3GPP TSG-RAN WG2 Meeting #131bis	R2-2507462

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.6.3

Source:	Nokia

Title:	On the open MAC issues for Rel-19 LTM

WID/SID:	NR_Mob_Ph4-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#130 the following was agreed for Conditional LTM:

Agreements on C-LTM - Upon CLTM execution, if TA is valid (i.e. TAT timer is running) at the first available CG occasion, and there is CG resource configured, RACH-less CLTM will be performed. - During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM.

In RAN2#131 the following was agreed for Conditional LTM:

Agreements on C-LTM - For C-LTM, if the UE receives more TAs beyond its capability, it’s up to UE implementation to decide which one is released

In this document we present our views about selected MAC issues that have been identified post-RAN2#131 meeting.

# 2	Discussion

## 2.1 	On the deactivation of target cell’s SP CSI-RSs used for L1-RSRP measurements (MAC-N02)

For the deactivation of SP CSI-RSs of candidate cells that were activated by the source cell, the following agreements were made:

- At RAN2#130: The UE deactivates SP CSI-RS resources of candidate cells (other than the target cell) after cell switch. FFS for the target cell. - At RAN2#131: The SP CSI-RS resources of the target cell are deactivated after CSI reporting in the first UL transmission. How this is captured in MAC is up to the MAC CR rapporteur.

The agreement text provided above appears to cover both the CSI-RSs used for L1-RSRP measurements and early CSI acquisition. However, it is important to note that RAN1 has specified two distinct features based on SP CSI-RSs:

- L1-RSRP measurements (RRM measurements):

  - These measurements are performed using one or two-port CSI-RSs (i.e., CSI-RSs used for beam management).

  - Measurements are reported to the serving cell to support LTM procedures (e.g., early DL/UL synchronization, cell switch, and/or target beam selection for cell switch).

  - Reporting is triggered by configurations provided by the serving cell (via CSI-MeasConfig of ServingCellConfig). Each candidate cell is associated with its own reporting configurations in its respective candidate cell configuration.

  - After a cell switch, the UE is expected to use measurement RSs associated with the reporting configuration included in the candidate cell configuration of the newly selected target cell.

  - The new serving cell may activate SP CSI-RS resources associated with its own reporting configurations.

  - There is no CSI report to the target cell for L1-RSRP measurements based on SP CSI-RSs activated by the previous source cell.

- Early CSI acquisition:

  - The UE performs measurements using multi-port CSI-RSs (configured separately from the CSI-RSs used for L1-RSRP measurements) and acquires CSI (e.g., CQI, PMI, RI, CRI).

  - Such measurements are directly reported to the target cell after reception of the cell switch command.

  - The LTM CSI report configurations for CSI acquisition for a candidate cell are configured separately in the respective LTM Candidate IE of each candidate cell.

  - For these measurements, the UE sends a CSI report to the target cell after receiving the cell switch command.

As per currently available RAN2 agreements, the deactivation of SP CSI-RSs associated with the target cell applies to CSI-RSs configured for CSI acquisition, since a CSI report, based on SP CSI-RSs activated by the previous source cell, is sent to the target cell. However, for L1-RSRP measurements based on SP CSI-RSs activated by the previous source cell, no report is sent to the target cell. Without further explicit clarification, this implies that SP CSI-RSs of the target cell used for L1-RSRP measurements may not be deactivated after a cell switch, leading to ambiguity.may not be deactivated after a cell switch, leading to ambiguity.may not be deactivated after a cell switch, leading to ambiguity.

Observation 1: There is no CSI report to the target cell for L1-RSRP measurements based on SP CSI-RSs activated by the previous source cell.

Observation 2: Without additional explicit clarification made in 3GPP, SP CSI-RSs for the target cell activated by the previous source cell for L1-RSRP measurement will not be deactivated, since the RAN2#131 agreement on deactivation (quoted above) was explicitly linked to the CSI reporting after the cell switch.

Proposal 1: Add a clarification that for L1-RSRP measurements, the UE deactivates SP CSI-RS resources of the target cell after cell switch.

## 2.2 Early synchronization for Conditional LTM

### 2.2.1 UE and network alignment on UE-maintained Tas (MAC-K02)

In an example scenario, the network may have already triggered early TA acquisition and provided corresponding TAs to the UE as many times as the UE capability. If the network decides to trigger early TA acquisition for another candidate and provide the estimated TA to the UE, the UE will have to discard one of the available TA values.

Leaving up to UE implementation which TAs to keep when the UE capability is reached creates ambiguity on the network side on what TAs are maintained by the UE. A consequence of this is that, for example, when the target/new serving cell receives the list of TAs and remaining TATs from the source, it does not know which of them are actually maintained at the UE, if they are more than UE capability. Therefore, it cannot know whether it should trigger early TA acquisition for the UE.

One way forward discussed during RAN2#131 to avoid ambiguity is that the NW triggers early UL sync only if the UE capability is not maxed out. This is very restrictive, especially considering that the UE maintains TAs and corresponding TATs upon cell switch. It is, therefore, important for RAN2 to agree on a mechanism that allows the NW to know which TAs are maintained by the UE, without over-restricting the possible network actions. To this end, some approaches have been proposed, like specifying the UE behavior by defining a rule about which TAs to maintain or signaling the maintained TAs. It is our view, that the former approach, regardless of the specific rule, has by far the lowest specification impact and does not add to UE complexity.

Proposal 2: Discuss on a method to enable UE and network alignment on the UE-maintained TAs, with the definition of a TA maintenance rule being the preferred option, due to its minimal specification and UE complexity impact.

## 2.3 Conditional LTM cell switch

### 2.3.1 CLTM execution

#### 2.3.1.1 TA handling when the serving cell is a candidate cell (MAC-V05, MAC-K01, MAC-N01)

As stated in the Introduction section, regarding the handling of the time alignment timers (TATs) of CLTM candidate cells, the following agreements were made in RAN2#129bis:

- Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and keep the early TA timer running. - C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running and starts the PTAG using the remaining time from the C-LTM timer.

These agreements define the UE handling of TATs of candidate cells (target and other candidate cells) and the PTAG TAT of the new serving cell, but they do not address if and how the TA and corresponding TAT of the previous serving cell should be used. Therefore, with the current agreements, the TA of the previous serving cell and the status of the corresponding TAT are not maintained after the cell switch. In case the previous serving cell is configured as a CLTM candidate and a subsequent conditional cell switch to that cell is triggered, the UE will have to obtain the TA again, performing an unnecessary random-access procedure either during RACH-based conditional cell switch or during early TA acquisition. We also note that such UE behavior is inconsistent with the one agreed for all other candidate cells, where the available TAs and corresponding TATs are maintained upon cell switch.

Observation 3: Upon a conditional cell switch, the TA and the corresponding TAT of the previous serving cell are not maintained, potentially leading to an unnecessary random-access procedure, in case that cell is a CLTM candidate.

The issue is depicted in Figure 1, where an example scenario with two CLTM candidates, with one of them corresponding to the current serving, is considered.

Figure 1: Issue with handling the TA of the previous serving cell.

In the current MAC open issues list for CLTM, the possibility of re-using current serving cell (source cell) TA, as well as the PTAG TAT as CLTM TAT after cell switch is discussed (MAC-V05, MAC-K01, MAC-N01), if current serving cell is configured as CLTM candidate target cell. The aim of such an approach is that the maintained TA and the CLTM TAT can be used for a potential subsequent CLTM cell switch to that cell.

A possible way to accomplish this is that, if the previous serving cell is configured as a CLTM candidate, upon cell switch, the UE stores the TA of the previous serving cell as the TA of the corresponding candidate configuration and starts the corresponding candidate TAT with the remaining time of the TAT of the PTAG of previous serving cell. An alternative solution is that the UE always maintains the TA and corresponding TAT of the candidate configuration corresponding to the serving cell by duplicating the TA and the status of the corresponding TAT of the serving cell. For example, when the UE receives a TAC MAC CE for the PTAG of the serving cell, in addition to the actions related to the serving cell TA and TAT, it should also update the TA and restart the TAT of the corresponding candidate configuration.

From the two alternatives considered, Alt. 1 is preferred, as it has a lower implementation complexity, since the UE does not need to maintain the candidate TAT corresponding to the serving cell while being served by it. A TP showing the required changes in the specification (in the branch treating the case when the CLTM target is not configured with two TAGs) is shown in Appendix A.1.

Proposal 3: UE stores the TA of the previous serving cell as the TA of the corresponding candidate configuration and starts the corresponding candidate TAT with the remaining time of the TAT of the PTAG of previous serving cell.

# 3	Conclusion

This document has made the following observations:

Observation 1: There is no CSI report to the target cell for L1-RSRP measurements based on SP CSI-RSs activated by the previous source cell.

Observation 2: Without additional explicit clarification made in 3GPP, SP CSI-RSs for the target cell activated by the previous source cell for L1-RSRP measurement will not be deactivated, since the RAN2#131 agreement on deactivation (quoted above) was explicitly linked to the CSI reporting after the cell switch.

Observation 3: Upon a conditional cell switch, the TA and the corresponding TAT of the previous serving cell are not maintained, potentially leading to an unnecessary random-access procedure, in case that cell is a CLTM candidate.

And proposed the following:

Proposal 1: Add a clarification that for L1-RSRP measurements, the UE deactivates SP CSI-RS resources of the target cell after cell switch.

Proposal 2: Discuss on a method to enable UE and network alignment on the UE-maintained TAs, with the definition of a TA maintenance rule being the preferred option, due to its minimal specification and UE complexity impact.

Proposal 3: UE stores the TA of the previous serving cell as the TA of the corresponding candidate configuration and starts the corresponding candidate TAT with the remaining time of the TAT of the PTAG of previous serving cell.

# Appendix

## A.1	TP for TA handling when the serving cell is a candidate cell

In the following, we provide a text proposal including the required changes in the MAC specification for handling the TA of the previous serving cell upon a CLTM cell switch:

5.2 Maintenance of Uplink Time Alignment

…

1>    when a conditional LTM cell switch procedure is triggered for a CLTM candidate cell or indicated by upper layer as specified in clause 5.y.3:

2>    if the CLTM candidate cell is not configured with two TAGs and the ltm-Candidate-TimeAlignmentTimer associated with the CLTM candidate cell is running as specified in clause 5.2x:

3>    if the (previous) serving cell is configured as an LTM candidate:

4>    store the TA value of the PTAG for the LTM candidate corresponding to the (previous) serving cell

4>    start or restart the ltm-Candidate-TimeAlignmentTimer associated with the LTM candidate corresponding to the serving cell with the length of the remaining time of the TimeAlignmentTimer of the PTAG

3>    apply the stored TA value associated with the CLTM candidate cell for the PTAG as specified in clause 6.1.3.4x;