3GPP TSG-RAN WG2 Meeting #131bis	R2- 2507463

Prague, Czech Republic, Oct. 13th-17th

Source:	CEWiT

Title:	6G Mobility Framework

Agenda item:	10.4

Document for:	Discussion

# Introduction

Below are the points from SID as drafted in RAN#108:

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

a) Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

b) AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

i. LCM procedures [RAN2, RAN1, RAN3, RAN4]

ii. Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

- Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

a) 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

b) Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4]

NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study.

c) Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

# Discussion

6G mobility framework is proposed to be built over the existing robust framework of 5G. Here we focus on below points:

•   How we could pick the components (such as early synchronization, AI/ML for mobility study outcomes, etc.) from 5G and incorporate it in 6G

•  How we could merge the existing separate procedures to form a single procedure to reduce the specification complexity.

•    How we could fill the gaps in the existing procedure and make the procedure complete

In this section we will describe some of the observations from 5G and how want it to be in 6G.

Legacy handover from Rel-15 handover makes use of filtered layer 3 measurements. This can be useful for reliable decision making with lesser ping pongs. An enhancement was brough in during Rel-18 where the mobility decision was taken based on L1 measurements. L1 measurements on the other hand were more instantaneous and hence could be used for more robust decision making. However, since filtering is optional for L1 measurements, it is possible that there could be ping pongs. However, if the handover is latency-tolerable, it is not a problem to pursue this direction. Hence it is proposed to keep all of these options in 6G.

Proposal 1: Both L1 and L3 measurement-based mobility to be supported in 6G

In legacy handovers of 5G (Rel-15) the network-initiated handovers and decision making was assumed to happen in RRC. The same was extended to MAC when LTM came in. This implementation flexibility is still expected in 6G. However, the UE needs to look out for only 1 message/indication from the network, unlike in 5G where there are multiple possibilities the UE must look out for. This simplifies the mobility procedure in 6G. Similarly all the UE-initiated enhancements that were introduced in Rel-16 and beyond, are also expected to continue being supported in 6G since it gives the UE a fallback route to stay in connected mode when the source cell is dying too fast. This way the UE will not have to do a cell-reselection.

Proposal 2: MAC/RRC based HO decisions with a single mechanism to indicate the decision to the UE are to be supported in 6G. Both UE-initiated and network-initiated handovers can be supported in 6G

Early synchronization is a very interesting feature that was introduced in Rel-18 as a part of LTM. This could be seen as a pluggable module. It could function well with the legacy L3 handovers as well. This saves a lot of interruption time associated with the legacy handovers since the major contribution is from the RACH process.

Proposal 3: Early synchronization should be supported in 6G

As proposed by various companies, 6G should support AI/ML from Day 1. As for this agenda item, the study on AI/ML for mobility in 5GA was directed towards 2 major goals – measurement reduction, handover performance improvement. If we incorporate these goals into 6G, we could have a good robust system where there is a good optimization of good enough measurements for timely decisions and resource utilization.

Proposal 4: Native AI/ML support for 6G mobility, with 5GA Rel20 as baseline

As there are Xn and N2 mode handovers, we would need 6G handovers with the same options. We need handovers where core involvement comes after the UE attaches to the target cell. This reduces interruption time. However, we also need mechanisms where brute-force handling is required, ifor instance, in the absence of Xn-like interface. Also the security key handling in Xn-like handovers needs to be better than the existing one. This is because there needs to be an internal handover again to bring about the key separation. We could resolve this issue by carefully architecting the security key derivation.

Proposal 5: Inter-gNB handovers could happen with or without direct interface between gNBs

Service continuity is the basis of user experience which becomes more important in 6G. Inter-RAT mobility can only be lossless in 5G. In 6G we could focus more on seamless, interruption free inter-RAT handovers. This becomes crucial if there are coverage holes in the system, and there is a possibility of more inter-RAT handovers. The same requirement goes with NTN-TN mobility as well. While we understand that all QoS cannot be met during these handovers, it would be useful to do a study on various mechanisms and the best possible QoS which could be included to have service continuity.

Proposal 6: Service continuity in Inter-RAT mobility to be supported for 6G-5G mobility

In 5G, there are no explicit clauses which instruct the UE to skip certain procedures and simplify the handover even if the target cell is a current serving cell. In 6G, inclusions of procedures such as serving cell swap could reduce the interruption time further. Currently there are several different procedures for network-initiated handovers (such as L3-based handover, LTM) as well as for UE-initiated handovers (such as CHO, CPAC, CLTM, etc.). We could simplify them and have a common mobility framework to reduce the complexity of the specification.

Proposal 7: Simplified handover procedures for handover between already connected serving cells such as primary and secondary cells

# Conclusion

In this contribution, we provided our proposals on 6G measurement and mobility as follows:

Proposal 1: Both L1 and L3 measurement-based mobility to be supported in 6G

Proposal 2: MAC/RRC based HO decisions with a single mechanism to indicate the decision to the UE are to be supported in 6G. Both UE-initiated and network-initiated handovers can be supported in 6G

Proposal 3: Early synchronization should be supported in 6G

Proposal 4: Native AI/ML support for 6G mobility, with 5GA Rel20 as baseline

Proposal 5: Inter-gNB handovers could happen with or without direct interface between gNBs

Proposal 6: Service continuity in Inter-RAT mobility to be supported for 6G-5G mobility

Proposal 7: Simplified handover procedures for handover between already connected serving cells such as primary and secondary cells

# References

- RAN2 #131bis Agenda v04

- RP-251881, New SID: Study on 6G Radio, June 9-13, 2025;

- TR 22.870, Study on 6G Use Cases and Service Requirements.