3GPP TSG RAN WG2 Meeting #131bis                          	   R2-2507464

Prague, Czech Republic, 13th – 17th October 2025

Agenda item:	8.5.2

Source:	Qualcomm Incorporated

Title:	Discussion on remaining MAC issues

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

In this contribution, we discuss the remaining MAC design issues.

# Discussion

## F_id for PRACH adaption

At RAN2 #130, possible collision issue of RA-RNTI for PRACH adaptation the following agreements were made [1].

Observation 1. It is not clear how f-id is used for the additional PRACH resources without any update in MAC spec.

Proposal 1. Add the following note for RA-RNTI in MAC spec:

- f-id is used for indicating the indexes of valid additional ROs and valid legacy ROs while overlapping in time.

- The index starts with the legacy ROs.

## OD-SIB1 for RedCap UEs

The following issue was brought up after the post meeting email discussion [POST131][111][NES] 38.321 CR.

Whether "initialDownlinkBWP-RedCap" can be configured before SIB1 request procedure (e.g. acquired by UE earlier) and can be used for PDCCH monitoring. If so, can the network know that for a redcap UE performing an OD-SIB1 request.

The initialDownlinkBWP-RedCap information element is included in SIB1 for RedCap UEs, which specifies the initial downlink bandwidth part (BWP) for the random access procedure for these devices, e.g., monitoring the RAR during RACH procedure. For OD-SIB1, RedCap UEs may use the DL BWP indicated by initialDownlinkBWP-RedCap for monitoring the RAR responding to its OD-SIB1 request.

Observation 2. To acquire OD-SIB1, a RedCap UE needs to know its initial DL BWP for monitoring the RAR responding to its OD-SIB1 request.

To support RedCap UEs with OD-SIB1, gNB needs to know that an OD-SIB1 request is transmitted from a RedCap UE and thus can transmit the RAR within the DL BWP indicated by initialDownlinkBWP-RedCap.

Observation 3. To support RedCap UEs with OD-SIB1, gNB needs to know that an OD-SIB1 request is transmitted from a RedCap UE.

To support RedCap UEs with OD-SIB1, RedCap UE’s specific parameters such as UL WUS configuration and initialDownlinkBWP-RedCap need to be included in UL WUS configuration. It’s too late to open any further design for RedCap UEs for OD-SIB1.

Therefore, we propose the following proposal for not supporting RedCap UE with OD-SIB1.

Proposal 2: Do not support RedCap UEs with OD-SIB1.

# Conclusion

In this contribution, we discussed the remaining MAC design issues and concluded with the following observations and proposals.

F_id for PRACH adaption

Observation 1: it is not clear how f-id is used for the additional PRACH resources without any update in MAC spec.

Proposal 1: add the following note for RA-RNTI in MAC spec:

- f-id is used for indicating the indexes of valid additional ROs and valid legacy ROs while overlapping in time.

- The index starts with the legacy ROs.

OD-SIB1 for RedCap UEs

Observation 2. To acquire OD-SIB1, a RedCap UE needs to know its initial DL BWP for monitoring the RAR responding to its OD-SIB1 request.

Observation 3. To support RedCap UEs with OD-SIB1, gNB needs to know that an OD-SIB1 request is transmitted from a RedCap UE.

Proposal 2: Do not support RedCap UEs with OD-SIB1.

# References

- “R2_130_R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM”, RAN2 #130 Malta, May 19th – 23rd, 2025.