3GPP RAN WG2 Meeting #131bis	R2-2507475

Prague, Czech Republic, October 13th – 17th 2025

Agenda Item:	8.1.2.2

Source:	InterDigital

Title:		Discussion on Applicability-related RILs: [E041], [E042], [C083], [C084], [Z001], [Z002], and [H010]

Document for:	Discussion, Decision

# Introduction

A RAN2 objective for AI/ML for Air Interface Work Item [1] is to design a general framework for one-sided AI/ML, including the signaling and protocol aspects of Life Cycle Management (LCM) to enable functionality selection, activation, deactivation, switching and fallback.

A list of remaining issues related to implementation of the LCM procedure has been provided as part of the AIML ASN.1 review [2] [3]. This contribution discusses these remaining issues, specifically those related to UE sided model for BM use case.

# Remaining Issues marked ‘ToDo’

The following issues and descriptions have been extracted from the AIML issues list [2] [3] as part of the ASN.1 review, and listed as “ToDo” by the TS 38.331 specification Rapporteur.

## [E041] Associate ID validity

Once an Associate ID is assigned to a particular deployment, there is no way for the network to reset/reassign the value to a different deployment in the future. This can cause challenges if hardware is replaced, IDs lost in failures/resets, or legacy mappings simply become invalid. The proponents note even though rare, such events can occur, and without a reset mechanism the UE and network may hold different interpretations of the same ID.

If the network chooses to update the Associated ID the new mapping must also be conveyed to the UE to avoid mismatch. Considering Associated ID is used for training purposes, such information may be stored (e.g., in a UE-side server) for an indefinite amount of time. To ensure no misunderstanding, the network must ensure that every UE which has ever collected data associated to that Assocaited ID must be informed of the change. This introduces many issues like privacy/security, signaling overhead, and general complexity.

Assigning a time limit to an Associated ID may circumvent this, however introduces a new set of problems. Aside from all the possible challenges of continuous model re-trainign and updates, Associated ID expiry would trigger data collection for the newly assigned value which may introduce signaling overhead. Furthermore, AIML operation would effectively be suspended on all cells with newly assigned Assocaited IDs until a model is retrained with considering the updated Assocaited ID.

The network already possesses considerable flexibility in how Assocaited ID values are assigned, and since the Associated ID space is 24 bits, a significant number of unique deployment options are already supported. It is therefore proposed that E041 be marked as “PropReject”

Proposal 1:	[E041] is marked as “PropReject”.

## [E042] Dedicated Flag to configure applicability reporting via UAI

RAN2#130 agreed the following:

“Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled.  Assume this applies to Option A and B, FFS if anything different needs to be done for option B (if specified)”

The current ASN.1 code captures the above agreement as follows:

OtherConfig-v19xy ::=                   SEQUENCE {

applicabilityReportConfig-r19                SetupRelease {ApplicabilityReportConfig-r19}                   OPTIONAL, -- Need M

dataCollectionPreferenceConfig-r19           SetupRelease {DataCollectionPreferenceConfig-r19}              OPTIONAL, -- Need M

loggedDataCollectionAssistanceConfig-r19     SetupRelease {LoggedDataCollectionAssistanceConfig-r19}        OPTIONAL  -- Need M

}

…

ApplicabilityReportConfig-r19 ::= SEQUENCE {

reportApplicabilityUAI-r19     ENUMERATED {true} OPTIONAL, -- Need R

applicabilityConfigList-r19    SEQUENCE (SIZE (1..maxNrofServingCells)) OF ApplicabilityConfig-r19                  OPTIONAL, -- Need R[RIL]: C083, AIML

...

}

…

applicabilityReportConfig Configuration for the UE to indicate the applicability of configurations subject to the applicability determination procedure.

applicabilitySetConfigId Indicates the ID of a set of prediction related parameters.

…

reportApplicabilityUAI If present, the field indicates the UE shall report applicability in UEAssistanceInformation message.

[E042] notes it was not discussed how to combine the flag enabling applicability reporting via UAI in ASN.1 for option B configurations, suggesting the following options:

- Remove the separate flag reportApplicabilityUAI-r19 and assume that applicapility reporting via UAI is implicitly configured when applicabilityReportConfig is included in otherConfig.

- Keep the flag reportApplicabilityUAI-r19 within applicabilityReportConfig, as it is.

- Move the flag reportApplicabilityUAI-r19 one level up, directly under otherConfig.

Although not explicitly discussed, the current implementation seems a reasonable reflection of the current RAN2 agreement. There seems no strong motivation to change the current text.

Proposal 2:	[E042] reportApplicabilityUAI is kept within applicabilityReportConfig as it currently is.

## [C083/C084] AddModList for applicabilityConfigList and applicabilitySetConfigList

[C083] notes “applicabilityConfigList-r19” is currently “Need R”, meaning a new list is provided every time UE receives the configuration. As a consequence, to update one entry for one cell the network must provide the whole list for all serving cells, otherwise the other entries for other cells will be deleted. It is further noted that change based applicability reporting is actually not implemented according to current procedural text since a UE has to report applicability status for each serving cell upon receiving this applicabilityConfigList-r19. To address these issues, the proponent suggests to adopt an AddModList structure for applicabilityConfigList-r19.

As noted by the proponent, the current implementation has several issues which can be resolved by adoption of an AddModList structure (which has obvious benefits from a flexibility and signaling overhead perspective). It is proposed that C083 be marked as “PropAgree”.

Proposal 3:	[C083] is marked as “PropAgree”.

[C084] notes that similar issues also exist for “applicabilitySetConfigList-r19”, and proposes similar changes to an AddModList structure. By the same reasonine as above, it is proposed that C084 be marked as “PropAgree”.

Proposal 4:	[C084] is marked as “PropAgree”

## [Z001/Z002] Release preference for inapplicable functionality

Applicability reporting via RRCReconfigurationComplete is currently implemented as below:

4> for each entry within applicabilitySetConfigList associated with the concerned serving cell, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation): 5> include an entry in the applicabilityInfoReportList and set the content as follows: 6> set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId; 6> set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId; 6> if the applicabilityStatus is set to inapplicable: 7> if the UE prefers to release the concerned ApplicabilitySetConfig, include releaseConfigurationPreference;

[Z001] notes that for the functionality provided in OtherConfig whose applicability has changed, the there is no harm in the UE maintaining the configuration. There is thus no need for UE to provide releaseConfigurationPreference in RRCReconfigurationComplete, and the corresponding procedural text above should be removed.

According to current ASN.1 implementation, including releaseConfigurationPreference is already optional. If the UE does not want to release the configuration in OtherConfig, it may simply not include the preference when reporting inapplicability. This is further supported by the procedural text, wherein upon applicability reporting with applicability status set to inapplicable, the UE may include this flag “if the UE prefers to release the concerned ApplicabilitySetConfig”. This leaves the possibility that the UE may always not include it if it prefers not to.

Proposal 5:	[Z001] is marked as “PropReject”.

[Z002] addresses a similar issue for applicability reporting via UAI. For similar reasons as above, it is proposed that this also be rejected, and if the UE does not wish to release the configuration upon reporting inapplicable it may simply not include the release flag.

Proposal 6:	[Z002] is marked as “PropReject”.

## [H010] Report configuration type in ApplicabilitySetConfig

ApplicabilitySetConfig-r19 is captured in ASN.1 as follows:

ApplicabilitySetConfig-r19 ::= SEQUENCE {

applicabilitySetConfigId-r19                ApplicabilitySetConfigId-r19                            OPTIONAL,   -- Need R [RIL]: N030 AIML, [RIL]: N035 AIML

resourcesForChannelMeasurement              CSI-ResourceConfigId                                    OPTIONAL,   -- Need R

resourcesForChannelPrediction-r19           CSI-ResourceConfigId                                    OPTIONAL,   -- Need R

associatedIdForChannelMeasurement-r19       AssociatedId-r19                                        OPTIONAL,   -- Need R

associatedIdForChannelPrediction-r19        AssociatedId-r19                                        OPTIONAL,   -- Need R

reportQuantity-r19                          ReportQuantity-r19                                      OPTIONAL,   -- Need R

reportConfigType                        CHOICE {[RIL]: H010 AIML

periodic                                SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH                   SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH                   SEQUENCE {

reportSlotConfig                        ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

reportSlotOffsetList                SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32),

p0alpha                                 P0-PUSCH-AlphaSetId

},

aperiodic                               SEQUENCE {

reportSlotOffsetList                SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32)

}

}                                                                                                   OPTIONAL,   -- Need R

nrofReportedPredictedRS-r19                 ENUMERATED {n1, n2, n3, n4}                             OPTIONAL,   -- Need R

nrofTimeInstance-r19                        ENUMERATED {n1, n2, n4, n8}                             OPTIONAL,   -- Need R

timeGap-r19                                 ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1}          OPTIONAL,   -- Need R

...

}

[H010] explains that reportConfigType in ApplicabilitySetConfig-r19 needs to only indicate the type of the report, i.e. periodic, semi persistent on PUCCH/PUSCH or aperiodic. Additional information such as reportSlotConfig etc. is not required for the UE to determine applicability, so is not needed in this configuration.

As noted by TS 38.331 specification Rapporteur in [2], the current implementation is based on the following RAN1 agreement, which assumes that legacy parameters are reused:

For option B of applicability check, RAN 1 assumes that at least the following RRC parameters are to be reused: - For both BM-Case 1 and BM-Case 2: - associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19 - For BM-Case 2: - TimeGap-r19, nroftimeinstance-r19, - Note: this doesn’t imply the associated ID is always present

From RAN2 perspective, the legacy reportConfigType parameter contains unnecessary information for applicability determination and the proposal in [H010] is correct. However, by strictly following RAN1 agreement above the legacy parameter for reportConfigType is to be used including all subparameters. Unless there is strong motivation in RAN2 (which doesn’t seem to be the case as both implementations work and overhead savings are minimal) there seems not enough motivation to overturn an explicit RAN1 agreement. It is therefore proposed [H010] is marked as “PropReject”.

Proposal 7:	[H010] is marked as “PropReject”.

# Conclusion

In this contribution the following proposals are made regarding issues identified during the AIML ASN.1 review:

Proposal 1:	[E041] is marked as “PropReject”.

Proposal 2:	[E042] reportApplicabilityUAI is kept within applicabilityReportConfig as it currently is.

Proposal 3:	[C083] is marked as “PropAgree”.

Proposal 4:	[C084] is marked as “PropAgree”

Proposal 5:	[Z001] is marked as “PropReject”.

Proposal 6:	[Z002] is marked as “PropReject”.

Proposal 7:	[H010] is marked as “PropReject”.

# References

RP-243244 – WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface – Qualcomm

AIML Comments file v029 – Rel-19 CR ASN.1 Review (2025-09)

AIML Review file v029  – Rel-19 CR ASN.1 Review (2025-09)