3GPP TSG-RAN WG2 Meeting #131bis	R2-2507493

Prague, Czech, 13-17 Oct, 2025

Source: 	Huawei, HiSilicon

Title: 	Remaining issues on LTE TN to NR NTN mobility

Agenda Item:	8.8.5

Document for:	Discussion and decision

# 1	Introduction

In RAN2#129bis meeting[1], the following agreements were made.

Agreements: 1. RAN2 confirms that redirection from LTE TN to NR NTN is not supported before Rel-19. 2. Extend the capability signaling for E-UTRA TN to NR NTN redirection to also indicate the support of IRAT E-UTRA TN to NR NTN cell reselection. E-UTRAN that broadcasts NR satellite assistance info in SIB33 may configure dedicated priority for an NR NTN frequency, only if it receives this capability common to E-UTRA TN to NR NTN redirection and cell reselection (based on NW implementation). 3. The redirection to NR NTN information element is enclosed in an NTN specific CarrierInfoNR, which is introduced in the choice structure as nr-19.

In this paper, we further discuss the remaining issue on LTE TN to NR NTN mobility postponed in the last meeting.

# 2	Discussion

In RAN2#131 meeting[2], the remaining issue on LTE TN to NR NTN mobility were left open for discussion:

- The smtc-r19 is absent in the CarrierInfoNR-r19 Proposal 1: The following options can be considered for UE to determine SMTC when the smtc-r19 is not present in the CarrierInfoNR-r19. - Option 1: UE uses the SMTC configured in the SIB24 when the smtc-r19 is not present in the CarrierInfoNR-r19. - Option 2: Add the sentence ‘In this release, the smtc is mandatory present in the CarrierInfoNR-r19’ in the field description. - Oppo prefers option 2. ZTE and Samsung agree and thinks this apply also to the corresponding TEI19 - vivo wonders if this is needed and we could live this to NW implementation. Ericsson and HW agree. CATT agrees Can come back in the next meeting - UE adjusts the SMTC according to the actual propagation delay Proposal 2: For the smtc-r19 in the CarrierInfoNR-r19, the UE should adjust the offset according to the actual propagation delay - Oppo/ZTE support. - Ericsson thinks we should have this discussion in general not only for NTN - After the discussion ZTE could agree this is not essential Can come back in the next meeting

Before addressing the specific open issues, it should be noted, in general, no further enhancements to the RRCRelease message should be discussed, as these kind of enhancements were not considered even for NTN–NTN mobility during the Rel-17 or Rel-18 discussions.

With respect to the specific issues raised in the previous meeting that remain open, our views are as follows

First issue is whether the UE should adjust the offset according to the actual propagation delay for the smtc-r19 is not present in the CarrierInfoNR-r19. In our views, there is no need for UE to adjust the offset. First it should be noted that the RRCRelease message is a dedicated UE message, and given the fact that the LTE TN coverage is quite small, the gNB can configure appropriated SMTC for the UE in RRCRelease message. Hence we propose

Proposal 1: RAN 2 agrees that there is no need UE should adjust the offset by itself.

Second issue is whether the smtc-r19 is mandatory present in the CarrierInfoNR-r19. In our view on this is that we can simply leave it to network implementation whether to include the smtc in the CarrierInfoNR-r19 or not.

Proposal 2:  RAN 2 agrees that it is up to the network implementation whether to include the smtc in the CarrierInfoNR-r19 or not.

# 3	Conclusion

In this contribution the following proposals are made for LTE to NR NTN mobility:

Proposal 1: RAN 2 agrees that there is no need UE should adjust the offset by itself .

Proposal 2: RAN 2 agrees that it is up to the network implementation whether to include the smtc in the CarrierInfoNR-r19 or not.

# 4	References

- R2-2502983, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 129bis, Wuhan, China, Apr 7th – 11th, 2025

- R2-2506203, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 131 Bangalore, India, Aug 25th – 29th , 2025