3GPP RAN WG2 Meeting #131bis	R2-2507507

Prague, Czech Republic, Oct. 13th – 17th, 2025

Agenda Item:	8.11.3

Source:	InterDigital, Inc.

Title:	RRC issues on SBFD

# 1. Introduction

In this contribution, we discuss a necessity of including early indication via MSG3 as RRC open issue.

# 2. Discussion

Early indication via MSG3

In RAN2#128 meeting [1], RAN2 discused this issue, however, RAN2 has not reached a conclusion until now.

In RAN1#120bis meeting, the following agreement was made [2]:

Agreement For CBRA, if a SBFD-aware UE transmits Msg1 in legacy-RO, the UE follows the legacy behavior during the random access procedure. - FFS: Whether specification change is necessary

When a SBFD-aware UE transmits MSG1 with legacy RO, then the UE may transmit MSG3 upon reception of MSG2. When the UE transmit the MSG3, then the UE can indicate the support of SBFD operation via the MSG3 (i.e., early indication) during the random access procedure.

This approach can be beneficial for SBFD-aware UE in RRC-IDLE/INACTIVE states, where the UE can decide on whether to connect to the selected cell based on SBFD operation or connect to the selected cell based on non-SBFD operation. For example, in case the measured CLI (based on received SSB) is low, the UE could connect to the cell based on SBFD operation. Otherwise, if the measured CLI (based on received SSB) is high, the UE could connect to the cell based on non-SBFD operation.

In another example, the UE can transmit random access preamble in ROs that coincide with SBFD symbols (e.g., via additional-ROs). As such, upon reception of the preamble, the gNB will recognize that the UE is SBFD-aware, and that the UE is operating based on SBFD operation. So, gNB can further transmit RAR in SBFD symbols. The SBFD-aware UE that has decided to perform RA based on legacy-ROs based on conditions of ROs selection, could indicate its support for SBFD operation later and as part of Msg3.

Since RAN2 has not reached a conclusion, also RAN1 made agreement (i.e., FFS whether specification change is necessary), RAN2 should discuss further to reach a conclusion regarding this issue as one of the RRC open issues.

Proposal 1. RAN2 should consider as RRC open issue whether to indicate SBFD support via MSG3 after MSG1 transmission via legacy RO.

# 3. Conclusion

Proposal 1. RAN2 should consider as RRC open issue whether to indicate SBFD support via MSG3 after MSG1 transmission via legacy RO.

# 4. References

[1] Chairman’s notes, 3GPP TSG RAN WG2 #128

[2] Chairman’s notes, 3GPP TSG RAN WG1 #120bis.