3GPP TSG-RAN WG2 Meeting #131bis             			       R2-2507515

Prague, Czech Republic, Oct. 13th-17th

Agenda item:	10.3.3

Source:	TCL

Title:	Considerations on 6GR general AI/ML framework

Document for:	Discussion and Decision

1	Introduction

At the RAN Plenary #108 meeting, the new SID on “New SID: Study on 6G Radio” was approved [1]. One of the main objectives of the study relates to AI/ML for 6GR are as follows.

(8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this contribution, we will show our views on general AI/ML framework in 6GR.

2. Discussion

2.1 General AI/ML framework

In 5G-Advanced, different WGs have studied the AI/ML general framework from their respective perspectives.

RAN3 has defined a basic functional framework for RAN intelligence, as shown in Figure 1[2]. This framework illustrates the data and information flows and consists of four main components: data collection, model training, model inference, and the actor.

Figure 1. Functional Framework for RAN Intelligence

RAN1 further explored a general AI/ML framework for the NR air interface based on the functional framework defined by RAN3, as illustrated in Figure 2[3]. This framework provides technical support for use cases addressed by RAN1 and RAN2, such as CSI feedback enhancement, beam management, positioning accuracy enhancement, CSI compression, and AI/ML-based mobility.

Figure 2: Functional framework for AI/ML for NR Air Interface

SA5 has mainly focused on providing AI/ML management capabilities and services for the 5GS. In particular, it specifies in detail the lifecycle management of ML entities, is depicted in the Figure 3[4]. The workflow involves 3 main phases; the training, deployment and inference phase, including the main operational tasks for each phase.

Figure 3: AI/ML operational workflow

Based on the above discussion, it can be observed that in the 5G-Advanced phase the SA and RAN working groups have independently defined functional frameworks for AI/ML for their respective use cases. However, it will result the duplication of work, where the similar issues are repeatedly discussed and specified across multiple WGs. On the other hand, there is a risk of inconsistent terminology and conflicting specifications if overlapping AI/ML functions are defined independently

It is observed that several potential issues with the multiple AI/ML functional frameworks, including functional redundancy, a risk of specification conflicts, and low efficiency in end-to-end collaboration. Considering the development of 6G native AI may require an efficient underlying architecture, it is proposed that early in the 6GR study phase, a future-proof AI/ML functional framework should be supported, using the 5G-A AI/ML framework for NR Air Interface as a baseline.

In 5G-Advanced, RAN and SA have respectively defined their functional frameworks for AI/ML.

6GR should support a future-proof functional framework for AI/ML that ensures forward compatibility and extensibility, using the 5G-A AI/ML framework for NR Air Interface as a baseline.

RAN2 shall coordinate with RAN3 and SA WGs to study a unified AI/ML framework, to prevent overlapping work.

2.2 LCM procedures

LCM framework

The LCM procedure is studied for the case that an AI/ML model has a model ID with associated information and/or for the case that a given functionality is provided by some AI/ML operations. However, LCM in 5G-A is use-case specific, which will result in multiple parallel schemes designs for the same AI/ML operations, such as data collection, inference configuration and reporting, which will bring some additional complexity and burden to the wireless system. To mitigate this, it is preferred that 6GR shall study a unified LCM framework designed to support diverse use cases across different WGs in a harmonized manner.

6GR should study a unified LCM framework designed to support diverse use cases.

Data collection is an essential aspect serving various purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update. Study on AI/ML data collection in 5G-Advanced have investigated both Control Plane (CP) based solution and User Plane (UP) based solution. It is observed that these approaches rely on the reuse of existing approaches, which in turn lead to the use of separate, use-case specific mechanisms for data collection. For example:

- For the AI/ML-based BM use case, data collection is based on the legacy CSI framework.

- For the AI/ML-based positioning use case, the LPP protocol is utilized for data collection.

- For the AI/ML-based mobility use case, legacy RRM measurement procedures are reused for data collection.

One the other hand, the same AI/ML data may be collected multiple times for different AI/ML use cases. For example, L1 beam measurement results are applicable to both AI/ML-based beam management and AI/ML-based mobility.

Based on above discussion, we think that in order to improve efficiency for data collection, it is important to design a harmonized data collection framework for 6G AI/ML. Hence, 6GR should specify a unified data collection framework to streamline data collection, thereby improving efficiency and eliminating redundancy.

6GR should specify a unified data collection framework, to enhance the efficiency of data collection and prevent redundancy.

Model location for inference

In 5G-Advanced, it was agreed that model inference can be deployed at the UE side, the network side, and the two-sided. For example, in the RRM prediction use case, both UE-sided and NW-sided models were studied, while two-sided models are exploring in the AI/ML for CSI compression use case.

Moreover, during the recent RAN1#122 meeting [5], the following highlighted assumption about model location for inference for AI/ML in 6GR was reached.

Agreement For 6GR AI/ML use cases identification/categorization, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following: - Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model

Hence, from our perspective, it is better to align with RAN1’s meeting agreement about model location for inference, that is, 6GR LCM should support the UE-sided model, NW-sided model, and two-sided model for model inference.

6GR LCM should support model location for inference, including UE-sided model, NW-sided model, and two-sided model.

Non-AI/ML operation

In 5G-Advanced, fallback to non-AI/ML operation is regarded as a key component of the LCM procedure. For instance, such non-AI/ML operation may be used when the inference configuration is inapplicable, system performance degrades, or the AI/ML model/functionality is disabled or not supported. While the mechanism was not fully standardized in 5G-A, we consider it important that 6GR continues to support non-AI/ML operation, which is critical to maintaining system reliability.

6GR LCM should support non-AI/ML operation.

2.3 AI agent

In 5G-Advanced, it has been confirmed through extensive evaluation that AI/ML model can enhance communication efficiency. For example, in AI/ML-based mobility, the predicated RRM measurement results can improve handover performance and reduces measurement efforts.

Recently, it is widely recognized that AI agents play a key role in enabling intelligent automation in wireless communication system. SA2’s document [6] has provided the definition of AI agent:

Al Agent: an automated intelligent entity capable of e.g interacting with its environment, acquiring contextual information, reasoning, self-learning, decision-making, executing tasks (autonomously or in collaboration with other Al Agents) to achieve a specific goal.

To realize the 6G native AI of a fully autonomous network with ubiquitous intelligence. AI agents, as intelligent entities with perception, learning, reasoning, and execution capabilities, are well suited to fulfill this role. Therefore, it is proposed that 6GR should natively support AI agents on both UE side and NW side, to enhance improve system performance.

Al Agent: an automated intelligent entity capable of e.g interacting with its environment, acquiring contextual information, reasoning, self-learning, decision-making, executing tasks (autonomously or in collaboration with other Al Agents) to achieve a specific goal.

6GR should natively support AI agents on both UE side and NW side, to improve the system performance.

3	Conclusion

Based on the discussion in the previous sections we propose the following:

- In 5G-Advanced, RAN and SA have respectively defined their functional frameworks for AI/ML.

- 6GR should support a future-proof functional framework for AI/ML that ensures forward compatibility and extensibility, using the 5G-A AI/ML framework as a baseline.

- RAN2 shall coordinate with RAN3 and SA WGs to study a unified AI/ML framework, to prevent overlapping work.

- 6GR should study a unified LCM framework designed to support diverse use cases.

- 6GR should specify a unified data collection framework to enhance the efficiency of data collection and prevent redundancy.

- 6GR LCM should support model location for inference, including UE-sided model, NW-sided model, and two-sided model.

- 6GR LCM should support non-AI/ML operation.

- 6GR should natively support AI agents on both UE side and NW side, to improve the system performance.

References

[1] RP-251881, “New SID: Study on 6G Radio”, RAN#108, June 2025.

[2] TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

[3] TR 37.817, Study on enhancement for Data Collection for NR and EN-DC,

[4] TR 28.908, Study on Artificial Intelligence/Machine Learning (AI/ML) management

[5] RAN1#122, Chairman notes.

[6] TR 22.870, Study on 6G Use Cases and Service Requirements.