3GPP TSG-RAN WG2 Meeting #131bis		       R2-2507534

Prague, Czech Republic, Oct. 13th-17th

Agenda Item:	8.1.2.2

Source:	ASUSTeK

Title:	Discussion on open issues for LCM

Document for:	Discussion and Decision

- Introduction

According to the RIL review and the comments, we’d like to discuss some open issues suggested by the rapporteur.

- Discussion

2.1 UE-side data collection

[X003] Start indication for UE-side data collection

[Description]: The presence of dataCollectionCandidateConfigList can implicitly indicate UE is about to collect data. There is no need to intorduce separate indication. On the other hand, start indication is useless without dataCollectionCandidateConfigList, since NW is not aware of the required configuration. If UE has already obtained the required configuration for data collection, UE doesn’t need to indicate start indication alone. We propose to remove the dataCollectionStart.

In our opinion, the dataCollectionStart could be used as a start indication, and should not be removed. Of the 2 interpretations provided by the rapporteur:

- the UE sends just a start indication and then it expects to receive the list of candidates from the network, or

- the UE sends just a start indication with the meaning that it can accept any data collection configuration that the NW may decide to provide (that assumes the list of candidates is optional on top of the start/stop procedure).

The use of start indication is for when no candidate configurations are provided by the NW. The NW may not provide candidate configurations due to the following cases: (i) not expecting the UE to collect data, or (ii) not letting the UE to select configuration.

Interpretation (i) may be suitable for case (i). By not providing the list of candidate configuration initially, the NW can save signalling overhead and provide the latest candidate configurations when requested. The downside of this approach is that, when there are multiple UEs that request for data collection, signalling overhead may increase because every UE may need to first request for a start, then indicate the preferred configuration. However, this can be solved by providing candidate configurations initially.

Interpretation (ii) may be suitable for case (ii). One alternative solution is for the NW to provide only one candidate configuration, but the available data collection configuration at the NW may be dynamic and the NW may need to update the candidate configuration accordingly, increasing signalling overhead.

The understanding could be clarified in the field description of the dataCollectionStart.

Proposal 1: [X003] Clarify the usage of dataCollectionStart. The UE sends just a start indication and then it expects to receive the list of candidates from the network.

[E040] Ensure that UE does not request a candidate UE-side data collection configuration for which it already has a UE-side data collection configuration

[Description]: According to the current procedural text in 5.7.4.3, the UE may request a candidate UE-side data collection configuration in UAI (received in otherConfig), even if it already has a corresponding UE-side data collection configuration (received in CSI-ReportConfig) for that candidate configuration. This is not a desired behaviour and it should be fixed.

We agree that UE should not request an already configured UE-side data collection configuration, but think the NW can release the candidate configurations accordingly.

Proposal 2: [E040] The NW can release the candidate configurations associated with configured ones accordingly, no need to enhance.

2.2 Applicability reporting

[H003][N021] Configuration for UE data collection restructuring/ Remove the CHOICE hierarchy from the configuration for CSI prediction and beam prediction

[Description]: The extra hierarchy introduced by forcing a choice between predictionConfiguration-r19 and configurationForChannelMonitoring-r19 is unnecessary and does not have the advantage of grouping many parameters together. It should be obvious to the configuring entity that one or the other should be configured.

[Description]: This is related to the following EN captured ubnder CSI-ReportConfig information element code: Editor's Note: FFS whether/how to group the parameters (and whether/how to update the field descriptions) for prediction, monitoring, and UE-side data collection based on the beam management and CSI prediction use cases. Currently “configurationForChannelPrediction-r19” is used for both inference and UE data collection. However, the notation “ForChannelPrediction” and “predictionConfiguration-r19” suggests that this parameter is only for inference. We think it is better to add a new parameter for data collection (e.g. configurationForUE-DataCollection-r19) and make “configurationForChannelPrediction-r19” applicable only for inference

Considering the different combinations of parameters for different use cases and LCM operations (e.g., inference, monitoring, data collection), we think that grouping can be useful. But we don’t think a CHOICE structure is needed and may introduce confusion due to complexity and some inconsistent naming (e.g., csi-InferencePrediction-r19 and configurationForChannelPrediction-r19). However, the parameters for data collection and inference identified by RAN1 is basically identical and it is clearly differentiated by the reportQuantity, so we think there is no need for extra grouping.

Proposal 3: [H003][N021] remove the CHOICE structure and do not introduce grouping for data collection for CSI-ReportConfig for AI/ML.

[E042] Necessity of dedicated flag to configure applicability reporting via UAI

[Description]: RAN2#130 agreed the following: “2 Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled. Assume this applies to Option A and B, FFS if anything different needs to be done for option B (if specified)” After adding the sets of inference related parameters for option B in otherConfig, it was not discussed how to combine this flag in ASN.1 with the configurations for option B.

The following possible solutions are provided:

- Remove the separate flag reportApplicabilityUAI-r19 and assume that applicability reporting via UAI is implicitly configured when applicabilityReportConfig is included in otherConfig.

- Keep the flag reportApplicabilityUAI-r19 within applicabilityReportConfig, as it is.

- Move the flag reportApplicabilityUAI-r19 one level up, directly under otherConfig.

We think either solution is fine without issues, but solution 3) is a cleaner/clearer approach. Therefore, we support solution 3).

Proposal 4: [E042] Prefer solution 3) to move the flag reportApplicabilityUAI-r19 one level up, directly under otherConfig.

- Conclusion

Proposal 1: [X003] Clarify the usage of dataCollectionStart. The UE sends just a start indication and then it expects to receive the list of candidates from the network.

Proposal 2: [E040] The NW can release the candidate configurations associated with configured ones accordingly, no need to enhance.

Proposal 3: [H003][N021] remove the CHOICE structure and do not introduce grouping for data collection for CSI-ReportConfig for AI/ML.

Proposal 4: [E042] Prefer solution 3) to move the flag reportApplicabilityUAI-r19 one level up, directly under otherConfig.

References

[1] AIML Comments for RIL

Annex