3GPP TSG-RAN WG2 Meeting #131bis	R2-2507536

Prague, Czech Republic, Oct. 13th-17th

Agenda Item:	8.5.3

Source:			ASUSTeK

Title:	Discussion on RA-RNTI for PRACH adaptation

Document for: 	Discussion and Decision

- Introduction

In the email discussion, there are some open issues on NES to be discussed. In this contribution, we would like to highlight on the calculation of RA-RNTI for PRACH adaptation.

- Discussion

In RAN1 #120 meeting, it has been agreed that the additional RO is invalid when the additional RO is overlapped with legacy valid RO in both time-domain and frequency-domain. This agreement does not rule out the situation that the additional RO and legacy RO are overlapped in time-domain and not overlapped in frequency domain. On the other hand, note that the variables in RA-RNTI calculation include the index of the first OFDM symbol of the PRACH occasion (s_id), the index of the first slot of the PRACH occasion in a system frame (t_id), the index of the PRACH occasion in the frequency domain (f_id) and the UL carrier used for Random Access Preamble transmission (ul_carrier_id). That is to say, RA-RNTI collision could occur when the additional RO and legacy RO are overlapped in time-domain.

Observation 1: RA-RNTI collision could occur when the additional RO and legacy RO are overlapped in time-domain.

In RAN2#130 meeting, it was agreed that there was no need to extend the RA-RNTI space for additional PRACH occasions, and f_id for the additional PRACH resources could be reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs. However, in the LS provided by RAN1 [1], it is pointed out by RAN1 that a separate configuration of Msg1-FDM for the additional PRACH resources is supported. Moreover, the UE is not expected to be configured more than 8 FDM-ed valid ROs, including legacy + additional ROs. There seems to be efforts to be made by RAN2 to address this issue.

Option 1: Rely on network implementation.

Option 2: Revisit the previous agreement and revise the RA-RNTI formula.

For Option 1, the network could schedule time domain resources to forbit legacy RO and additional RO overlapping. Also, the network should not schedule legacy ROs and additional ROs on more than 8 frequency resources for a NES-capable UE. However, such implementation limits the flexibility and reduce the efficiency of resource allocation.

For Option 2, it could be done by introducing a new offset to separate the two ROs. Note that when 2-step RA was implemented, an offset was also introduced in the formula to tell the difference between MSGB-RNTI and RA-RNTI (14×80×8×2). Similarly, another offset could be utilized for separating the type of the selected RO. That is,

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 × 2 × 2 × nes_id, where nes_id = 0 for legacy RO, nes_id = 1 for additional RO.

Proposal 1: Revisit the previous agreement and revise the RA-RNTI formula for RACH adaptation, e.g., introducing a new offset in RA-RNTI formula to separate the case that whether the selected RO is additional RO or legacy RO.

- Conclusion

Observation 1: RA-RNTI collision could occur when the additional RO and legacy RO are overlapped in time-domain.

Proposal 1: Revisit the previous agreement and revise the RA-RNTI formula for RACH adaptation, e.g., introducing a new offset in RA-RNTI formula to separate the case that whether the selected RO is additional RO or legacy RO.

- Reference

- R1-2506587, “Reply LS on RA-RNTI for PRACH adaptation”