3GPP TSG-RAN WG2 Meeting #131bis	R2-2507537

Prague, Czech Republic, Oct. 13th-17th

Agenda Item:	8.6.3

Source: 	ASUSTeK

Title:  	Discussion on MAC open issues for CLTM

Document for:	Discussion and Decision

- Introduction

In the email discussion for MAC open issue [1], open issues were listed and some were suggested to be discussed based on companies’ contribution:

Proposal 1: Following open issues are suggested to be discussed based on companies’ contributions: - Inter-CU LTM: MAC-V01, V02, - Event triggered L1 measurement: MAC-V03, S02, H03, H04(Part II), O02, Q01, - Conditional intra-CU LTM: MAC-V04, V05, K01, K02, N01, O03, M01.

In this contribution, we’d like to discuss open issues MAC-K01 and MAC-K02.

- discussion

MAC-K01: storing TA for LTM source Cell

In the current C-LTM procedure, when receiving a LTM Candidate Timing Advance Command MAC CE indicating a C-LTM candidate cell, the UE stores TA value indicated in the MAC CE for the indicated C-LTM candidate cell and start or restart ltm-Candidate-TimeAlignmentTimer associated with the indicated LTM associated with the indicated LTM candidate cell.

On the other hand, the Source Cell of the C-LTM procedure may also be a C-LTM candidate Cell, and the TA value of the Source Cell can still be valid when performing a C-LTM to a target Cell.

Observation 1:	When performing C-LTM to a target cell, the TA associated with a source cell of the LTM, which could be a C-LTM candidate Cell, can still be valid.

If the source cell is associated with a C-LTM candidate, then it would be beneficial to store the valid TA of the source cell for possible RACH-less C-LTM back to the source cell. In the current running CR, the UE does not stop ltm-Candidate-TimeAlignmentTimers for candidate cells upon MAC reset triggered by C-LTM to maintain TA validity for candidate Cells. However, if the ltm-Candidate-TimeAlignmentTimer for Source Cell’s LTM candidate configuration is not running at the time when C-LTM is triggered, the UE cannot store or maintain the TA for the Source Cell.

Observation 2:	When the UE performs C-LTM execution, the ltm-Candidate-TimeAlignmentTimer for C-LTM candidate configuration of the Source Cell may not be running, and the UE cannot store the valid TA for the C-LTM candidate configuration of the Source Cell.

To fix this scenario where candidate cell’s TA cannot be stored, we propose the UE to start or restart the corresponding C-LTM timer (ltm-Candidate-TimeAlignmentTimer) associated with the C-LTM candidate of the source cell upon C-LTM execution.

Figure 1

As illustrated in Figure 1, while the Source Cell is associated with a CLTM candidate configuration and the TA is valid, the new C-LTM timer (i.e., ltm-Candidate-TimeAlignmentTimer) is not running when it is a Serving Cell (since no LTM Candidate Timing Advance Command MAC CE is received). At timing t1, the UE determines to perform LTM on a candidate cell, where the TAT of the source cell has been running for n ms. The UE then stores the TA of the source cell for the corresponding C-LTM candidate that includes the source cell as the SpCell (e.g., the C-LTM-Candidate of which ltm-CandidatePCI is set to the PCI of the source cell). The UE then (re)starts the ltm-Candidate-TimeAlignmentTimer associated with the corresponding C-LTM candidate, with the length of the remaining time of the timeAlignmentTimer of the source cell (i.e., m-n ms).

Proposal 1:	(MAC-K01) Upon execution of a C-LTM to a candidate cell, if the source cell is a SpCell of a C-LTM candidate configuration, the UE stores the TA of the source cell for the C-LTM candidate configuration, and starts or restarts the ltm-Candidate-TimeAlignmentTimer associated with the C-LTM candidate with the length of the remaining time of the timeAlignmentTimer of the source cell.

MAC-K02: report stored TA for LTM candidate Cell

In the last meeting, it was agreed that if the UE receives more TAs beyond its capability, it’s up to UE implementation to decide which one is released:

For C-LTM, if the UE receives more TAs beyond its capability, it’s up to UE implementation to decide which one is released.

Since the NW does not know which TA is released, the exact candidate cells of which TA values are stored by UE is unclear to the NW, and it may cause the NW to initiate unnecessary random access procedure for early UL synchronization for the UE.

Observation 3: Since the NW does not know which TA is discarded by the UE when the number of stored TA exceeds UE capability, the NW could initiate unnecessary random access procedure for early UL synchronization for the UE.

To address this issue, we propose the UE to report candidate cells with stored TAs to the network when the UE discards or releases a TA due to the number of stored TA is beyond the UE’s capability.

Proposal 2:	(MAC-K02) The UE reports candidate cells with stored TA values to the network upon releasing a TA due to receiving more TAs beyond its capability.

- Text proposal for MAC-K01 (based on Draft 38.321 J00_JK2)

## 5.2b	Maintenance of UL Synchronization for CLTM candidate cell

The MAC entity shall for each CLTM candidate cell:

…

1>	when the Source Cell of a CLTM execution is a CLTM candidate cell:

2>	if two TAGs are configured for the CLTM candidate cell:

3>	store the TA values of the Source Cell;

3>	start or restart the ltm-Candidate-TimreAlignmentTimer and ltm-Candidate-TimeAlignmentTimerTAG2 associated with the CLTM candidate cell with the length of the remaining time of the timeAlignmentTimer of corresponding PTAGs of the Source cell;

2>	else:

3>	store the TA value of the Source Cell;

3>	start or restart the ltm-Candidate-TimeAlignmentTimer associated with the CLTM candidate cell with the length of the remaining time of the timeAlignmentTimer of PTAG of the Source cell.

- Text proposal for MAC-K02 (based on Draft 38.321 J00_JK2)

## 5.2b	Maintenance of UL Synchronization for CLTM candidate cell

The MAC entity shall for each CLTM candidate cell:

1>	when an LTM Candidate Timing Advance Command MAC CE described in clause 6.1.3.4b is received:

2>	if two TAGs are configured for the CLTM candidate cell:

3>	store the TA value in the LTM Candidate Timing Advance Command MAC CE for the indicated CLTM candidate cell for the indicated TAG as specified in clause 6.1.3.4b;

3>	start or restart the ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 associated with the indicated LTM candidate cell for the indicated TAG as specified in clause 6.1.3.4b;

2>	else:

3>	store the TA value in the LTM Candidate Timing Advance Command MAC CE for the indicated CLTM candidate cell as specified in clause 6.1.3.4b;

3>	start or restart the ltm-Candidate-TimeAlignmentTimer associated with the indicated LTM candidate cell as specified in clause 6.1.3.4b.

1>	when the CLTM candidate configuration(s) is released as specified in TS 38.331 [5]:

2>	stop the running ltm-Candidate-TimeAlignmentTimer and ltm-Candidate-TimeAlignmentTimerTAG2 associated with the corresponding CLTM candidate cell(s), if any;

2>	release the stored TA value for the corresponding CLTM candidate cell(s), if any.

1>	when a TA value of a CLTM candidate cell is discarded:

2>	instruct the Multiplexing and Assembly procedure to generate the LTM candidate TA report MAC CE as defined in clause 6.1.3.xx;

NOTE:	If the UE receives more TA values than it can store, it is up to the UE implementation which TA values to store or discard.

#### 6.1.3.xx	LTM candidate TA report MAC CEs

The LTM candidate TA report MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-xx. It has a fixed size and consists of a single octet containing seven C-fields and one R-field. The SCell Activation/Deactivation MAC CE with one octet is defined as follows (Figure 6.1.3.xx-1).

-	Ci: This field indicates the stored TA value status of the LTM candidate cell with ltm-CandidateId i. The Ci field is set to 1 to indicate that the UE has a stored TA value associated with the LTM candidate cell with ltm-CandidateId i. The Ci field is set to 0 to indicate that the UE does not have a stored TA value associated with the LTM candidate cell with ltm-CandidateId i;

Figure 6.1.3.xx-1: LTM candidate TA report MAC CE

- Conclusion

We have the following observation and proposal for MAC open issues for C-LTM:

Observation 1:	When performing C-LTM to a target cell, the TA associated with a source cell of the LTM, which could be a C-LTM candidate Cell, can still be valid.

Observation 2:	When the UE performs C-LTM execution, the ltm-Candidate-TimeAlignmentTimer for C-LTM candidate configuration of the Source Cell may not be running, and the UE cannot store the valid TA for the C-LTM candidate configuration of the Source Cell.

Proposal 1:	(MAC-K01) Upon execution of a C-LTM to a candidate cell, if the source cell is a SpCell of a C-LTM candidate configuration, the UE stores the TA of the source cell for the C-LTM candidate configuration, and starts or restarts the ltm-Candidate-TimeAlignmentTimer associated with the C-LTM candidate with the length of the remaining time of the timeAlignmentTimer of the source cell.

Observation 3: Since the NW does not know which TA is discarded by the UE when the number of stored TA exceeds UE capability, the NW could initiate unnecessary random access procedure for early UL synchronization for the UE.

Proposal 2:	(MAC-K02) The UE reports candidate cells with stored TA values to the network upon releasing a TA due to receiving more TAs beyond its capability.

- Reference

[1] R2-2507013 List of MAC open issues for R19 mobility