3GPP TSG-RAN WG2 Meeting #131-bis	R2- 2507550

Prague, Czech Republic, 13th – 17th Oct. 2025

Agenda Item:	8.6.2

Source:	MediaTek Inc.

Title:	Remaining CP issues in R19 mobility

Document for:	Discussion, Decision

Introduction

In this contribution, the remaining issues for CP in R19 mobility are discussed.

Discussion

CLTM co-exist with CHO

In the last meeting, the co-existence between CLTM and CHO were agreed by inheriting the rule from R18 LTM:

Observation 1: Network triggered L3 HO, CHO and network triggered PScell change (SN change/SN modification) can be configured with intra-CU LTM, while the coexistence of LTM and DAPS HO is not supported in Rel-18. Proposal 4: CLTM can coexist with network triggered L3 HO, CHO and network triggered PScell change, while the coexistence of CLTM and DAPS HO is not supported Agreed.

In stage-2 open issue, one company mentioned the meaning of “CLTM co-exist with CHO” and think it is not clear whether CLTM and CHO can be configured for the same candidate cell.

In our understanding, CLTM is an extension of CHO that introduces additional features such as L1 measurement reporting and early synchronization. However, these new features are optional, and CLTM can “fallback” to CHO-like mobility if they are not configured. For example, if only L3 measurement is configured and no early synchronization indication is provided, CLTM will operate similarly to CHO.

Therefore, from a signaling perspective, there is no need to configure two separate candidate configurations (CHO and CLTM) for the same candidate cell. The network can decide whether to trigger CLTM or CHO for a candidate cell based on its indication (e.g., by sending a TCI activation indication or a PDCCH order).

Running two sets of conditional mobility mechanisms on the same cell simultaneously may cause confusion for both the UE and the network. If the candidate configurations for CHO and CLTM are identical for a given cell, this would result in unnecessary signalling overhead. Furthermore, regarding the UE’s capability limit for the number of event evaluations (one for L1 condition and two for L3 condition), it is unclear whether CHO and CLTM share a common capability limit or have separate limits. This may lead to further compatibility issue.

Proposal 1: Simultaneous configuration of CLTM and CHO for the same candidate cell is not supported.

UE capability

Regarding the open issue of UE capability, there is a suggestion to introduce separate NR-DC capability for R19 Mobility.

In Rel18, ltm-MCG-NRDC-r18 and ltm-MCG-NRDC-Release-r18 are introduced for MCG LTM with NR-DC. ltm-MCG-NRDC-r18 Indicates whether the UE supports LTM for MCG with RACH with NR-DC configured as defined in TS 38.331 [9] and TS 38.321 [8]. UE indicating support for this feature shall also indicate support of ltm-MCG-IntraFreq-r18.. ltm-MCG-NRDC-Release-r18 Indicates whether the UE supports LTM for MCG with the release of NR-DC configuration as part of LTM execution when LTM cell switch command MAC CE is received. UE indicating support for this feature shall also indicate support of ltm-MCG-IntraFreq-r18.. In Rel-18, there was a restriction for MCG LTM as below: mrdc-SecondaryCellGroupConfig This field is used to configure and release an SCG in NR-DC and NE-DC. In an RRCReconfiguration message within an LTM-Config IE associated with the MCG, if this field is present its value can only be set to release. The restriction is removed in Rel-19, without any conditions. i.e. MCG LTM for intra-CU LTM, Inter-CU MCG LTM and CLTM may include mrdc-SecondaryCellGroupConfig set to setup. Since network needs to identify UEs which supports SCG addition/modification during MCG LTM, a new capability similar to the above capabilities may be added, as below. ltm-MCG-NRDC-Setup-r19 Indicates whether the UE supports LTM for MCG with the setup of NR-DC configuration as part of LTM execution when LTM cell switch command MAC CE is received. UE indicating support for this feature shall also indicate support of ltm-MCG-IntraFreq-r18.

Rapporteur suggests to reuse Rel-18 capabilities ltm-MCG-NRDC-r18 and ltm-MCG-NRDC-Release-r18 for Rel-19. However, the “mrdc-SecondaryCellGroupConfig set to setup” is prohibited in Rel-18 and newly added in Rel-19, according to the red text above. So, there are more scenarios introduced in Rel-19.

To be specific. In Rel-18, the following capabilities is indicated for the following cases:

- Case 1: UE executes MCG LTM upon which NR-DC is released (mrdc-SecondaryCellGroupConfig= release)

  - In this case, ltm-MCG-NRDC-Release-r18 is needed.

- Case 2: UE executes MCG LTM upon which NR-DC is kept without changes. (mrdc-SecondaryCellGroupConfig is absence and UE maintains the current SCG configuration.)

  - In this case, ltm-MCG-NRDC-r18 is needed.

However, In Rel-19, the following new case are introduced, for the SN key update purpose.

- Case 3: UE has NR-DC configured. UE executes MCG LTM upon which SCG configuration is modified (mrdc-SecondaryCellGroupConfig=setup)

Although the motivation is only for SN key update, this case is different from case 1 or 2. So a separate UE capability is needed for this case.

Proposal 2: A separate UE capability is introduced for MCG LTM with the setup of NR-DC configuration.

Moreover, for SN addition case, RAN2 had the following discussion and agreement:

RAN2#130 Proposal 1: Regarding MCG LTM with SN addition, RAN2 to discuss and confirm whether there is no restriction on only supporting MCG LTM with SN addition within the same SN. [Ericsson]: SN addition is SN addition and we don’t have different procedure to the same and different SN. [Apple]: The original intention was to allow sk-counter update due to MN change. However, if we allow SN change simultaneously, it may bring additional issue. [Vivo]: No need of restriction in the stage 2. [Xiaomi]: At least, we should clarify that inter-MN change with SN change shouldn’t be supported. [China Telecom]: This restriction (i.e. inter-MN change with SN change shouldn’t be supported) was already captured in the running CR. - No restriction on only supporting MCG LTM with SN addition within the same SN.

In our understanding, it implies that the following case is also valid:

- Case 4: UE does not yet have NR-DC configured and UE executes MCG LTM upon which NR-DC is setup (mrdc-SecondaryCellGroupConfig=setup)

However, it is not clear to us if this case is still for SN key change purpose. We suggest RAN2 to discuss the motivation of this case first and decide whether to introduce a separate UE capability for it.

Proposal 3: RAN2 to discuss if case 4 (UE does not yet have NR-DC configured and UE executes MCG LTM upon which NR-DC is setup) is a valid case and decide whether to introduce a separate UE capability for it.

Conclusion

In this contribution, the following observations and proposals are made:

Proposal 1: Simultaneous configuration of CLTM and CHO for the same candidate cell is not supported.

Proposal 2: A separate UE capability is introduced for MCG LTM with the setup of NR-DC configuration.

Proposal 3: RAN2 to discuss if case 4 (UE does not yet have NR-DC configured and UE executes MCG LTM upon which NR-DC is setup) is a valid case and decide whether to introduce a separate UE capability for it.

References

- R2_131_R18SL_R18R19 MOB_R19 NES_Session Notes_EoM

- R2-250xxxx Open issues on UE capability for Mob Ph4 v04_rapp

- R2-250xxxx List of 38.300 open issues for R19 mobility_v02_MediaTek