3GPP TSG-RAN WG2 Meeting #131bis                                 R2-2507558

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda item:		8.2.2

Source:		Samsung

Title:			Discussion on the remaining issues on A-IoT

Document for:	Discussion & Decision

- 1 Introduction

In RAN2#130, the AIoT WI was considered as completed from RAN2 perspective. Some issues are remaining open. In this contribution, we will share our views on the remaining issues.

- 2 Discussion

Issue 1: Case of “no upper layer data available”

In last meeting, the following agreements were achieved for the case of “no data available due to delay in NAS”

- The reader determines no data available case by SDU length 0.   As more data indication is mandatory, the device sets this bit to "0".

- The reader, in response to 0 SDU in the device’s MAC response, may send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device.

- The follow-up R2D Upper Layer Data Transfer message includes the Received Data Size field with the Received Data Size field set to value 0, without including the original command.

- RAN2 would like to check if there is a case where NAS doesn’t provide a response at all.   If this case exists, RAN2 will discuss this issue.

One unclear point is whether there is a case of “no upper layer data available”. This is related to the CT1 specification on AIoT NAS (TS24.369).

For read command and write command procedures, the device needs response with either complete or reject message to AIoTF. However, the following editor notes indicates that CT1 has not yet decided whether there is a response for the cases when read/write command not supported.

Editor's note: Whether to use the READ COMMAND REJECT message to reject the READ COMMAND message when the read command not supported is FFS. Editor's note: Whether to use the WRITE COMMAND REJECT message to reject the WRITE COMMAND message when the write command not supported is FFS.

Observation 1: CT1 has not determine the handling for the cases when read/write command not supported

On the other hand, CT1 also defines some unknown/unforeseen/erroneous cases, e.g.,

- Message too short

- Unknown or unforeseen message type

- Non-semantical mandatory information elements errors

For those cases, CT1 defines that the device shall ignore the AIoT NAS message. It seems that such “ignore” means that the AIoT device will not send any response message. If such understanding is correct, it indicates that the case of “no upper layer data available” is a valid case. However, it is better to get CT1’s confirmation.

Observation 2: CT1 defines some cases that the AIoT device shall ignore the received AIoT NAS message.

To determine the case of “no upper layer data available”, it is better to get the confirmation from CT1. RAN2 can indicate the above observations in the LS to CT1.

Meanwhile, if CT1 confirms the case of “no upper layer data available”, the reader needs to differentiate two different cases, i.e., “no upper layer data available due to delay NAS” and “no upper layer data available”. To address this, a straightforward solution can be:

- “0 SDU & MDI =0” represents “no upper layer data available”

- “0 SDU & MDI =1” represents “no upper layer data available due to delay NAS”

Although CT1 confirmation is needed, RAN2 can make a decision given “no upper layer data available” in advance considering such solution is straightforward and it is easy for agreement.

Proposal 1: RAN2 is kindly asked to prepare LS to CT1 to check the case of “no upper layer data available”. If CT1 confirms the possible case of “no upper layer data available”, the following solution can be considered:

- “0 SDU & MDI =0” represents “no upper layer data available”

- “0 SDU & MDI =1” represents “no upper layer data available due to delay NAS”

Issue 2: Paging ID length

According to latest SA2 LS (S2-2507793), SA2 intends to extend the length of Paging ID up to 600bits, which results in a large size paging message. Considering the simple design of AIoT system, the long message size will cause high probability of losing the message at the device side. Moreover, if the device losses one paging message, it may need to wait for a long period to perform access until receiving the next paging message. Thus, the large size of paging message may result in system performance degradation.

Proposal 2: RAN2 is kindly asked to agree that the extension of ID length will degrade the AIoT system performance.

From RAN2 perspective, the extension of Paging ID length is feasible. The specification change is to extend the Paging ID length field to 10bits. However, the system performance degradation cannot be neglected. So, we prefer to not extend the ID length.

Proposal 3: RAN2 is kindly asked to agree that the ID length extension is not encouraged.

Since SA2 is checking RAN2’s view on the feasibility of ID length extension, RAN2 can express the following in the reply LS:

- Extend the Paging ID is technically feasible from RAN2 point of view

- Large size of Paging ID may impact the AIoT system performance

- RAN2 suggests to not carry out the ID length extension if it can be avoided.

Proposal 4: RAN2 is kindly asked to prepare the reply LS to SA2 by indicating the followings:

- Extend the Paging ID is technically feasible from RAN2 point of view

- Large size of Paging ID may impact the AIoT system performance

- RAN2 suggests to not carry out the ID length extension if it can be avoided.

- 3 Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1: RAN2 is kindly asked to prepare LS to CT1 to check the case of “no upper layer data available”. If CT1 confirms the possible case of “no upper layer data available”, the following solution can be considered:

- “0 SDU & MDI =0” represents “no upper layer data available”

- “0 SDU & MDI =1” represents “no upper layer data available due to delay NAS”

Proposal 2: RAN2 is kindly asked to agree that the extension of ID length will degrade the AIoT system performance.

Proposal 3: RAN2 is kindly asked to agree that the ID length extension is not encouraged.

Proposal 4: RAN2 is kindly asked to prepare the reply LS to SA2 by indicating the followings:

- Extend the Paging ID is technically feasible from RAN2 point of view

- Large size of Paging ID may impact the AIoT system performance

- RAN2 suggests to not carry out the ID length extension if it can be avoided.