3GPP TSG-RAN WG2 #131bis                                                                                     R2-2507562

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	    10.4

Source: 	    MediaTek Inc.

Title: 	    Mobility for 6GR

Document for:	    Discussion and decision

# 1 Introduction

Study on 6G radio was approved in RP-251881[1] in RAN#108. One of the study objectives is Mobility for 6GR (for all RRC states), including related RRM (objective#4).

In this document, we discuss the general mobility principles, measurements, and procedure in both IDLE and CONNECTED mode and provide our suggestions on 6G mobility study.

# 2 Discussion

## 2.1 Background

In cellular communication systems, UEs may change their locations. To allow continued services, mobility procedures are designed to enable handovers between cells. In NR, mobility enhancements have been introduced release-by-release with the common objectives of higher reliability, lower interruption time and faster cell/beam switching:

- In Release 15, baseline mobility procedure is supported, the A3/A5 offset and TTT mechanisms are used to reduce Ping-Pong events and ensure the credibility of every cell switch.

- In Release 16, CHO was introduced to improve robustness, in which network configures candidate cells with associated conditions to UE in advance, and UE executes handover when the conditions are met. In this way, we can avoid the major cause of mobility failure that UE miss the handover command due to sudden channel fluctuation.

- In Release 16, DAPS (dual active protocol stack) handover was also introduced. A DAPS-capable UE maintains its connection with the source gNB until the handover to target gNB succeeds, so that the handover interruption is almost zero. However, the DAPS has high UE hardware requirement and it is not really deployed.

- In Release 18, we introduced a new mobility procedure called L1/L2-triggered mobility (LTM), aiming at mobility latency reduction. In LTM, RRC configurations of potential target cells (i.e., candidates) are provided to UE in advance, and UE applies RRC configurations indicated when the mobility switching procedure is triggered by the LTM command MAC CE. LTM allows UE fast switching between cells and ensures UE to be served by its best cell and beam with minimum interruption, by early preparation and pre-synchronization (downlink and uplink) towards candidate cells.

From the mobility enhancement evolution in NR, it is clear that high reliability and low interruption with fast mobility switching is the design trend in NR.

Observation 1: High reliability and low interruption with fast mobility switching is the design trend in NR.

## 2.2 Mobility Requirement

Traditionally, we have several key performance indicators (KPI) to evaluate mobility performance as shown in the following:

- Mobility failure rate: The probability that a triggered handover fails. The main reasons of mobility failure include (1) failed delivery of handover command or measurement report and (2) UE fails to connect to the indicated target cell e.g., RACH failure. (3) RLF caused by inappropriate timing of mobility decision

- Handover interruption: The period from when the UE disconnects from the source cell until it establishes a connection with the target cell, during which service is interrupted. Before UE can successfully communicate with the target cell, service is interrupted. Notice that although each interruption may be short, overall interruption can still cause user experience degradation if handover happens too frequently.

- Serving cell signal strength and throughput: The purpose of mobility procedure is to move UE to a better cell. A successful design should allow better serving cell signal strength distribution and thus higher throughput.

- Time of Stay (ToS) and Ping-Pong rate: Ping-Pong happens when UE switches from one cell to another and then switches back to the original cell after a short duration. UE’s ToS in a cell can be short if UE speed is high, cell coverage is small (e.g., high carrier frequency), or Ping-Pong rate is high. Shorter ToS means more frequent handover and thus more service interruption.

We understand that mobility failure is already quite low in 5G NR in particular for FR1. For FR2, CHO/C-LTM mechanism could effectively reduce the failure rate. So, there seems not much enhancement needed in this aspect.

In LTM, we were trying to reduce the handover interruption time to a value similar to beam switching. However, the LTM switching time (20ms ~ 30ms) is still too high for certain immersive communication requirements. Note that although DAPS could achieve theoretically 0ms interruption time, it is not deployed in real field due to its complexity and high UE hardware requirements. Therefore, minimizing interruption time with LTM-like procedure is an important aspect for RAN2 to study. The RAN1/RAN4 design will also impact interruption time and thus collaboration with RAN1/RAN4 is needed.

Proposal 1: RAN2 should aim to have a low-interruption mobility procedure (e.g., less than 10ms) in 6G. RAN2 to study how to reduce mobility interruption together with RAN1/RAN4.

## 2.3 Cross-WG sync on mobility design

According to the SID [1], the mobility objective is led by RAN2 but RAN1, RAN3, and RAN4 are also listed.

(4) Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

In our understanding, mobility is one of the topics that requires close inter-WG co-working. Some RAN1/RAN3/RAN4 designs would clearly impact the RAN2 mobility procedure. Some key aspects (e.g., reference signal design) that impact mobility performance are actually designed by RAN1. Without proper inputs from other WG, it would be difficult for RAN2 to make sufficient progress. Therefore, we assume that RAN2 should consider other WG inputs when designing the mobility procedure.

Proposal 2: RAN2 to study mobility procedure taking below R1/R2/R3 design into consideration:

- RAN1 – the mobility RS signal design. (e.g. which RS is used in which mobility procedure, what’s the periodicity assumption for the RS)

- RAN3 – The basic assumptions of RAN architecture that may impact mobility procedure (e.g., CU/DU split, path switching, and data forwarding between source and target)

- RAN4 – The initial measurement and mobility interruption analysis and its potential impact to mobility procedure

## 2.4 Mobility RS

Before RAN1 concluded, RAN2 should have some basic assumptions for the 6G reference signal (RS) used for 6G Mobility.

In 5G NR, the mobility functions in idle and connected modes are supported by the same RS (i.e., SSB). However, the requirements for cell/TRP/beam detection/measurement in idle mode are different from those for connected-mode mobility, and this difference is expected to become even more pronounced in 6GR as further reduction of network energy consumption for signals such as SSB are targeted for cells/TRPs/carriers with low/zero load, while enhanced mobility performance (e.g., fast link set-up and higher beam-level SNR) is targeted in connected mode. A single SSB cannot simultaneously satisfy the wide-beam, low-duty-cycle needs of idle mode and the narrow-beam, high-gain demands of connected mode. Therefore, reuse of the same RS for idle and connected mode for 6GR could be sub-optimal in catering for the different requirements of each mode.

To allow the 6GR design to optimally cater for both idle and connected mode mobility needs, separate RS for IDLE and CONNECTED should be considered for 6G. The RS design should of course be studied by RAN1 first but it would be good if RAN2 could also make some initial assumptions to speed up RAN2 procedure design. Of course, we should check with RAN1 later.

Proposal 3: RAN2 assumes that separate type of mobility RSs are used for IDLE and CONNECTED mode mobility procedure. RAN2 should discuss the impact to mobility procedure based on this assumption.

## 2.5 Measurement model

In current 5G NR system, the measurement model is defined in 38.300 as below.

Figure 1 – 5G Measurement Model from TS 38.300 Figure 9.2.4-1

The measurement model is suitable for legacy L3 handover defined in Rel-15. However, with the introduction of inter-cell beam management (ICBM) and LTM, the boundary between beam-based and cell-based mobility becomes indistinct. Since the UE ultimately chooses the best beam for data transmission, it is unclear whether a cell-level result (obtained by consolidating beam-level results) is still necessary. In 6G, we expect UE just report the measurement result per RS only.

Measurement reporting is also duplicated today in mobility and BM. In NR, SSB-based RSRP measurement result could be reported 1) in L1 via UCI or PUCCH, 2) in MAC-CE for LTM measurement event, and 3) in legacy RRC measurement report message. And since the introduction of LTM, the terminology “L1 measurement” and “L3 measurement” are widely used during the discussion. For the same beam, both L1 and L3 measurement refer to the same RS but with different requirement (i.e., L1 RSRP has a higher synchronization requirement),which is quite confusing and complicated. In 6G, we recommend a single, unified reporting mechanism that covers both beam-level and cell-level mobility to reduce SPEC complexity and implementation effort.

Overall, we suggest RAN2 to design a unified measurement framework with consistent reporting/procedure for both beam-level and cell-level mobility.

Proposal 4: RAN2 should aim to have a harmonized framework/procedure for different use cases. Considering the following direction:

- Measurement of Beam Management (BM) and CONNECTED mode mobility should be based on the same RS.

- Unified measurement report (e.g. in MAC-CE). No separate reporting in L1 and L3.

- Report per RS – FFS on beam consolidation

We expect to have a simplified measurement model for 6G as below figure. The UE reports measurement results per RS. The measurement process includes a UE-side filter (similar to L1 filter) and NW-configured filter (similar to L3 filter). The network also configures the triggering condition to send measurement report or the execution condition (like CHO) to trigger different procedure. There is no need to report measurement report for the same RS in different ways (e.g., different report containers or in BM/Mobility procedures). The difference on how well the UE has synchronized to the RS can be reported explicitly. E.g., sync status indication can be included in the measurement report.

Figure 2 – 6G Measurement Model

## 2.6 Mobility Procedure

### 2.6.1 IDLE mode procedure

5G idle mode mobility ensures robust and simple performance. The S-criteria is used to select valid cell and the ranking orders the valid cell by signaling quality. The reselection priority decides the frequency or RAT the UE should camp on. These mechanisms keep the UE reachable via paging and ready to initiate connection, while supporting UE-autonomous mobility without network awareness and minimizing power consumption through lightweight RRM measurements. 6G should retain this design to maintain a simple, efficient idle mobility foundation.

To ensure a smooth evolution to 6G, it is essential to maintain the principle of minimized UE capability in Idle state, as gNB does not know the UE's capabilities or its precise location. Any additional feature in Idle should be well-designed so that UEs not supporting it can still operate effectively. For example, for low power Radio (LR) design, RAN2 should discuss whether it is mandatory for all UE(s). If not, what’s the fallback UE behavior if not supporting LR.

Proposal 5: For 6GR idle mode mobility design:

- RAN2 should reuse 5G mechanisms/procedures (e.g. S-criteria, ranking, and reselection procedure, reselection priority) as baseline to enable long-DRX, UE-based mobility in idle state.

- Design features based on minimized UE capabilities. Analyze the impact of low power radio (LR) design in IDLE mode.

### 2.6.2 Connected mode procedure

One of the key pain points in 5G mobility is the mismatch between coverage and throughput. Specifically, mobility triggering conditions often rely on wide beam measurements, e.g., via SSB in 5G, which do not accurately reflect the user experience that is more closely associated with narrow beam reference signals, such as CSI-RS in 5G. To address this mismatch, multiple types of RS, including both wide-beam RS and narrow-beam RS, should be considered for the mobility triggering in 6G connected mode mobility.

Observation 2: The wide beam for mobility triggering evaluation in 5G cannot reflect the true user experience/throughput.

In the 6G connected mode procedure, we aim at achieving mobility with lower latency and minimal interruption. The  L1/L2 triggered mobility in 5G has demonstrated the benefit of latency reduction via lower layer measurement report and cell switch indication, as well as the low interruption through early synchronization/preparation and fast cell switch procedure. These features should be reused in 6G mobility as baseline to reduce the cell switch latency and interruption to achieve stable, high-performance cell switching.

Observation 3: The features of L1/L2 triggered mobility in 5G brings the benefit of low latency with minimized interruption, which can serve as the baseline in 6G mobility.

While mobility and MIMO beam management (BM) evolved separately in 5G NR, their similar objectives led to redundant, complex parallel processes for UEs and networks. This distinction becomes even less meaningful with the rise of cell-free concept/multi-TRP deployments in 6G. To reduce duplication and complexity, 6G should harmonize connected-mode mobility with beam management.

Observation 4: A harmonized connected-mode mobility procedure in 6G is benefit for reducing the redundant and duplicated procedure between MIMO beam management and mobility.

In 5G, UE initiated mobility is realized by conditional handover (Rel-16 CHO) and conditional LTM (Rel-19 C-LTM). Compared with network triggered mobility, UE initiated mobility have special benefit on reliability and UE’s flexibility:

- Improve Reliability: Mobility configuration is pre-configured, enabling immediate handover execution when radio conditions meet predefined thresholds. This proactive approach skips “MR->Network decision->HO command” procedure and prevents radio link degradation to critical failure levels where the handover might fail (i.e., too late handover). The handover command is omitted to prevent HOF caused by this signalling failure due to sudden channel fluctuation.

- UE autonomy and flexibility: Allows the UE to proactively decide optimal candidate cell by certain predefined conditions and pre-configured candidate configurations. The decision made by UE may be more accurate and reasonable due to more detailed measurement context of its radio environment, which allows UE to analyse more details that might not be included in the measurement report. In 6G, on-device AI could further leverage this rich measurement context to perform sophisticated candidate selection. This also avoid impractical report of massive environmental datasets to network.

C-LTM represents an evolutionary extension of CHO that introduces early candidate cell synchronization. Current R19 C-LTM limits UE autonomy to cell switching operations with network-controlled synchronization. In 6G, more UE initiation can be applied to early synchronization procedure to make more autonomous decisions during mobility procedures. For instance, the UE could select a candidate cell for early synchronization and notify the network of its decision. Initially, this approach could be applied to DL synchronization as a starting point.

In NTN mobility, additional information such as direction, distance, and Doppler shift can play a crucial role in determining the optimal timing for mobility. However, these factors are not currently reflected in existing measurement reports or mobility mechanisms. By enabling UE-initiated mobility, the UE could integrate these parameters into its analysis to make more precise mobility decisions.

We understand that UE initiated mobility requires more operation from network, such as pre-reserving candidate resources and forwarding UE context in advance. Such network complexity has limited its widespread deployment in NR, yet its evolution has never stopped. C-LTM was introduced in Rel-19 with extra network-UE coordination and pave the way for future design. As mobility performance requirements become increasingly stringent in 6G, we see a brighter future for UE-initiated mobility due to its reliability and accuracy. It is likely to be revitalized with new scenarios and solutions, along with better coordination between UE and Network. Therefore, we propose supporting UE-initiated mobility in day 1 in 6G.

Observation 5: The UE-initiated mobility will play a more important role in 6G to compensate NW-triggered mobility due to its reliability and accuracy.

Consider all the above-mentioned aspect, we have the following proposal for 6G CONNECTED mode mobility:

Proposal 6: Introduce a harmonized mobility procedure for 6GR CONNECTED mobility between beams, TRPs, and cells, with the supported of NW/UE triggered mobility and the design of pre-configuration, early sync, and fast switching.

With the increasing complexity of 6G scenarios and the challenges of coverage and throughput mismatches, the legacy rule-based handover procedures may no longer be suitable in all cases. While pre-configured mobility parameters (such as TTT and A3 offset) can optimize the average performance for all UEs within a cell, they may not be optimal for each individual UE or for every handover decision. Instead, it would be beneficial for certain mobility parameters to be dynamically adjusted by the UE based on its real-time link conditions to enhance the user experience.

AI is one of promising approaches for further enhancing 6G mobility. AI-based prediction can help UEs dynamically determine optimal parameters, or even decide whether or not to trigger a handover procedure. In NR AI Mobility SID [2], 3GPP proposes using AI to predict RRM measurements and the timing of measurement events defined by fixed configurations. We agree that standardizing AI-based mobility enhancements in 3GPP would be valuable. However, it is also important to allow for AI native mobility solutions or smart UE implementations (for example, to adjust parameters as described above) to provide greater flexibility and enable faster, more practical deployment.

To support this vision, some modifications to the specifications will be necessary. For example, allowing a range of acceptable mobility parameter values instead of fixed values, and introducing signaling for UE recommendations, etc. In 6G, we aim to design an extendable/flexible connected mode mobility procedure that allow to trigger (or not trigger) handover with new 6G features, including AI enhancement, such that the mobility can better reflect the real-time link conditions and improve the user experience.

Proposal 7: 6G mobility procedure should allow to trigger (or not trigger) connected mode mobility when necessary, from link condition (for data transmission) point of view.

# 3 Conclusions

Based on the discussion in section 2, we have the following observations and proposals:

Observation 1: High reliability and low interruption with fast mobility switching is the design trend in NR.

Observation 2: The wide beam for mobility triggering evaluation in 5G cannot reflect the true user experience/throughput.

Observation 3: The features of L1/L2 triggered mobility in 5G brings the benefit of low latency with minimized interruption, which can serve as the baseline in 6G mobility.

Observation 4: A harmonized connected-mode mobility procedure in 6G is benefit for reducing the redundant and duplicated procedure between MIMO beam management and mobility.

Observation 5: The UE-initiated mobility will play a more important role in 6G to compensate NW-triggered mobility due to its reliability and accuracy.

Proposal 1: RAN2 should aim to have a low-interruption mobility procedure (e.g., less than 10ms) in 6G. RAN2 to study how to reduce mobility interruption together with RAN1/RAN4.

Proposal 2: RAN2 to study mobility procedure taking below R1/R2/R3 design into consideration:

- RAN1 – the mobility RS signal design. (e.g. which RS is used in which mobility procedure, what’s the periodicity assumption for the RS)

- RAN3 – The basic assumptions of RAN architecture that may impact mobility procedure (e.g., CU/DU split, path switching, and data forwarding between source and target)

- RAN4 – The initial measurement and mobility interruption analysis and its potential impact to mobility procedure

Proposal 3: RAN2 assumes that separate type of mobility RSs are used for IDLE and CONNECTED mode mobility procedure. RAN2 should discuss the impact to mobility procedure based on this assumption.

Proposal 4: RAN2 should aim to have a harmonized framework/procedure for different use cases. Considering the following direction:

- Measurement of Beam Management (BM) and CONNECTED mode mobility should be based on the same RS.

- Unified measurement report (e.g. in MAC-CE). No separate reporting in L1 and L3.

- Report per RS – FFS on beam consolidation

Proposal 5: For 6GR idle mode mobility design:

- RAN2 should reuse 5G mechanisms/procedures (e.g. S-criteria, ranking, and reselection procedure, reselection priority) as baseline to enable long-DRX, UE-based mobility in idle state.

- Design features based on minimized UE capabilities. Analyze the impact of low power radio (LR) design in IDLE mode.

Proposal 6: Introduce a harmonized mobility procedure for 6GR CONNECTED mobility between beams, TRPs, and cells, with the supported of NW/UE triggered mobility and the design of pre-configuration, early sync, and fast switching.

Proposal 7: 6G mobility procedure should allow to trigger (or not trigger) connected mode mobility when necessary, from link condition (for data transmission) point of view.

# 4 References

[1] RP-251881:	“3GPP Work Item Description - Study on 6G Radio”, NTT DOCOMO, RAN#108

[2] TR 38.744: “Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR”