3GPP TSG-RAN WG2 Meeting #131bis		R2-2507565

Prague, Czech Republic, Oct. 13th-17th

Source:	China Telecom

Title:	Discussion on remaining RRC issues

Agenda Item:	8.12.3

Document for:	Discussion and Decision

- Introduction

In this contribution, we will discuss RIL[H400][408].

- Discussion

In RIL[H400][Z408], it is proposed to remove pucch-cell-r19 or remove servCellIndex-r19 from the CSI-ReportConfig -> reportTransmissionMode-r19 in the current draft running CR.  According to TS38.214, the PUCCH Resource can be configured via the pucchCell parameter within the CSI-ReportConfig. Therefore, the pucch-cell-r19 field should be retained to align with RAN1 spec.

We note that there are two methods to indicate the serving cell for PUSCH-based CSI reporting. One is indicated in ServingCellConfig -> CSI-MeasConfig -> CSI-ReportConfig. The other is indicated by servCellIndex-r19 in CSI-ReportConfig -> reportTransmissionMode-r19 -> pusch-ResourceOfModeB-19. If the result of indicated serving cell is always the same (case 1), then servCellIndex-r19 in CSI-ReportConfig -> reportTransmissionMode-r19 -> pusch-ResourceOfModeB-19 can be removed to avoid redundant configuration. If the serving cell used for legacy CSI reporting and UE-initiated CSI reporting can differ (case 2), then servCellIndex-r19 in CSI-ReportConfig -> reportTransmissionMode-r19 -> pusch-ResourceOfModeB-19 can be retained to support such flexibility. If case 2 is applicable, it is proposed to make servCellIndex-r19 an optional field: when present, it indicates the serving cell on which the UE-initiated CSI report is to be sent on PUSCH; when absent, the PUSCH resources are on the same serving cell as this report configuration.

(38.214) ##### 5.2.1.5.4 UE Initiated reporting … If the number of event instances determined by the counter for such reference signal is greater than or equal to eventInstanceCount, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. If timeAlignmentTimer associated with the Cell configured with the CSI-ReportConfig expires, the UE does not send a CSI report via PUSCH resource after transmission of UEIRI.

Proposal 1: servCellIndex-r19 in CSI-ReportConfig -> reportTransmissionMode-r19 -> pusch-ResourceOfModeB-19 can be optional field.

- Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: servCellIndex-r19 in CSI-ReportConfig -> reportTransmissionMode-r19 -> pusch-ResourceOfModeB-19 can be optional field.