3GPP TSG-RAN WG2 #131bis	R2-2507567

Prague, Czech, 13th – 17th October 2025

Agenda item:	8.19.2 (TEI19_[MINT_Ph2_EPS])

Source:	LG Electronics Inc., Nokia

Title: 	Introduction of MINT in EPS

Document for:	Discussion and Decision

1. Introduction

In this contribution, we discuss the necessary impacts to support MINT in EPS as requested by CT1 LS[1].

2. Discussion

RAN2 receives an LS [1] from CT1 to ask RAN2 to work on support of disaster roaming in EPS. MINT was introduced in Rel-17 to support roaming of UEs that are losing connectivity in their home PLMNs due to disaster, and the support of MINT in REl-17 is limited to RAN connected to 5GC.

According to the LS, RAN2 is tasked to work on the following:

- TASK#1: E-UTRAN connected to EPC also broadcasts disaster broadcast information included in SIB30

  - a)	disaster related indication; or

  - b)	"list of one or more PLMN(s) with disaster condition for which disaster roaming is offered by the available PLMN" where each PLMN with disaster condition is identified by its PLMN ID. The list will need to be able to hold at least the same amount of PLMN IDs as number of PLMNs which can share an E-UTRAN cell.

- TASK#2: E-UTRAN broadcasts access-barring parameters for access barring for disaster roaming services.

2.1 Disaster broadcast information

TASK#1: E-UTRAN connected to EPC also broadcasts disaster broadcast information included in SIB30

There are two options to implement TASK#1

- Option1: Within existing SIB30, introduce new fields corresponding to the disaster broadcast information related to MINT in EPS

- Option2: Introduce a new SIB to include new fields corresponding to the disaster broadcast information related to MINT in EPS

It is clear that option1 has less impact to 36331 and fulfils the requirement CT1 has asked. So, option1 is preferred.

Proposal1: Introduce disaster broadcast information fields for MINT in EPS in existing SIB30.

2.2 Access control for disaster roaming

TASK#2: E-UTRAN broadcasts access-barring parameters for access barring for disaster roaming services.

In EPS in 5GS, access barring is performed as part of Unified Access Control (UAV). Access Identity 3 (AI3) and corresponding access barring parameters were introduced to support access barring for disaster roaming. For access attempts subject to disaster roaming, NAS indicates AI3 to AS so that the access attempt is performing UAC checking with AI3 access barring parameters. Note that for access attempt with AI3, access barring check related to access categories is not applied. That is, if randomly drawn number is lower than access control threshold corresponding to AI3, the access is allowed, and barred otherwise.

In E-UTRAN connected to EPC, since UAC is not applicable, a different mechanism from UAC with AI3 should be applied to access barring for disaster roaming in EPS.

Approach1) Introduce a separate disaster roaming access barring for disaster roaming in EPS. Access attempt indicated as disaster roaming in EPS by upper layers only applies the disaster roaming access barring check but not others (other existing access barring control mechanisms).

In this approach, a separate access barring parameters for disaster roaming in EPS are introduced in SIB2. Then, for any access attempt indicated as disaster roaming in EPS by upper layers, the separate access barring parameters apply for access barring check.

In our view, the approach1 is straightforward and in line with CT1 intention for support of MINT in EPS. We also think this is more similar to access barring with AI3 in MINT in 5GS, where access barring with AI3 is completely de-coupled with access barring with others

Approach2) Apply existing LTE access control mechanism(s) to disaster roaming in EPS

In LTE, there are separate access barring mechanisms introduced across several releases, such as normal access class barring (ACB), ACB for emergency call/MO signaling/MO data, barring skipping for MMTEL video/voice, barring skipping SMS, ACB for CSFB, Application specific Congestion control for Data Communication (ACDC) barring. If we attempt to apply existing access barring mechanism(s) to disaster roaming in EPS, we may consider to applying ACB for MO signaling, where we still need to introduce separate ACB parameters for MO signaling related to disaster roaming, and apply the separate ACB parameters for MO signaling for disaster roaming access attempt.

In our view, it is unclear if reusing existing ACB mechanism to disaster roaming access attempts is in line with CT1 intention. This approach implies that disaster roaming access attempt is always mapped to a certain call type, e.g., MO signaling, if we decide to apply MO signaling. Furthermore, even though we apply existing ACB procedure (e.g.,ACB for MO signaling) to disaster roaming access, we anyway need to introduce separate barring fields corresponding to disaster roaming access barring (e.g., new fields corresponding to ACB for MO signaling for disaster roaming). For this reason, we see the approach2 less attractive.

Comparing approach1 and approach2, we propose to go for approach1, since approach1 is straightforward and more in line with existing access barring check with AI3 (i.e., access control for disaster roaming in 5GS). However, if other approaches are considered, it is better to ask CT1 and SA2 detailed requirements (or their intention) for access control against disaster roaming in EPS.

Proposal 2: Introduce a separate disaster roaming access barring for disaster roaming in EPS. Access attempt indicated as disaster roaming in EPS by upper layers only applies the disaster roaming access barring check but not others (other existing access barring control mechanisms)

Proposal 3: If other approaches including approach2 is considered, send an LS to CT1 and SA2 to ask for detailed requirements for access control against disaster roaming in EPS.

Other specification impact

For PLMN selection, UE NAS needs to know which PLMN supports disaster roaming and which disaster PLMNs are accepted based on reporting from AS, as described in section 4.2 of 36.304.

Section 4.2 or 36.304

| Idle Mode Process | UE Non-Access Stratum | UE Access Stratum |
| --- | --- | --- |
| PLMN Selection | Maintain a list of PLMNs in priority order according to TS 23.122 [5]. Select a PLMN using automatic or manual mode as specified in TS 23.122 [5] and request AS to select a cell belonging to this PLMN. For each PLMN, associated RAT(s) may be set. Evaluate reports of available PLMNs and, for E-UTRA if the UEs supports E-UTRA connected to 5GC, CN type(s) from AS for PLMN selection. Maintain a list of equivalent PLMN identities. Maintain a list of "PLMNs not allowed to operate at the present UE location". Maintain applicable disaster roaming information type for available PLMNs including potential disaster PLMNs for available PLMNs. | Search for available PLMNs. If associated RAT(s) is (are) set for the PLMN, search in this (these) RAT(s) and other RAT(s) for that PLMN as specified in TS 23.122 [5]. Perform measurements to support PLMN selection. Synchronise to a broadcast channel to identify found PLMNs (and CN type(s). Report available PLMNs with associated RAT(s) and, for E-UTRA if the UE supports E-UTRA connected to 5GC, CN type(s) to NAS on request from NAS or autonomously. Report applicable disaster roaming information for available PLMNs autonomously including potential disaster PLMNs. |

If disaster roaming in EPS is introduced on top of disaster roaming in 5GS, UE NAS needs to know CN type of the PLMN supporting the disaster roaming in order to distinguish whether the PLMN supporting disaster roaming is corresponding to disaster roaming support in 5GS or EPS.

Proposal 4: Update section 4.2 of 36.304 such that NAS maintains applicable disaster roaming information and CN type for available PLMNs including potential disaster PLMNs for available PLMNs, and AS reports applicable disaster roaming information for available PLMNs autonomously including potential disaster PLMNs and CN type.

Disaster roaming is basically a NAS capability. But since it involves a separate access barring, the disaster roaming in 5GS is specified as optional E-UTRA/5GC capability in 36.306.

It is optional for UE to support minimization of service interruption including reporting to NAS of disaster roaming information for available PLMNs and Access Barring check for Access Identity 3, as specified in TS 36.331 [5].

Then, it may be necessary to specify disaster roaming in EPS in 306 as well. Since this is optional capability without signaling (at least from AS capability perspecteive), it can be specified under section 6 (section for optional features without UE radio access capability parameters).

Proposal 5: Discuss whether to introduce a disaster roaming capability as optional capability without signaling in 36.306.

In 36.300, disaster roaming in 5GS is described, but this is not applicable to disaster roaming in EPS.

Proposal 6: Introduce description for disaster roaming in EPS in 36.300

CRs corresponding to proposals are provided in [2][3][4][5].

3. Conclusion

Proposal1: Introduce disaster broadcast information fields for MINT in EPS in existing SIB30.

Proposal 2: Introduce a separate disaster roaming access barring for disaster roaming in EPS. Access attempt indicated as disaster roaming in EPS by upper layers only applies the disaster roaming access barring check but not others (other existing access barring control mechanisms)

Proposal 3: If other approaches than proposal2 is considered, send an LS to CT1 and SA2 to ask for detailed requirements for access control against disaster roaming in EPS.

Proposal 4: Update section 4.2 of 36.304 such that NAS maintains applicable disaster roaming information and CN type for available PLMNs including potential disaster PLMNs for available PLMNs, and AS reports applicable disaster roaming information for available PLMNs autonomously including potential disaster PLMNs and CN type.

Proposal 5: To discuss whether to introduce a disaster roaming capability as optional capability without signaling in 36.306.

4. Reference

[1] R2-2506707, LS on Broadcasting Information on Disaster Condition of a PLMN from E-UTRAN in Case of Disaster Condition, CT1 (LG Electronics)

[2] R2-2507568, CR to 36.331 Introduction of MINT in EPS, LG Electronics Inc., Nokia

[3] R2-2507570, CR to 36.304 Introduction of MINT in EPS, LG Electronics Inc., Nokia

[4] R2-2507571, CR to 36.306 Introduction of MINT in EPS, LG Electronics Inc., Nokia

[5] R2-2507572, CR to 36.300 Introduction of MINT in EPS, LG Electronics Inc., Nokia