3GPP TSG RAN WG2 #131-bis			R2-2507584

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	10.4

Source:	BT Plc

Title:	High level requirements for 6GR mobility

Document for:	Discussion

# Introduction

The 6GR Study Item (SI) description was approved in RAN#108 [1]. One of the main objectives is the 6GR mobility lead by RAN2.

(4) Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] (7) Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: a) 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] b) Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. c) Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

This TDoc captures the main topics to be prioritized during the 6GR study phase

# Discussion

## Future proof 6GR mobility framework

Mobility is a fundamental aspect of 3GPP Radio Access Technologies (RATs). Following 3GPP’s historical trend, 6G will remain under specification for at least the next 10 years and is expected to be operational for no less than a decade. Therefore, the 6GR mobility framework must be designed to support both current and future service requirements.

- RAN2 should design a future proof 6GR mobility framework ready to support initial and future services without any framework modification.

## 6GR intra-frequency and inter-frequency mobility

Similar to previous 3GPP generations, it is expected that 6GR UEs natively support inter-frequency and intra-frequency mobility for all RRC UE states as shown in Figure 1.

Figure 1 UE intra-RAT mobility across differenet RRC 6GR states

UE capabilities for mandatory features are sometimes added to facilitate IODT and accelerate implementation. However, this often leads to capabilities being implemented based on commercial requirements, resulting in partially implemented functionality.

After several lessons learned from previous generations, it is reasonable to assume that in 6GR, only features with clear relevance, tangible benefits, and real commercial value will be specified. Consequently, there should be no need to include UE capabilities only for IODT purposes. For Non-Terrestrial Network (NTN) capable UEs, this principle extends to mobility between NTN and TN, regardless of the frequency band.

- RAN2 should study the 6GR mobility framework allowing mobility across all UE 6GR RRC states defined to be supported without any UE capabilities.

To achieve mobility across 6G bands, only 6G signalling should be required.

- RAN2 should focus the study under the assumption that the 6GR intra-frequency and inter-frequency mobility framework requires 6G signalling only.

Layer 3 (L3) measurements have supported mobility since LTE, with implementations well understood by vendors. This approach has proven reliable and suitable for most services deployed in LTE and 5G NR. However, the L3 measurement framework have reached their limits. As a result, 5G NR explored alternative solutions with varying commercial outcomes.

5G NR introduced Conditional Handover (CHO), Dual Active Protocol Stack (DAPS), and Layer 1/Layer 2 Triggered Mobility (LTM). DAPS failed to gain commercial traction due to its complexity outweighing the perceived benefits, and vendors have not implemented it. CHO has shown value in Non-Terrestrial Networks (NTN), but its benefits for Terrestrial Networks (TN) remain unclear, leading to limited adoption. In contrast, LTM has generated interest among MNOs, as it is expected to reduce handover delays.

Looking ahead, 3GPP is expected to work more efficiently than in 5G. To achieve this, it must prioritize the quality of features over the quantity.

5G NR mobility solutions without a clear implementation commitment, without a clear use case or, without a clear benefit vs cost can be considered a 3GPP failure.

Since 3GPP Release 18 [2], a framework based on Layer 1 and Layer 2 measurements has been under specification. The initial focus of Rel-18 LTM was to reduce interruption time and latency during intra-CU handovers due to mobility. In Release 19 [3], further mobility enhancements were introduced, including inter-CU LTM, event-triggered L2 reporting for LTM, and the use of CSI-RS reference signals for LTM-based mobility.

Rel-18 and Rel-19 are based on the principle of network coordination with potential candidate cells. However, this requires extensive signaling between network nodes to formulate the resulting RRC configuration for the UE.

In Release 20 [4], enhancements should specify greater flexibility for the network to dynamically change the LTM configuration using L2 signalling without RRC messages and to provide the means for the network to activate SCells dynamically as part of SpCell change with LTM cell switch command.

All lessons learned during Releases 18, 19, and 20 should be considered when defining the first LTM-6G-like framework.

6GR needs to consider all the lessons learnt during 5G NR LTM specification.

During Rel-18 specification period, it was acknowledge the complexity to add L1 and L2 measurements. That motivated to add L3 measurements to LTM framework initially intended for L1/L2 measurements only.

- RAN2 is encouraged to proceed with the 6GR LTM study item considering the lessons learned from 5G NR delivering an enhanced LTM framework compared with Rel-20.

As network slicing becomes relevant in 5G, mobility scenarios involving multiple slices with different deployment scopes are increasingly relevant. In practice, slices may be configured with nationwide coverage, or deployed locally for specific use cases such as industrial automation, public safety, or temporary events.

This diversity has introduced new challenges for mobility management. A UE may move between areas where the availability or configuration of slices changes, which can affect service continuity, QoS enforcement, and session anchoring.

To ensure robust support for such scenarios in 6G, RAN2 should study slice-aware mobility procedures that consider slice availability and suitability in the target cell, support seamless transitions between slices when necessary, and minimize service interruption and signaling overhead. These aspects are critical to ensure that mobility management remains efficient and scalable in multi-slice environments.

Incorporating these considerations into the study phase will help ensure that future specifications are aligned with realistic deployment models and operator needs, while enabling consistent user experience across heterogeneous slice configurations.

- RAN2 needs to include in the study the mobility when the network is configured with multiple slices and each slice can be deployed ad-hoc or nationwide.

## Inter-RAT mobility

For the 6G initial deployment scenarios, it is required to study the 6GR mobiliy framework in a way that the mobility framework supports mobility across all the UE states, as shown in Figure 2.

Figure 2 UE inter-RAT mobility across all different UE states

As for the inter-RAT use case, there is no need to introduce UE capabilities as mobility should be at the core of 6GR.

- RAN2 study prioritizes the inter-RAT mobility between 5G NR and 6GR without any UE capabilities

By the time the first commercial ready 6G specification is prepared for implementation, companies are expected to still be investing in 5G, especially considering that the full end-to-end 5G architecture is still ramping up.

Full end-to-end 5G ecosystem is still ramping up in most countries.

During the 6G launch period, companies will prioritize 6G, while maintaining limited resources in 5G for essential corrections and final developments. It is unrealistic to expect companies to continue investing simultaneously in 6G, 5G, and 4G during the 6G launch phase. At that time, companies involved in the 6G launch will be focused on multiple 6G IODT activities. In parallel, they will need to invest in ensuring inter-RAT mobility between 6G and 5G. Adding extra workload could compromise quality, or, to maintain quality, companies will need to increase their investment to guarantee interoperability with other 3GPP technologies.

RAN2 needs to focus the efforts to study the mobility across all UE RRC states between 5G and 6G.

During the transition from 4G to 5G, the number of exclusive 5G NR bands was limited. To address this, 5G NR specified Dynamic Spectrum Sharing (DSS), allowing 4G LTE and 5G NR to share the same spectrum. For 6GR, the idea is to adopt a similar approach by specifying Multi-RAT Spectrum Sharing (MRSS). Hopefully, MRSS will operate more efficiently, as DSS has proven to fall short of expectations, causing significant challenges for MNOs.

It is to early to know how MRSS will work, but RAN2 should design a mobility framework where the fact that the spectrum is shared among RATs becomes irrelevant. It should not impact in any way the UE behaviour and the UE performance to execute the mobility from and to any RRC state.

For spectrum sharing, the mobility solution should be transparent to any UE RRC mobility procedure, and the KPIs to define the mobility should meet the same criteria.

In 5G NR, the UE can be in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE. At this point in time, it is unclear how many RRC states will be specified in 6GR. What is clear is that the UE should support inter-RAT mobility from any RRC state.

To avoid multiple solutions that complicate implementatiosn, RAN2 should focus on the service supported by the UE. Services are categorized with a specific QoS and QoE. Regardless of the UE form factor or the UE connectivity type, the same 6GR mobility framework should be used. The mechanisms to trigger the mobility should be common to any UE.

- RAN2 to study a single 6GR intra-RAT and inter-RAT mobility framework that guarantees the performance of the service independently of the connectivity type and the MRSS solution without any impact to 5G UEs mobility performance, even the MRSS is done with partial or complete spectrum sharing.

# Conclusion

The observations done during the TDoc are summarised as follows:

Observation 1	5G NR mobility solutions without a clear implementation commitment, without a clear use case or, without a clear benefit vs cost can be considered a 3GPP failure.

Observation 2	6GR needs to consider all the lessons learnt during 5G NR LTM specification.

Observation 3	Full end-to-end 5G ecosystem is still ramping up in most countries.

Observation 4	RAN2 needs to focus the efforts to study the mobility across all UE RRC states between 5G and 6G.

Observation 5	For spectrum sharing, the mobility solution should be transparent to any UE RRC mobility procedure, and the KPIs to define the mobility should meet the same criteria.

The proposals to be considered by RAN2 are captured below:

Proposal 1	RAN2 should design a future proof 6GR mobility framework ready to support initial and future services without any framework modification.

Proposal 2	RAN2 should study the 6GR mobility framework allowing mobility across all UE 6GR RRC states defined to be supported without any UE capabilities.

Proposal 3	RAN2 should focus the study under the assumption that the 6GR intra-frequency and inter-frequency mobility framework requires 6G signalling only.

Proposal 4	RAN2 is encouraged to proceed with the 6GR LTM study item considering the lessons learned from 5G NR delivering an enhanced LTM framework compared with Rel-20.

Proposal 5	RAN2 needs to include in the study the mobility when the network is configured with multiple slices and each slice can be deployed ad-hoc or nationwide.

Proposal 6	RAN2 study prioritizes the inter-RAT mobility between 5G NR and 6GR without any UE capabilities

Proposal 7	RAN2 to study a single 6GR intra-RAT and inter-RAT mobility framework that guarantees the performance of the service independently of the connectivity type and the MRSS solution without any impact to 5G UEs, even the MRSS is done with partial or complete spectrum sharing.

# References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO, 3GPP TSG RAN#108, Prague, Czech Republic, June 9-13, 2025.

- NR_Mob_enh2-Core; leading WG: RAN2; REL-18; WID:RP-233970

- NR_Mob_Ph4-Core; leading WG: RAN2; REL-19; WID: RP-252111

- NR_Mob_Ph5; leading WG: RAN2; REL-20; WID: RP-252113