3GPP TSG-RAN-WG2 Meeting #131bis	R2-2507590

Prague, Czech Republic, 13th– 17th Oct, 2025

Agenda item	:		8.13.2

Source	: 	Sharp

Title	: 	[O503] [Z454] [Z455] [Z456] [O505] [K002] [E044] [H452] [H454] discussion on remaining issues related to C-plane procedure for multi-hop relay

Document for	:	Discussion and Decision

Introduction

RAN2 started ASN.1 review and RRC CR Rapp suggested to discuss some remaining issues[1]:

In this paper, we discuss the RIL of which status is set to ToDo.

Discussion

  - RIL related issues

O503: SUI initiation for discovery transmission resource requesting

O503 suggested to define a behaviour of an intermediate relay UE to use multi-hop specific threshold for determination on whether the UE should transmit SUI to request discovery transmission resource. We generally agree with the proposed change from O503, but an intermediate relay UE doesn’t need to check whether the condition of a remote UE is fulfilled as Rapp indicated.

- [O503] RAN2 to agree with the proposed change from O503 with removing the part that an intermediate relay UE check whether the condition of a remote UE is fulfilled.

Z454: Source L2 ID report

Z454 indicated that the specification does not state whether multi-hop remote UEs should report their source L2 ID. Regarding the proposed change, we agree with the first change, but the second change is not needed since the UE transmits a SUI message as a remote UE. Therefore, it is already clear which source ID should be reported in the SUI message.

3> if SIB12 includes sl-L2U2N-Relay or sl-L2U2N-MH-Relay and the UE is capable of L2 U2N remote UE: 4> include sl-SourceIdentityRemoteUE corresponding to the upstream direction and set it to the source identity configured by upper layer for NR sidelink L2 U2N relay communication transmission;

- [Z454] RAN2 to agree with the first change provided by the proposed change from Z454.

Z455: Paging ID report

Z455 mentioned that handling of paging ID lists in multi-hop relay scenarios is not clearly defined, especially when relays receive paging info for multiple unconnected child UEs. In the current CR, a last relay UE can report only one paging information in SUI even if the UE receives a paging information list. It is unreasonable that the list cannot be notified of the gNB even though it has reached the last relay UE. Providing multiple paging IDs to the gNB in ​​individual entries is possible with the legacy SUI format. However, in a multi-hop scenario, the last relay UE does not know the destination ID corresponding to the paging ID of the unreachable UE, so it is not possible to use the existing IE and procedure. For these reasons, we agree to introduce a paging ID list in the SUI.

- [Z455] RAN2 to agree to introduce a paging ID list in the SUI.

Z456: UE type in SUI message

According to Z456, it is unclear how to set contents of SUI message made by an intermediate relay UE. In Rel-17, remote/relay UE sends SUI to gNB for resource acquisition including UE-type information. And in Rel-18, U2U specific UE-type is introduced. In multi-hop case, the last relay UE should use the ue-Type as relayUE, and the remote UE should set the ue-Type as remoteUE. For the intermediate relay UE, RAN2 should discuss whether the new UE type is specified, or legacy UE type (remoteUE or relayUE) is reused. In our understanding, a CONNECTED intermediate relay UE is always a remote UE for receiving/sending RRC messages from/to gNB. But the intermediate relay UE may or may not have its own UP data. Therefore, we agree to define a new UE type because setting appropriate information elements for an intermediate relay UE to notify the gNB that it is an intermediate relay UE is friendly to network operations and clarifies UE behaviour.

- [Z456] RAN2 to introduce a new UE type in SUI for an intermediate relay UE to explicitly represent dual role.

And it should be discussed which information should be included if the SUI is sent from an intermediate relay UE. In Rel-18, Relay UE and Remote UE has specific report in sl-TxResourceReqL2U2NRelay in sl-TxResourceReqListCommRelay. One option is that an intermediate relay UE sets the fields in one or more entries as a relay UE and sets the fields in one entry as a remote UE. In that case, some information might be duplicated from NW perspective, e.g., sl-TxInterestedFreqListL2U2N, sl-TypeTxSyncListL2U2N, sl-PagingIdentityRemoteUE (it might also be reported by the (parent) last relay UE) and sl-CapabilityInformationSidelink (in case of 3-hop relay, a first relay UE sends information of a remote UE and an intermediate relay UE, an intermediate relay UE sends information of a first relay UE and a last relay UE, and a last relay UE sends information of an intermediate relay UE). Thus, to avoid such duplicated information reporting, intermediate relay UE specific information reporting is helpful, i.e. intermediate relay UE doesn’t indicate capability of a parent UE in SUI. The following table is summary of above discussion.

Table 1. ue-Type specific field settings in sl-TxResourceReqL2U2N-Relay within SUI

| IE | field | Rel-18 U2N relay UE | Rel-18 U2N Remote UE | Intermediate Relay UE |
| --- | --- | --- | --- | --- |
| sl-TxResourceReqL2U2N-Relay | SL-DestinationIdentity | x |  | x (for a child UE) |
|  | sl-TxInterestedFreqListL2U2N | x | x | x |
|  | sl-TypeTxSyncListL2U2N | x | x | x (current sync types for each frequency) |
|  | sl-LocalID-Request | x |  | x (for a child UE) |
|  | sl-PagingIdentityRemoteUE | x |  | ? (for a child UE) |
|  | sl-CapabilityInformationSidelink | x | x | x (for a child UE) |
| ue-Type |  | relayUE | remoteUE | (newType) |

- [Z456] Intermediate relay UE specific information reporting is introduced in SUI to avoid duplicated information reporting by multiple relay UEs.

O505: Paging delivery list

O505 said that the current specification doesn’t support multiple paging delivery in a SUI message, but it is not efficiently in multi-hop relay scenario. In legacy, when a relay UE needs to send multiple paging delivery for each remote UE, the relay UE sends SUI message including a paging message for each remote UE. However, in multi-hop scenario, in case that a remote UE is paged, relay UEs on the path should be wakeup and inteligent NW should page the relay UEs. Thus, it is natural to introduce a paging delivery list in UuMessageTransferSidelink to avoid signalling overhead due to multiple paging delivery.

- [O505] RAN2 to introduce a paging delivery list in UuMessageTransferSidelink to avoid signalling overhead due to multiple paging delivery.

K002: SIB/Paging information release due to SL RLF

K002 mentioned that the current specification doesn’t support triggering for releasing SIB/Paging information by intermediate relay UEs in IDLE/INACTIVE upon SL RLF. We think proponent assumed a case where an intermediate relay UE detects SL RLF on downstream hop. If so, we think it is reasonable to define the behaviour of intermediate relay UE in IDLE/INACTIVE. However, such assumption is not captured in the proposed text. Furthermore, the other events, e.g. PC5 link release, are not captured in the current specification. Therefore, RAN2 needs further check the procedural text.

- [K002] RAN2 to agree that intermediate relay UE in IDLE/INACTIVE sends remote UE information to release SIB/Paging information at parent UE when the UE detects some trouble on downstream. Procedural text should be further checked in CR.

E044: Prioritise candidate relay only based on RRC state

E044 mentioned that the note is not aligned with RAN2 agreement.

The note is below:

NOTE X:	The L2 U2N Remote UE may prioritize the selection or reselection of suitable NR sidelink U2N Relay UE based on any information available in the discovery message including the RRC State information. The RRC State information in the discovery message RRC container reflects the state of the UE that sends the discovery message.

And the RAN2 agreement is below:

Capture in a revision of the existing note in RRC running CR (relay reselection section 5.8.15.3) that the UE may prioritise relays based on any information available in the discovery message for the applicable model.

First, the selection of which relay UE to use depends on the implementation of the UE, and there is an appropriate selection method for each UE. Additionally, the application layer is also involved in this selection process, and the UE may choose a candidate relay based on other parameters (for example, the strength of the radio link measured at the AS layer). Therefore, RAN2 should not accept proposal E044, which suggests selecting the UE solely based on the RRC state.

- [E044] RAN2 to confirm the UE can select any candidate relay UE by UE implementation and the RRC state information is one of the factors to consider.

H452: hop count limitation

H452 suggested to introduce hop count limitation of discovery forwarding to avoid exceed the hop limitation specified in the definition. However, we assume that the hop limit mechanism is discussed and specified in SA/CT. Thus, RAN2 doesn’t need to introduce a duplicated mechanism. However, RAN2 can send LS to indicate the concern to SA2/CT1.

- [H452] RAN2 to reject the RIL H452 and send LS to indicate the concern (The forwarding of discovery messages beyond the hop limit restricted by the RAN WID) to SA2/CT1

H454: Notification message omission condition

In H454, the issue is the notification message omission by intermediate relay UEs in IDLE/INACTIVE does not consider hop count. According to the current RAN2 agreement, hop count “CAN” be considered as a AS parameter to determine whether to omit transmission of the notification message. We also think that the hop count should be related to the notification message omission condition. However, we believe that a trigger of the notification message should be “hop count change”, not “increase of hop count”. Hence, RAN2 needs to discuss whether to capture the hop count change as a notification message omission condition.

- [H454] RAN2 to capture “hop count change” as a notification message omission condition in the specification.

Conclusion

In this paper, we made following observations and proposals:

- [O503] RAN2 to agree with the proposed change from O503 with removing the part that an intermediate relay UE check whether the condition of a remote UE is fulfilled.

- [Z454] RAN2 to agree with the first change provided by the proposed change from Z454.

- [Z455] RAN2 to agree to introduce a paging ID list in the SUI.

- [Z456] RAN2 to introduce a new UE type in SUI for an intermediate relay UE to explicitly represent dual role.

- [Z456] Intermediate relay UE specific information reporting is introduced in SUI to avoid duplicated information reporting by multiple relay UEs.

- [O505] RAN2 to introduce a paging delivery list in UuMessageTransferSidelink to avoid signalling overhead due to multiple paging delivery.

- [K002] RAN2 to agree that intermediate relay UE in IDLE/INACTIVE sends remote UE information to release SIB/Paging information at parent UE when the UE detects some trouble on downstream. Procedural text should be further checked in CR.

- [E044] RAN2 to confirm the UE can select any candidate relay UE by UE implementation and the RRC state information is one of the factors to consider.

- [H452] RAN2 to reject the RIL H452 and send LS to indicate the concern (The forwarding of discovery messages beyond the hop limit restricted by the RAN WID) to SA2/CT1

- [H454] RAN2 to capture “hop count change” as a notification message omission condition in the specification.

Reference

- R2-250xxxx, SLRelay related RIL from ASN.1 review