3GPP TSG-RAN WG2 #131bis	Tdoc R2-2507604

Prague, Czech Republic, 2025-10-13 - 2025-10-17

Agenda Item:	8.19.2

Source:	Ericsson

Title:	Adapting TTT based on mobility state in NR

Document for:	Discussion

# 1	Introduction

In the last RAN2 meeting there was a proposal to introduce the speed-based TTT scaling mechanism from LTE in NR to improve handover success rate in high-speed train scenarios [1]. As there were concerns from some companies the proposal was not agreed but it was decided that the topic could be re-visited again in a later meeting.  In this contribution we describe our view on the matter.

# 2	Discussion

In LTE, a UE in RRC_CONNECTED can scale the time-to-trigger (TTT) handover parameter based on its estimated speed state (normal, medium, high). The speed state is estimated based on the handover rate (#HOs/Time period). The intention with the scaling is to receive the measurement report from the UE early enough so that the handover command can be sent successfully to the UE.

#### 5.5.6.2	Speed dependant scaling of measurement related parameters

The UE shall adjust the value of the following parameter configured by the E-UTRAN depending on the UE speed: timeToTrigger. The UE shall apply 3 different levels, which are selected as follows:

The UE shall:

1>	perform mobility state detection using the mobility state detection as specified in TS 36.304 [4] with the following modifications:

2>	counting handovers instead of cell reselections;

2>	applying the parameter applicable for RRC_CONNECTED as included in speedStatePars within VarMeasConfig;

1>	if high mobility state is detected:

2>	use the timeToTrigger value multiplied by sf-High within VarMeasConfig;

1>	else if medium mobility state is detected:

2>	use the timeToTrigger value multiplied by sf-Medium within VarMeasConfig;

1>	else:

2>	no scaling is applied;

As the network is also able to count the number of handovers, and since the TTT is configurable parameter in the measurement event configuration, TTT scaling could just as well be implemented on the network side. Unlike the UE side solution, the network-side solution can be used for all UEs and not only new UEs supporting the new functionality. To count the number of handovers the network could utilize the “Last visited cell” information available in the Xn/Ng handover request which contains information about the past cells and the time spent in each cell. Another option is to introduce proprietary parameter in the Xn/Ng handover request that allows the network to track the handover count.

Speed-based TTT scaling where the speed state is estimated based on handover rate can be implemented on the network side without requiring any new UE functionality.

Using number of handovers to estimate mobility state is not a very reliable method though if handovers are triggered for other reasons than mobility, for example for load-balancing. However, if the mobility state estimation is done on the network side it would be possible to exclude the non-mobility triggered handovers. The same is not possible if the estimation is done on the UE side as the UE doesn’t generally know what triggered the handover.

Estimating mobility state based on number of handovers is unreliable if handovers are triggered for other reasons than mobility, e.g. for load balancing reasons.

One advantage of the network-side solution is that handovers triggered for non-mobility reasons can be excluded in the handover count.

Estimating the mobility state based on the number of handovers is also not reliable if handovers if cell sizes vary. This was also the conclusion in the LTE Rel-11 study on Mobility Enhancements for Heterogenous Network (HetNet) [2] (see below). The study was for HetNet but also in regular macro network cell sizes vary a lot due to e.g. transmission power, frequency, antenna height and tilt, buildings, etc. Since cell sizes vary it’s difficult to find a single speed state threshold that works accurately across the network. However, we do acknowledge that the method can be reliable enough in certain scenarios like the high-speed train scenario where cell sizes are relatively similar and where the goal is to distinguish pedestrians on the side of the railway track from passengers onboard the train.

### 5.6.2	Overall observations on Mobility speed estimation

On mobility state estimation (MSE) and its impact on mobility performance, it was observed that the MSE is not as accurate in HetNet environments as in macro only deployments since it does not take into account cell sizes.

It was however agreed that possible enhancements to the UE-based MSE should serve the purpose of enhanced mobility performance (not only for the sake of enhancing the MSE estimate). There is consensus that enhancements should be considered to improve the mobility performance of HetNet. This includes UE and network based mechanisms.

Number of handovers is generally a poor metric to determine mobility state of the UE as cell sizes vary. However, for the high-speed train scenario it may be accurate enough.

Considering the above, we make the following proposal:

TTT scaling can be implemented on the network side. The UE side TTT scaling mechanism from LTE should not be introduced in NR.

An alternative to using the number of handovers to determine the mobility state is to look at the RSRP variation. RSRP is used already today for determining low mobility UEs in the RRM relaxation feature introduced in NR Rel-16 (see below). A similar method could be used for determining normal/medium/high speed UEs; basically, if RSRP varies fast the UE is moving fast and if RSRP varies slowly the UE is moving slowly. RSRP can be more reliable than the handover count method as it is not affected by variations in cell size or non-mobility triggered handovers.

#### 5.7.13.1	Relaxed measurement criterion for low mobility

The relaxed measurement criterion for UE with low mobility in RRC_CONNECTED is fulfilled when:

-	(SS-RSRPRef – SS-RSRP) < SSearchDeltaP-Connected,

Where:

-	SS-RSRP = current L3 RSRP measurement of the SpCell based on SSB (dB).

-	SS-RSRPRef = reference L3 RSRP measurement of the SpCell based on SSB (dB), set as follows:

-	After receiving low mobility criterion configuration, or

-	After MAC of the CG successfully completes a Random Access procedure after applying a reconfigurationWithSync in spCellConfig of the CG while low mobility criterion is configured, or

-	upon an indication from lower layers that a RACH-less handover or an LTM cell switch is successfully completed for the CG while low mobility criterion is configured, or

-	If (SS-RSRP - SS-RSRPRef) > 0, or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP-Connected:

-	The UE shall set the value of SS-RSRPRef to the current SS-RSRP value of the SpCell.

There are also other handover related parameters besides TTT that the network may want to tune based on the UE mobility state, for example preferred frequency layer, event specific offsets, hysteresis, etc. For this reason, a more generic mechanism may be preferred where the UE reports its mobility state and the network adjusts the mobility parameter rather than UE adjusting the mobility parameter itself.

We realize that introducing a new mechanism for mobility parameter tuning requires more thorough analysis and is probably too late to introduce for NR and rather something that should be explored for 6G.

For 6G more robust methods for determining mobility state (e.g. based on RSRP) should be explored. Also consider how other mobility parameters than TTT can be tuned.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Speed-based TTT scaling where the speed state is estimated based on handover rate can be implemented on the network side without requiring any new UE functionality.

Observation 2	Estimating mobility state based on number of handovers is unreliable if handovers are triggered for other reasons than mobility, e.g. for load balancing reasons.

Observation 3	One advantage of the network-side solution is that handovers triggered for non-mobility reasons can be excluded in the handover count.

Observation 4	Number of handovers is generally a poor metric to determine mobility state as cell sizes vary. However, for the high-speed train scenario it may be accurate enough.

Based on the discussion in the previous sections we propose the following:

Proposal 1	TTT scaling can be implemented on the network side. The UE side TTT scaling mechanism from LTE should not be introduced in NR.

Proposal 2	For 6G more robust methods for determining mobility state (e.g. based on RSRP) should be explored. Also consider how other mobility parameters than TTT can be tuned.

# References

R2-2506114, Discussion on MobilityStateParameters in RRC_CONNECTED, KDDI, RAN2#131

TR 36.839 Mobility Enhancements for Heterogenous Network

# Annex: Chair notes

R2-2506114	Discussion on MobilityStateParameters in RRC_CONNECTED	KDDI Corporation (TTC)	discussion

Proposal 1:  RAN2 should discuss whether to introduce the same functionality as LTE in NR, which changes the values of mobility-related parameters (e.g., timeToTrigger) based on the UE's mobility state and delivers mobilityStateParameters in RRCReconfiguration.

-	Apple points out that we had this discussion in NR and we agreed it was not needed.   KDDI thinks that after we implemented NR we noticed it was mistake.   Huawei thinks that it make sense so we should go back to discussion to understand why it was not introduced.

-	Ericsson doesn’t think the number of HO is a good indication of mobility state.

-	ZTE thinks from implementation perspective this would be useful in the field especially for high speed scenarios.

-	Qualcomm remembers that the counting is not very reliable, but the scenario that KDDI is bringing may be a relevant scenario and should be considered.

Consider the enhancements and discuss possible solution/way to address it

Proposal 2:　RAN2 should discuss the release from which to introduce the functionality.

Proposal 3:　RAN2 should discuss additional candidate solutions for NR Mobility State evaluation method.

Noted