3GPP TSG-RAN WG2 Meeting #131bis                                                       R2-2507607

Prague, Czech Republic, Oct 13th - 17th 2025

Agenda item:	10.2

Source: 	ZTE Corporation

Title: 	Consideration on the 6G UE Capability

Document for:	Discussion and Decision

Introduction

During RAN#108 meeting, the study on 6G radio was approved [1]. The objective on the UE Capability is listed as follows:

Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] … - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

In this paper, we will share our views on the 6G UE capability based on some observations of 5G.

Discussion

In this chapter, we will first discuss the UE capability reporting mechanism, then we share our views on the UE capability signaling framework.

  - UE Capability Reporting Framework

In this section, we first present our general understanding on the basic UE capability reporting procedure, then share our views on the dynamic capability update, and finally briefly discuss the issue related to the RACS.

    - 2.1.1 Basic UE Capability Reporting Procedure

In legacy 4G and 5G networks, UE capability reporting is triggered by a network request message. This request typically includes a capability filter that instructs the UE to report only the capabilities the network is interested in, as illustrated in Figure 1.

Fig 1: Legacy UE Capability Reporting Procedure

In 5G, a significant issue is the growing number of band combinations (BCs) that UEs are required to report. This results in capability signaling that exceeds two segments, particularly when the filter includes both FR1 and FR2 bands. For 6G, this problem should be addressed from both the network request and UE reporting perspectives.

From the UE reporting side, optimizations to the capability signaling structure should be considered—a topic that will be further discussed in the next chapter. From the network side, additional filtering granularity may be necessary. During early 5G discussions, there was also a lot of discussion on how to configure the filter, such as whether to include band combination information or MIMO layer parameters. However, no consensus was ultimately reached. For 6G, these issues should be re-evaluated in light of existing UE capability reporting challenges and the potential structure of 6G networks.

Proposal 1: The basic UE capability reporting procedure can be retained from 4G/5G, where the UE reports capabilities based on a network request. RAN2 to study how to optimize the filtering mechanism to enable additional granularity for the filtering of UE capabilities.

    - 2.1.2 Dynamic Capability Update

Regarding dynamic capability updates, a growing number of companies are showing interest in this functionality. In 5G, two distinct capability update procedures are defined in the legacy system:

- 1: Semi-persistent capability update, implemented via NAS re-attach procedure.

24501 5.5.1.3.2: If the UE is in the 5GMM-CONNECTED mode and the UE changes the radio capability for NG-RAN or E-UTRAN, the UE may locally release the established N1 NAS signalling connection and enter the 5GMM-IDLE mode.

- 2: Temporary capability update, implemented via the AS UAI-based procedure.

For the first procedure, the UE indicates "UE radio capability update needed" in the Registration message to the core network. The core network then releases the previously stored UE capabilities, so, when the UE accesses the network, the network will not receive any stored UE capabilities from the core network, this triggers the network to request the updated capabilities from the UE. The whole procedure requires local detach and re-attach.

The second procedure does not affect the CN, as the temporary capability updates are only stored in the RAN node during the connected state. This UAI-based mechanism handles scenarios such as overheating, power saving, and MUSIM. Several UAI-driven functionalities are already deployed in commercial 5G networks, including RRC state transfer, MIMO layer adaptation, and DRX configuration.

Observation 1: Multiple UAI-based functionalities for UE radio capability update have been commercialized in 5G networks.

For 6G, new scenarios like foldable phones and UE aggregation have been proposed. However, it remains unclear whether these new scenarios would impact the capability update framework. Taking the foldable phone as an example, a key design objective is to maintain consistent performance regardless of whether the device is folded or unfolded. Therefore, a change in the folding state should not trigger a capability update. A potential issue could arise from overheating in the folded state, but this is similar to the legacy overheating scenario already addressed in 5G. Consequently, as a first step, we should discuss whether any new 6G scenarios necessitate enhancements to the current capability update mechanism.

Proposal 2: Regarding dynamic capability updates, RAN2 should first discuss whether there are new scenarios in 6G that require enhancements to the current 5G UE capability update mechanism.

    - 2.1.3 RACS

In 5G, in addition to explicit UE capability reporting, the RACS concept was introduced in Release 16 as an implicit reporting method. It aims to reduce signaling overhead by defining UE capability templates and identifying UE capabilities through a capability ID. However, its implementation in 5G requires a template that covers all UE capabilities, which has hindered its commercialization. For 6G, the template-based concept can be revisited. RAN2 should discuss how to optimize the RACS mechanism—for example, by avoiding the need to encompass all UE capabilities in a single template.

Proposal 3: For the 6G UE capability reporting, the UE should be able to report capabilities using both explicit and implicit methods. RAN2 to study how to optimize the RACS mechanism in 6G to improve its feasibility.

  - UE Capability Signaling Framework

In this section, we first present our general understanding on the UE device types, then share our views on the 5G UE capability structure problems, and finally discuss the issue on the problematic UE identification.

    - 2.2.1 UE Device Types

In 5G, all new features introduced after 3GPP Release 15 are defined as optional. In the first release of 5G, UE capabilities such as RedCap were not introduced due to concerns that lower-end devices might impact the 5G user experience, thereby bringing negative impact to 5G market promotion. For 6G, to achieve scalable design and economies of scale, we need to support different device types from Day 1. However, we need to convey a clear message to the market about the use cases for each device type and their minimum capabilities. This approach can prevent market fragmentation and ensure a positive user experience. Therefore in 6G, from UE capability design perspective, different device types should be considered, and it is necessary to define different mandatory features specified for each device type per release version.

Proposal 4: Define different device types, with mandatory features specified for each device type per release version.

    - 2.2.2 5G UE Capability Structure Problems

As in the WID, one objective of the 6G UE capability is to improve and simplify the Radio signaling framework compared to 5G NR. To achieve this objective, first we need to identify the potential issues in the current 5G structure, and then discuss the potential improvements or simplifications based on these issues. During discussions on 5G UE capability, many new concepts were introduced, such as the decoupling of RF and BPC, as well as UL and DL. To decouple RF and BPC capabilities, the concepts of Band Combination and Feature Set were proposed. To decouple DL and UL, the feature set was further divided into DL feature set and UL feature set. These solutions have contributed to signaling simplification. However, the potential benefits of decoupling of RF and BPC, as well as UL and DL, have not been fully realized under the current architecture.

RF and BPC Decoupling

The intention of RF and BPC decoupling is to reduce the repeated reporting for Band Combinations (BCs) that share the same BPC capabilities. To implement this, one BC is linked to one FeatureSetCombinationID. However, due to the current FeatureSetCombination architecture (as shown in Fig. 2) and the fact that a single FeatureSet includes both UL and DL parts, the FeatureSetCombination reuse rate is very low in actual networks.

Fig 2: FeatureSetEntry List in 5G UE Capability Structure

Based on capability reporting from some commercial UEs, one UE reports 58 FeatureSetCombinations, but only 4 are reused; another reports 158, with only 33 reused. This means the UEs report 191 Band Combinations (BCs) using 158 FeatureSetCombinations.

Furthermore, even when BCs share the same BPC capability, the current structure does not support reporting capabilities for a group of BCs. This limitation further diminishes the potential benefits of RF and BPC decoupling.

Observation 2: The intention of the FeatureSetCombination was to implement RF and BPC decoupling. However, the reuse rate of FeatureSetCombinations is low with the current structure, and it does not support reporting capabilities for a group of BCs.

DL and UL Decoupling

The purpose of decoupling DL and UL was to prevent redundant DL reporting for BCs that share the same DL capabilities. However, this has not been effectively implemented in the current structure as follows.

FeatureSetCombination information element -- ASN1START -- TAG-FEATURESETCOMBINATION-START FeatureSetCombination ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand FeatureSetsPerBand ::= SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet FeatureSet ::= CHOICE { eutra SEQUENCE { downlinkSetEUTRA FeatureSetEUTRA-DownlinkId, uplinkSetEUTRA FeatureSetEUTRA-UplinkId }, nr SEQUENCE { downlinkSetNR FeatureSetDownlinkId, uplinkSetNR FeatureSetUplinkId } } -- TAG-FEATURESETCOMBINATION-STOP -- ASN1STOP

With the current structure, each FeatureSet includes both featureSetDownlink and featureSetUplink, the UE can’t report more than one featureSetUplink for the same featureSetDownlink. Taking Table 1 from a commercial UE log as an example, the UE reports BCs for the combination of bands 28, 41, and 79, with up to 10 BCs listed. Among these 10 BCs, there are three different DL combinations. For each DL combination, there are three or four UL combinations. In other words, 3 or 4 UL combinations share the same DL combination. However, the UE still need to report 10 BCs for this case.

Table 1: Multiple reporting of the same BC

| Index | Band Numbers | DL BW class | DL FeatureSet ID list | UL BW class | UL FeatureSet ID list | FeatureSet Comb |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 28/41/79 | a,a,a | 4,1,9 | a,a,a | 8,7,0 | 6 |
| 2 | 28/41/79 | a,a,a | 4,1,9 | _,a,_ | 0,7,0 | 12 |
| 3 | 28/41/79 | a,a,a | 4,1,9 | _,a,_ | 0,2,0 | 20 |
| 4 | 28/41/79 | a,a,c | 4,1,5 | a,a,_ | 6,7,0 | 4 |
| 5 | 28/41/79 | a,a,c | 4,1,5 | _,a,a | 0,7,7 | 14 |
| 6 | 28/41/79 | a,a,c | 4,1,5 | _,a,_ | 0,2,0 | 1 |
| 7 | 28/41/79 | a,c,a | 4,10,1 | a,a,a | 8,7,0 | 16 |
| 8 | 28/41/79 | a,c,a | 4,10,1 | a,a,a | 0,7,7 | 8 |
| 9 | 28/41/79 | a,c,a | 4,10,1 | a,a,_ | 0,2,0 | 13 |
| 10 | 28/41/79 | a,c,a | 4,10,1 | a,c,_ | 0,9,0 | 7 |

Even if a super BC (with DL bandwidth classes a, c, and c, and UL bandwidth classes a, c, and a) were used with 10 FeatureSetEntries under the current FeatureSetCombination structure, the DL repetition in each FeatureSetEntry would still be unavoidable.

Observation 3: A FeatureSet inherently contains both downlink and uplink components, which undermines the potential gains of DL and UL decoupling.

If Observations 1 and 2 are correct, a more flexible structure for RF/baseband decoupling and DL/UL decoupling should be considered for 6G. Furthermore, 6G is expected to support more flexible network architectures (e.g., UL-only cells). This will introduce a wider variety of band combinations, which will also require a more flexible BC reporting structure.

Proposal 5:  RAN2 to study more flexible structure for RF/baseband decoupling and DL/UL decoupling.

Redundant BC Reporting Structure

Furthermore In the current 5G UE capability structure, two separate Band Combination (BC) reporting branches exist: one for normal BC and the other for Uplink Transmit (UL Tx) Switching.

BandCombinationList-v1840 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1840 BandCombinationList-UplinkTxSwitch-r16 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-r16 BandCombinationList-UplinkTxSwitch-v1630 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1630 BandCombinationList-UplinkTxSwitch-v1640 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1640 BandCombinationList-UplinkTxSwitch-v1650 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1650 BandCombinationList-UplinkTxSwitch-v1670 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1670 BandCombinationList-UplinkTxSwitch-v1690 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1690

This kind of structure leads to signalling redundancy and complexity. For example, the difference between BCs that support TxSwitching and those that do not may only lie in parameters at the UL FSPC level, while many parameters at the PerBC or even PerFeatureSet level may be consistent. However, for normal CA under a BC and CA implemented through the UL TxSwitching method, the UE has to report the common capabilities part repeatedly in both 2 braches.

This dual-branch architecture introduces signaling redundancy and complexity. For instance, the difference between BCs that support Tx Switching and those that do not might only involve parameters at the UL FSPC level, while many parameters at the PerBC or even PerFeatureSet level could be identical. Nevertheless, for capabilities common to both normal CA under a BC and CA implemented via the UL Tx Switching method, the UE is required to report them redundantly across both branches.

Observation 5: The dual BC reporting branch structure (for normal BC and UL Tx switching CA, respectively) in the 5G UE capability framework leads to signaling redundancy and increased complexity, as common capabilities often need to be reported repeatedly.

Besides the problems in the above observations, the other issue of the current UE capability structure is about the high repetition rate of capability parameters. In 5G, each capability parameter has a defined granularity (e.g., per UE, per band, per band combination, etc.). However, in practice, different UEs may support different granularities for the same capability parameter. According to logs from commercial UEs, some devices support coarser granularities than specified for certain finely granular parameters. For example, in the case of per-BC CSI-RS-related capability reporting (Table 2), some UEs report the same capability values across all band combinations. Even in such cases, the UE is still required to report these capabilities per BC, resulting in a high repetition rate. A similar issue occurs with other fine-grained reporting schemes, such as per-band capabilities or per FeatureSet capabilities.

Table 2: High Repetition Rate of Capability Parameters

| BC | DL and UL cc number | CSI-RS port Num | CSI-RS beam Num | CSI report num |
| --- | --- | --- | --- | --- |
| N1-N41-N77-N79 | DL4CC;UL2CC | 164 | 16 | 16 |
| N1-N28-N41 | DL3CC | 164 | 16 | 16 |
| N1-N28-N41-N77 | DL4CC | 164 | 16 | 16 |
| N28-N41 | DL3CC;UL2CC | 164 | 16 | 16 |
| N28-N41-N77-N79 | DL4CC;UL2CC | 164 | 16 | 16 |
| N40-N41-N79 | DL3CC | 164 | 16 | 16 |
| … | … | … | … | … |
| N41-N41-N41-N41 | DL4CC | 164 | 16 | 16 |

Observation 6: With 5G structure, although some UEs support a coarser granularity in practice for certain capability parameters, they are still required to report them at the defined finer granularity, resulting in a high repetition rate.

Proposal 6: For the 6G UE Capability structure, RAN2 to study how to avoid repeated reporting problem in the 5G.

    - 2.2.3 Problematic UE Identification

In 4G and 5G networks, there is a critical need for the network to identify UE/chipset vendor information to perform targeted optimizations for problematic devices. One example scenario where this could be useful is the deployment of a new feature: due to flaws in protocol design or UE implementation, some UEs may fail to execute the corresponding network configurations. To prevent widespread issues, the network is often forced to adopt conservative settings or disable the new feature entirely for all of the UEs, which can significantly hinder its commercialization.

Observation 7: Because of some problematic UEs, the network has to adopt conservative settings or disable the new features entirely for all of the UEs to prevent widespread issues, which can significantly hinder new feature commercialization.

According to the 3GPP standards, the RAN node can only obtain this information based on the IMEISV sent from the core network (i.e. this is not entirely new mechanism). However, the AMF does not always forward the IMEISV, which leads to inefficient problem resolution. For the case that the network doesn’t indicate the IMEISV info, the network has to identify the possible problematic UEs by implementation, for example, to identify the type of terminal by looking for unique capabilities in the UE capability information that distinguish it from other terminals. This method is particularly cumbersome and prone to misidentification. As a result, some non-standard methods have been implemented through proprietary collaborations between the network and UE/chipset vendors.

Specification can be continuously revised/updated to address previously undetected issues, but whether already-released terminals in the live network are upgraded depends entirely on the users. This poses a significant challenge for maintaining problematic terminals in the live network. In the 6G UE capability reporting mechanisms, these issues should be discussed and resolved from the very beginning.

Proposal 7: RAN2 should discuss mechanisms to enable network identification of UE/chipset information in 6G.

Conclusion

Based on the above discussion, we have the following observations and proposals:

UE Capability Reporting Mechanism

Observation 1: Multiple UAI-based functionalities for UE radio capability update have been commercialized in 5G networks.

Proposal 1: The basic UE capability reporting procedure can be retained from 4G/5G, where the UE reports capabilities based on a network request. RAN2 to study how to optimize the filtering mechanism to enable additional granularity for the filtering of UE capabilities.

Proposal 2: Regarding dynamic capability updates, RAN2 should first discuss whether there are new scenarios in 6G that require enhancements to the current 5G UE capability update mechanism.

Proposal 3: For the 6G UE capability reporting, the UE should be able to report capabilities using both explicit and implicit methods. RAN2 to study how to optimize the RACS mechanism in 6G to improve its feasibility.

UE Capability Signaling Framework

Observation 2: The intention of the FeatureSetCombination was to implement RF and BPC decoupling. However, the reuse rate of FeatureSetCombinations is low with the current structure, and it does not support reporting capabilities for a group of BCs.

Observation 3: A FeatureSet inherently contains both downlink and uplink components, which undermines the potential gains of DL and UL decoupling.

Observation 4: The dual BC reporting branch structure (for normal BC and UL Tx switching CA, respectively) in the 5G UE capability framework leads to signaling redundancy and increased complexity, as common capabilities often need to be reported repeatedly.

Observation 5: With 5G structure, although some UEs support a coarser granularity in practice for certain capability parameters, they are still required to report them at the defined finer granularity, resulting in a high repetition rate.

Observation 7: Because of some problematic UEs, the network has to adopt conservative settings or disable the new features entirely for all of the UEs to prevent widespread issues, which can significantly hinder new feature commercialization.

Proposal 4: Define different device types, with mandatory features specified for each device type per release version.

Proposal 5:  RAN2 to study more flexible structure for RF/baseband decoupling and DL/UL decoupling.

Proposal 6: For the 6G UE Capability structure, RAN2 to study how to avoid repeated reporting problem in the 5G.

Proposal 7: RAN2 should discuss mechanisms to enable network identification of UE/chipset information in 6G.

Reference

[1] RP-251881  New SID: Study on 6G Radio  3GPP RAN