3GPP TSG-RAN2#131Bis														R2-2507609

Prague, Czech Republic, Oct. 13rd-17th

# Title:	Reply LS on compatibility issue for PEI and emergency PDU session

# Response to:	R2-2506725_R3-255906

# Release:	Release 17

# Work Item:	NR_UE_pow_sav_enh-Core

Source:	RAN2

To:	RAN3, SA2

Cc:	CT1

Contact Person:

#### Name:	Yuan Gao

#### Tel. Number:

#### E-mail Address:	gao.yuan66@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1 Overall description

RAN2 thanks RAN3 for the LS on the compatibility issue for PEI and emergency PDU session, RAN2 think the two scenarios identified by RAN3 would cause paging loss and should be addressed.

From RAN2 perspective, the following views are provided.

- When UE does not monitor PEI, but network send PEI, there is no critical issue, e.g. PAGING will not be missed, only some radio resource and network power will be waste.

- When UE monitors PEI, but network does not send PEI, PAGING may be missed, which should be avoided.

- When UE monitors PEI, network sends PEI, but the PEI subgroup is different (e.g. UE monitors CN assigned PEI and gNB sends UE-ID based PEI), PAGING may be missed, which should be avoided.

Based on these views, RAN2 discusses the solutions in the LS and provides the following feedback:

- Solution 4 and solution 5 should be rolled out, as they intend to disable PEI monitoring completely when there is emergency PDU session and would require RAN2 spec change.

- Solution 1 and solution 6 should not be considered as they intend to disable PEI monitoring based on CN assigned subgrouping and may bring in-consistent behavior for CN assigned PEI monitoring and UE-ID based PEI monitoring.

- Solution 2 is feasible and preferred if there is no R17/R18 UE supporting PEI in the real network.

- Solution 3 with NAS UE capability is feasible and preferred if there is R17/R18 UE supporting PEI in the real network.

- Prefer to select solution 2 or solution 3 based on whether there is R17/R18 UE supporting PEI in the existing network or not.

# 2 Actions

To SA2 and RAN3

# RAN2 respectfully asks SA2 and RAN3 to take the above information into account in related works.

# 3	Dates of next TSG CT WG1 meetings

TSG RAN2 Meeting #132	17 – 21 November 2025	Dallas US

TSG RAN2 Meeting #133	9 – 13 February 2025	Stor-Göteborg, SE