3GPP TSG-RAN WG2 Meeting #131bis	R2-2507612

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.17

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues on support of TDD mode for NB-IoT-NTN

WID/SID:	IoT_NTN_TDD - Release 19

Document for:	Discussion and Decision

# 1	Introduction

At RAN #106 the work item on NB-IoT TDD NTN mode [1] was agreed with the following objective:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4]

In this contribution we discuss remaining issues to support TDD NTN mode, which may impact RAN2 specification TS36.300 and TS36.321.

# 2	Discussion

## 2.1	Remaining Issue to 36.300

According to the WID and RAN1 discussions the NB-IoT TDD NTN frame structure is as illustrated in Figure 1.

Figure 1 NB-IoT TDD NTN frame structure.

The frame structure is described in TS 36.211:

## 4.4 Frame structure type 1 for IoT NTN TDD Frame structure type 1 is applicable to IoT NTN TDD in band 249. Each radio frame is Tf=307200Ts=10 ms long and consists of 10 subframes of length 30720Ts=1 ms, numbered from 0 to 9. Subframe i in frame nf has an absolute subframe number nsfabs=10nf+i where nf is the system frame number. The frame structure for IoT NTN TDD, at the uplink time synchronization reference point defined in clause 16.1.2 of TS 36.213 [4] consists of D=8 consecutive downlink subframes, followed by 50 consecutive guard period subframes, followed by U=8 consecutive uplink subframes, followed by 24 consecutive guard period subframes in each 90 ms interval. - The UE shall not assume any signal or channel being transmitted in subframes other than downlink subframes 3, 4, 5, 6, 7, 8, 9, and 0 across two consecutive radio frames. - The UE shall not transmit any signal or channel on a subframe other than the 8 consecutive uplink subframes.

RAN1 has agreed that the UL and DL occasions are paired. If the UE has transmitted in UL1 the UE will have to monitor for a response in DL1. This is also reflected in the use of fixed guard periods between the UL and DL subframes.

Observation 1: From UE’s perspective, the time separation between an UL and a DL occasion in the NB-IoT TDD NTN frame structure is 24 subframes.

The UE can only transmit UL during UL occasions and receive DL during the DL occasions. RAN2 did not discuss UE AS operation during the two gaps between a UL and a DL occasion or vice-versa. For example, during the two gaps, it is clear the UE should not trigger the RACH, SR or BSR transmission, nor perform RLF determination. Therefore, for NTN TDD UE AS operation during the two gaps, we think one approach can be to base the UE behavior on what was agreed for GNSS measurement gaps for NB-IoT. The stage 2 TS 36.300 defines the following:

NOTE: The AS operations (e.g. RLM related timers, dataInactivityTimer, CHO execution, neighbour cell measurement, RACH, SR, and BSR) are suspended when UE is performing GNSS acquisition and resumed when the GNSS acquisition is completed.

Differently from the NOTE, as agreed in RAN4, for the NB-IoT TDD NTN operation the neighbor cell measurement can still be performed outside the UL-DL pair of the serving cell. However the other UE AS operations such as RLM and RACH shall be suspended.

Proposal 1: RAN2 to capture UE AS operation during the gaps between UL and DL occasions in NB-IoT TDD NTN mode, by reusing the AS operation defined for GNSS measurement gap, with the exception of the neighbor cell measurements which can be performed outside the serving cell’s UL-DL pair.

Proposal 2: If P1 can be agreed, adopt the TP in Annex 1 for TS 36.300.

## 2.2	Remaining issue to 36.321

As indicated in Observation 1, the time gap length between an UL and a DL occasion is 24 subframes. However, the RTT is in the range 4-12.88 ms for a regenerative LEO600 deployment and 8-25.76 ms for a transparent LEO600 deployment depending on the UE’s elevation angle to the satellite. This means that the UL-DL occasion gap in some scenarios is significantly longer than the RTT and in other scenarios the RTT and the gap is quite comparable.

In current MAC specification, the RAR window and Contention Resolution timer start is based on the UE-eNB RTT. This may cause the issue that the start of the timer is not aligned with the UL-DL occasion gap significantly.  As an example, after Msg1 transmission, the RAR window starts based on X subframes + UE-eNB RTT. Starting the RAR window based on X (of table 5.1.4-1) and the UE-eNB RTT may result in most of the window occurring during the UL-DL occasions gap (due to the gap length=24 ms is quite larger than the UE-eNB RTT). This is problematic from UE energy consumption point of view and also limits the eNB scheduling flexibility, because it may reduce the overlap between the RAR window and the DL subframes in the DL occasion.

Observation 2: The RAR window and CR timer may occur primarily during the UL-DL occasion gap if the start of the window is based on UE-eNB RTT.

Based on this issue we propose that RAN2 discuss how to better align the UE’s PDCCH monitoring, based on the RAR window and Contention Resolution timer, with the DL occasions. We consider a feasible solution is to ensure the RAR window and Contention Resolution timer does not start before the next DL occasion.

This can be achieved by implementing the TP provided in Annex 2 (with the modification in red). Using this update, the RAR window and Contention Resolution timer will either start at the first DL subframe of the next DL occasion or start later if the UE-eNB RTT is long.

Proposal 3: The start of the RAR window and Contention Resolution timer shall be aligned with the start of the DL occasions, when the UE-eNB RTT is shorter than the UL-DL gap.

Proposal 4: If Proposal 3 is agreeable, RAN2 to adopt the TP implementation to TS36.321 in Annex 2.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: From UE’s perspective, the time separation between an UL and a DL occasion in the NB-IoT TDD NTN frame structure is 24 subframes.

Proposal 1: RAN2 to capture UE AS operation during the gaps between UL and DL occasions in NB-IoT TDD NTN mode, by reusing the AS operation defined for GNSS measurement gap, with the exception of the neighbor cell measurements which can be performed outside the serving cell’s UL-DL pair.

Proposal 2: If P1 can be agreed, adopt the TP in Annex 1 for TS 36.300.

Observation 2: The RAR window and CR timer may occur primarily during the UL-DL occasion gap if the start of the window is based on UE-eNB RTT.

Proposal 3: The start of the RAR window and Contention Resolution timer shall be aligned with the start of the DL occasions, when the UE-eNB RTT is shorter than the UL-DL gap.

Proposal 4: If Proposal 3 is agreeable, RAN2 to adopt the TP implementation to TS36.321 in Annex 2.

# Reference

[1] RP-243293, Iridum, “Revised WID on introduction of IoT-NTN TDD mode”, December 2024

# Annex 1: TP to 36.300

## 23.21.1 General … NOTE: For IoT NTN TDD, the AS operations (e.g. RLM related timers, dataInactivityTimer, CHO execution, RACH, SR, and BSR) are suspended when UE is served in a IoT NTN TDD cell and the current subframe is not one of the 8 consecutive downlink subframes or 8 consecutive uplink subframes.

# Annex 2: TP to 36.321

### 5.1.4 Random Access Response reception … If the UE is an NB-IoT UE: - if the random accesspreamble was transmitted in a non-terrestrial network: - if the non-terrestrial network operates in IoT-NTN TDD mode - If time distance from the last preamble repetition to the next DL subframe is larger than X subframes plus UE-eNB RTT, where value X is determined from Table 5.1.4-1 based on the used preamble format and the number of NPRACH repetitions; - RA Response window starts at the subframe that contains the end of the last preamble repetition plus the length from the last preamble repetition to the next DL subframe and has length ra-ResponseWindowSize for the corresponding enhanced coverage level; - else - RA Response window starts at the subframe that contains the end of the last preamble repetition plus X subframes plus UE-eNB RTT and has length ra-ResponseWindowSize for the corresponding enhanced coverage level; - else - RA Response window starts at the subframe that contains the end of the last preamble repetition plus X subframes plus UE-eNB RTT and has length ra-ResponseWindowSize for the corresponding enhanced coverage level, where value X is determined from Table 5.1.4-1 based on the used preamble format and the number of NPRACH repetitions; …