3GPP TSG-RAN2 Meeting #131bis	R2-2507627

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	8.4.3 MAC issues

Source: 	Ericsson

Title:  	LP-WUS MAC issue (Eri-001 and Proposal 1 in #213)

Document for:	Discussion and Decision

# Introduction

In this contribution LP-WUS collisions with Option 1-2 are discussed, see Eri-001 and Proposal 1 in email discussion #213.

# Discussion

## Collisions with DCP

For DCP in 38.300 it says that there can be multiple monitoring occasions before the OnDuration:

A UE can only be configured to monitor DCP when connected mode DRX is configured, and at occasion(s) at a configured offset before the on-duration. If short DRX cycle is configured, DCP is not applicable when short DRX cycle is used. More than one monitoring occasion can be configured before the on-duration. The UE does not monitor DCP on occasions occurring during active-time, measurement gaps, BWP switching, or when it monitors response for a CFRA preamble transmission for beam failure recovery (see clause 9.2.6), in which case it monitors the PDCCH during the next on-duration. If no DCP is configured in the active BWP, UE follows normal DRX operation.

DCP-Config-r16 ::=                  SEQUENCE {

ps-RNTI-r16                         RNTI-Value,

ps-Offset-r16                       INTEGER (1..120),

sizeDCI-2-6-r16                     INTEGER (1..maxDCI-2-6-Size-r16),

ps-PositionDCI-2-6-r16              INTEGER (0..maxDCI-2-6-Size-1-r16),

ps-WakeUp-r16                       ENUMERATED {true}            OPTIONAL,   -- Need S

ps-TransmitPeriodicL1-RSRP-r16      ENUMERATED {true}            OPTIONAL,   -- Need S

ps-TransmitOtherPeriodicCSI-r16     ENUMERATED {true}            OPTIONAL    -- Need S

}

It is not clear whether the UE needs to miss all the monitoring occasions before the OnDuration before it is required to monitor the next OnDuration:

RAN2 to discuss whether a clarification is needed for collisions with DCP.

## Collisions with LP-WUS

For collisions with LP-WUS option 1-1 RAN2 agreed that a similar rule should apply as for DCP, i.e. if the UE misses the LP-WUS before the OnDuration, then the UE has to monitor PDCCH during the next OnDuration:

Confirm the following RAN2#129bis working assumption for Option 1-1:

  - In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected).

For Option 1-2, UE does not start the lpwus-PDCCH-MonitoringTimer in collision cases, i.e. when the UE is not able to monitor the LP-WUS occasion(s). Can discuss if critical issue identified with this mechanism.

For collisions with option 1-2 RAN2 agreed that the UE is not required to start the lpwus-PDCCH-MonitoringTimer when the UE misses an LP-WUS occasion. Because with option 1-2 there can be frequent LP-WUS occasions and there is minor impact when the UE misses one of the LP-WUS occasions. This is correct when a very short LP-WUS periodicity is configured. The value range for the LP-WUS periodicity is depicted below:

- sl1, NULL,

- sl2, INTEGER {0..1},

- sl4, INTEGER {0..3},

- sl5, INTEGER {0..4},

- sl8 INTEGER {0..7}

- sl10, INTEGER {0..9}

- sl16, INTEGER {0..15}

- sl20, INTEGER {0..19}

- sl40, INTEGER {0..39}

- sl80, INTEGER {0..79}

The periodicity is in slots and dependent on the SCS. The steps are also a bit odd, e.g. 2, 4, 5, 8 slots. For LP-WUS 15, 30 and 120 kHz SCS is supported, which means that the value range is:

- {1, 2, …, 80} msec (15 kHz)

- {0.5, 1, …, 40} msec (30 kHz)

- {0.125, 0.25, …, 10} msec (120 kHz)

Metric and SCS dependency

It is better to have the LP-WUS periodicity in time and independent from the SCS, similar as the C-DRX parameters. For reference the short DRX cycle range is depicted below:

shortDRX                            SEQUENCE {

drx-ShortCycle                      ENUMERATED  {

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

For LP-WUS the larger values are perhaps not needed:

RAN2 to discuss whether the LP-WUS periodicity in connected mode should be in time and independent from the SCS for example {1, 2, 4, 6, 8, 10, 12, 16, 20, 24, 32, 40, 48, 64, 80, 160}.

For discussion sake it is assumed that the LP-WUS periodicity is {1, 2, …, 80, 160} msec.

In case the LP-WUS periodicity with option 1-2 is 160 ms, then this is comparable from a latency perspective with a LongDRX cycle of 160 ms with option 1-1. Thus when an LP-WUS periodicity of 160 ms is configured with option 1-2, then the UE should start the new timer when it misses an LP-WUS occasion.

When an LP-WUS periodicity of 160 ms is configured with option 1-2 then the UE should start the lpwus-PDCCH-MonitoringTimer.

But with a short LP-WUS periodicity the UE should be allowed to miss a few consecutive LP-WUS monitoring occasions. Thus we propose that the gNB can configure the number of consecutive LP-WUS monitoring occasions the UE is allowed to miss before it needs to start lpwus-PDCCH-MonitoringTimer:

The NW can configure the number of consecutive LP-WUS monitoring occasions the UE is allowed to miss before it needs to start the lpwus-PDCCH-MonitoringTimer.

# Summary

RAN2 is kindly asked to discuss collisions with LP-WUS and DCP:

Observation 1	When an LP-WUS periodicity of 160 ms is configured with option 1-2 then the UE should start the lpwus-PDCCH-MonitoringTimer.

Proposal 1	RAN2 to discuss whether a clarification is needed for collisions with DCP.

Proposal 2	RAN2 to discuss whether the LP-WUS periodicity in connected mode should be in time and independent from the SCS for example {1, 2, 4, 6, 8, 10, 12, 16, 20, 24, 32, 40, 48, 64, 80, 160}.

Proposal 3	The NW can configure the number of consecutive LP-WUS monitoring occasions the UE is allowed to miss before it needs to start the lpwus-PDCCH-MonitoringTimer.