3GPP TSG-RAN WG2 Meeting #131-bis	R2-2507632

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.7.3

Source:	Samsung

Title:	Outstanding LCP issues and related TPs

Document for:	Discussion & Decision

# Introduction

The basics of MAC design for R19 XR have been finalized, with many key issues now settled and implemented in the running CR (R2-2506619) agreed at RAN#109 (September 2025). On top of that, the MAC XR rapporteur has made a submission to the present meeting proposing further changes (R2-2506810). We do however believe that a couple of issues should be further discussed, and we outline them in this submission. We provide a TP for each of the issues, assuming that changes proposed in R2-2506810 are adopted on top of the ones already agreed in R2-2506619.

# Clarifying priority applied in HARQ Process ID selection / uplink grant prioritization

With regards to issue D.2 identified in the List of open issues, we do agree there is a possible issue here, and that interpretation could be made based on the current wording that the additionalPriority is always used in the HARQ Process ID selection / uplink grant prioritization, even though it appears (based on responses received during the discussion) that RAN2 recognize this was not the intended behaviour. We believe that a simple fix in our Proposal 1 below removes the ambiguity, while making minimal change to existing text.

Proposal 1. [Issue D.2] RAN2 is kindly asked to agree the following TP:

For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation selects an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize the HARQ Process ID with the highest priority, where the priority of HARQ process is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. In this selection, the priority of a logical channel configured with priorityAdjustmentThreshold shall be the highest priority that can be applied or has been applied for it in the LCP procedure for the MAC PDU (depending on whether the conditions specified in clause 5.4.3.1.3 are met). If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection among initial transmission and retransmission with equal priority, the UE shall prioritize retransmissions before initial transmissions. The priority of a HARQ Process for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than the priority of a HARQ Process for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU. If the MAC entity is not configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize retransmissions before initial transmissions. The UE shall toggle the NDI in the CG-UCI for new transmissions and not toggle the NDI in the CG-UCI in retransmissions.

<< Text omitted>>

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. In this determination, the priority of a logical channel configured with priorityAdjustmentThreshold shall be the highest priority that can be applied or has been applied for it in the LCP procedure for the MAC PDU (depending on whether the conditions specified in clause 5.4.3.1.3 are met). The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.

# Adding uplink grant prioritization as use-case for application of additionalPriority

As a result of R19 XR work, the MAC spec now states that “[priority] is the default priority used for a logical channel in the LCP procedure and uplink grant priority determination, unless specified otherwise”. This was added since now we can have two priority values configured for a LCH, and it was felt we should now make it clear which is the default one, and that this was the right place for it (= the definition of legacy priority).

We additionally agreed that additionalPriority – if configured and ‘active’ (i.e. threshold condition met) – will also be used for UL grant priority determination (see Section 2 above), by overriding the legacy priority (“highest priority that can be applied or has been applied for it in the LCP procedure for the MAC PDU”). Therefore we now have a default priority (= legacy priority) for LCP procedure and UL grant priority determination (unless specified otherwise). Our Proposal 2 introduces this into the MAC spec.

Proposal 2. RAN2 is kindly asked to agree the following TP:

#### 5.4.3.1	Logical Channel Prioritization

##### 5.4.3.1.1	General

The Logical Channel Prioritization (LCP) procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level. It is the default priority used for a logical channel in the LCP procedure and uplink grant priority determination, unless specified otherwise;

-	additionalPriority which is applied instead of priority for a logical channel in the LCP procedure and uplink grant priority determination when the conditions specified in clause 5.4.3.1.3 are met;

-	priorityAdjustmentThreshold which is used to determine whether additionalPriority or priority is applied in the LCP procedure;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

# Miscellaneous: whether to use priorityAdjustmentThreshold or enhancedLCP

As priorityAdjustmentThreshold is not optional within enhancedLCP, we can use configuration of either as the starting condition for testing whether a PDCP SDU is priority adjustable, or not. The current text opted for priorityAdjustmentThreshold. While there is no rule dictating this, we believe it could be replaced with enhancedLCP – this improves readability by immediately indicating that we are determining whether to apply the adjusted (enhanced) LCP procedure, and it also uses the higher-level ASN.1 parameter of the two. This is a purely editorial proposal, with no functional impact in our understanding.

Proposal 3. RAN2 is kindly asked to agree the following TP:

##### 5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover.

The MAC entity shall, when a new transmission is performed:

1>	if a logical channel is configured with enhancedLCP and its associated PDCP entity has an  SDU available for this transmission:

2> if the PDCP entity associated with this logical channel is configured with pdu-SetDiscard, and the PDU Set remaining time of the PDCP SDU (as defined in TS 38.323 [4]), evaluated at the time of the first symbol of this transmission, is less than the priorityAdjustmentThreshold; or

2>	if the PDCP entity associated with this logical channel is not configured with pdu-SetDiscard, and the remaining time of discardTimer of the PDCP SDU (as defined in TS 38.323 [4]), evaluated at the time of the first symbol of this transmission, is less than the priorityAdjustmentThreshold:

3> consider this PDCP SDU being priority adjustable.