3GPP TSG-RAN2#131bis	R2-2507643

Prague, Czech Republic, 13th – 17th Oct. 2025

Agenda Item:	8.9.2

Source:	Ericsson

Title:	RRC IoT NTN issues

Document for:	Discussion, Decision

# 1	Introduction

Here we discuss some outstanding issues for IoT NTN phase 3.

# 2	Discussion

Regarding this NOTE in 5.3.3.3x:

NOTE:	When receiving the CB-Msg3-EDT failure indication from lower layers, the RRC procedure is re-initiated. Which procedure (e.g. EDT [RIL]: N013, IoTNTN, random access procedure, CB-Msg3-EDT) is initiated[RIL]: V212, IoTNTN.

First sentence does not read well, further it is not for sure RRC needs to initiate something special for this case. For example, T300 may expire or may already have expired when the indication is received in RRC. Therefore, we propose:

In 5.3.3.3x, modify “When receiving the CB-Msg3-EDT failure indication from lower layers, the RRC procedure is re-initiated” to “When receiving the CB-Msg3-EDT failure indication from lower layers, an RRC procedure may be initiated”

IoT NTN is only supported for EPC, not for 5GC.

In 5.3.3.1b at the initiation of EDT using the RA procedure there are special conditions for EPC respectively for 5GC. However, for CB-Msg3-EDT there is no mentioning of the core – this may give the impression that 5GC is supported. Therefore, we propose:

In 5.3.3.1b, modify “In NTN, a BL UE, UE in CE mode A or NB-IoT UE can initiate EDT using the CB-Msg3-EDT procedure when all of the following conditions are fulfilled:” to “In NTN connected to EPC, a BL UE, UE in CE mode A or NB-IoT UE can initiate EDT using the CB-Msg3-EDT procedure when all of the following conditions are fulfilled:”

# 3	Conclusion

In this paper we have made these observations:

No table of figures entries found.

We propose:

Proposal 1	In 5.3.3.3x, modify “When receiving the CB-Msg3-EDT failure indication from lower layers, the RRC procedure is re-initiated” to “When receiving the CB-Msg3-EDT failure indication from lower layers, an RRC procedure may be initiated”

Proposal 2	In 5.3.3.1b, modify “In NTN, a BL UE, UE in CE mode A or NB-IoT UE can initiate EDT using the CB-Msg3-EDT procedure when all of the following conditions are fulfilled:” to “In NTN connected to EPC, a BL UE, UE in CE mode A or NB-IoT UE can initiate EDT using the CB-Msg3-EDT procedure when all of the following conditions are fulfilled:”