3GPP TSG-RAN WG2 Meeting #131-bis	R2-2507644

Prague, CZ, Oct 13-17, 2025

Agenda Item:	10.2

Source:	THALES, Airbus, Echostar, Novamint, Fraunhofer IIS

Title:	6G Radio Access Technology general aspects for NTN

Document for:	Discussion

# 1	Introduction

While in the context of 5G, the support of NTN was added once the basic design principles of the New Radio (NR) access were already defined, it is strongly recommended to define and specify the basic enablers to support NTN as part of the 6G radio access definition right from the initial 6G specification in release 21. This approach will allow for:

- Achieving ubiquitous connectivity and availability of services in remote and sparsely populated areas.

- Improving network resiliency, especially in situations where disasters disrupt terrestrial networks or damage undersea communication cables.

By integrating the fundamental NTN enablers in the initial stages of 6G development, users will experience several benefits, including seamless connectivity regardless of geographic location. This will empower individuals, communities, and industries in the most isolated regions, ensuring that they are not left behind in the digital revolution. Moreover, enhanced network resiliency will provide robust and reliable communication channels during emergencies, significantly reducing downtime and maintaining critical infrastructure and services. This harmonized design approach for both TN and NTN components will pave the way for a resilient, flexible, and inclusive communication network for future generations.

The 6G Radio Study Item approved at the RAN#108 meeting and updated at RAN#109 [1] states The main objectives related to the radio protocol architecture and the target to have a harmonized 6G radio design for TN and NTN:

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] a) Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE ca i) Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. […] j) Aim at a harmonized 6G Radio design for TN and NTN, including their integration. k) System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.pabilities reporting. […] (3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] a) User Plane architecture and protocol design [RAN2] b) Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] c) Access stratum security aspects, in alignment with requirements from SA3 [RAN2] d) Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] e) Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] […] (6) Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] a) Overall RAN architecture aspects [RAN3, RAN2] […] c) RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

The intent is therefore to achieve a 6G Radio design harmonized for the support of both TN and NTN deployment scenarios as well as deployment scenarios integrating NTN and TN components from Rel-21. Some aspects was discussed in [4]. In this contribution, we share our views on the support of NTN in the 6GR.

# 2	Discussion

## 2.1 	Design implications of native NTN support in 6GR

An harmonized radio interface that enables seamless integration between terrestrial and non-terrestrial in 6G systems is has been regarded as a fundamental design principle for the new 3GPP RAN1-led 6G Radio study [1].

By natively incorporating non-terrestrial networks (NTN), 6G can deliver unprecedented levels of performance, resilience, and global coverage, fulfilling the evolving demands of the future mobile ecosystem. Achieving comprehensive integration requires a unified and adaptable radio interface that seamlessly supports both terrestrial and satellite connections. This harmonization should address:

- Compatibility with diverse satellite orbits, such as Very LEO, LEO, MEO, and GEO.

- Robustness against challenges like Doppler shifts, significant and variable round-trip times (RTT), delay spreads, and low signal-to-noise ratios (SNRs).

- Mobility management and handover mechanisms to account for the high relative velocities and extended coverage areas of non-terrestrial networks.

Consider the support of a harmonized 6G radio interface design that supports further integration of terrestrial and non-terrestrial networks

## 2.2	6G Radio Access Network architecture for a native support of NTN

### 2.2.1	NTN Payload types and scenarios

Figure 1 : Proposal for a 6G NTN deployment scenario

Below are some considerations for practical NTN scenarios for 6G. The different scenarios are illustrated on the figure above. The following orbits should be considered:

- VLEO (very low Earth orbit) based scenario at minimum altitude of 300 km: Satellites operating at this altitude benefit from reduced latency due to their proximity to the surface of Earth, which can enhance communication and observation capabilities. At this altitude, the Round Trip Delay becomes closer to the one of terrestrial extended cells. Further, this closer proximity to Earth can offer more accurate ground metrics, and potentially lower cost for certain types of satellite operations. However, deployment at a VLEO orbit would require a very high number of satellites to generate overlapping footprint and hence service continuity in the targeted coverage. This is now made possible with the ever decreased launch cost. Typically, a maximum satellite beam footprint diameter of [250] km or lower should be supported.

- LEO (Low Earth Orbit) based scenarios (respectively 600 and 1200 km altitude):While these scenarios enable lower latency and higher data throughput in the context of 5G, they also introduce challenges such as frequent handovers and increased Doppler effects due to rapid satellite movement.

- MEO (Medium Earth Orbit) scenarios (10000 km altitude):  offer a balance between coverage area and latency, providing broader coverage than LEO and lower latency than GEO, but they still face moderate signal delays and more complex constellation management.

- GSO (geostationary, including GEO) based scenarios (35786 km altitude):  ensure continuous and stable coverage over large regions with simpler ground equipment, but their high altitude results in significant signal latency and reduced performance for delay-sensitive applications.

Additionally, the 6G radio access should also support multi orbit space segment combining at least a GSO with an NGSO based scenario, illustrated in Figure 1.

### 2.2.2	Types of cells supported on NTN

The different types of cells supported in Non-Terrestrial Networks (NTN) are:

1. Quasi-Earth Fixed Cells: Quasi-Earth fixed cells are radio coverage areas on the ground that are almost, but not completely, stationary relative to the Earth’s surface. These cells are typically associated with non-geostationary satellites (such as those in LEO and MEO) whose orbits cause their coverage footprints to move slowly or only slightly shift on the ground.

2. Earth Moving Cells: Earth moving cells are coverage zones that move continuously across the Earth’s surface. These are commonly linked to satellites in orbits where the coverage area does not stay fixed to one location but sweeps across different regions as the satellite passes overhead.

3. Earth Fixed Cells: Earth fixed cells are coverage zones that remain fixed relative to a specific geographic location on the Earth’s surface. This is mainly possible with geostationary satellites, which orbit at the same rotational speed as the Earth and thus "hover" over one position. The coverage area of an Earth fixed cell does not change over time.

Table 1 Types of cells

| Cell Type | Coverage Movement | Typical Satellite Orbit |
| --- | --- | --- |
| Quasi-Earth fixed | Slightly moves | LEO/MEO (with tracking) |
| Earth moving | Continuously changes place | LEO/MEO |
| Earth fixed | Stays over same location | GEO |

### 2.2.3	NTN Scenarios

The table below provides the list of reference practical scenarios (single orbit) to be supported by the 6G radio access with the related parameters.

Table 2 NTN reference orbit scenarios for 6G

| NTN Scenarios | Very LEO | LEO-600 | LEO-1200 | MEO | GSO |
| --- | --- | --- | --- | --- | --- |
| Orbit type | circular around the earth | circular around the earth | circular around the earth | circular around the earth | station keeping around a nominally fixed position in terms of elevation/azimuth with respect to a given earth point (NOTE 6) |
| Altitude | 300 km (NOTE 1) | 600 km | 12000 km | 2000 to ~35000 km | 35,786 km |
| Earth-fixed beams | Yes (Satellite generated steerable beams, NOTE 2) |  |  |  |  |
| Max beam footprint size (edge to edge) regardless of the elevation angle (NOTE 3) | [<250] km | [<250] km | [<250] km | [<250] km | [<250] km |
| Min Elevation angle for both sat-gateway and User Equipment | 10° for service link and 10° for feeder link |  |  |  |  |
| Max distance between satellite and User Equipment at min elevation angle | 1160 km | 1 932 km | 3 131 km | TBD km | 40 581 km |
| Max Round Trip Delay (propagation delay only NOTE 4) | 15.48 ms | 25.77 ms | 41.77 ms | 180 ms | 541.46 ms |
| Max Doppler shift (earth fixed user equipment) (NOTE 5) | 24.23ppm | 22.68ppm | 20.04ppm | TBD ppm | 0.93ppm |
| Max Doppler shift variation (earth fixed user equipment) (NOTE 4) | TBD ppm/s | 0.27 ppm/s | 0.13 ppm/s | TBD ppm/s | 0.000 045 ppm/s |
| NOTE 1: VLEO is characterized by altitudes ranging from approximately 300 to 600 kilometers above Earth. However, the lower the altitude, the higher the number of satellites is required to ensure service continuity and the higher the atmospheric drag which would need constant propulsion and re-boosting and hence reduced lifetime. NOTE 2: Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite. NOTE 3: This beam size refers to the Nadir pointing of the satellite. NOTE 4: Assuming transparent payload with service and feeder links and return NOTE 5: Max Doppler shift and Max Doppler shift variation in the absence of any device pre-compensation of satellite Doppler shift on the service link. NOTE 6: In case of GSO, the inclination may be non-zero. This means the satellite’s ground track may trace a figure-eight pattern (analemma) as seen from Earth. |  |  |  |  |  |

Study the 6G radio interface/access to support multi-orbit architecture and all the practical NTN deployment scenarios, orbits, and related service link characteristics in the table above.

## 2.2.4	NTN Payload types

The following payload types along with 6G functional mapping shall be supported [2]:

Table 3 6G NTN payload types with respect to 6G system architecture

| Payload type | 3GPP function on board | Feeder link type (NOTE 1) | Other capabilities |
| --- | --- | --- | --- |
| Transparent | Equivalent to RF repeater | 6G radio interface | - |
| Regenerative with RU processed payload | Radio Unit (RU) | Fronthauling of RU-Base station interface | - |
| Regenerative with full base station processed payload | BS | Backhauling of NG interface | Support of Inter Satellite Links |
| Regenerative with DU processed payload | Radio Unit and a Distributed unit | Base station internal interface (e.g. F1) | Inter Satellite transport links can be used |
| Regenerative embarking a full base station (BS) and part of Core Network | BS + UPF | Interface between the UPF/part of Core Network and the on-ground network |  |
| NOTE1: It refers to the radio link between satellite and a gateway. |  |  |  |

The 6G radio interface/access shall be defined to support the different NTN deployment architectures, e.g. transparent and multiple regenerative payloads with different 6G system functional splits between on board/on ground NTN nodes

## 2.3		Key capabilities for a harmonized 6G Radio access design for TN and NTN

### 2.3.1	6GR Device types

6G access network shall be able to serve a wide range of devices (User Equipment - UE). We shall distinguish the following ones:

- Smartphone and IoT devices with omnidirectional antenna which should be addressed with satellite networks operating in satellite service allocated frequency bands up to ~7 GHz.

- VSAT (Vehicle or building mounted) devices with directional antenna operating in all satellite service allocated frequency bands. Vehicle encompass cars, buses, railway trains, vessels and aeronautic platforms.

The 6G access network shall be defined to be able to serve devices with the following performances:

Table 4 NTN capable 6G UE types

| UE types | Min antenna gain | Min Transmit Power | Max Noise figure | UE Ground speed | Coverage |
| --- | --- | --- | --- | --- | --- |
| IoT and smartphones | -5 dB | 20 dBm | 7 dB | 300 km/h (on board high speed trains) | Outdoor, Light indoor (NOTE1) |
| Vehicle, vessel, aircraft, and building mounted | Equivalent to the one of 20 cm aperture antenna (FR2-NTN), 0.3 dBic (FR1-NTN) [3], [4] | Equivalent to 1 Watt (FR2-NTN) | [4] dB (FR2-NTN), 7 dB (FR1-NTN) | 1500 km/h (aircraft) | Outdoor only |
| NOTE1: adverse propagation conditions (e.g. Light indoor environment) requires to operate with a greater Maximum Coupling Loss than NR. This may lead to performance degradation (e.g. QoS, service rate, throughput, etc..). |  |  |  |  |  |

### 2.3.2	Duplexing mode

In all cases, the service link (which refers to the radio link between satellite and User Equipment) will provide the 3GPP defined NTN capable 6G radio interface.

It operates in satellite service allocated frequency bands which can be paired (Downlink and Uplink in separate bands) or unpaired (Downlink and Uplink in the same band).

While paired bands can apply to all orbits, unpaired band is restricted to the Low Earth Orbits (VLEO, LEO).

6G should support both duplexing methods — FDD (Frequency Division Duplexing), and TDD (Time Division Duplexing) as well as Half Duplex Frequency Division Duplexing (HD-FDD) mode.

The target Payload types are:

- Full-duplex FDD Satellite Payload:

  - Full duplex networks should support a mix of full and half duplex UEs

- Half-duplex FDD Satellite Payload: For both base station and terminal, a duplexer is not needed.

The targeted UE types are:

- Full and Half Duplex FDD Mode

The UE will have to support the frequency band and duplex mode of the satellite network.

Table 5 Duplex mode supported by 6GR NTN

| NTN capable UE type | Paired frequency band | Unpaired frequency band |
| --- | --- | --- |
| IoT and smartphones | FDD or HD-FDD | TDD |
| Vehicle and building mounted | FDD or HD-FDD | TDD |

The 6G Radio design shall be able to support NTN with all the duplex modes, i.e. FDD, TDD and HD-FDD at the UE and the network level

### 2.3.3 Seamless TN/NTN mobility in connected mode

Currently, 3GPP standards focus mainly on static use cases, including NTN/TN switching in idle mode. the potential for more dynamic use cases, such as service continuity during mobility, has not been sufficiently addressed.

The purpose is to ensure a global service continuity in both idle and connected modes. NTN is expected to provide coverage in overlapping areas with TN but also in areas beyond TN coverage.

While mobility from TN to NTN is mostly triggered by the unavailability of the TN service, the mobility from NTN to TN may be determined by other criteria.

In both cases, the UE shall be able to camp on a TN cell while being able to monitor NTN cells and vice versa.

### 2.3.4	GNSS Independent NTN Operation

Given that the GNSS service may be unavailable or degraded, NTN (GSO and NGSO, including VLEO) should be designed to operate independently of GNSS. For instance:

- Uplink pre-compensation (e.g. timing and frequency)

- Location-based mobility

- Location-based RRM measurements

- Geofencing

6G NTN shall be able to provide Positioning, Navigation and Timing (PNT) techniques, and exploit the integration of TN and NTN.

## 2.4	Services in 6G

### 2.4.1	Positioning, Navigation and Timing

The provision of robust, high-accuracy positioning is driven by regulatory requirements, demand for ultra-precise location services, need for GNSS independent solutions, and the expansion of service areas.

NTN is a key enabler of ubiquity, resilience, and wide area coverage representing a significant opportunity to enhance not only global connectivity but also positioning services especially in remote, underserved, and maritime areas where traditional terrestrial networks are not feasible or cost-effective. Complementing TN with NTN in 6G is crucial not only for connectivity but also for positioning services to maximize synergies between the two.

### 2.4.2	Broadcast services

Satellite communication's extensive coverage enables the delivery of broadcast services over wide and often transnational areas, supporting applications such as public information dissemination, emergency alerts, and multimedia content delivery on a large scale. However, in Non-Terrestrial Networks (NTN), the radio cells of satellites frequently span across country borders, while broadcast services may be subject to national regulations that restrict content to specific territories. This unique challenge highlights the importance of implementing geo-fencing mechanisms within the 6G system to ensure broadcast transmissions are confined to legally permissible areas.

To comply with these requirements, the 6G Radio with satellite access shall be equipped with capabilities that allow precise definition and enforcement of geographical boundaries for broadcast services. This includes mechanisms for user equipment to dynamically acquire service configuration, resource allocation, and control information, only when located within the targeted geographical area. Such geo-fencing ensures that broadcast content is only received by users within permitted regions, supporting both regulatory compliance and efficient use of satellite resources.

### 2.4.3	Multicast Services

Satellite networks are equally well-suited to support multicast services, delivering the same content to multiple users who are grouped based on service requirements. Multicast capabilities are essential for group communications in contexts such as corporate communications, public safety, and content distribution networks. A technical challenge arises when multicast groups span multiple beams or satellites, especially in non-GSO constellations, where frequent inter-satellite handovers occur and user equipment may move across jurisdictional boundaries.

To address this, the 6G system must enable efficient grouping, management, and routing for multicast sessions, while also implementing geo-fencing to limit service delivery to authorized geographic areas. This requires dynamic session management, beam-level routing, and continuous synchronization with national regulatory boundaries. As a result, multicast streams can be delivered to all group members located within the defined area, regardless of their movement between cells or satellites.

### 2.4.4	Regulatory services

Public Warning Systems (PWS):

Contemporary Public Warning Systems (PWS) uses a multicast or broadcast services to provide instantaneous, region-specific alerts during emergencies, offering far greater effectiveness than legacy approaches. Core components include an intuitive alert management portal and secure gateway, empowering authorities to swiftly craft and disseminate messages tailored to affected populations. Through the use of robust cell broadcast mechanisms, these systems deliver high-priority notifications to millions of mobile devices simultaneously, resilient to network congestion and independent of user opt-in.

Non-Terrestrial Networks (NTN) play a key role by extending these PWS capabilities to underserved or remote areas, ensuring no region is left unprotected during critical events.

Lawful Intercept (LI):

Lawful Intercept (LI) is a crucial legal and technical framework enabling authorized agencies to access and monitor communications in the interest of national security, crime prevention, and public safety. Telecommunications infrastructure must be equipped with mechanisms to selectively capture, store, and transfer intercepted data to law enforcement in accordance with national and international regulatory standards, such as those stipulated by the European Council.

NTN enhances LI by enabling interception capabilities across hybrid terrestrial-satellite environments, guaranteeing lawful compliance and intelligence accessibility even in regions lacking traditional network coverage.

Emergency Calling:

Reliable access to emergency call services (such as 112 or 911) is a fundamental human safety need (i.e: eCall, which uses 112 in Eruope, is a mandatory feature for all new cars sold in Europe since 2018), requiring robust telecommunications infrastructure capable of handling distress traffic under any circumstance. Modern emergency call solutions support accurate geolocation, rapid call setup, and prioritization within the network to ensure that callers are connected to emergency responders without delay.

By integrating NTN communication in 6G, individuals in rural, maritime, or otherwise isolated zones gain dependable access to voice services ensuring emergency calls can be placed and received without geographic constraints, significantly improving the overall efficacy of public safety initiatives.

Emergency Messaging:

Emergency messaging systems are engineered to disseminate critical information to the public or specific groups rapidly, using channels such as SMS, cell broadcast, and application-based alerts. These systems are designed to function even in network congestion scenarios, ensuring delivery reliability and message visibility. They also enable message delivery tracking, subscriber location analysis, and real-time updates during dynamic situations.

The inclusion of NTN communication infrastructure strengthens emergency messaging coverage, guaranteeing the continuity of information distribution in disaster-hit or remote areas where terrestrial networks may fail or be temporarily unavailable, thus supporting situational awareness for both citizens and first responders.

Study the support of the following services for NTN with 6G Radio

- Positioning, Navigation and Timing (PNT) services.

- Broadcast services over an intended area.

- Multicast services to a group of user equipment distributed over an intended area.

- PWS (Public Warning System) services over an intended area.

## 2.5	Other key features for 6GR to support NTN natively

The following table presents key building blocks for a harmonized 6G Radio access for TN and NTN.:

Table 6 Other key features for 6GR to support NTN natively

| Capability/service | Justification | Requirements |
| --- | --- | --- |
| NTN GNSS independent | Given that the GNSS service may be unavailable or degraded, NTN (GSO and NGSO, including VLEO) should be designed to operate independently of GNSS. | The 6G radio Access shall be able to accommodate the extended and variable Doppler and latency associated to NTN deployment scenarios. |
| Extended coverage | Provide service under adverse propagation conditions (e.g. in light indoor/in vehicle environment) | The 6G radio Access shall be able to support a higher Maximum Coupling Loss at the expense of the service rate (i.e. same as NB-IoT’s MCL) |
| Massive messaging capability | Support a high number of terminals willing to transmit quasi simultaneously low bandwidth services (i.e. messaging) within a beam | The 6G Radio Access shall be designed to provide an efficient grant free access protocol |
| Enhanced network verified UE location service | The current (Rel-18 defined) network verified UE location scheme may not be able to determine reliably the UE location with an accuracy of few kms with large beam size | The 6G Radio Access shall provide means for the network to determine the location of a UE without GNSS service |
| Integrated Communication And Sensing | ICAS capabilities within 6G are important, as ICAS allows terrestrial and satellite systems to address important scenarios like continuous monitoring of atmospheric conditions and space debris. These functions are crucial for safeguarding space operations and supporting accurate weather assessment, particularly in regions where terrestrial stations are sparse or absent. | The 6G radio Access shall support Integrated Sensing and Communication (ICAS) functionalities in NTN to enable for example detection, monitoring, and data analysis for space debris, atmospheric conditions, and weather prediction. |

RAN2 to study in priority the following NTN capabilities and services for the 6GR as part of the Rel-20 :

- GNSS independent NTN operation

- Extended coverage

- Flexible duplex mode support at UE level

- Support of HD-FDD at Network side

- Massive messaging capability

- Positioning, Navigation and Timing

- Enhanced network verified UE location service

- Seamless TN/NTN mobility in connected mode

- 6G NTN coexistence with IoT-NTN and NR-NTN

- ICAS (Integrated Communication And Sensing)

- Broadcast , multicast services

# 3	Conclusion

Proposal 1    Consider the support of a unified 6G radio interface that enables seamless integration of   terrestrial and non-terrestrial networks

- Study the 6G radio interface/access to support multi-orbit architecture and all the practical NTN deployment scenarios, orbits, and related service link characteristics in the table above.

Proposal 3   The 6G radio interface/access shall be defined to support the different NTN deployment   architectures, e.g. transparent and multiple regenerative payloads with different 6G system functional splits between on board/on ground NTN nodes

- Proposal 4   The 6G Radio design shall be able to support NTN with all the duplex modes, i.e. FDD, TDD and HD-FDD at the UE and the network level.

- Proposal 5    Study the support of the following services for NTN with 6G Radio

    - Positioning, Navigation and Timing (PNT) services.

    - Broadcast services over an intended area.

    - Multicast services to a group of user equipment distributed over an intended area.

    - PWS (Public Warning System) services over an intended area.

- RAN2 to study in priority the following NTN capabilities and services for the 6GR as part of the Rel-20 :

- GNSS independent NTN operation

- Extended coverage

- Flexible duplex mode support at UE level

- Support of HD-FDD at Network side

- Massive messaging capability

- Positioning, Navigation and Timing

- Enhanced network verified UE location service

- Seamless TN/NTN mobility in connected mode

- 6G NTN coexistence with IoT-NTN and NR-NTN

- ICAS (Integrated Communication And Sensing)

- Broadcast , multicast services

# References

RP-252912, “Revised SID: Study on 6G Radio”, NTT Docomo, CMCC, AT&T, Vodafone, Beijing, CN, September 2025.

R3-257192,” 6G RAN overall architecture for NTN” THALES, Prague, CZ, October, 2025.

3GPP TR 38.914, “	Study on 6G Scenarios and requirements”, v0.1.1, Beijing, CN, September 2025.

6GWS-250040, “Vision and priorities for 6G RAT to support NTN”, Thales, Incheon, KR, March 2025.