3GPP TSG-RAN WG2 Meeting #131-bis	R2-2507645

Prague, CZ, Oct 13-17, 2025

Agenda Item:	10.3.1

Source:	THALES, Airbus, Echostar, Novamint, Fraunhofer IIS

Title:	6G User Plane design aspects for NTN

Document for:	Discussion

# 1	Introduction

While in the context of the 5G, the support of Non-Terrestrial Networks (NTN) was added in Rel-17 to the NR access protocols, the 6G aims to design harmonized radio interface, including protocols, between TN and NTN to take into account the characteristics of the two access systems. This integrated approach will allow to:

- To design an optimized 6G protocol stack for NTN for better performances (throughput, latency, computational cost)

- To harmonize TN and NTN components for ubiquitous connectivity

More details on our proposed 6G radio interface and architecture is provided in [5].

In this contribution, we share our views for a 6G NTN and TN harmonized user plane protocol architecture.

# 2	Discussion

## 2.1	User plane protocol design for NTN architectures

The user plane protocol stack is a critical part for the overall 6GR system performance. It is responsible for transmitting user data in both the uplink (from terminal to the core network) and the downlink (from core network to terminal) sides. The basic functions the access network protocols should provide are:

- Data recovery for reliable communications

- Quality of Service management

- Radio resource allocation between users according to the channel quality

- Segmentation and multiplexing for data transmission on the physical layer

- Ciphering and integrity protection

In 5G, the user plane design integrates these mechanisms between different protocols :

- SDAP is responsible for mapping QoS bearers to radio bearers according to their quality of service.

- PDCP performs IP header compression, ciphering and integrity protection, retransmissions and in-sequence delivery, and removal duplicates in case of mobility.

- RLC protocol is responsible for segmentation and retransmission handling.

- MAC handles multiplexing/demultiplexing, scheduling, and HARQ retransmissions

For each UE and for each data radio bearer (signalling radio bearer for control plane), an entity at each protocol layer is instantiated. For the access node (i.e. gNB in 5G), the number of instantiated entities can be quite huge with a cost in terms of memory and computation. For a satellite where the gNB is fully embarked, the satellite payload and the related gNB(s) can handle hundreds of cells (illustrated below with a LEO-600 view field with 1058 beams that can be generated), with terminals in each cell, inducing a large memory and computational footprint on the satellite payload.

Figure 1 : View field of a LEO-600 satellite with 1058 beams

The satellite constrained SWaP (Size, Weight and Power) has to be used at its best for the user service, particularly regarding the service availability and throughput. So, it is beneficial for NTN performances to reduce user plane protocol costs and increase flexibility to share network functions in the NTN system, both between satellite nodes and between the space and the ground segment.

The envisioned RAN architectures for NTN, transparent, regenerative, split, are discussed in [5]. For the user plane design, it implies for:

- The transparent architecture: to have a data link layer (i.e. 6G-MAC) to support long RTT

- The regenerative architecture: to optimize the user plane protocol memory and computational footprint on the payload for a scalable system. One way to do this may be to move the crypto operations to an end-to-end approach for the user plane, e.g. end-to-end ciphering from the gateway to the terminal. We can also remove redundant operations such as retransmission and buffering.

- The split architecture: to reduce to the maximum the interdependencies between layers. The functional split architecture and interfaces are to be discussed in RAN3. Reducing the number of inter-dependicies between layers is necessary.

Proposal 1	The user plane of the 6G radio access shall be designed to support various NTN architectures :

- Transparent payload: Capability to accommodate extended RTT (e.g. up to 650 ms) at the MAC layer

- Regenerative payload with full 6G RAN on board: Optimize the memory and computational footprints of the user plane protocols

- Regenerative payload with partial 6G RAN functions on board (6G RAN architecture split): Optimize the number of interactions between the user plane protocols

## 2.2	Retransmission mechanisms at user plane level in NTN

In the satellite systems, the link budget is in general calculated to ensure reliable communications at the physical layer (to 10-5 error rate) to avoid retransmission mechanisms due to long RTT. The remaining errors are handled either at the point of presence (PoP, e.g. the gateway) or at the application endpoint by transport protocol retransmission mechanisms (e.g. QUIC or TCP). This tradeoff may increase latency in case of losses on the physical layer but reduce the need for multiple retransmission mechanisms at the different protocol layers. With a low error rate at the physical layer by design and FEC mechanisms, the HARQ may be deactivated; that is beneficial to reduce memory and computational footprint at the MAC layer.

Proposal 2	The 6G access radio protocol configurations may allow dynamically activation/deactivation of retransmission mechanisms at user plane level depending on the type of services and operating point

# 3	Conclusion

Proposal 1	The user plane of the 6G radio access shall be designed to support various NTN architectures :

- Transparent payload: Capability to accommodate extended RTT (e.g. up to 650 ms) at the MAC layer

- Regenerative payload with full 6G RAN: Optimize the memory and computational footprints of the user plane protocols

- Regenerative payload with partial 6G RAN functions (6GRAN architecture split): Optimize the number of interactions between the user plane protocols

Proposal 2	The 6G access radio protocol configurations may allow dynamically activation/deactivation of retransmission mechanisms at user plane level depending on the type of services and operating point

# References

RP-252912, “Revised SID: Study on 6G Radio”, NTT Docomo, CMCC, AT&T, Vodafone, Beijing, CN, September 2025.

RP-250810, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT Docomo, Deutsche Telekom, Incheon, KR, March 2025.

3GPP TR 38.914, “	Study on 6G Scenarios and requirements”, v0.1.1, Beijing, CN, September 2025.

6GWS-250040, “Vision and priorities for 6G RAT to support NTN”, Thales, Incheon, KR, March 2025.

R2-2507644, “6G Radio Access Technology general aspects for NTN”, THALES, Airbus, Echostar, Novamint, Fraunhofer IIS, Prague, CZ, October 2025.