3GPP TSG-RAN WG2 Meeting #131-bis	R2-2507647

Prague, CZ, Oct 13-17, 2025

Agenda Item:	10.4

Source:	THALES, Airbus, Echostar, Novamint, Fraunhofer IIS

Title:	6G Mobility aspects for NTN

Document for:	Decision

# 1	Introduction

The 6G Radio Study Item was approved at the RAN#108 meeting and updated at RAN#109 [1]. The main objectives related to the 6G mobility are:

The detailed objectives of the study are: Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

In this document we share our views for 6G mobility aspects and challenges related to NTN. 6G radio general aspects are discussed in [2]. For each identified challenge, targeted proposals are provided to guide the development of more seamless, resilient, and flexible 6G NTN solutions, ensuring improved service continuity and user experience across diverse scenarios.

# 2	Discussion

## 2.1	General

For 5G Non-Terrestrial Networks (NTN), the 3GPP standards in Release 17, Release 18, Release 19 and beyond have incrementally introduced and enhanced mobility types to address NTN scenarios. Release 17 laid the foundation by adapting existing 5G NR mobility procedures to cope with NTN-specific challenges such as large cell sizes, rapidly changing coverage footprints, and longer propagation delays. This included support for basic intra and inter-satellite handovers and tracking area updates.

Release 18 further refined NTN mobility by introducing predictive and assisted handover mechanisms, efficient beam management, and mechanisms to handle frequent service interruptions due to satellite movement. Release 19 and future work continue focusing on advanced seamless mobility, optimized handovers between terrestrial and non-terrestrial segments, support for higher mobility, as well as solutions for even lower latency and service continuity in dynamic orbital environments.

In the context of 6G RAN, robust support for both idle and connected mode mobility is required to enable zero interruption and zero packet loss

Study the support of 6G mobility with :

- Zero interruption time

- Zero Packet loss

## 2.2	Intra-RAT Mobility

### 2.2.1		6G NTN-TN service continuity

Currently, 3GPP standards focus mainly on static use cases, including NTN/TN switching in idle mode. the potential for more dynamic use cases, such as service continuity during mobility, has not been sufficiently addressed.

The purpose is to ensure a global service continuity in both idle and connected modes. NTN is expected to provide coverage in overlapping areas with TN but also in areas beyond TN coverage.

While mobility from TN to NTN is mostly triggered by the unavailability of the TN service, the mobility from NTN to TN may be determined by other criteria.

In both cases, the UE shall be able to camp on a TN cell while being able to monitor NTN cells and vice versa.

The related requirements on the UE differ on the type of targeted devices:

Table 1 : NTN capable 6G UE possible architecture characteristics

| NTN capable UE types | Modem | Antenna |
| --- | --- | --- |
| IoT and smartphones | Same for both NTN and TN | Same for both NTN and TN |
| Vehicle mounted | Same or distinct respectively both NTN and TN | One single antenna for TN FR1 and NTN FR1 below 10 GHz, for narrowband or IoT services 2 antennas: one for TN FR1 and another one for TN FR1 below 10 GHz, which could be an antenna more directive with higher gain than the TN 2 antennas, on for FR1 TN and another one for NTN in above 10 GHz (VSAT antenna) for broadband services |
| Building mounted | Same or distinct respectively both NTN and TN | Two for respectively NTN and TN |

Study a design for 6G procedures supporting TN to NTN and NTN to TN mobility for UEs in Idle and Connected mode.

### 2.2.2		6G NTN-NTN mobility

In satellite networks, frequent handovers are inevitable as satellites move, The handover time can reach up to 6.92 s for cell sizes smaller than 50 km, and up to 138.38 s for cell sizes of 1000 km, depending on the user equipment (UE) speed, assuming a constant satellite velocity of 7.56 km/s .

For users, these “service link” handovers occur as inter-beam (between satellite beams), inter-cell (between ground cells/base stations under satellite coverage) . They can interrupt connectivity, especially if switching occurs before the new connection is ready so it’s essential to make these transitions as rare and efficient as possible. By refining decision criteria, improving beam management, and carefully selecting target stations, service disruptions and processing demands can be reduced.

On the ground, “feeder link” handovers also reshape the network, influencing how traffic is routed. To maintain performance and avoid data loss, networks usually prepare new connections before breaking the old ones. Group handovers are another tool to avoid network overload when many users are affected at once.

Both the user device and the network benefit from smart selection: devices should choose networks and satellite types suited to their needs, while the network can optimize handover decisions based on quality requirements. Together, these strategies help maintain smooth communication in the ever-shifting landscape of satellite connectivity.

Additionally, the 6G radio access should also support multi orbit mobility between at least a GSO with an NGSO based scenario, where NGSO may cover VLEO, medium/high LEO and/or MEO.

Study mobility mechanisms for 6G satellite networks (NTN to NTN), at both idle and connected modes. The study should address:

- Inter-beam mobility

- Inter-cell mobility

- Inter-satellite mobility

In view of NTN characteristics, it is interesting to consider the following handover and mobility mechanisms for 6G study:

Time-based CHO: designed for quasi-Earth-fixed cells, such as GEO satellite beams. Handover events can be scheduled in advance because the satellite footprint remains stationary over the area, providing predictable periods for mobility control.

Location-based CHO: best suited for Earth-moving cells, such as those created by LEO or MEO satellite constellations. Handover decisions are triggered when the UE approaches a specified geographic threshold, leveraging knowledge of satellite trajectory and the user's position for more accurate and timely mobility.

RACH-less Handover: removes the need for the UE to perform the random access procedure during handover. This approach significantly reduces access latency and avoids extra delay, which is crucial in NTN where long propagation times would otherwise lengthen handover interruption.

Cell Switching (MAC-triggered, e.g. LTM): particularly effective in network architectures that use a Central Unit/Distributed Unit (CU/DU) split. The MAC layer directly triggers the cell or beam switch, allowing very fast changes between satellites or beams without involving higher-layer protocols, cutting down signaling and improving responsiveness.

Group Handover: optimized for quasi-Earth-fixed cell scenarios where a large number of UEs are served within the same satellite beam. Supports simultaneous handover of groups of UEs, reducing signaling storms and resource contention when many devices move at once (e.g., passengers on a moving train or aircraft).

RAN2 to consider the following mobility mechanisms in 6GR study:

- Time-based and Location-based Conditional Handover (CHO)

- RACH-less handover

- Cell switch mechanism triggered by MAC (e.g. LTM)

- Group handover

## 2.3 		Inter-RAT mobility

Inter-Radio Access Technology (Inter-RAT) mobility is a pivotal feature for 6G Non-Terrestrial Networks (NTN), particularly as NTN could be seen as an extension of terrestrial networks (TN) and the existing 5G NTN infrastructures to achieve ubiquitous coverage and enhanced resilience.

Inter-RAT handover occurs between different RATs that operate at distinct carrier frequencies, presenting unique challenges in maintaining service continuity. For 6G RAN, it is essential to support both idle and connected mode mobility across NTN and TN, specifically enabling seamless handovers between 6G NTN and NR TN/NTN, as well as between 6G NTN and LTE TN. These inter-RAT mobility procedures must be optimized to ensure minimal service interruption during handover, even in handheld scenarios characterized by fast-moving or transient connectivity conditions. Moreover, the handover mechanisms need to be highly reliable to achieve a low handover failure rate, thus safeguarding user experience and protecting operators’ terrestrial spectrum investments during the transition to ubiquitous NTN-enabled connectivity.

Inter-RAT mobility plays an essential role in the integration of 6G Non-Terrestrial Networks (NTN) with existing terrestrial network (TN) and 5G NTN infrastructures, facilitating ubiquitous coverage and resilient communications. Handover events involving different Radio Access Technologies (RATs) often occur between links on separate carrier frequencies, introducing complexity in coordinating smooth transitions while maintaining active service sessions.

In the context of 6G RAN, robust support for both idle and connected mode mobility is required to enable zero interruption and zero packet loss between 6G NTN and both NR TN/NTN or LTE TN domains. This interworking is especially critical for highly dynamic mobility scenarios.

Efficient inter-RAT handover design must mitigate service interruption and ensure a consistently very low packet loss (zero packet loss objective) , regardless of whether the device transitions from terrestrial to non-terrestrial coverage or vice versa. Achieving these goals is key to supporting operational continuity, preserving the integrity of terrestrial network investments, and accelerating the adoption of NTN capabilities in future communication ecosystems.

Consider the support of idle and connected mode mobility between

- 6G NTN and NR NTN

- 6G NTN and LTE TN

- 6G TN and 5G NR-NTN

## 2.5		Multi/Dual Connectivity

Dual Connectivity or Multi-connectivity between NTN and TN can be used for Coverage and Quality of Service enhancement. Also, it can ensure seamless mobility, exploiting Heterogeneous Network (HetNet) environment, connection to a large NTN macro cell for mobility anchor and connection to a local TN small cell for data transfer in optimal radio conditions. Thereby, multi-connectivity can provide a very fluent (no added data latency during handovers) and robust mobility (connection failures minimized). NTN-TN Multi-connectivity also provides increased throughput especially in case of VLEO and LEO; using multiple radio resources can obviously improve data rates experienced by users.

NTN-TN Dual connectivity may contribute to provide seamless mobility by eliminating handover interruption delays and errors, and optimizes capacity, coverage and mobility for devices connected in a heterogeneous NTN-TN network.

Given that the NTN may feature a different bandwidth compared to the one offered by TN, only part of the traffic (most critical one) may be duplicated via the NTN connectivity. Hence, the traffic is partially duplicated between the two NTN and TN connectivity.

Taking into account the diverse capacities offered by different satellite orbits (NGSO/GSO) and the integration of inter-satellite connections, coordinated processing of user-plane and control-plane information is achievable across GSO and non-GSO satellites. User equipment (UE) gains the ability to dynamically choose or hand over between GSO and non-GSO satellite nodes in order to align with changing service performance needs. Moreover, GSO and non-GSO satellites can operate in tandem to serve a single UE, where the GSO segment may be dedicated to managing signaling and network control, while non-GSO resources address user data transfer or deliver low-latency services for time-critical applications.

The 6G radio interface/access shall be defined to support multi-connectivity or Dual Connectivity between two NTN accesses (e.g. GSO and NGSO based) as well as between 6G NTN and 6G TN with simultaneous traffic flow in both/different access links.

# 3	Conclusion

- Study the support of 6G mobility with :

- Zero interruption time

- Zero Packet loss

Study a design for 6G procedures supporting TN to NTN and NTN to TN mobility for UEs in Idle and Connected mode.

Study mobility mechanisms for 6G satellite networks (NTN to NTN), at both idle and connected modes. The study should address:

- Inter-beam mobility

- Inter-cell mobility

- Inter-satellite mobility

RAN2 to consider the following mobility mechanisms in 6GR study:

- Time-based and Location-based Conditional Handover (CHO)

- RACH-less handover

- Cell switch mechanism triggered by MAC (e.g. LTM)

- Group handover

Consider the support of idle and connected mode mobility between

- 6G NTN and NR NTN

- 6G NTN and LTE TN

- 6G TN and 5G NR-NTN

The 6G radio interface/access shall be defined to support multi-connectivity or Dual Connectivity between two NTN accesses (e.g. GSO and NGSO based) as well as between 6G NTN and 6G TN with simultaneous traffic flow in both/different access links.

# References

RP-252912, “Revised SID: Study on 6G Radio”, NTT Docomo, CMCC, AT&T, Vodafone, Beijing, CN, September 2025.

R2-2507644, “6G Radio Access Technology general aspects for NTN”, THALES, Airbus, Echostar, Novamint, Fraunhofer IIS, Prague, CZ, October 2025.