3GPP TSG-RAN WG2 #131bis	R2-2507676

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	8.5.2

Source:	Ericsson

Title:	Discussion on NES RILs O004, O005, H127, H130, and more

Document for:	Discussion

# 1	Introduction

In RAN#105, the WID for network energy savings in Rel-19 was revised in [4]. In this contribution, we address the open issues related to the third objective below given in the revised WID as:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

# 2	Discussion

### 2.1 O004 and O005

These RILs are about whether paging adaptation mechanism introduced in Rel-19 applies also to LP-WUS and SL Relay. We think that it should be applicable to LP-WUS considering the similarity of LP-WUS to PEI. However, we have noticed that whether it should be applicable to LP-WUS is also captured as an open issue (C026) within the context of the LP-WUS WI. Therefore, we suggest leaving this discussion to the LP-WUS session.

Wait for the outcome of the related discussion in LP-WUS on whether paging adaptation mechanism applies to LP-WUS.

For SL Relay on the other hand, there seems to be no need for such applicability considering the motivation from network energy saving aspect and the complexity in case of application.

Paging adaptation mechanism does not apply to SL Relay.

### 2.2 H127

The description of the RIL is as follows: “In legacy spec, for CFRA, if occasions is absent, the corresponding field in RACH-ConfigCommon applies. With the introduction of additional RA resources, both legacy RA occasions and additional RA occasions can be configured in the RACH-ConfigCommon, it needs to be made clear which RA occasion is used when the field is absent in RACH-ConfigDedicated. Considering RAN2 has agreed that RACH adaptation is not applied for L3 HO command, the simplest way could be clarifying that legacy RA occasions (i.e. RA occasions not configured in addlRACH-Config-Adaptation-r19) are used when the field occasions is absent in RACH-ConfigDedicated.

RAN2 has made the following related agreements:

- RACH adaptation is not applied to the following cases (for non-serving cell case): 1) PDCCH order for additional RACH, 2) L3 HO command, 3) LTM CSC MAC-CE, 4) PDCCH order for LTM early UL sync.

- RAN2 confirms the newly introduced 1-bit indication in DCI 1_0 for C-RNTI, i.e., PDCCH order, applies to both CBRA and CFRA.

The use case for dedicated RACH configuration is not limited to handover. Even though, handover is a primary and classic use case for dedicated RA occasions/preambles, to enable a fast, non-contention-based connection to the target cell, the mechanism is also used in other scenarios where the network needs to configure dedicated RA resources to a UE. Some of those other use cases are: RRC Connection Re-establishment, SCG Change/Addition in Dual Connectivity, initial Access to a PSCell. If one considers the limitation for non-serving cell case as captured in the agreement above, there is at least the case for initial access. Note that, in 38.321, all use cases where RA adaptation do not apply are captured explicitly. Therefore, we do not think that the proposed change is needed.

The status of H127 is revised to “PropReject”.

### 2.3 H130

The description of the RIL is as follows: “According to 38.321, when an SCell is deactivated, the MAC entity shall not monitor PDCCH on the SCell or for the SCell:

1> if the SCell is deactivated: 2> not transmit SRS on the SCell; 2> not report CSI for the SCell; 2> not transmit on UL-SCH on the SCell; 2> not transmit on RACH on the SCell; 2> not monitor the PDCCH on the SCell; 2> not monitor the PDCCH for the SCell; 2> not transmit PUCCH on the SCell;

The above PDCCH monitoring behaviour inherits from LTE R10. With this restriction, the SSB adaptation indicated by DCI cannot apply to deactivated SCell. Even though we think dynamically changing the SSB periodicity to a smaller value for fast SCell activation is a valid use case, we prefer not to change the PDCCH monitoring behaviour or to revert RAN2 previous agreement of not introducing new MAC CE. This fast SCell activation can be enabled by OD-SSB operation. Under OD-SSB discussion, it is allowed to use the OD-SSB MAC CE to dynamically change the OD-SSB configuration including SSB periodicity. Alternatively, the network can use RRC reconfiguration to update the SSB periodicity of a deactivated SCell.”

We think that it would be better if we limit the impact to UE behavior regarding PDCCH monitoring if the SCell is deactivated, however the proposed change could be too restrictive considering that it may be beneficial to update the SSB periodicity dynamically for faster SCell activation.

SSB adaptation can be activated via RRC configuration, discuss whether such activation can only be performed when SCell is deactivated.

### 2.4 MAC issue 2 on initialDownlinkBwp-RedCap

Based on the existing SI request procedures and configurations, one can observe that a RedCap UE can use the common SI request resources to request SI when RedCap specific initial uplink BWP and RedCap specific SI request resources are not configured. When RedCap specific initial uplink BWP and RedCap specific SI request resources are configured, RedCap UEs can use RedCap specific SI request resource instead.

The RedCap specific SI request resource only applies to a normal UL carrier. When a RedCap UE initiates the SI request on the SUL carrier, it uses the common SI request resources regardless of whether RedCap specific initial downlink BWP is configured. When the SI request is initiated by using the RedCap specific SI request resources on the NUL carrier, a RedCap UE monitors the PDCCH on the RedCap specific BWP, if configured.

With these principles in mind, one may consider that it may be possible for the network to configure common SIB1 request resources and/or RedCap specific SIB1 request resources. The former may require an update in 38.321 to clarify that UE does not monitor initialDownlinkBWP-Redcap in that case, whereas the latter requires introduction of a RedCap specific OD-SIB1 request configuration and the associated procedure. We think that there is no need to have multiple mechanism here and prefer the former.

No need to introduce RedCap specific SIB1 request resources.

### 2.5 Stage 2

We suggest the following editorial revision to TS 38.300:

"Paging adaptation for network energy saving for UEs in RRC_IDLE and RRC_INACTIVE: in order to increase gNB sleeping time, the value of N and Ns are extended to increase the number of POs per PF and/or make PFs with sparser PFs. The UE supporting paging adaptation shall monitor PDCCH in POs separately signalled for paging adaptation, if configured. The UE supporting paging adaptation and PEI can monitor PEIs separately signalled for paging adaptation, if configured.

Adopt the TP above in the 38.300 CR.

# 3	Conclusion

In this contribution we discuss the open issues related to the third objective. Based on the discussion in the previous section, we propose the following:

Proposal 1	Wait for the outcome of the related discussion in LP-WUS on whether paging adaptation mechanism applies to LP-WUS.

Proposal 2	Paging adaptation mechanism does not apply to SL Relay.

Proposal 3	The status of H127 is revised to “PropReject”.

Proposal 4	SSB adaptation can be activated via RRC configuration, discuss whether such activation can only be performed when SCell is deactivated.

Proposal 5	No need to introduce RedCap specific SIB1 request resources.

Proposal 6	Adopt the TP above in the 38.300 CR.

# References

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.