3GPP TSG RAN WG2 #131bis	R2-2507677

Prague, Czech Republic, Oct 13th-17th, 2025

Agenda Item:	8.8.5

Source: 	Xiaomi, Samsung, OPPO, Apple

Title:  	Discussion on the RIL X500 (smtc in the CarrierInfoNR-r19) for LTE-NR NTN mobility

Document for:	Discussion and Decision

Introduction

This contribution focuses on the handling of the smtc field in CarrierInfoNR-r19 (RIL X500), which is critical for ensuring proper smtc configuration during redirection.

Discussion

According to the agreed CR [1], the nr-r19 is introduced in the RedirectedCarrierInfo and smtc-r19 is carried in the CarrierInfoNR-r19.

RedirectedCarrierInfo ::=			CHOICE {

eutra								ARFCN-ValueEUTRA,

geran								CarrierFreqsGERAN,

utra-FDD							ARFCN-ValueUTRA,

utra-TDD							ARFCN-ValueUTRA,

cdma2000-HRPD						CarrierFreqCDMA2000,

cdma2000-1xRTT						CarrierFreqCDMA2000,

...,

utra-TDD-r10						CarrierFreqListUTRA-TDD-r10,

nr-r15								CarrierInfoNR-r15,

nr-r17								CarrierInfoNR-r17,

nr-r19								CarrierInfoNR-r19

}

CarrierInfoNR-r19	::= SEQUENCE {

carrierFreq-r19					ARFCN-ValueNR-r15,

subcarrierSpacingSSB-r19			ENUMERATED {kHz15, kHz30, kHz120, kHz240},

smtc-r19							MTC-SSB-NR-r15				OPTIONAL,		-- Need OP

satAssistanceInfoList-r19			SEQUENCE (SIZE(1..maxSat-r17)) OF SatelliteId-r18

}

smtc The SSB periodicity/offset/duration configuration of the redirected target NR frequency. It is based on the timing reference of EUTRAN PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

Based on the field description of smtc, if the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

The measObjectNR is used to configure the RRC connected UE to perform the NR frequency measurement and the SMTC configuration related IE is copied as below. According to the measObjectNR, the SMTC configuration is mandatory present in the measObjectNR [2].

MeasObjectNR

The IE MeasObjectNR specifies information applicable for inter-RAT NR neighbouring cells.

MeasObjectNR information element

-- ASN1START

MeasObjectNR-r15 ::=				SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR-r15,

rs-ConfigSSB-r15					RS-ConfigSSB-NR-r15,

threshRS-Index-r15					ThresholdListNR-r15				OPTIONAL,		-- Need OR

maxRS-IndexCellQual-r15				MaxRS-IndexCellQualNR-r15		OPTIONAL,		-- Need OR

offsetFreq-r15						Q-OffsetRangeInterRAT			DEFAULT 0,

excludedCellsToRemoveList-r15			CellIndexList					OPTIONAL,		-- Need ON

excludedCellsToAddModList-r15			CellsToAddModListNR-r15			OPTIONAL,		-- Need ON

quantityConfigSet-r15				INTEGER (1.. maxQuantSetsNR-r15),

cellsForWhichToReportSFTD-r15		SEQUENCE (SIZE (1..maxCellSFTD)) OF PhysCellIdNR-r15	OPTIONAL,	-- Need OR

...,}

RS-ConfigSSB-NR-r15 ::=			SEQUENCE {

measTimingConfig-r15			MTC-SSB-NR-r15,

subcarrierSpacingSSB-r15	ENUMERATED {kHz15, kHz30, kHz120, kHz240},

...,

[[	ssb-ToMeasure-r15			CHOICE {

release						NULL,

setup						SSB-ToMeasure-r15

}											OPTIONAL	-- Need ON

]],

}

-- ASN1STOP

MeasObjectNR field descriptions

rs-ConfigSSB Indicates the SSB configuration for measuring the set of SS blocks within the SMTC measurement duration.

Therefore, the smtc configuration is essential for the for the RRC redirection from LTE to NR.

Observation 1: The SMTC configuration is mandatory present in the measObjectNR and the smtc configuration is essential for the RRC redirection from LTE to NR.

However, the measurement on the NR NTN frequency by an RRC connected UE in an LTE TN cell is currently not supported, and the Rel-20 WI for the E-UTRA TN to NR NTN HO will resolve this issue [3]. This implies that the NR NTN frequency can’t be configured in the measObjectNR currently. Therefore, if the smtc-r19 field is not present in CarrierInfoNR-r19, the SMTC configuration in measObjectNR cannot be used for the RRC redirection, since there is no corresponding NR NTN frequency configured in measObjectNR.

Observation 2: Measurement on the NR NTN frequency by a RRC Connected UE in an LTE TN cell is currently not supported.

Observation 3: The NR NTN frequency cannot be configured in the measObjectNR.

Observation 4: The Release-20 WI for E-UTRA TN to NR NTN handover will introduce support for measurement on the NR NTN frequency by the RRC Connected UE in an LTE TN cell.

Observation 5: If smtc-r19 is absent in CarrierInfoNR-r19, the UE cannot fall back to the SMTC from measObjectNR, due to the lack of a matching NR NTN frequency configuration.

Therefore, we think some clarification in the field description of smtc is needed.

Proposal 1: Clarify in the field description of smtc that ‘In this release, the smtc is mandatory present in the CarrierInfoNR-r19’.

The other issue is that the UE should adjust the offset according to the actual propagation delay for the smtc-r19 in the CarrierInfoNR-r19, which is the same as the cell reselection from LTE to NR NTN.

Proposal 2: For the smtc-r19 in the CarrierInfoNR-r19, the UE should adjust the offset according to the actual propagation delay.

Proposal 3: The following TP is provided for consideration.

smtc The SSB periodicity/offset/duration configuration of the redirected target NR frequency. It is based on the timing reference of EUTRAN PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. In this release, this field is mandatory present in CarrierInfoNR-r19. If the smtc-19 is present, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the UE's propagation delay difference between serving cell and neighbour cells equals to 0 ms, and the UE can adjust the offset based on the actual propagation delay.

Conclusions

In this contribution, we discussed the smtc issues for the RRC redirection and provide the observations and proposals as follows：

The smtc-r19 is absent in the CarrierInfoNR-r19

Observation 1: The SMTC configuration is mandatory present in the measObjectNR and the smtc configuration is essential for the RRC redirection from LTE to NR.

Observation 2: Measurement on the NR NTN frequency by a RRC Connected UE in an LTE TN cell is currently not supported.

Observation 3: The NR NTN frequency cannot be configured in the measObjectNR.

Observation 4: The Release-20 WI for E-UTRA TN to NR NTN handover will introduce support for measurement on the NR NTN frequency by the RRC Connected UE in an LTE TN cell.

Observation 5: If smtc-r19 is absent in CarrierInfoNR-r19, the UE cannot fall back to the SMTC from measObjectNR, due to the lack of a matching NR NTN frequency configuration.

Proposal 1: Clarify in the field description of smtc that ‘In this release, the smtc is mandatory present in the CarrierInfoNR-r19’.

UE adjusts the SMTC according to the actual propagation delay

Proposal 2: For the smtc-r19 in the CarrierInfoNR-r19, the UE should adjust the offset according to the actual propagation delay.

TP for the smtc

Proposal 3: The following TP is provided for consideration.

smtc The SSB periodicity/offset/duration configuration of the redirected target NR frequency. It is based on the timing reference of EUTRAN PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. In this release, this field is mandatory present in CarrierInfoNR-r19. If the smtc-19 is present, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the UE's propagation delay difference between serving cell and neighbour cells equals to 0 ms, and the UE can adjust the offset based on the actual propagation delay.

References

[1] R2-2506535, Introduction of LTE TN to NR NTN IDLE mode mobility

[2] 3GPP TS 36.331 V18.6.0, Radio Resource Control (RRC)

[3] RP-252890, Revise WID on E-UTRA TN to NR NTN handover enhancements