3GPP TSG-RAN WG2 #131bis	R2-2507678

# Prague, Czech Republic, 13th – 17th October 2025

Agenda Item:	 8.1.2.2

Source:	Ericsson

Title:	Discussion on RILs for LCM for UE side model for Beam Management (E040, E041, E042, B206/O300, O301/S045/N114)

Document for:	Discussion

# 1	Introduction

In this paper, we discuss and address some RILs identified for LCM for UE side model for Beam Management.

# 2 	Discussion

## 2.1 (E040) Ensure that UE does not request a candidate UE-side data collection configuration for which it already has a UE-side data collection configuration

For UE-side data collection, RAN2#131 agreed that a UE can request one or more UE-side data collection candidate configurations, out of a list of candidate configurations provided by the NW via otherConfig:

1 Multiple preferred configurations within the list of candidate configurations provided by NW can be indicated by the UE via UAI. 2 No prohibit timer is needed for UE indicating its preferred data collection configuration.

Thus, based on the agreements above, there is no limitation for the UE to ask again for a candidate configuration, even if it may have already received a corresponding UE-side data collection configuration from the NW within CSI-ReportConfig. In clause 5.7.4.2 a limitation was introduced, in the sense that the UE triggers a UAI to ask for candidate configurations only if it has not yet sent such a request (after receiving the configuration), or if the UE’s preference for the candidate configurations has changed.

1> if configured to provide its preference to be configured with radio measurement resources for UE-side data collection: 2> if the UE has a preference to be configured with radio measurement resources to perform UE-side data collection and did not transmit a UEAssistanceInformation message with dataCollectionPreference since it was configured to provide its preference to be configured with radio measurement resources to perform UE-side data collection; or 2> if the preference to be configured with radio measurement resources to perform UE-side data collection has changed since the last transmission of the UEAssistanceInformation message including dataCollectionPreference: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to report the UE preference to be configured with radio measurement resources for UE-side data collection;

However, this limitation is just for the trigger of the UAI and not for the content of the UAI, which is described in clause 5.7.4.3:

2> if the UE prefers to be configured with radio resources to perform data collection: 3> set dataCollectionStart to start; 3> for each serving cell configured with candidate UE-side data collection configuration(s) in dataCollectionCandidateConfigList and for which the UE has one or more preferred radio resource configuration(s): 4> include an entry in dataCollectionPreferredConfigurationList and set the content as follows: 5> set the dataCollectionServCellIndex to the serving cell index of the cell; 5> include in dataCollectionCandidateIdList the dataCollectionCandidateConfigId associated with preferred configuration(s) from dataCollectionCandidateConfigParameterList;

If, for example, the UE sends a request for candidate configuration #1 and then the UE would also like to send a request for candidate configuration #2, in the second request the UE can ask for both candidate configurations #1 and #2, even if the NW may have already provided a UE-side data collection configuration corresponding to candidate #1. Consequently, this is an unnecessary second request for candidate configuration #1, which would have to be processed by the NW. This is not a desired behaviour and it should be fixed.

One possible solution is based on NW implementation, where the NW de-configures a candidate configuration via otherConfig, when the NW provides a corresponding UE-side data collection configuration within CSI-ReportConfig. However, this means the NW needs to keep configuring and de-configuring candidate configurations based on whether there are corresponding UE-side data collection configurations configured at the same time or not. This solution thus introduces an additional amount of signaling.

Another solution is to rely on the UE implementation to compare all the fields (e.g. resources, associated IDs, etc.) within candidate configurations that it requested with the corresponding fields in UE-side data collection configurations received from the NW. While this is possible, it requires additional UE processing.

A third solution is to aid the UE in associating the requested candidate configurations with the actual UE-side data collection configurations from the NW. This can be achieved by adding the ID of the corresponding candidate configuration in the UE-side data collection configuration within CSI-ReportConfig. This is a small addition that would help the UE to check only the ID of the candidate configurations and to avoid sending redundant requests. The specification changes are presented below.

Text proposal

#### 5.7.4.3 Actions related to transmission of UEAssistanceInformation message The UE shall set the contents of the UEAssistanceInformation message as follows: <TEXT OMITTED> 2> if the UE prefers to be configured with radio resources to perform data collection: 3> set dataCollectionStart to start; 3> for each serving cell configured with candidate UE-side data collection configuration(s) in dataCollectionCandidateConfigList and for which the UE has one or more preferred radio resource configuration(s): 4> include an entry in dataCollectionPreferredConfigurationList and set the content as follows: 5> set the dataCollectionServCellIndex to the serving cell index of the cell; 5> include in dataCollectionCandidateIdList the dataCollectionCandidateConfigId associated with preferred configuration(s) from dataCollectionCandidateConfigParameterList; NOTE: The UE does not include in the list of preferred configurations the candidate configurations whose values of dataCollectionCandidateConfigId are included in the dataCollectionCandidateId in the CSI report configurations of the serving cell. <TEXT OMITTED> CSI-ReportConfig information element -- ASN1START -- TAG-CSI-REPORTCONFIG-START CSI-ReportConfig ::= SEQUENCE { reportConfigId CSI-ReportConfigId, carrier ServCellIndex OPTIONAL, -- Need S resourcesForChannelMeasurement CSI-ResourceConfigId, csi-IM-ResourcesForInterference CSI-ResourceConfigId OPTIONAL, -- Need R nzp-CSI-RS-ResourcesForInterference CSI-ResourceConfigId OPTIONAL, -- Need R <TEXT OMITTED> predictionConfiguration-r19 CHOICE { csi-InferencePrediction-r19 ENUMERATED {true}, configurationForChannelPrediction-r19 SEQUENCE { resourcesForChannelPrediction-r19 CSI-ResourceConfigId OPTIONAL, -- Need R associatedIdForChannelPrediction-r19 AssociatedId-r19 OPTIONAL, -- Need R associatedIdForChannelMeasurement-r19 AssociatedId-r19 OPTIONAL, -- Need R nrofReportedPredicted-RS-r19 ENUMERATED {n1, n2, n3, n4} OPTIONAL, -- Need R nrofTimeInstance-r19 ENUMERATED {n1, n2, n4, n8} OPTIONAL, -- Need R timeGap-r19 ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1} OPTIONAL, -- Need R, dataCollectionCandidateId-r19 DataCollectionCandidateConfigId-r19 OPTIONAL, -- Need R ... }, configurationForChannelMonitoring-r19 SEQUENCE { refToPredictionConfig-r19 CSI-ReportConfigId, nrofBestBeamForMonitoring-r19 ENUMERATED {n1, n2} OPTIONAL, -- Need R nrofTransmissionOccasion-r19 ENUMERATED {n1, n3, n7, n15} OPTIONAL, -- Need R timeInstanceFor-RS-PAI-r19 ENUMERATED {n1, n2, n8, spare1} OPTIONAL, -- Need R mappingToResourcesForChannelPrediction-r19 BIT STRING (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OPTIONAL, -- Need R timeInstanceFor-SGCS-r19 ENUMERATED {n1, spare3, spare2, spare1} OPTIONAL, -- Need R ... } }

Introduce the ID of the candidate data collection configuration (dataCollectionCandidateConfigId-r19) in the configuration of the corresponding UE-side data collection configuration (in CSI-ReportConfig), based on the TP above.

## 2.2 (E041) Associated ID validity

If UEs have collected data for a specific associated ID and trained a UE-side model based on that data, they will then be able to use the model for a later inference procedure. Once an associated ID value is signalled from the network to the UE, the meaning of that value at the UE (or UE-side entity) corresponds to the same beam property/deployment forever. This is since, according to current specifications, there is no way for the network to reset/reassign that value to other beam properties and inform the UE that it should forget the old meaning of that value.

We think it is an unreasonable requirement from the network point of view to assume that a given meaning holds for a given associated ID forever. It may happen that there is e.g. a hardware or software update in the network that makes the previous configurations and hence the allocations of the associated IDs obsolete. The UE and the network must have the same understanding of an associated ID value also after such changes. It is thus important for the UE (or UE-side entity) to understand that properties for an associated ID have changed, so that it can handle any stored UE-side model for that associated ID accordingly. Any such model would then typically not be useful for inference any longer.

It should therefore be possible for a network to indicate to a UE that an associated ID is no longer valid, so that the UE (or UE-side entity) can handle its corresponding UE-side model accordingly, e.g. release or retrain it.

The meaning of an associated ID will need to be changed at some point in time when there is a change of associated radio conditions, e.g. due to replacement of hardware or change of configuration in the mobile radio network. The corresponding trained UE-side models would then no longer be useful for inference.

The network should be able to indicate to a UE that an associated ID is no longer valid, so that the UE (and/or associated training server) can handle its corresponding UE-side model accordingly, e.g. release or retrain it.

For the network indication to the UE, one could consider that the network sends a separate indication that an associated ID is no longer valid. It is then not fully clear when such an indication would be sent, since the network does not know for which associated IDs the UE has a UE-side model.

An alternative is to instead include a separate version identification for the associated ID. The UE then stores a version number with the associated ID and in case there is a change of the associated ID, the network then provides a different version number for it. The UE can then release any stored UE model for the previous version of the associated ID. This option would have the least impact to the specification and we thus propose to introduce the following TP (for this solution) in the WI RRC CR for AI/ML for Phy. Considering that the case where the network would need to change the meaning of an associated ID can be assumed to be very seldom, it would probably be enough with just a few values for the version number.

Text Proposal

#### –	AssociatedId

The IE AssociatedId indicates that the UE may assume similar properties of a DL Tx beam or beam set/list associated with the same value. The AssociatedID value is unique within a PLMN, i.e. it can only be associated with one same/similar beam deployment within the same PLMN.

AssociatedId information element

-- ASN1START

-- TAG-ASSOCIATEDID-START

AssociatedId-r19 ::=        SEQUENCE {

associatedIdNumber-r19     BIT STRING (SIZE (24)),

versionNumber-r19           INTEGER (0..3)               OPTIONAL      -- Need R

}

-- TAG-ASSOCIATEDID-STOP

-- ASN1STOP

AssociatedId field descriptions

versionNumber Indicates the version number of the associated ID. If the UE receives a new versionNumber for an associated ID, it shall consider any other version of the same associated ID as no longer valid. If the versionNumber is not included, the UE can assume that the received associated Id has the same versionNumber as the one previously received by the UE, if any.

RAN2 to introduce the TP for inclusion of a version number for a provided associated ID in the WI RRC CR for AI/ML for Phy.

## 2.3 (E042) Necessity of dedicated flag to configure applicability reporting via UAI

RAN2#130 agreed the following:

2 Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled. Assume this applies to Option A and B, FFS if anything different needs to be done for option B (if specified)

After adding the sets of inference related parameters for option B in otherConfig, it was not discussed again how to combine the flag in ASN.1 agreed above (and also included in otherConfig) with the configurations for option B. Thus, in the current RRC specifications, there is a separate flag reportApplicabilityUAI within ApplicabilityReportConfig, i.e. at the same level with the list applicabilityConfigList, containing the parameters for option B.

ApplicabilityReportConfig-r19 ::= SEQUENCE { reportApplicabilityUAI-r19 ENUMERATED {true} OPTIONAL, -- Need R applicabilityConfigList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF ApplicabilityConfig-r19 OPTIONAL, -- Need R ... }

Let us discuss some possible signalling solutions.

- One solution is to keep the flag reportApplicabilityUAI within applicabilityReportConfig, as it is. This means that the flag can be set (i.e. applicability reporting via UAI can be configured) independently of whether one or both of option A and option B are configured. Nonetheless, we think some minor changes are needed in the procedural text, for clarity.

Text proposal for solution (1):

#### 5.7.4.2 Initiation <TEXT OMITTED> 1> if configured to report assistance information about the applicability of configurations subject to the applicability determination procedure based on reportApplicabilityUAI: 2> if the applicability status of configurations subject to the applicability determination procedure has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or in UEAssistanceInformation): 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to report assistance information about the applicability of configurations subject to the applicability determination procedure;

- Another solution is to remove the flag reportApplicabilityUAI and assume that applicability reporting via UAI is implicitly configured when applicabilityReportConfig is included in otherConfig. This would optimize the signalling in the sense that the separate flag would be spared, but it would mean that whenever option B is configured, applicability reporting via UAI is also implicitly configured, i.e. there is no possibility to configure option B without applicability reporting via UAI (and only with applicability reporting via RRCReconfigurationComplete).

Text proposal for solution (2):

ApplicabilityReportConfig-r19 ::= SEQUENCE { reportApplicabilityUAI-r19 ENUMERATED {true} OPTIONAL, -- Need R applicabilityConfigList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF ApplicabilityConfig-r19 OPTIONAL, -- Need R ... }

- A third solution is to move the flag reportApplicabilityUAI one level up, directly under otherConfig. This is similar to solution (1), except that the flag would be even more separated from the configurations for option B.

Text proposal for solution (3):

#### 5.7.4.2 Initiation <TEXT OMITTED> 1> if configured to report assistance information via UEAssistenceInformation message about the applicability of configurations subject to the applicability determination procedure: 2> if the applicability status of configurations subject to the applicability determination procedure has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or in UEAssistanceInformation): 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to report assistance information about the applicability of configurations subject to the applicability determination procedure; <TEXT OMITTED> OtherConfig-v1900 ::= SEQUENCE { gapOccasionCancelRatioReportConfig-r19 SetupRelease {GapOccasionCancelRatioReportConfig-r19} OPTIONAL, -- Need M lpwus-OffsetPreferenceConfig-r19 SetupRelease {LPWUS-OffsetPreferenceConfig-r19} OPTIONAL, -- Need M applicabilityReportConfig-r19 SetupRelease {ApplicabilityReportConfig-r19} OPTIONAL, -- Need M reportApplicabilityUAI-r19 ENUMERATED {true} OPTIONAL, -- Need R dataCollectionPreferenceConfig-r19 SetupRelease {DataCollectionPreferenceConfig-r19} OPTIONAL, -- Need M loggedDataCollectionAssistanceConfig-r19 SetupRelease {LoggedDataCollectionAssistanceConfig-r19} OPTIONAL, -- Need M assisted-SSB-MTC-Config SetupRelease {Assisted-SSB-MTC-Config-r19} OPTIONAL -- Need M } <TEXT OMITTED> ApplicabilityReportConfig-r19 ::= SEQUENCE { reportApplicabilityUAI-r19 ENUMERATED {true} OPTIONAL, -- Need R applicabilityConfigList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF ApplicabilityConfig-r19 OPTIONAL, -- Need R ... }

RAN2 to discuss the following three signalling options for configuring applicability reporting via UAI:

- Keep the flag reportApplicabilityUAI within applicabilityReportConfig;

- Remove the flag reportApplicabilityUAI and assume that applicability reporting via UAI is implicitly configured when applicabilityReportConfig is included in otherConfig;

- Move the flag reportApplicabilityUAI one level up, directly under otherConfig.

## 2.4 (B206/O300) Transfer of applicability information in handover command preparation

In RILs B206/O300 it is proposed to enhance either the inter-node HandoverPreparationInformation message, or the UAI from the UE, so that the applicability information does not contain only applicability changes (according to the latest UAI from the UE, as in legacy), but rather the full list of applicability for all configurations, when the UAI is forwarded from source to target gNB.

First of all, we note that such enhancements are not compliant with the RAN2 agreements:

RAN2#128: „Source cell UAI (as is) can be sent from source cell to target cell using existing signaling. No further optimizations will be considered in RAN2 related to UAI.” RAN2#129: “Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.” “Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause”

Thus, to adopt these enhancements, we would need to revert the previous agreements above. This should be done only if very well justified and, in our view, there is no relevant use case for these changes. The applicability status included in the UAI refers only to configurations for source cell/gNB. When the UE is handed over to a target gNB, it is expected that other configurations relevant to the target gNB are provided by the target gNB in the HO Command message. The UE then responds with the applicability in the HO Complete message (i.e. RRCReconfigurationComplete). Thus, we do not see the benefit to change the current UAI content and procedures for the purpose of sending additional applicability information (about the configurations of the source gNB) to the target gNB, that is not of interest to the target gNB anyway.

Do not introduce further specification changes to enhance applicability reporting via UAI in case of handover.

## 2.5 (O301/S045/N114) Missing purpose for UE-side data collection request

The RILs O301/S045/N114 discuss issues related to the start indication from the UE to the NW, for UE-side data collection. Overall, we think that indeed some clarification on the purpose of this indication is needed, since different signalling on the start indication, preferred list of candidate configurations to start, and preferred list of configurations to stop data collection was agreed in different stages of the work, and it may not be clear which combinations of these parameters are possible or make sense.

In our view, the start indication is a baseline feature, whereas the list of candidate configurations can be optionally configured by the NW, to allow the UE to send preferred candidates. Thus, in our view, it is possible that the UE can send a start indication even if the NW did not provide a list of preferred candidates and the UE can thus not indicate specific preferred candidates. Furthermore, we think that the UE, as in current specifications, should have the possibility to send a separate list of preferred candidate configurations to be started and a separate list of UE-side data collection configurations to be stopped. A single list where the UE includes only the preferred candidates to start and the NW interprets the missing candidates as configurations to stop may be ambiguous. This is since the UE may omit a candidate due to two different reasons and the NW cannot know which one of them it is:

- The UE requests the NW to stop the omitted candidate

- The UE already has a UE-side data collection configuration for the omitted candidate and wants to keep collecting data for it, so it does not make sense to request it again (see also RIL E040).

To avoid extensive changes, we suggest to keep the ASN.1 structure as it is (with a separate start indication, a list of preferred candidates, and a list of configurations to stop) and add the following clarification in the procedural text.

Text proposal

#### 5.7.4.3 Actions related to transmission of UEAssistanceInformation message The UE shall set the contents of the UEAssistanceInformation message as follows: <TEXT OMITTED> 2> if the UE prefers to be configured with radio resources to perform data collection: 3> if the UE did not receive any candidate UE-side data collection configuration(s) in dataCollectionCandidateConfigList: 4> set dataCollectionStart to start; 3> else, for each serving cell configured with candidate UE-side data collection configuration(s) in dataCollectionCandidateConfigList and for which the UE has one or more preferred radio resource configuration(s): 4> include an entry in dataCollectionPreferredConfigurationList and set the content as follows: 5> set the dataCollectionServCellIndex to the serving cell index of the cell; 5> include in dataCollectionCandidateIdList the dataCollectionCandidateConfigId associated with preferred configuration(s) from dataCollectionCandidateConfigParameterList;

The UE can send an indication to start UE-side data collection without a list of preferred candidate configurations, if the NW allowed the UE to send start/stop requests but did not provide a list of candidate configurations. Adopt the TP above.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The meaning of an associated ID will need to be changed at some point in time when there is a change of associated radio conditions, e.g. due to replacement of hardware or change of configuration in the mobile radio network. The corresponding trained UE-side models would then no longer be useful for inference.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Introduce the ID of the candidate data collection configuration (dataCollectionCandidateConfigId-r19) in the configuration of the corresponding UE-side data collection configuration (in CSI-ReportConfig), based on the TP above.

Proposal 2	The network should be able to indicate to a UE that an associated ID is no longer valid, so that the UE (and/or associated training server) can handle its corresponding UE-side model accordingly, e.g. release or retrain it.

Proposal 3	RAN2 to introduce the TP for inclusion of a version number for a provided associated ID in the WI RRC CR for AI/ML for Phy.

Proposal 4	RAN2 to discuss the following three signalling options for configuring applicability reporting via UAI:

a)	Keep the flag reportApplicabilityUAI within applicabilityReportConfig;

b)	Remove the flag reportApplicabilityUAI and assume that applicability reporting via UAI is implicitly configured when applicabilityReportConfig is included in otherConfig;

c)	Move the flag reportApplicabilityUAI one level up, directly under otherConfig.

Proposal 5	Do not introduce further specification changes to enhance applicability reporting via UAI in case of handover.

Proposal 6	The UE can send an indication to start UE-side data collection without a list of preferred candidate configurations, if the NW allowed the UE to send start/stop requests but did not provide a list of candidate configurations. Adopt the TP above.