3GPP TSG-RAN WG2 #131bis	R2-2507679

# Prague, Czech Republic, 13th – 17th October 2025

Agenda Item:	 8.1.3

Source:	Ericsson

Title:	Discussion on RILs for NW-side data collection (Z004, J008, J009, S044, H007, H002)

Document for:	Discussion

# 1	Introduction

In this paper, we discuss and address open issues and provide analysis on RILs for the NW-side data collection.

# 2 	Discussion

## 2.1 (Z004, J008, J009, S044) Handling of logging entities in VarCSI-LogMeasReport

When the UE is configured with a new CSI logged measurement configuration (i.e. for a new csi-LoggedMeasurementConfigId), a corresponding entry is added also within the UE variable used for storing the logged measurements (VarCSI-LogMeasReport). In case it is the first logging configuration for the cell, an entity is also first introduced on cell level.

A couple of issues have been raised for this:

- (S044) An entity in VarCSI-LogMeasReport may be empty at the time of reporting the log to the network

- (Z004) The UE cannot add new entities in VarCSI-LogMeasReport if the buffer for data logging is full

For issue a) there are proposed changes related to this both for S044 and for J008, which is addressing a slightly different issue about how to include the logs in multiple UEInformationResponse messages. The proposed changes for both these RILs clarify that only entities with logged measurements should be reported and should thus solve a possible issue with reporting empty entities.

For issue b) Z004 includes a proposed change where the entities are not added to VarCSI-LogMeasReport at reception of the CSI logged measurement configuration, but instead at the actual logging. The proposed change for Z004 does however not seem to be complete, since the logging is there still performed for the entities within VarCSI-LogMeasReport (which should then not be present). An alternative TP (based on 38.331, v19.0.0) with some further changes is provided below. It is proposed to include this TP in the WI RRC CR for AI/ML for Phy.

TP for addition of entities in VarCSI-MeasLogReport at measurement logging

#### 5.5a.1.3	Reception of CSI-LoggedMeasurementConfig by the UE

Upon receiving csi-LoggedMeasurementConfigToAddModList in the csi-MeasConfig of a serving cell, the UE shall:

1>	for each CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList:

2>	if the current UE configuation for the serving cell includes the CSI logged measurement configuration associated with the given csi-LoggedMeasurementConfigId:

3>	modify the CSI logged measurement configuration according to the configuration received in csi-LoggedMeasurementConfigToAddModList;

2>	else:

3>	add the received CSI logged measurement configuration to the UE configuration;

2>	if the cell identity of the serving cell for which the measurements shall be logged, i.e. the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, is not included in an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport:

3>	include an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport;

3>	set cellId to the CGI of the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, if available. If the CGI is not available for that cell, set cellId to the ARFCN and PCI of the serving cell;

2>	if not already present, include an entry in csi-LogMeasInfoList in VarCSI-LogMeasReport and set refCSI-LoggedMeasurementConfigId to the csi-LoggedMeasurementConfigId associated to the CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList;

2>	perform measurements logging as specified in 5.5a.3.2.

#### 5.5a.3.2	Initiation

The UE shall:

1>	for each CSI logged measurement configuration associated with a refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport, perform the logging of measurements for the corresponding serving cell associated with cellId, in accordance with the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList:

2>	if the csi-LoggedMeasurementEventTriggerConfig is not included and the buffer for network-side data collection is not full:

3>	perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

2>	if the csi-LoggedMeasurementEventTriggerConfig is included and the buffer for network-side data collection is not full:

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the entering condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the entering condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the leaving condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the leaving condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	stop performing the logging for the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList;

2>	when performing the loggingupon receiving measurement results from the lower layers for a CSI logged measurement configuration for a serving cell:

3>	if the cell identity of the serving cell for which the measurement results are received, i.e. the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, is not included in an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport:

4>	include an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport with cellId set to the CGI of the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, if available. If the CGI is not available for that serving cell, set cellId to the ARFCN and PCI of the serving cell;

3>	if not already present for the serving cell, include an entry in csi-LogMeasInfoList within VarCSI-LogMeasReport and set refCSI-LoggedMeasurementConfigId to the csi-LoggedMeasurementConfigId of the CSI logged measurement configuration in csi-LoggedMeasurementConfigToAddModList;

3>	for each CSI logged measurement configuration associated to refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport:

34>	set the csi-RS-MeasResultList and SSB-MeasResultList to include the quantities that were receivedthe UE is logging measurements for, upon receiving the quantities from the lower layers;

34>	if the time between the measurements that are logged and included in this instance of csi-LogMeasInfoList and the measurements for the previous instance of csi-LogMeasInfoList with the same refCSI-LoggedMeasurementConfigId, for the same serving cell, is longer than the logging periodicity (if configured) or the periodicity of the measurement resources (if the logging periodicity is not configured):

45>	set the timeGap to true;

2>	when the memory reserved for the logged measurement information for data collection becomes full, stop logging;

2>	when the memory reserved for the logged measurement information for data collection is no longer full, resume logging.

A related part that seems to be missing, with or without a change for Z004, is however that the corresponding entities in VarCSI-LogMeasReport should also be removed at some point in time. The RIL J009 addresses this specifically for the case where the logged measurements are reported. It has a proposed change where empty entries within csi-LogMeasInfoCellList are removed after transmission of UEInformationResponse message with csi-LogMeasReport, when the logged measurements are discarded. This would then mean that there is a 1-1 mapping between presence of the entities in VarCSI-LogMeasReport and that there are logged measurement results stored for them, i.e. the corresponding entities in csi-LogMeasInfoCellList and csi-LogMeasInfoList will only be present if they include any logged measurement results. With such a 1-1 mapping there should then not be any need for the proposed changes in S044 and for J008, i.e. for issue a) above.

To achieve the 1-1 mapping the corresponding entities in VarCSI-LogMeasReport should however be removed at all cases where the logged measurement are discarded. The wording “discard the logged measurement entries included in VarCSI-LogMeasReport” is used for the other cases in 38.331 where the logged measurements are discarded. RAN2 should discuss if this is sufficient or if it needs to be clarified that this means that the UE also discards the corresponding entities in csi-LogMeasInfoCellList and/or csi-LogMeasInfoList within VarCSI-LogMeasReport.

The proposed change for Z004 is included in the WI RRC CR for AI/ML for Phy, based on Text Proposal in this document.

The proposed change for J009 is included in the WI RRC CR for AI/ML for Phy. The proposed changes for S044 and J008 are not included.

RAN2 to discuss whether there is a need to clarify that “discard the logged measurement entries included in VarCSI-LogMeasReport” also includes that the corresponding entities in csi-LogMeasInfoCellList and/or csi-LogMeasInfoList within VarCSI-LogMeasReport are discarded.

## 2.2 (H007) Handling of logged measurements at configuration modification/release

The RIL H007 addressed the handling of logged measurements in case there is a modification or release of a CSI logged measurement configuration, with the proposed change that any logged measurements for that configuration should be released in both cases.

The case of modification of an existing CSI logged measurement configuration could be avoided by network implementation, but it is a case that is still supported in the specification. We therefore tend to agree with the proposal to add that any logged measurements for the modified csi-LoggedMeasurementConfigId should be discarded.

For the case of release of a CSI logged measurement configuration we however think that it would be a normal case to first release the configuration and later retrieve the logged measurements. That would not be possible if the logged measurements are automatically discarded if the configuration is released. The network would then need to always retrieve the data before the configuration can be released, which in turn means that the UE would need to have the logging configuration for some additional time (until the data has been successfully retrieved). This would then mean that the UE would need to perform logging in vain (between the fetching of data until the configuration is released by the network).

As commented for H007, the issue with not discarding the data at release of the logging configuration is in case the network then afterwards configures another logging configuration with the same csi-LoggedMeasurementConfigId. This case we however think can be handled by instead discarding any logged measurements (if available for that csi-LoggedMeasurementConfigId) when the logging configuration is added.

We here provide a corresponding Text Proposal, which we propose be included in the WI RRC CR for AI/ML for Phy.

Text Proposal

#### 5.5a.1.3	Reception of CSI-LoggedMeasurementConfig by the UE

Upon receiving csi-LoggedMeasurementConfigToAddModList in the csi-MeasConfig of a serving cell, the UE shall:

1>	for each CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList:

2>	if the current UE configuation for the serving cell includes the CSI logged measurement configuration associated with the given csi-LoggedMeasurementConfigId:

3>	discard any logged measurement entries included in VarCSI-LogMeasReport for this csi-LoggedMeasurementConfigId;

3>	modify the CSI logged measurement configuration according to the configuration received in csi-LoggedMeasurementConfigToAddModList;

2>	else:

3>	discard any logged measurement entries included in VarCSI-LogMeasReport for the csi-LoggedMeasurementConfigId for the serving cell, if available;

3>	add the received CSI logged measurement configuration to the UE configuration;

2>	if the cell identity of the serving cell for which the measurements shall be logged, i.e. the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, is not included in an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport:

3>	include an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport;

3>	set cellId to the CGI of the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, if available. If the CGI is not available for that cell, set cellId to the ARFCN and PCI of the serving cell;

2>	if not already present, include an entry in csi-LogMeasInfoList in VarCSI-LogMeasReport and set refCSI-LoggedMeasurementConfigId to the csi-LoggedMeasurementConfigId associated to the CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList;

2>	perform measurements logging as specified in 5.5a.3.2.

The UE should discard any logged measurements included in VarCSI-LogMeasReport for the csi-LoggedMeasurementConfigId of a CSI logged measurements configuration that is added or modified. The provided Text Proposal should be included in the WI RRC CR for AI/ML for Phy.

## 2.3 (H002) Retaining logged measurement at LTM

The solution to keep logged measurements at a reconfiguration with sync, which was agreed at RAN2#130, was discussed further at the RAN2#131 meeting with the following agreement:

- RAN2 confirm that the solution agreed in RAN2#130 is applicable to regular HO and CHO (i.e. 1-bit indication corresponding to each candidate cell configuration in RRCReconfiguration is provided).

The corresponding agreement at RAN2#130 was:

- (RRC-18) 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration with synch is introduced.  In case of HO, the source sends the 1-bit indication to target cell in HO preparation message. This 1-bit indication is included in HO command by target cell (if the target cell wants to keep the data).   We should have single UE behaviour, when it receives the indication it keeps it, otherwise it removes it.   Notify RAN3

A retainLoggedMeasurements field was introduced both in the RRCReconfiguration message from the network to the UE and in the HandoverPreparationInformation from the source gNB to the target/candidate gNB.

The agreement at RAN2#131 says that the solution is applicable for regular HO and CHO. However, as described in the RIL H002, it does not mention LTM and it was requested to confirm that it is also supported for LTM. The case of subsequent LTM was raised as a possible issue during the discussions.

For the UE, if the retainLoggedMeasurements field is included in the RRCReconfiguration message that is executed at an LTM cell switch (thus including reconfiguration with sync), it should act according to it just as for a normal HO or a CHO. Since the HandoverPreparationInformation message is used for configuration of inter-gNB LTM candidates, the candidate gNBs will include the field in the RRCReconfiguration message in the LTM candidate configuration just as for HO or CHO. This would then be the case independent if the LTM candidate configuration is executed as a first LTM cell switch or a subsequent LTM cell switch.

We therefore propose RAN2 to clarify that the solution to retain un-retrieved data through an indication in RRCReconfiguration (with sync) is applicable also at LTM (initial or subsequent).

RAN2 to clarify that the solution to retain logged measurements through an indication in RRCReconfiguration (with sync) is applicable also at LTM, both initial and subsequent.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The proposed change for Z004 is included in the WI RRC CR for AI/ML for Phy, based on Text Proposal in this document.

Proposal 2	The proposed change for J009 is included in the WI RRC CR for AI/ML for Phy. The proposed changes for S044 and J008 are not included.

Proposal 3	RAN2 to discuss whether there is a need to clarify that “discard the logged measurement entries included in VarCSI-LogMeasReport” also includes that the corresponding entities in csi-LogMeasInfoCellList and/or csi-LogMeasInfoList within VarCSI-LogMeasReport are discarded.

Proposal 4	The UE should discard any logged measurements included in VarCSI-LogMeasReport for the csi-LoggedMeasurementConfigId of a CSI logged measurements configuration that is added or modified. The provided Text Proposal should be included in the WI RRC CR for AI/ML for Phy.

Proposal 5	RAN2 to clarify that the solution to retain logged measurements through an indication in RRCReconfiguration (with sync) is applicable also at LTM, both initial and subsequent.