3GPP TSG-RAN WG2 #131bis	Tdoc R2-2507689

Prague, Czech Republic, 2025-10-13 - 2025-10-17

Agenda Item:	8.19.2

Source:	KDDI Corporation

Title:	Discussion on speed dependant scaling of measurement related parameters

Document for:	Discussion

1	Introduction

In the last RAN2 #131 meeting, we proposed the need for speed-dependent scaling of measurement-related parameters within TEI-19[1]. This contribution proposes to continue the discussion on three candidate approaches for it. These approaches were identified through offline discussions following RAN2#131, reflecting diverse operational experiences and technical perspectives from multiple operators and vendors.

2	Discussion

At RAN2#131, We proposed the need for speed-dependent scaling of measurement-related parameters within TEI-19. While initial discussions highlighted the need for such mechanisms, more concrete proposals emerged through subsequent offline exchanges among operators and vendors.

As a result, three candidate approaches were identified:

- Option 1: No standardization; rely on vendor-specific network implementation.

- Option 2: Reuse LTE-based mechanisms, such as handover count-based mobility state estimation.

- Option 3: Use RSRP variation-based estimation, such as relaxed measurement criteria.

The following points summarize the current understanding and concerns regarding each option:

- Option 1 offers flexibility; however, it may result in inconsistent implementations and a lack of transparency, particularly in multi-vendor environments. There is no guarantee that network vendors will implement specific features, which means NR may not deliver the functionalities required by operators. Therefore, this option may not be acceptable for some operators.

- Option 2 has proven effective in highway and railway scenarios, offering a familiar framework for operators. It has benefits from operational precedent but may not scale well in complex or mixed cells deployment scenarios.

- Option 3 is promising in terms of adaptability. It may be better suited for heterogeneous network environments, however, lacks sufficient commercial validation and consensus. It requires further validation.

In addition, we proposed the need for inclusion of medium mobility state definitions such as LTE. This proposal is based on actual commercial network usage, where medium mobility conditions are frequently encountered and relevant to parameter tuning.

Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Clarify the applicability and limitations of each candidate option, including identifying effective deployment scenarios such as highways, railways, and heterogeneous networks.

Proposal 2	Initiate a survey among operators and vendors to gather feedback on the feasibility and preference for each option. This will ensure that the standardization process is informed by practical insights and deployment realities across different regions and technologies.

Proposal 3	Consider the inclusion of medium mobility state definitions in the standard, based on commercial network usage.

Proposal 4	Conduct a survey among operators to assess whether Option 1 is acceptable, specifically considering the risk that there is no guarantee network vendors will implement the required features. The survey should clarify if operators can tolerate the possibility that NR may not provide the functionalities they need due to vendor-specific implementations.

We believe that further discussion on the above items will help clarify the direction of this TEI-19 topic and facilitate consensus among stakeholders. We invite interested parties to share their views and support continued exploration of speed dependant scaling mechanisms.

References

R2-2506114, Discussion on MobilityStateParameters in RRC_CONNECTED, KDDI, RAN2#131