3GPP TSG-RAN WG2 Meeting #131-bis	R2-2507690

# Prague, Czechia, October 13–17, 2025

Agenda Item:	8.8.2

Source:	Ericsson

Title:	Discussion on various RILs for NR NTN

Document for:	Discussion/Decision

# Introduction

At RAN2#131, RAN2 has completed the work related to DL coverage enhancements and broadcast services for NTN (see WID [2]).

According to the rapporteur’s assessment of the collected RILs, however, certain open issues exist. The following RILs are marked as “ToDo”:

A200, C003, C008, E010, E011, S024, H250, N085, Q200, V208, X250, Z256, and Z258.

In what follows, we address those RILs one by one.

# Discussion

## 2.1	DL coverage enhancements RILs

### S024

[Description]: RAN2 agreement is that

-	The UE reports an indication of the N closest reference locations via UE assistance information, e.g. bitmap or list of indices of the locations.

The UE can report the N closest reference locations via the RRCReconfigurationComplete message.

“UE can ..” in our understanding means UE optionally reports in RRCReconfigurationComplete message. If we implement in the current way below, it enforces UE always reports in RRCReconfigurationComplete message, which obviously is not aligned with the agreement!

2>	if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state:

3>	include referenceLocationReport;.

[Proposed Change]: Add “if available” at the end.

2>	if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state:

3>	include referenceLocationReport, if available;

[Comments]:

[vivo]: The proposed change is fine to us

[Rapp] In our understanding of the agreement, the “can” indicates an alternative way of reporting. Let’s clarify this in the next meeting. We are unsure what is the concern and why “if available” would be applicable. A UE needs to know its accurate location to initiate and maintain its connection to the network.

In our view, it is beneficial for both the UE (in terms of UE energy consumption) and the network (in terms of radio resource usage) that the UE provides the initial referenceLocationReport via the RRCReconfigurationComplete message, provided that the UE is configured by the network to provide the N closest reference locations in the preceding RRCReconfiguration message. Ideally, the UE should rely on the UEAssistanceInformation message only for providing an updated referenceLocationReport, e.g., upon UE movement. We expect such updates to be a possible but rare event, since NTN cells are large and the majority of UEs are staying in RRC_CONNECTED mode for a short time. Moreover, provisioning of updates to a previous referenceLocationReport is so far the only case identified in RAN2 where the UE cannot piggyback the information onto the RRCReconfigurationComplete message and must hence rely on the UAI procedure (as discussed at RAN2#131).

- [S024] It is beneficial for UE and network that the UE provides referenceLocationReport in the RRCReconfigurationComplete message, which, so far, was believed to be always possible for the initial report after the UE has been configured in the RRCReconfiguration message.

It is unclear why (in which case) the referenceLocationReport would not be available in RRC_CONNECTED mode, as suggested by the company that has raised the RIL.

- [S024] Since the UE needs accurate location information for accessing and maintaining the connection to an NTN cell, it is uncertain the why referenceLocationReport would not be timely available in RRC_CONNECTED mode.

For deriving the referenceLocationReport, the UE needs 1) the configuration for UE-assisted SMTC selection/configuration and 2) its GNSS position. The configuration is provided by the network via dedicated RRC signaling, otherwise the UE anyway does not need to report this information. If the reference locations are absent in dedicated RRC signaling, they are taken from broadcasted system information. In our understanding, a UE in RRC_CONNECTED state maintains up-to-date system information and monitors for an SI change indication in any paging occasion at least once per modification period (see TS 38.331). Regarding its GNSS position, we believe the UE needs to know its accurate location before entering and while being in RRC_CONNECTED mode. We consider the following excerpt from TS 38.300 v18.6.0:

#### 16.14.2.2	Timing Advance and Frequency Pre-compensation

For the serving cell, the network broadcast valid ephemeris information and Common TA parameters. The UE shall have valid GNSS position as well as ephemeris and Common TA before connecting to an NTN cell. To achieve synchronisation, before and during connection to an NTN cell, the UE shall compute the RTT between UE and the RP based on the GNSS position, the ephemeris, and the Common TA parameters (see clause 4.2 in TS 38.213 [38]), and autonomously pre-compensate the TTA for the RTT between the UE and the RP as illustrated in Figure 16.14.2.1-1 (see clause 4.3 of TS 38.211 [52]).

The UE shall compute the frequency Doppler shift of the service link, and autonomously pre-compensate for it in the uplink transmissions, by considering UE position and the ephemeris. If the UE does not have a valid GNSS position and/or valid ephemeris and Common TA, it shall not transmit until both are regained.

In connected mode, the UE shall be able to continuously update the Timing Advance and frequency pre-compensation.

The UE may be configured to report Timing Advance during Random Access procedures or in connected mode. In connected mode, event-triggered reporting of the Timing Advance is supported.

Figure 16.14.2.2-1: Void

While the pre-compensation of the instantaneous Doppler shift experienced on the service link is to be performed by the UE, the management of Doppler shift experienced over the feeder link and transponder frequency error is left to the network implementation.

Following this, the UE must know its GNSS position before entering and while being in RRC_CONNECTED mode, especially before transmitting in UL. We therefore conclude that the UE knows its accurate location to properly perform autonomous TA and frequency Doppler shift pre-compensation, otherwise the UE could not even transmit the RRCReconfigurationComplete message.

Based on the current 3GPP specifications, it is ensured that the UE has all the necessary information to derive the referenceLocationReport at any time in RRC_CONNECTED mode.

- [S024] The UE has all the necessary information to derive the referenceLocationReport at any time in RRC_CONNECTED mode.

Reject RIL S024. The UE always provides the initial referenceLocationReport in the RRCReconfigurationComplete message after being configured to report the N closest reference locations in the RRCReconfiguration message.

### H250

[Description]: The closest reference location information in the UAI can be used for both SMTC configuration and gap configuration. Besides, it is preferred that we use “reference location information reporting” because it is different from directly reporting UE location. Similar changes (referring to gap configuration) need to be made to multiple other places.

[Proposed Change]:

2>	if the assisted-SSB-MTC-Config is set to setup:

3>	consider itself to be configured to provide closest reference location information for assisting SMTC and measurement gap configuration in RRC_CONNECTED state in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide closest reference location information for assisting SMTC and measurement gap configuration in RRC_CONNECTED state.

[Comments]:

[Rapp] Regarding the first change, we consider it is incorrect since the UE does not provide a “reference location” but an index/bitmap (whatever RAN2 eventually decides). Measurement gap considerations need to be addressed if inter-frequency is agreed for SMTC enhancements.

We understand the concerns of the company that has raised the RIL. However, the proposed change may lead to a misunderstanding that the UE is providing a reference location, which is a 48-bit IE well defined in the 3GPP specs, where in fact the UE is providing a pointer to a reference location via a bitmap, or an index (or indices). Besides, we think the addition of “closest reference” here is not essential, because the fact that the UE reports information about the closest reference locations is anyway described specifically in Section 5.7.4.3 “Actions related to transmission of UEAssistanceInformation message”.

For RIL H250, reject the addition of “closest reference” to the UAI procedure description since the UE reports a pointer (via index or bitmap).

In addition to SMTC(s), the UE needs a suitable measurement gap configuration to perform neighbour cell measurements for inter-frequency but not for intra-frequency. We thus agree to the addition of “and measurement gap” provided that location-based SMTC selection is extended to the inter-frequency case, otherwise this addition should be removed.

For RIL H250, accept the addition “and measurement gap” if RAN2 agrees location-based SMTC selection applies to inter-frequency.

### V208

[Description]: In the post email discussion, there is an argument on whether a bitmap or a reference location index should be used for the UE reference location report. With index indicating, the network can identify which one location is the nearest one or second-nearest one, based on the listed order in the reporting list. However, we don’t see index is beneficical. The network can configure the parameter N to control the number of locations that can be reported. Such information is sufficient for configuring N SMTCs for UE. The network doesn’t require which location is the nearest one.

[Proposed Change]: RAN2 confirms that bitmap of 6 bit is used for UE reference location report.

[Comments]:

ZTE: We still prefer to use index, and the reason is that even when NW configured UE to report 4, it is possible that NW can still have the possibility to configure UE with less SMTCs, e.g., 2 SMTCs. And report the reference Location in order in such scenarios is beneficial.

[Ericsson] We agree with vivo’s proposal.

[Rapp] The final reporting format be discussed in the next meeting. Please bring contributions addressing signaling such as message size and structure.

We agree with the understanding of the company that has raised the RIL. The network configures the parameter N depending on the UE’s capabilities regarding parallel SMTCs and with the intention of later configuring all the SMTCs associated with the N reported reference locations, not a subset of those SMTCs. The reason is that the network does not know how close/far the UE is to/from those N reference locations that the UE has reported as “closest”. For example, for N=2, the UE may be

- approximately in between the two reference locations, or

- close to one reference location and relatively far from the other one, or

- close to the cell center of the serving cell and thus relatively far from both reference locations.

To avoid unnecessary UE energy consumption for neighbor-cell measurements, it is best to configure the UE with as little SMTCs as necessary. In case 2), it would be sufficient to configure the UE only with the SMTC associated with the one reference location that is closest. In case 3), it might even be ok not to configure any of the corresponding SMTCs (here considering intra-frequency). However, the network cannot distinguish between those three cases. Consequently, the network always needs to configure all the SMTCs associated with the N reported reference locations. This is the case regardless of whether the UE reports the reference locations via bitmap or sorted list of indices. There is thus no advantage to specify referenceLocationReport as a list of indices sorted based on the UE’s distance from the reference locations, while a bitmap is more signaling efficient. If RAN2 were to adopt a list-of-indices-based approach, the parameter N has in fact no purpose, since the network may then always configure the maximum value, which was not RAN2’s intention. We thus agree with the RIL reporter’s proposal to adopt a bitmap-based approach.

- [V208] UE reference location reporting as agreed so far does not enable the network to always configure an optimal (i.e., smallest possible) set of SMTCs, regardless of whether a list-of-indices or a bitmap approach is selected.

Accept RIL V208. The IE referenceLocationReport is a 6-bit bitmap.

### X250

[Description]: As the cell size of neighbor cells can be different according to TR38.821, UE cannot decide which SMTC to select purely based on the closeness of neighbour cells, as UE may be out of coverage of the closest neighbour cell if it happens to be with a small cell size.

[Proposed Change]: Broadcast a distance threshold together with the reference location.

[Comments]:

[Rapp] Cells of different sizes has not yet been considered in NTN deployments, but it may be a reality in multi-orbit scenarios. To be discussed next meeting.

We agree that, in the presence of neighbor cells of different sizes, location-based SMTC selection may not work very well if the reference locations are placed in the cell centers of the neighbor cells. However, the reference locations are defined from serving cell’s perspective and there is no restriction on where to place them. The issue of different cell sizes can be overcome by network implementation. One suitable strategy is to place each reference location on the cell edge between the serving cell and the corresponding neighbor cell. Other suitable strategies exist, so long as each reference location is placed somewhere on a straight line between the center of the serving cell and the center of the respective neighbor cell. There is thus no clear benefit of adding a distance threshold for each of the reference locations. On the contrary, adding a number of distance thresholds creates a significant additional signaling overhead. This is particularly problematic for system information broadcast, where the message sizes are limited and the message scheduling is inflexible, such that adding further information may even result in reducing the coverage.

- [X250] The suggested problem of different cell sizes for location-based SMTC selection can be solved by network implementation.

However, a single (common) distance threshold for all reference locations would be useful to overcome the issue described for RIL V208 (see above). Namely, if the network has configured a common distance threshold applying to all the reference locations, the UE would in the referenceLocationReport not include those reference locations that are farther away than the threshold distance. In other words, the UE may report less than N reference locations in case some of the N closest reference locations are not close enough to be relevant (according to the distance threshold). This helps to avoid unnecessary UE energy consumption for irrelevant neighbor-cell measurements, as the network would not configure the SMTCs associated with the reference locations that were not reported.

- [X250] A common distance threshold for all reference locations would help to unnecessary UE energy consumption for irrelevant neighbor-cell measurements.

For RIL X250, the network can optionally configure one common distance threshold for all reference locations.

### N085

[Description]: A UE may not only support less SMTCs than signalled in SMTC4 and SMTC5, but also only a single periodicity.

[Proposed Change]: Use “or” instead of and for SMTC4 and SMTC5 to be supported

smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports only a single periodicity or less SMTCs than what is included in smtc4list or smtc5list, it is up to the UE to select which SMTCs to consider. The total number of configurable SMTCs across smtc4list and smtc5list is 6. The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2. If an entry in smtc5list is present but the pci-List, periodicity and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list, if present.

[Comments]:

[Rapp] A UE that only supports one periodicity can safely ignore smtc5list in SIB2. In the MO, provided network has UE capabilities, this will not be configured. We agree with the intention that this should be somewhat captured.

We share the concerns of the company that has raised the RIL but prefer to address it differently. Location-based SMTC selection and the additional SMTC periodicity are two separate features, so they should be kept separate as much as possible.

First, a UE that does not support the additional SMTC periodicity must apply the legacy SMTC periodicity, which is given by smtc, or smtc1 respectively. Moreover, as discussed in [3], the legacy SMTC periodicity must be equal to (or larger than) the largest SSB periodicity among serving cell and intra-frequency neighbor cells, and it is not an issue to monitor SSB transmissions at a certain period using an SMTC window of a longer period, i.e., measuring less frequently. Consequently, the UE can set up SMTC windows based on the legacy SMTC periodicity and any of the SMTC offsets given in smtc5list. The offset of an entry in smtc5list can be considered for location-based selection even if the entry is associated with an additional periodicity.

- [N085] Location-based SMTC selection should be applicable independent of the additional SMTC periodicity. A UE that does not support the additional SMTC periodicity may select any entry of smtc5list and apply the legacy SMTC periodicity as provided in smtc.

The proposed change may be understood as: A UE may ignore an entry in smtc5list if the entry is associated with the additional SMTC periodicity and if the UE does not support the feature. To avoid that misunderstanding, we propose adopting the following changes instead:

smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in smtc4list and smtc5list, it is up to the UE to select which SMTCs to consider. If a UE does not support an additional SMTC periodicity, it ignores the periodicity provided in smtc5list, if any, and instead applies the periodicity provided in smtc. The total number of configurable SMTCs across smtc4list and smtc5list is 6. The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2. If an entry in smtc5list is present but the pci-List, periodicity and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list, if present.

For RIL N085, introduce the following clarification: “If a UE does not support an additional SMTC periodicity, it ignores the periodicity provided in smtc5list, if any, and applies the periodicity provided in smtc.”

### A200

[Description]: The current assumption from CR rapporteur is SMTC in SIB2 is associated with reference location in SIB19, and it is used for serving cell measurement. We think it is not always the case. Note that when NW provides smtc in SIB2, it should associate with the largest SSB periodicity of serving and neighbor cell. For example, if serving cell has a SSB periodicity of 160ms, and neighbor cell has a SSB periodicity of 20ms, the smtc in SIB2 has to be with 160ms periodicity. That is, it is actually for neighbor cell measurement. And smtc5list can provide another periodicity of 20ms for serving cell measurement.

Therefore, we would like to clarify that there should be an explicit reference location associated with legacy smtc configured in SIB2.

[Proposed Change]: Allow smtc in SIB2 to associate with a reference location in RefLocList-r19.

[Comments]:

[Rapp] In our understanding, smtc is used for the serving cell. However, this needs to be clarified in RAN2 first.

We agree with the observations made by the company that has raised the RIL. Namely, if the legacy SMTC (i.e., smtc in SIB2) applies to the serving cell, the network may not be able to configure the SMTC periodicity (for that legacy SMTC) equal to the SSB periodicity of the serving cell. This is true if the SSB periodicity of at least one intra-frequency neighbor cell is larger than the SSB periodicity of the serving cell, as clarified by RAN4. However, it does not break RAN4 requirements to configure an SMTC periodicity that is larger than the SSB periodicity of an associated cell. Nevertheless, one may argue the downside is that the UE may not benefit from the denser SSB transmissions in the serving cell, provided that the UE measures on the serving cell using SMTC.

- [A200] If the legacy SMTC (i.e., smtc in SIB2) applies to the serving cell, the UE may not benefit from denser SSB transmissions in the serving cell (provided that the UE measures on the serving cell using SMTC).

In our understanding, the legacy SMTC always applies to the serving cell and should therefore always be used, i.e., the legacy SMTC is not applicable to location-based selection. RAN2 first needs to clarify whether this is true, and if so, consider the potential 3GPP spec impact (also across RAN groups) for overcoming this restriction. If already in legacy smtc in SIB2 does not necessarily apply to the serving cell or if the spec impact is limited, the proposed solution is a good way forward.

For RIL A200, clarify that legacy SMTC (smtc in SIB2) applies to the serving cell and is not used for location-based SMTC selection.

### Q200

[Description]: Currently only fixed cell is supported as no moving reference location is defined. Either clarification should be added or we need to agree this is not supported in moving cell.

[Proposed Change]: in FD, Add clarification that for moving cell the reference location is associated with serving cell epoch time.

[Comments]:

In our understanding, location-based SMTC selection is not necessary in the case of Earth-moving cells. This is because the coverage areas of the cells are moving much faster than the UE may possibly move. Consequently, there is only one (or few) neighbor cell(s) that the UE may reselect, or be handed over to; those neighbor cell(s) whose coverage areas are moving toward the UE at a very high speed. There is thus no reason for the UE to measure on any other neighbor cells, since the coverage areas of those other neighbor cells are moving away from the UE at very high speed. Moreover, the network is aware of the NTN deployment and knows which one (or few) neighbor cell(s) the UE needs to measure on and hence which SMTC(s) the UE needs, and this applies regardless of the UE’s location within the serving cell’s coverage area. It is thus best to explicitly exclude the Earth-moving cell scenario for location-based SMTC selection in Rel-19, also to avoid potential future maintenance work regarding this.

- [Q200] Location-based SMTC selection is not necessary for Earth-moving cells, because only one or few neighbor cells are relevant for measurement, as the coverage areas of the other neighbor cells are moving away from the UE at very high speed, regardless of the UE’s location.

Reject Q200 Earth-moving cell scenario is down-prioritized for location-based SMTC selection in Rel-19.

### C008

[Description]: Same as RIL C006.

[Proposed Change]: Modify “The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2.” as “The total number of different SMTC periodicities across smtc and smtc5list is 2.”. Illustrate that “If smtc5list is present, the two SMTC periodicities capable UE ignores smtc4list.”.

smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in smtc4list and smtc5list, it is up to the UE to select which SMTCs to consider. The total number of configurable SMTCs across smtc4list and smtc5list is 6. The total number of different SMTC periodicities across smtc and smtc5list is 2. If an entry in smtc5list is present but the pci-List, periodicity and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list, if present. If smtc5list is present, the two SMTC periodicities capable UE ignores smtc4list.

[Comments]:

[Rapp] The first change is fine. In our view, the second change is incorrect. RAN2 has agreed delta signalling with smtc4list to precisely avoid configuring two lists (smct4list and smtc5list) with the same information (pci-list and offset).

[Qualcomm] The design here is to apply delta configuration so current text is correct that total number does not exceed, may be should say 7. If smtc5 is present, the smtc4 shall be present. But if smtc4 is present, it is not necessary smtc5 to be included.

We follow the thinking of the company that has raised the RIL in the sense that there is no SMTC periodicity in smtc4list. The field description needs to be updated, so that it does not mention smtc4list in the context of SMTC periodicities. We hence agree with the first change that is proposed for RIL C008. However, the second change (the addition of “If smtc5list is present, the two SMTC periodicities capable UE ignores smtc4list.”) is not needed, since smtc4list is anyway not associated with any additional SMTC periodicity. The second change is actually incorrect, since certain contents of smtc4list may be reused to replace absent IEs in smtc5list (delta-signaling).

- [C008] The first change is correct. The second change, however, is incorrect and in fact prevents delta signaling for smtc5list as agreed by RAN2.

For RIL C008, agree to the first change (the removal of “smtc4list”), but reject the second change (the addition of “If smtc5list is present, the two SMTC periodicities capable UE ignores smtc4list.”).

### E010

[Description]: The current solution for UE-assisted SMTC selection in RRC_CONNECTED mode requires two RRC reconfigurations of the UE, for each UE upon each transition to RRC_CONNECTED mode. This may cause significant signaling overhead and pose a scalability issue for NTN, where cells are large and radio resources are extremely scarce. The additional, second RRC reconfiguration is needed, because at the time when the usual, first RRC reconfiguration is performed, the network does not yet know which are the relevant SMTCs to be configured for the UE.

[Proposed Change]: The above problem can be avoided for UEs transitioning from RRC_INACTIVE to RRC_CONNECTED mode by allowing the UEs to report the N closest reference locations, i.e., by allowing them to add referenceLocationReport, in the RRCResumeComplete message based on prior UE configuration. For UEs transitioning from RRC_INACTIVE to RRC_CONNECTED mode, AS security is enabled after reception of the RRCResumeRequest message by the network. Hence, the RRCResume and RRCResumeComplete message are subject to the same protection (i.e., cyphering and integrity protection) as the RRCReconfiguration and RRCReconfigurationComplete message.

This change enables the network to efficiently resume RRC connections of UEs (without RRC reconfiguration).

[Comments]:

[Qualcomm] This probably can be discussed to do it properly as the network may need to add new reference location list in RRCResume. Otherwise after being inactive so long, just reporting outdated reference location is not useful. Please change to ToDO.

Regarding the comment, it is true that if the UE has moved to a different cell (e.g., due to UE or satellite movement), the reference location set stored by the UE may be outdated. Nonetheless, we note that in a typical NTN quasi-Earth-fixed cell deployment, it is highly unlikely that the reference location set will change due to satellite movement since the set will be configured by the operator (e.g., OAM) during the deployment phase based on satellite footprint characteristics. In any case, similar to other UAI reports, the UE releases assisted-SSB-MTC-Config upon initiating RRCResumeRequest see section 5.3.13.2. Therefore, if the network would like to obtain this report, it would need to include it in the RRCResume message. The network can optionally include a new set of reference locations in the configuration or refer to the list in system information. This prevents the UE from using a potentially outdated configuration.

Accept E010. Take the TP in Annex as a baseline.

## 2.2	MBS-related RILs

### C003

[Description]: We prefer to clarify how UE determines the service area if the Target Service Area in the USD or the ISA in SIBXX are not totally aligned. Since the UE is not expect to receive MBS service outside the service area, it would be better for the UE to get a precise service area scope.

[Proposed Change]: Add the following NOTE to clarify the relationship between the Target Service Area in the USD and the ISA in SIBXX.

NOTE: If the service area information is broadcast in an NTN cell, the UE ignores the service area information in USD

[Comments]:

[Rapp] Even thought it is unlikely that both ways of delivering the information may be configured simultaneously, RAN2 should further discuss which one is prioritized (if needed).

[Qualcomm] Which ever area UE wants to use should work pointing to (may be not exactly same) almost same region. Otherwise this should be error case.

It is highly unlikely that a network operator would select both ways to deliver the same information to the UE: ISA and Service Announcement. Provided the Service Announcement is a mandatory feature of MBS, we understand this should have prevalence. In any case, the evaluation of whether the UE is inside or outside the ISA (or TSA) is up to implementation. Therefore, even if this problem were to happen due to network misconfiguration, this can be simply solved by implementation.

Reject C003. How to use ISA (or TSA) is up to UE implementation, if both were to be simultaneously configured.

### E011

[Description]: Following RAN2 agreements, a UE may initiate the establishment or release when it enters/leaves the ISA of the MBS service in question. However, the text captured in 5.9.3.2 is not sufficient to ensure these limitations. For instance, there are some clauses such as “upon start of the MBS session” that would allow a UE to acquire establish the MRBs even outside the ISA..

[Proposed Change]: It is simpler to include the geofencing limitation in the general configuration of broadcast MRBs (section 5.9.3.1) so that it applies both to initial establishment/release but also to updates. Here an example:

#### 5.9.3.1	General

The broadcast MRB configuration procedure is used by the UE to configure PDCP, RLC, MAC and the physical layer upon starting and/or stopping to receive a broadcast MRB transmitted on MTCH, or upon modification of a configuration of a broadcast MRB received by the UE. The procedure applies to MBS capable UEs that are interested to receive or that are receiving an MBS broadcast service that are in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED with an active BWP with common search space configured by searchSpaceMTCH or searchSpaceMCCH and are located within the Intended Service Area associated with the MBS service, if any.

[Comments]:

[Qualcomm]We suggest this to change to ToDO. This should not be applicable to legacy UEs. It should be applicable to only UEs capable of supporting ISA and this is not clear to us.

Following previous RAN2 agreements, it should be captured how the ISA applies to the configuration. There are some clauses such as “upon start of the MBS session” that would allow a UE to acquire establish the MRBs even outside the ISA. Therefore, a simple geofencing limitation in the general configuration of broadcast MRBs (section 5.9.3.1) so that it applies both to initial establishment/release but also to updates

Although we understand Qualcomm’s concern, this should not be an issue since the procedural text does not affect legacy, nor applies to non-ISA capable UEs (e.g., terrestrial UE). These devices will not understand the ISA signaling and will not execute this specific procedural option. In RRC, there are some other examples where ISA is referred while there is no explicit mention to ISA NTN capable UE, for instance, in section 5.9.3.2 “upon leaving the ISA(s) associated with an MBS broadcast service the UE is interested in.”

Agree E011.

## 2.3	RAN1 parameters RILs

### Z256

[Description]: SearchSpaceLinkingIdCE is only used to linked two SS, while searchSpaceId for the additional SS for pagingSearchSpace is still needed to be added in PDCCH-ConfigCommon

[Proposed Change]: Add pagingSearchSpaceExt in PDCCH-ConfigCommon per below:

PDCCH-ConfigCommon information element

-- ASN1START

-- TAG-PDCCH-CONFIGCOMMON-START

PDCCH-ConfigCommon ::=              SEQUENCE {

controlResourceSetZero              ControlResourceSetZero                                  OPTIONAL,   -- Cond InitialBWP-Only

commonControlResourceSet            ControlResourceSet                                      OPTIONAL,   -- Need R

searchSpaceZero                     SearchSpaceZero                                         OPTIONAL,   -- Cond InitialBWP-Only

commonSearchSpaceList               SEQUENCE (SIZE(1..4)) OF SearchSpace                    OPTIONAL,   -- Need R

searchSpaceSIB1                     SearchSpaceId                                           OPTIONAL,   -- Need S

searchSpaceOtherSystemInformation   SearchSpaceId                                           OPTIONAL,   -- Need S

pagingSearchSpace                   SearchSpaceId                                           OPTIONAL,   -- Need S

ra-SearchSpace                      SearchSpaceId                                           OPTIONAL,   -- Need S

...,

[[

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                               SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                              SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT          SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT               SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT                                                   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}                                                                                           OPTIONAL    -- Cond OtherBWP

]],

[partailly omitted]

[[

applyIndicatedTCI-State-r18            ENUMERATED {first, second, both, none}                OPTIONAL,   -- Cond FollowUTCI

commonSearchSpaceListExt-r18           SEQUENCE (SIZE(1..4)) OF SearchSpaceExt-v1800         OPTIONAL,   -- Need R

searchSpaceMulticastMCCH-r18           SearchSpaceId                                         OPTIONAL,   -- Need R

searchSpaceMulticastMTCH-r18           SearchSpaceId                                         OPTIONAL    -- Need S    ]],

[[

pagingSearchSpaceExt                   SearchSpaceId                                           OPTIONAL,   -- Cond PagingSearchSpace

]]

}

-- TAG-PDCCH-CONFIGCOMMON-STOP

-- ASN1STOP

PDCCH-ConfigCommon field descriptions

pagingSearchSpace, pagingSearchSpaceExt ID(s) of the search space(s) for paging (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive paging in this BWP (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial downlink BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, an (e)RedCap UE in RRC_INACTIVE while SDT procedure is ongoing and T319a is not running, if CG-SDT is selected and if extended CG-SDT periodicity is configured (i.e. cg-SDT-PeriodicityExt is configured), or an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE while SDT procedure is not ongoing, shall monitor paging in the initial DL BWP that includes CORESET#0.

| Conditional Presence | Explanation |
| --- | --- |
| FollowUTCI | The field is absent if the field followUnifiedTCI-State is present or if more than one value for the field coresetPoolIndex is configured in controlResourceSet for the same bandwidthpart. Otherwise, it is optionally present, Need R. |
| InitialBWP-Only | If SIB1 is broadcast the field is mandatory present in the PDCCH-ConfigCommon of the initial BWP (BWP#0) in ServingCellConfigCommon except it is the RedCap-specific initial BWP not including CD-SSB and the entire CORESET#0 in which case the field is absent, Need R; it is absent in other BWPs and when sent in system information. If SIB1 is not broadcast and there is an SSB associated to the cell, the field is optionally present, Need M, in the PDCCH-ConfigCommon of the initial BWP (BWP#0) in ServingCellConfigCommon (still with the same setting for all UEs). In other cases, the field is absent. |
| InitialBWP-Paging | This field is optionally present, Need R, if this BWP is the initialDownlinkBWP or initialDownlinkBWP-RedCap including CD-SSB and the entire CORESET#0, and pei-Config is configured in DownlinkConfigCommonSIB. Otherwise, this field is absent. |
| OtherBWP | This field is optionally present, Need R, if this BWP is not the initialDownlinkBWP and pagingSearchSpace is configured in this BWP. Otherwise this field is absent. |
| PagingSearchSpace | This field is optional present, need R, if pagingSearchSpace is present. Otherwise this field is absent. |

In our understanding, this RIL may come from a misinterpretation of RAN1 agreements and the introduction of search space (SS) linking (an existing feature since Rel-17) for the purpose of DL CE in NTN Rel-19. The fact that two SS of the same type are linked involves that the UE needs to monitor both. Regardless the type of SS (e.g., paging, random access, etc.), the UE uses searchSpaceLinkingId-CE to establish the connection between. Therefore, if a paging SS is linked to another SS, it is evident that the second should be also monitored for and the extra signaling proposed in this RIL is not necessary. Further details should be present in RAN1 and can be simply pointed out from RRC with a reference.

Reject Z256.

## 2.4	LTE to NR NTN RILs

### E801

[Description]: Provided the large size of NTN cells compared to TN cells. We think there is a low probability that a TN cell will see many satellites in the same carrier frequency operating within its cell coverage. Therefore, we wonder why we introduce the flexibility in signalling to configure multiple Satellite IDs per carrier frequency in SIB24 and RRCConnectionRelease. We understand it should be enough with one, especially in the dedicated message. Otherwise, we fear that the UE may need to try multiple combinations of ephemeris (propagation delay and doppler compensation) to scan a single neighbour carrier frequency which could have an impact on performance.

[Proposed Change]:

Use a single satellite ID per carrier frequency instead of a sequence.

CarrierFreqNR-v19xy ::=		SEQUENCE {

satAssistanceInfoList-r19		SatelliteId-r18	OPTIONAL				-- Need OR

}

[Comments]:

Rapporteur’s comments: Thanks for raising this RIL. This RIL was raised during post email discussion phase after RAN2#131. We can further discuss based on companies’ contributions.

The typical size of an NTN cell for a LEO satellite is about 50km radius. This footprint is considerably larger than a typical TN cell. Therefore, in most scenarios, a UE in a TN cell will only see a single satellite providing coverage in a certain carrier frequency to the area where it is located. In other words, it is unlikely that multiple satellites with serve the same area in the same frequency.

In the current specification, multiple Satellite IDs per carrier frequency in have been configured in SIB24 and RRCConnectionRelease. In addition to potential interference and coexistence issues, we consider this may be problematic if the UE needs to try multiple satellites, i.e., combinations of carrier frequency and ephemeris (e.g., propagation delay and doppler compensation) to perform measurements and potentially execute a redirection mobility procedure. Thus, unless there is any specific technical reason, we consider it sufficient and less prone to errors to limit the number of satellite IDs to 1 in SIB24 and RRCConnectionRelease.

Accept E801. Limit the number of satellites IDs to 1 in SIB24 RRCConnectionRelease.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	[S024] It is beneficial for UE and network that the UE provides referenceLocationReport in the RRCReconfigurationComplete message, which, so far, was believed to be always possible for the initial report after the UE has been configured in the RRCReconfiguration message.

Observation 2	[S024] Since the UE needs accurate location information for accessing and maintaining the connection to an NTN cell, it is uncertain the why referenceLocationReport would not be timely available in RRC_CONNECTED mode.

Observation 3	[S024] The UE has all the necessary information to derive the referenceLocationReport at any time in RRC_CONNECTED mode.

Observation 4	[V208] UE reference location reporting as agreed so far does not enable the network to always configure an optimal (i.e., smallest possible) set of SMTCs, regardless of whether a list-of-indices or a bitmap approach is selected.

Observation 5	[X250] The suggested problem of different cell sizes for location-based SMTC selection can be solved by network implementation.

Observation 6	[X250] A common distance threshold for all reference locations would help to unnecessary UE energy consumption for irrelevant neighbor-cell measurements.

Observation 7	[N085] Location-based SMTC selection should be applicable independent of the additional SMTC periodicity. A UE that does not support the additional SMTC periodicity may select any entry of smtc5list and apply the legacy SMTC periodicity as provided in smtc.

Observation 8	[A200] If the legacy SMTC (i.e., smtc in SIB2) applies to the serving cell, the UE may not benefit from denser SSB transmissions in the serving cell (provided that the UE measures on the serving cell using SMTC).

Observation 9	[Q200] Location-based SMTC selection is not necessary for Earth-moving cells, because only one or few neighbor cells are relevant for measurement, as the coverage areas of the other neighbor cells are moving away from the UE at very high speed, regardless of the UE’s location.

Observation 10	[C008] The first change is correct. The second change, however, is incorrect and in fact prevents delta signaling for smtc5list as agreed by RAN2.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Reject RIL S024. The UE always provides the initial referenceLocationReport in the RRCReconfigurationComplete message after being configured to report the N closest reference locations in the RRCReconfiguration message.

Proposal 2	For RIL H250, reject the addition of “closest reference” to the UAI procedure description since the UE reports a pointer (via index or bitmap).

Proposal 3	For RIL H250, accept the addition “and measurement gap” if RAN2 agrees location-based SMTC selection applies to inter-frequency.

Proposal 4	Accept RIL V208. The IE referenceLocationReport is a 6-bit bitmap.

Proposal 5	For RIL X250, the network can optionally configure one common distance threshold for all reference locations.

Proposal 6	For RIL N085, introduce the following clarification: “If a UE does not support an additional SMTC periodicity, it ignores the periodicity provided in smtc5list, if any, and applies the periodicity provided in smtc.”

Proposal 7	For RIL A200, clarify that legacy SMTC (smtc in SIB2) applies to the serving cell and is not used for location-based SMTC selection.

Proposal 8	Reject Q200 Earth-moving cell scenario is down-prioritized for location-based SMTC selection in Rel-19.

Proposal 9	For RIL C008, agree to the first change (the removal of “smtc4list”), but reject the second change (the addition of “If smtc5list is present, the two SMTC periodicities capable UE ignores smtc4list.”).

Proposal 10	Accept E010. Take the TP in Annex as a baseline.

Proposal 11	Reject C003. How to use ISA (or TSA) is up to UE implementation, if both were to be simultaneously configured.

Proposal 12	Agree E011.

Proposal 13	Reject Z256.

Proposal 14	Accept E801. Limit the number of satellites IDs to 1 in SIB24 RRCConnectionRelease.

# 4	References

R2-2501415, Report of [AT129][304][R19 NR NTN] LS to RAN1 on DL coverage enhancements, Huawei, RAN2#129, February 2025.

RP-243300, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, 3GPP TSG RAN#106, Dec. 2024.

R2-2506154, Discussion on Reply LS on SMTC enhancements, Ericsson, RAN2#131, Bengaluru, India, August 25–29, 2025.

# 5	Annex

## 5.1	Relevant agreements for DL CE

During RAN2#130, the following agreements related to DL coverage enhancements were captured:

WA: We introduce a mechanism to assist the NW to configure the SMTCs in connected mode

RAN2 to agree that the maximum configured SMTCs per frequency for idle/inactive UEs is 6.

To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications)

We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported

We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

- Alt 1: UE provide the closest N reference locations/neighbor cells to network.

- Alt 2: UE reports the selected SMTCs from configured SMTC set to network.

No enhancement for cell selection/reselection for DL CE is pursued (can re-discuss this if we will finally introduce a new barring scheme)

During RAN2#131, the following agreements related to SMTC enhancements were captured:

The maximum number configured SMTCs for idle/inactive is 7 and it also includes the SMTC of the serving cell (This updates a previous decision to have a maximum of 6 STMCs).

We introduce a capability to support STMC enhancements to FR1 (FFS on FR2 depending on RAN4 progress)

RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band). If RAN4 cannot finish the corresponding RRM requirement design, we can introduce some statement in the specs in the future to limit the configuration to one periodicity only.

RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements

In connected mode the NW will only configure up to 4 STMC.

UE Assistance Information message will include information to the NW to (re)configure the most relevant SMTCs for the area where the UE is located

The procedure for UE assisted SMTC configuration in connected mode is follows:

- - For a UE indicating the relevant capability (FFS the name of the capability and it this is merged with some other capability) the network configures the UE via reconfiguration message with a list of reference locations and a parameter N, thereby configuring the UE to report the N closest reference locations (FFS whether also a distance threshold is indicated). If the list of reference locations is not provided, the list of reference locations provided in system information is reused.

- - The UE reports an indication of the N closest reference locations via UE assistance information, e.g. bitmap or list of indices of the locations.

- - The UE can report the N closest reference locations via the RRCReconfigurationComplete message.

- - The network configures the relevant SMTC(s). The maximum number of configured SMTCs is according to legacy UE capability in terms of SMTC support (same as legacy).

- - The UE provides an update on the set of the N closest reference locations, if changed, via UE assistance information. Upon receiving an update, the network reconfigures the relevant SMTC(s), if changed.

## 5.2	TS 38.331 TP for E010

#### 5.3.13.4	Reception of the RRCResume by the UE

[…]

1>	if the RRCResume message includes the assisted-SSB-MTC-Config:

2>	if the assisted-SSB-MTC-Config is set to setup:

3>	consider itself to be configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state;

[…]

2>	if the UE is configured in this RRCResume message to provide location information for assisted SMTC configuration in RRC_CONNECTED state:

3>	include referenceLocationReport;

[...]

#### –	RRCResume

RRCResume-v1900-IEs ::=             SEQUENCE {

assisted-SSB-MTC-Config-r19             SetupRelease {Assisted-SSB-MTC-Config-r19}                      OPTIONAL, -- Need M

nonCriticalExtension                SEQUENCE {}                                                     OPTIONAL

}

#### –	RRCResumeComplete

RRCResumeComplete-v1900-IEs ::=         SEQUENCE {

applicabilityReportList-r19             ApplicabilityReportList-r19                                             OPTIONAL,

referenceLocationReport-r19             BIT STRING (SIZE (6))                                                   OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                                             OPTIONAL

}