3GPP TSG-RAN WG2 meeting #132	R2-2508002

Agenda Item:	2.2

Source: 	ETSI MCC

Title:		Report of 3GPP TSG RAN WG2 meeting #131bis, Prague, Czech Republic

Report of 3GPP TSG RAN WG2 meeting #131bis

Prague, Czech Republic

13 - 17 October, 2025

Document for: Approval

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

# Contents

Organisation of the meeting	7

Statistics/Executive Summary	7

General	8

1	Opening of the meeting	8

1.1	Call for IPR	8

1.2	Network usage conditions	8

1.3	Other	8

2	General	9

2.1	Approval of the agenda	9

2.2	Approval of the report of the previous meeting	9

2.3	Reporting from other meetings	9

2.4	Instructions	9

2.5	Others	11

3	Incoming liaisons	11

4	EUTRA Rel-17 and earlier	11

4.1	EUTRA corrections Rel-17 and earlier	11

4.3	Positioning corrections Rel-16 and earlier	13

5	NR Rel-15 and Rel-16	13

5.1	Common	13

5.1.1	Stage 2 and Organisational	14

5.1.2	User Plane corrections	14

5.1.2.1	MAC	14

5.1.2.2	RLC PDCP SDAP BAP	14

5.1.3	Control Plane corrections	14

5.1.3.1	NR RRC	14

5.1.3.2	UE capabilities	16

5.1.3.3	Other	16

5.3	NR Positioning Support	16

6	NR Rel-17	16

6.1	Common	16

6.1.1	Stage 2 and Organisational	17

6.1.2	User Plane corrections	17

6.1.3	Control Plane corrections	17

6.1.3.1	NR RRC	17

6.1.3.2	UE capabilities	20

6.1.3.3	Other	20

6.3	NR positioning enhancements	20

7	NR Rel-18	21

7.0	Common	21

7.0.1	UE Capabilities	21

7.0.2	Rel-18 corrections	21

7.0.2.1	RACH-less HO	21

7.0.2.2	NR network-controlled repeaters	21

7.0.2.3	NR support for UAV	21

7.0.2.4 	Mobile Terminated Small Data Transmission	21

7.0.2.5	IDC enhancements for NR and MR-DC	21

7.0.2.6	Mobile IAB (Integrated Access and Backhaul) for NR	21

7.0.2.7	Timing Resiliency and URLLC Enh	22

7.0.2.8	Enhanced support of reduced capability NR devices	22

7.0.2.9	Further NR coverage enhancements	22

7.0.2.10	Network energy savings for NR	22

7.0.2.11	Further enhancement of data collection for SON MDT in NR and EN-DC	22

7.0.2.12 Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR	23

7.0.2.13 NR MIMO evolution	23

7.0.2.14 Enhancements of NR Multicast and Broadcast Services	23

7.0.2.15 Enhancement on NR QoE management and optimizations for diverse services	23

7.0.2.16	XR Enhancements for NR	24

7.0.2.17	NR NTN enhancements	24

7.0.2.18	IoT NTN enhancements	26

7.0.2.19	Enhanced NR Sidelink Relay	26

7.0.2.20	NR Sidelink evolution	27

7.0.2.21	Expanded and improved NR positioning	27

7.0.2.22	Further NR mobility enhancements	30

7.0.2.23	TEI18	32

7.0.2.24	Others	32

8	NR Rel-19	33

8.0	General	33

8.0.1	ASN.1 Review	33

8.0.2	Other	36

8.1	AI/ML for NR air interface	37

8.1.1	Organizational	37

8.1.2	Functionality based LCM	41

8.1.2.1	LCM for NW-sided model for Beam Management use case	41

8.1.2.2	LCM for UE-sided model for Beam Management use case	41

8.1.2.3	LCM for Positioning use case	47

8.1.3	NW side data collection	47

8.4	Low-power wake-up signal and receiver for NR (LP-WUS/WUR)	56

8.4.1	Organizational	56

8.4.2	RRC issues	58

8.4.3	MAC issues	63

8.4.4	Other issues	64

8.5	Network Energy Saving Enh.	67

8.5.1	Organizational	67

8.5.2	Control plane	69

8.5.3	User plane	78

8.6	Mobility Enhancement Ph4	79

8.6.1	Organizational	80

8.6.2	Control plane	81

8.6.3	User plane	86

8.7	XR Enhancements Ph3	90

8.7.1	Organizational	90

8.7.2	RRC corrections	92

8.7.3	User plane corrections	94

8.7.4	Other corrections	99

8.8	NTN for NR Ph3	100

8.8.1	Organizational	100

8.8.2	RRC corrections	103

8.8.3	Idle mode corrections	111

8.8.4	Other corrections	112

8.8.5	LTE to NR NTN mobility corrections	112

8.9	IoT NTN Ph3	113

8.9.1	Organizational	114

8.9.2	RRC corrections	116

8.9.3	MAC corrections	121

8.9.4	Other corrections	123

8.10	SON/MDT Ph4	126

8.10.1	Organizational	126

8.10.2	Papers related to RILs	127

8.10.3	Other	130

8.11	Evolution of NR duplex operation: Sub-band full duplex (SBFD)	132

8.11.1	Organizational	132

8.11.2	MAC issues	132

8.11.3	Other aspects	134

8.12	NR MIMO Phase 5	137

8.12.1	Organizational	137

8.12.2	MAC issues	137

8.12.3	Others	141

8.13	NR sidelink multi-hop relay	144

8.13.1	Organizational	144

8.13.2	Control plane	145

8.13.3	User plane corrections	156

8.13.4	Others	158

8.14	Additional topological enhancements	158

8.15	NavIC L1 SPS A-GNSS support	158

8.16	BDS B2b in A-GNSS	159

8.17	IoT-NTN TDD mode	159

8.18	LTE-based 5G Broadcast	162

8.18.1	Organizational	162

8.18.2	RRC corrections	162

8.18.3	Other corrections	162

8.19	TEI19	163

8.19.1	RAN2-led	163

8.19.2	Other WG-led	168

8.20	NR Others	169

8.20.1	RAN4	169

8.20.2	Other WGs	174

9	NR Rel-20	179

9.1	AI/ML for PHY Ph2	179

9.2	Ambient IoT Ph2	179

9.2.1	Organizational	179

9.2.2	Topology 2	179

9.3	AI/ML for mobility	183

9.4	Mobility Enh Ph5	183

9.5	XR Enhancements Ph4	183

9.6	SON/MDT Ph5	183

9.7	IoT NTN Ph4	183

9.7.1	Organizational	183

9.7.2	Other	185

9.8	E-UTRA TN to NR NTN HO	190

10	6GR Rel-20 - Study on 6G Radio Access Technology	190

10.1	Organizational	190

10.2	General aspects	192

10.3	Radio protocol architecture	204

10.3.2	Control plane	212

10.3.3	Common User plane and Control plane	223

10.4	Mobility	236

11	Breakout session reports	241

11.1	Session on R18 and R19 Mobility	241

11.2	Session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others	241

11.3	Session on NES, NR NTN and IoT NTN	241

11.4	Session on positioning and sidelink relay	242

11.5	Session on XR and LTE-based 5G Broadcast	242

11.6	Session on maintenance and SON/MDT	242

Closing of the meeting	242

Annex A:	List of participants	242

Annex B:	List of Tdocs	242

Annex C:	Incoming liaison statements	242

Annex D:	Outgoing liaison statements	247

Annex E:	List of agreed in principle CRs	248

Annex F:	List of email discussions during the meeting	252

Annex G:	Post-meeting email discussions after RAN2#131bis	253

Guidelines for email discussions:	253

Deadlines	253

Short email discussions, Deadline Oct. 24th, 10:00 UTC	253

Long email discussions, for R2-132, Oct. 31st 10:00 UTC (unless otherwise stated)]	255

Annex H:	History	257

# Organisation of the meeting

Meeting:	3GPP TSG RAN2#131bis

Meeting location:	Prague, Czech Republic

Duration:	13 - 17.10.2025

Host:	ETSI

TSG RAN WG2 Chair	Diana Pani (InterDigital) (diana.pani@interdigital.com)

TSG RAN WG2 Vice chair:	Kyeongin Jeong (Samsung) (kyeongin.j@samsung.com)

TSG RAN WG2 Vice chair:	Erlin Zeng (CATT) (erlin.zeng@catt.cn)

TSG RAN WG2 MCC Support:	Juha Korhonen (ETSI MCC) (juha.korhonen@etsi.org)

Email reflector:	3GPP_TSG_RAN_WG2@LIST.ETSI.ORG

Technical documents:	ftp://ftp.3gpp.org/tsg_ran/WG2_RL2/TSGR2_131bis/Docs

Next meetings:	TSG RAN2#132	17 - 21.11.2025, Dallas, USA

TSG RAN2#133	09 - 13.02.2026, Gothenburg, Sweden

# Statistics/Executive Summary

TSG RAN2#131bis was a normal face-to-face meeting, with a possibility for one-way remote access.

The topics discussed were:

-	NR Rel-19 ASN.1 Review, AI/ML for NR air interface, Ambient IoT, TEI19, AI/ML for PHY Ph2, 6GR Rel-20 - Diana Pani (Chair)

-	Further NR mobility enhancements, Mobility Enhancement Ph4 - Antonino Orsino

-	NR MIMO evolution, Low-power wake-up signal and receiver for NR (LP-WUS/WUR), Evolution of NR duplex operation: Sub-band full duplex (SBFD), NR MIMO Phase 5, NR Others - Erlin Zeng (VC)

-	EUTRA corrections Rel-17 and earlier, UE capabilities, NR NTN enhancements, Network Energy Saving Enh, NTN for NR Ph3, IoT NTN Ph3, IoT-NTN TDD mode, TEI19, NR Others, IoT NTN Ph4  - Sergio Parolari

-	NR Rel-17 RRC, Enhanced NR Sidelink Relay, Expanded and improved NR positioning, NR sidelink multi-hop relay, TEI19 - Nathan Tenny

-	XR Enhancements for NR, XR Enhancements Ph3, LTE-based 5G Broadcast, TEI19 - Dawid Koziol

-	EUTRA corrections Rel-17 and earlier, NR Rel-15, Rel-16 and Rel-17, RACH-less HO, Mobile IAB for NR, Network energy savings for NR, Further enhancement of data collection for SON MDT in NR and EN-DC, Enhancement on NR QoE management and optimizations for diverse services, TEI18, SON/MDT Ph4 - Mattias Bergström

The statistics from this meeting are:

-	460 f2f participants

-	1142 Tdoc numbers allocated with 1131 available contributions. (See the attached tdoc list)

-	61 incoming liaison statements, which were all noted.

-	20 outgoing liaison statements

-	68 at-meeting email discussions

-	37 email approvals/discussions scheduled after the RAN2#131 meeting (18 short and 19 long), see Annex G for details.

Number of CRs or draftCRs submitted: 266. Out of these, 80 were agreed in principle. See Annex E for details.

# General

This meeting was an ordinary meeting and had full decision power, i.e. full decision power to make agreements and approvals according to RAN WG2 terms of reference, without any need to ratify decisions at a later RAN2 or other meeting.

# 1	Opening of the meeting

## 1.1	Call for IPR

The attention of the delegates of this Working Group is drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of. The delegates were asked to take note that they were hereby invited: - to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP. - to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Statement and the Licensing declaration forms (https://www.etsi.org/images/files/IPR/etsi-ipr-form.doc)

NOTE:	IPRs may be declared to the Director-General or Chairman of the SDO, but not to the RAN WG2 Chairman.

## 1.2	Network usage conditions

1/ 	To avoid email system overload, please don’t attach files and documents to emails e.g. for offline email discussions, but instead use files placed on the meeting server instead. Inbox/Drafts folder is used for meeting offline discussions.

## 1.3	Other

In accordance with the Working Procedures it is reaffirmed that: (i) compliance with all applicable antitrust and competition laws is required; (ii) timely submissions of work items in advance of TSG or WG meetings are important to allow for full and fair consideration of such matters; and (iii) the chair will conduct the meeting with strict impartiality and in the interests of 3GPP

Note on (i): In case of question please contact your legal counsel.

Note on (ii): WIDs don’t need to be submitted to the RAN2 meeting and will typically not be discussed here either.

Consensus principles reminder The attention of the delegates to the meeting is drawn to the fact that 3GPP endeavours to reach consensus on all decisions and therefore depends on a cooperative spirit of the Individual Members. In particular, Individual Members are encouraged to seek a consensus-based solution and only to sustain objections as a very last resort, and where absolutely necessary and well justified. The leadership will conduct the present meeting in a manner whereby informal methods of reaching consensus are encouraged, whilst ensuring that well justified concerns are taken into account

RAN endorsed working principle for 6G (RP-250766) 3GPP to create lean and streamlined standards for 6G, e.g., by dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, avoiding excessive configurations, etc. Any exception to the above shall be well justified.

# 2	General

## 2.1	Approval of the agenda

R2-2506701	Agenda for RAN2#131bis	Chairman	agenda

Approved

## 2.2	Approval of the report of the previous meeting

R2-2506702	RAN2#131 Meeting Report	MCC	report	Late

Approved

## 2.3	Reporting from other meetings

## 2.4	Instructions

CRs

- Use latest CR template version 12.3 for all CRs submitted to RAN2 meeting

Rel-18 and earlier maintenance CRs

- Only essential/critical corrections are expected

- Editorial and clarification corrections should be sent to be reviewed and approved by spec rapporteurs prior to submission.

- Editorials corrections should be collected and submitted by spec rapporteurs.

- NOTE: the tdoc limit applies to all CRs (i.e. WI spec rapporteurs are NO longer expected to submit individual contributions).  They can submit a company CR where they also include miscellaneous corrections that have been sent to them.

Rel-18 UE capabilities

-	EUTRA UE capabilities corrections are covered by separate CRs

-	RAN1/RAN4 NR UE capabilities (new) and corrections are covered in Rel-18 common MegaCRs (38306 and 38331) covering all rel-18 WIs (end outcome).

-	UE capabilities in LPP 37355 and SLPP 38355 are covered in the main CRs for the Positioning WI.

Rel-19 CRs

- CR already agreed in principle but not yet officially agreed must be submitted to RAN2#132 for formal approval under in-principle agreed CRs AIs

- CR editors / Rapporteurs continue to support maintenance related to their respective CR / WI and are required to follow drafting rules

- Single correction CR per spec coordinated by CR editor/rapporteurs will be agreed per feature for RAN#132

- Rapporteurs (except for RRC) should create open issue list for correction phase.  See below.

- CR editors / Rapporteurs should gather miscellaneous and non-controversial issues, if any, for their respective specification prior to submission deadline.  Other companies are expected to give editorial inputs to the rapporteurs and not have contributions on such issues.

- Emails to CR editors/rapporteurs should follow the following naming convention when sending emails to rapporteurs:

[Pre_RAN2#131bis][CR xx.yyy] Clarification CRs

- The organizational AIs for each WIs are reserved for rapporteurs only.  CR rapporteurs are expected to submit only 1 CR per spec.

- Companies are expected to submit Tdocs with TP (not CRs).   More specifically, the Tdoc should contain description of open issues/proposal and the proposed corrections/TP in the contribution itself.   Small issues can be included in the tdoc with just short justification same level of detail as in cover sheet.

- RRC ASN.1 changes can be drafted in a NBC way until ASN.1 is frozen, to avoid unnecessary RRC overhead.   The focus should be on drafting the changes in the best possible way.

- Inter-op analysis on Rel-19 CR coverpages in NOT needed

Open issues

- A list of open issues for correction phase is expected to be created per CR per WI (except for RRC specification - issues will be maintained in RIL list) and shared as soon as possible.  The list of CR open issues should be completed by Sept. 19th from CR editors/rapporteurs.  Companies can contribute to the open issue list and input (if requested) possible resolution.

- Rapporteur and/or company identifying issue can provide proposal on how to resolve the issues

  - For each issue, rapporteurs are requested to explicitly indicate whether further contribution input on the open issue is needed.   Input should be requested only for difficult to resolve issues and/or new open issues for which there wasn’t sufficient discussion time to resolve it.

  - Rapporteurs should critically consider the need for contribution on an issue.  If the issue can be resolved with a quick offline during the meeting, then the issue should be marked as to be resolved offline without contributions on that topic.

- Stage 2 corrections and UE capability corrections should be given to rapporteur directly over email discussion and no contributions are expected, unless really needed as specified by rapporteur.

- Companies should follow rapporteurs guidance (i.e. only address open issues for which the rapporteur indicates further input is needed).

- Companies should clearly indicate the open issue number they are addressing in their section and proposal, e.g. Proposal x: (RIL-1, MAC-1, etc) Agree to bla bla

ASN.1 and Handling of RILs

- Please review Hakan's email instructions on ASN.1 review.  Instructions are found at:  Directory Listing /ftp/Email_Discussions/RAN2/[Misc]/ASN1 review/Rel-19 2025-09

- Companies are expected to provide their TPs/Comments in the RIL Comment file and not submit contributions.   WI CR and RRC spec Rapporteurs can identify the critical RILs that require further contribution inputs.

- Single Tdoc containing 1 or more RIL resolutions per WI is expected. Companies are highly encouraged to work offline to resolve the issues.

Rel-19 UE capabilities

-	EUTRA UE capabilities are covered by separate CRs

-	All NR UE capabilities will be included common Mega CRs (38306 and 38331) covering all Rel-19 WIs (end outcome).

During the work on NR UE caps:

-	In a Common Rel-19 Agenda Item (AI): RAN1 and RAN4 feature corrections are handled jointly under a common AI, with some explicit exceptions. UE capabilities will be included in UE cap MegaCR directly from UE capability rapporteur

-	In WI-specific Rel-19 Agenda Items: RAN2 specific UE capabilities are handled per WI and endorsed as individual CRs.  Final endorsed CRs will be merged into mega CR post meeting.

Tdoc limitations

Tdoc limitations doesn’t apply to Rapporteur Input, i.e.

-	Assigned summary rapporteur input of the summary.

-	Email / offline discussions outcomes by discussion rapporteur,

-	Limit of 1 WI/SI  rapporteurs input for WI planning.  The work plan is not expected to be updated/submitted every meeting, unless needed.   It can include progress of other WG groups in the same Tdoc (i.e. separate Tdocs on other WG agreements are not required).

-	TS rapporteur input for TS maintenance.

-	Contact Company of a LSin that triggers RAN2 action may submit one tdoc to facilitate the LS reply. This only applies to one of the contact companies in case there are several (default the first).

Tdoc limitations doesn’t apply to Input created at the meeting, revisions, assigned documents etc.

Tdoc limitations doesn’t apply to shadow / mirror CRs (Cat A), or In-Principle Agreed CRs.

Tdoc limitations applies to all other submitted tdocs (e.g. discussion tdoc and CR tdoc are counted as two).

Postponed CRs still count towards tdoc limit unless 3 or more companies are co-sourcing it.

For each R19 feature, 1 additional tdoc on top of the limit is allowed for a primary co-sourcing company for co-sourced contribution with 4 or more companies (this also applies to RILs).

Tdoc request/submission for RAN2#131bis deadlines:

- Tdoc Submission deadline: Oct 3rd, 2025

## 2.5	Others

R2-2506703	RAN2 Handbook	MCC	discussion	Late

# 3	Incoming liaisons

Note: LSs are moved to the respective agenda items if any.

# 4	EUTRA Rel-17 and earlier

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.1	EUTRA corrections Rel-17 and earlier

(NB_IOTenh4_LTE_eMTC6-Core; leading WG: RAN1; REL-17; WID: RP-211340)

(UPIP_EN-DC_UE; leading WG: RAN3; REL-17; WID: RP-213669)

(LTE TEI17)

Essential corrections to LTE Rel-17 topics not covered by other agenda items.

(NB_IOTenh3-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed: June 20; WID: RP-200293); REL-15 and Earlier NB-IoT WIs are in scope but not listed explicitly (long list).

(LTE_eMTC5-Core; LTE_eMTC5-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed:  June 20; WID: RP-192875;), REL-15 and Earlier eMTC WIs are in scope but not listed explicitly (long list).

(LTE_feMob-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed: June 20; WID: RP-190921);

(LTE_terr_bcast-Core, LTE_DL_MIMO_EE-Core, LTE_high_speed_enh2-Core; LTE TEI16 Non-positioning);

(LTE_NBIOT_eMTC_NTN; leading WG: RAN1; REL-17; WID: RP-211601)

REL-16 and Earlier EUTRA WIs are in scope but not listed explicitly (long list), Except Positioning WI, which is addressed by AIs below.

NOTE that LTE corrections related to NR WIs or Joint NR LTE WIs should be submitted to NR AIs below.

NOTE that LTE corrections which are the same as an NR correction should be submitted to the respective NR AI (so the NR CR and LTE CR can be treated together).

This Agenda Item is treated in the Maintenance Breakout session (Corrections for LTE_NBIOT_eMTC_NTN might be treated in the NTN breakout session)

MINT

R2-2507248	Clarification that MINT applicability only applies to E-UTRA connected to 5GC [MINT]	Huawei, HiSilicon	CR	Rel-17	36.306	17.10.0	1926	-	F	TEI17

R2-2507249	Clarification that MINT applicability only applies to E-UTRA connected to 5GC [MINT]	Huawei, HiSilicon	CR	Rel-18	36.306	18.6.0	1927	-	A	TEI17

R2-2507252	Clarification that MINT applicability only applies to E-UTRA connected to 5GC [MINT]	Huawei, HiSilicon	CR	Rel-19	36.306	19.0.0	1928	-	A	TEI17

-	Samsung thinks we don’t need to capture EUTRA/5GC since its clear from the title heading of “6.18 E-UTRA/5GC features”. Huawei anyway wants to clarify it in the description.

-	Samsung also think reference to RRC is enough.

-	Lenovo thinks no change is needed as the Rel-19 CR will be designed correctly and named properly. Xiaomi agrees

Not pursued

SPS for TDD

R2-2507316	Correction to uplink grant allocation for Semi-Persistent Scheduling for TDD	TOYOTA Info Technology Center, Lenovo	CR	Rel-19	36.321	19.0.0	1597	-	F	LTE_LATRED_L2-Core, TEI19

-	Nokia agrees with the intention but thinks its editorial. ZTE wants to change this from R14. Huawei wonders if this should be cat D. Qualcomm agrees with ZTE.

R2-2507325	Correction to preallocated uplink grant for TDD	TOYOTA Info Technology Center, Lenovo	CR	Rel-19	36.321	19.0.0	1598	-	F	LTE_eMob-Core, TEI19	Revised

R2-2507459	Correction to preallocated uplink grant for TDD	TOYOTA Info Technology Center, Lenovo	CR	Rel-19	36.321	19.0.0	1598	1	F	LTE_eMob-Core, TEI19	R2-2507325

-	Qualcomm thinks its editorial, and think it can be merged with the previous CR. Ericsson agrees with Qualcomm. Toyota thinks we need to do something. Huawei want to merge and change from Rel-14.

Both above are agreeable but to be merged into one Rel-19 CR with magic sentence. CR to be seen next meeting.

SON/MDT

R2-2507413	Correction on event triggered based logged MDT configuration	Huawei, HiSilicon, CMCC, ZTE	CR	Rel-17	36.331	17.14.0	5164	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2507700	Correction on event triggered based logged MDT configuration	Huawei, HiSilicon, CMCC, ZTE	CR	Rel-17	36.331	17.14.0	5164	1	F	NR_ENDC_SON_MDT_enh-Core

R2-2507414	Correction on event triggered based logged MDT configuration	Huawei, HiSilicon, CMCC, ZTE	CR	Rel-18	36.331	18.7.0	5165	-	A	NR_ENDC_SON_MDT_enh-Core

R2-2507415	Correction on event triggered based logged MDT configuration	Huawei, HiSilicon, CMCC, ZTE	CR	Rel-19	36.331	19.0.0	5166	-	A	NR_ENDC_SON_MDT_enh-Core

-	Samsung think the LTE RRC spec works fine as necessary wording is above in the same section.

Not pursued

R2-2506976	Correction on GNSS position acquisition	Xiaomi	CR	Rel-17	36.331	17.14.0	5158	-	F	LTE_NBIOT_eMTC_NTN-Core

- Ericsson thinks this should be normative text

- Google thinks the original note is ok, the UE would not stay in connected for too long in the other cases. Xiaomi thinks the UE could stay for long in connected mode

- HW wonders if we need to consider the resume case in this EPC case

- QC is ok with adding “or resuming”

- Samsung thinks the Note already covers all the cases and it’s also already clear from Stage 2

- Vivo and CATT think the current note covers all the cases and no changes are needed. MTK agrees

RAN2 reconfirms that the UE may need to re-acquire the GNSS position before establishing or resuming the connection, or performing EDT or transmission using PUR procedures to avoid interruption during the connection, EDT or transmission using PUR procedures (no spec changes needed)

Not pursued

R2-2506977	Correction on GNSS position acquisition	Xiaomi	CR	Rel-18	36.331	18.7.0	5159	-	A	LTE_NBIOT_eMTC_NTN-Core

Not pursued

R2-2507477	Introduce UE capability for UE coarse location reporting	Xiaomi	CR	Rel-17	36.306	17.10.0	1930	-	F	LTE_NBIOT_eMTC_NTN-Core

- HW thinks we already had this discussion in Rel-17 and we decided not to have this. Nokia and ZTE agrees.

- QC thinks we decided to specify that the information is sent if available and then there was no need for the capability

- Vivo is ok to have a capability for the eMTC case

- Samsung thinks it would be clearer to have this

RAN2 understands that all UEs supporting (IoT/NR) NTN should understand the request to report the coarse location.

RAN2 reconfirms that a UE supporting (IoT/NR) NTN may not report the coarse location information if not available.

Not pursued

R2-2507478	Introduce UE capability for UE coarse location reporting	Xiaomi	CR	Rel-18	36.306	18.6.0	1931	-	A	LTE_NBIOT_eMTC_NTN-Core

Not pursued

R2-2507479	Introduce UE capability for UE coarse location reporting	Xiaomi	CR	Rel-17	36.331	17.14.0	5169	-	F	LTE_NBIOT_eMTC_NTN-Core

Not pursued

R2-2507480	Introduce UE capability for UE coarse location reporting	Xiaomi	CR	Rel-18	36.331	18.7.0	5170	-	A	LTE_NBIOT_eMTC_NTN-Core

Not pursued

## 4.3	Positioning corrections Rel-16 and earlier

(LTE_NavIC-Core, LTE TEI16 Positioning), REL-15 and Earlier WIs related to positioning are in scope but not listed explicitly (long list).

Tdoc Limitation: 1 tdoc

# 5	NR Rel-15 and Rel-16

Essential corrections only.

Tdoc Limitation: 3 Tdocs in total for agenda item 5 (incl. its sub agenda items) and agenda item 6 (incl. its sub agenda items)

In case a correction need to be reflected in both NR TS and LTE TS, the corrections should be submitted under one single AI (so the NR and LTE correction can be treated together), the sub-Ais below this

## 5.1	Common

Includes the following WIs and input that doesn’t fit elsewhere.

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_IAB-Core; leading WG: RAN2; REL-16; started: Dec 18; target Aug 20; WID: RP-200840)

(NR_unlic-Core; leading WG: RAN1; REL-16; started: Dec 18; Closed June 20; WID: RP-192926).

(NR_IIOT-Core; leading WG: RAN2; REL-16; started: Mar 19; Completed: Jun 20; WID: RP-200797)

(NR_UE_pow_sav-Core; leading WG: RAN1; REL-16; started: Mar 19; Completed Jun 20; WID: RP-200494).

(NR_2step_RACH-Core; leading WG: RAN1; REL-16; started: Dec 18; Completed: June 20; WID: RP-200085).

(SRVCC_NR_to_UMTS-Core; leading WG: RAN2; REL-16; started: Dec 18; Completed; Mar 20; WID: RP-190713)

(RACS-RAN-Core, leading WG: RAN2; REL-16; started: Mar 19; completed: Jun 20; WID: RP-191088)

(NG_RAN_PRN-Core; leading WG: RAN3; REL-16; started: Mar 19; completed: June 20; WID: RP-200122)

(NR_eMIMO-Core, leading WG: RAN1; REL-16; started: Jun 18; target; Aug 20; WID: RP-200474)

(NR_CLI_RIM; leading WG: RAN1; REL-16; started: Dec 18; Completed: Jun 20; WID: RP-191997)

(NR_L1enh_URLLC-Core, leading WG: RAN1; REL-16; Completed: June 20; WID: RP-191584)

(LTE_NR_DC_CA_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Target Aug 20; WI RP-200791)

(NR_Mob_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed June 20; WID: RP-192277).

(NR_SON_MDT-Core; leading WG: RAN3; REL-16; started: Jun 19; Completed June 20; WID: RP-191776)

(5G_V2X_NRSL-Core; leading WG: RAN1; REL-16; started: Mar 19; completed; Aug 20; WID: RP-200129)

(NR_HST, NR_RRM_enh-Core, NR_RF_FR1, NR_RF_FR2_req_enh, NR_n66_BW, LTE_NR_B41_Bn41_PC29dBm-Core, NR_CSIRS_L3meas,)

(NR TEI16)

LTE mob enh corrections that are common with NR mobility enhancements should be submitted to this AI.

### 5.1.1	Stage 2 and Organisational

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 36.300, 37.340

### 5.1.2	User Plane corrections

User Plane corrections will be handled in the User Plane break out session

#### 5.1.2.1	MAC

F-field

R2-2507636	Correction to F field in MAC subheader for SL-SCH	NTT DOCOMO, INC., Qualcomm Incorporated, Samsung	CR	Rel-16	38.321	16.21.0	2132	-	F	5G_V2X_NRSL-Core

R2-2507637	Correction to F field in MAC subheader for SL-SCH	NTT DOCOMO, INC., Qualcomm Incorporated, Samsung	CR	Rel-17	38.321	17.14.0	2133	-	A	5G_V2X_NRSL-Core

R2-2507638	Correction to F field in MAC subheader for SL-SCH	NTT DOCOMO, INC., Qualcomm Incorporated, Samsung	CR	Rel-18	38.321	18.7.0	2134	-	A	5G_V2X_NRSL-Core

-	Docomo indicates that R17 and R18 CRs need to be revised to account for flexible sized MAC CEs, i.e. need to add “size of the MAC SDU or variabled sized MAC CEs is less than 256 bytes”.

-	Xiaomi agrees with this. SHARP agrees and indicates we need R19 CRs too.

-	Lenovo indicates we need to remove NR-DC as impacted architecture option.

In-principle agreed but from R17 the change should be “size of the MAC SDU or variable sized MAC CEs is less than 256 bytes”. For all releases, remove NR-DC as impacted arch option. Make a R19 CR.

#### 5.1.2.2	RLC PDCP SDAP BAP

### 5.1.3	Control Plane corrections

#### 5.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, e.g. 36331, Stage-2 etc.

Single SCS

R2-2507107	Clarification of the single SCS per frequency restriction	Apple	CR	Rel-15	38.331	15.30.0	5511	-	F	NR_newRAT-Core

R2-2507108	Clarification of the single SCS per frequency restriction	Apple	CR	Rel-16	38.331	16.21.0	5512	-	A	NR_newRAT-Core

R2-2507109	Clarification of the single SCS per frequency restriction	Apple	CR	Rel-17	38.331	17.14.0	5513	-	A	NR_newRAT-Core

R2-2507110	Clarification of the single SCS per frequency restriction	Apple	CR	Rel-18	38.331	18.7.0	5514	-	A	NR_newRAT-Core

-	ZTE think the RAN1 agreement is not correctly interpreted. CATT agrees with ZTE, CATT think that for a particular freq the SCS should be the same.

-	Nokia thinks that the change should be in the IDLE-mode spec instead.

-	ZTE wonders if there are issues in the field, Apple havnt seen this yet. ZTE thinks then we don’t need this. Apple anyway wants this restriction.

-	ZTE think we can capture something in the minutes instead.

[AT131bis][604][Maint] SCS per frequency restriction (Apple)

Scope:

    - Discuss offline (face-to-face preferred, if suitable) if we need a change and if so what the change would be.

Intended outcome:

    - In-principle agreed CR(s) in R2-2507834, R2-2507835, R2-2507836, R2-2507837, if needed.

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2507834	Clarification of the single SCS per frequency restriction	Apple, Nokia, Samsung Inc.	CR	Rel-15	38.300	15.21.0	1049	-	F	NR_newRAT-Core

R2-2507835	Clarification of the single SCS per frequency restriction	Apple, Nokia, Samsung Inc.	CR	Rel-16	38.300	16.20.0	1050	-	F	NR_newRAT-Core

R2-2507836	Clarification of the single SCS per frequency restriction	Apple, Nokia, Samsung Inc.	CR	Rel-17	38.300	17.14.0	1051	-	F	NR_newRAT-Core

R2-2507837	Clarification of the single SCS per frequency restriction	Apple, Nokia, Samsung Inc.	CR	Rel-18	38.300	18.7.0	1052	-	F	NR_newRAT-Core

R2-2507908	Clarification of the single SCS per frequency restriction	Apple, Nokia, Samsung Inc.	CR	Rel-19	38.300	18.7.0	1053	-	F	NR_newRAT-Core

All five are in-principle agreed

PWS

R2-2507227	On continuing acquiring PWS notification(s) until it re-acquires schedulingInfoList in SIB1	Samsung	CR	Rel-15	38.331	15.30.0	5520	-	F	NR_newRAT-Core

R2-2507228	On continuing acquiring PWS notification(s) until it re-acquires schedulingInfoList in SIB1	Samsung	CR	Rel-16	38.331	16.21.0	5521	-	A	NR_newRAT-Core

R2-2507230	On continuing acquiring PWS notification(s) until it re-acquires schedulingInfoList in SIB1	Samsung	CR	Rel-17	38.331	17.14.0	5522	-	A	NR_newRAT-Core

R2-2507231	On continuing acquiring PWS notification(s) until it re-acquires schedulingInfoList in SIB1	Samsung	CR	Rel-18	38.331	18.7.0	5523	-	A	NR_newRAT-Core

-	Ericsson agrees that there is such a note in LTE-spec, but there are no inter-operability issues, however Ericsson are OK with a CR if from Rel-19. Huawei also think there are no inter-operability issues and there are different ways to address the issue so no CR is needed.

-	Samsung are OK to not add this change, but want to capture a common understanding in the minutes. Huawei says that there are other ways than the proposed and we can simply leave it to UE implementation

Not pursued, as RAN2 assumes the issue can be addressed by UE implementation

SON/MDT

R2-2507416	Correction on event triggered based logged MDT configuration	Huawei, HiSilicon, CMCC, ZTE	CR	Rel-16	38.331	16.21.0	5531	-	F	NR_SON_MDT-Core

R2-2507699	Correction on event triggered based logged MDT configuration	Huawei, HiSilicon, CMCC, ZTE	CR	Rel-16	38.331	16.21.0	5531	1	F	NR_SON_MDT-Core

R2-2507417	Correction on event triggered based logged MDT configuration	Huawei, HiSilicon, CMCC, ZTE	CR	Rel-17	38.331	17.14.0	5532	-	A	NR_SON_MDT-Core

R2-2507418	Correction on event triggered based logged MDT configuration	Huawei, HiSilicon, CMCC, ZTE	CR	Rel-18	38.331	18.7.0	5533	-	A	NR_SON_MDT-Core

R2-2507419	Correction on event triggered based logged MDT configuration	Huawei, HiSilicon, CMCC, ZTE	CR	Rel-19	38.331	19.0.0	5534	-	A	NR_SON_MDT-Core

-	Samsung think this is not needed since not essential, current spec is clear enough from the field description. If we do this there will be some issues and there are similar clarifications needed if we do this change. Xiaomi agrees that this is not essential. Nokia agrees with Xiaomi and think that the conditions are checked above in the same section.

-	MediaTek wonders if there is a field issue? Huawei has not seen any issues in field but NR and LTE specs are not aligned and prefer to align.

Not pursued

SSB-less SCell

R2-2507595	Clarification of SSB-less SCell	Ericsson	CR	Rel-15	38.331	15.30.0	5549	-	F	NR_newRAT-Core

R2-2507596	Clarification of SSB-less SCell	Ericsson	CR	Rel-16	38.331	16.21.0	5550	-	A	NR_newRAT-Core

R2-2507597	Clarification of SSB-less SCell	Ericsson	CR	Rel-17	38.331	17.14.0	5551	-	A	NR_newRAT-Core

R2-2507598	Clarification of SSB-less SCell	Ericsson	CR	Rel-18	38.331	18.7.0	5552	-	A	NR_newRAT-Core

-	Nokia suggests that we should follow the progress of the R19 discussion. Nokia says we can postpone to next meeting.

Postponed

#### 5.1.3.2	UE capabilities

UE cap corrections 38306, 38331

#### 5.1.3.3	Other

This agenda item addresses the idle and inactive behaviour specified in 38.304 or 36.304, LTE-specific changes for the applicable WIs, Other parts not covered elsewhere.

## 5.3	NR Positioning Support

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_pos-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Jun 20; WID: RP-200218).

(NR TEI16 Positioning)

Stage 2 corrections shall be discussed with the specification rapporteur (Sven Fischer sfischer@qti.qualcomm.com) before submission. Stage 2 CRs not discussed with the specification rapporteur will not be treated.

# 6	NR Rel-17

Essential corrections only.  Editorial/clarifications should be sent to be reviewed and approved by spec rapporteurs prior to submission.  Editorials should only be submitted by spec rapporteurs.

Tdoc Limitation: 3 Tdocs in total for agenda item 5 (incl. its sub agenda items) and agenda item 6 (incl. its sub agenda items)

## 6.1	Common

(NR_MG_enh-Core; leading WG: RAN4; REL-17; WID: RP-211591)

(NR_UDC_enh-Core; leading WG: RAN2; REL-17; WID: RP-211203)

(NG_RAN_PRN_enh-Core; leading WG: RAN3; REL-17; WID: RP-202363)

(NR_IAB_enh-Core; leading WG: RAN2; REL-17; WID: RP-211548)

(NR_UE_pow_sav_enh-Core; leading WG: RAN2; REL-17; WID: RP-212630)

(LTE_NR_DC_enh2-Core; leading WG: RAN2; REL-17; WID: RP-201040)

(LTE_NR_MUSIM-Core; leading WG: RAN2; REL-17; WID: RP-212610)

(NR_Slice-Core; leading WG: RAN2; REL-17; WID: RP-212534)

(NR_QoE-Core; leading WG: RAN3; REL-17; WID: RP-211406)

(NR_ext_to_71GHz-Core; leading WG: RAN1; REL-17; WID: RP-212637)

(NR_cov_enh-Core; leading WG: RAN1; REL-17; WID: RP-211566): non-RACH-indication parts

(NR_redcap-Core; leading WG: RAN1; REL-17; WID: RP-211574)

(NR_feMIMO-Core; leading WG: RAN1; REL-17; WID: RP-212535)

(NR_SmallData_INACTIVE-Core, leading WG: RAN2; REL-17; WID: RP-212594)

(NR_IIOT_URLLC_enh-Core; leading WG: RAN2; REL-17; WID: RP-210854)

(NR_MBS-Core; leading WG: RAN2; REL-17; WID: RP-201038)

(NR_ENDC_SON_MDT_enh-Core; leading WG: RAN3; REL-17; WID: RP-201281)

(NR_NTN_solutions-Core; leading WG: RAN2; REL-17; WID: RP-211557)

(NR_SL_enh-Core; leading WG: RAN1; REL-17; WID: RP-202846)

(NR_SL_Relay-Core; leading WG: RAN2; REL-17; WID: RP-212601)

PRACH partitioning items

(NR TEI17)

Includes Rel-17 Work Items without specific R2 Agenda Item, e.g. RAN1 and RAN4 led items, SA2 and CT1 led items (was previously “Rel-17 Other”)

Includes aspects that does not fit under the more specific AIs, e.g. multi-WI aspects.

Corrections for NR_NTN_solutions-Core might be treated in the NTN breakout session.

### 6.1.1	Stage 2 and Organisational

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 37.340, (36.300 if applicable)

Emergency PDUs

R2-2506723	Reply LS on emergency call back and paging (R3-255883; contact: Qualcomm)	RAN3	LS in	Rel-17	NR_newRAT-Core, NR_redcap-Core	To:SA2	Cc:RAN2, CT1, RAN

-	Qualcomm think we should just note it, no RAN2 action.

Noted

PEI during emergency

R2-2506725	LS on compatibility issue for PEI and emergency PDU session (R3-255906; contact: ZTE)	RAN3	LS in	Rel-17	NR_UE_pow_sav_enh-Core	To:SA2	Cc:CT1, RAN2

-	ZTE thinks that the decision will be made in SA2 which solution to pick. ZTE suggest that we postpone this whole issue, including the CRs below from vivo.

Noted

R2-2507023	Correction on PEI in emergency PDU session	Vivo, Nokia (Rapporteur)	CR	Rel-17	38.300	17.14.0	1037	-	F	NR_UE_pow_sav_enh-Core

R2-2507024	Correction on PEI in emergency PDU session	Vivo, Nokia (Rapporteur)	CR	Rel-18	38.300	18.7.0	1038	-	A	NR_UE_pow_sav_enh-Core

R2-2507025	Correction on PEI in emergency PDU session	Vivo, Nokia (Rapporteur)	CR	Rel-19	38.300	19.0.0	1039	-	A	NR_UE_pow_sav_enh-Core

R2-2507608	Consideration on the LS on compatibility issue for PEI and emergency PDU session	ZTE Corporation, Sanechips	discussion	Rel-17	NR_UE_pow_sav_enh-Core

R2-2507609	Reply LS on compatibility issue for PEI and emergency PDU session	ZTE Corporation, Sanechips	LS out	Rel-17	NR_UE_pow_sav_enh-Core	To:RAN3, SA2	Cc:CT1

Postponed to if/when we hear anything from SA2

### 6.1.2	User Plane corrections

User Plane Related aspects will be handled in the User Plane break out session. (exception: TEI new proposals if any).

### 6.1.3	Control Plane corrections

#### 6.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, except UE caps.

R2-2506794	Correction on previousPSCellId in SCGFailureInformation	CATT	CR	Rel-17	38.331	17.13.0	5487	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2506795	Correction on previousPSCellId in SCGFailureInformation	CATT	CR	Rel-18	38.331	18.6.0	5488	-	A	NR_ENDC_SON_MDT_enh-Core

R2-2506796	Correction on previousPSCellId in SCGFailureInformation	CATT	CR	Rel-19	38.331	18.6.0	5489	-	A	NR_ENDC_SON_MDT_enh-Core

R2-2507554	Correction on SCGFailureInformation	Samsung, Ericsson	CR	Rel-17	38.331	17.14.0	5545	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2507566	Correction on SCGFailureInformation	Samsung, Ericsson	CR	Rel-18	38.331	18.7.0	5546	-	A	NR_ENDC_SON_MDT_enh-Core

Discussion on both above

-	CATT think that we should not exclude the PSCell addition failure case. Ericsson think that both these CR sets are needed and complementing each other. ZTE think that the Samsung CRs are correct and only want to cover PSCell change.

-	CATT and Ericsson think we can merge these CRs.

Postponed to next meeting to see if/what change is needed.

R2-2507620	Correction on setting timeSinceCHO-Reconfig when the failure is due to RLF	Ericsson	CR	Rel-17	38.331	17.14.0	5555	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2507621	Correction on setting timeSinceCHO-Reconfig when the failure is due to RLF	Ericsson	CR	Rel-18	38.331	18.7.0	5556	-	A	NR_ENDC_SON_MDT_enh-Core

R2-2507622	Correction on setting timeSinceCHO-Reconfig when the failure is due to RLF	Ericsson	CR	Rel-19	38.331	19.0.0	5557	-	A	NR_ENDC_SON_MDT_enh-Core

-	Samsung wonders if we can say “the PCell”, i.e. remove the source? CATT agrees with the CR as is. Xiaomi wants to have “the last PCell”. Samsung says that we use “the PCell” in more places.

The three above are in-principle agreed but change to “the PCell”

71 GHz

R2-2507004	Correction on subcarrierSpacing values in IE SCS-SpecificCarrier	Huawei, HiSilicon	CR	Rel-17	38.331	17.14.0	5500	-	F	NR_ext_to_71GHz-Core

R2-2507005	Correction on subcarrierSpacing values in IE SCS-SpecificCarrier	Huawei, HiSilicon	CR	Rel-18	38.331	18.7.0	5501	-	A	NR_ext_to_71GHz-Core

R2-2507006	Correction on subcarrierSpacing values in IE SCS-SpecificCarrier	Huawei, HiSilicon	CR	Rel-19	38.331	19.0.0	5502	-	A	NR_ext_to_71GHz-Core

-	Ericsson agrees but think that this CR is mandatory implemented. Qualcomm think the CR is correct.

All are in-principle agreed

MINT

R2-2507066	Correction on uac-BarringFactorForAI3 absence case [MINT]	Huawei, HiSilicon	CR	Rel-17	38.331	17.14.0	5505	-	F	TEI17

R2-2507067	Correction on uac-BarringFactorForAI3 absence case [MINT]	Huawei, HiSilicon	CR	Rel-18	38.331	18.7.0	5506	-	A	TEI17

R2-2507068	Correction on uac-BarringFactorForAI3 absence case [MINT]	Huawei, HiSilicon	CR	Rel-19	38.331	19.0.0	5507	-	A	TEI17

-	Samsung think the field description is clear already so there is no need to change. Lenovo agrees with Samsung, and think that the field description should be read together and there is no need to change the procedure text. MediaTek thinks this clarification is good. Qualcomm also think this is the correct behaviour and we can agree the CR. Lenovo can agree if it makes others happy.

All are in-principle agreed

QoE

R2-2507390	Correction on pdcp-Config for SRB4	Samsung, Ericsson	CR	Rel-17	38.331	17.14.0	5527	-	F	NR_QoE-Core

-	Samsung says that this has been discussed offline (together with the Rel-18 CR) and this is agreeable.

In-principle agreed

Relays – To be handled in Nathan’s session

R2-2507097	Correction on PC5 Relay RLC channel configuration	Apple, CATT	CR	Rel-17	38.331	17.14.0	5508	-	F	NR_SL_relay-Core

Not pursued

Discussion:

Ericsson understand it is up to the network to make sure the configurations are internally consistent.  Apple confirm that this is the intention, and the proposal is to clarify that the SL dedicated configuration and SRAP configuration should both be taken into account.  Ericsson think the UE should be able to understand the relationship; Apple indicate that we do not show the dependency today on the SRAP configuration.

Huawei think the spec is already clear and the add/modify/release part handles it.

Chair wonders if an implementation reading the spec today would do anything wrong.  Apple understand that the relay has to set up SRAP configurations blindly for all the remote UEs with all the relay RLC channels, and we have no procedure now that says what to do when the SRAP configuration changes.

ZTE think Apple’s observation is correct, and we could capture the SRAP configuration in the same sentence as the sl-ConfigDedicatedNR.

Nokia understand that nothing is broken, just not optimized.  Apple understand that over-configuring the RLC channels would be an incorrect behaviour by the relay.  Nokia think it does not break the system and this is for Rel-17, which has been frozen for a long time; Apple think it is not what we intended to design.  Huawei agree with Nokia that we should only consider changes to Rel-17 if they are critical.

Chair asks if a relay UE that implements the sensible configuration is in violation today.  Ericsson think it depends on the network to have consistent configurations.

OPPO think since we have a common understanding of the correct implementation and there is some lack of clarity in the spec, adding a NOTE could be a compromise way forward.  Nokia think we could capture it in the minutes; they have some concern about adding the NOTE to a frozen release.

Apple interpret that a relay UE doing the right thing would be in violation of the spec today, because it applies a configuration different from what the spec tells it to do.

LG have no strong opinion but think Apple’s proposal is reasonable, and we could consider taking a CR from Rel-18.

Huawei think capturing it in the minutes would be enough.  ZTE think we could capture it in Rel-19 (which is not frozen), as they see the change as being necessary for aligned UE behaviour.

Nokia think a UE doing the right thing is not in violation.  ZTE think it may violate what the network expects, and without guidance we could see two different UE behaviours.  Nokia think both implementations are valid.  Chair understands that the “extra” RLC channels would never be used because the SRAP configuration cannot direct traffic to them.  Apple think a relay UE implementing the spec must set up the extra channels, or else it is going outside what the spec tells it to do.

Apple think a change from Rel-19 would be all right, but we should capture something for the earlier releases at least in the minutes.

LG agree we could put something in the minutes and take the CR from Rel-19 with the magic sentence.

Agreement:

RAN2 confirm that a L2 U2N relay UE initiates a PC5 Relay RLC channel addition towards a L2 U2N Remote UE if the sl-RLC-ChannelID received in sl-ConfigDedicatedNR is indicated for this L2 U2N Remote UE in accordance with sl-SRAP-ConfigRelay, and a PC5 Relay RLC channel with the received sl-RLC-ChannelID was not configured before.

R2-2507098	Correction on PC5 Relay RLC channel configuration	Apple, CATT	CR	Rel-18	38.331	18.7.0	5509	-	A	NR_SL_relay-Core

Not pursued

R2-2507099	Correction on PC5 Relay RLC channel configuration	Apple, CATT	CR	Rel-19	38.331	19.0.0	5510	-	A	NR_SL_relay-Core

Update to category F and add magic sentence

Agreed in principle with these changes as R2-2507792

#### 6.1.3.2	UE capabilities

UE cap corrections 38306, 38331.

MIMO

R2-2507164	Correction on UL Tx switching MIMO coherence capabilities	ZTE Corporation	CR	Rel-17	38.331	17.14.0	5515	-	F	NR_RF_FR1_enh-Core

R2-2507165	Correction on UL Tx switching MIMO coherence capabilities	ZTE Corporation	CR	Rel-18	38.331	18.7.0	5516	-	A	NR_RF_FR1_enh-Core, NR_MC_enh-Core

R2-2507166	Correction on UL Tx switching MIMO coherence capabilities	ZTE Corporation	CR	Rel-19	38.331	19.0.0	5517	-	A	NR_RF_FR1_enh-Core, NR_MC_enh-Core

-	Xiaomi think this is just reference info and want to postpone this and can discuss it with the mega CR. ZTE says there is no mega CR for Rel-16. Qualcomm think this is good to clarify and support the CR. Xiaomi wants more time during the meeting. Ericsson think we can just IPA it and if there really are issues we can come back next meeting, so that gives Xiaomi even more time.

All are in-principle agreed, but “of 2 bands” is only needed from Rel-18.

RAN2 (in-principle) agrees these CR since it is addressing an inconsistency where only one (out of two) feature index-references are present in the ASN.1. RAN2 acknowledges that there are some missing feature index-references in the ASN.1, however RAN2 does not intend to address all of those missing index-references. Further, the field description in 38.306 is what defines a feature.

R2-2507481	Introduce UE capability for UE coarse location reporting	Xiaomi	CR	Rel-17	38.306	17.14.0	1368	-	F	NR_NTN_solutions-Core

Not pursued

R2-2507482	Introduce UE capability for UE coarse location reporting	Xiaomi	CR	Rel-18	38.306	18.7.0	1369	-	A	NR_NTN_solutions-Core

Not pursued

R2-2507483	Introduce UE capability for UE coarse location reporting	Xiaomi	CR	Rel-17	38.331	17.14.0	5535	-	F	NR_NTN_solutions-Core

Not pursued

R2-2507484	Introduce UE capability for UE coarse location reporting	Xiaomi	CR	Rel-18	38.331	18.7.0	5536	-	A	NR_NTN_solutions-Core	Revised

R2-2507495	Introduce UE capability for UE coarse location reporting	Xiaomi	CR	Rel-18	38.331	18.7.0	5536	1	A	NR_NTN_solutions-Core	R2-2507484

Not pursued

#### 6.1.3.3	Other

Including idle and inactive behaviour specified in 38.304 or 36.304.

## 6.3	NR positioning enhancements

(NR_pos_enh-Core; leading WG: RAN1; REL-17; WID: RP-210903)

# 7	NR Rel-18

## 7.0	Common

Rel-18 WIs not covered under an explicit AI in 7.x.  Multi-WI Rel-18 items, e.g. cross-WI-issues not handled under another WI. UE capabilities.

### 7.0.1	UE Capabilities

Multi-WI handling of Rel-18 feature lists and UE capability Mega CRs.

### 7.0.2	Rel-18 corrections

Essential corrections only. For smaller corrections please contact CR editor / Rapporteur directly.  Coordinate with rapporteurs and chair if input above limit is required

Tdoc limitation: 4

#### 7.0.2.1	RACH-less HO

Corrections to generalized RACH-less HO procedure, including NTN, mIAB, and overlapping sections of the LTM cell switch procedure

R2-2507399	Correction on releasing CFRA resources in case of RACH-less handover	Ericsson	CR	Rel-18	38.331	18.7.0	5528	-	F	TEI18

-	ZTE thinks that the text is needed in case of fallback to RACH-based. Ericsson think that there are other issues if such fallback would happen. Vivo agrees with ZTE think that RACH-less and RACH-based can both be configured and such fallback can happen. Huawei think current spec is OK since it says “if configured”. QC think wonders why NW would configure both RACH-less and RACH-based but if possible we need to understand if/how the spec works.

[AT131bis][601][Maint] Correction on RACH-less (Ericsson)

Scope:

    - Discuss offline (face-to-face preferred, if suitable) if and how the spec should be updated. Consider if fallback from RACH-less to RACH-based is valid.

Intended outcome:

    - In-principle agreed CR(s) in R2-2507831, if needed

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

-	Ericsson reports after the offline that it seems that current procedure it should be fine for the case on when CFRA resources are provided together with RACH-lessHO, hence no CR is needed

R2-2507399 is not pursued

#### 7.0.2.2	NR network-controlled repeaters

(NR_NetConRepeater; leading WG: RAN1; REL-18; WID: RP-230175)

#### 7.0.2.3	NR support for UAV

(NR_UAV-Core; leading WG: RAN2; REL-18; WID: RP-230782 and LTE WID: RP-230783 )

#### 7.0.2.4 	Mobile Terminated Small Data Transmission

(NR_MT_SDT-Core; leading WG: RAN2; REL-18; WID: RP-222993)

#### 7.0.2.5	IDC enhancements for NR and MR-DC

(NR_IDC_enh-Core; leading WG: RAN2; REL-18; WID: RP-221281)

#### 7.0.2.6	Mobile IAB (Integrated Access and Backhaul) for NR

( NR_mobile_IAB -Core; leading WG: RAN3; REL-18; WID: RP-232669)

R2-2507631	Correction on application of restrictions to mIAB-MT	Samsung	CR	Rel-18	38.304	18.4.0	0449	-	F	NR_mobile_IAB-Core

-	Huawei think its correct and find it editorial as it is only for clarification. Huawei want to soften the wording on the cover page, e.g. no interoperability issues.

-	Nokia says that “NCR” is mentioned on the cover page where IAB is intended.

-	Xiaomi says there are IAB-MT and mIAB. Samsung thinks that there is some inconsistent use of IAB and mIAB, and found that this missing definition was the most appropriate.

In-principle agreed but the inter-operability issue section should be removed

#### 7.0.2.7	Timing Resiliency and URLLC Enh

(NR_TRS_URLLC; leading WG: RAN3; REL-18; WID: RP-230754)

#### 7.0.2.8	Enhanced support of reduced capability NR devices

(NR_redcap_enh-Core; leading WG: RAN1; REL-18; WID: RP-232671)

#### 7.0.2.9	Further NR coverage enhancements

(NR_cov_enh2-Core; leading WG: RAN1; REL-18; WID: RP-221858)

#### 7.0.2.10	Network energy savings for NR

(Netw_Energy_NR-Core; leading WG: RAN1; REL-18; WID: RP-223540)

R2-2507274	MAC correction on UE transmissions during Cell DRX	Huawei, HiSilicon	CR	Rel-18	38.321	18.7.0	2128	-	F	Netw_Energy_NR-Core

R2-2507275	Stage-2 correction on UE transmissions during Cell DRX	Huawei, HiSilicon	CR	Rel-18	38.300	18.7.0	1043	-	F	Netw_Energy_NR-Core

R2-2507276	MAC correction on UE transmissions during Cell DRX	Huawei, HiSilicon	CR	Rel-19	38.321	19.0.0	2129	-	A	Netw_Energy_NR-Core

R2-2507277	Stage-2 correction on UE transmissions during Cell DRX	Huawei, HiSilicon	CR	Rel-19	38.300	19.0.0	1044	-	A	Netw_Energy_NR-Core

All in-principle agreed but the titles of the CRs does not need to have “MAC” and “Stage-2”.

#### 7.0.2.11	Further enhancement of data collection for SON MDT in NR and EN-DC

(NR_ENDC_SON_MDT_enh2-Core; leading WG: RAN3; REL-18; WID: RP-221825)

RLF report

R2-2506790	Correction on nrPreviousCell logging in RLF report	CATT	CR	Rel-18	38.331	18.6.0	5483	-	F	NR_ENDC_SON_MDT_enh2-Core

R2-2506791	Correction on nrPreviousCell logging in RLF report	CATT	CR	Rel-19	38.331	18.6.0	5484	-	A	NR_ENDC_SON_MDT_enh2-Core

Both are in-principle agreed

SHR

R2-2506792	Correction on timeSinceSHR in SHR	CATT	CR	Rel-18	38.331	18.6.0	5485	-	F	NR_ENDC_SON_MDT_enh2-Core

R2-2506793	Correction on timeSinceSHR in SHR	CATT	CR	Rel-19	38.331	18.6.0	5486	-	A	NR_ENDC_SON_MDT_enh2-Core

-	ZTE thinks this is not an essential correction.

Not pursued

CPAC

R2-2507519	Correction to CPAC MRO	ZTE Corporation, CMCC, Huawei, Ericsson, Sanechips	CR	Rel-18	38.331	18.7.0	5538	-	F	NR_ENDC_SON_MDT_enh2-Core

R2-2507520	Correction to CPAC MRO	ZTE Corporation, CMCC, Huawei, Ericsson, Sanechips	CR	Rel-19	38.331	19.0.0	5539	-	A	NR_ENDC_SON_MDT_enh2-Core

-	Samsung are OK with the CR overall but think the first change should be “mobility robustness optimization for SCG failure as defined in 38.306 [X]”. ZTE is OK with this but think we can update the wording to exactly refer to the capability names.

In-principle agreed but the capability names should be aligned to 306.

#### 7.0.2.12 Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR

(NR_DualTxRx_MUSIM-Core; leading WG: RAN2; REL-18; WID: RP-233071)

#### 7.0.2.13 NR MIMO evolution

(NR_MIMO_evo_DL_UL-Core; leading WG: RAN1; REL-18; WID: RP-233028)

R2-2506731	Reply LS on maximum transmission power for STxMP (R4-2511781; contact: vivo)	RAN4	LS in	Rel-20	NR_MIMO_evo_DL_UL-Core	To:RAN1	Cc:RAN2

Noted

On dependency of group-based beam reporting

R2-2507547	Correction on dependency of group-based beam reporting	Nokia	CR	Rel-18	38.331	18.7.0	5543	-	F	NR_MIMO_evo_DL_UL-Core

Discussion

-	Qualcomm think this CR is correct so agree.

R2-2507548	Correction on dependency of group-based beam reporting	Nokia	CR	Rel-19	38.331	19.0.0	5544	-	A	NR_MIMO_evo_DL_UL-Core

The two CRs above are in principle agreed.

#### 7.0.2.14 Enhancements of NR Multicast and Broadcast Services

(NR_MBS_enh-Core; leading WG: RAN2; REL-18; WID: RP-231829)

#### 7.0.2.15 Enhancement on NR QoE management and optimizations for diverse services

(NR_QoE_enh-Core; leading WG: RAN3; REL-18; WID: RP-223488)

R2-2506724	Reply LS on MBS Communication Service Type (R3-255896; contact: Nokia)	RAN3	LS in	Rel-18	NR_QoE_enh-Core	To:SA4	Cc:RAN2, SA5

Noted

R2-2507546	Correction on pdcp-Config for SRB4 and SRB5	Samsung, Ericsson	CR	Rel-18	38.331	18.7.0	5542	-	F	NR_QoE_enh-Core

-	Xiaomi are OK with the motivation but want to define a default config. The session chair thinks that it would be NBC. Nokia thinks that adding a default would be clean but that is NBC.

[AT131bis][602][Maint] Correction SRB4 and SRB5 (Samsung)

Scope:

    - Discuss offline (face-to-face preferred, if suitable) if we should add a default config for SBT4 and SBR5 or if we should agree the Samsung CR

Intended outcome:

    - In-principle agreed CR(s) in R2-2507832, if needed

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

-	Samsung clarifies that the original CRs are fine. Xiaomi and Samsung think that the approach with the default config is feasible, but since the approach with the default config is NBC, we do this instead.

R2-2507546 is in-principle agreed. And a Rel-19 mirror is to be seen in the next meeting.

#### 7.0.2.16	XR Enhancements for NR

(NR_XR_enh-Core; leading WG: RAN2; REL-18; WID: RP-230786)

R2-2507028	Discussion on DSR triggering for R18 XR	vivo	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: RAN2 to discuss which is the correct understanding on the current specification, and whether any change on MAC is needed:

-	alt1: “PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU” in 5.4.9 in MAC means “PDCP SDUs that are buffered for the LCG but have not been completely transmitted in any MAC PDU”. -> SDU segment could trigger DSR

-	alt2: “PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU” in 5.4.9 in MAC means “PDCP SDUs that are buffered for the LCG but have not been partially transmitted in any MAC PDU”. -> SDU segment cannot trigger DSR

Proposal 2: If the change on MAC is needed in P1, from which release should it start, i.e. Rel-18 or Rel-19.

R2-2507282	Discussion on DSR triggering for RLC segment	LG Electronics Inc.	discussion	Rel-18	NR_XR_enh-Core

Proposal 1. RAN2 confirms that in Rel-18 remaining RLC segment of the PDCP SDU may trigger DSR, if the remaining PDCP discard timer value is less than RemainingTimeThreshold.

Proposal 2. Clarify “have not been transmitted in any MAC PDU” in the MAC spec to “have not been completely transmitted by MAC PDU,” in order to report the delay information of RLC segment if a PDCP SDU is partially transmitted. Use TP in Annex A as a baseline.

R2-2507473	Clarification on DSR Triggering	Ericsson	discussion 	Rel-18	NR_XR_enh-Core

Observation 1	There are many examples in specification that a SDUs refers to the complete PDU while the SDU segments refer to part of the PDU data.

Observation 2	An RLC SDU is a PDCP PDU and a PDCP PDU is a PDCP SDU including PDCP headers.

Proposal 1	RAN2 to confirm that the transmission or reception of an RLC SDU segment is not equivalent to a transmission or reception of the said RLC SDU or the PDCP SDU.

Proposal 2	RAN2 agrees that the interpretation in Proposal 1 remains in Release 18 and any remaining RLC SDU segments of a PDCP SDU can trigger a DSR.

DISCUSSION:

- Xiaomi thinks that alt. 1 can work, but NW could also figure out there are segments left in the buffer. But are OK to confirm this way.

- Ericsson thinks that with the agreement it should be clear enough already.

- Vivo prefers to have a simple clarification in MAC. Current spec can be interpreted both way.

- OPPO also prefers to specify in MAC to avoid confusion in future.

- Nokia is OK to clarify.

- MTK is a bit worried about introducing “completely” as then we can start misinterpreting the rest of the specs. MTK thinks the specs is clear as it is.

- LGE thinks that in other cases we indicate whether this is part of SDU of a whole SDU. “Any PDU” is confusing. At least this should be changed, i.e. remove “any”. Vivo has similar view.

- Lenovo also thinks this should be clarified as it may come back.

- Apple thinks chair notes are sufficient.

- Huawei thinks some clarification is useful, but maybe “completely” may not be the best word.

RAN2 understanding is that: “PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU” in 5.4.9 in MAC means “PDCP SDUs that are buffered for the LCG but have not been completely transmitted in any MAC PDU”. -> SDU segment could trigger DSR

We will try to clarify this in MAC. FFS the wording and CB next meeting

#### 7.0.2.17	NR NTN enhancements

(NR_NTN_enh-Core; leading WG: RAN1; REL-18; WID: RP-232669)

R2-2506990	Correction on eventD2 with multiple moving reference locations	CSCN, ZTE Corporation, Sanechips, Huawei, Hisilicon, Xiaomi, CATT, OPPO	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1:	RAN2 can further discuss and down select from the following options to address the case when multiple neighbor cells with moving reference locations fulfil eventD2 and would trigger measurement report:

−	Option 1: For multiple moving reference locations on the same frequency, the network should configure separate MeasObjectNR for each reference location, where only one MO can perform SSB measurements.

−	Option 2: Similar as the eventA processing mechanism, use a cellsTriggeredList to include cells with moving reference locations that meet the entering/leaving condition for eventD2 during the "Measurement report triggering" and "Measurement reporting" procedures. Add information in MeasResults to indicate which neighbour cell (s) triggered event D2 is FFS.

−	Option 3: Add the moving reference location(s) for eventD2 to the ReportConfigNR.

- Apple thinks this is a valid issue, and prefers option 2 (if we go for option 3 we need to add more).

- Vivo thinks we can leave this to NW implementation to only configure one, alternatively we can go for option 1. QC agrees

- QC thinks there could be a problem also for multiple configuration of D1 events, for privacy considerations.

- Nokia thinks in case we can go for option 1

- Ericsson thinks none of the options are necessary and a NW implementation could address this case

- CSCN thinks we could rule out option 3 and could accept option 1

- Xiaomi prefers option 1.

Continue in offline 310

[AT131bis][310][R18 NR NTN] eventD2 (CSCN)

Scope: Continue the discussion on correction for eventD2

Intended outcome: summary of the offline discussion and revised CRs (in R2-2507774, R2-2507775, R2-2507776, R2-2507777)

Deadline for offline discussion summary: Thursday 2025-10-16 11:00

R2-2507774	Report of [AT131bis][110][R18 NR NTN] eventD2	CSCN	discusison	Rel-19	NR_NTN_enh-Core

Proposal 1: Select option2(R2-2507765) to address the case when multiple neighbor cells with moving reference locations fulfil eventD2 and would trigger measurement report.

- Ericsson thinks that we could clarify that if the NW configures multiple eventD2 each eventD2 implicitly corresponds to a reference location, with no need for ASN.1 changes (option 4)

- Apple prefers 2 but it‘s ok to consider 3 and 4, but not 1

We rule out option 1

- QC could accept option 4 also for Rel-18 if it’s worded as a note

- HW and ZTE think option 3 is ok, but we would need to fix the capability

- Ericsson thinks that option 3 has an impact to the capability, not option 4. CSCN agrees

- Ericsson thinks that if we go for a Rel-19 change with a capability we also need a Rel-18 CR to limit the behaviour in Rel-18

WA: We go for a R19 CatF CR adding in ReportConfigNR the PCI corresponding to the reference location in the MO that triggered eventD2. We also introduce a corresponding new R19 UE capability for this (come back in the next meeting with a Rel-18 and a Rel-19 CR)

R2-2506996	Corrections on eventD2	CSCN, ZTE Corporation, Sanechips, Huawei, Hisilicon, OPPO	CR	Rel-18	38.331	18.7.0	5496	-	F	NR_NTN_enh-Core

Revised in R2-2507775

R2-2507775	Corrections on eventD2	CSCN, ZTE Corporation, Sanechips, Huawei, Hisilicon, OPPO	CR	Rel-18	38.331	18.7.0	5496	1	F	NR_NTN_enh-Core

R2-2506997	Corrections on eventD2	CSCN, ZTE Corporation, Sanechips, Huawei, Hisilicon, CATT, OPPO	CR	Rel-18	38.331	18.7.0	5497	-	F	NR_NTN_enh-Core

Revised in R2-2507776

R2-2507776	Corrections on eventD2	CSCN, ZTE Corporation, Sanechips, Huawei, Hisilicon, CATT, OPPO	CR	Rel-18	38.331	18.7.0	5497	1	F	NR_NTN_enh-Core

R2-2506998	Corrections on eventD2	CSCN, ZTE Corporation, Sanechips, Huawei, Hisilicon, OPPO	CR	Rel-18	38.331	18.7.0	5498	-	F	NR_NTN_enh-Core

Revised in R2-2507777

R2-2507777	Corrections on eventD2	CSCN, ZTE Corporation, Sanechips, Huawei, Hisilicon, OPPO	CR	Rel-18	38.331	18.7.0	5498	1	F	NR_NTN_enh-Core

Working Assumption:

1.	We go for a R19 CatF CR adding in ReportConfigNR the PCI corresponding to the reference location in the MO that triggered eventD2. We also introduce a corresponding new R19 UE capability for this (come back in the next meeting with a Rel-18 and a Rel-19 CR)

#### 7.0.2.18	IoT NTN enhancements

(IoT_NTN_enh-Core; leading WG: RAN2; REL-18; WID: RP-223519)

#### 7.0.2.19	Enhanced NR Sidelink Relay

(NR_SL_relay_enh-Core; leading WG: RAN2; REL-18; WID: RP-223501)

R2-2507076	Correction on UE capability for MP split	ZTE Corporation, Sanechips, CATT	CR	Rel-18	38.306	18.7.0	1357	-	F	NR_SL_relay_enh-Core

Agreed in principle

Discussion:

Qualcomm wonder if it is needed considering the capability name.  ZTE clarify that the split DRB is separate from PDCP duplication.

R2-2507077	Correction on UE capability for MP split	ZTE Corporation, Sanechips, CATT	CR	Rel-19	38.306	19.0.0	1358	-	A	NR_SL_relay_enh-Core

Agreed in principle

R2-2507213	Correction to SI reception by remote UE for multi path	LG Electronics Inc.	CR	Rel-18	38.331	18.7.0	5422	1	F	NR_SL_relay_enh-Core	R2-2505543	Revised

Revised in R2-2507553

R2-2507553	Correction to SI reception by remote UE for multi path	LG Electronics	CR	Rel-18	38.331	18.7.0	5422	3	F	NR_SL_relay_enh-Core	R2-2507213

Impact analysis to be added

Agreed in principle with this change, as R2-2507795

R2-2507214	Correction to SI reception by remote UE for multi path	LG Electronics	CR	Rel-19	38.331	19.0.0	5422	2	A	NR_SL_relay_enh-Core	R2-2505543

Withdrawn (needs a new CR number)

R2-2507697	Correction to SI reception by remote UE for multi path	LG Electronics	CR	Rel-19	38.331	19.0.0	5563	-	A	NR_SL_relay_enh-Core

Impact analysis to be added

Agreed in principle with this change, as R2-2507796

R2-2507215	Correction to SI reception by remote UE for multi path	LG Electronics Inc.	discussion	Rel-18	NR_SL_relay_enh-Core

Noted

R2-2507474	U2U Relays, Peer Remote UE Control Plane Procedures	Ericsson, Nokia	CR	Rel-18	38.300	18.7.0	1045	-	F	NR_SL_relay_enh-Core

Steps 4 through 9 to be replaced with steps 5 through 9

Agreed in principle with this change, as R2-2507793

Rel-19 mirror agreed in principle as R2-2507794

Discussion:

Huawei wonder if there are no data from the other side to be transmitted, if the peer procedures would be needed; they understand that if there are data, the peer side will initiate the procedures already, so the spec is OK without the CR.

Samsung think the CR is technically correct, but we have a general assumption in some places that the peer will do the same procedure.  Ericsson understand that it matters here because it is U2U and the setup procedure is controlled by the UEs themselves.  To Huawei’s comment, Ericsson understand that when we set up the link it should be done in both directions.

Apple do not see that step 4 needs to be performed by both sides.  Ericsson think the end-to-end link needs to be set up bidirectionally.  Nokia agree with Apple.

#### 7.0.2.20	NR Sidelink evolution

(NR_SL_enh2-Core; leading WG: RAN1; REL-18; WID: RP-230077)

#### 7.0.2.21	Expanded and improved NR positioning

(NR_pos_enh2-Core; leading WG: RAN1; REL-18; WID: RP-232670)

Including outcome of email discussion [Post131][410][POS] Stage 2 descriptions for Rel-18 positioning (CATT)

R2-2506821	Report of [Post131][410][POS] Stage 2 descriptions for Rel-18 positioning (CATT)	CATT	discussion	Rel-18	NR_pos_enh2-Core

Noted

Discussion:

ZTE wonder why the eDRX extension should not be included, since we have the feature in Rel-18.  Qualcomm indicate that we did not agree on this feature in the positioning session and it was only for eRedCap.  Ericsson point out that it was mentioned in our WID and we are supporting it; they think it would be enough to provide a reference to 38.300 and say it is supported.

CATT confirm that 38.300 has a sentence about the eDRX extension, and they are fine with referring to it.

Huawei indicate that the extension was for RRC_INACTIVE in Rel-18, and they wonder if for us it applies in RRC_IDLE from Rel-17.  CATT think we would need separate sentences in the clauses for positioning in idle and inactive respectively.

Qualcomm note that some reference numbers are missing in the added text.

Qualcomm doubt if there is meaningful impact to positioning from this feature.  vivo agree with Qualcomm because there is no clear indication of the relationship between DRX and positioning.

Huawei checked 38.305 and found that in RRC_INACTIVE, there is no mention of the alignment between PRS and DRX; they think it would be more useful to document in stage 2 that this alignment is for power saving, but if we do not want to be this specific, we could just say the features can be supported together.  Ericsson think we should not capture too much detail about the purpose of configuring something that is supported in the spec, and a simple pointer would be good enough.

Qualcomm wonder what the UE behaviour would be related to this spec change.  Nokia have some sympathy with Qualcomm’s comment and think a simple statement that “this is supported” does not shed much light, so they would prefer to have some linkage to positioning if we have the text at all.

Huawei understand that there should be no impact to UE behaviour.

Ericsson think there is stage 3 text related to the longer SRS periodicity.

Xiaomi wonder if the Rel-19 CR should cover FH.  CATT indicate that the Rel-19 CR here is a true shadow without Rel-19-specific content.  Qualcomm think there is confusion due to two versions of FH for RedCap and non-RedCap, so they think it should be kept separate if captured in stage 2.

R2-2506824	Corrections on LPHAP, carrier phase, bandwidth aggregation and frequency hopping for positioning	CATT, Ericsson, Nokia, ZTE Corporation	CR	Rel-18	38.305	18.6.0	0187	3	F	NR_pos_enh2-Core	R2-2505124

Agreed in principle

R2-2506825	Corrections on LPHAP, carrier phase, bandwidth aggregation and frequency hopping for positioning(R19 Cat. A CR)	CATT, Ericsson, Nokia, ZTE Corporation	CR	Rel-19	38.305	18.6.0	0198	-	A	NR_pos_enh2-Core

Agreed in principle

R2-2506969	Correction on AdditionalSpectrumEmission in SL positioning	ZTE Corporation	CR	Rel-18	38.331	18.7.0	5494	-	F	NR_pos_enh2-Core

Agreed in principle

Discussion:

Huawei wonder if this is only for the dedicated pool; if so, it should be made clear.  ZTE indicate that for the shared pool, the SL frequency is the same as for data, so the frequency is already included in SIB12 with the additional spectrum emission.

R2-2506970	Correction on AdditionalSpectrumEmission in SL positioning	ZTE Corporation	CR	Rel-19	38.331	19.0.0	5495	-	A	NR_pos_enh2-Core

Agreed in principle

R2-2507040	Correction for the description of rangeAndOrDirection	Huawei, HiSilicon	CR	Rel-18	38.355	18.6.0	0016	-	F	NR_pos_enh2-Core	Revised

Revised in R2-2507355

R2-2507355	Correction for the description of rangeAndOrDirection	Huawei, HiSilicon	CR	Rel-18	38.355	18.6.0	0016	1	F	NR_pos_enh2-Core	R2-2507040

Not pursued

Discussion:

Chair wonders if it is a mandatory CR.  Huawei think so.

Qualcomm think it is not needed because this is the interpretation we always use; they understand that the usage of 0..180 to mean -90..90 is typical.  They think for AoA we have a reference to a RAN1 TR in LPP/NRPPa.  Huawei think the SA2 description suggests otherwise, and maybe they need to align to our convention.

Ericsson agree with Qualcomm and think there are other places where we use 0..180; they think something could be clarified in SA2.

CATT agree with Qualcomm about the existing LPP convention; they think it is CT4’s responsibility to translate from LPP representation to SA2’s representation.

R2-2507349	Correction for the description of rangeAndOrDirection	Huawei, HiSilicon	CR	Rel-19	38.355	19.0.0	0017	-	A	NR_pos_enh2-Core

Not pursued

R2-2507246	Correction on RequestLocationInformation for DL-TDOA and DL-AOD	Samsung, Qualcomm	CR	Rel-18	37.355	18.6.0	0562	-	F	NR_pos_enh2-Core

Not pursued (can be done from Rel-19)

Discussion:

Ericsson think it is not critical to fix in Rel-18 but could be done from Rel-19.  Huawei agree with Ericsson and think it is a small issue.

Qualcomm think it could be handled in the ASN.1 review.

Samsung think we could agree to capture it in Rel-19.

Agreement:

RAN2 confirm that “location measurements” in the description of RequestLocationInformation for DL positioning in 37.355 should read “location information” and this issue can be fixed from Rel-19.

R2-2507328	Correction on NCD-SSB Configuration for Positioning	China Telecom	CR	Rel-18	38.331	18.7.0	5524	-	F	NR_pos_enh2-Core

Needs to be revised for ASN.1 backward compatibility

Postponed

Discussion:

ZTE checked RAN1 agreements and did not find an indication that NCD-SSB is supported as a reference.

CATT checked with RAN1 and understand that NCD-SSB can be used, but they are not sure why the offset is required since the UE can still find the NCD-SSB according to the SSB configuration.

China Telecom understand that this is a full configuration, so the offset is needed.

Huawei think we should clarify whether this is for intra- or inter-band; for intra-band they understand there is no need for the offset, and inter-band SSB was introduced for NES and has the offset.  They have no strong view on whether to support it, but they think the CR needs to be revised to be ASN.1 BC.

Nokia looked at the RAN1 parameter list and did not find the offset mentioned, so they understand that the need should be confirmed by RAN1.

Ericsson note that we did SRS for positioning in Rel-16 and the time offset was added only in Rel-17, so there would not be anything in the parameter list; they are not sure if anything is broken without the CR, but they think other parts of the RRC configuration include it and it may not be necessary to duplicate it here.

Ericsson note that for SRS for MIMO we do not provide the offset, but the difference here may be that neighbour cells are also involved; they would like more time to check.

CATT agree with Nokia that it should be checked with RAN1 colleagues.

Huawei agree with Ericsson that we should not cause impact to other features; they think it would be good to consult internally with companies’ RAN1 colleagues.

R2-2507330	Correction on NCD-SSB Configuration for Positioning	China Telecom	CR	Rel-19	38.331	19.0.0	5525	-	A	TEI19, NR_pos_enh2-Core

Postponed

R2-2507533	Correction on processing of sidelink grant on Dedicated SL-PRS resource pool	ASUSTeK	CR	Rel-18	38.321	18.7.0	2131	-	F	NR_pos_enh2-Core

First change to be replaced with “2> if sl-PRS-MaxNum-Transmissions is configured and the number of transmissions of the SL-PRS has not reached the selected maximum number of SL-PRS transmissions; or 2> if sl-PRS-MaxNum-Transmissions is not configured:”

Agreed in principle as R2-2507797

Mirror CR agreed in principle as R2-2507798

Discussion:

Huawei are not sure the third change is correct: There are two SR procedures, one for normal transmission and one for sidelink, and they think we may not need to specify a particular one.  ASUSTeK clarify that the current reference just points to a MAC CE, and the parallel requirements in the same paragraph point to the procedures.

Huawei think the first change may be wrong because removing the condition would remove it also for scheme 2.

LG understand that the CR would remove the reference to the parameter completely.

Xiaomi think the first change is not needed because the parameter will not be checked if it is not configured for scheme 1.  ASUSTeK think then it should be removed from the section that is common to both schemes.

Ericsson think we should fix the misnamed RRC parameter in Rel-18.  Chair thinks there are other cases.  Huawei agree that there are other cases and we could let it pass now.

Withdrawn/Not available

R2-2507152	Correction on RequestLocationInformation for DL-TDOA and DL-AOD	Samsung	CR	Rel-18	37.355	18.6.0	0561	-	F	NR_pos_enh2-Core	Withdrawn

#### 7.0.2.22	Further NR mobility enhancements

(NR_Mob_enh2-Core; leading WG: RAN2; REL-18; WID:RP-233970)

R2-2506726	LS on the handling of inter-DU L2 reset for LTM (R3-255907; contact: Ericsson)	RAN3	LS in	Rel-18	NR_Mob_enh2-Core	To:RAN2

R2-2507400	Handling of inter-DU L2 reset for LTM	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core

R2-2507386	On inter-DU Layer 2 Reset in LTM		Nokia		discussion	Rel-18	NR_Mob_enh2-Core

Discussion

- ZTE shares same view with Nokia that L2 reset is always done for inter-DU

- Ericsson thinks that network control this and if there are network implementations which can support this we should not restrict it.

- Xiaomi thinks that RAN3 should decide it

- Huawei thinks we do not need to prevent network implementation, but we are not asking RAN3 to do any new signalling

- Nokia still thinks some work needs to be done in RAN3

- CATT thinks network implementation can do this but without any RAN3 impact.

- Samsung shares Nokia’s view

RAN2 understanding is that in case of inter-DU L2 reset is always done

[AT131bis][101][MOB] LS to RAN3 of L2 reset (Ericsson)

Scope: Discuss working of the reply LS to send to RAN3 about L2 reset.

Intended outcome: Agreeable LS in R2-2507711.

Deadline: CB on Wednesday.

R2-2507711	Draft Reply LS to RAN3 on L2 reset	Ericsson	LS out	Rel-18	NR_Mob_enh2-Core	To:RAN3

The LS is approved in R2-2507727

R2-2507727	Reply LS to RAN3 on L2 reset	RAN2	LS out	Rel-18	NR_Mob_enh2-Core	To:RAN3

=> Approved

R2-2507616	Correction on the execution of SCG LTM	CATT	CR	Rel-18	38.331	18.7.0	5554	-	F	NR_Mob_enh2-Core

R2-2507630	Correction on the execution of SCG LTM	CATT	CR	Rel-19	38.331	19.0.0	5558	-	A	NR_Mob_enh2-Core

Discussion

- Huawei think change is okay but CR coverpage needs some work

- Xiaomi supports the change

For Rel-19 CR change to be added in section about SCG

CR coverpage should clarify better the consequence if not approved

CB [102] on Thursday. Agreeable in principle CRs in R2-2507712 (R18) and R2-2507713 (R19)

R2-2507712	Correction on the execution of SCG LTM	CATT	CR	Rel-18	38.331	18.7.0	5554	1	F	NR_Mob_enh2-Core

R2-2507713	Correction on the execution of SCG LTM	CATT	CR	Rel-19	38.331	19.0.0	5558	1	A	NR_Mob_enh2-Core

The CRs are in principle agreed

R2-2507026	Correction on stop of cg-RRC-RetransmissionTimer upon configuredGrantTimer expiration	vivo	CR	Rel-18	38.321	18.7.0	2124	-	F	NR_Mob_enh2-Core

R2-2507027	Correction on stop of cg-RRC-RetransmissionTimer upon configuredGrantTimer expiration	vivo	CR	Rel-19	38.321	19.0.0	2125	-	A	NR_Mob_enh2-Core

Discussion

- Ericsson thinks this current text is not a problem and that that field is used also for other cases

- CATT thinks this is not correct

- Huawei wonders what is the effect of the change

- Nokia agree with Huawei and Ericsson that change is not needed

CR is not pursued

R2-2507220	Corrections on validation of reported idle/inactive and reselection measurements	Samsung	CR	Rel-18	38.331	18.7.0	5519	-	F	NR_Mob_enh2-Core

Discussion

- Samsung clarify that EMR is applicable to both NR and LTE

- Huawei points that tab are not used in ASN.1 but rather spaces.

Style in ASN.1 should be fixed and coverpage has a sentence which is not completed

CB on Thursday [103]. Agreeable in principle CRs in R2-2507714 (R18) and R2-2507715 (R19)

R2-2507714	Corrections on validation of reported idle/inactive and reselection measurements	Samsung	CR	Rel-18	38.331	18.7.0	5519	1	F	NR_Mob_enh2-Core

R2-2507715	Corrections on validation of reported idle/inactive and reselection measurements	Samsung	CR	Rel-19	38.331	19.0.0	5564	-	A	NR_Mob_enh2-Core

The CRs are agreed in principle.

R2-2507381	Corrections on Rel-18 UE capabilities for LTM	Huawei, HiSilicon	CR	Rel-18	38.306	18.7.0	1363	-	F	NR_Mob_enh2

R2-2507382	Corrections on Rel-18 UE capabilities for LTM	Huawei, HiSilicon	CR	Rel-19	38.306	19.0.0	1364	-	A	NR_Mob_enh2

Discussion

- MTK is fine with the CR but example in the CR coverpage is wrong

- Nokia is okay with most of changes but ltm-MAC-CE-JointTCI-r18 implies that “joint” TCI are only used for the serving cell and this is not correct

CB on Thursday [104]. Agreeable in principle CRs in R2-2507716 (R18) and R2-2507717 (R19)

R2-2507716	Corrections on Rel-18 UE capabilities for LTM	Huawei, HiSilicon	CR	Rel-18	38.306	18.7.0	1363	1	F	NR_Mob_enh2

R2-2507717	Corrections on Rel-18 UE capabilities for LTM	Huawei, HiSilicon	CR	Rel-19	38.306	19.0.0	1364	1	A	NR_Mob_enh2

The CRs are in principle agreed

R2-2507526	Miscellaneous corrections on mobility enhancements	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.7.0	5540	-	F	NR_Mob_enh2-Core

R2-2507527	Miscellaneous corrections on mobility enhancements	ZTE Corporation, Sanechips	CR	Rel-19	38.331	19.0.0	5541	-	A	NR_Mob_enh2-Core

Discussion

- Nokia thinks that change to procedural text is fine but maybe change in field desciptions are not needed

- Huawei is okay to add field descriptions but most of time are not needed.

Changes related to LTM+Sidelink, LTM+MBS, and LTM+QoE are removed from the CR.

CRs are agreed in principle unseen in R2-2507718 (R18) and R2-2507719 (R19)

R2-2507718	Miscellaneous corrections on mobility enhancements	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.7.0	5540	1	F	NR_Mob_enh2-Core

R2-2507719	Miscellaneous corrections on mobility enhancements	ZTE Corporation, Sanechips	CR	Rel-19	38.331	19.0.0	5541	1	A	NR_Mob_enh2-Core

Withdrawn

R2-2506812	Correction on the execution of SCG LTM	CATT	CR	Rel-18	38.331	18.6.0	5490	-	F	NR_Mob_enh2-Core	Withdrawn

R2-2506813	Correction on the execution of SCG LTM	CATT	CR	Rel-19	38.331	18.6.0	5491	-	A	NR_Mob_enh2-Core	Withdrawn

#### 7.0.2.23	TEI18

R2-2507191	Correction on uplink power control for Type-1 CG-PUSCH [PL RS Type 1 CG]	Ofinno	CR	Rel-18	38.331	18.7.0	5518	-	F	TEI18

-	Xiaomi wants to instead delete “by the SRI field in DCI”. Nokia indicated that in the revision history the “R2-2505783” should be removed.

In-principle agreed but the change should be to remove “by the SRI field in DCI”. And remove “R2-2505783” from the revision history

#### 7.0.2.24	Others

Including NR Others, Multi-WI Rel-18 items, e.g. cross-WI-issues not handled under another WI

8Rx

R2-2506729	LS on 8Rx UE receiver capability definition update request (R4-2509151; contact: China Telecom, Ericsson)	RAN4	LS in	Rel-18	NR_ENDC_RF_FR1_enh2-Perf	To:RAN2

Noted

R2-2507395	Correction on R18 8Rx UE receiver capability definition	China Telecom, Ericsson, Huawei, HiSilicon	CR	Rel-18	38.306	18.7.0	1365	-	F	NR_ENDC_RF_FR1_enh2-Perf

R2-2507396	Correction on R18 8Rx UE receiver capability definition	China Telecom, Ericsson, Huawei, HiSilicon	CR	Rel-19	38.306	19.0.0	1366	-	A	NR_ENDC_RF_FR1_enh2-Perf

In-principle agreed but tick the “N”s in the Other specs affected-field

UL Tx switching

R2-2507167	Correction on UL Tx switching MIMO coherence capabilities	ZTE Corporation	CR	Rel-18	38.306	18.7.0	1359	-	F	NR_MC_enh-Core

R2-2507168	Correction on UL Tx switching MIMO coherence capabilities	ZTE Corporation	CR	Rel-19	38.306	19.0.0	1360	-	A	NR_MC_enh-Core

-	Docomo agrees with this CR. Vivo are OK with the CR. Nokia think that the “R18” is not needed, but are OK with the CRs since its not critical.

Both are in-principle agreed

R2-2507599	Clarification of supported band pairs for UL TX switching	Ericsson	CR	Rel-18	38.331	18.7.0	5553	-	F	NR_MC_enh-Core

-	Nokia think its not very critical. ZTE and Ericsson agrees what the intended behaviour should be but they don’t agree on what current spec or the CR achieves hence an offline is needed.

[AT131bis][603][Maint] UL TX switching (Ericsson)

Scope:

    - Discuss offline (face-to-face preferred, if suitable)

Intended outcome:

    - In-principle agreed CR(s) in R2-2507833, if needed

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2507833	Clarification of supported band pairs for UL TX switching	Ericsson	CR	Rel-18	38.331	18.7.0	5553	1	F	NR_MC_enh-Core

In-principle agreed

Old revisions/Withdrawn

R2-2506880	Correction on R18 8Rx UE receiver capability definition	China Telecom, Ericsson, Huawei, HiSilicon	CR	Rel-18	38.306	18.6.0	1354	-	F	NR_ENDC_RF_FR1_enh2-Perf	Withdrawn

R2-2506881	Correction on R18 8Rx UE receiver capability definition	China Telecom, Ericsson, Huawei, HiSilicon	CR	Rel-19	38.306	18.6.0	1355	-	A	NR_ENDC_RF_FR1_enh2-Perf	Withdrawn

# 8	NR Rel-19

## 8.0	General

R2-2506711	LS on updated Rel-19 RAN1 UE features lists for LTE after RAN1#122 (R1-2506429; contact: NTT DOCOMO, AT&T)	RAN1	LS in	Rel-19	IoT_NTN_Ph3, IoT_NTN_TDD, LTE_terr_bcast_Ph2	To:RAN2	Cc:RAN4

Noted

### 8.0.1	ASN.1 Review

Contributions on common ASN.1 identified issues and cross-WI identified issues.  RILs specific to WI are expected to be discussed in corresponding WI.

Rapporteur will create separate list that only include the RILs to be discussed in common session.  One contribution covering the common session RILs is expected per company.   Additional tdoc can be submitted for co-sourced contributions with 4 or more companies.

R2-2507222	LTE ASN.1 Review file	Samsung	discussion	Rel-19	Late

Noted

R2-2507223	LTE RIL List	Samsung	discussion	Rel-19	Late

Noted

R2-2507224	LTE ASN.1 Class 0 Issues	Samsung	discussion	Rel-19	Late

Noted

R2-2507685	38331 ASN.1 Review file v000	Ericsson	discussion	Rel-19	TEI19

Noted

R2-2507686	38331 ASN.1 Comments file v000	Ericsson	discussion	Rel-19	TEI19

Noted

R2-2507687	38331 ASN.1 RIL list v000	Ericsson	discussion	Rel-19	TEI19

Noted

R2-2507695	LTE ASN.1 Review comment file	Samsung	discussion	Rel-19

Noted

(C)LTM and SBFD RA

O000

R2-2507617	Discussion on cross-WI ASN.1 Issues (O000,O001,O003, O004, O005)	OPPO	discussion	Rel-19	NR_duplex_evo-Core, NR_Mob_Ph4-Core, NR_LPWUS-Core, Netw_Energy_NR_enh-Core, NR_SL_relay_multihop-Core	Late

Noted

Agreements

[O000] R2 confirm SBFD-RACH is not applicable to early UL sync.  Update RRC spec to restrict this.

[O000] R2 confirm SBFD-RACH applicability to CLTM execution.  No further specifications changes are required.

[O000] R2 confirm SBFD-RACH applicability to LTM execution upon CSC for intra-DU case as per current specs

(C)LTM and sidelink

Z152

O001

R2-2507617	Discussion on cross-WI ASN.1 Issues (O000,O001,O003, O004, O005)	OPPO	discussion	Rel-19	NR_duplex_evo-Core, NR_Mob_Ph4-Core, NR_LPWUS-Core, Netw_Energy_NR_enh-Core, NR_SL_relay_multihop-Core	Late

- Agreements

[O001] R2 confirm that LTM/CLTM is not applicable to SL intermediate-relay UE.

[O001] R2 confirm that CLTM is not applicable to SL relay UE.

(C)LTM and QoE

Z153

- ZTE explains that there is a paragraph that explains how the failure is reported that may need to be updated.

The co-existance of LTM and QoE is not supported (i.e. they are not configured at the same time)

(C)LTM and MBS

Z154

The co-existance of LTM and MBS is not specified (i.e. can be supported but no further specification work)

PO bundling, NES and LPWUS

O005

C026

R2-2507236	Discussion about LP-WUS RILs H050, E043, C026, V001, Z052 and V002	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

- RAN2 confirms that LP-WUS in idle/inactive can co-exist with Rel-19 paging adaptation mechanism in NES, and the LP-WUS mechanism will not be affected by paging adaptation.

Noted

R2-2506864	[C026] Co-existence of LP-WUS with paging adaptation	CATT	discussion	Rel-19	NR_LPWUS-Core

- []RAN2 confirms that LP-WUS in idle/inactive can co-exist with Rel-19 paging adaptation mechanism in NES, and the LP-WUS mechanism will not be affected by paging adaptation.

Noted

R2-2507617	Discussion on cross-WI ASN.1 Issues (O000,O001,O003, O004, O005)	OPPO	discussion	Rel-19	NR_duplex_evo-Core, NR_Mob_Ph4-Core, NR_LPWUS-Core, Netw_Energy_NR_enh-Core, NR_SL_relay_multihop-Core	Late

- [O005] R2 discuss to support co-configuration of LP-WUS and PO-bundling, i.e., separate LP-WUS configuration for legacy PO and R19 bundled PO.

Noted

RAN2 confirms that LP-WUS in idle/inactive can co-exist with Rel-19 paging adaptation mechanism in NES, and the LP-WUS mechanism will not be affected by paging adaptation.  Further discussions can take place in LP-WUS session on addressing this.

PO bundling, NES and SL Relay

O004

R2-2507617	Discussion on cross-WI ASN.1 Issues (O000,O001,O003, O004, O005)	OPPO	discussion	Rel-19	NR_duplex_evo-Core, NR_Mob_Ph4-Core, NR_LPWUS-Core, Netw_Energy_NR_enh-Core, NR_SL_relay_multihop-Core	Late

[O004] R2 not purse co-configuration of PO bundling and SL Relay in a same cell.

NES RA and SBFD RA

O003

R2-2507617	Discussion on cross-WI ASN.1 Issues (O000,O001,O003, O004, O005)	OPPO	discussion	Rel-19	NR_duplex_evo-Core, NR_Mob_Ph4-Core, NR_LPWUS-Core, Netw_Energy_NR_enh-Core, NR_SL_relay_multihop-Core	Late

[O003] R2 confirm that it is not supported to configure addlRACH-Config-Adapt-r19 within rach-ConfigCommon (no suffix) and rach-ConfigCommon-r17, together with sbfd-RACH-SingleConfig-r19 enabled, but it is not supported to configure addlRACH-Config-Adapt-r19 within sbfd-RACH-DualConfig-r19.

NES Case1 with respect of the SSBless Scell.

E024

R2-2507334	Discussion on RILS E023, E024, E025, X201, O006, J002, H128, H129, J005, Z101, Z102, V503 	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core	Late

Proposal 1	RAN2 to conclude that Case 1 does not require always on SSB, even in reference cells.

Continue in NES session and check how Rel-18 is impacted by this

Enhancing the readability of RRC procedure text

H202

R2-2507055	Enhancing the readability of RRC spec  [H202]	Huawei, HiSilicon	discussion	TEI19

Proposal 1: 	For better readability of RRC procedure text, RAN2 to separate UE procedures belonging to different features within a long section to separate section for each feature. Take the TP in appendix A as an example.

-	Qualcomm and Samsungs thinks we can try.   Samsung think that we should examine on a case by case basis.  Huawei thinks we can do by email, examine chapters and agree which chapter to re-examine and then propose TPs.   Nokia thinks that we should do it as a separate effort from ASN.1 and the freeze.   Oppo is concerned that if we do it for all features it will be a mess so we should do it one by one.

For Rel-19 proposed text we can do an exercise where we identify sections which can be separated, without introducing inconsistencies in previous releases.  Determine whether we do any further updates in RAN2#132

[POST131bis][020][RRC] Readability of RRC procedural text (Huawei)

Intended outcome:  Identify sections that can benefit from update procedure and doesn’t introduce inconsistencies in previous releases.   No draft CR or TPs for RAN2#132

Deadline:  long

Proposal 2:	For better readability of ASN.1, similar to SetupRelease, RAN2 to use more parameterized types for CHOICE with sub-fields. Adopt the TP in appendix B.

-	Qualcomm, Mediatek and Samsung thinks it is a bit late for NR.   Huawei understands and thinks we can do it for 6G.

Not supported

.

-	Ericsson thinks we can do better in general

OtherConfig for XR, LPWUS, AIML, NTN UAI report

H201

RRC Rapp assumes issue resolved at CR impl.

ON ASN.1 review for next meeting (READ THE MAIL FROM RAPPORTEUS)

-	WI rapporteurs should update the RIL list and status

-	New RILs can be introduced in main file.

-	Rapporteurs update the WI CRs (short email discussion review) and resulting result goes into the ASN.1 folder to continue review on separate WI CRs.

Discussion

-	Ericsson thinks that some issues that impact more than one WI they should be lifted into separate CR.

### 8.0.2	Other

This AI is reserved for Rel-19 LSs from other WGs.  No contributions are expected on these LSs for this meeting

Reserved for UE capability rapporteur input .

R2-2506710	LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#122 (R1-2506426; contact: NTT DOCOMO, AT&T)	RAN1	LS in	Rel-19	NR_AIML_air, NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_Mob_Ph4, NR_NTN_Ph3, NR_MC_enh2, TEI19, NR_LBCA_Sw	To:RAN2	Cc:RAN4

Noted

R2-2506720	LS on Rel-19 higher layers parameters list Post RAN1#122 (R1-2506626; contact: Ericsson)	RAN1	LS in	Rel-19	NR_AIML_air, NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_Mob_Ph4, NR_XR_Ph3, NR_NTN_Ph3, IoT_NTN_Ph3, IoT_NTN_TDD, NR_MC_enh2, NR_LBCA_Sw, LTE_terr_bcast_Ph2, TEI19	To:RAN2, RAN3	Cc:RAN4

Noted

[POST131bis][003][RRC] RAN1 parameters (Ericsson)

Intended outcome: Report to RAN1 the final parameter names

Deadline:  Long

R2-2506734	LS on Rel-19 RAN4 UE feature list for NR (version 2) (R4-2511884; contact: CMCC)	RAN4	LS in	Rel-19	NR_ENDC_RF_Ph4, NonCol_intraB_ENDC_NR_CA_Ph2, NR_ATG_enh, NR_RRM_Ph5, Netw_Energy_NR_enh, NR_LPWUS, NR_Mob_Ph4, NR_XR_Ph3, NR_FR1_lessthan_5MHz_BW_Ph2, NR_LBCA_Sw, NR_FR1_7MHz_BW, NR_IoT_NTN_req_test_enh, NR_AIML_air, NR_NTN_Ku_bands, NR_NTN_Ph3	To:RAN2	Cc:RAN1

Noted

R2-2507586	Corrections on Rel-19 RAN1/4 UE capability	Xiaomi	CR	Rel-19	38.306	19.0.0	1370	-	F	NR_LPWUS, NR_MIMO_Ph5

Use a baseline for next revision

R2-2507587	Corrections on Rel-19 RAN1/4 UE capability	Xiaomi	CR	Rel-19	38.331	19.0.0	5547	-	F	NR_LPWUS, NR_AIML_air, NR_ENDC_RF_Ph4

-	Lenovo is concerned that this is not aligned with the feature list.   Vivo confirms that it reflects RAN1 intention.

Use a baseline for next revision

[POST131bis][004][UE caps] UE capability CR (Xiaomi)

Intended outcome: Update UE capability CRs post RAN1/RAN4 LS and combine with RAN2 CRs.

Deadline:  long

R2-2507144	Discussion on remaining issues of UE capability	OPPO	discussion	Rel-19	TEI19, NR_LBCA_Sw

Noted

R2-2506721	Reply LS on NR Femto node shared by PLMN and PNI-NPN (R3-252337; contact: LGE)	RAN3	LS in	Rel-19	5G_Femto, NR_WAB_5GFemto-Core, eNPN	To:SA2	Cc:RAN2

Noted

R2-2506745	Reply LS on FS_VMR_Ph2 solution impacts to RAN (Additional ULI) (S2-2504110; contact: Qualcomm)	SA2	LS in	Rel-19	VMR_Ph2	To:RAN3	Cc:RAN2

Noted

## 8.1	AI/ML for NR air interface

(NR_AIML_air-Core; leading WG: RAN1; REL-19; WID: RP-250792 and SID: RP-243245)

Time budget: 0 TU

Tdoc Limitation: 3 tdocs

### 8.1.1	Organizational

LS, Rapporteur input, including workplan.

Including LSs R2-2506752, R2-2506751, and R2-250675 on R20 data collection and data set and parameter sharing.  No contributions should be submitted to address these LSs.

LS

RAN2 in ‘To’

R2-2506722	Reply LS on Logged Data Handling During Handover (R3-255824; contact: Nokia)	RAN3	LS in	Rel-19	NR_AIML_air-Core	To:RAN2	Cc:SA5

Noted

R2-2506751	Reply LS on signalling feasibility of dataset and parameter sharing (S2-2508104; contact: Samsung)	SA2	LS in	Rel-19	NR_AIML_air-Core	To:RAN2	Cc:RAN, RAN1, RAN3, SA, SA3, SA5

Noted

R2-2507707	Discussion on SA2 LS on non-OTA data set transfer	Samsung	discussion	Rel-19	NR_AIML_air-Core

Reply is sent to SA2 clarifying to answer their questions on the details of the options asked (based on existing RAN2 agreements)

[AT131bis][007][AI PHY] LS to SA2 on dataset parameter options (Samsung)

Intended outcome: Agree to LS over email

Deadline:  Thursday

R2-2507907	Reply LS on signalling feasibility of dataset and parameter sharing	RAN2	LS out	Rel-19	NR_AIML_air_Ph2	To:SA2	Cc:RAN, RAN1, RAN3, SA, SA3, SA5

Remove RAN and SA from cc

The LS is approved with change in R2-2507929

R2-2506757	Reply LS on signalling feasibility of dataset and parameter sharing (S5-254083; contact: Huawei)	SA5	LS in	Rel-19	NR_AIML_air-Core	To:RAN2	Cc:RAN, SA, RAN1, RAN3, SA2, SA3

Noted

R2-2507134	LS on input data from UE for case 3b (S2-2507583; contact: Samsung)	SA2	LS in	Rel-19	NR_AIML_air, NR_AIML_air-Core, AIML_CN	To:RAN2	Cc:CT1, CT4

Noted

R2-2507148	[Draft] Reply LS on input data from UE for Case 3b	Samsung	LS out	Rel-19	NR_AIML_air-Core	To:SA2	Cc:CT1, CT4

-	Nokia explains that we didn’t even discuss case 3b, so we have to indicate that RAN2 doesn’t support case 3b in Rel-19.  Ericsson thinks that we didn’t do any enhancements, but it doesn’t mean we don’t support it.  Qualcomm, Apple and Vivo agrees and vivo thinks we can explain a bit about how it works.   Apple points out that we had an agreement that it can be supported without RAN2 impact.

Both can be supported by existing LPP protocol.   Add that RAN2 didn’t study the details of case 3b, but we assume the ground truth is the location position in the LMF.

[AT131bis][008][AI PHY] Reply LS on positioning (Samsung)

- -	Intended outcome: agree by email

- -	Deadline:  Thursday

R2-2507922	[Draft] Reply LS on input data from UE for Case 3b	Samsung	LS out	Rel-19	AIML_CN, NR_AIML_Air, NR_AIML_air-Core	To:SA2	Cc:CT1, CT4

The LS is approved

R2-2507460	Open Issues LCM for Positioning	Ericsson	discussion	Rel-19	NR_AIML_air-Core	Late (Moved from 8.1.2.3)

Proposal 1: Reply LS to SA2 says “existing LPP procedure can be reused for data collection with regards to ground truth data and its related data (e.g., quality indicator of ground truth data, time stamp of ground truth data)”.

Rel-20 LSs moved from 9.1

R2-2506744	LS on specification of dataset and model parameters exchange (RP-252966; contact: Qualcomm, InterDIgital)	RAN	LS in	Rel-20	NR_AIML_air_Ph2	To:SA	Cc:SA2, SA5, RAN2

Noted

R2-2506752	LS on UE data collection and data transfer (S2-2508119; contact: Nokia)	SA2	LS in	Rel-20	FS_AIML_CN_Ph2	To:RAN2	Cc:RAN1, RAN3

[AT131bis][009][AI PHY] Reply LS to SA2 on data collection (Nokia)

Intended outcome: Agree to LS

Deadline:  CB Friday

R2-2507708	Offline on Reply LS on UE data collection and data transfer (S2-2508119/R2-2506752)	Nokia (Rapporteur)	discussion	Rel-20	FS_AIML_CN_Ph2

Not treated

R2-2507901	Report about [AT131bis][009][AI PHY] Reply LS to SA2 on data collection	Nokia	discussion	Rel-19	NR_AIML_air-Core

Noted

R2-2507902	Reply LS on UE data collection and data transfer	Nokia	LS out	Rel-19	NFS_AIML_CN_Ph2	To:SA2	Cc:RAN1, RAN3

-	Xiaomi thinks that it is important to tell SA2 that RAN2 will work on it in R20 and decisions will be made in RAN.

-	Mediatek thinks that it is important for RAN2 to address these issues in R20. Vivo is concerned that it would give impression that SA2 will have to wait.

-	Apple thinks it should be included as RAN2 will be studying this.

-	Oppo thinks that we didn’t discuss UE radio capabilities

RAN2 concluded that in some use-cases additional information (e.g. UE data collection and feature related radio capabilities, and user consent) may be relevant for selection of UEs, but RAN2 have not discussed where (i.e. gNB, CN, UE server) this information would be considered.

Add the following: SA2 can continue their work and take these answers into account to provide feedback to RAN by December.   Further RAN2 would like to indicate to SA2 that data collection will be further discussed as part of 5GA AI/ML for PHY Rel-20.

The LS is approved in R2-2507928 with the last two paragraphs above

RAN2 in ‘CC’ (To be Noted)

R2-2506755	Reply LS on per-UE UE performance metrics (S5-253854; contact: Huawei)	SA5	LS in	Rel-18	NR_AIML_NGRAN-Core	To:RAN3	Cc:RAN2

Noted

R2-2506759	Reply to LS on Continuous MDT (S5-254110; contact: Ericsson, Magenta, CATT, ZTE, Huawei)	SA5	LS in	Rel-19	NR_AIML_NGRAN_enh-Core	To:RAN3	Cc:RAN2

Noted

CRs, open issues, email discussion reports, RILs

37.320

R2-2507421	TS 37320 Open Issues for Air Interface Feature	Huawei, HiSilicon	other	Rel-19	NR_AIML_air-Core

Conclusion: Based on the email discussion, the changes due to the following issues are agreed:

Issue-01, Issue-02, Issue-03, Issue-04, Issue-05 (merged into Issue-06), Issue-06, Issue-07 (changes proposed by Apple).

Noted

R2-2507420	Correction on AI for Air Interface Feature in TS 37320	Huawei, HiSilicon	CR	Rel-19	37.320	19.0.0	0146	-	F	NR_AIML_air-Core

Endorsed

37.355

R2-2507411	Remaining LPP open issues for feature "AI/ML for NR air interface"	Qualcomm Incorporated (Rapporteur)	discussion	Late

Noted

R2-2507412	Corrections to DL AI/ML Positioning	Qualcomm Incorporated (Rapporteur)	draftCR	Rel-19	37.355	19.0.0	F	NR_AIML_air-Core	Late

The CR is endorsed and will be updated after ASN.1 review

38.300

R2-2506958	Report of TS38300 Open Issues on AI for Air Interface Feature	vivo(Rapporteur)	report	NR_AIML_air-Core

Conclusion: There is no open issues requiring online discussion

Noted

R2-2506959	Introduction of AI for Air interface feature in 38300	vivo(Rapporteur)	draftCR	Rel-19	38.300	19.0.0	NR_AIML_air-Core	Withdrawn

R2-2506995	Introduction of AI for Air interface feature in 38300	vivo(Rapporteur)	CR	Rel-18	38.300	18.7.0	1006	2	B	NR_AIML_air-Core	R2-2506498

=> Revised in R2-2507688

R2-2507688	Introduction of AI for Air interface feature in 38300	vivo(Rapporteur)	CR	Rel-18	38.300	18.7.0	1006	3	B	NR_AIML_air-Core

R2-2507696  Introduction of AI for Air interface feature in 38300	vivo(Rapporteur)	CR	Rel-19	38.300	18.7.0	1006	3	B	NR_AIML_air-Core

Use as a baseline for further review

[POST131bis][010][AI PHY] stage 2 CR (Vivo)

Intended outcome:  CR to be submitted for approval next meeting

Deadline:  Long

38.305

R2-2506779	Summary of open issue email discussion on 38.305 CR for AIML Positioning	CATT	discussion	Rel-19	NR_AIML_air-Core

Do not capture the following sentence in 38.305 Clause 7.6.1 for “Procedures for On-Demand PRS transmission” according to RAN2#131 agreement “The UE asks specific TRPs for PRS transmission with on-demand PRS configuration, i.e., within NR-On-Demand-DL-PRS-Request”:“UE-initiated On-Demand PRS transmission procedure allows the UE to request DL-PRS configuration information for specific TRPs.”

Do not capture the following RAN2 understanding in 38.305: “RAN2 understand that when multiple UE-based positioning methods are requested by an LMF, the derivation of a single location estimate by the UE is up to UE implementation.”

Noted

R2-2506780	Corrections on 38.305 CR for AIML Positioning	CATT	CR	Rel-19	38.305	18.6.0	0197	-	F	NR_AIML_air-Core

Remove sentence from above first agreement

The CR is endorsed with the sentence deleted above.  Any comments on the CR or new identified issues should be sent directly to rapporteur.

38.306

R2-2507589	Report of open issue in [POST131][043][AI PHY] UE capabilities (Xiaomi)	Xiaomi	discussion	Rel-19

-	Samsung and ZTE think it is important to have the UE capability indication.  Apple thinks the assumption is that the data is 100 or 500bits.  Samsung thinks that RAN1 didn’t consider the RAN2 impact as the UE would have to log for several seconds.   Xiaomi thinks that we have a minimum of 64kB but it can support more. We can consider for next generation to indicate.   Qualcomm, Oppo,  and Vivo agrees we don’t  need the capability.  Huawei, BT, CMCC, and Ericsson thinks that if the UE supports it should indicate as if the network knows it would select UEs differently and set the threshold differently.

-	Mediatek thinks that the network can chose multiple UEs to collect the data so for Rel19 we don’t need additional capability.

No UE capability is introduced to indicate ‘whether UE can support other memory sizes for logged measurement of network-side data collection’ in Rel-19.

R2-2507588	Corrections on AI air UE capability	Xiaomi, Oppo	draftCR	Rel-19	38.306	19.0.0	F	NR_AIML_air

The CR is endorsed.  Any comments on the CR or new identified issues should be sent directly to rapporteur.

38.331

R2-2507681	AIML Comments file	Ericsson	report

Noted

R2-2507682	AIML Review file	Ericsson	report

Noted

R2-2507680	Corrections to AIML for NR air interface	Ericsson	CR	Rel-19	38.331	19.0.0	5561	-	F	NR_AIML_air-Core

[POST131bis][011][AI PHY] RRC CR (Ericsson)

Intended outcome:  Continue reviewing and updating CRs and get input on remaining ASN.1 open issues using the general ASN.1 process.

Deadline:  Short

=> Endorsed in R2-2507934

R2-2507934	Corrections to AIML for NR air interface	Ericsson	CR	Rel-19	38.331	19.0.0	5561	1	F	NR_AIML_air-Core

=> Endorsed

### 8.1.2	Functionality based LCM

Corrections only.  Companies should follow guidance from rapporteurs.

#### 8.1.2.1	LCM for NW-sided model for Beam Management use case

LCM related to NW-sided model for beam management use case.

No contributions expected for this meeting.

#### 8.1.2.2	LCM for UE-sided model for Beam Management use case

Corrections only.  Companies should follow guidance from rapporteurs.

ASN.1 RILs

[N021]/[H003]/[A105]/[S047] ‘Choice hierarchy for CSI prediction and beam prediction – [Proposed Status: ToDo]

- To be treated in Offline considering relevant proposals e.g. R2-2506778 (P1-P5), R2-2507090 (P3), R2-2507117 (P1-P5), R2-2507338 (P7), R2-2507534 (P3), R2-2507654 (P2), R2-2507670 (P1-P3), and R2-2507673 (P3).

[AT131bis][012][AI PHY] Offline on RILs (Ericsson)

Intended outcome: Agree to RILs

Deadline:  Thursday

R2-2507698	Report of [AI/ML (ASN.1 issues) [[N021]/[H003]/[A105]/[S047] and [Z004][J008][J009]	Ericsson	discussion

Noted

Agreements

1	[N021/H003/A105/S047] The parameters for beam management performance monitoring and CSI prediction performance monitoring are grouped in two separate fields.

2	[N021/H003/A105/S047] The parameters for beam management inference and beam management UE-side data collection are grouped in the same field. This field’s name should be general enough to cover both inference and data collection.

3	[N021/H003/A105/S047] (7/11 companies agree) A CHOICE structure is kept for the fields containing parameters introduced in CSI-ReportConfig for AIML PHY Rel-19.

4	[Z004/J008/J009] The UE logs a cell/configuration ID in VarCSI-LogMeasReport only when a measurement result for this cell/configuration is logged (and not upon receiving the logging configuration).

5	[Z004/J008/J009] The UE discards logged cell/configuration IDs from VarCSI-LogMeasReport, if there are no more logged measurements for them after reporting measurements in UEInformationResponse.

6	[H008/B204] RILs H008/B204 to be resolved in the CR, based on changes for N021/H003/A105/S047.

7	[S050] RIL S050 to be resolved after RAN2#131bis.

8	[H002] No specification changes are made to explicitly exclude or to enhance retaining logged data for LTM.

9	[Z005/H009] RILs Z005/H009 to be resolved after receiving a reply from RAN1.

10	[Z007] RIL Z007 to be resolved after RAN2#131bis.

[E040] Redundant UE data collection config request – [Proposed Status: ToDo]

R2-2507678	Discussion on RILs for LCM for UE-side models for beam management (E040, E041, E042, B206, O300, O301, S045, N114)	Ericsson	discussion

Proposal 1: Introduce the ID of the candidate data collection configuration (dataCollectionCandidateConfigId-r19) in the configuration of the corresponding UE-side data collection configuration (in CSI-ReportConfig), based on the TP above.

Noted

R2-2506960	Discussion on RILs [E040], [E041], [N028], [E042], [H002], [X003], [Z001,Z002]	vivo	discussion	NR_AIML_air-Core

Proposal 1. (E040) No specification change is needed, as it is reasonably assumed that the UE can correctly determine whether to receive the UE-side data collection configuration corresponding to the requested candidate UE-side data collection configuration.

Noted

Discussion

-	Vivo, Nokia, Apple don’t think anything is needed.   Apple explains that a reasonable implementation can handle this.

-	Ericsson asks if we can at least have a clarification note.

No specification change is needed, as it is reasonably assumed that the UE can correctly determine whether to receive the UE-side data collection configuration corresponding to the requested candidate UE-side data collection configuration.

[E041] Associated ID validity – [Proposed Status: ToDo]

R2-2507678	Discussion on RILs for LCM for UE-side models for beam management (E040, E041, E042, B206, O300, O301, S045, N114)	Ericsson	discussion

Proposal 2: The network should be able to indicate to a UE that an associated ID is no longer valid, so that the UE (and/or associated training server) can handle its corresponding UE-side model accordingly, e.g. release or retrain it.

Proposal 3: RAN2 to introduce the TP for inclusion of a version number for a provided associated ID in the WI RRC CR for AI/ML for Phy.

Noted

R2-2507118	Open issues on LCM of AI/ML based beam management (including E041/E042/X003/H010/E040/Z001/Z002)	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 3 (RIL E041): It is left to NW implementation to maintain validity of associated ID in Rel-19, e.g. NW updates or de-configures the associated ID when it is out of date.

Noted

It is left to NW implementation to maintain validity of associated ID in Rel-19, e.g. NW updates or de-configures the associated ID when it is out of date.

[E042] Dedicated flag to configure applicability reporting via UAI – [Proposed Status: ToDo]

R2-2507678	Discussion on RILs for LCM for UE-side models for beam management (E040, E041, E042, B206, O300, O301, S045, N114)	Ericsson	discussion

Proposal 4	RAN2 to discuss the following three signalling options for configuring applicability reporting via UAI:

a)	Keep the flag reportApplicabilityUAI within applicabilityReportConfig;

b)	Remove the flag reportApplicabilityUAI and assume that applicability reporting via UAI is implicitly configured when applicabilityReportConfig is included in otherConfig;

c)	Move the flag reportApplicabilityUAI one level up, directly under otherConfig.

Noted

Option a)

R2-2507338	Discussion on RILs [E041][E042][Z001][Z002][H010][Z007][E040][N021][H003][B204][X004] for AIML	LG Electronics Inc.	discussion	Rel-19

Proposal 2.   [E042] Keep the flag reportApplicabilityUAI-r19 within applicabilityReportConfig, as it is.

Noted

Option b)

R2-2507090	Remaining issues in LCM for BM use case	Samsung	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: (E042) RAN2 agree with Solution 1) i.e. remove the separate flag reportApplicabilityUAI-r19 and assume that applicapility reporting via UAI is implicitly configured when applicabilityReportConfig is included in otherConfig.

Noted

Option c)

R2-2507534	Discussion on open issues for LCM	ASUSTeK	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: [E042] Prefer solution 3) to move the flag reportApplicabilityUAI-r19 one level up, directly under otherConfig.

Noted

-	Qualcomm, Apple, and Xiaomi think that the feature is broken unless the UAI is always configured.    Interdigital indicates that we agreed it is optional, but agrees with Qualcomm and Xiaomi.   Ericsson doesn’t think it is broken and we would make a lot of changes if we have to remove this.

-	Qualcomm thinks that the UE behaviour is not clear when applicability reporting is not configured.

-	Interdigital thinks that we can add a note that it is expected that the network always configures applicabliyt reporting.   ZTE thinks that for option A we have to always have UAI configured but for options B it can be optional.   Xiaomi thinks this is applicable for both, as even for option B the UE can change applicability.

-	Qualcomm explains that for CSI report config the network needs to know if applicability has changed.

[E042] Applicability reporting via UAI is always enabled for option A and B.   Remove flag from RRC CR and update procedural text to indicate that it is configured if option A or B are configured.  Wording offline.

[Z001]/[Z002] releaseConfigurationPreference indication – [Proposed Status: ToDo]

R2-2507295	On the remaining issue and RILs Z001 Z002 for LCM of UE side model	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 3: [Z001/Z002] IE ApplicabilitySetConfig is not applicable for the inference related parameter sets. RAN2 to adopt the text proposal provided in the Annex.

Noted

R2-2507118	Open issues on LCM of AI/ML based beam management (including E041/E042/X003/H010/E040/Z001/Z002)	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 8 (RIL Z001/Z002): Reject Z001/Z002, i.e., allow the UE to indicate “release” for one set of inference related parameters in OtherConfig in applicability report (via RRCReconfigurationComplete/UAI).

Noted

PropReject (RIL Z001/Z002) i.e., allow the UE to indicate “release” for one set of inference related parameters in OtherConfig in applicability report (via RRCReconfigurationComplete/UAI).

[X003] Redundant indication of dataCollectionStart – [Proposed Status: ToDo]

R2-2507652	[X003][O301/S045/N114][X004][E040]Discussion on open issues of UE data collection	Xiaomi

Discussion

- Proposal: [X003] is marked as PropAgree. TP in section 4.1 is adopted.

Noted

R2-2507476	Discussion on UE-sided data collection-related RILs: [E040], [X003], [X004]	InterDigital	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: [X003] RAN2 understands UE may send a start indication alone to request the dataCollectionCandidateconfiglist (if not already provided). UE does not re-send start indication when subsequently indicating the preferred configuration.

Noted

R2-2507925

Agreements

1	[X003]: Change the name of the “start” indication to a “request” indication. The condition of sending this “request” indication (if it is configured by NW to do so) is the UE has no candidate list. Update the procedural text in RRC to reflect this condition.

2	[O301]/[S045]/[N114]: Enhancing the “request” indication to be per-use case and/or per-serving cell will not be pursued in Release 19.

Noted

[O301]/[S045]/[N114] Missing purpose for UE-side data collection request – [Proposed Status: ToDo]

R2-2506764	[O301] Missing purpose for UE side data collection request	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: [O301] RAN2 to discuss which solution below is preferred to address the issue:

Solution 1: Along with ‘start’ indicator, UE should also indicate the data collection use case type, e.g. {BM, CSI prediction, spare 2, spare 1}.

Solution 2: ‘start’ indicator is signaled per use case.

R2-2507090	Remaining issues in LCM for BM use case	Samsung	discussion	Rel-19	NR_AIML_air-Core

Proposal 6: (S045) the start indication (i.e., dataCollectionStart-r19) is introduced per serving cell, and the TP proposed in S045 is adopted.

[X004]/[S041] Candidate data configuration – [Proposed Status: ToDo]

R2-2507652	[X003][O301/S045/N114][X004][E040]Discussion on open issues of UE data collection	Xiaomi	discussion

[X004] is marked as PropAgree. TP in 4.2 is adopted.

[H010] Report configuration type in ApplicabilitySetConfig – [Proposed Status: ToDo]

R2-2507673	Remaining issues (including H003, H008, H010) in LCM for UE-sided model for BM/CSI prediction	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: [H010] In ApplicabilitySetConfig-IE, reportSlotOffsetList, pucch-CSI-ResourceList and p0alpha in reportConfigType are not relevant to applicability check. The parameter reportConfigType can be simplified to ENUMERATED {periodic, aperiodic, semiPersistentOnPUCCH }. The TP is in section 4.1.

Proposal 2-1: [H010] If Proposal 2 cannot be agreed directly by RAN2, RAN2 should check this issue with RAN1.

Noted

R2-2507118	Open issues on LCM of AI/ML based beam management (including E041/E042/X003/H010/E040/Z001/Z002)	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 6 (RIL H010): Reject H010 because reportSlotConfig/reportSLotOffsetList are needed for CPU occupation calculation and thereby necessary for the UE to determine applicability of ApplicabilitySetConfig-r19 (i.e. option B).

Noted

(RIL H010) PropReject

[C083]/[C084] AddModList for applicabilityConfigList and applicabilitySetConfigList – [Proposed Status: ToDo]

R2-2506777	Discussion on RIL[C083][C084]	CATT, CBN	discussion	Rel-19	NR_AIML_air-Core

[C083][C084] Adopt the AddModList and ReleaseList IE structure for both applicabilityConfigList-r19 and applicabilitySetConfigList-r19.

[B206]/[O300] Transfer of applicability information in HO command preparation – [Proposed Status: ToDo]

R2-2506927	[B206][O300] Incomplete applicability info during handover	Lenovo, Oppo, Samsung, ZTE, Apple	discussion	Rel-19	Late

Proposal 1	RAN2 seeks for a solution that allows the source gNB to transfer complete applicability information to target gNB

a.	Alternative 1： [B206] Allow UAI in HandoverPreparationInformation message to convey complete applicability information.

b.	Alternative 2: [O300] UE reports complete applicability information in UAI.

c.	Alternative 3: A new IE in HandoverPreparationInformation message to convey complete applicability information.

Noted

R2-2507652	[X003][O301/S045/N114][X004][E040]Discussion on open issues of UE data collection	Xiaomi	discussion

Proposal 5: [B206] is PropReject. The target gNB can always request UE to report its full applicability information that is applicable to the target gNB after UE’s handover.

Noted

Agreements

[B206][O300] A new IE in HandoverPreparationInformation message to convey complete applicability information.

[H008]/[B204] Configuration restrictions in predictionConfiguration – [Proposed Status: ToDo]

R2-2507673	Remaining issues (including H003, H008, H010) in LCM for UE-sided model for BM/CSI prediction	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

Proposal 4 [H008]: Field description for predictionConfiguration-r19 needs to be added as follows, in order to ensure that it is always set in alignment with reportQuantity-r19.

R2-2507338	Discussion on RILs [E041][E042][Z001][Z002][H010][Z007][E040][N021][H003][B204][X004] for AIML	LG Electronics Inc.	discussion	Rel-19

Proposal 8. 	[B204] Adopt the revised TP for B204, i.e., clarify that ‘none-BM-r19’ and ‘none-CSI-r19’ are intended for UE-side data collection in the field description for reportQuantity.

[S050] UAI retransmission after mobility – [Proposed Status: ToDo]

R2-2507090	Remaining issues in LCM for BM use case	Samsung	discussion	Rel-19	NR_AIML_air-Core

Proposal 9: [S050] UE does not resend the UAI for applicability reporting in below cases:

a.	If the UAI was send 1s before the handover execution.

b.	After LTM cell switch or CHO execution.

[S049] Applicability reporting needs to consider RRCResumeComplete – [Proposed Status: changed to PropAgree]

R2-2507090	Remaining issues in LCM for BM use case	Samsung	discussion	Rel-19	NR_AIML_air-Core

Proposal 8: [S049] Consider applicability reported in RRCResumeComplete while reporting applicability in UAI or RRCReconfigurationComplete. S049 is agreed.

Other issues

Distinguish single vs. multi-cell Associated ID

R2-2507118	Open issues on LCM of AI/ML based beam management (including E041/E042/X003/H010/E040/Z001/Z002)	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 1 (Remaining FFS): On top of 24-bit associated ID, introduce 1-bit indication on whether it is applied to single cell or multiple cells.

Noted

R2-2507090	Remaining issues in LCM for BM use case	Samsung	discussion	Rel-19	NR_AIML_air-Core

Proposal 10: No explicit indication to distinguish cell specific and multi-cell specific associated ID.

Noted

No explicit indication to distinguish cell specific and multi-cell specific associated ID.

Periodic CSI report changing from ‘not applicable’ to ‘applicable’

R2-2507295	On the remaining issue and RILs Z001 Z002 for LCM of UE side model	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: When the previously configured inference configuration with periodic CSI reporting changes from "inapplicable" to "applicable", and the applicability report is transmitted via the RRCReconfigurationComplete , UE shall not activate this inference configuration automatically. The handling should be consistent with that when the report is sent via UAI.

-	Oppo thinks that we can just rely on UAI

-	Xiaomi thinks that we should only send the UAI not RRCreconfiguraiton complete as this just confuses the situations.  Qualcomm thinsk that the UE should use RRC complete if it is available.

- -	Interdigital thinks we can capture this Understanding is: when the configuration becomes from inapplicable to applicable the UE sends an UAI.   It is expected that the network can reconfigure the UE.

Noted

Not Treated

AIML support for SCG cells in NR-DC

R2-2506778	Discussion on RIL[N021][H003]	CATT	discussion	Rel-19	NR_AIML_air-Core

R2-2506927	[B206][O300] Incomplete applicability info during handover	Lenovo	discussion	Rel-19	Late

R2-2507117	Discussion on RIL issues related to predictionConfiguration-r19 (A105/N021/H003)	Apple, ZTE Corporation, Sanechips	discussion	Rel-19	NR_AIML_air-Core

R2-2507181	On Simplifying Procedures and ASN.1 for AI/ML	Nokia	discussion	Rel-19	NR_AIML_air-Core

R2-2507295	On the remaining issue and RILs Z001 Z002 for LCM of UE side model	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2507345	Discussion on RIL issues related to [Z001/Z002] [E042] [X003] LCM for UE-sided Model for Beam Management	SHARP	discussion

R2-2507475	Discussion on Applicability-related RILs: [E041], [E042], [C083], [C084], [Z001], [Z002], and [H010]	InterDigital	discussion	Rel-19	NR_AIML_air-Core

R2-2507624	Issues related to AI for Air interface feature in TS 38.300	Nokia	discussion	Rel-19	NR_AIML_air-Core

R2-2507654	Discussion on open issues of AIML LCM [E041, H003/A105/S047, E042, Z001/Z002, B206]	Xiaomi	discussion	Rel-19	NR_AIML_air

R2-2507670	Corrections for CSI report configuration (H003, N021, S047, A105)	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

#### 8.1.2.3	LCM for Positioning use case

Corrections only.  Companies should follow guidance from rapporteurs.

Not Treated

R2-2507088	Discussion on remaining issue for AI positioning	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core	Late

R2-2507460	Open Issues LCM for Positioning	Ericsson	discussion	Rel-19	NR_AIML_air-Core	Late (Moved from 8.1.2.3)

### 8.1.3	NW side data collection

Corrections only.  Companies should follow guidance from rapporteurs.

[H007] Handling of logged data during logged measurement configuration modification and release

R2-2507296	On RIL Z010, Z011, Z004/J009, Z005/H009, Z007, J008, H007 and RAN centric NW side data collection	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Observation 7: Discarding the collected data when associated logging configuration is released or modified will cause the collected data loss which is waste the UE effort to collect the data.

Observation 9: NW can be aware of situation before releasing the CSI-LoggedMeasurementConfig, so that the NW is able to do the right things when releasing the old csi-LoggedMeasurementConfig and adding a new csi-LoggedMeasurementConfig with the same configuration Id.

Proposal 9: To capture the following note in the subclause 5.5x.1.3:

‘It is up to NW implementation to guarantee that UE will not log the measurements for a csi-LoggedMeasurementConfigId is from different csi-LoggedResourceConfig in the VarCSI-LogMeasReport’

Noted

R2-2507337	Discussion on RILs [J008][J009][N028][H002][H007][Z004][Z005][H009][L002] for AIML	LG Electronics Inc.	discussion	Rel-19

Proposal 4.	[H007] Adopt proposed TP from H007 in [1], i.e., clarify the UE operation when it receives modified logging configuration, to ensure alignment between UE behaviour and network expectations.

Noted

R2-2507119	Open issues on NW-side data collection (including H007/Z005/H009/H002/Z007)	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 3 (RIL H007): Following similar principle of legacy logged MDT, NW is expected to retrieve data before providing an updated configuration, and all the remaining logged data (if any) will be discarded upon reception of a modified logging configuration or release a logging configuration.

Noted

Discussion

-	Nokia supports LG as we should specify something.

-	Interdigital, Ericsson, ZTE, Samsung supports ZTE’s proposal as it is a corner case and releasing every time is not efficient.

-	Xiaomi thinks that if the network releases it would be a waste.  We should go with apple or LGs proposal.

-	Qualcomm, Vivo supports apple.  Huawei has a concern that all data would be lost and there has to be some logic to release the data and support’s LGs proposal.

-	Samsung has concerns with LG and Apple, the UE shouldn’t release the data.

-	ZTE thinks that the network can retrieve the data before the configuration.   Nokia supports ZTEs concern.

-	Nokia and Fujitsu think it is ok for network implementation but the UE should be clear on the expected behaviour.

- Agreement on [H007]

- 1	The UE keeps the data (as in current specification).   It is up to the network to ensure how logged configurations are handled with the understanding that the UE will keep the data and will not do any special handling with configurations are released/modified.

- 2	For agreement 1, add a NOTE: the UE keeps the data for a loggedconfigurationID when that configuration is released/modified.

[N028] Reuse of A1/A2 events for NW-side logging

R2-2507298	Discussion on RILs related to NW side data collection	Nokia	discussion	Rel-19	NR_AIML_air-Core	Late

Observation 1: The current implementation does not follow the conventional specification structure defined for other event-based features and it is not easily expandable for a new event.

(RIL-N028) Simplify the procedure by adapting the changes as proposed in the Section 2.1.1.

Treated offline and noted

[H002] Retaining Logged Measurements During LTM

R2-2506928	[H002][H007] Handling of logged data in UE	Lenovo	discussion	Rel-19

Proposal 1: [H002] RAN2 is suggested to not support the data retaining for LTM case in Rel19.

R2-2507431	Remaining issues for NW side data collection (RILs: N028, H002, H007, Z005, H009) 	InterDigital Pennsylvania	discussion	Rel-19	NR_AIML_air_Ph2-Core

Proposal 2: RAN2 to confirm that no special handling is needed regarding the retraining/discarding of data during LTM (i.e., current retainLoggedMeasurement handling for HO is sufficient also for LTM).

R2-2507119	Open issues on NW-side data collection (including H007/Z005/H009/H002/Z007)	Apple	discussion	Rel-19	NR_AIML_air-Core

Observation 2: On H002, although the solution agreed in RAN2#130 may not be applied to subsequent LTM, smart NW implementation may still apply the solution to LTM. For example, NW may configure all the candidate LTM cells from the same infra-vendor based on its implementation.

Proposal 5 (RIL H002): Capture the following NOTE in TS 38.300 (TP in Appendix 1).

NOTE:	It is up to Network implementation whether to include the indication in the RRCReconfiguration message for regular HO or CHO or LTM.

[Z005] Start/stop performing L1 measurement in Logged NW side data collection

[H009] Interactions with PHY for NW-side data collection

R2-2507296	On RIL Z010, Z011, Z004/J009, Z005/H009, Z007, J008, H007 and RAN centric NW side data collection	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Observation 5: The interaction between RRC and PHY layer is missing in the current text procedure regarding measurement logging, as a result, PHY layer still performs the L1 measurement when measurement logging has been stopped in RRC layer which may cause the unnecessary power consumption, PHY layer does not perform the corresponding L1 measurement in time when RRC layer start/resume data logging which may result in nothing can be logged by RRC.

Proposal 5: Add the interlayer interaction between RRC layer and PHY layer in the text procedure regarding L1 measurement logging in subclause 5.5x.3.2:

- Indicate to lower layer to start the corresponding L1 measurement when performing the logging

- Indicate to lower layer to stop the corresponding L1 measurement when stopping performing the logging.

R2-2507337	Discussion on RILs [J008][J009][N028][H002][H007][Z004][Z005][H009][L002] for AIML	LG Electronics Inc.	discussion	Rel-19

Proposal 6. 	[Z005/H009] Not support TPs from Z005 and H009 in [1]. The decision whether to measure or not is UE implementation, and RAN2 specify the logging-related behavior only at the RRC layer.

[Z007] The field description of CSI-LogMeasReportReq (regarding multiplexing of legacy SON/MDT and AIML logged data)

R2-2507338	Discussion on RILs [E041][E042][Z001][Z002][H010][Z007][E040][N021][H003][B204][X004] for AIML	LG Electronics Inc.	discussion	Rel-19  (moved from agenda 8.1.2.2)

Proposal 5.   [Z007] Not support TP from Z007 in [1], i.e., the current specification is sufficient to capture that multiplexing of legacy SON/MDT reports and AIML logged data is not supported

R2-2506961	Discussion on RILs [Z004,J008,J009], [H007], [Z005,H009], [Z007]	vivo	discussion	NR_AIML_air-Core

Proposal 5.	 (Z007) The current procedural text does not capture the RAN2 agreement of “Multiplexing of legacy SON/MDT report and AIML logged data is not supported in the same UE information response message”.  Suggest to add the related description in the field description of CSI-LogMeasReportConfig.

[Z004] The timing point of logging the cell Id and configuration Id for the data collection

[J008] Setting csi-LogMeasInfoList in UEInformationResponse

[J009] Discard entries in csi-LogMeasInfoCellList after sending UEInformationResponse

- To be treated offline [Z004][J008][J009] considering the contributions below

R2-2507296	On RIL Z010, Z011, Z004/J009, Z005/H009, Z007, J008, H007 and RAN centric NW side data collection	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: The concern from J009 is no longer exited if UE creates the entry only when at least one measurement result for this entry is logged.

R2-2506961	Discussion on RILs [Z004,J008,J009], [H007], [Z005,H009], [Z007]	vivo	discussion	NR_AIML_air-Core

Proposal 1.	 (Z004) (J009) No specification change is needed, i.e.,

- The logging of cell Id information and configuration Id information in the VarCSI-LogMeasReport is performed upon receiving logging configuration.

- The UE does not discard the entries in csi-LogMeasInfoCellList from VarCSI-LogMeasReport if the corresponding csi-LogMeasInfoList is empty.

Proposal 2.	(J008) Upon reporting the logging measurement, the UE should ignore the entries in csi-LogMeasInfoCellList from VarCSI-LogMeasReport, in which the corresponding csi-LogMeasInfoList is empty.

R2-2507298	Discussion on RILs related to NW side data collection	Nokia	discussion	Rel-19	NR_AIML_air-Core	Late

Proposal 2: (RIL-Z004) Support procedural text in Clause 5.5x.1.3 and Clause 5.5x.3 as it is.

Proposal 3: (RIL-J008/J009/S044) Additional clarification is required for logged measurement entries and additional entries in Clause 5.7.10.3.

Not Treated:

R2-2507091	Discussion on NW side data collection	Samsung	discussion	Rel-19	NR_AIML_air-Core

R2-2507163	[J008][J009] Logged data reporting for NW-side data collection	Sharp	discussion	Rel-19	Late

R2-2507653	[Z004][H002][H007][Z007] Discussion on open issues of NW data collection	Xiaomi

R2-2507669	RRC corrections for NW-sided data collection [H002], [H007], [H009/Z005]	Huawei, HiSilicon	Rel-19	NR_AIML_air-Core

R2-2507679	Discussion on RILs for NW-side data collection (Z004, J008, J009, S044, H007, H002)	Ericsson	discussion

## 8.2	Ambient IoT

(Ambient_IoT_solutions, leading WG: RAN1; REL-19; WID: RP-250796)

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

### 8.2.1	Organizational

LS, Rapporteur input, including workplan, etc.

Including outcome of [POST130][027][AIoT] MAC Running CR (Huawei) and [POST130][028][AIoT] 38.300 Running CR (CMCC)

Email Discussion Outputs

R2-2507030	Summary of A-IoT MAC open issues (outcome of [POST131][021][AIoT] MAC spec)	Huawei, HiSilicon	report	Rel-19

Noted

R2-2507709	Summary of A-IoT MAC open issues offline	Huawei, HiSilicon	report	Rel-19

Noted

Agreements 1. RAN2 concludes the follow for the proposals in post-email summary R2-2507030:  (5-1) For permanent disabled device, no change to MAC and 38.300 spec (unless CT1 request us later). Companies understand it is clear in SA2 and/or CT1 spec. 2 (5-2) For paging ID check in re-access case, both transaction ID and paging ID are to be checked by the device. (i.e., no change to the current spec.) 3 (5-3) For Random ID Response message monitoring, RAN2 to agree the following change to MAC spec: 5.3.1.3 Reception of Random ID Response message Once the Access Random ID message is transmitted, the device shall monitor for Random ID Response message until it has received K message(s) of the Access Trigger message or the A-IoT Paging message or R2D Upper Layer Data Transfer message addressed to the device (i.e., the device shall not monitor for the Random ID Response message after that). The K is configured in the A-IoT Paging message.. 4. (5-4) For R2D Upper Layer Data Transfer message monitoring, no spec change is needed.

Rel19 CR

R2-2507029	A-IoT MAC rapporteur CR	Huawei, HiSilcon	CR	Rel-19	38.391	19.0.0	0001	-	F	Ambient_IoT_Solutions-Core

Endorsed and will be updated after RAN2#131bis

[POST131bis][017][AIoT] MAC CR  (Huawei)

Intended outcome: update and review revised MAC CR with agreements from this meeting and highlight any new open issues.

Deadline:  Long

LS to RAN2

R2-2506704	LS on delayed A-IoT D2R NAS messages (C1-255165; contact: Huawei)	CT1	LS in	Rel-19	Ambient_IoT_Solutions, AmbientIoT-CT	To:RAN2	Cc:SA2

Noted

R2-2506708	LS on the maximum supported AIoT NAS container length (C1-255679; contact: Lenovo)	CT1	LS in	Rel-19	AmbientIoT-CT	To:RAN2	Cc:RAN1, RAN3

Noted

Agreements

RAN2 replies to CT1:

	We clarify the answer is only about R19 in general.

	For the response to single upper layer command in D2R, RAN2 supports the 125 bytes considering that SA1 requirement is 125 bytes.

	To explain to CT1 that the D2R segmentation is not intended to support the NAS SDU larger than SA1 requirement.

	RAN2 understands for R2D upper layer data for single upper layer command, the maximum size of one R2D NAS container is: 125bytes – 6bytes = 119bytes

[AT131bis][13][AIoT] LS to CT1 (Lenovo)

Intended outcome: agree to LS by email

Deadline:  Thursday

R2-2506712	LS on Ambient IoT Stage-2 TP (R1-2506523; contact: CMCC)	RAN1	LS in	Rel-19	Ambient_IoT_Solutions	To:RAN2

Already implemented in August meeting

Noted

R2-2506748	Reply LS to Reply LS on the removal of service type information (S2-2507689; contact: LGE)	SA2	LS in	Rel-19	AmbientIoT-ARC, Ambient_IoT_Solutions	To:RAN3, RAN2	Cc:RAN1

Noted

R2-2506750	LS on AIoT Device Permanent ID Length (S2-2507793; contact: Huawei)	SA2	LS in	Rel-19	AmbientIoT-ARC	To:RAN2	Cc:RAN1, CT4, SA3

Noted

-	Apple thinks that this impacts the flexibility of future paging message.   Ericsson and Xiaomi thinks that we should highlight the space pressure and the 1000bits limit.   ZTE thinks that there is a good reason to do this to compete with RF ID.   Mediatek agrees there is a use cases but we can address all the atoms in the universe about 270bits, so it would be good to highlight the space limitation.   Huawei explains that RF ID carries more information and it is not just one ID per device.

Agreements RAN2 replies to SA2: - From RAN2 point of view, extending the AIoT Device permanent ID to 256-bit and 496-bit is feasible, based on the calculation of existing fields size in this release. - However, the less overhead of paging message, the better coverage performance for paging message reception. Highlight space pressure with 1000bits and ask SA2 to consider the space pressure from RAN2.

[AT131bis][014][AIoT] LS to SA2 (Huawei)

Intended outcome:  Agree to LS by email

Deadline:  Thursday

R2-2507912	[Draft] Reply LS on AIoT Device Permanent ID Length	Huawei, HiSilicon	LS out	Rel-19	Ambient_IoT_solutions	To:SA2	Cc:RAN1, CT4, SA3

=> Revised in R2-2507931

R2-2507931	Reply LS on AIoT Device Permanent ID Length	RAN2	LS out	Rel-19	Ambient_IoT_solutions	To:SA2	Cc:RAN1, CT4, SA3

The LS is approved

R2-2506753	Reply LS on paging ID length (S3-252933; contact: CATT)	SA3	LS in	Rel-19	AmbientIoT-SEC	To:SA2, RAN2, CT4	Cc:RAN3

Noted

LS with RAN2 in CC

R2-2506709	LS on Structure updates of AIoT Identifiers (C4-253575; contact: CICT)	CT4	LS in	Rel-19	AmbientIoT-CT	To:SA2, RAN3	Cc:SA3, RAN2

Noted

R2-2506920	[Draft] Reply LS on the maximum supported AIoT NAS container length	Lenovo	LS out	Rel-19	Ambient_IoT_solutions	To:CT1, RAN3

To be update over email discussion

R2-2507913	Reply LS on the maximum supported AIoT NAS container length	RAN2	LS out	Rel-19	Ambient_IoT_solutions	To:CT1	Cc:RAN3

=> Revised in R2-2507932

R2-2507932	Reply LS on the maximum supported AIoT NAS container length	RAN2	LS out	Rel-19	Ambient_IoT_solutions	To:CT1	Cc:RAN3

The LS is approved

### 8.2.2	A-IoT

Corrections only.  Companies should follow guidance from rapporteurs.

Paging ID extension

R2-2507347	Remaining issues in R19 Ambient-IoT	ZTE Corporation, Sanechips	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 1(Issue 1-3):  RAN2 to reply to SA2 that it is feasible to extend the AIoT Device permanent ID to 256-bit and 496-bit from signaling overhead point of view, even though less overhead is preferred from the performance point of view.

Proposal 2(Issue 1-3): If AIoT Device permanent ID is extended as requested by SA2, the length of the paging ID length field needs to be extended to 10 bits, so that maximum size of about 600 bits paging ID can be indicated.

=> Not treated

R2-2507101	Discussion on remaining open issues of A-IoT	Apple	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 7: 	RAN2 reply to SA2 that only 256-bit Paging ID can be considered.

Wait for SA2 conclusion before finalizing length ID field size

Maximum NAS container size

Not treated:

R2-2506962	Remaining open issues on R19 Ambient IoT	vivo	discussion	FS_Ambient_IoT_solutions

Proposal 4: (Issue 4-6) RAN2 to reply to CT1 with the following information:

The maximum supported AIoT NAS container length in R2D direction is 119 bytes;

The maximum supported AIoT NAS container length in D2R direction is 123 bytes.

R2-2507557	Remaining open issues for Rel-19 A-IoT	Xiaomi	discussion	Rel-19	Ambient_IoT_Solutions	R2-2506986

Proposal 5: (Issue 4-6) RAN2 replies to CT1 that the maximum supported TB sizes in R2D and D2R directions are 952 bits and 991 bits, respectively.

Security parameter in paging

R2-2506902	Issue 1-7 Security parameter in A-IoT paging	CMCC, China Unicom, China Telecom, Huawei, HiSilicon, ZTE Corporation, Sanechips, Nokia, Fujitsu, Interdigital, Xiaomi, vivo, LGE, Panasonic, Transsion Holdings, Quanray Electronics, ETRI	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 1:	Add a 128-bit field in paging message to carry the security parameter, which is optionally present with 1-bit to indicate its presence.

Noted

R2-2506915	Discussion on A-IoT remaining issues	Spreadtrum, UNISOC	discussion	Rel-19

Proposal 3: The 128 bits security parameter should be mandatory in paging message according to SA3 agreements.

Noted

Discussion

-	Samsung things that we can make this optional from singaling point of view but whether it is mandatory present it is up to SA3.

-	Qualcomm thinks that SA3 have in their specification that it is always included.   Why don’t we discuss this SA3.

-	Huawei doesn’t have an intention to change SA3 agreements, this is only for signalling purposes.  This one bit will also help with future releases.    And it is up for discussion on how to align the SA3 behaviour.

-	Mediatek thinks that only reason seems to be for future compatibility bit, but not sure what will change.  CMCC thinks that for Rel-20 there may be other security information.

-	Xiaomi thinks that it can be up to the operator to determine whether to include it or not.   Apple indicates that now Xiaomi is showing a different reason and that should be discussed in SA3.   Xiaomi explains that in R20 we can have a simple security algorithm.

-	ZTE thinks that future compatibility is one reason but of course another reason it is coverage.   ZTE thinks that if we do it this way then if SA3 changes the agreements this works.  Mediatek agrees but thinks that we should decouple future compatibility with SA3 decision.

-	Interdigital explains that we are challenging SA3 decision for now but just discussing the tradeoff with adding one bit to potentially simplify lives later on.

-	LG thinks that if we want the future compatibility we can use another bit instead of extension bit, like a version bit.   Huawei thinks that this would be specific to security so we can have version bit for security not paging message.

-	Nokia understands that the SA3 agreement is that the UE shall support and not that the network shall always set it.

-	Futurewei indicates that everytime we include this 128bits it means it is power consuming.   And agrees with ZTE that maybe SA3 will change their decision.   Qualcomm thinks that if they won’t change their mind in R19 they will not for R20.   Huawei thinks that it is not clear whether the authentication procedure is used every time.

-	Oppo indicates that we also have the case for paging for all in which case we will not have security.

-	ZTE asks that even if authentication is needed is the 128bits always needed.

Agreements Send an LS to SA3, cc:CT1 - Question on whether the security parameter has to be included in every paging message. - Clarify that we have different types of messages, paging message and subsequent paging. - While it is feasible from signaling perspective, RAN2 has concerns - RAN2 has discussed the following concerns and downsides with making the 128bits mandatory for every paging message: - deployments where it may not be always needed - overhead is high and complexity for devices - power consumption overhead with authentication (time consuming, etc) - coverage In RAN2 agree to: - Add a 1 bit optionality bit for 128bits security field in paging message. For now, we state in our specification that this bit is set to present in this release according to SA3 TS. If SA3 confirms that it can be optional after LS reply it shall be updated.

[AT131bis][015][AIoT] LS to SA3  (CMCC)

Intended outcome:  LS to SA3 on 128bits security

Deadline:  Thursday

R2-2507906	Draft LS on Security parameter in A-IoT paging	CMCC	LS out	Rel-19	Ambient_IoT_Solutions-Core	To:SA3	Cc:CT1

The LS is approved in R2-2507920

R2-2507920	LS on Security parameter in A-IoT paging	RAN2	LS out	Rel-19	Ambient_IoT_Solutions-Core	To:SA3	Cc:CT1, RAN3

=> Approved

New case with no NAS response

R2-2507258	Remaining issues of Ambient IoT	Qualcomm Incorporated	discussion	Ambient_IoT_Solutions-Core

Proposal 2: RAN2 confirms, in addition to delayed response, it is valid that in some cases A-IoT NAS doesn’t provide a response at all.

Noted

R2-2507558	Discussion on the remaining issues on A-IoT	Samsung	discussion	Rel-19	Ambient_IoT_Solutions-Core

Proposal 1: RAN2 is kindly asked to prepare LS to CT1 to check the case of “no upper layer data available”. If CT1 confirms the possible case of “no upper layer data available”, the following solution can be considered:

“0 SDU & MDI =0” represents “no upper layer data available”

“0 SDU & MDI =1” represents “no upper layer data available due to delay NAS”

Noted

Discussion

-	Xiaomi and Samsung that there is an integrity failure case, but it is not clear whether the device has to send something to the AF.

On the case where NAS doesn’t provide a response at all (not for integrity failure)

-	Vivo thinks that Samsungs proposal makes sense as we have two different cases, delayed and no responses.

-	Apple and Ericsson thinks that we don’t need to handle this in RAN2.   Xiaomi explains that the reader doesn’t know that the UE will not respond.   Qualcomm explains that today’s protocol we will send 0 SDU and drain the power.   Ericsson thinks that we should just not send anything (no 0 SDU).

-	Mediatek thinks that there is benefits for the reader to understand whether it expects anything or not.

-	Transsion thinks that the reader can’t take action if it doesn’t know what case it is.

-	ZTE wonders how the device knows why the failure occurred and if there is an integrity failure the device should not transmit.   Huawei explains that UE implementation.

-	ZTE thinks that at least for integrity failure according to SA3 there should be no response.   Nokia agrees that the issue is similar to NR.    Honor thinks that for integrity failure the AS shouldn’t respond.    Interdigital thinks that this is not the only reason for failure.

On MDI

-	Huawei thinks that for now we only have one case “other than integrity failure” so we can use the 1 MDI bit value and if we have to handle integrity failure we can consider adding another bit later.

-	Qualcomm thinks we may need to update further so maybe we can wait to see if we get a response from SA3 or we can make it future proof.

“0 SDU & MDI =0” represents “no upper layer data available”

“0 SDU & MDI =1” represents “no upper layer data available due to delay NAS”

- Agreements

- RAN2 confirms, in addition to delayed response, it is valid that in some cases A-IoT NAS doesn’t provide a response at all.

- For cases other than integrity failure, AS will indicate no NAS response expected to reader.   FFS how (e.g. using 0 SDU & MDI, or new indication).

- For integrity failure, for now RAN2 assumes that there is no AS response to the reader.  Ask SA3 ccCT1 whether a similar mechanism (e.g. AS response to the reader) can be used to indicate to reader no NAS response due to integrity failure.

[AT131bis][016][AIoT] LS to SA3 on integrity failure (Xiaomi )

Intended outcome: agree to LS by email

Deadline:  Thursday

R2-2507914	[Draft] LS on integrity failure	Xiaomi	LS out	Rel-19	Ambient_IoT_solutions	To:SA3, CT1	Cc:SA2, RAN3

Update action: RAN2 respectfully asks SA3 to address Question 1 and other WGs to provide feedback (if needed) to the above questions for the case of integrity check failure of an R2D A-IoT NAS message.

The LS is approved in R2-2507915 with the change above

R2-2507915	LS on integrity failure	RAN2	LS out	Rel-19	Ambient_IoT_solutions	To:SA3	Cc:SA2, RAN3, CT1

=> Approved

Paging ID Type (wait for SA2 and discuss in November)

Not treated:

R2-2506839	Ambient-IoT Remaining Issues	NEC	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 2.2:

One new paging ID type field is included in CBRA paging message, which is used to indicate the type of paging ID field from either single device ID or group device ID (filter information).

One new paging ID protection field is included in both CBRA and CFA paging message, which is used to indicate whether protection procedure for paging id is applied or not.

R2-2507347	Remaining issues in R19 Ambient-IoT	ZTE Corporation, Sanechips	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 4 (Issue 1-7): A 3 bit field is introduced in paging message to indicate the paging ID type (i.e. device permanent ID, concealed T-ID, stored T-ID, or filtering information) and whether the stored T-ID type shall be updated with a command in case stored T-ID type is used.

R2-2507426	Open issues for TS 38.391	Ericsson	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 5	Rename paging ID field name as paging NAS container and paging ID length field as paging NAS container length.

Overall guidance

After December, NBC changes should be avoided as much as possible similar to NR MAC process.

Not treated:

R2-2506765	Discussion on segmentation order issue	Transsion Holdings	discussion	Rel-19

R2-2506921	Discussion on max NAS message size	Lenovo	discussion	Rel-19

R2-2506929	Discussion on remaining open issues on A-IOT	OPPO	discussion	Rel-19	Ambient_IoT_Solutions

R2-2506986	Remaining open issues for Rel-19 A-IoT	Xiaomi	discussion	Rel-19	Ambient_IoT_Solutions	Revised

R2-2507031	A-IoT remaining issues related to other WGs	Huawei, HiSilicon	discussion	Rel-19

R2-2507197	Views on remaining issue 3-7	Ofinno	discussion	Rel-19	Ambient_IoT_Solutions

R2-2507207	Discussions on remaining issues for Rel-19 AIoT	Futurewei	discussion	Rel-19	38.391	Ambient_IoT_Solutions-Core

R2-2507210	Discussion on open issues for AIoT	LG Electronics Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2507256	Ambient IoT open issues	Nokia	discussion	Ambient_IoT_Solutions

R2-2507453	Handling of NAS Layer Errors with MDI Field	InterDigital France R&D, SAS	discussion	Rel-19

R2-2507535	Discussion on Ambient IoT open issues	ASUSTeK	discussion	Rel-19	Ambient_IoT_Solutions

R2-2507552	On remaining issues for AIoT MAC	NTT DOCOMO, INC.	discussion	Rel-19

R2-2506942	Discussion on remaining issues for ambient IoT	CATT	discussion	Rel-19	Ambient_IoT_Solutions

## 8.4	Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

(NR_LPWUS-Core; leading WG: RAN1; REL-19; WID RP-251200)

Time budget: 0 TU

Tdoc Limitation: 3 tdocs

### 8.4.1	Organizational

LS, Rapporteur input, etc.

R2-2506727	LS on allocation of CN assigned subgroup ID for LP-WUS (R3-255941; contact: NTT DOCOMO)	RAN3	LS in	Rel-19	NR_LPWUS-Core	To:SA2	Cc:RAN2, RAN1

Noted

38.304

R2-2506862	List of open issues for LP-WUS 38.304 CR	CATT	discussion	Rel-19	NR_LPWUS-Core

Noted

R2-2506861	Corrections on LP-WUS in TS 38.304	CATT	CR	Rel-19	38.304	18.4.0	0447	-	F	NR_LPWUS-Core

Endorsed

[Post131bis][207][LPWUS] CR for TS 38.304 (CATT)

Intended outcome: Update the CR for endorsement

Deadline:  Short

38.331

R2-2507008	LPWUS RILs resolutions (based on review file v17)	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

All the RILs marked with PropAgree are agreeable, will be reflected in the RRC CR

R2-2507007	Miscellaneous corrections on RRC for Rel-19 LP-WUS WUR	vivo (Rapporteur)	CR	Rel-19	38.331	19.0.0	5503	-	F	NR_LPWUS-Core

Endorsed

[Post131bis][208][LPWUS] CR for TS 38.331 (vivo)

Intended outcome: Update the CR for endorsement

Deadline:  Short

=> Endorsed in R2-2507939 (38.331)

R2-2507939	Miscellaneous corrections on RRC for Rel-19 LP-WUS WUR	vivo (Rapporteur)	CR	Rel-19	38.331	19.0.0	5503	1	F	NR_LPWUS-Core

=> Endorsed

38.321

R2-2507104	Open issues on Rel-19 LPWUS 38.321 CR	Apple(Rapporteur)	discussion	Rel-19

Noted

Proposal 2: Confirm UE does not monitor LP-WUS when short DRX cycle is used in either DRX group when secondary DRX group is configured.

Proposal 3: Confirm UE enables the LP-WUS monitoring only based on RRC configuration.

Proposal 4: Confirm in Option 1-2 when UE is in CDRX active time but LP-WUS monitoring timer is not running, the legacy UE operation on CSI/SRS transmission in active time is applied.

Proposal 5: Confirm in Option 1-2 when UE is in CDRX active time and LP-WUS monitoring timer is running, the legacy UE operation on CSI/SRS transmission in active time is applied.

P2

-	Qualcomm wonder is this for O1-1 or O1-2. Apple think it is for O1-1, and think there is no impact to MAC if we agree with P2. NEC agree with the principle but see some impact to at least stage 2 spec.

For Option 1-1, confirm UE does not monitor LP-WUS when short DRX cycle is used in either DRX group when secondary DRX group is configured.

P3

-	Nokia think this is related to some discussions under RRC topic.

-	Xiaomi think the intention here is to exclude the option of enabling/disabling via L1/L2 signaling. Ericsson, Huawei, OPPO share this understanding.

Activation/deactivation of LP-WUS monitoring via L1/L2 signaling is not supported.

P4

-	Ericsson think it is generally ok but the ‘legacy operation’ may not be crystal clear.

-	Apple think there is MAC spec impact if we agree. Xiaomi not sure.

-	vivo think we should agree P4 and discuss remaining details in CR.

Confirm in Option 1-2 when UE is in CDRX active time but LP-WUS monitoring timer is not running, the legacy UE operation on CSI/SRS transmission in active time is applied.

P5

-	vivo think the behaviour is not the same as the legacy.

Confirm in Option 1-2 when UE is in CDRX active time and LP-WUS monitoring timer is running, the legacy UE operation on CSI/SRS transmission in active time other than when OnDurationTimer is running is applied.

[Post131bis][209][LPWUS] CR for TS 38.321 (Apple)

Intended outcome: Update the CR for endorsement

Deadline:  Short

The following 5 contributions are moved to 8.4.4

R2-2507639	Miscellaneous corrections for LP-WUS	Ericsson	CR	Rel-19	38.300	19.0.0	1046	-	F	NR_LPWUS-Core	Revised

R2-2507640	Miscellaneous corrections for LP-WUS	Ericsson	CR	Rel-19	38.300	19.0.0	1046	1	F	NR_LPWUS-Core	R2-2507639

R2-2507369	Open issue of LP-WUS in TS37.340	ZTE Corporation, Sanechips	report	Rel-19	NR_LPWUS-Core

R2-2507370	Correction on LP-WUS in TS 37.340	ZTE Corporation, Sanechips, Xiaomi	CR	Rel-19	37.340	19.0.0	0424	-	F	NR_LPWUS-Core

R2-2507156	List of open issues for Rel-19 LP-WUS UE capabilities	Huawei, HiSilicon (Rapporteur)	discussion	Rel-19

### 8.4.2	RRC issues

Issues related to RILs, other remaining RRC issues

Co-existence of LP-WUS in idle/inactive and paging adaptation (C026)

R2-2506864	[C026] Co-existence of LP-WUS with paging adaptation	CATT	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: Support the co-existence of LP-WUS and paging adaptation in RRC_IDLE/INACTIVE.

Proposal 2: lpwus-PoNumPerLo-r19 is common for both legacy paging and Rel-19 paging adaptation.

Proposal 3: Separate lpwus-LoFrameOffsetList-r19 is introduced for Rel-19 paging adaptation.

Proposal 4: If separate parameter for paging adaptation with LP-WUS, i.e., separate lpwus-LoFrameOffsetList, is signaled in system information, a UE supporting paging adaptation and LP-WUS can monitor the LP-WUS occasion according the parameters.

R2-2507009	[V001-V006, C026, H053/054, E036] Discussion on RRC open issues for LP-WUS WUR	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 9: (C026) LP-WUS could coexist with all Rel-19 features, unless critical issue is identified, detailed spec change discussed in CR phase.

Discussion

-	CATT think with the main session agreement we should further discuss the detailed impact.

-	Nokia think R1 may be discussing this also.

[AT131bis][201][LPWUS] Impact with co-existence with LPWUS and  paging adaptation (CATT)

Scope: Discuss the potential impact if LPWUS and paging adaptation are configured at the same time, to reach the same understanding on the impact, also on whether specification work is needed

Intended outcome: Summary in R2-2507731.

Deadline: before Thursday CB

R2-2507731	Impact with co-existence with LPWUS and paging adaptation

Noted

RAN2 confirms that LP-WUS and paging adaptation can be configured and used together.

P2

-	OPPO thinik O2 is clearer so prefer it. Ericsson, NEC, LG E also support O2.

-	Xiaomi think O2 is ok in general, but want to check whether there is additional issue since now it extend the length of the list to 8.

-	ZTE wonder if we go O2 whether we need to extend the length of the list, and not sure if there is R1 impact. Vivo think no impact to R1.

Separate LP-WUS configuration for legacy UEs and UEs supporting paging adaptation. Introduce a separate lpwus-LoFrameOffsetList configuration for LP-WUS with paging adaptation.

Empty UAI for report for LP-WUS time offset (E009, V001, C031, Z052)

R2-2506863	[E009][H050][V001][C031][H055]Discussion on RRC open issues	CATT	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: If the UE has no preference on offset for LP-WUS monitoring of the cell group, the UE do not include timeOffset in the LPWUS-OffsetPreference IE.

R2-2507041	Discussion on [RIL] O701 Whether UE can report an empty preference time offset	OPPO	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1	RAN2 discuss and decide the following options on whether allow UE report an empty preference time offset to NW:

Option1: It is allowed to report an empty preference time offset for LP-WUS monitoring to NW and adopt the TP1 in Appendix.

Option2: It is not allowed to report an empty preference time offset for LP-WUS monitoring to NW and adopt the TP2 in Appendix.

Discussions

-	OPPO prefer Option 1.

-	Xiaomi, ZTE, Ericsson agree with CATT P1, since this is similar as legacy way.

-	Nokia think if UE support this feature then UE should always have some value to report, so prefer Option 2.

If the UE has no preference on offset for LP-WUS monitoring of the cell group, the UE do not include timeOffset in the LPWUS-OffsetPreference IE.

UAI for disabling LP-WUS (H050, E043)

R2-2507331	[H050][H053][H054][H055] Discussion on LP-WUS RILs	Huawei, HiSilicon	discussion	Rel-19	Late

Noted

Proposal 1: UE can send UAI to the network indicating to disable the LP-WUS functionality or whether the LP-WUS can be enabled again.

Proposal 2: When to send the UAI is up to UE implementation, without additional trigger conditions/thresholds.

R2-2507626	LP-WUS issues (E035, E036, E037, E043, 38304-2, C026)	Ericsson	discussion	Rel-19	NR_LPWUS-Core	Late

Noted

Proposal 9	NW can configure an exit condition for LP-WUS monitoring in connected mode based on RLM/BFD measurements.

R2-2507236	Discussion about LP-WUS RILs H050, E043, C026, V001, Z052 and V002	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1	[H050, E043] UE is able to notify network about its preference on whether to apply LP-WUS mechanism, preferably, via UAI.

R2-2507344	Discussion about LP-WUS RILs E034, E035, E037, E043, V002, H050, H053, H054	Nokia	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 2: [H050, E043] The NW can configure LR measurement triggering condition for UAI for LP-WUS.

R2-2507350	Discussion on RIL in LP-WUS RRC	Qualcomm Incorporated	discussion	NR_LPWUS-Core

Noted

Proposal 1	For both of Option 1-1 and Option 1-2, UE needs to indicate to the gNB when the UE enters or leave LP-WUS mode.

Proposal 2	For option 1-2, gNB needs to send a switching command to UE in order for UE switching from LP-WUS mode to legacy PDCCH monitoring with/without DCP.

Discussions

-	InterDigital ok with the Nokia proposal.

-	Huawei think this proposal is for the connected state. Qualcomm think this is to handle the rather frequent cases when UE moves.

-	Samsung disagree with this new UAI and think we can rely on NW configuration.

-	Xiaomi think for the connected state the NW has some info about the radio condition, so not sure why UAI is critical here. NEC agree with Huawei proposal, and think NW may not know the LR measurement well.

-	Ericsson think the use case of this proposed UAI is not so clear, and think if UE is moving fast this UAI may not be timely and helpful to the NW.

Discussions in CB session

-	Ericsson not sure how such UAI, if introduced, is triggered by UE, and think simply relying on UE implementation is not so clear solution. InterDigital, Nokia share this view, and think UE behaviour should be specified.

-	OPPO do not think there is an issue w/o such UAI. Nokia think there is issue since NW has no idea about whether UE can receive the LPWUS, and also UE may keep monitor even if it is out of LPWUS coverage.

-	Qualcomm think the trigger should be when UE moving out of the LPWUS coverage. Xiaomi think there is no LR measurement for UEs in connected state so this does not work. LG E think for the case described by Qualcomm, NW/UE can rely on measurement report. Nokia, InterDigital think it is difficult to only leave it to NW implementation.

-	ZTE think in the offline discussions everyone think there is an issue.

-	Qualcomm think we can just close this issue by rejecting the RILs. Vivo think that is fine. Nokia has strong concern and want to discuss further.

Relationship between thresholds for LP-WUS monitoring and RRM relaxation/offloading (V002-V006)

R2-2507009	[V001-V006, C026, H053/054, E036] Discussion on RRC open issues for LP-WUS WUR	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 3: (V002) The MR-based thresholds of the entry condition for R19 RRM relaxation should be lower than or equal to the thresholds of the entry condition for LP-WUS monitoring.

Proposal 4: (V003) The LR-based thresholds of the entry condition for R19 RRM relaxation should be lower than or equal to the thresholds of the entry condition for LP-WUS monitoring.

Proposal 5: (V004) The MR-based thresholds of the entry condition for serving cell RRM offloading should be higher than or equal to the thresholds of the entry condition for LP-WUS monitoring.

Proposal 6: (V005) The LR-based thresholds of the entry condition for serving cell RRM offloading should be higher than or equal to the thresholds of the entry condition for LP-WUS monitoring.

Proposal 7: (V006) The LR-based thresholds of the exit condition for serving cell RRM offloading should be higher than or equal to the thresholds of the exit condition for LP-WUS monitoring.

R2-2507082	Remaining issues on LP-WUS paging monitoring and proposed TP to 3331,304	Xiaomi Communications, Huawei, HiSilicon, ZTE Corporation, Sanechips, Qualcomm Incorporated, Ericsson, Apple, Lenovo	discussion

Noted

Proposal 1	The UE is allowed to monitor LP-WUS when the UE is in either Rel-19 RRM offload state or in Rel-19 RRM relaxation state. No extra entry/exit condition for LP-WUS monitoring is needed.

R2-2507236	Discussion about LP-WUS RILs H050, E043, C026, V001, Z052 and V002	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 4	[V002, V003, V004, V005] Merging LP-WUS monitoring thresholds into Offloading/Relaxation condition, e.g., UE starts the LP-WUS monitoring either in Offloading or Relaxation scenarios, there are no separate thresholds for LP-WUS monitoring.

Discussions

-	InterDigital think if we merge these conditions there might be R1 impact. Vivo think there is no impact.

-	Nokia think we have discussed this a lot, and think there may be R4 impact, which is too late to have. OPPO share this view. Vivo think there is no further impact to R4 either.

Discussion in the CB on Xiaomi P1

-	Multiple companies cannot accept Xiaomi P1 after some offline discussions.

Discussion in the CB on vivo P5/6/7

-	InterDigital think we do not need this, since they are up to NW implementation. OPPO, LG E agree with this view. Nokia think nothing is broken w/o these proposals.

-	Ericsson these are useless, and think we should only specify as Xiaomi P1 proposed.

The following are the general understanding in R2 except for two companies (such understanding does not require specification change):

The MR-based thresholds of the entry condition for serving cell RRM offloading should be higher than or equal to the thresholds of the entry condition for LP-WUS monitoring.

The LR-based thresholds of the entry condition for serving cell RRM offloading should be higher than or equal to the thresholds of the entry condition for LP-WUS monitoring.

The LR-based thresholds of the exit condition for serving cell RRM offloading should be higher than or equal to the thresholds of the exit condition for LP-WUS monitoring.

RAN2 impacts for Type 1 and Type 2 LR (H053, H054, E036)

R2-2507331	[H050][H053][H054][H055] Discussion on LP-WUS RILs	Huawei, HiSilicon	discussion	Rel-19	Late

Noted

Proposal 3: RAN2 to create the separate sets of entry/exit thresholds for both LR types defined by RAN4.

R2-2507626	LP-WUS issues (E035, E036, E037, E043, 38304-2, C026)	Ericsson	discussion	Rel-19	NR_LPWUS-Core	Late

Noted

Proposal 4	Introduce a flag in SIB to prohibit Type 2 WUR to use LP-WUS in the cell e.g. when LP-WUS cannot be configured with sufficient guard band for type 2 WUR.

Discussions

Need to create the separate sets of entry/exit thresholds for both LR types

-	Xiaomi think R4 has not defined different RRM requirements, so not sure if separate sets of thresholds are useful.

-	Apple think type 1 or 2 has no impact to R2, and think we do not need to discuss here. CATT agree, and think NW implementation will handle the configurations properly. Vivo also agree, and think the different noise figure of both types only impact the test case. InterDigital also agree, and think it introduces great spec impact.

-	Qualcomm support Huawei proposal.

-	Nokia think different RF accuracy have been defined by RAN4, so think Huawei proposal is good.

-	OPPO have concern since we introduce too many different threshold, so want to ask R4 whether this proposal is needed.

-	HW think the coverage performance of the two types are different.

-	Ericsson think w/o separate thresholds, NW will have to configure as per the worse case scenario. Nokia agree. Vivo think this is up to NW implementation.

-	vivo think R4 does not consider separate capabilities for type 1 and type 2.

-	Huawei, QC think we can send LS to R4 to ask whether there is issue. Apple do not want to send a LS and think if there is issue R4 should let us know.

Need to introduce flag in SIB to prohibit type 2 WUR

-	Qualcomm, Huawei think this restrict LPWUS usage.

-	Nokia think if we do not have separate thresholds for both types then we need this.

-	Ericsson think R4 didn't have time for sufficient discussions. Vivo think more discussions will be had in R4.

H053, H054, E036 are rejected.

TTT for RRM relaxation entry (E035)

R2-2507626	LP-WUS issues (E035, E036, E037, E043, 38304-2, C026)	Ericsson	discussion	Rel-19	NR_LPWUS-Core	Late

Noted

Proposal 10	NW can configure a TimeToTrigger (TTT) with the RRM relaxation entry condition.

Discussion on the need of TTT

-	InterDigital support the proposals, and want to also have a TTT for the exit condition.

-	NEC think the intention is fine but wonders whether R4 also have similar mechanism. Xiaomi agree. Ericsson and Nokia do not think the R4 timer is for the same purpose.

-	CATT think this is small optimization in this late stage.

-	Xiaomi think this is not needed.

-	Nokia, Qualcomm support the proposal.

-	Huawei think when R4 defined the measurement requirements, they assume UE will anyway take multiple samples to get the results, so to some extend the intention is already fulfilled.

-	vivo do not think we need to postpone. OPPO do not see need to this new timer.

E035 is rejected.

R2-2506863	[E009][H050][V001][C031][H055]Discussion on RRC open issues	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2506864	[C026] Co-existence of LP-WUS with paging adaptation	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2506953	Discussion on LP-WUS RRC remaining issue 	NEC	discussion	Rel-19	NR_LPWUS-Core

R2-2507009	[V001-V006, C026, H053/054, E036] Discussion on RRC open issues for LP-WUS WUR	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2507041	Discussion on [RIL] O701 Whether UE can report an empty preference time offset	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2507082	Remaining issues on LP-WUS paging monitoring and proposed TP to 3331,304	Xiaomi Communications, Huawei, HiSilicon, ZTE Corporation, Sanechips, Qualcomm Incorporated, Ericsson, Apple, Lenovo	discussion

R2-2507155	Discussion on open issues 38304-1 and 38304-3 for R19 LP-WUS	Huawei, HiSilicon	discussion	Rel-19

R2-2507236	Discussion about LP-WUS RILs H050, E043, C026, V001, Z052 and V002	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2507331	[H050][H053][H054][H055] Discussion on LP-WUS RILs	Huawei, HiSilicon	discussion	Rel-19	Late

R2-2507344	Discussion about LP-WUS RILs E034, E035, E037, E043, V002, H050, H053, H054	Nokia	discussion	Rel-19	NR_LPWUS-Core

R2-2507350	Discussion on RIL in LP-WUS RRC	Qualcomm Incorporated	discussion	NR_LPWUS-Core

R2-2507504	RRC issues on LP-WUS	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2507626	LP-WUS issues (E035, E036, E037, E043, 38304-2, C026)	Ericsson	discussion	Rel-19	NR_LPWUS-Core	Late

### 8.4.3	MAC issues

Remaining MAC issues

On P1 in R2-2507104 (Proposal 1: Revisit the UE operation in LP-WUS collision for Option 1-2, which should be based on critical issues in the current RAN2 agreements. (based on contribution))

R2-2507627	LP-WUS MAC issue (Eri-001 and Proposal 1 in #213)	Ericsson	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 3	The NW can configure the number of consecutive LP-WUS monitoring occasions the UE is allowed to miss before it needs to start the lpwus-PDCCH-MonitoringTimer.

R2-2507105	Remaining issues of LP-WUS in RRC_CONNECTED	Apple	discussion	Rel-19

Noted

Proposal 1: Capture the configuration restriction that the LP-WUS cycle is shorter than long DRX cycle for Option 1-2.

R2-2507530	LP-WUS MAC Issues	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1(Eri-001): RAN2 Confirms that UE can always be reachable, whether it monitors LP-WUS or monitors PDCCH directly.

R2-2506865	Discussion on MAC open issues	CATT	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: No enhancement is needed for handling on lpwus-PDCCH-MonitoringTimer in LP-WUS collision cases.

Discussion

-	Nokia have sympathy to Ericsson proposal, think that solve the issue that NW cannot reach the UE in the time period.

-	Xiaomi share the same view as CATT and think it is optimization. Xiaomi think NW already can have good control of the UE monitoring, e.g., based on the measurements. ZTE, OPPO agree.

-	ZTE think we already discussed and think NW knows such collision.

-	LG E think the issue should be addressed and agree with Ericsson.

-	NEC think we can consider Apple proposals as a possible compromise.

On P6 in R2-2507104 (Proposal 6: Further discuss the necessity to explicitly describe “option1-1/1-2” based on 38.300 description.  (based on contribution))

R2-2507105	Remaining issues of LP-WUS in RRC_CONNECTED	Apple	discussion	Rel-19

Noted

Proposal 3: Capture the term of Option 1-1 and Option 1-2 in TS 38.300.

R2-2507308	LP-WUS options description	InterDigital	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal: The usage of LP-WUS Option 1-1 and Option 1-2 naming convention is not applied for TS 38.300 nor TS 38.321.

Discussions

-	vivo think in RRC we mention Options 1-1/1-2, but they need to refer to some place. Vivo think these terms are not in R1 spec either. Vivo think if Option 1 and 2 are not the best wording we an consider further refinement.

-	Ericsson think it would be good if we only use these terms in RRC but in a self explainary way.

R2-2507174	MAC open issues	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 3: RAN2 confirms that the MR does not need to wake up for an available CG or SR occasions if there is no actual uplink transmission on that CG/SR occasion.

Discussion

-	Apple, Xiaomi and Huawei think this is not essential.

-	Ericsson wonder how frequent is the issue happening.

-	QC think since UE do not operate in MR and LR at the same time, so this issue might not exist.

-	LG E think there is a confliction btw MR and LR, and we should solve.

RAN2 understand that uplink transmission should not be impacted by LP-WUS operation.

R2-2506865	Discussion on MAC open issues	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2506923	Remaining MAC open issues for  LP-WUS	Lenovo	discussion	Rel-19

R2-2506954	Discussion on LP-WUS MAC remaining issue 	NEC	discussion	Rel-19	NR_LPWUS-Core

R2-2506981	Discussing on remaining MAC open issues	Xiaomi	discussion	Rel-19	NR_LPWUS-Core

R2-2507010	Discussion on MAC open issues for LP-WUS WUR	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2507042	Discussion on the remaining issue on LP-WUS in RRC_CONNECTED	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2507105	Remaining issues of LP-WUS in RRC_CONNECTED	Apple	discussion	Rel-19

R2-2507174	MAC open issues	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_LPWUS-Core

R2-2507308	LP-WUS options description	InterDigital	discussion	Rel-19	NR_LPWUS-Core

R2-2507351	Remaining issues in LP-WUS MAC	Qualcomm Incorporated	discussion	NR_LPWUS-Core

R2-2507530	LP-WUS MAC Issues	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2507627	LP-WUS MAC issue (Eri-001 and Proposal 1 in #213)	Ericsson	discussion	Rel-19	NR_LPWUS-Core

### 8.4.4	Other issues

Issues related to IDLE/INACTIVE, Changes to Stage 2, UE capabilities, and other remaining issues if not covered by the previous agenda items

Low mobility criterion related

R2-2507011	Discussion on other open issues for LP-WUS WUR	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: (38304-1) “low mobility” criterion based on LR measurements is not needed for LP-WUS.

Proposal 2: (38304-3) There is no specification impact for R16 low mobility criterion when UE exits fully offloading.

R2-2507505	Open issues on LP-WUS	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: [38304-1] Support to evaluate low mobility criterion with LR measurements.

Proposal 2: [38304-3] Do not consider the specification impact on low-mobility criteria when a UE exits measurement offloading.

Confirm there is no specification impact for R16 low mobility criterion when UE exits fully offloading.

Discussion on “low mobility” criterion based on LR measurements

-	Ericsson think this is essential issue and think we should address. Ericsson think we should have the means to configure the UE not using LPWUS, to avoid the issue of not having the measurement for a long time. Nokia also think this is critical and support the proposal from InterDigital.

-	QC support vivo proposal. QC think in the RRM meas relaxation mode NW can have the measurements.

-	OPPO see some benefit to introduce low mob criteria.

-	ZTE support the proposal from InterDigital. ZTE think it may be a comprise if we only use MR results to evaluate low mob for the entry condition.

Chair: we will discuss again in the next meeting. Joint proposal is greatly encouraged.

38.300

R2-2507639	Miscellaneous corrections for LP-WUS	Ericsson	CR	Rel-19	38.300	19.0.0	1046	-	F	NR_LPWUS-Core	Revised

=> Revised in R2-2507640

R2-2507640	Miscellaneous corrections for LP-WUS	Ericsson	CR	Rel-19	38.300	19.0.0	1046	1	F	NR_LPWUS-Core	R2-2507639

Will be updated in the post meeting email discussion

Discussions

-	Nokia point out that with the addition then the old sentence ‘LP-WUS is monitored outside active-time’ is not needed.

[Post131bis][210][LPWUS] CR for TS 38.300 (Ericsson)

Intended outcome: Update the CR for endorsement

Deadline:  Short

37.340

R2-2507369	Open issue of LP-WUS in TS37.340	ZTE Corporation, Sanechips	report	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: For MR-DC, UE can be configured to report MCG specific UE assistance information if the MN is a gNB and/or SCG specific UE assistance information if the SN is a gNB, if it has preferred time offset for PDCCH monitoring after LP-WUS monitoring.

Proposal 2: UE can report preferred time offset for PDCCH monitoring after LP-WUS monitoring for MCG and/or SCG independently for NR-DC.

Discussion

-	Qualcomm wonder how the NW use this information if it is reported to the SN. QC think such report is per UE not per CG.

-	Xiaomi think this is fine, since this is similar as we did for DCP. Xiaomi wonder whether the values can be different for MCG and SCG. ZTE think it can and it is the intention P2.

For MR-DC, UE can be configured to report MCG specific UE assistance information if the MN is a gNB and/or SCG specific UE assistance information if the SN is a gNB, if it has preferred time offset for PDCCH monitoring after LP-WUS monitoring.

UE can report preferred time offset for PDCCH monitoring after LP-WUS monitoring for MCG and/or SCG independently for NR-DC.

R2-2507370	Correction on LP-WUS in TS 37.340	ZTE Corporation, Sanechips, Xiaomi	CR	Rel-19	37.340	19.0.0	0424	-	F	NR_LPWUS-Core

Endorsed

UE capability

R2-2507156	List of open issues for Rel-19 LP-WUS UE capabilities	Huawei, HiSilicon (Rapporteur)	discussion	Rel-19

Noted

R2-2506965	[LPWUS-Cap-OI-1] Discussion on open issue for LP-WUS UE capabilities	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal: The UE capabilities for LP-WUS operation in IDLE/INACTIVE should also be added outside the Rel-19 paging container in UE capability information message. Which contents to be added can be discussed:

Option 1: only indicating the supported band list for LP-WUS regardless of the receiver type and other capabilities;

Option 2: indicating the supported band list for LP-WUS and the receiver type.

R2-2507531	LP-WUS Other Issues	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1 	(LPWUS-Cap-OI-1): RAN2 confirms that gNB can decode Rel-19 paging container (i.e., UE-RadioPagingInfo-r19) although it is defined using OCTET STRING. No further enhancement is necessary for the LP-WUS capability report.

Discussion

-	ZTE think it is possible for the NW to decode. HW think even though it is possible but it unnecessarily complicates NW implementation.

-	Ericsson support Huawei proposal, because it give a clean structure of the signalling.

-	Xiaomi think this topic should be discussed in the main session and think it is not limited to LPWUS.

-	vivo think it is useful since at least some NW vendor will not decode the info in the container.

The UE capabilities for LP-WUS operation in IDLE/INACTIVE should also be added outside the Rel-19 paging container in UE capability information message. Detailed signalling can be further discussed.

R2-2507253	Correction to R19 LP-WUS UE Capabilities	Huawei, HiSilicon	draftCR	Rel-19	38.306	19.0.0	NR_LPWUS-Core

Endorsed.

[Post131bis][211][LPWUS] CR for TS 38.306 (Huawei)

Intended outcome: Update the CR for endorsement

Deadline:  Short

R2-2506965	[LPWUS-Cap-OI-1] Discussion on open issue for LP-WUS UE capabilities	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

R2-2507011	Discussion on other open issues for LP-WUS WUR	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2507043	Discussion on the remaining issues on RRM measurement	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2507083	Remaining issues on RRM relaxation and proposed TP to TS 38.304	Xiaomi Communications	discussion

R2-2507253	Correction to R19 LP-WUS UE Capabilities	Huawei, HiSilicon	draftCR	Rel-19	38.306	19.0.0	NR_LPWUS-Core

R2-2507352	Paging monitoring in LP-WUS CONNECTED state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

R2-2507505	Open issues on LP-WUS	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2507531	LP-WUS Other Issues	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2507618	Discussion on open issues in 38.304 for LP-WUS	Nokia	discussion	Rel-19	NR_LPWUS-Core

R2-2507628	LP-WUS critical issues (E008, 38304-1 and -3)	Ericsson	discussion	Rel-19	NR_LPWUS-Core	Late

## 8.5	Network Energy Saving Enh.

(Netw_Energy_NR_enh-Core; leading WG: RAN1; REL-19; WID: RP-242354)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

### 8.5.1	Organizational

Incoming LS, CR rapporteurs’ inputs, etc.

R2-2506719	Reply LS on RA-RNTI for PRACH adaptation (R1-2506587; contact: Ericsson)	RAN1	LS in	Rel-19	Netw_Energy_NR_enh	To:RAN2

Noted

- Stage 2 open issues

R2-2507272	Report of email discussion [POST131][108][NES] stage-2 CR	Huawei, HiSilicon	report	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1 (4/5): Add the following Note for gap determination:

NOTE x:	When the serving cell is associated with both SSB and OD-SSB in different frequency, “the SSB associated to the initial DL BWP” includes both SSB and OD-SSB.

-	LG supports the proposal but wonders if we should have a general clarification, not only for measurement gaps. Apple and ZTE think this would require a lot of changes.

Agreed

R2-2507273	Network Energy Savings Enhancements miscellaneous stage-2 corrections	Huawei, HiSilicon	CR	Rel-19	38.300	19.0.0	1042	-	F	Netw_Energy_NR_enh-Core

Endorsed

- MAC open issues

R2-2507178	Report of [POST131][111][NES] 38.321 CR	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

MAC Issue 1: RA-RNTI for PRACH adaptation (described in LS R1-2506587)

Noted

MAC issue 2: whether "initialDownlinkBWP-RedCap" can be configured before SIB1 request procedure (e.g. acquired by UE earlier) and can be used for PDCCH monitoring. If so, can the network know that for a redcap UE performing an OD-SIB1 request.

Noted

MAC Issue 3: Monitoring adap-SSBPeriodicityIndication-RNTI during C-DRX

-	rapporteur indicates this is already covered in the rapporteur CR

Noted

R2-2507179	Miscellaneous MAC corrections for R19 NES	InterDigital	CR	Rel-19	38.321	19.0.0	2127	-	F	Netw_Energy_NR_enh-Core

Endorsed

Revised in R2-2507769 to reflect the progress on the remaining MAC issues (in offline 307)

R2-2507769	Miscellaneous MAC corrections for R19 NES	InterDigital	CR	Rel-19	38.321	19.0.0	2127	1	F	Netw_Energy_NR_enh-Core

-	MTK suggests to add FDM to the list of abbreviations

Endorsed

Add FDM to the list of abbreviations in the version submitted to the next meeting

- UE capability open issues

R2-2507367	Open issues on Rel-19 NES UE capability	ZTE Corporation, Sanechips	report	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1 [OPPO001]: Confirm the following understanding on (e)RedCap UE paging capability reporting at both UE and network side and no spec change is needed:

-	From network perspective, if (e)RedCap UE indicate support for paging adaption, NW can configure paging adaption in initialDownlinkBWP-RedCap-r17 and/or initialDownlinkBWP and (e)RedCap UE will follow the configuration in the DL BWP it uses.

-	From UE perspective, A (e)RedCap UE is supposed to use the RedCap-specific initialDownlinkBWP if configured and monitor the adapted POs if configured as part of the (e)RedCap-specific initialDownlinkBWP and if it supports this Rel-19 paging adaption. Otherwise, it uses the initialDownlinkBWP and monitors the adapted POs if provided and if it supports this Rel-19 paging adaption.

-	Oppo is ok not to introduce a new capability but thinks some description about the intended behaviour is needed

Agreed

Proposal 2: Agree the following changes in paging adaption capability description and paging adaption PEI capability description:

Change 1 [Ericsson001]:

pagingAdaptation-r19

Indicates whether the UE supports paging adaption, in which the value range for parameter N and Ns as defined in TS 38.331[9] are extended to make it possible to have increased interval between Paging Frames and compensate the decrease in the number of Paging Frames.	UE	No	No	No

-	rapporteur indicates this is already covered in the rapporteur CR

Agreed

Change 2 [Ericsson002]:

pagingAdaptionPEI-SupportBandList-r19

Indicates whether the UE supports receiving paging early indication in DCI format 2_7 as specified in TS 38.304 [21] for a list of frequency band for paging adaption. The UE shall support UEID based subgrouping for a frequency band if it indicates supporting of paging early indication reception for the frequency band. The set of OFDM symbols within a slot where UE can monitor the PEI PDCCH in Type 2A CSS is the same as the requirement for paging PDCCH in Type 2 CSS for IDLE and INACTIVE mode UEs.

A UE supporting this feature shall also indicate support of pagingAdaptation-r19.

A UE supporting this feature shall also indicate support receiving paging early indication in DCI format 2_7 as specified in TS 38.304 [21] for the same band(s) in pei-SubgroupingSupportBandList-r17.	UE	No	No	No

-	rapporteur indicates this is already covered in the rapporteur CR

Agreed

From:

R2-2506849	Remaining issue on Redcap UE for OD-SIB1 and Paging Adaptation	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 3	R2 discuss the solution to differentiate the support of PO bundling (i.e., pagingAdaptation-r19) and PEI support for PO bundling (i.e., pagingAdaptionPEI-SupportBandList-r19) for non-(e)Redcap UE and (e)Redcap UE, e.g., via the TP on 38.306 as above.

R2-2507368	Correction on Rel-19 NES UE capability	ZTE Corporation, Sanechips, Ericsson	CR	Rel-19	38.306	19.0.0	1362	-	F	Netw_Energy_NR_enh-Core

Agreed in principle

- ASN.1 review

R2-2507660	NES Comments File	Ericsson	report	Rel-19	Netw_Energy_NR_enh-Core

R2-2507661	NES Review File	Ericsson	report	Rel-19	Netw_Energy_NR_enh-Core

R2-2507662	Conclusions for NES RILs	Ericsson	report	Rel-19	Netw_Energy_NR_enh-Core

PropAgree: X204, X205, C182, N002, H126, H100, H101, H102, J003, J004, V501, L203, L204, L205, L206, L209, L210, N003, S030, S031, S035, Z103, A101, A102

X204, X205, C182, N002, H126, H100, H101, H102, J003, J004, V501, L203, L204, L205, L206, L209, L210, N003, S030, S031, S035, Z103, A101, A102 are Agreed

PropReject: X202??, X203, O007, C181??(already in the CR?), C183, C184??, C185, H125??(already in the CR?), H103, H129??, V500??, L201??, L202, L207, L208, S034, S029??

-	Xiaomi thinks X202 should be ToDo

-	LG thinks L201 is ToDo

X203, O007, C183, C185, H103, L202, L207, L208, S034 are Rejected

ToDo: O004, O005, X200, X201, O006, J001, J002, N001, H127, H128, H130, H131, J005, E023, E024, E025, Z101, Z102, A103, V503

O004, O005, X200, X201, O006, J001, J002, N001, H127, H128, H130, H131, J005, E023, E024, E025, Z101, Z102, A103, V503, C181, H125, C184, H129, V500, S029, X202, L201 are ToDo

Duplicate: S028, V502

R2-2507663	Corrections for Network Energy Saving	Ericsson	CR	Rel-19	38.331	19.0.0	5559	-	F	Netw_Energy_NR_enh-Core

-	rapporteur indicates that PropAgree RILs are implemented in this version of the CR. The coversheet might need to be updated to remove the reference to C181 and H125

Endorsed

Revised in R2-2507783 to reflect the meeting agreements

R2-2507783	Corrections for Network Energy Saving	Ericsson	CR	Rel-19	38.331	19.0.0	5559	1	F	Netw_Energy_NR_enh-Core

Discussed in [Post131bis][302]

[Post131bis][302][NES] RRC CR (Ericsson)

Scope: Update the RRC CR with meeting agreements

Intended outcome: endorsed CR in R2-2507783

Deadline:  short

=> Endorsed in R2-2507783 (38.331)

Agreements:

For Stage 2:

1.	Add the following Note for gap determination:

NOTE x:	When the serving cell is associated with both SSB and OD-SSB in different frequency, “the SSB associated to the initial DL BWP” includes both SSB and OD-SSB.

For UE capability:

1.	RAN2 confirms the following understanding on (e)RedCap UE paging capability reporting at both UE and network side and no spec change is needed:

-	From network perspective, if (e)RedCap UE indicate support for paging adaption, NW can configure paging adaption in initialDownlinkBWP-RedCap-r17 and/or initialDownlinkBWP and (e)RedCap UE will follow the configuration in the DL BWP it uses.

-	From UE perspective, A (e)RedCap UE is supposed to use the RedCap-specific initialDownlinkBWP if configured and monitor the adapted POs if configured as part of the (e)RedCap-specific initialDownlinkBWP and if it supports this Rel-19 paging adaption. Otherwise, it uses the initialDownlinkBWP and monitors the adapted POs if provided and if it supports this Rel-19 paging adaption.

R2-2507768	LS on OD-SSB 	Apple	Rel-19	Netw_Energy_NR_enh-Core	To:RAN1 Cc:RAN4

Approved

### 8.5.2	Control plane

Essential RRC corrections (including the issues related to RILs), 38.304, stage-2, and UE capability corrections. Note stage-2 corrections may be handled with lower priority.

- RRC issue 1: [E023/E024/E025/H128/J002/O006/V503] The relationship between SSB-less and OD-SSB case 1

R2-2507162	[J001][J002][J005] Discussion on OD-SSB and SSB Adaption	Sharp	discussion	Rel-19	Late

Proposal 3: An OD-SSB case#1 SCell is not an SSB-less SCell.

R2-2507334	Discussion on RILS E023, E024, E025, X201, O006, J002, H128, H129, J005, Z101, Z102, V503 	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core	Late

Proposal 1: RAN2 to conclude that Case 1 does not require always on SSB, even in reference cells.

-	HW thinks this is restrictive from NW perspective

Continue in offline 302

R2-2507115	Control plane open issues on Rel-19 NES (including RIL E204/E205/A103/X200/O005)	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Observation 1: Specifications changes to TS 38.331 are needed to differentiate the following two scenarios:

•	Scenario 1 (SSB-less SCell): absoluteFrequencySSB is absent in the concerned SCell, and the UE obtains its timing reference from the SpCell or a SCell in the same or different frequency band.

•	Scenario 2 (OD-SSB case 1): absoluteFrequencySSB is absent in the concerned SCell, but od-ssb-absoluteFrequency-r19 is configured in SCellConfig.

Observation 2: As agreed in RAN2#130, NW may configure ServingCellMO for SSB-less SCell if the UE reports it supports capability intraF-NeighMeasForSCellWithoutSSB. Otherwise, NW doesn’t configure ServingCellMO.

Proposal 1 (E024/E025): RAN2 confirm Table.1 on how to configure different scenarios with SSB-less SCell and OD-SSB SCells.

RAN2 confirms the understanding in Table below

-	Ericsson wonders if also the field description plays a role (and if we need to change the field description for absoluteFrequencySSB). Apple agrees we need to update the field description

|  | absoluteFrequencySSB in ServingCellConfigCommon | OD-SSB-r19 in SCellConfig | ssbFrequency in ServingCellMO | ServingCellMO-OD |
| --- | --- | --- | --- | --- |
| Scenario 0: SCell associated with AO-SSB only | Present | Absent | Present | Absent |
| Scenario 1-a: SSB-less SCell with ServingCellMO configured | Absent | Absent | Present | Absent |
| Scenario 1-b: SSB-less SCell with ServingCellMO not configured | Absent | Absent | Absent | Absent |
| Scenario 2: OD-SSB case 1 (i.e. only associated with OD-SSB) | Absent | Present, and its od-ssb-absoluteFrequency-r19 is present | Absent | Present, and its ssbFrequency is same as od-ssb-absoluteFrequency-r19) |
| Scenario 3: AO-SSB and OD-SSB in same frequency | Present | Present, but its od-ssb-absoluteFrequency-r19 is absent. | Present (same as absoluteFrequencySSB) | Absent |
| Scenario 4: AO-SSB and OD-SSB in different frequency | Present | Present, and its od-ssb-absoluteFrequency-r19 is present and different from absoluteFrequencySSB. | Present (same as absoluteFrequencySSB) | Present, and its ssbFrequency is same as od-ssb-absoluteFrequency-r19 |

Proposal 2 (E024/E025): To differentiate SSB-less SCell and OD-SSB case 1, adopt the following spec changes:

- Update the field description of absoluteFrequencySSB to preclude OD-SSB 1 case1 from SSB-less SCell.

- In field description of ServingCellMO, clarify that ServingCellMO is configured for OD-SSB case 1.

- In presence condition of ssbFrequency (i.e. SSBorAssociatedSSB2) under MeasObjectNR, clarify that ssbFrequency is mandatory present in OD-SSB case 1.

-	HW and vivo don’t agree with the first change. vivo thinks it should refer to the OD-SSB configuration not just the frequency

-	HW and ZTE think we don’t need to discuss the third change based on the decision in the previous meeting

Continue in offline 302

Observation 3: RAN1 is discussing whether the UE can perform SSB-less operation (i.e. obtain timing from SpCell or anther SCell) in OD-SSB case 1, when OD-SSB is not activated. However, in case it is agreed in RAN1, RAN2 just need to add some text in field description of absoluteFrequencySSB (i.e. no need to impact ASN.1).

Proposal 3 (E024/E025): RAN2 wait RAN1 conclusion whether the UE can perform SSB-less operation (i.e. obtain timing from PCell or anther SCell) in OD-SSB case 1, when OD-SSB is not activated.

Continue in offline 302

R2-2506936	[H126][L201][X200][A103][H128][H129][X201][H131][H130][H127] Control plane issues	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: Agree [H128], i.e., remove the conditional presence of ODssbAOssb, od-ssb-absoluteFrequency-r19 is OPTIONAL, need R.

- RRC issue 2: [X201] Whether to configure ServingCellMO or ServingCellMO-OD in OD-SSB case 1

R2-2507050	RIL X200, X201, X202, X203 and remaining open issues for NES	Xiaomi	discussion	Netw_Energy_NR_enh-Core

Proposal 2: RAN2 to agree for case 1 (no AO-SSB) when OD-SSB is activated, the measurement is based on servingCellMO, i.e., does not pursue the situation where the ssbFrequency in servingCellMO is different from od-ssb-absoluteFrequency.

-	HW support this option

R2-2507334	Discussion on RILS E023, E024, E025, X201, O006, J002, H128, H129, J005, Z101, Z102, V503 	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core	Late

Proposal 2: RAN2 to agree servingcellMO is not configured for Case1 SCell and OD-SSB specific MO is used for both CSI-RS measurements and OD-SSB measurements.

-	LG supports this alternative

-	Nokia thinks both would work but maybe this alternative would simplify the text

servingcellMO is not configured for OD-SSB measurement in Case1 SCell and OD-SSB specific MO is used for both CSI-RS measurements and OD-SSB measurements after OD-SSB is activated.

- RRC issue 3: how CSI-RS based serving cell measurements would work in OD-SSB

R2-2507614 Control Plane issues [N001[N002][X200] [N003]	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core	Late

Proposal 4: RAN2 to clarify how the CSI-RS based serving cell measurements would work when OD-SSB is configured e.g. based on the table 1.

RAN2 assumes the following table describes how the CSI-RS based serving cell measurements would work when OD-SSB is configured

| Scenario | servingCellMO contains | servingCellMO-OD contains | What is measured (OD-SSB not activated) for serving cell | What is measured (OD-SSB activated) for serving cell |
| --- | --- | --- | --- | --- |
| AO-SSB and OD-SSB both configured and on same frequency | AO-SSB CSI-RS OD-SSB on same freq |  | AO-SSB, CSI-RS | OD-SSB, CSI-RS |
| AO-SSB and OD-SSB both configured and on different frequency | AO-SSB CSI-RS | OD-SSB CSI-RS2 | AO-SSB, CSI-RS | OD-SSB, CSI-RS2 |
| Only OD-SSB configured | CSI-RS | OD-SSB CSI-RS2 | CSI-RS | OD-SSB, CSI-RS2 |
| Only OD-SSB configured |  | OD-SSB CSI-RS2 |  | OD-SSB, CSI-RS2 |

R2-2507334	Discussion on RILS E023, E024, E025, X201, O006, J002, H128, H129, J005, Z101, Z102, V503 	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core	Late

Proposal 7: Agree that UE performs CSI-RS measurements for serving cell based on servingcellMO (if configured) when OD-SSB is not active and based on OD-SSB specific MO when OD-SSB is active.

Proposal 8: Send LS to RAN4 on these RAN2 agreements.

-	Apple is not sure we need to send a LS to RAN4. Nokia agrees

- RRC issue 4: [L201/202] Whether/how to clarify OD-SSB SMTC and SSBAdapt SMTC are only for servng cell measurement

R2-2506848	Left Issues on On-Demand SSB and SSB adaptation (X200, L201, L202, O006)	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core	Late

Proposal 2: [L201] To align with R4 conclusion, R2 confirm the adaptive SMTC usage for OD-SSB does not affect the SMTC usage for neighbouring cell, which is to be performed as in legacy.

Agreed

Proposal 3: [L202] To align with R4 conclusion, R2 confirm the adaptive SMTC usage for SSB adaptation does not affect the SMTC usage for neighbouring cell, which is to be performed as in legacy.

Agreed (can check if any further changes are needed or if this is already covered by the latest RRC CR)

-	ZTE supports both

-	LG supports p2 and thinks p3 is already resolved by the latest RRC CR

- RRC issue 5: [C184] The needs of different ssb-ToMeasure configurations for OD-SSB

R2-2506817	[C184]Impact of od-ssb-PositionsInBurst on ssb-ToMeasure	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: A clarification that OD-SSB-PositionsInBurst is configured as a subset of ssb-ToMeasure within the associated MeasObjectNR should be added to the field description of the OD-SSB-PositionsInBurst.

R2-2507334	Discussion on RILS E023, E024, E025, X201, O006, J002, H128, H129, J005, Z101, Z102, V503 	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core	Late

Proposal 6: RAN2 to agree that OD-SSB specific MO includes values for ssb-ToMeasure to match the OD-SSB activated by the MAC CE.

-	Apple and vivo think this is sufficient

Continue in offline 301

- RRC issue 6: [H130] Whether SSB adaptation is applicable to deactivated SCell

R2-2507614 Control Plane issues [N001[N002][X200] [N003]	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core	Late

Proposal X: Allow direct SSB adaptation activation with RRC configuration.

-	Xiaomi and ZTE thinks that SSB adaptation can be applicable to deactivated Scell without further spec changes (based on group DCI in the Pcell). LG agrees

-	Nokia and Apple think that at the moment there are no RAN4 requirements for SSB adaptation for deactivated SCell

Continue in offline 301

R2-2506936 [H126][L201][X200][A103][H128][H129][X201][H131][H130][H127] Control plane issues	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 8: Agree [H130], i.e., add a conditional presence for adap-SSB-Config-r19: The field is optionally present, Need M, for an activated SCell. It is absent otherwise.

- RRC issue 7: [S029] Enhancement of neighbor cell measurement when OD-SSB is not activated

R2-2506879	handling of RRC open issues	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal: for a measObject (in the measurement identity list), if a SCell corresponding to the frequency of the measObject is configured with on demand SSB and always on/periodic SSB is not configured for the SCell and on demand SSB is not activated and this measObject is associated only with a measurement reporting of a type/event which needs measurements of this SCell: UE ignore the measObject and do not measure neighbor cells according to measObject.

Continue in offline 301

- RRC issue 8: [X200/A103/N001/J001] Procedural text of OD-SSB SMTC setup

R2-2507115	Control plane open issues on Rel-19 NES (including RIL E204/E205/A103/X200/O005)	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4 (A103/X200): RAN2 adopt the following spec changes to simplify and align the procedure text of handling SMTC for OD-SSB and SSB adaptation:

- In ASN.1, modify SSB-MTCxList-r19 and SSB-MTC-SSBAdapt-r19 to use same IE type (e.g. SSB-MTCx-r19) which explicitly includes the mapped SSB periodicity.

- In Section 5.5.2.10, modify the procedure of OD-SSB and SSB adaptation with aligned wording that the SMTC with same mapped SSB periodicity is selected.

Continue the discussion after progress on the other higher level principles

R2-2506936	[H126][L201][X200][A103][H128][H129][X201][H131][H130][H127] Control plane issues	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: On [X200][A103]: Extend the maximum number of OD-SSB specific SMTCs from 6 to 12, no change to the procedure text is needed.

- RRC issue 9: [O005] Applicability of paging adaptation to LP-WUS

R2-2507326	[E023,…][L201][O005] Discussion on RRC open issues	LG Electronics France	discussion	Rel-19	38.331	Netw_Energy_NR_enh-Core	Late

Proposal 7. Do not support scenario co-configuring R19 LP-WUS and R19 paging adaptation.

Coexistence of paging adaptation and LP-WUS is confirmed and further details are to be discussed in LP-WUS session

R2-2507115	Control plane open issues on Rel-19 NES (including RIL E204/E205/A103/X200/O005)	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5 (O005): Wait RAN1 conclusion on whether paging adaptation can be applied to LP-WUS.

- RRC issue 10: [H126/H131/V500/H129] other issues

R2-2506936	[H126][L201][X200][A103][H128][H129][X201][H131][H130][H127] Control plane issues	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: On the PropAgreed [H126], update the wording to “This field is ignored for serving cellon-demand SSB measurements during fast measurement window.”

Agreed

Proposal 7: On [X201][H131]: Clarify that in the following cases the legacy servingCellMO is used for deriving serving cell measurement results:

-	if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is not configured

-	if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is not configured with od-ssb-absoluteFrequency

-	if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is configured with od-ssb-absoluteFrequency and OD-SSB transmission is not activated

-	if absoluteFrequencySSB is not configured in ServingCellConfigCommon and od-ssb is configured and OD-SSB transmission is activated

In the following case servingCellMO-OD is used for deriving serving cell measurement results:

-	if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is configured with od-ssb-absoluteFrequency and OD-SSB transmission is activated

Continue in offline 301

R2-2506966	Remaining CP open issues of NES	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: [V500] Remove the RRC state restriction before the UE acquires SIB1 upon reception of SI change and PWS notification in the OD-SIB1 cell, and adopt the TP1 in the Annex 5.1.

-	Samsung supports the proposal. Also Apple supports the proposal

The UE acquires SIB1 from broadcast upon reception of SI change and PWS notification in the OD-SIB1 cell

R2-2507614 Control Plane issues [N001[N002][X200] [N003]	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core	Late

Proposal 6: Agree [H129], i.e., add the following to the field description of od-ssb-absoluteFrequency: This field is only included when the on-demand SSB has a different SSB frequency from the always-on SSB of the SCell.

Add the following to the conditional presence of od-ssb-absoluteFrequency: This field is only included when the on-demand SSB has a different SSB frequency from the always-on SSB of the SCell (and ensure that case 1 is also reflected)

[AT131bis][301][NES] RRC open issues (Ericsson)

Scope: Continue the discussion on RRC open issues 5, 6, 7, 10

Intended outcome: summary of the offline discussion (in R2-2507761)

F2F time: Wednesday morning coffee break 10:30-11:00

Deadline for offline discussion summary: Thursday 2025-10-16 14:00

[AT131bis][302][NES] SSB-less vs OD-SSB case 1 (Apple)

Scope: Continue the discussion on RRC open issues 1

Intended outcome: summary of the offline discussion (in R2-2507762)

F2F time: FFS

Deadline for offline discussion summary: Tuesday 2025-10-14 14:00

R2-2507761	Report of [AT131bis][301][NES] RRC open issues	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1	Discussion on the impact of od-ssb-PositionsInBurst on ssb-ToMeasure is postponed to next meeting.

Discussion on the impact of od-ssb-PositionsInBurst on ssb-ToMeasure is postponed to next meeting

Proposal 2	[S029] is not pursued.

[S029] is Rejected

Proposal 3	[V500] is agreed and the proposed text in R2-2506966 is endorsed.

[V500] is Agreed and the proposed text in R2-2506966 is endorsed.

Proposal 4	[H127] is not pursued.

[H127] is Rejected

Proposal 5	[H130] is not pursued.

[H130] is Rejected

-	RRC rapporteur suggests to continue discussion on whether there is a need for clarification in the meeting minutes and if there is a need to activate adaptation via RRC configuration.

-	HW understands that if this RIL is rejected we also assume that a UE can receive DCI2-9 from Pcell for SSB adaptation status for deactivated Scells

Can come back in the next meeting to check whether we can confirm the understanding that a UE can receive DCI2-9 from Pcell for SSB adaptation status for deactivated Scells

Proposal 6	[C181] is agreed.

[C181] is Agreed.

Proposal 7	Introduce a new Rel-19 IE to group po-NumPerPEI-r19, payloadSizeDCI-2-7-r19, pei-FrameOffset-r19, where each parameter is mandatory.

Introduce a new Rel-19 IE to group po-NumPerPEI-r19, payloadSizeDCI-2-7-r19, pei-FrameOffset-r19, where each parameter is mandatory.

Proposal 8	X202 is not pursued.

[X202] is Rejected.

Proposal 9	RAN2 confirms that legacy servingCellMO is used for deriving serving cell measurement results in the following cases:

- if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is not configured.

- if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is not configured with od-ssb-absoluteFrequency

- if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is configured with od-ssb-absoluteFrequency and od-ssb transmission is not activated

RAN2 confirms that legacy servingCellMO is used for deriving serving cell measurement results in the following cases:

- - if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is not configured.

- - if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is not configured with od-ssb-absoluteFrequency

- - if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is configured with od-ssb-absoluteFrequency and od-ssb transmission is not activated

Proposal 10	RAN2 confirms that servingCellMO-OD is used for deriving serving cell measurement results in the following cases:

- if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is configured with od-ssb-absoluteFrequency and OD-SSB transmission is activated.

- if absoluteFrequencySSB is not configured in ServingCellConfigCommon and od-ssb is configured and od-ssb is activated.

RAN2 confirms that servingCellMO-OD is used for deriving serving cell measurement results in the following cases:

- - if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is configured with od-ssb-absoluteFrequency and OD-SSB transmission is activated.

- - if absoluteFrequencySSB is not configured in ServingCellConfigCommon and od-ssb is configured and od-ssb is activated.

Continue the discussion on the procedural text for OD-SSB and CSI-RS measurements and for STMC setup for SSB adaptation/OD-SSB in a Post meeting email discussion

[Post131bis][301][NES] OD-SSB  CSI-RS measurements (Xiaomi)

Scope: Discuss procedural text for OD-SSB and CSI-RS measurements and for SMTC setup for SSB adaptation/OD-SSB

Intended outcome: email discussion summary

Deadline:  long

R2-2507762	Report of [AT131bis][302][NES SSB-less vs OD-SSB case 1	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: When AO-SSB in reference cell is present (i.e. absoluteFrequencySSB is present) and OD-SSB is not configured, the UE can obtain timing from SpCell or another SCell (i.e. SSB-less operation).

When AO-SSB in reference cell is present (i.e. absoluteFrequencySSB is present) and OD-SSB is not configured, the UE can obtain timing from SpCell or another SCell (i.e. SSB-less operation).

Proposal 2: When AO-SSB in reference cell is present (i.e. absoluteFrequencySSB is present) and OD-SSB is deactivated, RAN2 assume that the UE can obtain timing from SpCell or another SCell (i.e. SSB-less operation as legacy Rel-18 behavior). And when OD-SSB is activated, the UE obtains timing reference based on OD-SSB. RAN2 updates CR based on this assumption.

When AO-SSB in reference cell is present (i.e. absoluteFrequencySSB is present) and OD-SSB is deactivated, RAN2 assume that the UE can obtain timing from SpCell or another SCell (i.e. SSB-less operation as legacy Rel-18 behavior).

Proposal 3: RAN2 determine whether to send LS to RAN1 to check if any issue in Thursday CB.

We send an LS to RAN1 (cc RAN4) in R2-2507768 with the RAN2 agreements and updated field description asking if they see any issues with this

Proposal 4: To reflect RAN2 assumption, RAN2 agree the following CR on field description of absoluteFrequencySSB:

absoluteFrequencySSB

Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent and od-ssb-r19 is absent in SCellConfig, or if the field is absent and od-ssb-r19 is present in SCellConfig but OD-SSB is not activated, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. if the field is absent and OD-SSB is activated, the UE obtains timing reference based on OD-SSB.

For PCell, this field corresponds to the CD-SSB.

The field description of absoluteFrequencySSB is updated as follows:

Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent and od-ssb-r19 is absent in SCellConfig, or if the field is absent and od-ssb-r19 is present in SCellConfig but OD-SSB is not activated, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For PCell, this field corresponds to the CD-SSB.

Agreements:

[E023/E024/E025/H128/J002/O006/V503]:

1.	When AO-SSB in reference cell is present (i.e. absoluteFrequencySSB is present) and OD-SSB is not configured, the UE can obtain timing from SpCell or another SCell (i.e. SSB-less operation).

2.	When AO-SSB in reference cell is present (i.e. absoluteFrequencySSB is present) and OD-SSB is deactivated, RAN2 assume that the UE can obtain timing from SpCell or another SCell (i.e. SSB-less operation as legacy Rel-18 behavior).

3.	The field description of absoluteFrequencySSB is updated as follows:

Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent and od-ssb-r19 is absent in SCellConfig, or if the field is absent and od-ssb-r19 is present in SCellConfig but OD-SSB is not activated, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For PCell, this field corresponds to the CD-SSB.

4.	[X201]: servingcellMO is not configured for OD-SSB measurement in Case1 SCell and OD-SSB specific MO is used for both CSI-RS measurements and OD-SSB measurements after OD-SSB is activated.

5.	[L201]: To align with R4 conclusion, R2 confirms the adaptive SMTC usage for OD-SSB does not affect the SMTC usage for neighbouring cell, which is to be performed as in legacy.

6.	[L202]: To align with R4 conclusion, R2 confirms the adaptive SMTC usage for SSB adaptation does not affect the SMTC usage for neighbouring cell, which is to be performed as in legacy (can check if any further changes are needed or if this is already covered by the latest RRC CR)

7.	[O005]: Coexistence of paging adaptation and LP-WUS is confirmed and further details are to be discussed in LP-WUS session

8.	[H126]: update the wording to “This field is ignored for serving cellon-demand SSB measurements during fast measurement window

9.	[V500]: The UE acquires SIB1 from broadcast upon reception of SI change and PWS notification in the OD-SIB1 cell

10.	[H129]: Add the following to the conditional presence of od-ssb-absoluteFrequency: This field is only included when the on-demand SSB has a different SSB frequency from the always-on SSB of the SCell (and ensure that case 1 is also reflected)

Additional Agreements:

11.	Discussion on the impact of od-ssb-PositionsInBurst on ssb-ToMeasure is postponed to next meeting

12.	[S029] is Rejected

13.	[V500] is Agreed and the proposed text in R2-2506966 is endorsed.

14.	[H127] is Rejected

15.	[H130] is Rejected (Can come back in the next meeting to check whether we can confirm the understanding that a UE can receive DCI2-9 from Pcell for SSB adaptation status for deactivated Scells)

16.	[C181] is Agreed.

17.	Introduce a new Rel-19 IE to group po-NumPerPEI-r19, payloadSizeDCI-2-7-r19, pei-FrameOffset-r19, where each parameter is mandatory.

18.	[X202] is Rejected.

19.	RAN2 confirms that legacy servingCellMO is used for deriving serving cell measurement results in the following cases:

- if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is not configured.

- if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is not configured with od-ssb-absoluteFrequency

- if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is configured with od-ssb-absoluteFrequency and od-ssb transmission is not activated

20.	RAN2 confirms that servingCellMO-OD is used for deriving serving cell measurement results in the following cases:

- if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is configured with od-ssb-absoluteFrequency and OD-SSB transmission is activated.

- if absoluteFrequencySSB is not configured in ServingCellConfigCommon and od-ssb is configured and od-ssb is activated.

R2-2506817	[C184]Impact of od-ssb-PositionsInBurst on ssb-ToMeasure	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506848	Left Issues on On-Demand SSB and SSB adaptation  (X200, L201, L202, O006)	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core	Late

R2-2506849	Remaining issue on Redcap UE for OD-SIB1 and Paging Adaptation	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506879	handling of RRC open issues	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506936	[H126][L201][X200][A103][H128][H129][X201][H131][H130][H127] Control plane issues	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506966	Remaining CP open issues of NES	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2507050	RIL X200, X201, X202, X203 and remaining open issues for NES	Xiaomi	discussion	Netw_Energy_NR_enh-Core

R2-2507115	Control plane open issues on Rel-19 NES (including RIL E204/E205/A103/X200/O005)	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2507162	[J001][J002][J005] Discussion on OD-SSB and SSB Adaption	Sharp	discussion	Rel-19	Late

R2-2507326	[E023,…][L201][O005] Discussion on RRC open issues	LG Electronics France	discussion	Rel-19	38.331	Netw_Energy_NR_enh-Core	Late

R2-2507334	Discussion on RILS E023, E024, E025, X201, O006, J002, H128, H129, J005, Z101, Z102, V503 	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core	Late

R2-2507465	Discussion on remaining RRC issues	Qualcomm Incorporated	discussion

R2-2507509	User Plane issues for NES	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh

R2-2507614	Control Plane issues [N001[N002][X200] [N003]	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core	Late

R2-2507676	Discussion on NES RILs O004, O005, H127, H130, and more	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

### 8.5.3	User plane

Essential MAC corrections.

- MAC Issue 1: RA-RNTI for PRACH adaptation (described in LS R1-2506587)

  - Issue 1a: whether/how to capture RA-RNTI for additional RACH resource in MAC spec

R2-2507177	[MAC issue 1] RA-RNTI for PRACH adaptation	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1:  Adopt the TP in Annex A for TS 38.321, to specify that if the UE selects an additional RO (configured by addlRACH-Config-Adapt) and the RO overlaps in the frequency domain with legacy ROs, the UE interprets f_id as the index of the PRACH occasion in the frequency domain plus the number of legacy FDMed ROs.

-	Apple thinks we should consider time overlapping.

-	Samsung is ok for capture this in MAC but we need to avoid referring to “legacy”

We capture RA-RNTI for additional RACH resource in MAC spec

Discuss the actual TP in offline 307

  - Issue 1b: whether to capture the restriction of f_id of additional RO <=8 in RRC

R2-2506818	Discussion on MAC open issues	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: Clarify in the field description of msg1-FDM in addlRACH-Config-Adapt that the network ensures that the total value of msg1-FDM-r19 in addlRACH-Config-Adapt and msg1-FDM (without suffix) in rach-ConfigGeneric configured in the associated RACH-ConfigCommon is not more than 8.

-	Interdigital supports this and thinks this is aligned to RAN1

-	LG and Nokia think this is not truly aligned to RAN1 understanding as they refer to valid values

-	Apple thinks we could clarify in the field description that “a UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs)”

We clarify in the field description that a UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) (exact wording can be further discussed)

- MAC issue 2: whether "initialDownlinkBWP-RedCap" can be configured before SIB1 request procedure (e.g. acquired by UE earlier) and can be used for PDCCH monitoring. If so, can the network know that for a redcap UE performing an OD-SIB1 request.

Option 1 The UE monitors PDCCH based on searchspace#0 coreset#0 configuration in OD-SIB configuration

Option 2: The UE monitors PDCCH in redcap specific initial DL BWP

Option 3: not support redcap UE for OD-SIB1

-	IDC thinks we could discuss whether we have separate preambles for RedCap UEs

-	Samsung thinks the only acceptable solution is to go for option 1

-	Xiaomi would like to preclude option 2. Apple agrees option 2 would require more specification work

-	Nokia, ZTE, Ericsson, CATT, vivo, QC support option 1

-	LG support option 1 and option 3.

All UEs (including (e)RedCap UEs) perform an OD-SIB1 request in the same way. The UE monitors PDCCH based on searchspace#0 coreset#0 configuration in OD-SIB configuration

Discuss the actual changes in offline 307

R2-2507116	User plane open issues on Rel-19 NES	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2a: If initialDownlinkBWP-RedCap is configured and Redcap UE has acquired OD-SIB1-Config earlier, the Redcap UE monitors the PDCCH in initialDownlinkBWP, irrespective of whether OD-SIB1 request is triggered in NUL or SUL.

Proposal 2b: RAN2 adopt the TP in Appendix.

R2-2507271	Discussion on remaining User Plane issues of NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: [MAC-2] Introduce a RedCap specific OD-SIB1 request configuration and specify the corresponding OD-SIB1 request procedure, by which the NW/UE could know whether to send/receive the PDCCH on the RedCap specific BWP.

R2-2507464	Discussion on remaining MAC issues	Qualcomm Incorporated	discussion

Proposal 2: Do not support RedCap UEs with OD-SIB1.

[AT131bis][307][NES] MAC open issues (Interdigital)

Scope: Continue the discussion on remaining MAC open issues

Intended outcome: revised MAC CR (in R2-2507769)

Deadline for revised MAC CR: Thursday 2025-10-16 14:00

Agreements:

1.	We capture RA-RNTI for additional RACH resource in MAC spec

2.	We clarify in the field description that a UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) (exact wording can be further discussed)

3.	All UEs (including (e)RedCap UEs) perform an OD-SIB1 request in the same way. The UE monitors PDCCH based on searchspace#0 coreset#0 configuration in OD-SIB configuration

R2-2506818	Discussion on MAC open issues	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506878	handling of MAC open issues	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506967	Remaining UP open issues of NES	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2507051	Remaining MAC open issues for NES	Xiaomi	discussion	Netw_Energy_NR_enh-Core

R2-2507116	User plane open issues on Rel-19 NES	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2507140	Remaining MAC open issues on common signal/channel adaptation	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2507161	[MAC issue 2] Discussion on OD-SIB1 for RedCap UE	Sharp	discussion	Rel-19

R2-2507177	[MAC issue 1] RA-RNTI for PRACH adaptation	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2507251	Discussion on NES user plane open issues	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2507271	Discussion on remaining User Plane issues of NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2507464	Discussion on remaining MAC issues	Qualcomm Incorporated	discussion

R2-2507536	Discussion on RA-RNTI for PRACH adaptation	ASUSTeK	discussion	Rel-19	Netw_Energy_NR_enh-Core

## 8.6	Mobility Enhancement Ph4

(NR_Mob_Ph4-Core; leading WG: RAN2; REL-19; WID: RP-252111)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

### 8.6.1	Organizational

Incoming LS, CR rapporteurs’ inputs, etc.

R2-2506740	LS on definition of CSI-RS based L1 intra/inter-frequency measurement (R4-2512334; contact: Apple)	RAN4	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN2	Cc:RAN1

Noted

R2-2506816	Report of [POST131][115][MOB] Open issues on UE capability (CATT)	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

[AT131bis][105][MOB] Capabilities (CATT)

Scope: Discuss open issues in R2-2506816, RIL V400, and P2 and P3 of R2-2507550.

Intended outcome: Discussion summary in R2-2507720 and CRs (if possible) in R2-2507721 (331) and R2-2507722.

Deadline: CB on Thursday.

R2-2507720	Report of	[AT131bis][105][MOB] Capabilities (CATT)	CATT	report

Proposal 1: (Ericsson-1)For cltm-ExecutionConditionL1-r19, UE supporting this feature shall also indicate support of intraFreqL1-MeasConfig-r18 for at least one band combination.

Proposal 2: (Samsung-1) A separate UE capability is introduced for MCG LTM with the SCG change or SCG addition.

Proposal 3: (Samsung-2)Leave it to RAN4 on whether to define a new UE capability for the supported maximum number of LTM Event configurations.

Proposal 4: [V400] NO separate indicator to enable Rel19 MCG LTM recovery is introduced in the RRC specification, i.e., we reuse the existing attemptLTM-Switch IE.

All above proposals are agreed

R2-2507721	Correction on 38331 for UE capability for Mob Ph4	CATT	draftCR	Rel-19	38.331	19.0.0	F	NR_Mob_Ph4-Core

R2-2507722	Correction on 38306 for UE capability for Mob Ph4	CATT	draftCR	Rel-19	38.306	19.0.0	F	NR_Mob_Ph4-Core

The CRs are endorsed to be merged in capability Rapporteur’s CR

R2-2507170	38.300 open issue list for R19 mobility	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Discussion

- Apple thinks that there is no need for offline

Comeback

- QC would like this to not be for the same cell. NEC, MTK agree.

- ZTE thinks that we can go with what we have agreed in Rel-18.

CLTM and CHO can be configured together (no specification changes are required)

R2-2507121	Miscellaneous corrections for stage-2 in Rel-19 Mobility Enhancements	Apple Inc	CR	Rel-19	38.300	19.0.0	1040	-	F	NR_Mob_Ph4-Core

Discussion

- ZTE thinks that changes about security description should also apply to intra-CU case. We can just remove inter-CU.

Remove the “inter-gNB” from the changes about security

The CR is agreed in principle unseen in R2-2507723

R2-2507723	Miscellaneous corrections for stage-2 in Rel-19 Mobility Enhancements	Apple Inc	CR	Rel-19	38.300	19.0.0	1040	1	F	NR_Mob_Ph4-Core

R2-2507012	Miscellaneous corrections on MAC for Mob Ph4	vivo (Rapporteur)	CR	Rel-19	38.321	19.0.0	2123	-	F	NR_Mob_Ph4-Core

Noted. Will be updated in post email discussion.

R2-2507013	List of MAC open issues for R19 mobility	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2507014	Discussion on MAC open issues for R19 mobility	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Tdocs above noted.

[Post131bis][109][MOB] MAC corrections (vivo)

Scope: Capture agreements from the meeting and update issues list.

Intended outcome: Agreeable in principle CR in R2-2507728.

Deadline: Short.

=> Endorsed in R2-2507728

R2-2507728	Miscellaneous corrections on MAC for Mob Ph4	vivo (Rapporteur)	CR	Rel-19	38.321	19.0.0	2123	1	F	NR_Mob_Ph4-Core

=> Endorsed

R2-2507401	Mobility Review file	Ericsson	report	Rel-19	NR_Mob_Ph4-Core

R2-2507402	Mobility Comments file	Ericsson	report	Rel-19	NR_Mob_Ph4-Core

=> Revised in R2-2507658

R2-2507658	Mobility Comments file	Ericsson	report	Rel-19	NR_Mob_Ph4-Core

R2-2507403	Mobility RILs conclusions	Ericsson	report	Rel-19	NR_Mob_Ph4-Core

Tdocs above noted. RILs status will be updated in post email discussion.

R2-2507404	Corrections on RRC for mobility enhancements Phase 4	Ericsson	CR	Rel-19	38.331	19.0.0	5529	-	F	NR_Mob_Ph4-Core

Noted. Will be updated in post email discussion.

[Post131bis][110][MOB] RRC corrections (Ericsson)

Scope: Capture agreements from the meeting and update RILs list.

Intended outcome: Agreeable in principle CR in R2-2507729 and update of RILs list in R2-2507730 (RIL Comments file) and R2-2507881 (Excel sheet).

Deadline: Short.

=> Endorsed in:

R2-2507729 (38.331)

R2-2507730 (RIL list)

=> Noted in R2-2507881

Agreements

-	For cltm-ExecutionConditionL1-r19, UE supporting this feature shall also indicate support of intraFreqL1-MeasConfig-r18 for at least one band combination.

-	A separate UE capability is introduced for MCG LTM with the SCG change or SCG addition.

-	Leave it to RAN4 on whether to define a new UE capability for the supported maximum number of LTM Event configurations.

-	No separate indicator to enable Rel19 MCG LTM recovery is introduced in the RRC specification, i.e., we reuse the existing attemptLTM-Switch IE.

-	V400 is marked as PropReject.

-	CLTM and CHO can be configured together (no specification changes are required)

### 8.6.2	Control plane

Essential RRC corrections (including the issues related to RILs), stage-2, and UE capability corrections. Note stage-2 corrections may be handled with lower priority.

1. [E005] Handling of radio bearers during LTM cell switch

R2-2507405	Issue with handling of radio bearers during the LTM cell switch [E005]	Ericsson, MediaTek Inc., Samsung, Huawei, HiSilicon, ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

=> Revised in R2-2507659

R2-2507659	Issue with handling of radio bearers during the LTM cell switch [E005]	Ericsson, MediaTek Inc., Samsung, NEC, Huawei, HiSilicon, ZTE Corporation, Sanechips, Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: RAN2 to agree on one of these two solutions to solve the problem with the bearer handling in LTM:

a.	Solution 1: It is clarified in the procedural text that, upon the execution of an LTM cell switch procedure, the UE releases all RLC bearers (configurations and bearers itself).

b.	Solution 2: A new field is introduced within the LTM candidate configuration (which is set by the candidate cell) which indicates to the UE to release all RLC bearers (configurations and bearers itself).

Discussion

- QC wonder if this changes the handling for the RLC bearers

- CATT think that target needs to anyway read the container to provide a delta

- Huawei explains that handling for radio bearer configuration is different because some fields of the configuration cannot be changed

- MTK clarify that is clear in the spec that change of some parameters of the RLC radio bearer configuration is forbiden

- Apple is fine with solution 1

- MTK is also fine with solution 1

It is clarified in the procedural text that, upon the execution of an LTM cell switch procedure with security key change, the UE releases all RLC bearers (configurations and bearers itself).

We capture the agreement in the RRC Rapporteur’s CR

The RIL E005 is marked as PropAgree

2. [M202] Handling report configuration for early CSI acquisition

R2-2506814	[M202] control plane issues for LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1 [M202]: To support early CSI reporting for subsequent LTM, UE should maintain the ltm-CSI-ReportConfig-r19 for early CSI reporting associated with the LTM candidate.

R2-2506924	[B110] [B111] [M202] Maintenance of CSI resource and CSI report configuration after cell switch	Lenovo	discussion	Rel-19	Late

Proposal 1: UE releases LTM-CSI-ReportConfig IE in LTM-candidate IE e.g., after CSI reporting at the target cell after or during cell switch triggered by LTM.

Discussion

- CATT suggest to configure the IE to setupRelease and let the network to handle what the UE should do

- MTK agree with CATT

- Huawei also agree with CATT

The field ltm-CSI-ReportConfig-r19 within LTM-Candidate IE is changed to SetupRelease

We capture the agreement in the RRC Rapporteur’s CR

The RIL M202 is marked as PropAgree

3. [H150] Configuration of execution conditions

R2-2507378	RRC issues for LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core	Late

Proposal 1: Use a single field for execution conditions i.e.,

-	in LTM-Candidate, remove ltm-ExecutionCondition;

-	remove the corresponding procedure text in 5.8.5.18.6;

-	in the RRCReconfiguration message contained in ltm-CandidateConfig, allow including ltm-Config but with ltm-ServingCellExecutionCondition as the only present field.

Discussion

- Ericsson clarify that serving conditions are for the case on when serving cell is not a candidate cell

- Huawei clarify that we can only have in one place and UE apply what it receives

- MTK think that Huawei solution is not very clean. Huawei mentions that we can have a UE variable.

- Nokia same view as MTK

CB [106] on Thursday. We try to produce a TP in R2-2507724, otherwise we pospone.

R2-2507724	38.331 TP for LTM execution conditions	Huawei, HiSilicon	discussion 	Rel-19	NR_Mob_Ph4-Core

The TP in R2-2507724 can be taken as baseline in RRC post email discussion.

The RIL H150 is marked as PropAgree.

4. [H151] Configuration of L1 event-trigger report & execution condition

R2-2507378	RRC issues for LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core	Late

Proposal 3a: Create a new ToAddModList and ToReleaseList in MAC-CellGroupConfig for LTM L1 event-triggered reports.

Proposal 3b: Create a new ToAddModList and ToReleaseList in MAC-CellGroupConfig for L1 events used for execution conditions.

Discussion

- Nokia understand that this needs also to be part of servingCellConfig of the SpCell

- QC agrees with Nokia

- Samsung agree with Nokia

We clarify in field description that ltm-CSI-ReportConfig and ltm-CSI-ResourceConfig can only be configured within ServingCellConfig of the SpCell.

We capture the agreement in the RRC Rapporteur’s CR

The RIL H151 is marked as PropAgree

5. [X153][X152] Configuration of L1 event-trigger report & execution condition

R2-2507434	[X153] [X152] Discussion on RILs X153 and X152	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: Regarding issues pointed out in observation 1 (X153) and observation 2 (X152), following corrections can be considered and the proposed TP in Annex can be agreed:

-	Change the bullet number of the procedure for whether to perform RLC re-establishment and PDCP data recovery (AM DRB) based on the Rel-18 ID (i.e., the corresponding L2 reset operation as defined in Rel-18) to the next level bullet of the conditions about Rel-19 ID, rather than the same level bullet.

-	Replace the condition “else if the field ltm-NoSecurityChangeID is not configured for the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or” with "1> else:".

-	Change the procedure for the update of Rel-18 ID of serving cell to the first level bullet (i.e., “1>”).

Discussion

- Huawei thinks the TP is okay but for the first bullet (1>) we need to also check whether there is a value (on top of whether the Rel-19 IDs are different).

- MTK agree with Xiaomi and change proposed by Huawei

TP in R2-2507434 is agreed to be merged in the RRC Rapporteur’s CR with the change proposed by Huawei (check in first 1> whether there is a value (on top of whether the Rel-19 IDs are different)

RIL X153 and X152 are marked as PropAgree

5. [Z155] The missing description for VarLTM-ServingCellNoSecurityChange

R2-2507528	Discussion on RIL issue [Z155][Z157]	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1a: [Z155] RAN2 to confirm that the NW can configure MCG LTM with NoSecurityChangeID in ltm-Config and SCG LTM with NoSecurityChangeID in ltm-ConfigNRDC simultaneously, for the coexistence of Intra-CU MCG/SCG LTM and Inter-CU SCG/MCG LTM.

Proposal 1b: [Z155] The UE can maintain two independent VarLTM-ServingCellNoSecurityChange, one associated with the ltm-Config and one associated with the ltm-ConfigNRDC, if both are configured.

Discussion

- Nokia understand that security key change at both MCG and SCG is not allowed

- Xiaomi shares the same view as Nokia

- MTK think that ZTE analysis is correct

- Vivo asks whether network is in control of the IDs

The TP proposed in R2-2507528 is agreed to be merged in the RRC Rapporteur’s CR

The RIL Z155 is marked as PropAgree

6. [Z157] Release SCG configuration in case of MCG LTM with SCG configuration

R2-2507528	Discussion on RIL issue [Z155][Z157]	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: [Z157] RAN2 to discuss how to handle the release of the current SCG configuration in case of MCG LTM with SCG configuration:

•	Option 1: The network does not set mrdc-ReleaseAndAdd for MCG LTM with SCG configuration, and the UE autonomously release the SCG part of the current UE configuration upon LTM cell switch execution, i.e. follow the same actions as LTM cell switch triggered on the SCG (see TP2-1 in the Annex).

•	Option 2: The network always sets mrdc-ReleaseAndAdd for MCG LTM with SCG configuration, but the UE shall not release the LTM configuration for the SCG if MR-DC release is triggered due to LTM cell switch execution for MCG LTM with SCG configuration (see TP2-2 in the Annex).

Discussion

- Huawei is okay with Option 2 but TP is not ok. We need to clarify what is released depending on which case.

- MTK slightly prefer Option 1.

CB [107] on Thursday. ZTE to provide a TP in R2-2507725 for Option 2 according to Huawei comment.

R2-2507725	TP on release on current SCG configuration in case of MCG LTM with SCG configuration	ZTE Corporation

The TP in R2-2507725 is endorsed to be merged in the Rapporteur’s RRC CR.

The RIL Z157 is marked as PropAgree.

7. [S036] Release SCG configuration in case of MCG LTM with SCG configuration

R2-2507238	[S036][S037]Discussion on Mobility RILs	Samsung	discussion	Late

Proposal 1: [S036] Update L1-MeasConfigNRDC to include CSI-RS measurement related capabilities.

We add capability for CSI-RS measurements (and potentially new capabilities coming from RAN1 or RAN4) in INM

We capture the agreement in the RRC Rapporteur’s CR

The RIL S036 is marked as PropAgree

8. [S037] Mandatory LTM information in inter-node RRC message

R2-2507238	[S036][S037]Discussion on Mobility RILs	Samsung	discussion	Late

Proposal 2: [S037] Absence of LTM configuration and Reference configuration in in CG-Config and CG-ConfigInfo means that the receiver maintains the values informed via the previous message.

Discussion

- ZTE is fine for the reference configuration but for the ltm-Config we can wait in case it should be removed due to RAN3.

We add ltm-ReferenceConfigurationSCG, ltm-Config, ltm-ReferenceConfigurationMCG in the exeption list in section 11.2.3 of TS 38.331.

We capture the agreement in the RRC Rapporteur’s CR

The RIL S037 is marked as PropAgree

9. [N101] Description of reference configuration

R2-2507436	Remaining Open Issues for RRC	Nokia	discussion

Proposal 4: The description of reference configuration is modified to reflect the changes corresponds to Inter-CU SCG LTM. The proposed changes are given in Annexure.A.3

Discussion

- Huawei think that what is apply for what is already clear from the procedural text. We can clarify that it is used to provide a configuration which is used for a set of non-completed LTM candidate configurations.

- Nokia is fine with Huawei proposal

In the description ReferenceConfig IE it is clarified that this is a configuration which is used for a set of non-completed LTM candidate configurations.

We capture the agreement in the RRC Rapporteur’s CR

The RIL N101 is marked as PropAgree

10. [N102] Field description of ltm-ConfigurationSCG

R2-2507436	Remaining Open Issues for RRC	Nokia	discussion

Proposal 5: The restrictions on inclusion of ltm-configurationSCG in RRC-Configuration for certain cases should be moved to the top-level structure LTM-Config-NRDC. TP provided in A.4

Discussion

- MTK agree with the change

- Huawei is also okay with the change

The restriction which is now in ltm-ConfigurationSCG is moved to field description of ltm-ConfigNRDC.

We capture the agreement in the RRC Rapporteur’s CR

The RIL N102 is marked as PropAgree

Agreements

-	It is clarified in the procedural text that, upon the execution of an LTM cell switch procedure with security key change, the UE releases all RLC bearers (configurations and bearers itself).

-	The RIL E005 is marked as PropAgree

-	The field ltm-CSI-ReportConfig-r19 within LTM-Candidate IE is changed to SetupRelease.

-	The RIL M202 is marked as PropAgree

-	The TP in R2-2507724 can be taken as baseline in RRC post email discussion.

-	The RIL H150 is marked as PropAgree.

-	We clarify in field description that ltm-CSI-ReportConfig and ltm-CSI-ResourceConfig can only be configured within ServingCellConfig of the SpCell.

-	We capture the agreement in the RRC Rapporteur’s CR

-	The RIL H151 is marked as PropAgree

-	TP in R2-2507434 is agreed to be merged in the RRC Rapporteur’s CR with the change proposed by Huawei (check in first 1> whether there is a value (on top of whether the Rel-19 IDs are different)

-	RIL X153 and X152 are marked as PropAgree

-	The TP proposed in R2-2507528 is agreed to be merged in the RRC Rapporteur’s CR

-	The RIL Z155 is marked as PropAgree

-	The TP in R2-2507725 is endorsed to be merged in the Rapporteur’s RRC CR.

-	The RIL Z157 is marked as PropAgree.

-	We add capability for CSI-RS measurements (and potentially new capabilities coming from RAN1 or RAN4) in INM.

-	The RIL S036 is marked as PropAgree

-	We add ltm-ReferenceConfigurationSCG, ltm-Config, ltm-ReferenceConfigurationMCG in the exeption list in section 11.2.3 of TS 38.331.

-	The RIL S037 is marked as PropAgree

-	In the description ReferenceConfig IE it is clarified that this is a configuration which is used for a set of non-completed LTM candidate configurations.

-	The RIL N101 is marked as PropAgree

-	The restriction which is now in ltm-ConfigurationSCG is moved to field description of ltm-ConfigNRDC.

-	The RIL N102 is marked as PropAgree

R2-2507093	RRC open issues for R19 mobility	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506814	[M202] control plane issues for LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506924	[B110] [B111] [M202] Maintenance of CSI resource and CSI report configuration after cell switch	Lenovo	discussion	Rel-19	Late

R2-2507015	Discussion on RRC open issues for R19 mobility	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2507238	[S036][S037]Discussion on Mobility RILs	Samsung	discussion	Late

R2-2507378	RRC issues for LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core	Late

R2-2507434	[X153] [X152] Discussion on RILs X153 and X152	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2507436	Remaining Open Issues for RRC	Nokia	discussion

R2-2507528	Discussion on RIL issue [Z155][Z157]	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2507550	Remaining CP issues in R19 mobility	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

### 8.6.3	User plane

Essential MAC corrections.

[AT131bis][108][MOB] MAC corrections (vivo)

Scope: Discuss MAC-V03, MAC-O02, MAC-S02, MAC-H03, MAC-H04 (II Part), MAC-O01, MAC-O03, MAC-M01, MAC-K02. If there is time, other issues also can be included but only if there is good support.

Intended outcome: Report in R2-2507726.

Deadline: Offline on Tuesday Oct 14th (10:00-11:00, main room).

R2-2507726	Discussion report on [AT131bis][108][MOB] MAC corrections (vivo)	vivo	discussion

(MAC-V03) If an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is modified or removed, RRC layer should trigger MAC layer to remove the measurement reporting entry, stop the periodical reporting timer, and reset the associated information (e.g. timeToTrigger) for this ltm-CSI-ReportConfigId.

(MAC-O02) The SR resource for (truncated) MR MAC CE has the same priority as the SR resource for BFR MAC CE.

(MAC-O02) If the SR resource for (truncated) MR MAC CE overlaps with SR resource for BFR MAC CE (for an SCell or a BFD-RS set), it’s up to UE implementation which SR resource to be selected. Update the Note in the current MAC specification.

(MAC-S02) Not agree to move beams from the BEAM_ENTERING_LIST and BEAM_LEAVING_LIST when they are reported in a truncated measurement report. Keep the current MAC spec. as it is.

(MAC-H03) Proponent companies to comment the CR during post-meeting email discussion. Rapporteur will consider them accordingly.

For MAC-H04 (2nd Part), take the below change as the starting point:

Text in the current CR:

1> for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig:

is changed as:

1> for each ltm-CSI-ReportConfigId included in the SpCell for L1 measurement and event-triggered report:

(MAC-O01) Agree the TP in R2-2507094.

(MAC-O03) The UE releases the stored TA value and stops the corresponding CLTM TAT if the NW releases the execution condition for the LTM candidate cell.

(MAC-M01) The case that“UE can only perform RACH-based LTM for CLTM recovery”should be captured in the specification.

(MAC-K02) UE will not perform any report for the case: if the UE receives more TAs beyond its capability, UE releases some TA by UE implementation. No spec change is needed.

Discussion

- MTK for M01 the CFRA case should also be valid. Vivo is fine and thinks we can remore the (CBRA)

- Lenovo think that legacy in Rel-18 is only for CBRA. Nokia agree and don’t want to optimize. Samsung explains that in current spec UE will only do CBRA.

All proposals above are agreed

1. MAC-V01 RACH based LTM with MIMO 2TA

R2-2507014	Discussion on MAC open issues for R19 mobility	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: [MAC-V01] RAN2 selects from the following three options to address the mismatch issue that the TCI state ID indicated in the LTM Cell Switch Command MAC CE and TA value in the RAR within the RACH-based LTM procedure are associated with different TAGs:

-	Option 2: Target DU sends a new TCI state in RACH Msg 4.

-	Option 3: If the mismatch issue occurs, UE follows the TCI state associated with the RACH-based LTM procedure. Otherwise, UE follows the indicated TCI state in the LTM cell switch command.

-	Option 4: UE selects the SSB associated with the same TAG ID as the TAG ID associated with the indicated TCI state in LTM Cell Switch Command MAC CE during the RACH-based LTM procedure.

R2-2507573	User Plane issues for CLTM and event triggered L1 MR	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: (MAC-V01) Rel-18 agreement and solution can be applied to Rel-19 inter-CU LTM, i.e., no spec impact is needed.

Discussion

- LG prefers to have the same as Rel-18

- Xiaomi agree with LG. Oppo agree. Nokia agree.

- Vivo think that in Rel-18 there was no time and maybe now we can solve it. Samsung explains that Rel-18 solution is also fine for Rel-18.

For the RACH based LTM with MIMO 2TA, Rel-18 agreement and solution can be applied to Rel-19 inter-CU LTM, i.e., no spec impact is needed.

MAC-V01 is closed

2. MAC-V02 CFRA resource in LTM Cell Switch Command MAC CE

R2-2507014	Discussion on MAC open issues for R19 mobility	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: [MAC-V02] For inter-CU LTM, a target cell specific CFRA resource pool could be provided by target gNB-DU to source gNB-DU, and the CFRA resource are assigned by S-DU in the LTM cell switch command MAC CE.

R2-2507078	Handling of MAC open issues on C(LTM)	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: No need to discuss and specify anything in R19 for MAC-V02.

Discussion

- Ericsson agree is fine with Samsung proposal. LG agree. CATT agrees.

For MAC-V02 we do not specify any special handling, and we rely on what is already specified in Rel-18.

MAC-V02 is closed

3. MAC-V04 TA update of CLTM TAT

R2-2507014	Discussion on MAC open issues for R19 mobility	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 7: [MAC-V04] Upon receiving an LTM Cell switch Command MAC CE with valid TA, UE shall store the TA value for the target cell and start or restart the CLTM TAT of the target cell.

R2-2507573	User Plane issues for CLTM and event triggered L1 MR	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 5: (MAC-V04) When the UE receives LTM cell switch command including a TA value and this command indicate one LTM candidate cell, the UE does NOT maintain this TA value as a TA value for a candidate cell and does NOT start associated TAT for CLTM.

Discussion

- Xiaomi supports vivo proposal. Nokia, MTK are also fine.

- Oppo support sharp proposal.

- LG things vivo proposal is just an optimization

- ZTE thinks this if we maintain TAT for candidate than UE needs to maintain 2 TATs

- Huawei thinks vivo proposal may create issue if not many TAs are supported

- Samsung has similar view as LG. CATT is also fine

- Ericsson has no strong view but agree with Huawei and the others.

If network wants the UE to store the TA in the LTM cell switch command for a CLTM candidate cell, the TA MAC CE (for CLTM) can be sent from the target after the LTM cell switch procedure is completed.

MAC-V04 is closed

4. MAC-O03 Handling of TA when CLTM candidate is released (TA in LTM CSC MAC CE)

R2-2507094	MAC open issues for R19 mobility	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: Upon release of the CLTM execution condition, the UE stops CLTM TAT if running even if the corresponding LTM candidate configuration is kept.

R2-2507457	Discussion on remaining User Plane issues	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 7: (MAC-O03) The UE keeps any TA validity timer for an LTM candidate which has its execution conditions removed.

Discussion

- Samsung supports Oppo proposal.

- Nokia thinks that we don’t need to specify anything. But if we specify something then Oppo proposal is fine.

Upon release of the CLTM execution condition, the UE stops CLTM TAT if running even if the corresponding LTM candidate configuration is kept.

MAC-O03 is closed

5. MAC-V05/MAC-K01/MAC-N01 Handling of TA when CLTM candidate is released (TA in RAR and TA MAC CE)

R2-2507014	Discussion on MAC open issues for R19 mobility	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 8: [MAC-V05] Upon receiving an (Absolute) Timing Advance Command MAC CE or a RAR, UE shall store the TA value and start or restart the CLTM TAT of the candidate cell(s) that share the same PCI as the serving cell(s) of the TAG.

R2-2507573	User Plane issues for CLTM and event triggered L1 MR	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 6. (MAC-V05, MAC-K01, MAC-N01) When the UE receives a TAC MAC CE for a serving cell, UE does NOT store a valid TA value for a CLTM candidate cell and update its CLTM TAT of the source cell for possible RACH-less CLTM back to the source cell.

Discussion

- Samsung think that since the UE releases the TA then this RILs are not an issue anymore.

For issues MAC-V05/MAC-K01/MAC-N01 we do not any enhancements.

MAC-V05/MAC-K01/MAC-N01 are closed

6. MAC-F02 Threshold for beam selection

R2-2507190	[MAC-F02] Threshold for beam selection	Ofinno	discussion	Rel-19	NR_Mob_Ph4

Proposal 1.	For beam selection for L3-based RACH-less CLTM based on a configured threshold, RAN2 to agree the configured threshold is one of:

ALT1: Reusing existing Rel-18 threshold (cg-RRC-RSRP-ThresholdSSB), according to TP1 for ALT1;

ALT2: Introduce new Rel-19 threshold (cg-LTM-RSRP-ThresholdSSB), according to TP2 for ALT2.

Discussion

- Samsung prefer to re-use existing threshold. Xiaomi agrees.

For beam selection for L3-based RACH-less CLTM based on a configured threshold, the existing cg-RRC-RSRP-ThresholdSSB is re-used.

MAC-F02 is closed

Agreements

-	(MAC-V03) If an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is modified or removed, RRC layer should trigger MAC layer to remove the measurement reporting entry, stop the periodical reporting timer, and reset the associated information (e.g. timeToTrigger) for this ltm-CSI-ReportConfigId.

-	(MAC-O02) The SR resource for (truncated) MR MAC CE has the same priority as the SR resource for BFR MAC CE.

-	(MAC-O02) If the SR resource for (truncated) MR MAC CE overlaps with SR resource for BFR MAC CE (for an SCell or a BFD-RS set), it’s up to UE implementation which SR resource to be selected. Update the Note in the current MAC specification.

-	(MAC-S02) Not agree to move beams from the BEAM_ENTERING_LIST and BEAM_LEAVING_LIST when they are reported in a truncated measurement report. Keep the current MAC specification as it is.

-	(MAC-H03) This issue will be addressed during the MAC CR post-meeting email discussion.

-	For MAC-H04 (2nd Part), take the below change as the starting point:

Text in the current CR:

1> for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig:

is changed as:

1> for each ltm-CSI-ReportConfigId included in the SpCell for L1 measurement and event-triggered report:

-	(MAC-O01) Agree the TP in R2-2507094.

-	(MAC-O03) The UE releases the stored TA value and stops the corresponding CLTM TAT if the NW releases the execution condition for the LTM candidate cell.

-	(MAC-M01) The case that“UE can only perform RACH-based LTM for CLTM recovery”should be captured in the specification.

-	(MAC-K02) UE will not perform any report for the case: if the UE receives more TAs beyond its capability, UE releases some TA by UE implementation. No spec change is needed.

-	For the RACH based LTM with MIMO 2TA, Rel-18 agreement and solution can be applied to Rel-19 inter-CU LTM, i.e., no spec impact is needed.

-	MAC-V01 is closed

-	For MAC-V02 we do not specify any special handling, and we rely on what is already specified in Rel-18.

-	MAC-V02 is closed

-	If network wants the UE to store the TA in the LTM cell switch command for a CLTM candidate cell, the TA MAC CE (for CLTM) can be sent from the target after the LTM cell switch procedure is completed.

-	MAC-V04 is closed

-	Upon release of the CLTM execution condition, the UE stops CLTM TAT if running even if the corresponding LTM candidate configuration is kept.

-	MAC-O03 is closed

-	For issues MAC-V05/MAC-K01/MAC-N01 we do not any enhancements.

-	MAC-V05/MAC-K01/MAC-N01 are closed

-	For beam selection for L3-based RACH-less CLTM based on a configured threshold, the existing cg-RRC-RSRP-ThresholdSSB is re-used.

-	MAC-F02 is closed

R2-2506815	Discussion on SP CSI-RS and CSI-IM for early CSI acquisition	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2507304	Collision between PUSCH for early CSI and measurement gap	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2507078	Handling of MAC open issues on C(LTM)	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2507094	MAC open issues for R19 mobility	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2507190	[MAC-F02] Threshold for beam selection	Ofinno	discussion	Rel-19	NR_Mob_Ph4

R2-2507379	MAC issues for LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2507435	Discussion on mobility MAC open issues	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2507457	Discussion on remaining User Plane issues	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2507462	On the open MAC issues for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

R2-2507485	LTM MAC remaining issues 	Qualcomm Incorporated	discussion

R2-2507529	Discussion on MAC open issues	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2507537	Discussion on MAC open issues for CLTM	ASUSTeK	discussion	Rel-19	38.321	NR_Mob_Ph4-Core

R2-2507551	Remaining MAC issues in R19 mobility	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2507573	User Plane issues for CLTM and event triggered L1 MR	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

## 8.7	XR Enhancements Ph3

(NR_XR_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-250107)

Time budget: 0 TU

Tdoc Limitation: 3 tdocs

### 8.7.1	Organizational

LS, rapporteur input, open issues lists etc.

Rapporteur CRs

R2-2506810	Corrections for XR enhancements	Qualcomm France	CR	Rel-19	38.321	18.6.0	2122	-	D	NR_XR_Ph3

R2-2507016	Miscellaneous corrections on RLC for R19 XR	vivo	CR	Rel-19	38.322	19.0.0	0066	-	F	NR_XR_Ph3-Core

R2-2507054	Correction to RRC spec for R19 XR	Huawei, HiSilicon	CR	Rel-19	38.331	19.0.0	5504	-	F	NR_XR_Ph3-Core	Late

H201 will be removed from RRC CR as it is handled by RRC spec rapporteur

The CRs above are endorsed as a baseline for further changes after the meeting

PDCP CR will be provided after this meeting

[AT131bis][508][XR] Update 38.331 CR (Huawei)

Scope: Update and review 38.331 CR

Intended outcome: Agreeable CR

Deadline:  Short

Open issues

R2-2507052	R19 XR RRC comment file	Huawei, HiSilicon	discussion	NR_XR_Ph3-Core	Late

R2-2507053	R19 XR RRC review file	Huawei, HiSilicon	discussion	NR_XR_Ph3-Core	Late

The following RILs are proposed as PropAgree: H200, S039, S055, Z203

The following RILs are proposed as PropReject: V051, O400, S038

The following RILs are ToDo in XR WI: V050, N091

The following RIL is ToDo in general ASN.1 AI: H202

The following RILs are agreed: H200, S039, S055, Z203

The following RILs are rejected: O400

The following RIL will be handled in general ASN.1 AI: H202

R2-2507017	List of RLC open issues for R19 XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: Following open issues are suggested to be discussed based on companies’ contributions: RLC-X01, RLC-V01, RLC-N01, RLC-N02.

Proposal 2: Proponent companies are invited to provide proposals on the following issues, which Rapporteur understands there is no problem: RLC-E01, RLC-H02, RLC-S01.

Proposal 3: Following open issues are suggested to be discussed during the CR phase: RLC-H01, RLC-V02.

Noted

R2-2507245	Offline 504 on XR Stage 2 Open Issues	Nokia (Rapporteur)	discussion	Rel-19	NR_XR_Ph3-Core

Summary: No open issues identified and the Stage 2 can be kept as is.

Noted

R2-2507130	PDCP open issues for XR	LG Electronics Inc. (Rapporteur)	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1 (S001). Include “i:th” in the definition of “Non-delay-reporting PDCP SDU”.

Proposal 2 (H001). Discuss whether to change the definition of “Non-delay-reporting PDCP SDU” so that each of a non-delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is exclusive.

Proposal 3 (N001). Discuss whether to capture a NOTE to specify that “whether to re-send the gap report after HO can be left to UE implementation”.

(S001) Include “i:th” in the definition of “Non-delay-reporting PDCP SDU”.

R2-2507430	Summary of [POST131][508][XR] Discussion on XR MAC open issues 	Qualcomm France	discussion

Proposal 1.  Adopt the TPs proposed in Issue A.2, B.2, C.4 and C.5.

Proposal 2.  Improve the wordings for the issues raised in Issue C.3 and D.1.

Proposal 3.  Discuss more on Issue D.2.

Rapporteur suggestion: Discuss wording for D.2 offline considering TPs from Tdocs 6841, 7301, 7057, 7309

Issue A.2, B.2, C.4, C.5, C3. and D.1 are agreed (already part of rapporteur CR)

### 8.7.2	RRC corrections

Corrections to TS 38.331 which require Tdoc submission as per RIL list.

Simultaneous configuration of auto-ReTx and enhanced polling (V050)

R2-2507160	Views on RIL050 and RIL051	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: [RIL-V050] We propose to allow configuring both thresholds (remaining time based RLC polling and remaining time based RLC retransmission) for the same RLC entity.

R2-2507018	[V050, V051] Discussion on RRC open issues for R19 XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: [V050] Capture in the field description that the value of remainingTimeThresholdRLC-ReTx should be set lower than remaingTimeThresholdRLC-Polling.

DISCUSSION:

- Ericsson thinks that we can leave it up to NW implementation how to set the thresholds.

- LGE has not strong view on whether to capture the limitation.

- Huawei thinks we can leave this up to NW implementation. There seem to be no issues, as it depends when the network wants to include polling

- Vivo asks what is the aim of polling after retransmission has been triggered.

- Huawei clarifies that the transmitter may want to move the window.

- Vivo thinks this was not the intention to introduce this feature.

- Samsung is OK to leave up to NW.

[RIL-V050] Allow configuring both thresholds (remaining time based RLC polling and remaining time based RLC retransmission) for the same RLC entity.

[V050] Do not capture in the field description that the value of remainingTimeThresholdRLC-ReTx should be set lower than remaingTimeThresholdRLC-Polling.

UL rate query as a subset of UL rate control (V051)

R2-2507160	Views on RIL050 and RIL051	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: [RIL-V051] Specify that ul-RateQueryConfigList is the subset of ul-RateControlConfigList.

R2-2507018	[V050, V051] Discussion on RRC open issues for R19 XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: [V051] Clarify in the field description that the QoS flow(s) configured in rate query should be the subset of QoS flow(s) configured for rate control.

DISCUSSION:

- LGE agrees with the proposals, but the network should ensure this. Ericsson agrees.

- Nokia think this may have an impact on MAC specifications. This may require us to refer to QoS flows separately for DL and UL.

- OPPO agrees that this should be like this, but we do not have to clarify this in RRC. We usually do not specify such restrictions. QCM, Lenovo agrees.

[V051] Do not clarify in the field description that the QoS flow(s) configured in rate query should be the subset of QoS flow(s) configured for rate control.

[V051] RAN2 assumes the network implementation will configure it properly, i.e. the QoS flow(s) configured in rate query should be the subset of QoS flow(s) configured for rate control.

UAI transmission and prohibit timer for UAI (N091, S038)

R2-2507510	RIL N091 and S038 on UAI for measurement gap skipping	Nokia, Nokia Shanghai Bell, Huawei	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: (RIL-N091) Adopt the TP above to change the first condition to transmission of UAI for preference for a measurement gap configuration other than the UAI message itself.

Proposal 2: (RIL-S038) If the prohibit timer T346o checking is moved to the 1> level, it should be reset upon new MG configuration to avoid preventing reported preference for the newly configured MG, otherwise the specification should be left as it is.

R2-2507629	RRC Corrections for XR	Samsung	discussion	Rel-19	Late

Proposal 1: [N091] No specification change is considered for UAI to be sent without delay for a new configuration of gap occasion ratio.

Proposal 2: [S038] Agree to keep T346o not running check as general condition and place it in 1> bullet. (Adopt TP1)

DISCUSISON on N091:

- QCM supports Nokia’s proposal.

- Fujitsu thinks a clarification is not necessary as we agreed to have a common timer for all configurations. We do not have to optimize. Xiaomi agrees.

- Nokia thinks this is not an optimization. Prohibit timer should be applied only after information has been sent, not before.

- Ericsson asks what the problem is if we send it later. Is there any impact on the performance?

- Nokia thinks it is better to provide this information earlier if we can.

- Lenovo thinks Nokia’s proposal captures RAN4 intended behaviour and supports it.

- Ofinno supports the TP from Nokia as it follows the current UAI principles. There is no need to delay.

- Samsung is OK with the trigger proposed by Nokia.

- ZTE thinks it is not critical, but is OK with the change.

- MTK is also OK with the change as this is consistent with UAI.

- LGE thinks that after sending UAI due to new trigger we need to restart the prohibit timer. Nokia agrees.

DISCUSISON on S038:

- Xiaomi thinks we should keep the timer where it is, otherwise it conflicts with N091 resolution.

- Vivo thinks we should move the timer upwards.

(RIL-N091) We allow UAI triggering for a new MG configuration that has not been sent in UAI before, use the following TP from R2-2507510 as a baseline:

(S038) Keep the prohibit timer where it is.

Agreements for RRC - H201 will be removed from RRC CR as it is handled by RRC spec rapporteur - The following RILs are agreed: H200, S039, S055, Z203 - The following RILs are rejected: O400 - The following RIL will be handled in general ASN.1 AI: H202 - [RIL-V050] Allow configuring both thresholds (remaining time based RLC polling and remaining time based RLC retransmission) for the same RLC entity. - [V050] Do not capture in the field description that the value of remainingTimeThresholdRLC-ReTx should be set lower than remaingTimeThresholdRLC-Polling. - [V051] Do not clarify in the field description that the QoS flow(s) configured in rate query should be the subset of QoS flow(s) configured for rate control. - [V051] RAN2 assumes the network implementation will configure it properly, i.e. the QoS flow(s) configured in rate query should be the subset of QoS flow(s) configured for rate control. - (RIL-N091) We allow UAI triggering for a new MG configuration that has not been sent in UAI before, use the following TP from R2-2507510 as a baseline: - (S038) Keep the prohibit timer where it is.

R2-2506840	Discussion on RRC for XR	CATT,CBN	discussion	Rel-19	NR_XR_Ph3-Core

R2-2507300	XR RRC Corrections	ZTE Corporation, Sanechips	discussion

R2-2507470	N091, S038	Ericsson	discussion	Rel-19

### 8.7.3	User plane corrections

Corrections to 38.321, 38.322 and 38.323 for all features.

RLC-X01

R2-2507084	Remaining issues on DSR and proposed TP	Xiaomi Communications	discussion

Proposal 1.  	RAN2 to confirm whether the calculation of non-delay reporting data in RLC should follow RLC’s own buffer status or just follow PDCP’s indication.

Proposal 2.  If the calculation of non-delay reporting data in RLC should follow RLC’s own buffer status, then PDCP only needs to indicate the delay reporting data to RLC. Thus, RLC will calculate the non-delay reporting RLC SDU based on its own buffer status.

R2-2506841	Leftover Issue on User Plane	CATT	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: [RLC-X1] RLC layer follows the same order as in PDCP layer for SDU handling， remove the RLC calculation of the non-delay reporting data.

(RLC-X01) We keep the spec as it is.

RLC-V01

R2-2507020	Discussion on RLC open issues for R19 XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1	[RLC-V01] The receiving RLC entity should indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires for HFN synchronization and avoiding packet delivery delay.

R2-2507129	Remaining open issues related to RLC enhancements	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1. (RLC-V01) No need to indicate the SN of obsolete RLC SDUs to the receiving PDCP entity after t-RxDiscard expires.

(RLC-V01) No need to indicate the SN of obsolete RLC SDUs to the receiving PDCP entity after t-RxDiscard expires.

RLC-N01

R2-2507310	Remaining RLC open issues on avoiding unnecessary re-transmissions	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: Update the RX_Next state variable description in the TS 38.322 as follows:

DISCUSISON:

- Ericsson’s understanding was that the current text works as the receiver just assumes tit was fully received.

- Huawei does not think the change is necessary.

- LGE thinks that in case of fake ACK the SDUs are actually not received. Has not strong view, but IDT proposal is fine.

- Nokia would prefer to clarify in the text and prefer their TP.

- Lenovo would also prefer to clarify. The current text is not correct.

- Ericsson suggest to clarify that when the timer expires, then SDUs are considered received and then we do not clarify in the definition.

(RLC-N01) We will clarify RX_Next state variable update when t-RxDiscard expires. FFS an exact wording and where to do the change (e.g. procedure or variable definition) (offline)

RLC-N02

R2-2507159	UP Open Issues	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 5: [RLC-N02] The NOTE in clause 5.3.3.3 is updated as below:

R2-2507532	User plane corrections for XR Enhancements Ph3	NTT DOCOMO INC..	discussion	Rel-19

Proposal 3.	It is no problem to stop and reset the running t-PollRetransmit even when all RLC SDUs with SNs up to and including POLL_SN are already negatively acknowledged. There is no change of NOTE in 5.3.3.3 other than adding “If stopReTxDiscardedSDU is configured” at the beginning of the sentence

DISCUSSION:

- LGE thinks Nokia’s change is aligned with the procedure. We can remove borth NACK and ACK from the note. Samsung has the same view.

- Apple disagrees with LGE and Samsung. This proposal does not always work. We just need a change from NTT DCM.

- Ofinno suggests not to speak of “positively or negatively”, just “acknowledged.

(RLC-N02) In NOTE in 5.3.3.3 we add “If stopReTxDiscardedSDU is configured” at the beginning of the sentence.

(RLC-N02) FFS whether any other changes are needed, e.g. as in R2-2507159 (offline)

RLC-E01

R2-2507471	RLC-E01, RLC-X01	Ericsson	discussion	Rel-19

Proposal 1	(RLC-E01) Allow the RLC Rx entity to receive ‘full SNs’ before triggering the t-RxDiscard timer i.e., start t-RxDiscard if RX_Next_Highest> RX_Next +1.

DISCUSSION:

- LGE thinks that this is not critical as there are other ways of detecting RLF.

- Ericsson thinks there are situations where PHY layer will not detect RLF and they would like to be able to detect such case.

- Xiaomi thinks this scenario is very rare, current spec is fine.

- Sharp, Samsung agrees with Xiaomi and LGE. Proper configuration should cover this.

- Ericsson would prefer to be careful on these rare cases as well.

(RLC-E01) No change is needed.

RLC-H02

R2-2507056	Discussion on remaining issues for RLC for R19 XR	Huawei, HiSilicon	discussion	NR_XR_Ph3-Core

Proposal1: (RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Specify the content of the RLC STATUS PDU triggered by the t-RxDiscard expiry in a separate condition.

DISCUSSION:

- Nokia thinks that this is already clear from other sections.

- LGE thinks that according to current specification this case will not happen. Current spec is OK.

- Fujitsu indicates that the variable is not updated when t-Rx discard expires, so there can be a problem.

- Huawei and Lenovo think the situation can happen so it is better to clarify.

- MTK also thinks this can happen.

Discuss offline RLC-H02

RLC-S01

R2-2507315	Discussion on open issues for RLC and PDCP	Samsung	discussion	Rel-19

[RLC-S01] Proposal 5: PDCP triggers a PDCP SN gap report when stopReTxDiscardedSDU is configured in all RLC entities and at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. (Adopt TP3)

DISCUSSION:

- LGE thinks we have already discussed this and we agreed not to modify the specs. LGE thinks even if it is under HARQ, we can report to receiving entity. LGE thinks Samsung’s change is not complete, but LGE prefers to change nothing.

(RLC-S01) No change needed as per the previous agreement

R2-2507903 Discussion on the RLC STATUS PDU at t-RxDiscard expiry	Huawei, HiSilicon

R2-2507905 Discussion report on [AT131bis][501][XR] Discuss remaining RLC issues (vivo)	vivo

(RLC-N01) Try to capture it in the procedure “If t-RxDiscard is configured and expires, the receiving side of an AM RLC entity consider the corresponding SDUs as received.”, if problem is identified or companies have strong concern during CR phase, it will be postponed.

(RLC-N01) Add “or t-RxDiscard expires” at the end of definition of RX_Next.

(RLC-N02) Companies will bring contribution in next meeting to explain the use cases and the TP.

(RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Companies are invited to bring contributions on the solutions to address it.

RLC-V02 is closed without any spec impact.

RLC-H01 is updated according to the CR in R2-2507016.

Agreements for RLC - (RLC-X01) We keep the spec as it is. - (RLC-V01) No need to indicate the SN of obsolete RLC SDUs to the receiving PDCP entity after t-RxDiscard expires. - (RLC-N01) We will clarify RX_Next state variable update when t-RxDiscard expires. FFS an exact wording and where to do the change (e.g. procedure or variable definition) - (RLC-N01) Try to capture it in the procedure “If t-RxDiscard is configured and expires, the receiving side of an AM RLC entity consider the corresponding SDUs as received.”, if problem is identified or companies have strong concern during CR phase, it will be postponed. - (RLC-N01) Add “or t-RxDiscard expires” at the end of definition of RX_Next. - (RLC-N02) In NOTE in 5.3.3.3 we add “If stopReTxDiscardedSDU is configured” at the beginning of the sentence. - (RLC-N02) FFS whether any other changes are needed, e.g. as in R2-2507159 - (RLC-E01) No change is needed. - (RLC-S01) No change needed as per the previous agreement - (RLC-N02) Companies will bring contribution in next meeting to explain the use cases and the TP. - (RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Companies are invited to bring contributions on the solutions to address it. - RLC-V02 is closed without any spec impact. - RLC-H01 is updated according to the CR in R2-2507016.

PDCP-H001

R2-2507058	Discussion on non-delay-reporting PDCP SDU definition	Huawei, HiSilicon	discussion	NR_XR_Ph3-Core

Proposal 1:	To align with the procedure description as in data volume calculation, the definition of non-delay-reporting PDCP SDU should be changed as “a non-delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is a PDCP SDU that will be transmitted prior to any of the delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold but after all delay-reporting PDCP SDUs associated with the i-1:th DSR-ReportingThreshold, and that is not a delay-reporting PDCP SDU associated with any of the k:th DSR-ReportingThreshold where k<=i”

R2-2507279	Remaining open issues for DSR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2. [PDCP-H001] Keep the current definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

DISCUSSION:

- Fujitsu supports Huawei’s proposal. In addition, procedural text could be simplified. There are several issues with the definition, e.g. it is not aligned with how delay-critical data was defined.

- OPPO supports LGE proposal, the procedure is clear. Huawei’s proposal is not entirely accurate.

- Apple thinks this has been discussed already, it ‘s not worth to reopen the discussion.

- Xiaomi, Nokia, Samsung thinks there is no issue.

[PDCP-H001] Keep the current definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

PDCP-N001

R2-2507159	UP Open Issues	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: [PDCP-N001] The agreement on RLC-12 from the RAN2#131 meeting is captured in the clause 5.16.1 of the PDCP specification as follows:

NOTE: 	It is up to UE implementation whether to re-send a PDCP SN gap report when upper layer requests a PDCP entity re-establishment (e.g., based on a previously submitted PDCP SN gap report not having been successfully delivered before PDCP entity re-establishment).

R2-2507315	Discussion on open issues for RLC and PDCP	Samsung	discussion	Rel-19

[PDCP N001] Proposal 7: RAN2 to not introduce a NOTE specifying re-sending the PDCP SN gap report after HO can be left to UE implementation.

DISCUSSION:

- Sharp prefers to capture the note as currently it is not clear in specs that UE can do this.

- CATT thinks we can capture sth in chair notes.

- OPPO thinks no need to capture in specs. We do not have to capture UE implementation for this.

- Lenovo is fine with the note, the current specs may suggest this is not allowed.

- Nokia thinks the UE should not re-send whenever it wants.

- Samsung has strong concerns on this note.

- LGE thinks this behaviour should be limited to AM DRB as UM DRB state variables are reset. LGE wonders whether this is useful for AM DRB.

(PDCP-N001) Capture a NOTE in the clause 5.16.1 of the PDCP specification that the UE can (re)send the PDCP SN gap report when upper layer requests a PDCP entity re-establishment. FFS exact wording and whether this should be limited to AM DRB (to be handled during CR update)

Agreements for PDCP - (S001) Include “i:th” in the definition of “Non-delay-reporting PDCP SDU”. - [PDCP-H001] Keep the current definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU. - (PDCP-N001) Capture a NOTE in the clause 5.16.1 of the PDCP specification that the UE can (re)send the PDCP SN gap report when upper layer requests a PDCP entity re-establishment. FFS exact wording and whether this should be limited to AM DRB (to be handled during CR update)

MAC – D.2

R2-2507812	Summary of [AT131bis][502][XR] Remaining MAC issues (Qualcomm)

(MAC-D.2) Adopt the following TP for clause 5.4.1 in TS 38.321:

MAC – other

R2-2507305	XR user plane corrections	NEC	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: Like Bj, we propose moving specification on applied priority determination out of the LCP procedure to allow greater flexibility for UE implementation.

Proposal2: for clearness and simplicity, define two MAC CE names “UL rate control recommendation” and “UL rate control Query” but share the same MAC CE format and its description.

DISCUSSION on P1:

- LGE thinks this is not essential, the UE implementation is not restricted.

- Xiaomi agrees this is up to UE implementation.

DISCUSSION on P2:

- Samsung thinks this is reasonable. Lenovo agrees, this is now confusing to have a single name for two different MAC CEs.

- QCM clarifies they followed legacy style. In the procedure it is still clear as they are differentiated, e.g. by using UL or DL.

- Ericsson think it makes sense, but at this stage they prefer to keep it as it is.

P1 and P2 from R2-2507305 are not pursued

R2-2507632	Outstanding LCP issues and related TPs	Samsung	discussion

Proposal 2. RAN2 is kindly asked to agree the following TP:

Proposal 3. RAN2 is kindly asked to agree the following TP:

DISCUSSION on P2:

- Nokia thinks it is not needed, we already refer to LCP

- OPPO thinks anyway this is according to LCP, so this is OK. Do not want to impact legacy text.

- LGE agrees with Nokia and OPPO. No need to describe additional details here.

DISCUSSION on P3:

- QCM thinks it is clear enough already. Enhanced LCP is too generic.

P2 and P3 from R2-2507632 are not pursued

Agreements for MAC - Issue A.2, B.2, C.4, C.5, C3. and D.1 are agreed (already part of rapporteur CR) - (MAC-D.2) Adopt the following TP for clause 5.4.1 in TS 38.321: - P1 and P2 from R2-2507305 are not pursued - P2 and P3 from R2-2507632 are not pursued

R2-2506926	Discussion on avoiding unnecessary retransmissions	Lenovo	discussion	Rel-19

R2-2506964	On the definition of non-delay-reporting PDCP SDU	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

R2-2507019	Discussion on MAC open issues on rate control for R19 XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2507057	Discussion on remaining issues for MAC for R19 XR	Huawei, HiSilicon	discussion	NR_XR_Ph3-Core

R2-2507112	Open Issues of RLC CR for Rel-19 XR	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2507192	Discussion on XR User Plane Open Issues	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

R2-2507299	XR RLC Issues	ZTE Corporation, Sanechips	discussion

R2-2507301	XR Scheduling enhancement open issues	ZTE Corporation, Sanechips	discussion

R2-2507309	Remaining MAC open issues	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2507311	Remaining RLC open issue on timely re-transmissions	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2507342	Discussion on PDCP open issues	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2507343	Discussion on RLC open issues	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2507472	H001, N001	Ericsson	discussion	Rel-19

R2-2507516	Discussion on open issues of XR RLC AM enhancements	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

Withdrawn

R2-2506931	Discussion on remaining issues for RLC	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core	Withdrawn

### 8.7.4	Other corrections

Including corrections to stage-2, UE capabilities etc.

R2-2506842	Discussion on UE Capabilities for XR	CATT	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: Use reporting threshold(s) instead of multiple reporting thresholds to describe UE capability delayStatusReportNonDelayReportingData and multipleEntryDelayStatusReport.

DISCUSSION:

- Xiaomi thinks current text is OK, but we could also change to say “using multiple entry DSR MAC CE”.

- Vivo thinks procedure was already updated according to the same way as in CATT’s TP, so they support it.

- LGE agrees the change is needed, prefer CATT’s text.

- Nokia supports Xiaomi’s proposal.

- OPPO prefers CATT’s TP.

Use “reporting threshold(s)” instead of “multiple reporting thresholds” to describe UE capability delayStatusReportNonDelayReportingData and multipleEntryDelayStatusReport.

## 8.8	NTN for NR Ph3

(NR_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-251954 )

LTE_TN_NR_NTN_mob, leading WG: RAN2, Rel-19 WID: RP-251974 )

Time budget: 0 TU

Tdoc Limitation: 3 tdocs

### 8.8.1	Organizational

LS, Rapporteur input, including open issues lists, etc.

Rapporteur inputs do not count towards the tdoc limitation.

Including the lists of open issues, if any, raised in the following email discussions:

[Post131][301][R19 NR NTN] Stage2 CR (Thales)

[Post131][303][R19 NR NTN] 38.304 CR (ZTE)

[Post131][304][R19 NR NTN] capability CR (Apple)

[Post131][315][R19 NR NTN] MAC CR (Interdigital)

[Post131][316][LTE NR NTN mob] Stage2 CR (Samsung)

[Post131][318][LTE NR NTN mob] capability CR (vivo)

- Stage 2 open issues

R2-2507648	Report of [Post131][301][R19 NR NTN] Open issues for Stage2 (Thales)	THALES	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	Endorse R2-2507649 to capture in the stage 2 specification the following:

- The PDCCH repetition and SIB1 repetition are also supported for TN in section 19

- The deprioritization of the frequency of a service the UE is interested in when the UE is outside ISA

- The related enhancements for measurements in Downlink Coverage enhancements

-	ZTE thinks the second change should be moved to the NTN specific section

Changes are endorsed but can continue the discussion on where to put the second one exactly

Observation 1	On the applicability of OCC to RACH-less handover, two companies support this (including the proponent), two companies think it is supported by default and no specs change is needed, and two companies think it is a non-essential feature and that further RAN1 and RAN4 inputs are needed

Proposal 2a	RAN2 to confirm if OCC is applicable for RACH-less handovers by suitable network implementation.

Proposal 2b	RAN2 to discuss if an LS to RAN1 and RAN4 is needed for confirmation (to support or not)

Proposal 2c	RAN2 to discuss if a clarification in the stage 2 specification is needed if Proposal 2a is agreed. It can be revised according to the LS replies from RAN1 and RAN4 if an LS to RAN1 and RAN4 is sent

-	Ericsson thinks this is not covered by the WID and it would not work

-	QC and Samsung thinks this would work, at least in FR1

-	Thales is not sure about the complexity/gain trade-off

-	QC thinks this is about msg5, not msg3

We send an LS to RAN1/RAN4 indicating that Stage3 specs in RAN2 do not prevent the use of OCC for RACH-less handover and asking whether this could be supported without any further updates to their specs

Draft an LS in R2-2507764 (Thales)

R2-2507649	Miscellaneous Stage 2 corrections for NR NTN phase 3	THALES (Rapporteur)	CR	Rel-19	38.300	19.0.0	1047	-	F	NR_NTN_Ph3-Core

-	vivo thinks that RAN1 did not agreed that repetition of PDSCH carrying Msg4 is supported for TN

Endorsed as baseline, but can continue the discussion on where to put the second changes exactly and on the change repetition of PDSCH carrying Msg4

Revised in R2-2507779

R2-2507779	Miscellaneous Stage 2 corrections for NR NTN phase 3	THALES (Rapporteur)	CR	Rel-19	38.300	19.0.0	1047	1	F	NR_NTN_Ph3-Core

Agreed in principle

[AT131bis][306][NR NTN] LS on OCC for RACH-less HO (Thales)

Scope: Draft an LS to RAN1/RAN4 on OCC for RACH-less HO based on meeting agreements

Intended outcome: Draft LS (in R2-2507767)

F2F time: FFS

Deadline for Draft LS: Thursday 2025-10-18 11:00

R2-2507767	[Draft] LS on OCC for RACH-less HO	Thales	LS out	Rel-19	NR_NTN_Ph3-Core	To:RAN1/RAN4

Change “de” to “do” and add “update”

Revised in R2-2507780

R2-2507780	LS on OCC for RACH-less HO	Thales	LS out	Rel-19	NR_NTN_Ph3-Core	To:RAN1, RAN4

Revised in R2-2507909

R2-2507909	LS on OCC for RACH-less HO	Thales	LS out	Rel-19	NR_NTN_Ph3-Core	To:RAN1, RAN4

Approved

- Idle mode open issues

R2-2507521	Open issues on NR NTN 38.304	ZTE Corporation,  Sanechips	report	Rel-19	NR_NTN_Ph3-Core

Open issue 1: The TAI/Cell list received in target service area in USD could be used by UE for service continuity purpose.

Proposal 1: Update NOTE0j as : ‘NOTE 0j:	The ISA(s) can be ISA(s) provided in SIBxx and/or target service area  in USD. It is up to UE’s implementation to decide whether it is inside ISA(s) or not.’

Agreed

Open issue 2: With the support of common PDCCH repetition, the UE, the UE will monitor the pagingSearchSpace and its linked CSS for paging reception. In the TS 38.304 spec, the PO concept should be applied to SearchSpace linked with pagingSearchSpace.

Proposal 2: When common PDCCH repetition is configured for paging, down select between below two options for PO determination:

- Opt1: The search space linked with pagingSearchSpace is also considered as pagingSearchSpace. UE will apply behavior as specified in TS 38.304 section 7.1 to determine the PDCCH monitoring occasion, no specs change is needed for this purpose.

- Opt2:  Update TS 38.304 sub-clause 7.1 as per below

When SearchSpaceId other than 0 is configured for pagingSearchSpace or when SearchSpaceId other than 0 is configured for the SearchSpace linked with pagingSearchSpace, if configured, the UE monitors the (i_s + 1)th PO…

RAN2 understands there is no impact to RAN2 Stage3 specs

R2-2507522	Correction on broadcast service in NTN	ZTE Corporation, Sanechips	CR	Rel-19	38.304	19.0.0	0448	-	F	NR_NTN_Ph3-Core

Agreed in principle

- UE capability open issues

R2-2507122	Report of [Post131][304][R19 NR NTN] Open issues for capability (Apple)	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: No open issue is identified for Rel-19 NR NTN UE capability.

Noted

- ASN.1 review for NR NTN

R2-2507693	ASN.1 comment file and RIL assessment for NR NTN Rel-19	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

PropAgree: C002, C007, E012, E013, E014, N086, S025, S026, S027, O710, V202, Z253, and Z254.

C002, C007, E012, E013, E014, N086, S025, S026, S027, O710, V202, Z253, and Z254 are Agreed

PropReject: C006??, C009??, H251??, H252??, H253??, H254, N081, N082, N083, N084, V200??, V201??, V203, V206??, Z255?? and Z257.

H254, N081, N082, N083, N084, V203, and Z257 are Rejected

ToDo: A200, C003, C008, E010, E011, S024, H250, N085, Q200, V208, X250, Z256, and Z258.

A200, C003, C008, E010, E011, S024, H250, N085, Q200, V208, X250, Z256, Z258, C006, C009, H251, H252, H253, V200, V201, V206, Z255 are ToDo

Duplicate: H252, V204, V205, V207, V209, Z251, Z252, X251 and C005.

R2-2507694	ASN.1 review file for NR NTN	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2507692	Initial corrections to NR NTN Phase 3	Ericsson	CR	Rel-19	38.331	19.0.0	5562	-	F	NR_NTN_Ph3-Core

Endorsed

Revised in R2-2507785 to reflect the meeting agreements

R2-2507785	Initial corrections to NR NTN Phase 3	Ericsson	CR	Rel-19	38.331	19.0.0	5562	1	F	NR_NTN_Ph3-Core

Discussed in [Post131bis][303]

[Post131bis][303][NR NTN] RRC CR (Ericsson)

Scope: Update the RRC CR with meeting agreements

Intended outcome: Endorsed CR in R2-2507785

Deadline:  short

=> Endorsed in R2-2507946 (38.331)

R2-2507946	Initial corrections to NR NTN Phase 3	Ericsson	CR	Rel-19	38.331	19.0.0	5562	2	F	NR_NTN_Ph3-Core

=> Endorsed

Agreements:

For Stage2:

1.	capture the following in the stage 2:

- The PDCCH repetition and SIB1 repetition are also supported for TN in section 19

- The deprioritization of the frequency of a service the UE is interested in when the UE is outside ISA

- The related enhancements for measurements in Downlink Coverage enhancements

For idle mode:

1.	Update NOTE0j as: ‘NOTE 0j:	The ISA(s) can be ISA(s) provided in SIBxx and/or target service area in USD. It is up to UE’s implementation to decide whether it is inside ISA(s) or not.’

R2-2507781	LS on Location-based SMTC enhancement 	Qualcomm	LSout	Rel-19	NR_NTN_Ph3-Core	To:RAN4

Remove Draft

Revised in R2-2507789

R2-2507789	LS on Location-based SMTC enhancement 	Qualcomm	LSout	Rel-19	NR_NTN_Ph3-Core	To:RAN4

Approved

- ASN.1 review for LTE TN to NR NTN mobility

R2-2506870	RIL status on LTE TN to NR NTN mobility	CATT	discussion	Rel-19	LTE_TN_NR_NTN_mob

RILs marked as ProAgree: V230??

Note: The wording may be slightly different from what is proposed. Please check the Rapporteur’s comments.

RILs marked as ToDo: S905, X500, O711, E801

V230, S905, X500, O711, E801 are ToDo

R2-2506869	Corrections on LTE TN to NR NTN IDLE mode mobility in TS 38.331	CATT	CR	Rel-19	36.331	18.6.0	5157	-	F	LTE_TN_NR_NTN_mob

Revised in R2-2507786 to reflect the meeting agreements

R2-2507786	Corrections on LTE TN to NR NTN IDLE mode mobility in TS 36.331	CATT	CR	Rel-19	36.331	18.6.0	5157	1	F	LTE_TN_NR_NTN_mob

Discussed in [Post131bis][304]

[Post131bis][304][LTE TN NR NTN] RRC CR (CATT)

Scope: Update the RRC CR with meeting agreements

Intended outcome: endorsed CR in R2-2507786

Deadline:  short

=> Endorsed in R2-2507786

### 8.8.2	RRC corrections

Corrections to TS 38.331.

- Reference location report

[X250/V205] (provision of a distance threshold)

Continue in offline 303

R2-2506807	Discussion on RIL X250 X251 V205 for DL coverage enhancement	Xiaomi, ZTE, CSCN, Samsung	discussion	Rel-19

Proposal 1: [RIL X250] RAN2 agrees to provide a distance threshold together with the reference location for location based SMTC selection in idle/inactive mode and UE assisted SMTC configuration in connected mode.

Proposal 2: [RIL V205] For UE assisted SMTC configuration in connected mode, UE only reports the closest reference locations that satisfy the distance threshold up to the configured maximum report number.

R2-2506834	Discussion on RILs [V200][S024][S025][H250][V208][X250] regarding SMTC Enhancement	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 8: (RIL X250) Introduce a distance threshold for each reference location in the reference location list.

Proposal 9: (RIL X250) UE only sets the reference location bit to 1 when the corresponding reference location is the closest reference location and the UE is within the associated threshold range.

[V208/X251] (bitmap vs indices)

Proposal 3: [RIL X251] Reference location report is in the format of a list of reference location index, instead of a bitmap.

R2-2506834	Discussion on RILs [V200][S024][S025][H250][V208][X250] regarding SMTC Enhancement	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 7: (RIL V208) Confirm that bitmap is used to indicate the N closest reference location.

-	Ericsson, HW, Nokia and CATT support this

Confirm that bitmap is used to indicate the N closest reference location.

R2-2506807	Discussion on RIL X250 X251 V205 for DL coverage enhancement	Xiaomi, ZTE, CSCN, Samsung	discussion	Rel-19

Proposal 3: [RIL X251] RAN2 to discuss whether Reference location report is in the format of a list of reference location index, instead of a bitmap.

[S024/S025/C005/Z502] (RRCReonfigurationComplete vs UAI)

R2-2507496	RIL S024 S025 C006	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1a: Reporting close reference locations in RRCReconfigurationComplete message is not mandatory to UE. UE only includes the report in RRCReconfigurationComplete when it is available.

-	Ericsson thinks the UE knows its location in connected mode and then it could include the report in RRCReconfigurationComplete without delay

-	Nokia, ZTE, vivo supports this proposal

Reporting close reference locations in RRCReconfigurationComplete message is not mandatory to UE. UE only includes the report in RRCReconfigurationComplete when it is available

R2-2506867	[C005][S024][S025]Discussion on the mechanism of UE reporting the N closest reference location	CATT	discussion	Rel-19	NR_NTN_Ph3-Core	Late

Proposal 1: Down selection from the following options for UE reporting the N closest reference locations.

-	Option 1 [C005]: restrict the UE has to report the reference locations in the RRCReconfigurationComplete message for the first time.

-	Option 2 [S024][S025][Z252]: make the UE reports the reference locations in the RRCReconfigurationComplete message as an optional behaviour, and if the UE does not report the reference locations in the RRCReconfigurationComplete message for the first time, report it in the UEAssistanceInformation message.

[V204/H250] (applicability to inter-frequency)

Continue in offline 303

R2-2506834	Discussion on RILs [V200][S024][S025][H250][V208][X250] regarding SMTC Enhancement	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 6: (RIL H250) NW configures the location information reporting for assisted SMTC configuration for each frequency, and UE separately reports the N closest reference location for each frequency.

R2-2507691	[RIL H250] Applicability of SMTC enhancements to inter-frequency	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	RAN2 first decides which inter-frequency scenario(s) is(are) considered before discussing whether the Rel-19 SMTC enhancements are applicable to the inter-frequency case, and how.

Proposal 2	For ASN.1 review, down-select from the following inter-frequency scenarios assuming uniform cell sizes across different frequencies:

1)	Inter-frequency neighbor cells are non-overlapping but adjacent to the serving cell,

2)	Inter-frequency neighbor cells overlap the serving cell and intra-frequency neighbor cells without spatial offset (cell centers are identical),

3)	Inter-frequency neighbor cells overlap the serving cell and intra-frequency neighbor cells with spatial offset (cell centers are different).

Proposal 3	It is RAN2’s intention to enable the UE, in RRC_IDLE/INACTIVE state, to select SMTCs across frequencies based on location information to prevent the UE from having to perform unnecessary inter-frequency measurements.

Proposal 4	Extending location-based/UE-assisted SMTC selection to the inter-frequency scenario where intra- and inter-frequency cells perfectly overlap (also referred to as Scenario 2) is down-prioritized in Rel-19.

Proposal 5	Reference locations from a single set can be associated with multiple frequencies (or SMTCs for multiple frequencies).

Proposal 6	For Scenario 3, inter-frequency reference locations can be derived from intra-frequency reference locations and an specific offset.

Proposal 7	Support additional SMTC periodicity to the inter-frequency case.

R2-2507440	Remaining issues on beam hopping with multiple SMTC offsets [Q200]	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 3	RAN2 first needs to decide whether more than 4 SMTCs configuration is applicable for inter-frequency measurements.

Proposal 4	If SMTC enhancement is applicable for inter-frequency measurements, it is up to UE how to select SMTC based on location and use multiple periodicities for IDLE mode inter-frequency measurements.

Proposal 5	If SMTC enhancement is applicable for inter-frequency measurements, a new UE capability to support two periodicities for connected mode inter-frequency measurements is introduced and an LS is sent to RAN4 for any issue/concern.

Proposal 6	Total number of satellites across intra and inter-frequency measurements at a given time instance does not exceed 4.

[H250/H252] (terminology, report content in RRCReconfigurationComplete)

R2-2506935	[H250][H251][H252][H253] Discussion on remaining RRC issues	Huawei, HiSilicon	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: [H252] RAN2 to down select from the following:

Option1: Clarify that the referenceLocationReport in RRCConfigurationComplete only reports the single closest reference location, it cannot report multiple reference locations.

Option2: Add closestLocsToReport to RRCConfiguration (outside of OtherConfig)

Option3: Remove referenceLocationReport from RRCConfigurationComplete

-	Ericsson thinks we could add the “N closest…” in the procedural text

For [H252] we add the “N closest…” in the procedural text

Proposal 2: [H250] Use “closest reference location information” to replace “location information” in the UAI reporting procedures, and clarify that this this information is for “assisting SMTC and measurement gap configuration”.

Use “closest reference location information” to replace “location information” in the UAI reporting procedures, and clarify that this this information is for “assisted SMTC and measurement gap configuration” (can come back to this after the discussion on inter-frequency support)

[Q200] (Location-based SMTC selection for Earth moving cell scenario)

R2-2507440	Remaining issues on beam hopping with multiple SMTC offsets [Q200]	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	Location-based SMTC enhancement is also supported in moving cell scenario. The existing movingReferenceLocation is reused for this purpose (i.e., network can update the movingReferenceLocation with epoch time without notifying UE).

-	Thales thinks we should support this only if does not imply extra work

-	Ericsson wonders if there is a requirement to support this

-	Xiaomi thinks for idle/inactive there should be no problem

-	QC thinks it could be fine not to support this but we need more clarification that moving cell do not broadcast smtc5

-	Ericsson thinks smtc5 is also for multiple periodicity

Location-based SMTC enhancement is not supported in moving cell scenario (to be captured in Stage2)

We specify that smtc5 with multiple offsets is not broadcast in a Earth-moving cell scenario (to be captured in the smtc5 field description in system information)

Send a LS to RAN4 in R2-2507781 (QC) to inform them of this decision

Proposal 2	Location-based SMTC is sufficient to solve issue of selecting four SMTCs both in connected mode and idle mode measurements and there is no need to introduce second solution to solve the same issue in Rel-19.

[A200] (reference location can be associated with smtc)

R2-2507123	Open issues on NR NTN (A200/V204)	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: The Rel-15 smtc in SIB2 should be allowed to associate with a reference location in RefLocList-r19.

-	Samsung thinks this is not needed as the UE needs to apply legacy SMTC. Vivo agrees

-	HW tends to agree there could be a problem

[A200] Postponed to the next meeting

- SMTC5 list

[C006/C008/C009] (co-existence of SMTC5 SMTC4 list)

R2-2506866	[C006][C008][C009]Corrections on the smtc5list	CATT	discussion	Rel-19	NR_NTN_Ph3-Core	Late

Proposal 1: Make the following corrections to the field description of smtc4list, smtc5list under SIB2:

[C006] Remove the limitation of “The total number of configurable SMTCs across smtc4list and smtc5list is 6.”;

[C008] Modify “The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2.” as “The total number of different SMTC periodicities across smtc and smtc5list is 2.”;

[C008] Illustrate that “If smtc5list is present, the two SMTC periodicities capable UE performs measurement only according to the SMTCs in the smtc5list”.

-	Ericsson thinks we could clarify that the maximum number of offsets should be 6

-	QC thinks the same applies to periodicities. Ericsson thinks this already captured (as part of C008)

-	HW thinks the current spec is fine

- -	Ericsson suggests to clarify that the maximum number of offsets should be 6. Nokia/CATT agree. HW disagrees as this would hint that it’s possible to specify more than 6 SMTCs

We change the limitation as “The total number of configurable SMTCs offsets across smtc4list and smtc5list is 6.”

Proposal 2: [C009] For the smtc5list in MeasObjectNR, if the periodicity parameter is present for an SSB-MTC5 configuration, the UE uses this configured parameter; if the periodicity parameter is absent for an SSB-MTC5 configuration, the UE uses the periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration. Reflect that in section 5.5.2.10.

-	Ericsson thinks in connected mode the NW knows the UE capability and would configure smtc accordingly

-	Ericsson thinks we could clarify that SMTC5 in MO is used to configure one additional periodicity

-	QC thinks we could only rely on stmc5 in this case and not rely on smtc4

-	vivo thinks we could have a unified solution with idle

RAN2 understands that if the NW wants to reuse the periodicity in smtc1, the NW needs to configure smtc4 (we capture something accordingly in the field description)

R2-2507496	RIL S024 S025 C006	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 2: Agree C006, and clarify in FD that UE shall setup SMTC according to smtc5list if both smtc4list and smtc5list are configured in SIB.

[H251] (offset in smtc when smtc5list is present)

R2-2506935	[H250][H251][H252][H253] Discussion on remaining RRC issues	Huawei, HiSilicon	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 3: [H251] Modify the field description on smtc in SIB2, so that UE does not need to consider neighbour cell propagation delay when smtc5list is configured.

[H251] Postponed to the next meeting

- Broadcast service related RILs

[V206] (mbs-SessionAreaList-r19 Need code)

R2-2506833	Discussion on RIL V206 	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: (RIL V206) Change the need code for mbs-SessionAreaList-r19 from Need R to Need S.

Change the need code for mbs-SessionAreaList-r19 from Need R to Need S.

Proposal 2: (RIL V206) Add in the FD of mbs-SessionAreaList-r19 that UE considers the associated service can be received within the entire cell area if the list is absent.

-	Ericsson suggests to capture something in Stage2

-	HW thinks this should go in Stage3. Xiaomi/Apple/Toyota agree.

Add in the FD of mbs-SessionAreaList-r19 that UE considers the associated service can be received within the entire cell area if the list is absent

R2-2507625	Remaining RRC issues for NR NTN	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 3: (V206) The Need code for mbs-SessionAreaList-r19 is fine in “Need R”.

[C003] (ISA vs TargetServiceArea)

Continue in offline 303

R2-2506868	[C003] Clarification of how the UE uses the service area information in SIB and USD	CATT	discussion	Rel-19	NR_NTN_Ph3-Core	Late

Proposal 1: Add a NOTE “NOTE: If the service area information is broadcast in the SIB27, the UE ignores the Target Service Area in the USD.” in section 5.2.2.4.29 Actions upon reception of SIB27. Adopt the TP in Annex.

[C003] Postponed to the next meeting

R2-2507380	Remaining issues for MBS broadcast over NTN	Huawei, HiSilicon	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 2: To determine the intended service area of an MBS broadcast service of interest:

- when SIB27 is not broadcast, the UE can use the target area info in the USD;

- when SIB27 is broadcast, the UE shall use SIB27 and the MBSBroadcastConfiguration message..

R2-2507523	RRC corrections related to RILs	ZTE Corporation,  Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core	Late

Proposal 1: For MRB establishment/MRB release/MII initiation, when both ISA in SIB and target service area included in USD are provided, UE prioritize usage of ISA in SIB.

R2-2507625	Remaining RRC issues for NR NTN	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 2: (C003) As the ISA is not aligned with the target service area in USD, the RRC procedures should be performed according to the SIBXX ISA.

[V200] (SIB2 validity)

Continue in offline 303

R2-2506834	Discussion on RILs [V200][S024][S025][H250][V208][X250] regarding SMTC Enhancement	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: (RIL V200) Clarify that UE in RRC CONNECTED is required to acquire and maintain the valid SIB2 in RRC CONNECTED once the assisted SMTC configuration without a list of reference location is configured.

-	QC could accept to move the info on reference location to SIB19

-	Xiaomi thinks this is not needed. Ericsson also thinks this is not needed

-	vivo still thinks there is some ambiguity.

-	Apple thinks the NW would have to use dedicated signalling for this. Huawei agrees as it thinks that the spec does not mandate the UE to maintain SIB2 in connected mode

-	QC thinks SIB2 is not an essential SIB

-	Xiaomi thinks that it’s not likely that the reference location will change in the Earth-fixed case

[V200] is Rejected

Proposal 2: (RIL V200) Clarify that UE in RRC CONNECTED can stop acquiring and maintaining the valid SIB2 in RRC CONNECTED if the assisted SMTC configuration is released or reconfigured with a list of reference location.

[H253] (warningAreaCoordinatesSegment-r19 Need code )

Continue in offline 303

R2-2506935	[H250][H251][H252][H253] Discussion on remaining RRC issues	Huawei, HiSilicon	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 4: [H253] Change the need code for warningAreaCoordinatesSegment-r19 in SIB7 from “Need R” to “Need S”, and clarify in the field description that if the field is missing, warningAreaCoordinatesSegment for other segments of the same warning message applies.

[O710] (warningMessageSegmentNumber and warningMessageSegmentType clarification)

R2-2507044	Discussion on [RIL]O710 geo-fencing for ETWS	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

RAN2 to discuss and confirm which understanding is correct on the segment number and segment type parameters for warning area coordinates segment and adopt the corresponding TP:

Option 1-1: The current warningMessageSegmentNumber and warningMessageSegmentType is reused for warningAreaCoordinatesSegment. And NW configure the same number of segments of warning message and warning area coordinate. Adopt the TP1 in Annex.

Option1-2: The current warningMessageSegmentNumber and warningMessageSegmentType is reused for warningAreaCoordinatesSegment. And NW can configure the different number of segments of warning message and warning area coordinate. Adopt the TP2 in Annex.

-	Oppo thinks the rapporteur CR is already aligned to this and we just need to add warningMessageSegmentType

-	Oppo thinks the same correction for IoT NTN

Fix the description for warningMessageSegmentType in SIB7 (same applies to IoT NTN)

Option2: Define the separate segment number and segment type parameters for warningAreaCoordinatesSegment and adopt the TP3 in Annex

[Z258/Z256] (miscellaneous)

R2-2507523	RRC corrections related to RILs	ZTE Corporation,  Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core	Late

Proposal 4: RAN2 agrees on Z256, and confirms there is no need to update 304 specs when common PDCCH repetition is configured in 304 specs.

[Z256] is rejected

Proposal 5: RAN2 confirms sidelink UE can support MBS feature in NTN, and agrees on Z258.

[Z258] is rejected (issue to be discussed in cross-WI discussion)

R2-2507625	Remaining RRC issues for NR NTN	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: (Z256) pagingSearchSpaceExt in PDCCH-ConfigCommon is not needed to introduce.

(RRC rapporteur view on RILs)

R2-2507690	Discussion on various RILs for NR NTN	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	Reject RIL S024. The UE always provides the initial referenceLocationReport in the RRCReconfigurationComplete message after being configured to report the N closest reference locations in the RRCReconfiguration message.

Proposal 2	For RIL H250, reject the addition of “closest reference” to the UAI procedure description since the UE reports a pointer (via index or bitmap).

Proposal 3	For RIL H250, accept the addition “and measurement gap” if RAN2 agrees location-based SMTC selection applies to inter-frequency.

Proposal 4	Accept RIL V208. The IE referenceLocationReport is a 6-bit bitmap.

Proposal 5	For RIL X250, the network can optionally configure one common distance threshold for all reference locations.

Proposal 6	For RIL N085, introduce the following clarification: “If a UE does not support an additional SMTC periodicity, it ignores the periodicity provided in smtc5list, if any, and applies the periodicity provided in smtc.”

Proposal 7	For RIL A200, clarify that legacy SMTC (smtc in SIB2) applies to the serving cell and is not used for location-based SMTC selection.

Proposal 8	Reject Q200 Earth-moving cell scenario is down-prioritized for location-based SMTC selection in Rel-19.

Proposal 9	For RIL C008, agree to the first change (the removal of “smtc4list”), but reject the second change (the addition of “If smtc5list is present, the two SMTC periodicities capable UE ignores smtc4list.”).

Proposal 10	Accept E010. Take the TP in Annex as a baseline.

Proposal 11	Reject C003. How to use ISA (or TSA) is up to UE implementation, if both were to be simultaneously configured.

Proposal 12	Agree E011.

Proposal 13	Reject Z256.

Agreements:

1.	[S024]: Reporting close reference locations in RRCReconfigurationComplete message is not mandatory to UE. UE only includes the report in RRCReconfigurationComplete when it is available

2.	[H252]: We add the “N closest…” in the procedural text

3.	[H250]: Use “closest reference location information” to replace “location information” in the UAI reporting procedures, and clarify that this this information is for “assisted SMTC and measurement gap configuration” (can come back to this after the discussion on inter-frequency support)

4.	[V208/X251] Confirm that bitmap is used to indicate the N closest reference location.

5.	[Q200]: Location-based SMTC enhancement is not supported in moving cell scenario (to be captured in Stage2). We specify that smtc5 with multiple offsets is not broadcast in a Earth-moving cell scenario (to be captured in the smtc5 field description in system information)

6.	[C006]: We change the limitation as “The total number of configurable SMTCs offsets across smtc4list and smtc5list is 6.”

7.	[C009]: RAN2 understands that if the NW wants to reuse the periodicity in smtc1, the NW needs to configure smtc4 (we capture something accordingly in the field description)

8.	[V206] Change the need code for mbs-SessionAreaList-r19 from Need R to Need S. 	Add in the FD of mbs-SessionAreaList-r19 that UE considers the associated service can be received within the entire cell area if the list is absent

9.	[O710]: Fix the description for warningMessageSegmentType in SIB7 (same applies to IoT NTN)

10. 	[Z256] is Rejected

11. 	[Z258] is Rejected (issue to be discussed in cross-WI discussion)

12.	[V201] is Rejected

[AT131bis][303][NR NTN] RRC open issues (Ericsson)

Scope: Continue the discussion on applicability to inter-frequency [V204/H250] and on provision of a distance threshold [X250/V205]

Intended outcome: summary of the offline discussion (in R2-2507763)

F2F time: FFS

Deadline for offline discussion summary: Wednesday 2025-10-15 17:00

Scope: Discuss proposals for RILs C003, V200, H253

Intended outcome: summary of the offline discussion (in R2-2507782)

Deadline for offline discussion summary: Friday 2025-10-17 09:00

R2-2507763	Report of [AT131bis][303][NR NTN] RRC open issues	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1	Without clarity on the considered inter-frequency scenario(s), it is impossible to conclude on either the applicability of the Rel-19 SMTC enhancements to inter-frequency or the introduction of a distance threshold.

Observation 2	There are at least four different inter-frequency scenarios for NTN deployments that can be considered.

Observation 3	There are two main types of inter-frequency scenarios, adjacent coverage and overlapping/overlaid coverage.

Observation 4	Location-based SMTC selection based on a limited set of reference locations does not work for scenarios where the inter-frequency neighbor cells are much smaller than the serving cell.

Observation 5	In the LS exchange with RAN4, RAN2 has so far not considered the inter-frequency scenario.

Observation 6	For inter-frequency Scenarios 1 and 3, location-based SMTC selection in RRC_IDLE/INACTIVE state is not necessary and UE-assisted SMTC selection in RRC_CONNECTED state is not necessary, except for UEs that do not support parallelSMTC-r17.

Observation 7	The existing solution for UE-assisted SMTC selection in RRC_CONNECTED state is independent of scenarios/frequencies and can already be used for the inter-frequency case.

Observation 8	For inter-frequency Scenarios 1 and 3, location-based/UE-assisted SMTC selection for inter-frequency may be beneficial from the UE energy consumption perspective (across RRC states).

Observation 9	For inter-frequency Scenario 2, location-based/UE-assisted SMTC selection is not necessary.

Observation 10	For Scenario 1 and 3 (and in principle Scenario 4), the additional SMTC periodicity is useful for the inter-frequency case, whereas for Scenario 2, it is not necessary.

Proposal 1	We extend the Rel-19 SMTC enhancements to inter-frequency, using as a baseline the Rel-19 agreed solutions for intra-frequency (for both location-based SMTC selection in RRC_IDLE/INACTIVE state and UE-assisted SMTC configuration in RRC_CONNECTED state). If modifications for inter-frequency are needed, Scenarios 1, 2, 3 (adjacent and overlaid inter-frequency neighbor cells of similar size) are prioritized.

-	QC thinks we also need a UE capability for this. Apple wonders which capability and thinks we can discuss this aspect later. QC thinks the capability is about the support for 2 periodicities in the inter-frequency case (per UE capability)

-	vivo thinks we just need one capability

We extend the Rel-19 SMTC enhancements to inter-frequency, using as a baseline the Rel-19 agreed solutions for intra-frequency (for both location-based SMTC selection in RRC_IDLE/INACTIVE state and UE-assisted SMTC configuration in RRC_CONNECTED state). If modifications for inter-frequency are needed, Scenarios 1, 2, 3 (adjacent and overlaid inter-frequency neighbor cells of similar size) are prioritized.

We introduce a new (per-UE) capability with signalling for the support in RRC connected of 2 periodicities in the inter-frequency case

Proposal 2	The introduction of a new distance threshold (that is associated with the Rel-19 reference locations) is not pursued due to lack of consensus.

-	vivo thinks we could introduce distance threshold at least for idle mode, for which there is no privacy issue. Nokia could be ok as long as we leave the behaviour to UE implementation

-	CSCN and ZTE think this could be useful also in connected mode. Also think that one way to address the privacy concern issue is to have a UE capability for this.

-	Toyota thinks this should not be an optional feature

-	Nokia thinks this is not useful in idle. QC/Google agree with Nokia

The introduction of a new distance threshold (that is associated with the Rel-19 reference locations) is not pursued due to lack of consensus

Agreements:

1. 	[V204/H250] We extend the Rel-19 SMTC enhancements to inter-frequency, using as a baseline the Rel-19 agreed solutions for intra-frequency (for both location-based SMTC selection in RRC_IDLE/INACTIVE state and UE-assisted SMTC configuration in RRC_CONNECTED state). If modifications for inter-frequency are needed, Scenarios 1, 2, 3 (adjacent and overlaid inter-frequency neighbor cells of similar size) are prioritized. We introduce a new (per-UE) capability with signalling for the support in RRC connected of 2 periodicities in the inter-frequency case

2.	[X250/V205] The introduction of a new distance threshold (that is associated with the Rel-19 reference locations) is not pursued

R2-2507782	Report of [AT131bis][303][NR NTN] RRC open issues – second round	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

PropReject: H253

[H253] is rejected

ToDo: C003, V200

Agreements:

[H253] is rejected

[V200] is Rejected

- Non-RIL related

R2-2507634	Service continuity in MBS NTN	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core	R2-2505822

Proposal 1	Broadcast future neighbour cells providing the same MBS service(s) as the serving cell.

Proposal 2	A UE is informed of an area over with uniform MTCH configuration parameters for a certain MBS service regardless the serving cell or satellite.

Can be discussed in the next meeting

R2-2506907	Remaining issues on broadcast service for NR NTN	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Reuse the existing mbs-NeighbourCellList-r17 in MBSBroadcastConfiguration to provide the future neighbour cells providing the same MBS service(s) as the serving cell for NTN MBS.

Proposal 2: The optimization that inform UE whether the cell replacing the serving cell provides an MBS service with identical MTCH configuration parameters is not needed.

R2-2507151	RRC corrections for Rel-19 NR NTN DL-CE	DENSO CORPORATION	discussion	NR_NTN_Ph3-Core

Proposal 3: RAN2 to discuss necessity of introducing prohibit timer of UEAssistanceInformation for reporting closest reference locations

R2-2507329	Downlink Coverage Enhancements for NTN for NR Phase3	TOYOTA Info Technology Center	discussion	Rel-19	38.331	NR_NTN_Ph3-Core

Proposal 1: Agree on the 38.331 CR provided in Annex A in this document, to indicate that the UE may use the refLocList association between reference locations and SMTC configurations, when selecting which SMTCs to consider if the UE supports less SMTCs than what is included in smtc4list and smtc5list.

R2-2506807	Discussion on RIL X250 X251 V205 for DL coverage enhancement	Xiaomi, ZTE, CSCN, Samsung	discussion	Rel-19

R2-2506833	Discussion on RIL V206 	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2506834	Discussion on RILs [V200][S024][S025][H250][V208][X250] regarding SMTC Enhancement	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2506866	[C006][C008][C009]Corrections on the smtc5list	CATT	discussion	Rel-19	NR_NTN_Ph3-Core	Late

R2-2506867	[C005][S024][S025]Discussion on the mechanism of UE reporting the N closest reference location	CATT	discussion	Rel-19	NR_NTN_Ph3-Core	Late

R2-2506868	[C003] Clarification of how the UE uses the service area information in SIB and USD	CATT	discussion	Rel-19	NR_NTN_Ph3-Core	Late

R2-2506907	Remaining issues on broadcast service for NR NTN	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2506935	[H250][H251][H252][H253] Discussion on remaining RRC issues	Huawei, HiSilicon	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2506989	Discussion on the remaining issue of Downlink Coverage Enhancements	CSCN	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2507044	Discussion on [RIL]O710 geo-fencing for ETWS	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2507123	Open issues on NR NTN (A200/V204)	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2507151	RRC corrections for Rel-19 NR NTN DL-CE	DENSO CORPORATION	discussion	NR_NTN_Ph3-Core

R2-2507329	Downlink Coverage Enhancements for NTN for NR Phase3	TOYOTA Info Technology Center	discussion	Rel-19	38.331	NR_NTN_Ph3-Core

R2-2507380	Remaining issues for MBS broadcast over NTN	Huawei, HiSilicon	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2507440	Remaining issues on beam hopping with multiple SMTC offsets [Q200]	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2507496	RIL S024 S025 C006	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2507523	RRC corrections related to RILs	ZTE Corporation,  Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core	Late

R2-2507538	Discussion on closest reference location reporting	ASUSTeK	discussion	Rel-19	38.331	NR_NTN_Ph3-Core

R2-2507625	Remaining RRC issues for NR NTN	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2507634	Service continuity in MBS NTN	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core	R2-2505822

R2-2507664	Discussion on RIL N085, S024, X250, A200	Nokia, Nokia Shanghai Bell	discussion

R2-2507675	Discussion on RIL V204-C003-Z253-Z255-V206-V207 for NR NTN Ph3	Xiaomi	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2507690	Discussion on various RILs for NR NTN	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2507691	[RIL H250] Applicability of SMTC enhancements to inter-frequency	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

### 8.8.3	Idle mode corrections

Corrections to TS 38.304.

R2-2506835	Discussion on 304 Open Issue 2 regarding PDCCH Repetition 	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

Observation: The RAN1 design enables the UE to use two linked search spaces for PDCCH reception for paging reception with soft combining, but the existing RAN2 specs do not account for the second associated search space in paging reception scenario.

Proposal: (304-2) For paging reception, RAN2 captures that PDCCH monitoring occasion(s) are determined according to pagingSearchSpace and its associated common search space (if configured).

-	after further checking vivo thinks the current 38.304 spec is ok but still think some reference to RAN1 spec is needed in RRC

-	Ericsson indicates there is already a reference to RAN1 spec in the field description iand this is sufficient. Nokia agress

[V201] is Rejected

R2-2507610	Discussion on Paging Search Space monitor for PDCCH repetition	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: RAN2 postpone discussion on updating TS38.304 regarding paging monitoring for linked search spaces until RAN1 finalizes the UE capability and expected UE behavior for PDCCH reception on linked Paging Search Spaces.

Proposal 2: It is unnecessary to introduce pagingSearchSpaceExt in PDCCH-ConfigCommon. The RIL Z256 can be rejected.

### 8.8.4	Other corrections

Corrections to TS 38.300, TS 38.306 and TS 38.321.

R2-2507254	Considerations on two SMTC periodicities	Nokia, Nokia Shanghai Bell	discussion	NR_NTN_Ph3-Core

Proposal 1: In case RAN4 cannot complete requirements in time, instead of limiting the configuration to one periodicity only, do not revert RAN2 solution but add a note to stage 2 specification mentioning that for 2 configured SMTCs the RAN4 requirements may not apply and time to complete measurements will be up to UE implementation.

-	QC and vivo think we can wait and see RAN4 progress first.

R2-2507288	Support for OCC RACH-less	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

### 8.8.5	LTE to NR NTN mobility corrections

Corrections to all specs for LTE_TN_NR_NTN_mob.

R2-2507045	Discussion on [RIL] O711 remaining issue on dedicated priority	OPPO, Xiaomi	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	Similar to redirection, include the satellite ID information in the FreqPriorityNR IE in RRCConnectedRelease message when E-UTRA provide the dedicated priority for NR NTN frequency.

-	HW thinks this is not essential and fear we might need similar changes for TEI19 items

-	Samsung can accept this for this WI but we should not do it for any TEI19 items

-	ZTE thinks this is an optimization not a correction.

-	Ericsson has some sympathy for this

-	Xiaomi thinks this is needed. QC also supports

-	Huawei thinks we don’t have anything similar also for NR NTN to NR NTN. Oppo thinks it should not be a reason to reject this

RAN2 understands that the NTN frequency in FreqPriorityNR should also be configured in SIB24 (no specs impacts)

[O711] is Rejected

R2-2507285	RRC corrections related to [S905]	Samsung	discussion	Rel-19	LTE_TN_NR_NTN_mob	Late

Proposal 1: Agree S905 according to the Text Proposal in Annex A.

-	vivo is ok with the intention and could also be fine with the actual TP

Agree S905 as:

- satAssistanceInfoList

- List of satellite ID(s), used to associate with the satellite assistance information for neighbour cell measurements on this frequency for the purpose of redirection. Each satellite ID included in this list corresponds to a satelliteId configured in neighSatelliteInfoListNR within nr-r19 or neighSatelliteInfoList within eutra-NTN-r19via SystemInformationBlockType33.

Proposal 2: Reject O711.

Proposal 3: Reject E801, keep the satellite assistance info list.

From 8.8.2:

R2-2507690	Discussion on various RILs for NR NTN	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 14	Accept E801. Limit the number of satellites IDs to 1 in SIB24 RRCConnectionRelease.

-	CATT prefers to keep the current spec as it is

-	Samsung agrees there should not be many satellites but doesn’t think we should limit to 1.

-	Nokia supports E801

-	Xiaomi thinks the redirection is to a frequency not to a satellite or a cell

-	vivo thinks the current spec is future proof and there is no need to limit. Thales agrees

[E801] is Rejected

R2-2507677	Discussion on the RIL X500 (smtc in the CarrierInfoNR-r19) for LTE-NR NTN mobility	Xiaomi, Samsung, OPPO, Apple	discussion	Rel-19	LTE_TN_NR_NTN_mob-Core

- The smtc-r19 is absent in the CarrierInfoNR-r19

Observation 1: The SMTC configuration is mandatory present in the measObjectNR and the smtc configuration is essential for the RRC redirection from LTE to NR.

Observation 2: Measurement on the NR NTN frequency by a RRC Connected UE in an LTE TN cell is currently not supported.

Observation 3: The NR NTN frequency cannot be configured in the measObjectNR.

Observation 4: The Release-20 WI for E-UTRA TN to NR NTN handover will introduce support for measurement on the NR NTN frequency by the RRC Connected UE in an LTE TN cell.

Observation 5: If smtc-r19 is absent in CarrierInfoNR-r19, the UE cannot fall back to the SMTC from measObjectNR, due to the lack of a matching NR NTN frequency configuration.

Proposal 1: Clarify in the field description of smtc that ‘In this release, the smtc is mandatory present in the CarrierInfoNR-r19’.

-	ZTE thinks this is not needed. Vivo/CATT agree.

-	QC thinks smtc cannot be absent in both places

-	ZTE thinks there is no RAN4 requirement to configure STMC in this case

- UE adjusts the SMTC according to the actual propagation delay

Proposal 2: For the smtc-r19 in the CarrierInfoNR-r19, the UE should adjust the offset according to the actual propagation delay.

R2-2507493	Remaining issues on LTE TN to NR NTN mobility	Huawei, HiSilicon	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: RAN 2 agrees that there is no need UE should adjust the offset by itself.

Smtc

The SSB periodicity/offset/duration configuration of the redirected target NR frequency. It is based on the timing reference of EUTRAN PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If smtc-r19 is configured, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the UE's propagation delay difference between serving cell and neighbour cells equals to 0 ms, and UE can adjust the offset based on the actual propagation delay.

Come back in the next meeting ([V230] is still ToDo)

Proposal 2: RAN 2 agrees that it is up to the network implementation whether to include the smtc in the CarrierInfoNR-r19 or not.

Agreed

R2-2507525	Clarification on SMTC configuration for redirection	ZTE Corporation,  Sanechips	discussion	Rel-19	LTE_TN_NR_NTN_mob

Proposal 1: No need to mandate NW to provide SMTC configuration in the field description for smtc configuration in carrierInfoNR-r19 or let UE use the SMTC provided in SIB24 when the smtc-r19 is not present in the CarrierInfoNR-r19.

No need to mandate NW to provide SMTC configuration in the field description for smtc configuration in carrierInfoNR-r19.

Proposal 2: Specify UE behavior to autonomous adjust offset in smtc configured in carrierInfoNR-r19 based on it’s actual propagation delay, and adopt the TP provided in Annex.

Agreements:

1. 	[O711] is Rejected (RAN2 understands that the NTN frequency in FreqPriorityNR should also be configured in SIB24 (no specs impacts))

2.	[S905]: Change description of satAssistanceInfoList as:

“List of satellite ID(s), used to associate with the satellite assistance information for neighbour cell measurements on this frequency for the purpose of redirection. Each satellite ID included in this list corresponds to a satelliteId configured in neighSatelliteInfoListNR within nr-r19 or neighSatelliteInfoList within eutra-NTN-r19 via SystemInformationBlockType33.”

3.	[E801] is Rejected

4.	[X500] is Rejected. No need to mandate NW to provide SMTC configuration in the field description for smtc configuration in carrierInfoNR-r19

## 8.9	IoT NTN Ph3

(IoT_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-252504)

Time budget: 0 TU

Tdoc Limitation: 3 tdocs

### 8.9.1	Organizational

LS, Rapporteur input, including open issues lists, etc.

Rapporteur inputs do not count towards the tdoc limitation.

Including the lists of open issues, if any, raised in the following email discussions:

[Post131][305][R19 IoT NTN] Stage2 CR (Ericsson)

[Post131][307][R19 IoT NTN] MAC CR (Mediatek)

[Post131][308][R19 IoT NTN] 36.304 CR (Nokia)

[Post131][309][R19 IoT NTN] capability CR (Qualcomm)

- Incoming LS

R2-2506717	Reply LS on on CB Msg3 EDT for IoT NTN Ph3 (R1-2506553; contact: MediaTek)	RAN1	LS in	Rel-19	IoT_NTN_Ph3	To:RAN2

Noted

R2-2506737	LS on CB-msg3-EDT (R4-2512173; contact: MediaTek)	RAN4	LS in	Rel-19	IoT_NTN_Ph3-Core	To:RAN2

Noted

- MAC open issues

R2-2507561	Remaining MAC open issues in Rel-19 IoT NTN	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core

Proposal 1: For CB-Msg3 transmission, the UE shall prioritize CB-Msg3-EDT data over TA report MAC CE. (if concluded to support TA report for CB-Msg3-EDT in RIL S901 discussion)

Come back after the RRC discussion

Proposal 2: RAN2 to discuss whether to define the backoff parameter in CB-Msg4 in unit of CB-Msg3 transmission window periodicity.

Noted

Proposal 3: RAN2 to discuss whether to capture CB-Msg3 scrambling with CB-RNTI as below

- Move the CB-RNTI derivation description from clause 5.1a.3 CB-Msg4 reception to clause 5.1a.2	CB-Msg3 transmission.

Move the CB-RNTI derivation description from clause 5.1a.3 CB-Msg4 reception to clause 5.1a.2	CB-Msg3 transmission.

- In Table 7.1-2: RNTI usage, define CB-RNTI is used for CB-Msg3 scrambling

-	Ericsson thinks there is no scrambling for Msg3

-	vivo thinks that no matter what we don’t need to capture this in RAN2 specs.

-	QC thinks this is the same case as EDT and this should be covered in RAN1 specs

In Table 7.1-2: RNTI usage, define CB-RNTI is used for CB-Msg3 scrambling

Proposal 4: RAN2 to discuss whether to change “HARQ ACK resource” field in CB-MSG4 from 2 bits to 4 bits for eMTC.

RAN2 does not add more HARQ ACK resources indications for CB-Msg4 for eMTC

R2-2507555	Corrections for CB-MSG3 EDT	MediaTek Inc.	CR	Rel-20	36.321	19.0.0	1599	-	F	IoT_NTN_Ph3-Core

Revised in R2-2507656

R2-2507656	Corrections for CB-MSG3 EDT	MediaTek Inc.	CR	Rel-19	36.321	19.0.0	1599	1	F	IoT_NTN_Ph3-Core

Can further check the values in table 7.2-x: Backoff Multification Factor for CB-Msg3-EDT

Endorsed

Idle mode open issues

R2-2507439	Rapporteur Summary for [Post131][308][R19 IoT NTN] 36.304 CR	Nokia , Nokia Shanghai Bells	discussion

Noted

The NOTE in 5.2.4.4 agreed in current version is not changed

For paging monitoring relaxation at satellites outside NAS configured list, RAN2 to discuss based on contributions at RAN2-131-bis.

For paging monitoring relaxation at satellites outside NAS configured list, RAN2 to discuss further during online discussion. Following key issues needs to be addressed by the proponents for better conclusion on this issue in the online meeting

o	Handling of system information changs and its impact to paging monitoring.

o	Clarification on the probability of the scenario in SF architecture to establish the benefits

The corrections provided in Annexure A is considered for Rel-19 CR. Also provided in R2-2507563

R2-2507563	Miscellaneous Corrections for TS36.304	Nokia 	CR	Rel-19	36.304	19.0.0	0885	-	F	IoT_NTN_Ph3-Core

Endorsed

- UE capabilities open issues

R2-2507443	Open issues on Rel-19 IoT NTN UE capabilities	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1	No UE radio capability signaling is introduced to indicate whether UE supports S&F mode operation

-	Ericsson thinks the eNB needs to know the UE capability to release the UE in the transition from normal to S&F mode. QC thinks the release procedure could start from the MME but we could consider the possibility to let the eNB to start this

An AS UE radio capability signaling is introduced to indicate whether UE supports S&F mode operation (to be captured in a CR for next meeting)

Proposal 2	Update the current field description of pws-Support-r19 by adding “It is optional for a PWS-capable NB-IoT UE to support the reception of PWS message on an acceptable cell”.

-	HW is not sure we need an extra capability for this

Proposal 3	RAN2 decide whether PWS capable UE supports acceptable cell in NB-IoT TN.

Proposal 4	If P3 is agreed, introduce TN network indication of support of PWS to prevent UEs camping on cells that will not support PWS broadcasting.

Proposal 5	For TN, RAN2 clarify that for pws-Support-r19 in TN, TDD/FDD differentiation is not needed.

-	vivo thinks we need to wait for SA1 response first

For pws-Support-r19,TDD/FDD differentiation in TN is not needed

- ASN.1 review

R2-2507060	RIL status on IoT NTN Ph3	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_Ph3-Core

PropAgree: V212, V213, V216, N011, N013, W801

V212, V216, N011, N013, W801 are Agreed

PropReject: V210, N012

V210, N012 are Rejected

ToDo: V211, C001, V215, S900, S901

V211, C001, V215, S900, S901, V213, Z001, Z002, S902 are ToDo

Duplicate: G001, V214

R2-2507059	Rapporteur correction on IoT NTN Ph3	Huawei, HiSilicon	CR	Rel-19	36.331	19.0.0	5160	-	F	IoT_NTN_Ph3-Core

Revised in R2-2507261

R2-2507261	Rapporteur correction on IoT NTN Ph3	Huawei, HiSilicon	CR	Rel-19	36.331	19.0.0	5160	1	F	IoT_NTN_Ph3-Core	R2-2507059

Endorsed (but can come back to changes for V213)

Revised in R2-2507787 to reflect the meeting agreements

R2-2507787	Rapporteur correction on IoT NTN Ph3	Huawei, HiSilicon	CR	Rel-19	36.331	19.0.0	5160	2	F	IoT_NTN_Ph3-Core

Discussed in [Post131bis][305]

[Post131bis][305][IoT NTN] RRC CR (Huawei)

Scope: Update the RRC CR with meeting agreements

Intended outcome: endorsed CR in R2-2507787

Deadline:  short

=> Endorsed in R2-2507787

Agreements:

For MAC:

1.	Move the CB-RNTI derivation description from clause 5.1a.3 CB-Msg4 reception to clause 5.1a.2 CB-Msg3 transmission.

2.	In Table 7.1-2: RNTI usage, define CB-RNTI is used for CB-Msg3 scrambling

3.	RAN2 does not add more HARQ ACK resources indications for CB-Msg4 for eMTC

For UE capability:

1.	An AS UE radio capability signaling is introduced to indicate whether UE supports S&F mode operation

2.	For pws-Support-r19,TDD/FDD differentiation in TN is not needed

### 8.9.2	RRC corrections

Corrections to TS 36.331.

[V211] (Indicate the regenerative operation mode to NAS)

R2-2506836	Discussion on RIL V211	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation: The upper layer may misunderstand the cell still operates in S&F mode since no information is forwarded to upper layer even if the cell changes from S&F operation mode to regeneration payload mode, i.e., the sf-OperationMode is not present in SIB1 and the t-ModeSwitching is not present in SIB31.

Proposal: (RIL V211) RAN2 to further decide which option is taken to indicate the upper layer that the cell operates in the regeneration payload mode.

-	Option 1: UE indicates to the upper layer that the cell operates in regeneration payload mode if the sf-OperationMode is not present in the SIB1.

-	Option 2: Clarify t-ModeSwitching is always present in SIB31 if the cell is capable of S&F mode.

-	HW/QC/Oppo think we already agreed to send the switching time to NAS

-	ZTE is not sure if the NW always have the information about the switching time and then does not support option 2, but it’s open to consider option 1. MTK agrees and see no harm to go for option 1

-	vivo/Xiaomi are ok to clarify according to option 2

-	Google thinks that if we go for option 2 we need to inform CT1

R2-2507650	[V211][C001][S901] Discussion on RILs for IoT-NTN	Google	discussion	Rel-19	IoT_NTN_Ph3-Core	Late

[V211] Indicate the regenerative operation mode to NAS

Observation 1	Based on the current specification, the UE NAS won’t be notified when the cell changes its operation mode from the S&F mode to the normal mode.

Observation 2	NAS layer with a running T3451 determines whether it can initiate a NAS procedure in a cell, based on the current operation mode (i.e., S&F mode or normal mode) of the cell.

Observation 3	NAS layer needs to trigger an abnormal case, upon receiving an EMM casue#83 in a cell operating in the normal mode.

Proposal 1	The UE RRC indicates to upper layers that the cell is operating in normal mode, upon acquiring a SIB1 that does not include a sf-OperationMode.

The UE RRC indicates to upper layers that the cell is operating in normal mode, upon acquiring a SIB1 that does not include a sf-OperationMode

R2-2507046	Discussion on RRC open issues for IoT NTN	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1	For a NTN cell operating in S&F mode, t-ModeSwitching shall be always configured by network.

[S900] (Accumulate SI decoding across SI windows for PWS SIB)

R2-2507284	RRC corrections related to [S900][S901]	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core	Late

Proposal 1: Agree S900 – a scheduling configuration in SIB1 indicates whether a UE may accumulate SI decoding for PWS SIB. Agree text proposal in Annex A.

-	Ericsson agrees with this proposal

-	Nokia thinks the same could apply to other SIB

Can come back in the next meeting

R2-2507611	Discussion on remaining RILs for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 5: The NB-IoT UE is allowed, but not required, to accumulate SI messages across SI windows.

Observation 6: change of the ETWS and CMAS SI message content does not trigger the SI modification procedure.

Proposal 4: Allow the NB-IoT UE implementation to attempt to accumulate PWS (ETWS/CMAS) SI messages across SI windows. No specification impact is expected.

-	CATT agrees with the proposal

[S901] (Clarification on TA report)

R2-2507284	RRC corrections related to [S900][S901]	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core	Late

Observation 1: Including the TA report MAC CE will cause at least 3 bytes of overhead and is not useful for CB-Msg3-EDT.

Proposal 2: Agree S901. TA report is not triggered by RRC for CB-Msg3-EDT. Agree text proposal in Annex A.

-	Ericsson thinks that in majority of cases there will be no further information so there is no need to have this. Nokia, Google, Xiaomi agree

-	vivo and QC think this comes for free and we should not disable the feature. Wonder what is the problem to include this if there is space

-	MTK sugest to go for a simple approach and go for this p2

TA report is not included during the CB-Msg3-EDT procedure.

We capture text proposal in Annex A into the RRC CR

R2-2506837	Discussion on RIL S901 and RIL V214	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: (RIL S901) RAN2 to capture in the RRC specification that when ta-Report or ntn-TA-Report is included in SystemInformationBlockType2(-NB), reporting of timing advance is enabled during Random Access or CB-Msg3-EDT.

Proposal 2: (RIL S901) RAN2 to capture in the MAC specification that the UE may be configured to report information about timing advance during a Random Access procedure or a CB-Msg3-EDT procedure.

[V215] (Removal of p0-UE-PUSCH-r19)

Continue in offline 308

R2-2506838	Discussion on RIL V215 	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1: As there is a dependency of power control for PUSCH and preamble, p0-UE-PUSCH configured for PUR transmission is utilized as a power offset to adjust the target P0 for PUR transmission, based on the link performance gap between the preamble and PUR PUSCH.

Observation 2: The parameter cb-Msg3-InitialReceivedTargetPower-r19, a dedicated parameter explicitly configuring the target P0 for CB-Msg3-EDT PUSCH transmission, decouples the power control for CB-Msg3-EDT PUSCH entirely from preambleInitialReceivedTargetPower.

Proposal: (RIL V215) RAN2 agrees to remove the parameter p0-UE-PUSCH-r19 from cb-Msg3-ConfigSIB(-NB).

[C001] (sf-OperationModeNeigh)

Continue in offline 308

R2-2506872	[C001] Correction on the operation mode of neighbour cells	CATT	discussion	Rel-19	IoT_NTN_Ph3-Core	Late

Observation 1: If we support providing the neighbour cell’s barring status, the following issues need to be considered:

- How could the serving cell obtain the neighbour cell’s barring status is unclear. The enhancement on X2 interface in RAN3 is needed;

- Changes of neighbour cell’s barring status will lead to frequent updates of system information, which is not conducive to UE energy consumption.

- It is unclear how the UE uses the neighbour cell’s barring status.

Observation 2: It is sufficient for the UE to obtain the neighbour cell’s barring status by reading that cell’s SIB1 according to legacy behaviour.

Proposal 1: For the information of neighbour cell in SIB33, only the operation mode of neighbour cell, not the barring status, should be provided. Adopt the TP in Annex.

R2-2507219	Discussion on leftover issues with Store and Forward satellite operation	ETRI, Korea University	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 4	RAN2 to clarify that the S&F mode signalled in SIB33 per neighbour satellite indicates only one of S&F mode and normal mode without any barring information.

Proposal 5	RAN2 to adopt the proposed change for [C001].

[Z001] definition of windowSize and windowPeriodicity

Continue in offline 308

R2-2507086	RRC corrections for R19 IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core	Late

Proposal 1: It’s suggested to define windowSize and windowPeriodicity with the unit of (N)PUSCH resources periodicity as below:

R2-2507441	Remaining issues on CB-EDT	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 2	For windowSize, use 2 bits of information to signal n1, n2, n3 and n4 where value n1 should correspond to duration of (1 * pusch-Periodicity-r19) and n2 to (2 * pusch-Periodicity-r19) and so on.

Proposal 3	Select the range of values for windowPeriodicity-r19 which are in multiple of 10ms and larger than or equal to windowSize.

[Other - S&F architecture indication]

R2-2506980	Discussion on remaining RRC open issues on S&F operation	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 3: RAN2 to consider indicating S&F architecture option (Split MME or Full CN) in SIB31.

Proposal 4: UE does not initiate user plane establishment and user plane data transfer in the cell indicating split MME architecture.

-	HW thinks the network knows the architecture and behaves accordingly. No need for further indications.

-	QC thinks this could be useful in the mobility scenario

-	ZTE prefers not to include this indication

-	CATT thinks the NW can inform the UE via NAS in Attach Procedure and no further AS indication is needed

-	Xiaomi thinks we could ask SA2 if this is needed. MTK agrees this is up to SA2 but we can wait for their indication

From 8.9.4:

R2-2507437	Remaining Open Issues related to SF Architecture aspects	Nokia , Nokia Shanghai Bells	discussion

Observation 1: Architecture option is common across all satellites of given PLMN. UE can deduce the architecture option based on NAS signalling.

Proposal 1: S&F Architecture option need not be indicated via any RRC signalling including SIB indication.

S&F Architecture option is not indicated via any RRC signalling including SIB indication (Can come back to if we receive an indication from SA2 to cover the scenario of mixed (split MME and Full CN) deployments)

[Other - SF mode indication and SI update trigger]

R2-2506980	Discussion on remaining RRC open issues on S&F operation	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: Change of the S&F mode indication and the S&F mode transition time do not trigger SI update procedure.

-	QC thinks we should rely on existing timers

-	Samsung is fine if this refers to a change of the S&F mode in the neighbour cell

We clarify in RRC that a change of the S&F mode indication and the S&F mode transition time of neighbour cells in SIB33 does not trigger SI update procedure.

Proposal 2: UE can re-acquire SIB33 upon transition time expiry for new S&F mode indication and transition time.

RAN2 understands that in idle mode the UE may re-acquire SIB33 upon transition time expiry for new S&F mode indication and transition time (if available) (check if anything needs to be added to an existing note for this)

From 8.9.4:

R2-2507635	Remaining open issues for store and forward	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1	The modification period for SIB1-NB is fixed at 4096 radio frames (i.e., 40.96 seconds) which may be longer than the remaining NGSO satellite service time after the operation mode transition.

Observation 2	The barring bit and the “S&F operation” indication in SIB1-NB may not be updated timely following the change of satellite operation mode.

Proposal 2	Regardless modification period boundaries, SIB1-NB is updated and re-acquired immediately after the satellite operation mode transition.

-	Google thinks this is related to legacy barring indication

-	QC thinks that if we don’t support this the NW would have to page all the UEs

-	vivo thinks we can leave this to NW implementation (e.g. start barring UEs before operation mode transition)

-	MTK thinks we could indicate that the NW can change the S&F mode in SIB1-NB without waiting the modification period

-	Aalyria, ZTE and Google thinks this proposal would have impacts to legacy UEs

RAN2 understands that in case the operation mode transition time is not aligned to the modification period legacy mechanisms shall apply (e.g. paging for UEs in idle) (no new enhancements are introduced)

[Z002] Signalling for SCS list configuration

Continue in offline 308

R2-2507086	RRC corrections for R19 IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core	Late

Proposal 2: It’s suggested to use the following way to define npusch-SubCarrierSetList-r19:

npusch-SubCarrierSetList-r19 ::=		CHOICE {

npusch-SubCarrierSetList-khz15			SEQUENCE (SIZE(1..12)) OF INTEGER (0..18),

npusch-SubCarrierSetList-khz3dot75		SEQUENCE (SIZE(1..48)) OF INTEGER (0..47)

}

[Other – misc]

R2-2507441	Remaining issues on CB-EDT	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 4	start timeAlignmentTimer when TA command is received in a successful CMR for a serving cell belonging to a TAG.

Proposal 6	Support new Msg4 multicast response format for legacy 4 step RACH or EDT procedure.

R2-2507643	RRC IoT NTN issues	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1	In 5.3.3.3x, modify “When receiving the CB-Msg3-EDT failure indication from lower layers, the RRC procedure is re-initiated” to “When receiving the CB-Msg3-EDT failure indication from lower layers, an RRC procedure may be initiated”

Proposal 2	In 5.3.3.1b, modify “In NTN, a BL UE, UE in CE mode A or NB-IoT UE can initiate EDT using the CB-Msg3-EDT procedure when all of the following conditions are fulfilled:” to “In NTN connected to EPC, a BL UE, UE in CE mode A or NB-IoT UE can initiate EDT using the CB-Msg3-EDT procedure when all of the following conditions are fulfilled:”

R2-2507149	RRC corrections for Rel-19 Store & Forward operation	DENSO CORPORATION	discussion	IoT_NTN_Ph3-Core

Observation 1: Satellite may need to set same value of t-ModeSwitching-r19 in t-Service also, since starting S&F mode equals to NTN service stop from legacy UE point of view.

Proposal 1: RAN2 to add NOTE to clarify that S&F capable UE can ignore t-Service, if the value of t-Service is same as t-ModeSwitching-r19 in normal mode

Other comments

R2-2507283	Discussions on RILs [V211][C001][V215]	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: Agree C001, change the values in sf-OperationModeNeigh to s&f and normal.

Proposal 2: On V215, RAN2 to discuss whether to remove both p0-UE-PUSCH/p0-UE-NPUSCH and alpha/alpha-NB.

Proposal 3: Reject V211.

[AT131bis][308][IoT NTN] RRC open issues (Huawei)

Scope: Continue the discussion on RIL V215, C001, Z001, Z002

Intended outcome: summary of the offline discussion (in R2-2507771)

F2F time: Thursday morning coffee break

Deadline for offline discussion summary: Thursday 2025-10-16 11:00

R2-2507771	Report of [AT131bis][308][IoT NTN] RRC open issues	Huawei	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: The S&F mode indication for neighbour satellite signalled in SIB33 contains no barring information.

Agreed

Proposal 2: If the S&F mode indication for a neighbour satellite is absent from SIB33, UE assume the corresponding neighbour satellite is operating in normal mode.

Agreed

Proposal 3: Change the unit of windowSize to PUSCH periodicity for IoT NTN CB-Msg3-EDT configuration.

Change the unit of windowSize to PUSCH periodicity for IoT NTN CB-Msg3-EDT configuration. We keep the unit of windowPeriodicity in tens of ms (aim to decide on the actual range for both in the next round of ASN.1 review)

Proposal 5: The alpha parameter for power control is kept in the CB-Msg3-EDT configuration. For the p0-UE-PUSCH parameter, wait for RAN1 progress.

Agreed

Proposal 6: Update the configuration npusch-SubCarrierSetList as follows:

npusch-SubCarrierSetList-r19 ::=		CHOICE {

npusch-SubCarrierSetList-khz15			SEQUENCE (SIZE(1..12)) OF INTEGER (0..18),

npusch-SubCarrierSetList-khz3dot75		SEQUENCE (SIZE(1..48)) OF INTEGER (0..47)

}

Agreed

Proposal 4: Keep the PUSCH configuration separate from the window configuration as in the current specification. FFS whether we need to clarify that the PUSCH resource outside of the window is invalid for CB-Msg3-EDT.

Keep the PUSCH configuration separate from the window configuration as in the current specification.

RAN2 understands that the PUSCH resource outside of the window is invalid for CB-Msg3-EDT (FFS if/how to clarify this in the spec)

Proposal 7: Further discuss whether extra indications need to be introduced in SIB2(-NB) to indicate whether UE can perform up-CB-Msg3-EDT and cp-CB-Msg3-EDT.

-	HW thinks this is a problem only if the NW configures CB-msg3-EDT without configuring legacy EDT

-	vivo thinks it’s more flexible to have 2 indications. Nokia/Ericsson/Xiaomi think this could be safer.

-	Huawei can accept to have this

-	ZTE thinks that if we go for this we need to refer to the presence of these indications in a number of places in RRC

We introduce extra indications in SIB2(-NB) to indicate whether UE can perform up-CB-Msg3-EDT and cp-CB-Msg3-EDT.

Agreements:

1.	[V211] The UE RRC indicates to upper layers that the cell is operating in normal mode, upon acquiring a SIB1 that does not include a sf-OperationMode

2.	[S901] TA report is not included during the CB-Msg3-EDT procedure

3.	S&F Architecture option is not indicated via any RRC signalling including SIB indication (Can come back to if we receive an indication from SA2 to cover the scenario of mixed (split MME and Full CN) deployments)

4.	We clarify in RRC that a change of the S&F mode indication and the S&F mode transition time of neighbour cells in SIB33 does not trigger SI update procedure.

5.	RAN2 understands that in idle mode the UE may re-acquire SIB33 upon transition time expiry for new S&F mode indication and transition time (if available) (check if anything needs to be added to an existing note for this)

6.	RAN2 understands that in case the operation mode transition time is not aligned to the modification period legacy mechanisms shall apply (e.g. paging for UEs in idle) (no new enhancements are introduced)

[C001]:

7.	The S&F mode indication for neighbour satellite signalled in SIB33 contains no barring information.

8.	If the S&F mode indication for a neighbour satellite is absent from SIB33, UE assume the corresponding neighbour satellite is operating in normal mode.

[Z001]:

9.	Change the unit of windowSize to PUSCH periodicity for IoT NTN CB-Msg3-EDT configuration. We keep the unit of windowPeriodicity in tens of ms (aim to decide on the actual range for both in the next round of ASN.1 review)

10.	Keep the PUSCH configuration separate from the window configuration as in the current specification.

11.	RAN2 understands that the PUSCH resource outside of the window is invalid for CB-Msg3-EDT (FFS if/how to clarify this in the spec)

12.	[V215] The alpha parameter for power control is kept in the CB-Msg3-EDT configuration. For the p0-UE-PUSCH parameter, wait for RAN1 progress.

13.	[Z002] Update the configuration npusch-SubCarrierSetList as follows:

npusch-SubCarrierSetList-r19 ::=	CHOICE {

npusch-SubCarrierSetList-khz15		SEQUENCE (SIZE(1..12)) OF INTEGER (0..18),

npusch-SubCarrierSetList-khz3dot75	SEQUENCE (SIZE(1..48)) OF INTEGER (0..47)

}

14.	[902] We introduce extra indications in SIB2(-NB) to indicate whether UE can perform up-CB-Msg3-EDT and cp-CB-Msg3-EDT.

### 8.9.3	MAC corrections

Corrections to TS 36.321.

- TA report during CB-Msg3-EDT

R2-2506979	Discussion on remaining MAC open issues on CB-Msg3-EDT	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1: TA report is not essential for CB-Msg3-EDT as such, but can be useful for potentially subsequent RRC connection, i.e. for the configuration of differential K-offset. Even if UE includes TA report in CB-Msg3, it would be useless if network does not want to send UE to connected mode.

Observation 2: If network wants to send UE to connected mode and really needs UE’s TA report, network can still configure (e.g. in CB-Msg4 or RRCConnectionReconfiguration message) UE to report TA based on event trigger.

Proposal 1: [MAC-Q001, RRC-S901] TA reporting during CB-Msg3-edt is not supported.

Addressed in the RRC discussion (TA report is not included during the CB-Msg3-EDT procedure)

- Details of Backoff scheme for CB-Msg3-EDT

R2-2507642	MAC IoT NTN issues	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1	The back off index 0 to 15 indicate a back off parameter value of 3, 6, 9,… 45, 48 multiples of the CB-Msg3-transmission window periodicity.

-	QC thinks the existing way is also fine and we could stick to that. HW agrees

-	MTK supports this because it’s good to align the attempts to the transmission window boundaries

-	Nokia thinks we could go for option B1 in NEC paper

-	NEC prefers Ericsson proposal (or A2 in their paper)

-	Samsung is fine as long as the final Backoff value is interpreted in ms

-	Xiaomi supports this proposal

The back off index 0 to 15 indicate a back off parameter value of [FFS] multiples of the CB-Msg3-transmission window periodicity.

R2-2507087	MAC corrections for R19 IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: For CB-Msg3 EDT procedure, Backoff Indicator also needs to include the resource information of time occasion and frequency occasion where collisions are detected.

-	Xiaomi doesn’t think this is needed and wonders how this is indicated in BI. ZTE thinks we could use an index

-	vivo, Ericsson, MTK agree with Xiaomi.

-	ZTE thinks the NW only needs to know where the collision happens. ZTE thinks this is needed because of msg4 multiplexing and the fact that BI is read by multiple UEs.

R2-2507306	CB-EDT relevant MAC open issues	NEC	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: When cb-Msg3-TxWindow is configured, it is necessary to review the BI values indicated by the 4-bit BI field and consider the following options:

-	Option A: The Backoff field indicates the number of DSA window periods

o	A1: without a table, 4bit BI field indicates the number of DSA window periods directly

o	A2: with a new table using periodicity as unit for both eMTC CB-Msg3 and NB-IOT CB-Msg3.

-	Option B: Larger backoff values in ms are introduced

o	B1: by utilizing the reserved code points in the current BI parameter values tables.

o	B2: two new BI tables with bigger values in ms are defined for eMTC CB-Msg3 and NB-IOT CB-Msg3 respectively.

- CB-RNTI for Msg3 scrambling

R2-2507286	Various MAC corrections on CB-Msg3-EDT	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: Confirm CB-RNTI is also used for CB-Msg3 scrambling and that it is captured in MAC specifications.

R2-2507642	MAC IoT NTN issues	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 3	RAN2 do not mention CB-RNTI in relation to the CB-Msg3

- HARQ ACK resources indications for CB-Msg4 for eMTC

R2-2507642	MAC IoT NTN issues	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1	Adding more indications for HARQ ACK resources for eMTC is a new feature for RAN1 that require many changes in RAN1 spec. RAN1 work for this WI is now in maintenance.

Proposal 2	RAN2 do not add more HARQ ACK resources indications for CB-Msg4 for eMTC.

RAN2 does not add more HARQ ACK resources indications for CB-Msg4 for eMTC

- On completing CB-Msg3-EDT without RRC message

R2-2507286	Various MAC corrections on CB-Msg3-EDT	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 2: On completing CB-Msg3-EDT without RRC message: clarify that successful completion is for the specific case of no RRC message received.

-	MAC rapporteur indicates this is already captured in the MAC CR

Proposal 3: Agree text proposal in Annex A.

R2-2507242	Issues on early termination of CB-Msg3-EDT without RRC message	Google	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 2	Terminating (successfully) an EDT procedure without an RRC message is a new procedure/feature introduced in Rel-19, and is tightly connected to UE’s RAI capability.

Proposal 4	RAN2 to discuss whether the support for terminating (successfully) a CB-Msg3-EDT without an RRC message is a mandatory or an optional feature for the UE supporting CB-Msg3-EDT.

-	MTK thinks this could be considered as part of the feature with no separate capability

We don’t introduce a separate UE capability for terminating (successfully) a CB-Msg3-EDT without an RRC message (a UE supporting cp-CB-Msg3-EDT also support this functionality)

Proposal 5	If the support for the terminating CB-Msg3-EDT without an RRC message is an optional feature according to P4, RAN2 to discuss how to capture this optionality in TS 36.306, among the following options: A) the existing rai-SupportEnh-r16 also indicates the support for terminating CB-Msg3-EDT without an RRC message; B) a new UE radio access capability parameter is introduced to indicate the support for terminating CB-Msg3-EDT without an RRC message; C) a new optional feature without UE radio access capability parameter is introduced to mark the support for terminating CB-Msg3-EDT without an RRC message.

- CQI and AS RAI reporting in Msg3

R2-2507242	Issues on early termination of CB-Msg3-EDT without RRC message	Google	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1	NB-IoT UEs are not allowed to transmit DCQR using “DCQR and AS RAI MAC CE” in CB-Msg3, which may hinder these UEs from informing the network of whether DL data following the UL data in CB-Msg3 is expected or not (i.e., may conflict with the RAN2#131 agreement).

Proposal 1	NB-IoT UEs are allowed to transmit the “DCQR and AS RAI MAC CE” in CB-Msg3, when rai-ActivationEnh is configured in SIB2-NB.

-	QC thinks this is already possible. Google thinks this is only clear for connected mode

-	Nokia and Nordic think we could use the NAS procedure.

Come back in the next meeting to decide whether NB-IoT UEs are allowed to transmit the “DCQR and AS RAI MAC CE” in CB-Msg3, when rai-ActivationEnh is configured in SIB2-NB

Proposal 2	If P1 is agreed, consider the TP in Annex as the baseline for amending the current 36.321 running CR.

Proposal 3	NB-IoT UEs shall set ‘Quality Report’ to the value ‘noMeasurement’ in the “DCQR and AS RAI MAC CE”, when transmitting the “DCQR and AS RAI MAC CE” in a CB-Msg3 and in a non-anchor carrier.

R2-2507286	Various MAC corrections on CB-Msg3-EDT	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 4: To support CQI (DCQR) and AS RAI reporting in CB-Msg3-EDT, add another condition in MAC to “Msg3 DCQR”.

Proposal 5: Agree text proposal in Annex B.

Agreements:

1.	The back off index 0 to 15 indicate a back off parameter value of [FFS] multiples of the CB-Msg3-transmission window periodicity.

3.	We don’t introduce a separate UE capability for terminating (successfully) a CB-Msg3-EDT without an RRC message (a UE supporting cp-CB-Msg3-EDT also support this functionality)

### 8.9.4	Other corrections

Corrections to TS 36.300, TS 36.304 and TS 36.306.

- relaxation of IDLE mode task based on the S&F monitoring list

R2-2506871	Discussion on relaxation of IDLE mode task based on the S&F monitoring list	CATT, Samsung, Thales, Google	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1: SA2 agreed that the S&F monitoring list is used by the UE assist with saving power and determining which satellites to receive MT data/signalling.

Proposal 1: When a S&F monitoring list is configured by upper layers, the UE may stop the IDLE mode tasks (e.g. cell (re)selection, paging monitoring, etc.) if the UE determines that it is out of coverage of all target satellite(s) indicated by the list. Endorse the TP in Annex.

-	HW sees some values for this but thinks we should revise to say that the relaxation is about idle mode tasks in NTN and then this applies “… when the UE determines that it is out of coverage of all target satellite(s) indicated by the list and by SIB32”. Samsung agrees

-	Nokia, Ericsson and ZTE don’t agree with p1

R2-2507438	Remaining Open issues for idle mode operation	Nokia , Nokia Shanghai Bells	discussion

Observation 1: Based on the procedure text given in TS23.401, we can deduce that UE can attempt MT data reception only in the NAS configured satellite list.

Observation 2: MME initiated paging for NAS procedure /Paging for system information modification is possible in the satellites outside the NAS configured satellite list.

Proposal 1: UE based paging monitoring relaxation based on NAS configured satellite list is not supported.

UE based paging monitoring relaxation based on NAS configured satellite list is not supported

Proposal 2: RAN2 to confirm that paging monitoring relaxation is only applicable when the cell is operating in SF mode only. UE switches to regular paging monitoring when the SF mode is changed to normal mode.

-	Xiaomi and QC don’t think this is needed

-	HW thinks that shutting down the cell is not an option

-	Samsung thinks the right way to do it is what suggested by CATT, this is clearly interior

- Paging enhancement in Store and Forward satellite operation

R2-2507061	Paging enhancement in Store and Forward satellite operation	Huawei, HiSilicon, Apple, Nokia, Ericsson, CENC	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal: An indication is introduced in the System information to indicate that the UE could skip monitoring paging.

-	Apple supports having an indication for this

-	MTK thinks this is enhancement is not needed

-	Nordic also think we don’t need such enhancement at this stage

-	Toyota thinks there could be some benefit

-	ZTE this could be useful but to make it work eNB and MME should be aligned, so some RAN3 work might be needed

-	Ericsson think we could have an indication in SIB31 “no more DL data in the buffer” without mandating any UE behaviour. Huawei thinks it’s a possible compromise

-	QC thinks this impacts the system and other groups need to be involved

Come back in the next meeting to check whether we can have an indication (likely in SIB31) without mandating any UE behaviour

Send a LS to SA2 (Cc: RAN3) saying that RAN2 is discussing to have an indication in SIB to inform UEs than no paging is expected (so that the UE may skip paging) and asking them if any paging is expected from MME to RAN when the feeder link is not available (can further work on the LS text in the Post meeting email discussion)

[Post131bis][307][IoT NTN] LS to SA2 (Huawei)

Scope: discuss the LS to SA2 on paging enhancements

Intended outcome: Approved LS in R2-2507930

Deadline:  short

=> Approved in R2-2507930

R2-2507930	LS to SA2 on the paging in Store and Forward	RAN2	LS out	Rel-19	IoT_NTN_Ph3-Core	To:SA2	Cc:RAN3

=> Approved

R2-2506978	Discussion on remaining 36.304 open issues on S&F operation	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

Observations:

1)	Paging monitoring is not only for downlink data arrival (e.g. CN-paging), but also to receive SI update notification. Missing SI update will make future communication (e.g. triggered by UL data) using outdated cell configurations.

2)	The S&F mode satellite can still change its mode to normal, therefore it still can forward core network’s paging.

3)	For each UE, downlink data might come at different time, therefore it would be a corner case that a S&F mode satellite does not store any data for any UEs.

Proposal 1: RAN2 not to pursue paging monitoring relaxation.

- Impact of the S&F mode on AS

Come back in the next meeting

R2-2507244	Impact of the S&F mode transition time on AS	Google	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1	As the S&F mode indication (i.e., sf-OperationMode) and the legacy barring indications can only change at the boundaries of SI modification periods, the S&F mode transition time (i.e., t-ModeSwitching) signaled in SIB31 allows UEs to know more precisely when the serving cell changes its operational mode.

Observation 2	If a non-S&F-capable UE considers a cell as barred because it's currently operating in S&F mode, the UE shall exclude the cell as a candidate for cell selection/reselection for 300 seconds, even if the cell will revert to the normal mode before the 300-second timer expires.

Proposal 1	A Rel-19 non-S&F-capable UE can exclude a barred cell as the candidate for cell selection/reselection until t-ModeSwitching, if sf-OperationMode is signaled by the cell with the value ‘notBarred’ (TP is attached in Annex A).

Proposal 2	A Rel-19 non-S&F-capable UE can utilize the transition time from the normal mode to the S&F mode to determine when to trigger the measurement for cell reselection (TP is attached in Annex B).

R2-2507635	Remaining open issues for store and forward	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

- Specification impact of S&F cell de-prioritization

Proposal 1	Capture in normative text that a Rel-19 UE may exclude S&F cells as candidate cell for cell reselection.

- Operation mode transition without cell change

Observation 1	The modification period for SIB1-NB is fixed at 4096 radio frames (i.e., 40.96 seconds) which may be longer than the remaining NGSO satellite service time after the operation mode transition.

Observation 2	The barring bit and the “S&F operation” indication in SIB1-NB may not be updated timely following the change of satellite operation mode.

Proposal 2	Regardless modification period boundaries, SIB1-NB is updated and re-acquired immediately after the satellite operation mode transition.

Observation 3	A UE excludes a barred cell (or frequency) from the candidate list for cell re-selection for 300 seconds.

Observation 4	During the service time of a LEO satellite, a UE may not re-select to a cell previously determined as barred, even if the satellite changes operation mode (e.g., non-S&F) and the barring bit is deactivated.

Proposal 3	The UE excludes a barred neighbouring cell due to S&F operation mode only until the operation mode transition time.

R2-2507494	UE behaviour related to the Satellite ID list for Store and Forward	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: When determining whether to perform measurement in IDLE, the UE should consider both NAS information (i.e., Satellite list and NAS wait timer) and the information in SIB32 (i.e., t-ServiceStart and ephemeris). The text proposal for capturing this in TS 36.304 is provided in the Annex

- Acceptable cell camping

Come back in the next meeting

R2-2507287	Idle mode and capability-related corrections	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: Acceptable cell camping on a TN cell can be allowed if there is a way of preventing UEs to camp on the cell as an acceptable cell.

Proposal 2: Introduce a network indication of support of PWS broadcasting. If indicated, a UE may camp on the cell as an acceptable cell.

R2-2507089	Other corrections for R19 IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 2a: Whether PWS can be supported for TN can be left to NW and UE implementation, e.g., whether SIB10~12-NB messages can be broadcast from NB-IoT TN can be left to TN implementation.

Proposal 2b: Under any scenario, a UE must not refrain from camping on a network only because it does not support PWS provision. There is also no need to introduce any kind of NW indication about PWS broadcasting.

- UE AS capability for support of S&F

R2-2507047	Discussion on other open issues for Rel-19 IoT-NTN	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 3	RAN2 assumes no need to introduce AS capability signalling for UE supporting of S&F mode for now, given the introduced NSA capability for this. Send LS to RAN3 for further check/confirmation.

Agreements:

1.	UE based paging monitoring relaxation based on NAS configured satellite list is not supported

R2-2506873	Correction on the any cell selection state for NB-IoT	CATT	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2506944	Discussion on remaining issues on Store&Forward	Transsion Holdings	discussion	Rel-19

R2-2507047	Discussion on other open issues for Rel-19 IoT-NTN	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2507359	Discussion on S&F Idle Mode Procedures	TOYOTA ITC	discussion	Rel-19	IoT_NTN_Ph4-Core

R2-2507437	Remaining Open Issues related to SF Architecture aspects	Nokia , Nokia Shanghai Bells	discussion

## 8.10	SON/MDT Ph4

(NR_ENDC_SON_MDT_Ph4-Core; leading WG: RAN3; REL-19; WID: RP-234038)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

### 8.10.1	Organizational

LS, CR rapporteur’s miscellaneous non-controversial corrections, etc.

LSs

R2-2506728	LS on geographical area scope MDT (R3-255960; contact: CATT)	RAN3	LS in	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	To:RAN2, SA5

Noted

Capabilities

R2-2506784	Summary of open issue email discussion on SONMDT UE capabilities	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: RAN2 to discuss whether the UE capability for “Logged MDT enhancement for Slicing” is optional WITHOUT signalling or optional WITH signalling.

-	CATT prefers without signalling. Nokia prefers to have signalling because the network use the capability indication to understand which UEs to select. Qualcomm think that we can use the existing capability indication for slice based cell reselection. Nokia says there is not a one to one mapping since you might support slice based resection without the logging.

-	Session chairs wonders if this is 1 bit? CATT confirms.

The “Logged MDT enhancement for Slicing” is optional WITH signalling.

R2-2506785	Corrections on SONMDT UE Capabilities	CATT	CR	Rel-19	38.306	18.6.0	1353	-	F	NR_ENDC_SON_MDT_Ph4-Core

Endorsed and to be updated based on further agreements, e.g. add sigalling for “Logged MDT enhancement for Slicing”

[AT131bis][605][SONMDT] Corrections on SONMDT UE Capabilities (CATT)

Scope:

    - Updated 38.306 CR based on further agreements

Intended outcome:

    - Endorsed CR(s) in R2-2507839 and R2-2507840

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2507839   Corrections on SONMDT UE Capabilities CATT     draftCR  Rel-19    38.331    19.0.0       F     NR_ENDC_SON_MDT_Ph4-Core

R2-2507840   Corrections on SONMDT UE Capabilities CATT     draftCR  Rel-19    38.306    19.0.0       F     NR_ENDC_SON_MDT_Ph4-Core

Both are endorsed and to be merged in the mega CR.

R2-2507422	Correction on R19 SONMDT in TS 36.331	Huawei, HiSilicon	CR	Rel-19	36.331	19.0.0	5167	-	F	NR_ENDC_SON_MDT_Ph4-Core

-	Nokia indicates that one of the RIL-status for B001 is not updated, it should say PropAgree but says “ToDo” now.

Endorsed and to be updated based on further ASN.1 review.

R2-2507423	WI Comments file for TS 36.331 CR for R19 SONMDT	Huawei, HiSilicon	other	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Noted

R2-2507424	WI Review file for TS 36.331 CR for R19 SONMDT	Huawei, HiSilicon	other	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Noted

R2-2507665	SONMDT Comment file	Ericsson	report	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

All PropAgree are agreed, except E028 which should be ToDo.

All PropReject are rejected

Noted

R2-2507666	SONMDT Review file	Ericsson	report	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Noted

R2-2507667	Corrections on RRC for Rel-19 SONMDT features	Ericsson	CR	Rel-19	38.331	19.0.0	5560	-	F	NR_ENDC_SON_MDT_Ph4-Core

-	Nokia says that they want to discuss E028 further.

Endorsed and to be updated based on further ASN.1 review. However, E028 should be ToDo pending further discussion.

### 8.10.2	Papers related to RILs

Papers related to identified RILs

C057

R2-2506781	Discussion on MDT RIL[C057]	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

-	Ericsson think that the scenario in the procedural text is different. The intetion is that the UE shall log the cell the UE ended up camping on when the UE didn’t find a highest priority slice cell.

The UE shall log the cell the UE ended up camping on for the last failure occurrence.

C058, C060

R2-2506782	Discussion on SON RIL[C058][C060]	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: [C058] To align the time info included in RLF report and SCGFailureInformation, i.e., set timeBetweenLastFulfillmentAndEvent to the elapsed time between the point in time of fulfilling the last triggering event of the fulfilled execution condition and the SCG RLF, which means PSCell change/addition failure is not taken into account.

Proposal 2: [C060] RAN2 to clarify that the change in procedure description of logging timeSinceCHO-Reconfig is implemented from Rel-18, i.e., “the reception of the last conditionalReconfiguration including the condRRCReconfig of the target PCell in the source PCell” should be “the reception of the last applied conditionalReconfiguration including the condRRCReconfig of the target PCell in the source PCell”.

On P1:

-	Nokia think we don’t use SCG RLF. Ericsson RLF and PSCell change failure are equally important. CATT says they want to align RLF and PSCell change failure report.

On P2:

-	Ericsson think this is a Rel-18 change so we should look at a Rel-18 CR. CATT think we can put this in RRC spec rapp CR. Ericsson is OK with this, but think there are other problems. Nokia wants to see a proper CR. Session chair says we can wait to the next meeting and can see a proper CR.

C058 is agreed

X551, X554

R2-2506993	Discussion on Rel-19 SONMDT RILs	Xiaomi	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: (X551) To align with the RAN2 agreement that “the UE suspends logging if it cannot obtain location information”, RAN2 to confirm the correctness of one of the following interpretations of the current procedural text:

Option 1: The UE’s “skipping” of logging for an interval is also a form of “suspending”.

Option 2: The UE’s “skipping” of logging for an interval is not a form of “suspending”.

Proposal 2: (X551) If RAN2 agrees that the UE’s “skipping” of logging for an interval qualifies as “suspending”, RAN2 to adopt the TP (First change) proposed in Tdoc R2-2506993.

Proposal 3: (X554) RAN2 to agree that MDT optimization for NTN is restricted to (quasi-) Earth-fixed NTN cell scenarios (not NTN Earth-moving cell scenarios) and adopt the TP (Third change) proposed in Tdoc R2-2506993.

Proposal 4: (X554) RAN2 to send a LS to inform RAN3 of the conclusion regarding the applicable scenarios for MDT optimization for NTN.

On P1/P2:

-	Samsung thinks current spec is fine its mainly about that the NW shouldn’t get these measurements when the UE skips/suspends, and the current spec achieves this. Nokia agrees with Samsung. ZTE think we discussed this already and companies thought that there is no real confusion and current spec is fine.

X551 is rejected, unless critical issues are found.

On P3/P4:

-	Nokia think this is not needed since the reference location is different in R17 and R19, current spec is fine. Qualcomm thinks we can change the name. Nokia would be OK to rename it, e.g. “circularAreaReferenceLocation” and “circularAreaRadius”. Ericsson think we should rename since RAN3 has not restricted.

We rename “referenceLocation” to “circularAreaReferenceLocation” and “radius” to “circularAreaRadius”.

R2-2507409	[RIL] N064, N065: RLF, SHR logging when CHO only HO is performed	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: RAN2 confirms that the relevant case is when a complimentary CHO-only is executed when the CHO with candidate SCG is also configured.

Proposal 2: RAN2 agrees that the current specification does not fully address the case when a complimentary CHO-only is executed when the CHO with candidate SCG is also configured (e.g., nothing is logged when the two configurations are received together, and SHR is only created if one of the legacy near failure scenario happens).

Proposal 3: RAN2 agrees that based on current specification the UE logs information that is not relevant for the root cause analyses (e.g., logging happens about the condition fulfilment status at the time of configuration of the CHO-only is received).

Proposal 4: RAN2 agrees to address the case when a complimentary CHO-only is executed when the CHO with candidate SCG is also configured by logging the relevant information when this happens.

On P1:

-	ZTE agrees and this is already considered in the spec. Samsung think this was not the intention. Huawei think that the interesting scenario is that the UE gets a complementary CHO-only when the UE already has a CHO with candidate SCG.  Huawei understands that this is challenging the previous agreements, the intention was different than what Nokia suggests.

Not pursued, the intention is not that a complimentary CHO-only is executed when the CHO with candidate SCG is also configured.

R2-2507582	[S20][S021]Discussion on SON/MDT RILs	Samsung	discussion	Late

Proposal1: Update the check for highest priority while logging slicing related information in logged measurements as following in TS 38.331:

4>	if the UE has performed cell reselection using reselection priorities for slice-based cell reselection and was not able to camp on a suitable cell that supports the NSAG ID with the highest priority received from the upper layer (see TS 24.501 [23]) and present in the used FreqPriorityListDedicatedSlicing or FreqPriorityListSlicing  (as specified in TS 38.304 [20]) during the last logging interval:

Proposal 2: UE does not duplicate NSAG ID in different entries within logged measurements.

Proposal 3: Use “camped-on” instead of “select” in “failure in attempt to select a cell” and remove “if it is different from servCellIdentity” in the condition for logging reselectedCellId.¨

Proposal 4: ReselectedCellId is logged only for Intra-NR handover.

On P1:

-	Qualcomm think that the highest priority is from UE side, i.e. from upper layers in the UE, not in the sys info. ZTE agrees with Qualcomm. SAamsung thinks that we should clarify that it’s the slices received from NAS. Nokia is OK but its already clear from the 304.

On P2:

-	Ericsson thinks that the CN can change. Samsung says only at registration. Nokia says that the CN can update any time. Nokia think we might lose info so they are not supportive.

Agreements - Use “camped-on” instead of “select” in “failure in attempt to select a cell” and remove “if it is different from servCellIdentity” in the condition for logging reselectedCellId. - ReselectedCellId is logged only for Intra-NR reselection.

R2-2507668	Addressing RILs for Rel-19 SONMDT features [E015, C057, E051]	Ericsson	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	RAN2 to align the descriptions for UE capabilities introduced for Rel-19 SON/MDT between TS 38.331 and TS 38.306:

a.	Use “RLF Report for time or location based CHO in NTN” for UE capability of supporting MRO enhancement in NTN.

b.	Use “SHR and RA report enhancement for MCG LTM” for UE capability of supporting both SHR and RA report.

Proposal 2	If UE is unable to find a cell based on NSAG priority information and reselects to a cell where it declares failure and goes to any cell selection state, it logs the cell information as reselectedCellId in logged MDT report.

Proposal 3	The current implementation of ltm-RecoveryCellId in RLF report is compatible with inter-CU LTM. RAN2 remove the Editor’s Note from the relevant text in RRC spec.

Agreements - RAN2 to align the descriptions for UE capabilities introduced for Rel-19 SON/MDT between TS 38.331 and TS 38.306: - a. Use “RLF Report for time or location based CHO in NTN” for UE capability of supporting MRO enhancement in NTN. - b. Use “SHR and RA report enhancement for MCG LTM” for UE capability of supporting both SHR and RA report. - The current implementation of ltm-RecoveryCellId in RLF report is compatible with inter-CU LTM. RAN2 remove the Editor’s Note from the relevant text in RRC spec.

R2-2507671	Discussion on R19 SONMDT RIL [H310]	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: [H310] It is proposed to add a spare value (for the future use) in the IE fulfilledConfigWhenChoOnly-r19.

H310 is agreed

R2-2507672	Discussion on R19 SONMDT RIL [H301]	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: [H301] In both TS 38.331 and 38.306, if the UE supports RLF-Report for conditional handover with time-based or location-based trigger condition, it is proposed to add “e.g. condEventD1, condEventD2 or condEventT1)” for clarification.

Clarify in 38.306 what time-based or location-based trigger conditions are, i.e. that it comprises condEventD1, condEventD2 and condEventT1.

H301 is rejected

Withdrawn

R2-2507233	Discussion about SON MDT RILs	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	Late

- ZTE indicated offline that they withdraw this paper.

### 8.10.3	Other

Critical corrections, if any.

R2-2507234	Correction to NTN MDT CR 37.320 Rel-19	ZTE, Sanechips	CR	Rel-19	37.320	19.0.0	0145	-	F	NR_ENDC_SON_MDT_Ph4-Core

-	Nokia thinks this is a RAN3 text so should be discussed there. And they have a CR there.

Not pursued by RAN2, RAN3 can discuss this if they want

R2-2507235	Correction to successful LTM cell switch CR 38.300 Rel-19	ZTE, Sanechips	CR	Rel-19	38.300	19.0.0	1041	-	F	NR_ENDC_SON_MDT_Ph4-Core

-	Qualcomm think RAN3 discussed the current working and find this CR unnecessary. Ericsson think in we should change to “In case of MCG LTM cell switch”. Qualcomm would be OK to do what Ericsson suggests. Nokia agrees with the change from ZTE.

Postponed

R2-2507410	Some MRO related enhancements	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: RAN2 confirms that the current specification fails in logging the information about the chronological fulfilment of the two execution conditions of the last executed CHO with candidate SCGs in case of RLF shortly after performing a successful CHO with candidate SCGs.

Proposal 2: RAN2 agrees that

1)	The UE can be configured with new time threshold criterion, which is used to check the time being set in the field timeConnFailure.

2)	A new branch is added to RLF content determination clause to add the missing information about the chronological fulfilment of the two execution conditions of the last executed CHO with candidate SCGs in case of RLF shortly after performing a successful CHO with candidate SCGs.

- Huawei wonders if this is related to any RIL. Nokia says no, but can add a RIL if needed. Xiaomi diesnt support this since is a rare case. ZTE agrees with Nokias intention but would like to postpone. Ericsson think that there is no critical problem.

Adopt the CHO-approach (i.e. first triggering event and time between events) for logging time between fulfilment of execution condition for the case when CHO with candidate SCGs is successfully executed but followed by RLF.

R2-2507623	Stage-2 corrections for SONMDT features	Ericsson	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	RAN2 to discuss changes to stage-2 description of SHR according to the TP in Annex 1.

Proposal 2	RAN2 to discuss changes to stage-2 description of logged MDT according to the TP in Annex 2.

The first TP needs polishing offline

The TP in Annex 2 is agreeable

[AT131bis][606][SONMDT] Stage-2 corrections for SONMDT (Ericsson)

Scope:

    - Discuss offline (face-to-face preferred, if suitable)

Intended outcome:

    - In-principle agreed CR(s) in R2-2507838

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2507838	Stage 2 additions on MDT for slicing	Ericsson	CR	Rel-19	37.320	19.0.0	0147	-	B	NR_ENDC_SON_MDT_Ph4-Core

In-principle agreed but no need to write the “stage 2” in the final Tdoc title. Find a better name for the final CR.

R2-2506783	Discussion on PLMN ID list for NTN MDT (LS R3-255960)	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Moved from 8.10.1

-	ZTE thinks there are existing fields which can be used for providing PLMN ID lists. Huawei agrees with ZTE but think maybe we need to check the spec a bit more in detail.

To be addressed during ASN.1 review

## 8.11	Evolution of NR duplex operation: Sub-band full duplex (SBFD)

(NR_duplex_evo-Core; leading WG: RAN1; REL-19; WID: RP-251874)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

### 8.11.1	Organizational

Incoming LS, Rapporteur input, etc..

LSin

R2-2506718	Reply LS on CSI-RS based CFRA using SBFD RO (R1-2506556; contact: ZTE)	RAN1	LS in	Rel-19	NR_duplex_evo-Core	To:RAN2	Cc:RAN4

Noted

38.321

R2-2507158	Summary of Rel-19 SBFD MAC open issue discussions for maintenance	Samsung	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposed WF for MAC-1:

Following the majority view, no correction will be considered for MAC-1 in Rapp CR. Companies that deem further discussion necessary, can submit tdoc contribution for MAC-1.

Discussion

-	Ericsson think the condition is not needed since it is already clear in stage 2

No correction will be considered for MAC-1 in the MAC CR.

R2-2507080	Correction on MAC spec for R19 SBFD	Samsung	CR	Rel-19	38.321	19.0.0	2126	-	F	NR_duplex_evo-Core

Discussion

-	Xiaomi think the ‘RO’ in the OCT 4 of Enhanced LTM Cell Switch Command MAC CE should be moved to the other type since the current byte is optional and it may not exist. Samsung think this comment is valid.

RO is placed next to the NCC field. CR will be updated accordingly (will be handled via post meeting email).

[Post131bis][212][SBFD] CR for TS 38.321 (Samsung)

Intended outcome: Update the CR for endorsement

Deadline:  Short

R2-2506820	Open issues in TS 38.300 on Rel-19 Evolution of NR duplex operation (SBFD)	CATT	discussion	Rel-19	NR_duplex_evo-Core

=> Moved to 8.11.3

### 8.11.2	MAC issues

Remaing MAC issues

MAC-2: How to clarify "the preambleReceivedTargetPower used in Msg3 transmission power derivation is that configured for legacy RO, in the case that Msg3 is transmitted in non-SBFD symbols and SBFD RACH config option 2 is configured."

R2-2507255	Discussion on the remaining MAC open issues	Samsung	discussion	Rel-19	NR_duplex_evo-Core

Noted

[MAC-2] Proposal 1: RAN2 to prioritize Options 3 and 4, over Options 1 and 2, to minimize the risk of further mis-interpretation, where

- Option 1: "indicate the preambleReceivedTargetPower included in the rach-ConfigCommon or the rach-ConfigCommon-r17".

- Option 2: "indicate the preambleReceivedTargetPower included in the rach-ConfigCommon".

- Option 3: "indicate the preambleReceivedTargetPower not included in the sbfd-RACH-DualConfig".

- Option 4: "indicate the preambleReceivedTargetPower configured for the first PRACH occasions".

R2-2506822	Remaining Issues on Random Access	CATT	discussion	Rel-19

Noted

[MAC-2] Proposal 1: Positive statement is slightly preferred, that is to clarify that for RACH configuration option 2, if Msg3 PUSCH transmission is performed in non-SBFD symbols, preambleReceivedTargetPower included in rach-ConfigCommon can be used.

R2-2507280	Remaining MAC issues on SBFD	LG Electronics Inc.	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1. [MAC-2] Use the format of ‘not in yyy’ to specify that preambleReceivedTargetPower configured for non-SBFD RO is used if Msg3 PUSCH is transmitted in non-SBFD symbols

R2-2507264	Remaining issue of SBFD	Qualcomm Incorporated	discussion	NR_duplex_evo-Core

Noted

Proposal 1: For the case that Msg3 is transmitted in non-SBFD symbols and SBFD RACH config option 2 is configured, it is to clarify the preambleReceivedTargetPower used is the one included in the RACH-ConfigCommon.

If Msg3 PUSCH transmission is performed in non-SBFD symbols, indicate the preambleReceivedTargetPower configured for the first PRACH occasions".

MAC-3: Whether, and (if support) how, to support the compensation for the difference in preamble received target power between SBFD RO and legacy RO (in addition to the compensation for the power ramping difference that is already supported), for ensuring preamble transmit power continuity, during RO type switching.

R2-2507517	Discussion on SBFD MAC open issues	Xiaomi	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 2: [MAC-3] During RO type switching, there is no need to compensate the difference in preamble received target power between SBFD RO and legacy RO. No specification change is needed.

R2-2507266	MAC Issues - SBFD	Nokia 	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1: To support the compensation for the difference in preamble received target power between SBFD RO and legacy RO (in addition to the compensation for the power ramping difference that is already supported), for ensuring preamble transmit power continuity, during RO type switching.

Discussion

-	Charter support Nokia proposal since there is power difference in different ROs.

-	ZTE do not think the power difference is large and think UE can adjust anyway.

-	Samsung think the difference can be addressed by proper NW configuration, so no need to have compensation. Vivo agree this view. CATT and Huawei think R1 already discussed this issue, and think this is out of R2 scope. CATT, InterDigital, and vivo support Xiaomi proposal.

-	Apple think Nokia proposal is useful, and think for 2step RACH and 4step RACH the coverage of msg3 is similar so no there wasn’t pathloss difference, and think the case of SBFD RO is different.

-	Huawei think no compensation is acceptable in R1.

-	Nokia think R2 can discuss this issue. Ericsson agree with this view and think Nokia proposal make sense.

-	Nokia think this needed since it give NW more flexibility in configuration. Nokia think there is power difference also due to the fact that some of the gNB panels are used for UL direction.

-	OPPO think it is difficult to estimate the exact difference, and do not think R1 has clear view.

-	WI Rapp suggest to discuss R1 if there is real issue about this. Samsung, CATT agree.

R2-2507003	Discussion on residual issues for MAC spec	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

Noted

Postponed

Proposal for MAC-x: Agree the TP of bypassing the immediate Msg1 repetition number fallback when the UE performs RO type fallback.

Discussion

-	ZTE agree with the issue but want to refine the changes.

-	Samsung think this could be solved by NW configuration. LG E also think this not critical.

R2-2506822	Remaining Issues on Random Access	CATT	discussion	Rel-19

R2-2506971	Discussion on MAC-2 and MAC-3 for SBFD	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2507003	Discussion on residual issues for MAC spec	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

R2-2507255	Discussion on the remaining MAC open issues	Samsung	discussion	Rel-19	NR_duplex_evo-Core

R2-2507264	Remaining issue of SBFD	Qualcomm Incorporated	discussion	NR_duplex_evo-Core

R2-2507266	MAC Issues - SBFD	Nokia 	discussion	Rel-19	NR_duplex_evo-Core

R2-2507280	Remaining MAC issues on SBFD	LG Electronics Inc.	discussion	Rel-19	NR_duplex_evo-Core

R2-2507363	MAC remaining issues	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

R2-2507517	Discussion on SBFD MAC open issues	Xiaomi	discussion	Rel-19	NR_duplex_evo-Core

R2-2507576	Discussion on UE transmit power continuity during RO type switching	vivo	discussion	Rel-19	NR_duplex_evo-Core

### 8.11.3	Other aspects

Issues related to RILs, other remaing RRC issues, Changes to Stage 2, UE capabilities, and other remaining issues if not covered by the previous agedam items

38.331

R2-2507000	WI SBFD ASN.1 Review file	Huawei, HiSilicon (Rapporteur)	discussion	Rel-19	NR_duplex_evo-Core

Noted

R2-2507001	WI SBFD ASN.1 Comments file	Huawei, HiSilicon (Rapporteur)	discussion	Rel-19	NR_duplex_evo-Core

Noted

R2-2506999	Corrections to WI SBFD	Huawei, HiSilicon (Rapporteur)	CR	Rel-19	38.331	19.0.0	5499	-	F	NR_duplex_evo-Core

Will be updated using post meeting email discussion.

[Post131bis][213][SBFD] CR for TS 38.331 (Huawei)

Intended outcome: Update the CR for endorsement

Deadline:  Short

=> Endorsed in R2-2507944

R2-2507944	Corrections to WI SBFD	Huawei, HiSilicon (Rapporteur)	CR	Rel-19	38.331	19.0.0	5499	1	F	NR_duplex_evo-Core

=> Endorsed

C100, C104, L701

R2-2506972	Discussion on RIL [C100][C104][L701]	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1: For RIL C100, in BeamfailureRecoveryConfig, support to add ‘or of the fallback CBRA’ in the field description of ra-OccasionType, instead of deleting ‘of CFRA’.

Proposal 2: For RIL C104, move the ra-OccasionType-r19 to be under CFRA field in RACH-ConfigDedicated, and to add ‘or of the fallback CBRA’ in the field description.

Proposal 3: For RIL L701, there is no need to restrict that rach-ConfigCommon and sbfd-RACH-DualConfig with same FeatureCombination should be provided in the same additionalRACH-Config.

R2-2507281	[L701][C100][C104] RIL issues on SBFD	LG Electronics Inc.	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1. [L701] Configure rach-ConfigCommon and sbfd-RACH-DualConfig in the same additionalRACH-Config IE, if the both rach-ConfigCommon and sbfd-RACH-DualConfig are configured for a specific FeatureCombination.

Proposal 3. [C100][C104] Clarify that ra-OccasionType in BeamFailureRecoveryConfig and RACH-ConfigDedicated IE indicates the second PRACH occasions for CFRA or for fallback case from CFRA to CBRA to be used by a SBFD aware UE.

For RIL C100, in BeamfailureRecoveryConfig, support to add ‘or of the fallback CBRA’ in the field description of ra-OccasionType, instead of deleting ‘of CFRA’.

For RIL C104, move the ra-OccasionType-r19 to be under CFRA field in RACH-ConfigDedicated, and to add ‘or of the fallback CBRA’ in the field description.

P3 in R2-2507281

-	Ericsson, Nokia agree with ZTE proposal.

-	Samsung, CATT, Xiaomi, Qualcomm, Apple think LG E proposal is ok.

If both rach-ConfigCommon and sbfd-RACH-DualConfig are configured for the same FeatureCombination, rach-ConfigCommon and sbfd-RACH-DualConfig are configured in the same additionalRACH-Config IE.

38.300

R2-2506820	Open issues in TS 38.300 on Rel-19 Evolution of NR duplex operation (SBFD)	CATT	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1: There is one open issue in TS 38.300: whether SBFD with CA operation is required in clause X.1 General.

Proposal 2: Add more cases of RO type selection to TS 38.300: 1) CFRA triggered by dedicated RRC signalling, 2) CBRA RO selection for initial PRACH transmissions. Take the CR in R2-2506823 as baseline.

R2-2506823	Introduction of Rel-19 Evolution of NR duplex operation (SBFD)	CATT	CR	Rel-19	38.300	18.6.0	1008	2	F	NR_duplex_evo-Core	R2-2506604

Will be updated using post meeting email discussion, can take into account proposals and TP in R2-2507002 and R2-2507364.

[Post131bis][214][SBFD] CR for TS 38.300 (CATT)

Intended outcome: Update the CR for endorsement

Deadline:  Short

R2-2507002	Discussion on issues for Stage-2 spec	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1: No need to add description on CA for stage-2 spec.

R2-2507364	Remaining issue for Stage 2 spec	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

Noted

R2-2507508	SBFD with CA for stage-2 spec	InterDigital, Inc.	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1: Support SBFD operation with CA scenario for stage-2 specification.

Proposal 2: Adopt the changes to stage-2 38.300 specification provided in the appendix.

P1in R2-2507508

-	Qualcomm do not think we should discuss this in R2, and think R1 can discuss whether to add it or not.

-	Ericsson think it is good to capture system wise aspect in stage 2. Ericsson think we could provide some wording regarding CA+SBFD and then let R1 check. Qualcomm think this should be discussed in R1.

-	ZTE explains that CA and SBFD are agreed by R1 and think there is no stage 3 impact to R2 spec. OPPO agree. ZTE support P1.

-	CATT do not support to capture this, and think the proposed change i.e., ‘a carrier’ from InterDigital is not so clear. Qualcomm think more clarification from R1 may be useful, and think technically it is supported to configured SBFD on more than one carriers.

-	InterDigital think R2 is responsible to stage 2.

-	OPPO wonder if SBFD is configured for CA, is there any restriction that SBFD is only configured for one of the carriers. Samsung think there is such restriction. Huawei think there is some ambiguity.

-	CATT think it is useful to send LS to R1. Huawei think we can ask R1 to provide the TP.

Send LS to R1, ask R1 to provide TP regarding the combination of CA and SBFD.

[AT131bis][206][SBFD] LS to R1 and R4 on SBFD and CA (InterDigital)

Intended outcome: Approve the LS via email (to R1, cc R4) in R2-2507746.

Deadline: before end of the Meeting

R2-2507746	LS on SBFD and CA	RAN2	LS out	Rel-19	NR_duplex_evo-Core	To: RAN1	Cc:RAN4

The LS is approved.

R2-2506972	Discussion on RIL [C100][C104][L701]	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2507002	Discussion on issues for Stage-2 spec	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

R2-2507267	Other Aspects of SBFD	Nokia 	discussion	Rel-19	NR_duplex_evo-Core

R2-2507281	[L701][C100][C104] RIL issues on SBFD	LG Electronics Inc.	discussion	Rel-19	NR_duplex_evo-Core

R2-2507364	Remaining issue for Stage 2 spec	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

R2-2507507	RRC issues on SBFD	InterDigital, Inc.	discussion	Rel-19	NR_duplex_evo-Core

R2-2507508	SBFD with CA for stage-2 spec	InterDigital, Inc.	discussion	Rel-19	NR_duplex_evo-Core

R2-2507518	Discussion on SBFD RRC open issues	Xiaomi	discussion	Rel-19	NR_duplex_evo-Core

R2-2507577	Discussion on RRC Remaining issues for SBFD	vivo	discussion	Rel-19	NR_duplex_evo-Core

## 8.12	NR MIMO Phase 5

(NR_MIMO_Ph5-Core; leading WG: RAN1; REL-19; WID: RP-242394)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

### 8.12.1	Organizational

LSs and rapporteur input, etc.

R2-2506738	LS on event triggered L1-RSRP reporting if eventDetectionTimeWindowLength-r19 is configured (R4-2512232; contact: Qualcomm)	RAN4	LS in	Rel-19	NR_MIMO_Ph5-Core	To:RAN1	Cc:RAN2

Noted

38.321

R2-2507497	Report of Rel-19 MIMO MAC open issues for maintenance	Samsung	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

During FR2 UL gap, the UE performs PUCCH transmission for UE Initiated Report Indication (for both mode-A and mode-B) and transmission of CG Type 1 for mode-B UE-initiated CSI reporting. Consider the proposed TP as baseline.

To determine DRX Active Time at symbol n, UE considers UE Initiated Report Indication sent for mode-A UE-initiated CSI reporting until 4 ms prior to symbol n when evaluating all DRX Active Time conditions. Adopted the proposed TP.

The existing Aperiodic CSI Trigger State Subselection MAC CE is used for CSI trigger state subselection for UEI-CSI reporting. Adopted the proposed TP.

for UEI reporting, MAC does not consider overlapping handling for 1) mode-A/B PUCCH resource overlapping with other resources and 2) mode-B PUSCH resource overlapping with other resources. No MAC impact.

[Post131bis][215][MIMOevo] CR for TS 38.321 (Samsung)

Intended outcome: Update the CR for endorsement

Deadline:  Short

38.331

R2-2507593	Review file for MIMO ASN.1 review	Ericsson	discussion	Late

Noted

R2-2507594	Comment file for MIMO ASN.1 review	Ericsson	discussion	Late

Noted

R2-2507592	Corrections for MIMO Phase 5	Ericsson	CR	Rel-19	38.331	19.0.0	5548	-	F	NR_MIMO_Ph5-Core	Late

Endorsed

[Post131bis][216][MIMOevo] CR for TS 38.331 (Ericsson)

Intended outcome: Update the CR for endorsement

Deadline:  Short

=> Endorsed in R2-2507938

R2-2507938	Corrections for MIMO Phase 5	Ericsson	CR	Rel-19	38.331	19.0.0	5548	1	F	NR_MIMO_Ph5-Core

=> Endorsed

### 8.12.2	MAC issues

Remaining MAC issues

UE behavior for the different caes, a) the TAT for mode-B PUSCH is expired while the TAT for PUCCH is running, and b) if the TAT for mode-B PUCCH is expired while the TAT for PUSCH is running, clears the CG for UEI

R2-2506847	Discussion on remaining MAC issue	China Telecom	discussion	NR_MIMO_Ph5-Core

Noted

Proposal 1[Issue-1]: Prefer Option-1, i.e., release the associated PUCCH. And it is OK with both TPs.

R2-2507021	Discussion on MAC open issues for UEI BMR	vivo	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1	Upon STAG TAT expiry associated with a SCell configured with a UEI report configuration with Mode B, if the PUCCH resource of the UEI report configuration is configured on a PCell or PUCCH-SCell of a different TAG with a running TAT, the UE:

Option 2a: does not release the PUCCH resource. If a UEI beam report is triggered for the SCell, the UE transmits the PUCCH and does not transmit the PUSCH for the UEI beam report.

Option 2b: does not release the PUCCH resource. If a UEI beam report is triggered for the SCell, the UE does not transmit the PUCCH for the UEI beam report.

R2-2507199	Remaining MAC issues in MIMO	Ofinno	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1	For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for Type1 CG PUSCH is expired while the TAT for PUCCH is running, RAN2 to discuss and down-select one of the following options:

Option 1: UE releases the PUCCH. (Adopt the TP in the Appendix 1).

Option 2: UE does not release the PUCCH and not transmit the UEIRI on the PUCCH. (Adopt the TP in the Appendix 2).

Proposal 1	For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for Type1 CG PUSCH is expired while the TAT for PUCCH is running, RAN2 to discuss and down-select one of the following options:

Option 1: UE releases the PUCCH. (Adopt the TP in the Appendix 1).

Option 2: UE does not release the PUCCH and not transmit the UEIRI on the PUCCH. (Adopt the TP in the Appendix 2).

Discussions

P1

-	Qualcomm think PUCCH is specifically configured for this feature so no need to keep the resource.

-	ZTE think there is no need to release, and think there is no need to change the specification if we go with Option 2.

-	Nokia, Ericsson, Asustek, Huawei prefer O1. Ericsson think we need to clarify the UE behavior in the spec.

-	LG E support O2 and think O1 is just optimization. ZTE agree. LG E think the PUCCH resource could be shared btw HARQ ACK/NACK and UEIBM. Asustek not sure if such resource configuration is reasonable.

-	OPPO think UE still transmit on PUCCH and think NW implementation can disable the transmission in PUCCH.

-	Ofinno think we should just go with the O1 since it get majority’s support.

-	ZTE think NW will reconfigure anyway because BM is important, so no need to require UE to release the PUCCH automatically. Ofinno think from UE perspective it is a waste to transmit in this case.

For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for PUCCH is expired while the TAT for Type1 CG PUSCH is running, the UE clears the Type1 CG PUSCH. (Adopt the TP in the Appendix 3 as baseline).

UEIRI on PUCCH if the BWP configured with mode-B PUSCH is deactivated

R2-2507154	[Issue-2] Discussion on MAC remaining issue for MIMO	SHARP Corporation	discussion	NR_MIMO_Ph5-Core

Noted

Proposal 1: UE does not transmit a PUCCH for beam measurement report notification in Mode B if the UL BWP for the associated PUSCH is deactivated.

R2-2507600	Consideration on the Remaining MAC Issues of UEIBM	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 2: If the BWP carrying the Mode-B CG PUSCH is deactivated, the UE shall not transmit the UEI Report Indication on PUCCH. RAN2 is to confirm whether this should be specified as a defined behavior or left to network implementation to avoid.

Discussion

P2

-	ZTE, CATT, Nokia think it is up to NW implementation.

-	QC think this is an error case and think NW should ensure this does not happen.

-	Ofinno think it is not reasonable to always require the BWP for UEIBM to be activated, because there may be need to schedule other transmission on other BWPs.

-	Ericsson think we need to clarify the UE behavior in the spec.

If the BWP/SCell carrying the Mode-B CG PUSCH is deactivated, the UE shall not transmit the UEI Report Indication on PUCCH.

TAT expired for any TCI state of multi-panel SDM for sDCImTRP 2TA

R2-2506941	Discussion on remaining MAC issues	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 3 [Issue-5]: If UE is configured with sDCI mTRP two TA and UL multi-panel transmission with SDM mode, UE clears the CG resource if at least one TCI state indicated by the DCI for the CG resource is associated with the expired TAT.

R2-2507199	Remaining MAC issues in MIMO	Ofinno	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 4	When a serving cell is configured with multipanelSchemeSDM and with two TAGs, to avoid unsuccessfully decoding due to only partial PUSCH transmission/MIMO layer/TRP has valid TA, RAN2 to discuss and down-select one of the following options:

•	Option 1: UE clears any CG/PUSCH for SP-CSI if any of the two activated TCI states for the CG/PUSCH for SP-CSI is associated with a TAG of an expired TAT. (Adopt the TP in the Appendix 5).

•	Option 2: UE does not transmit any CG/PUSCH for SP-CSI if any of the two activated TCI states for the CG/PUSCH for SP-CSI is associated with a TAG of an expired TAT. (Adopt the TP in the Appendix 6).

R2-2507498	MAC open issues	Samsung	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 3: For multipanelSchemeSDM, UE clear the CG/PUSCH for SP-CSI if all of the activated TCI state(s) for the CG/PUSCH for SP-CSI is associated with the TAG(s) of the expired TAT(s). This is aligned with legacy behaviour; no change is needed for this.

Proposal 4: For multi-panel SDM scheme, NW should ensure to indicate in DCI both two TCI states with running TAT(s) for multipanelSchemeSDM, so that UE can use the indicated TCI states to transmits all MIMO layers of the TB. Discuss whether to specify that “UE does not transmit any uplink transmission via multi-panel SDM scheme if any TCI state to be applied for the multi-panel SDM transmission is associated to a TAG for which the TAT is expired.”

Discussion

-	ZTE think O1 in Ofinno P4 is preferable.

-	Qualcomm not sure if this issue is real. And think if we need a solution then P1 is reasonable.

-	Ofinno think it is not easy for NW to always ensure like proposed by Samsung P4.

-	Samsung do not agree with O1 from Ofinno P4.

DRX active time for mode-A UEI reporting

R2-2506941	Discussion on remaining MAC issues	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 4 [Issue-10]: For mode-A UEIBR, keep the current wording in the MAC spec that UE considers the DRX as active time while a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH has not been received after transmitting UE Initiated Report Indication on PUCCH.

R2-2507265	MAC Open issues	Nokia 	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 2:  If a PDCCH scheduling a mode-A UEI CSI report is not received after transmission of UEIRI, UE stays in the active state until the next PUCCH ressource for transmitting UE Initiated Report Indication. No MAC spec change.

R2-2507209	Remaining issues on UEI beam reporting	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 3. For Mode-A, RAN2 introduce a new timer to handle Active time based on UEI reporting.

Proposal 4. For Mode-A, the new timer is started upon UEIRI transmission and is stopped upon expiry of the timer.

Discussions

P3 in R2-2507209

-	CATT think for legacy SR we do not have similar mechanism. LG E think these are different cases.

-	Asusteck agree with P3.

-	Ericsson wonders what is the new timer and how does it work.

P2 in R2-2507265

-	Asusteck think this is fine but we need to change the MAC spec.

-	CATT also ok with P2.

-	Ericsson support as well.

-	LG E wonders what is the UE behaviour on the next PUCCH resource. Nokia think UE will transmit based on the trigger, nothing new.

-	LG E has concern if NW configured a very short PUCCH periodicity, the granularity could be very short (such as on the slot level), and in this case not sure how it works.

[AT131bis][203][MIMOevo] Remaining MAC issues (Samsung)

Intended outcome: Proposals in R2-2507734 for the remaining MAC issues.

Deadline: before Friday CB

R2-2507734	[AT131bis][203][MIMOevo] Remaining MAC issues	Samsung	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1: For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for Type1 CG PUSCH is expired while the TAT for PUCCH is running, and if the configured PUCCH resource id is only associated to this type-1 CG PUSCH, UE releases the PUCCH.

Discussion in CB

P1

-	ZTE has concern for the case if this is the only PUCCH that UE gets, and if the resource is released then there is no way to report ACK/NACK. OPPO, Ofinno, Nokia, vivo think this is not typical case so this issue mentioned by ZTE is not so critical.

-	ZTE want to check with R1. Vivo think we can just check internally so no need to send a LS.

-	Question from HW: what is the meaning of ‘only associated to this type-1 CG PUSCH’

-	Question from ZTE: how should UE decide whether a PUCCH is ONLY associated to type-1 CG PUSCH

- ?? Baseline, can confirm in the next meeting: For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for Type1 CG PUSCH is expired while the TAT for PUCCH is running, and if the configured PUCCH resource id is only associated to this type-1 CG PUSCH (i.e., not associated with any other UCI), UE releases the PUCCH.

(Update the agreement on Monday): For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for PUCCH is expired while the TAT for Type1 CG PUSCH is running, and if the Type1 CG is only associated to this PUCCH for mode-B UEI reporting, the UE clears the Type1 CG PUSCH.

When a serving cell is configured with multipanelSchemeSDM and with two TAGs, to avoid unsuccessfully decoding due to only partial PUSCH transmission/MIMO layer/TRP has valid TA, UE does not transmit any PUSCH if any TCI states used for multi-panel SDM is associated to an expired TAT. Specify this UE behavior in MAC.

Proposal 4: For DRX, after transmitting UEIRI on PUCCH, select one option:

Option 1 (5/10): UE stays in active time until receiving PDCCH scheduling. Keep the current MAC spec.

Option 2 (5/10): UE stays in active time until the next PUCCH resource occasion. Update MAC spec.

Discussion in CB

P4

-	Nokia think O1 will extend the UE actively which is not needed, so prefer O2. Ofinno also support O2. Asustek think part of the reason is that coapneis have different views regarding how PUCCH is handled in this case, so perhaps we should check with R1 (internally if not sending a LS. )

Chair: P1/P4 can be discussed in the next meeting, companies are encouraged to check internally with R1 if needed.

R2-2506847	Discussion on remaining MAC issue	China Telecom	discussion	NR_MIMO_Ph5-Core

R2-2506906	Discuss on MIMO MAC issues	CMCC	discussion	Rel-19	NR_MIMO_Ph5-Core	Withdrawn

R2-2506941	Discussion on remaining MAC issues	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2507021	Discussion on MAC open issues for UEI BMR	vivo	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2507154	[Issue-2] Discussion on MAC remaining issue for MIMO	SHARP Corporation	discussion	NR_MIMO_Ph5-Core

R2-2507199	Remaining MAC issues in MIMO	Ofinno	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2507209	Remaining issues on UEI beam reporting	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2507265	MAC Open issues	Nokia 	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2507377	MAC issues for MIMO	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2507498	MAC open issues	Samsung	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2507539	Discussion on MIMO MAC open issues	ASUSTeK, Ofinno, Ericsson	discussion	Rel-19	38.321	NR_MIMO_Ph5-Core

R2-2507600	Consideration on the Remaining MAC Issues of UEIBM	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

### 8.12.3	Others

Issues related to RILs, other remaining RRC issues, Changes to Stage 2, and other issues if not covered by the previous agenda items

Coexistence of UL-only TRP and (C)-LTM (H403)

R2-2507376	RRC issues for MIMO	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core	Late

Noted

Proposal 4: Confirm that, when the TA is already acquired, it is possible to perform an initial (RACH-less) uplink transmission to a cell configured with intra-cell asymmetric DL sTRP/UL mTRP using a pre-stored PL offset for the TCI state used.

Proposal 5: Leave it to the mobility enhancement session (whether) to specify the necessary signalling.

R2-2506852	Clarification on the coexistence between LTM or CLTM and UL-only TRP	OPPO	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

LTM

Proposal 1: For both Rel-18 LTM and Rel-19 LTM, the configuration (including pathloss offset) of the asymmetric DL sTRP/UL mTRP can be included in the LTM candidate configuration, without adding the pathloss offset in (enhanced) LTM Cell Switch Command MAC CE.

Proposal 2: The UL-only TRP is not used for the early RACH procedure of LTM. No extra specification change is needed.

C-LTM

Proposal 3: The gNB by implementation can include the configuration (including pathloss offset) of the asymmetric DL sTRP/UL mTRP in the candidate configuration of C-LTM. No extra specification change is needed.

Proposal 4: The UL-only TRP is not used for the early RACH procedure for C-LTM. No extra specification change is needed.

R2-2507605	[Z408][K103][H400][H403][Z409]RIL Issues for MIMO	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 2: No need to include the PL offset in the LTM Cell Switch Command.

Proposal 3: RAN2 should first clarify the usage scenario for configuring the PL offset for the candidate cell, and subsequently determine whether and how to include it.

R2-2507499	RIL S001 H403	Samsung	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 2: Not pursue H403, i.e., pathloss offset is not supported for LTM candidate TCI states.

Discussion

-	Qualcomm do not see it a valid case, because the PL offset is configured when there is no DL measurement from the UL only TRP. CATT agree.

-	Ericsson and Qualcomm support Samsung proposal.

H403 is not pursed.

On servCellIndex for mode-B CG (Z408, K103, H400)

R2-2507376	RRC issues for MIMO	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core	Late

Noted

Discussion

-	HW clarifies their proposal is to ‘Update the description of the CSI-ReportConfig IE according to the TP in 2.1.’

The changes proposed by P1 is generally agreeable, can refine the wording (e.g., details regarding mode A / mode B) in the post meeting email discussions.

R2-2507565	Discussion on remaining RRC issue	China Telecom	discussion

Noted

Proposal 1: servCellIndex-r19 in CSI-ReportConfig -> reportTransmissionMode-r19 -> pusch-ResourceOfModeB-19 can be optional field.

Discussion

-	Ofinno prefer to remove servCellIndex for mode-B CG, ZTE also agree. Nokia fine with removing but think it may be useful to inform R1 about the agreement.

-	Asustek think we need some clarification in the description.

Remove servCellIndex-r19 in modeB-r19 in CSI-ReportUE-IBR-r19.

R2-2507540	Discussion on RIL [K103]	ASUSTeK	discussion	Rel-19	38.331	NR_MIMO_Ph5-Core

Noted

K103 is not pursued.

Proposal 1:  (RIL-K103) Use a single parameter ConfiguredGrantConfigIndexMAC to indicate Type-1 CG PUSCH resource for the PUSCH transmission in mode-B UE-initiated beam reporting.

S001

R2-2507499	RIL S001 H403	Samsung	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1: For S001, add in field description that cri-TypeI-SinglePanelRI-Restriction-r19 and cri-TypeI-SinglePanelN1-N2-CBSR-r19 are optionally present if codebookType is set to typeI-SinglePanel, cri-TypeII-RI-Restriction-r19 and cri-TypeII-N1-N2-CBSR-r19 are optionally present if codebookType is set to typeII-r16. They are not configured together with codebookType in codebookConfig-r19.

For S001, add in field description that cri-TypeI-SinglePanelRI-Restriction-r19 and cri-TypeI-SinglePanelN1-N2-CBSR-r19 are optionally present if codebookType is set to typeI-SinglePanel, cri-TypeII-RI-Restriction-r19 and cri-TypeII-N1-N2-CBSR-r19 are optionally present if codebookType is set to typeII-r16. They are not configured together with codebookType in codebookConfig-r19.

Z409

R2-2507605	[Z408][K103][H400][H403][Z409]RIL Issues for MIMO	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 4: Send an LS to RAN1 to confirm whether there is a need to define additionalOneSlotOffsetDoppler-r19 as a list.

N122, C252, C253

R2-2507549	Stage 2 and RRC aspects	Nokia	discussion	Rel-19	NR_MIMO_Ph5-Core	Late

Noted

Proposal 3: [N122] Generalize and use the same field description for pathlossOffset under TCI-State and TCI-UL-State IEs, clarifying that the field applies to TCI states of UL-only TRPs; use existing conditional presence tag “JointTCI” in TCI-State IE to distinguish the joint TCI state case.

Generalize and use the same field description for pathlossOffset under TCI-State and TCI-UL-State IEs, clarifying that the field applies to TCI states of UL-only TRPs; use existing conditional presence tag “JointTCI” in TCI-State IE to distinguish the joint TCI state case.

H401

R2-2507376	RRC issues for MIMO	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core	Late

Noted

Proposal 3: Ask RAN1:

-	whether RAN1 will capture the UE behaviour for the case where a report is triggered while there are no resources for UEIRI transmission or mode-B transmission in the active BWP;

-	whether there is a particular reason why the resources for UEIRI transmission and for mode-B transmission on PUSCH are in a single BWP (unlike Rel-15 CSI reporting on PUCCH), which could make the above scenario difficult to avoid.

Stage 2

R2-2507549	Stage 2 and RRC aspects	Nokia	discussion	Rel-19	NR_MIMO_Ph5-Core	Late

Noted

Proposal 1: Clarify the asymmetric TRP scenario in TS 38.300 according to the text proposal in Annex A.

Discussions

-	CMCC think UL TRP is used in WID, and also think there is the case when the TRP is sending SSB so no need to change to UL only TRP.

Discussions on CB

-	CMCC explain that after some offline checking, perhaps we do not change the current spec. Nokia explain that these exiting descriptions are already after discussions in R1 and were compromise there already.

R2-2506852	Clarification on the coexistence between LTM or CLTM and UL-only TRP	OPPO	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2507376	RRC issues for MIMO	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core	Late

R2-2507499	RIL S001 H403	Samsung	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2507540	Discussion on RIL [K103]	ASUSTeK	discussion	Rel-19	38.331	NR_MIMO_Ph5-Core

R2-2507549	Stage 2 and RRC aspects	Nokia	discussion	Rel-19	NR_MIMO_Ph5-Core	Late

R2-2507565	Discussion on remaining RRC issue	China Telecom	discussion

R2-2507605	[Z408][K103][H400][H403][Z409]RIL Issues for MIMO	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2507657	[K103][H402] Discussion on RIL related issues	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

## 8.13	NR sidelink multi-hop relay

(NR_SL_relay_multihop; leading WG: RAN2; REL-19; WID: RP-250188)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

### 8.13.1	Organizational

LSs and rapporteur input

SRAP open issues and rapporteur CR

R2-2506805	SRAP open issues for NR sidelink multi-hop relay	OPPO	report	NR_SL_relay_multihop

Noted

R2-2507150	Miscellaneous SRAP corrections for multi-hop U2N Relay	OPPO, ASUSTeK	CR	Rel-19	38.351	19.0.0	0042	-	F	NR_SL_relay_multihop

[Post131bis][402][Relay] Rel-19 relay SRAP CR (OPPO)

Scope: Update the CR in R2-2507150 in line with decisions of RAN2#131bis.

Intended outcome: Endorsable CR to RAN2#132

Deadline: Long

38.304 open issues

R2-2507183	Open issues on Rel-19 multihop relay 38.304 CR	MediaTek Inc.	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

MAC open issues

R2-2507455	MAC Open Issues Discussion	InterDigital France R&D, SAS	discussion

Noted

Capability open issues

R2-2507559	Open issues on Rel-19 Relay Capability	Samsung	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal: The remote UE operation is not a prerequisite of the relay UE operation, and no further specification change is needed.

Agreement:

The remote UE operation is not a prerequisite of the relay UE operation, and no further specification change is needed.

ASN.1 review and RRC rapporteur CR

K002, W500, and W501 are PropReject  but with related documents under AI 8.13.2

E049, E050, and H453 are PropAgree and implemented in the CR but have related documents under AI 8.13.2

R2-2507488	Corrections to WI SLRelay	Huawei, HiSilicon (Rapporteur)	CR	Rel-19	38.331	19.0.0	5537	-	F	NR_SL_relay_multihop-Core	Late

[Post131bis][403][Relay] Rel-19 relay RRC CR (Huawei)

Scope: Update the CR in R2-2507488 in line with decisions of RAN2#131bis.

Intended outcome: Endorsed CR (to be taken into ASN.1 review)

Deadline: Short

=> Endorsed in R2-2507943

R2-2507943	Corrections to WI SLRelay	Huawei, HiSilicon (Rapporteur)	CR	Rel-19	38.331	19.0.015537	2	F	NR_SL_relay_multihop-Core

=> Endorsed

R2-2507489	WI SLRelay ASN.1 Review file	Huawei, HiSilicon (Rapporteur)	discussion	Rel-19	NR_SL_relay_multihop-Core	Late

Noted

R2-2507490	WI SLRelay ASN.1 Comments file	Huawei, HiSilicon (Rapporteur)	discussion	Rel-19	NR_SL_relay_multihop-Core	Late

Noted

### 8.13.2	Control plane

Impact to 38.331 (except for capability issues), 38.304

RIL documents (prioritise ToDo RILs)

Note: J011, X500 are ToDo without documents

O503/O505/O508

R2-2506804	Discussion on control plane correction for multi-hop U2N relay	OPPO	discussion	Rel-19	NR_SL_relay_multihop

Noted

Proposal 1	(O505) RAN2 to discuss whether to introduce sl-PagingDelivery-List to enable the Paging delivery of multiple remote UEs within the same UuMessageTransferSidelink message.

Discussion:

Qualcomm think it is an optimisation that only applies to a rare case (when multiple remote UEs are paged at exactly the same time).

Apple think from a signalling perspective the proposal is harmless, and there may be many child UEs resulting in a reasonable probability of simultaneous paging.

NEC agree with Qualcomm and think simultaneous paging would be unusual, and the proposal costs some specification effort.

Huawei agree that it is an optimisation.

Proposal 2	(O508) Add the missing discovery transmission threshold condition as proposed in O508.

Proposal 3	(O503) Add the missing conditions of SUI initiation for discovery transmission at the intermediate relay and last relay as proposed in O503.

Discussion:

Huawei wonder why the intermediate relay UE needs to check the remote UE threshold condition for discovery, since it will check anyway when it connects.  OPPO agree that it will check when it connects, but they understand that it should be checked also for every discovery transmission based on Rel-17 principles.

OPPO clarify that this aligns with the agreement to reuse the remote UE threshold value for the intermediate relay UE threshold.

Huawei wonder if the last relay UE really needs to check if it has a PC5 connection with the child.  OPPO indicate that it is not checking this condition for transmission, but it creates two different cases.

Agreements:

Do not introduce sl-PagingDeliveryList in this release.  O505 can be closed.

Add the missing discovery transmission threshold condition (as in issue O508).  Refer to TP in R2-2506804.  O508 can be closed.

Add the missing conditions of SUI initiation for discovery transmission at the intermediate relay and last relay (as in issue O503).  Refer to TP in R2-2506804.  O503 can be closed.

B100/B101/B102

R2-2506925	[B100][B101][B102] issues for notification message	Lenovo	discussion	Rel-19

Noted

Proposal 1: The case that the intermediate relay UE in idle/inactive can trigger a notification message after receiving release message from its parent has been agreed in RAN2#131 meeting. But it is not captured in RRC specification. RAN2 to agree TP#1 for this case.

Discussion:

Huawei indicate that so far we have not mixed upper-layer and AS-layer conditions; they understand that the intermediate relay UE can release the child UE in upper layers without an AS trigger for the message.

InterDigital wonder if we should then revert the agreement to send the notification message.

Lenovo understand that Huawei’s interpretation would restrict the intermediate relay UE behaviour to always release the child, which would change the agreement and need to be documented in the spec.

NEC recall that we sent the agreement to SA2 for confirmation but think we did not hear back, and they think we should keep the agreement unless told otherwise.  InterDigital think the agreement aligns with other upstream failure cases.

Proposal 2: The intermediate relay UE in RRC connected state can be triggered to transmit the notification message towards the child UE after receiving the release message from its parent relay UE. RAN2 to agree TP#2.

Proposal 3: The intermediate relay UE can be triggered to transmit the notification message towards the child UE upon intermediate Relay UE's RRC connection failure including RRC connection reject as specified in 5.3.3.5 and 5.3.13.10, and T300 expiry as specified in 5.3.3.7, and RRC resume failure as specified in 5.3.13.5. RAN2 to agree TP#3.

Discussion:

Huawei think there is no impact from this case to Uu resources, so they wonder if the network will really reject the RRC connection.  Lenovo indicate that from the intermediate perspective it will establish an end-to-end connection, and the network is allowed to reject, e.g., for load reasons.

Apple support the proposal and think we should document the case since the UE cannot get into connected state.  OPPO have the same understanding.

Agreements:

Capture in RRC that the intermediate relay UE in idle/inactive can trigger a notification message after receiving release message from its parent, as in issue B100.  Refer to TP in R2-2506925.  B100 can be closed.

The intermediate relay UE in RRC connected state can be triggered to transmit the notification message towards the child UE after receiving the release message from its parent relay UE, as in issue B101.  Refer to TP in R2-2506925.  B101 can be closed.

The intermediate relay UE can be triggered to transmit the notification message towards the child UE upon intermediate Relay UE's RRC connection failure including RRC connection reject as specified in 5.3.3.5 and 5.3.13.10, and T300 expiry as specified in 5.3.3.7, and RRC resume failure as specified in 5.3.13.5, as in issue B102.  Refer to TP in R2-2506925.  B102 can be closed.

Z454/Z456/Z455

R2-2506983	Discussion on RIL [Z452][Z454][Z455][Z456][Z458][Z459]	ZTE Corporation, Sanechips	discussion	Rel-19	NR_SL_relay_multihop

Noted

Proposal 2.	[Z454]MH remote UE will report it’s own SRC L2 ID to network. And adopt the TP in annex clause.

Proposal 3.	[Z455]Introduce a paging ID list in SL-TxResourceReqL2U2N-Relay-r17 included in SUI message.

Proposal 4.	[Z456]RAN2 is suggested to discuss below two solutions for UE to set UE type in SUI message:

Solution1: Clarify that intermediate relay UE will use different L2 IDs for remote UE discovery function and relay UE discovery function, which may need double check with SA2.

Solution2: It is possible that UE use same L2 ID for remote UE and relay UE discovery, a new UE type is needed.

Discussion:

ZTE clarify that P4 can focus on whether there is a need to introduce a new UE type for the intermediate relay.

Qualcomm think P3 is not needed; the SUI will only include the direct child remote UE.  NEC have the same understanding.  ZTE think if the relay and remote are both in idle/inactive, the last relay UE will report both paging IDs.

NEC understand that each UE reports its own paging ID through its own SUI.  OPPO understand that the relay UE reports the paging ID for the remote UE, and the connected UE does not need to report its own paging ID.

OPPO understand the list is needed since currently we can only report a single destination.

Qualcomm consider that each intermediate relay UE reports only its direct child(ren).  ZTE ask if there is a two-hop case where the remote and intermediate are both in idle, which UE is responsible for receiving the paging message for the remote UE, the intermediate or last relay UE?  Qualcomm think this goes to the question of whether the intermediate relay UE can monitor paging directly.  ZTE consider that the last relay UE may need to know the paging ID of the remote UE in order to monitor its paging.

Qualcomm think it can be done with multiple SUI messages.

Apple think there is a difference between Qualcomm’s discussion of the intermediate relay UE and ZTE’s proposal for the last relay UE; they understand that for the last relay UE, ZTE’s proposal is needed so it can know the paging ID of the downstream remote(s).

NEC have concerns on the specification impact; we have not made any changes within the SUI, and we would need to have a new Rel-19 field for this case with corresponding procedural text.

Qualcomm would prefer to postpone the issue for more offline discussion on how paging monitoring works.  Huawei think this discussion was on a different issue.

ZTE think multiple SUIs will not work because the UE only reports its direct child(ren), and a new SUI replaces the old SUI from network perspective.  Qualcomm wonder what we do then about multiple remote UEs for the same relay UE in legacy operation; ZTE clarify that each SUI needs to include all the children.  OPPO think we discussed this previously and excluded reporting of indirect child UEs, so Qualcomm’s suggestion would require additional PC5-RRC signalling.  Apple agree with OPPO and see ASN.1 impact from Qualcomm’s proposal.

Qualcomm wonder if the ZTE proposal includes only paging ID or also other information.  ZTE clarify that it is just the paging ID of indirect child UEs.

Xiaomi think on P4, the current spec is clear that the L2IDs for remote and relay operation will be different, and no need to check with SA2.  Qualcomm understand that the intermediate relay UE will set the appropriate role for whatever role it is acting in at the moment (remote or relay).

ZTE clarify that the UE type is in the root of the SUI message, so each message can only report one UE type.  Xiaomi think it will report different types in different instances of the SUI.

Apple think Xiaomi’s suggestion does not work because the network will drop the previous SUI.

NEC think a new UE type is needed and the impact is clear.

Qualcomm think we can leave it to intermediate relay UE implementation and the network will know which UE is an intermediate relay.

OPPO indicate that in previous releases, the UE role is used by the network to provide different configurations, e.g., the appropriate threshold conditions for the role, so they see that a new type is helpful.

ZTE think if we do not have a new UE type, we would have some spec impact to clarify that it is left to UE implementation.  Qualcomm agree that there would be something to document.

NEC think with no new UE type, we would have to have new discussions on when the intermediate acts as a remote and when it acts as a relay.

Ericsson have the same understanding as Qualcomm and think the gNB will distinguish the two roles; they see the proposal as an optimization.

OPPO understand that both options are feasible for the network, explicit indication with a new UE type or implicit solution where the network relies on knowledge that the UE is an intermediate relay.

Xiaomi think the current spec works and a new UE type would have SA2 impact.  ZTE do not see the SA2 impact.

Chair understands that from network implementation perspective, it is a question of whether the SUI always replaces the previous version.  ZTE understand that it does, under general principles of uplink messages.  Apple have the same understanding as ZTE; we do not have delta signalling in the uplink.

Qualcomm think even if the SUI overrides previous versions, a network-based solution can work, and we already expect the network to remember some information from SUIs.

Ericsson think we could add a NOTE to describe the expected network behaviour; they see that introducing a new UE type is a bigger spec change.

Agreements:

MH remote UE will report its own SRC L2 ID to network as in issue Z454.  Refer to TP in R2-2506983.  Z454 can be closed.

Introduce a paging ID list in SL-TxResourceReqL2U2N-Relay-r17 included in SUI message, used to report paging IDs of indirect child UEs at least for last relay UE, as in issue Z455.  Refer to TP in R2-2506983.  Z455 can be closed.  FFS (related to A500) whether the intermediate relay UE in RRC_CONNECTED can monitor paging for child UEs.

Z456 can be left to UE implementation to set the UE type and network implementation to identify which UE is an intermediate.

X501/X502/X503/X505 (last three are PropReject)

R2-2506994	Discussion on Rel-19 SL MH-Relay RILs	Xiaomi	discussion	Rel-19	NR_SL_relay_multihop

Noted

Proposal 1: (X501) To clarify that the understanding of “UE has SIB request information to provide” also includes the requested SIB(s) from the child UE, RAN2 is requested to agree to adopt the TP (first change) proposed in R2-2506994.

Discussion:

Apple understand the concern but think the problem is that we did not capture anything in the reception behaviour about the SIB forwarding, and they think something should be added in the SIB request reception instead of in the definition of “SIB request information”.

Xiaomi think both solutions can resolve the issue and they are open to either.

OPPO slightly prefer the option from Apple, which can address the paging equivalent issue as well.

Agreement:

For paging and system information forwarding, describe the triggering of RemoteUEInformationSidelink when the request from a child UE is received.  X501 can be closed (captured in this agreement).  Details to be checked in CR implementation/ASN.1 review.

Proposal 2: (X502) To avoid confusion regarding “paging information” when handling actions related to the transmission of the RemoteUEInformationSidelink message, RAN2 to clarify “paging information” as “paging UE ID/paging cycle” and agree to adopt the TP (second change) proposed in R2-2506994.

Discussion:

Huawei understand that the term “paging information” is used in two contexts at least since Rel-17, and they do not see a need to change it now.  Apple think this is an overloaded term and we could fix it from Rel-17.  Huawei think it is clear in the context of each procedure.

Agreement:

X502 is not captured in the CR now, but checking can continue in the ASN.1 review if there is interest.

Proposal 3: (X503) RAN2 to agree to adopt the TP (third change) proposed in R2-2506994, which revises Figure 5.8.9.8.1-1 to support the transmission of the RemoteUEInformationSidelink message to both the remote UE and the child UE.

Discussion:

Huawei think we have generally reused the figures without changing them, because the intermediate/child UEs will in any case function also as remote UEs.

Apple think this is a forwarding behaviour, and it would make sense to add “child UE” in the notation.

ZTE support the change and think it is about the logical functions, not a physical function as described by Huawei’s comment.

Agreement:

Revise Figure 5.8.9.8.1-1 to support the transmission of the RemoteUEInformationSidelink message to both the remote UE and the child UE.  X503 can be closed.

Proposal 4: (X505) RAN2 is requested to agree to adopt the TP (fourth change) proposed in R2-2506994, which removes the prefix of the “mh-indicationType” IE.

Discussion:

Huawei think the change is incorrect because there should be a distinct name from the legacy field.

OPPO understand it is a procedural/ASN.1 misalignment, since the procedural text refers to it as indicationType, and we should align.  Apple, ZTE, and OPPO understand that we aligned the procedural text to the new type in Rel-18.

Agreement:

Align the procedural text to use the name mh-IndicationType in the RemoteUEInformationSidelink message where appropriate.  X505 can be closed.

A500/O505/X501/H451

R2-2507103	ASN.1 issues for SI/Paging forwarding (A500/O505/X501/K002/H451)	Apple	discussion	Rel-19	NR_SL_relay_multihop

Noted

[Focus on A500 and related issues]

Proposal 2 	Specify RemoteUEInformationSidelink transmission behaviour for RRC_CONNECTED intermediate relay UE in TS 38.331.

Discussion:

Huawei think the changes related to SFN-DFN offset are not needed, because we did not adopt it for multihop so far.  Apple indicate this is related to P3 below, but they are combined in the TP and need to be teased apart.

OPPO think the SFN-DFN offset is not a concern, since the relay UE in multihop will not indicate support of forwarding the offset (pending TEI19 discussion).  Apple clarify that the intention is to have all the multihop behaviour in the newly added text at the end of the TP section.

Proposal 3 	Single-hop and Multi-hop SI/Paging omponent are to be separately specified in clarity in 5.8.9.8.2  for the transmission of RemoteUEInformationSidelink.

Proposal 4 	The triggering of the transmission of RemoteUEInformationSidelink is also described in respective subclauses when SL RLF occurs or when child UE’s request is received.

Agreement:

Specify RemoteUEInformationSidelink transmission behaviour for RRC_CONNECTED intermediate relay UE in TS 38.331.  Refer to second and third sections of the TP in R2-2507103.  A500 can be closed.

E044

R2-2507427	Discussion on RIL E044 and RIL E029	Ericsson	discussion	Rel-19	NR_SL_relay_multihop

Noted

Proposal 1	Include the condition in the note 4 that the note is only applicable to multi-hop L2 U2N Relay communication.

Proposal 2	Update the note 4 that the L2 U2N Remote UE may prioritize the selection or reselection of suitable NR sidelink U2N Relay UE based on the RRC state information in the discovery message.

Proposal 3	Adopt the TP for RIL E044 captured in clause 4.

Discussion:

Chair understands we agreed any information can be used, not just the RRC state.  Ericsson think the RRC state is clear and we can leave the rest to implementation.

Huawei think the NOTE currently is aligned with what we agreed.  Apple think we could say “e.g., RRC state”.  Ericsson would be OK with Apple’s suggestion.

Xiaomi could accept “e.g.”.  Toyota agree that we agreed to use any information, notably the hop count, so we should not make it specific to the state.

Toyota think the agreement of the last meeting was clear and the hop count is visible from the NAS message; they consider that the current text is already a compromise.

Agreement:

Include the condition in the note 4 that the note is only applicable to multi-hop L2 U2N Relay communication.  Do not otherwise update the text of the note.  E044 can be closed.

H455/H454/H452

R2-2507491	Discussion on Multi-hop Relay RILs [H452],[H454] and [H455]	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core	Late

Noted

[H455] How to restrict inter-gNB direct/single-hop to multi-hop indirect path switching

Proposal 1: Add an indication in the measurement report to indicate whether the candidate Relay UE is on a single-hop or multi-hop target path as indicate in the text proposal in the Annex 1.

Discussion:

NEC understand the intention and tend to agree with it, but they do not think the method is perfect because it does not capture the details of the hop count.  They would prefer to include hop count in the report.

LG think we already agreed that only RRC_CONNECTED UEs are candidates for the target, and the SUI should allow the network to know the hop count.  Huawei are concerned about the inter-gNB case, which can be supported for single-hop only.

Qualcomm think even for the intra-gNB case there can be a problem before the SUI is sent.

OPPO think there is no problem for the intra-gNB case because the target is necessarily RRC_CONNECTED per the WID.

Xiaomi agree with OPPO and think we just need to provide service continuity for the intra-gNB case.

Qualcomm indicate that in the intra-gNB case, the gNB cannot know if the target path is single-hop or multihop, so it can only select RRC_CONNECTED targets even though that restriction is not there for single-hop.

Lenovo support the proposal and think Qualcomm’s case is correct, as well as the situation of inter-gNB candidates where the serving gNB does not know if they are single- or multihop.

Toyota understand from the discussion that there is a problem to solve, and we should either take Huawei’s proposal or NEC’s; they see that NEC’s solution with the hop count is more consistent with the information that we already have.

OPPO wonder if hop count 1 means single-hop or multihop with only one link, in terms of the capability of the relay UE; they understand that a multihop relay acting as a last relay UE should not be considered for inter-gNB path switch.

Qualcomm think we could clarify this issue in the field description; for example, absent means single-hop, while hop count 1 means a multihop relay acting as a last relay UE.

LG think if the indication is introduced in the measurement report, it needs to be transferred to the target gNB.  Huawei indicate that the source can take the decision without passing the information to the target.

LG have some concern about a corner case where the target changes from single-hop to multihop during the procedure, but they think we can solve it somehow.  Toyota want to avoid having the same problem happen repeatedly.  Huawei think a relay reselection during the procedure will cause the target to change its discovery signalling and the remote will recognize the issue.

NEC note that the hop count in discovery is an optional parameter, and we have to figure out what to do if it is absent.

OPPO think a single-hop/multihop indicator is simpler, and for multihop, the serving gNB knows the hop count since everything is RRC_CONNECTED and intra-gNB.

CATT agree with OPPO and Xiaomi and think it is not in scope to address multihop inter-gNB scenarios.

Apple think this issue is in scope because it interferes with the intra-gNB solution.

Huawei indicate that the hop count is mandatory in Rel-19 discovery per CT1 spec.

OPPO think the hop count is not needed for RRC_CONNECTED intra-cell candidates, and they wonder if we should ask whether the upper layer can provide the RSC to AS layer to determine the hop status in case the hop count is omitted.

Agreement:

Add an indication in the measurement report to indicate the hop status of the candidate relay UE (single-hop or multihop); details (e.g., mandatory or optional, what to report when the hop count is absent) can be discussed in CR implementation/ASN.1 review.  H455 can be closed.

[H452] Condition for Discovery message transmission

Proposal 2: The UE should check the hop count condition against the configured hop limit before transmitting a discovery message as indicated in the text proposal in the Annex 2.

Discussion:

ZTE think it should be an SA2 decision, because we do not have a hop limit in AS.  Ericsson think we do not need to resolve this right away.

Qualcomm indicate that this criterion is already in SA2 spec.  LG think it is an upper layer decision.

Agreement:

Rely on SA2 to limit discovery message transmission above the hop count limit.  H452 can be closed.

[H454] Notification message from Intermediate Relay UE

Proposal 3: The intermediate relay UE in RRC_IDLE or RRC_INACTIVE state should send a notification to all its child UE(s) when the hop count of the target path increases, allowing the child UE(s) to determine whether to maintain the PC5 connection with the intermediate relay UE as indicated in the text proposal in the Annex 3

Discussion:

Apple wonder if this is specific to the path switch case.  They wonder if we need the notification in addition to discovery signalling.  Huawei explain that the hop count change will be seen by the relay UE first and it may be useful to notify the child(ren) before discovery.

Ericsson think we should not be considering an upper-layer metric so much in RAN2, and there could be other metrics like QoS that we do not look at.

NEC recall that we agreed that the remote UE can perform reselection according to the hop count, and they also think we can rely on the discovery signalling.

Apple think the proposal just reduces the latency of reselection for the remote(s) and it is not urgent to send the notification right away.

Huawei intend to clarify the NOTE that says when we can suppress the message, and it should indicate guidance to the relay rather than normative behaviour.  Chair is not sure that the proposal changes the specified functionality.

Kyocera understand that cases other than the same-serving-cell case are not addressed in the note; for example, if everyone is in idle, there is an RRC setup in flight, and the relay reselects without notification, what happens?  Chair understands we rely on the failure of the RRC procedure to recover from this.

Huawei intend to allow the remote UE(s) to reselect if they need to without waiting for discovery.

ZTE think we should consider accumulated QoS instead of ho pcount, as this is the root reason for wanting to reselect.  Apple think we should not re-discuss this issue.

Agreement:

Rely on discovery signalling to address the case of reselection in the same serving cell without notification from the intermediate relay UE, including where it causes an increase of hop count.  H454 can be closed (no spec change except to fix the language of the NOTE to replace “may not” with “may be omitted” or something similar).

Additional documents related to H455 (note: R2-2507353 contains an additional proposal)

R2-2506844	Intra-gNB Service Continuity for Multi-hop U2N Relay	CATT	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2507353	Remaining issue on U2N multi-hop U2N relay control plane	Qualcomm Incorporated	discussion	NR_SL_relay_multihop-Core

R2-2507451	Correction on Restricting Service Continuity for Inter-gNB cases	InterDigital France R&D, SAS	discussion	Rel-19

Documents on RILs not marked ToDo

K002 (PropReject but with multiple documents)

R2-2507541	[K002] Required SIB and Paging information release due to SL RLF	ASUSTeK	discussion	Rel-19	38.331	NR_SL_relay_multihop

Noted

Proposal 1:  When a L2 Intermediate U2N Relay UE in RRC_IDLE/RRC_INACTIVE detects sidelink radio link failure with its child UE, the L2 Intermediate U2N Relay UE should initiate the Remote UE information for NR sidelink communication procedure to its parent UE to release the required SIB(s) and the related Paging information associated with the child UE.

Discussion:

Apple think this is one of the motivations for the paging information list, allowing a relay to cancel for multiple child UEs at once.

Huawei think any reasonable UE implementation will do this and we do not necessarily need normative text.  Apple think we should not leave things out of the spec.

ZTE wonder if we need to capture this specific cause or just have a general statement for “upon change of the requested SIB/paging”; they are afraid there could be many different causes for corner cases.

Xiaomi agree with Huawei that it should be left to implementation to avoid expanding the spec.  Apple think it is something the UE has to do.  CATT wonder what the consequence is if we do not do it; Apple clarify that the monitoring is wasted, and CATT are not sure this is a big problem that requires fixing in the spec.

ZTE think if we do not capture the release, the parent may need to spend extra time/energy to monitor paging that is not needed downstream.

Ericsson agree to capture the behaviour to avoid unnecessary monitoring and signalling.

Huawei note that in idle/inactive there is no data transfer, so they are not sure sidelink RLF will even happen in this case.  ASUSTeK understand that it can happen due to sidelink traffic between the UEs.  Apple think there can be ongoing procedures between the UEs even without data relaying; Ericsson agree that we do not exclude the local traffic scenario.

Huawei understand that the remote UE will do relay reselection.  Apple think this does not help the upstream UE reduce monitoring/forwarding.

Huawei think it is a bit strange to consider idle/inactive when the rest is for connected.

LG think if the failure happens during setup, the intermediate has not requested monitoring yet and there is nothing to cancel, and SL-SRB2/SL-SRB3 may not be there.

Agreement:

When a L2 Intermediate U2N Relay UE in RRC_IDLE/RRC_INACTIVE detects sidelink radio link failure with its child UE, the L2 Intermediate U2N Relay UE should initiate the Remote UE information for NR sidelink communication procedure to its parent UE to release the required SIB(s) and the related Paging information associated with the child UE.  Details of implementation to be worked out in CR implementation/ASN.1 review.  K002 can be closed.

O503/Z454/Z455/Z456/O505/E044/H452/H454

R2-2507590	[O503] [Z454] [Z455] [Z456] [O505] [K002] [E044] [H452] [H454] discussion on remaining issues related to C-plane procedure for multi-hop relay	Sharp	discussion	Rel-19	NR_SL_relay_multihop-Core

Proposal 7. [K002] RAN2 to agree that intermediate relay UE in IDLE/INACTIVE sends remote UE information to release SIB/Paging information at parent UE when the UE detects some trouble on downstream. Procedural text should be further checked in CR.

W500/W501 (PropReject)

R2-2506946	[W500][W501]Discussion on SUI for multi-hop U2N Relay	NEC Corporation	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal 1: It is suggested to adopt RIL[W500] to allow Last Relay UE to request local ID for L2 Intermediate U2N Relay UE.

Proposal 2: It is suggested to adopt RIL[501] to allow Last Relay UE to receive the paging UE ID from peer L2 Intermediate U2N Relay UE.

Discussion:

Apple think it works today because the intermediate relay UE acts as a remote UE.  NEC think there are multiple remote UEs and the last relay UE needs to know which one is involved.

ZTE understand that from the parent UE’s perspective, there is no such thing as a downstream intermediate and it will always be considered as a remote.

Apple think there is no ambiguity for the gNB, and we understand that an indirectly connected child cannot send direct signalling to a UE multiple hops upstream (“grandparent”).

CATT agree with Apple and ZTE but think the scenario can be further discussed.

Apple consider that the second change (on paging ID) is not needed since we are already introducing multiple paging IDs in the ASN.1, but we can check after implementation of this meeting’s agreements.

Agreement:

W500 and W501 remain open (to be checked after implementation of this meeting’s agreements).

E049/E050 (PropAgree, both labelled E049 in comments file)

R2-2507428	discussion on RIL E049 and RIL E050	Ericsson	discussion	Rel-19	NR_SL_relay_multihop

Noted

Proposal 1	Correct the need code for the field sl-SRAP-ConfigRelay-ToAddModList-r19 and sl-SRAP-ConfigRelay-ToReleaseList-r19 from Need R to Need N.

Proposal 2	Include UE procedure texts handling release action for the fields sl-SRAP-ConfigRelay-ToAddModList-r19.

Proposal 3	Delete the field sl-SRAP-ConfigRelay-ToAddModList-r19.

Proposal 4	Update the description for the IE SL-RelayUE-ConfigMH as the below

- The IE SL-RelayUE-ConfigMH specifies the threshold configuration information for NR sidelink Last U2N Relay UE (in case of multi-hop L2 U2N Relay communication) or Intermediate U2N Relay UE or First U2N Relay UE.

Discussion:

Ericsson clarify that they understand the RIL implementations need some adjustment, and P3 is a new issue; they understand that the ToAddModList here is not useful.

Huawei think we should see a TP next meeting.

Ericsson will create a corresponding ASN.1 review issue.

Agreement:

E049 and E050 remain open for further discussion of how to implement.  Issues can be discussed in CR implementation/ASN.1 review and/or next meeting.

H453 (PropAgree; note that R2-2506843 contains an additional proposal)

R2-2506843	Further Discussion on Control Plane Leftover Issues	CATT	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal 2:  RAN2 agrees to multiply the timers in ue-TimersAndConstants with the hop count (eg t300 * hop count) for multi-hop relay T300/T301/T319 cases.

Discussion:

Huawei indicate that the current CR implementation adds the case where the timers are not provided in the MH case.  CATT understand that this aligns with the proposal and everything is OK.

R2-2506877	Remaining CP issues for Multi-hop Relay	NEC	discussion	Rel-19	NR_SL_relay_multihop

Noted

Proposal-1: For multi-hop scenarios involving two or more Relay UEs, allow the Remote UE to choose to whether apply the legacy t300-RemoteUE or t300-RemoteUE*hopCount based on UE implementation.

Other issues

[AT131bis][401][Relay] Rel-19 relay RRC non-RIL issues (Huawei)

Scope: F2F offline to briefly discuss the RRC proposals not related to RILs, with the goal to prioritise what needs to be treated online and progress towards consensus where possible.

Intended outcome: Report to Wednesday relay session in R2-2507791

Schedule: Monday 2025-10-13 16:30 CET in Brk2

Deadline: Tuesday 2025-10-14 19:00 CET (for report availability)

R2-2507791	Report of [AT131bis][401][Relay] Rel-19 relay RRC non-RIL issues (Huawei)	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal 1: Once the intermediate relay UE moves in the cell coverage, it is left to intermediate relay UE implementation whether to release the paging-related information in the parent relay UE for itself and for its its child UE(s). There are no spec impacts.

Proposal 2: RAN 2 assumes that parallel RRC Reconfiguration procedure will not be performed by the network when providing SRB 0 and SRB 1 mapping configuration for the multi hop relay UEs in R19.

Proposal 3: Flexiblity for the Intermediate relay UEs to include the received paging information in the sl-PagingInfo-RemoteUE or sl-PagingInfo-RemoteUE-List based on its implimintation is not pursued.

Proposal 4: RAN2 to discuss and confirm to capture in the specifications the case for discovery model B, where an intermediate relay UE that already has a PC5 connection with a parent UE may send a response message directly to the remote UE—without forwarding the solicitation message to its parent UE—if the PC5 AS conditions with the child UE are met.

Discussion:

Apple wonder if P1 really has no spec impact; they see that the request to the parent relay should be cancelled.  Huawei indicate that this is a temporary condition where the intermediate relay may be moving in and out of coverage, and we should be able to avoid constant updates, but they do not see that we need to specify behaviour for this.  Apple think this will result in double monitoring by the intermediate and the parent, and we should require the intermediate to update when it changes monitoring status.

Huawei think we similarly did not specify in-and-out-of-coverage behaviour for the remote UE in Rel-17.

OPPO understand that we do not have a clear specification today; we say “if the UE has paging information to provide” or “if any paging information has changed”, so they think it can cover the proposed behaviour without spec impact.  Apple think this language applies to the request, not the paging monitoring, and they think our agreement to minimize double monitoring means we should not allow this.

Qualcomm have a similar view to Apple and think the easiest thing is to let the last relay UE monitor paging always.  However, they do not see double paging as a big problem since the remote UE will ignore the duplicate page; they think this can happen even on Uu.

Huawei agree that double paging does not cause any systemic harm.

OPPO understand we agreed that in idle/inactive the intermediate relay can monitor paging, and it is FFS only for the connected case.

Apple and Qualcomm think the intermediate relay UE should never release the paging information and we can accept the double paging.  Qualcomm think a good intermediate relay UE should not monitor directly.

Apple wonder if P2 has spec impact.  ZTE understand that if there is nothing specified, the network may do anything.

OPPO think it is the same as in Rel-17, when the network is expected to configure the relay first and then the remote to make sure the relay can forward the configuration for the remote, with no spec impact.

Qualcomm think P4 is in upper-layer scope and we do not need to capture it in RAN2.

LG agree that the operation is in SA2 scope, but whether to check the AS conditions is in RAN2 scope.

Xiaomi also think it is an upper-layer issue and we should not agree to ignore a message received from upper layers.

Huawei indicate that we do not have this case captured in the spec now; we have the case with no PC5 connection and we forward the message after checking the threshold, and to support the case with a connection we would have to add something.

Apple understand based on Huawei’s explanation that there is no issue; the UE should check the AS condition first, and then if it passes, upper layers should handle it.

LG understand that the last relay UE checks the RSRP before sending the response message, and the original intention may be that the last relay UE selects the best path, so the intermediate relay UE has to check the RSRP.

Huawei indicate that we have a case where the intermediate relay UE checks the condition when it does not have a PC5 link with the parent, and the issue here is whether to do the same thing in the case where it does have a PC5 link.

Qualcomm agree that the intermediate checks the receiving link in all cases and then forwards the message to upper layers; they think there is no special behaviour in the AS layers here.  Huawei understand that the message is forwarded in different directions in the two cases, and upper layers rely on us to forward it.

Qualcomm understand that the upper layer will handle the forwarding.  Huawei wonder if the solicitation message will be sent to upper layers only on condition that the AS condition is met.

LG consider that the intermediate relay UE does not currently check the AS condition for forwarding; Apple understand that this is because it checks before passing the solicitation message to upper layers.  Huawei indicate that the intermediate relay checks for the link, then checks the condition, and accordingly forwards the message to the parent.  Qualcomm think the exact order of checks is a UE internal implementation issue, but they agree that we specify in RAN2 which node the message should be forwarded to, as in U2U relay.

OPPO understand LG’s concern is that a bullet for sending the response message would need to be added in our spec.

Agreements:

Once the intermediate relay UE moves in the cell coverage, it is left to intermediate relay UE implementation whether to start monitoring paging directly for its child UEs. There are no spec impacts.

RAN2 assumes that parallel RRC Reconfiguration procedures for child UEs will not be performed by the network when providing SRB0 and SRB1 mapping configuration for the multi hop relay UEs in R19.  No spec impact is expected, but RAN2 understand that a parallel configuration is likely to mean that the parent relay cannot tell if a packet is valid and will drop the packet.

Flexibility for the Intermediate relay UEs to include the received paging information in the sl-PagingInfo-RemoteUE or sl-PagingInfo-RemoteUE-List based on its implementation is not pursued.

For discovery model B, an intermediate relay UE that already has a PC5 connection with a parent UE may send a response message directly to the remote UE—without forwarding the solicitation message to its parent UE—provided the PC5 AS conditions with the child UE are met.  How this is realised in the UE layers is up to implementation.  Capture the possibility to transmit the response to the remote UE in our spec.

R2-2507257	Discussion on discovery and relay reselection for multi-hop U2N relay	LG Electronics Inc.	discussion	Rel-19	NR_SL_relay_multihop

R2-2507259	Discussion on the control plane procedure for multi-hop U2N relay	LG Electronics Inc.	discussion	Rel-19	NR_SL_relay_multihop

R2-2507452	Corrections on Notification Message Handling	InterDigital France R&D, SAS	discussion	Rel-19

R2-2507492	Remaining issues for Multi-hop Relay	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

### 8.13.3	User plane corrections

Impact to 38.351, 38.321, and 38.323.

SRAP-6

R2-2506803	Discussion on user plane correction for multi-hop U2N Relay	OPPO	discussion	Rel-19	NR_SL_relay_multihop

Noted

Discussion:

Apple wonder if the proposal actually changes the default behaviour; they think we could use the default behaviour for all hops and not change the spec.  OPPO understand that the default SL-RLC1 is for the case of the first message when neither the remote nor relay has a dedicated RRC configuration, and in this case all hops besides the first should already be in RRC_CONNECTED and have dedicated configuration.

LG have the same understanding and think the proposal is aligned with Rel-17 behaviour; if the first relay UE is in connected it can use the dedicated configuration.

Apple understand that the proposal requires the network to give a dedicated configuration to the intermediate relay UE(s), and they do not see the need for this constraint.

OPPO consider that in Rel-17, default was only used for the first hop, and we have never discussed using it on the other hops, so they do not see that there is a new requirement in the proposal.

Ericsson think we should keep the flexibility for the network and agree with Apple, but they think the gNB can (not “must”) provide the dedicated signalling.

OPPO understand that the network will provide dedicated configuration for all the bearers including SRB0, so they do not see the concern for SRB1.  Apple indicate that configuring SRB1 would use up a channel ID and the network may want to conserve it.  OPPO do not think the additional channel ID is mandatory; the network can configure multihop bearers using the same channel ID, and there is nothing special about SRB1 in this respect.

Apple note that the LCID for SRB1 is reserved, and if the network sends a dedicated configuration there is a risk that it uses an additional LCID.

Chair wonders if anything is broken if we have the flexibility for the network as suggested by Apple and Ericsson.  OPPO do not see a functional problem, but they think there would be some spec impact, and they note that the intermediate relay UE loses some ability to do error checking on SRB1.  Apple think it can associate the packet with a source remote UE from the SRAP header on SRB1 (unlike SRB0), so they see nothing broken.

Xiaomi do not think OPPO’s proposal restricts the flexibility, since the relay UEs will always be connected as remote UEs first.  Apple note that SL-RLC1 is bidirectional, and they think the configuration as a remote UE can use the default SL-RLC1 rather than a dedicated configuration as in Rel-17.

ZTE indicate that we already ask the network to provide a dedicated channel for SRB0, and the added flexibility for SRB1 is not too significant; they would rather rely on dedicated configuration as proposed by OPPO.  Apple think the current spec allows the network to do this.

Sharp see that there would be impact to the RRC spec for error handling from the Apple/Ericsson interpretation.

Agreement:

At the link between remote UE and the first relay UE, the link between intermediate relay UEs, and the link between intermediate relay and the last relay, or the link between the last relay and the network, the network may provide a dedicated configuration for SL-RLC1.  If the network does not provide such a configuration, the default PC5 SL-RLC1 configuration is used.

Proposal 1	(SRAP-6) For the DL and UL SRB1 of remote UE in multi-hop U2N Relay:

-	At the link between remote UE and the first relay UE, the default PC5 RLC channel (i.e., SL-RLC1) can be used as in R17 single hop U2N Relay;

-	At the link between intermediate relay UEs or the link between intermediate relay and the last relay, or the link between the last relay and the network, the RLC channel is configured by the network via dedicated RRC message.

R2-2507591	(SRAP-6) discussion on remaining issues related to U-plane procedure for multi-hop relay	Sharp	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Discussion:

Sharp clarify that some additional text is needed for the uplink case.

Apple think we do not need this level of detail in the spec as the behaviour is similar to the Rel-17 case; in particular, they think that the requirement to release SL-RLC1 is not right since it may be used for the UE’s own setup as a remote UE.

OPPO checked the Rel-17 spec and found that the release behaviour aligns with the TP.  Apple understand that this is a different case since the intermediate relay UE(s) can have traffic as a remote UE.

OPPO wonder how the UE can know if a dedicated SRB1 configuration is for its own traffic or for relay traffic.  ZTE understand that the dedicated configuration will have an associated L2ID that indicates this.  OPPO do not see the use case for the network to provide a dedicated configuration for relay but leave the default configuration for the UE’s own traffic; Apple think it is a possible case.

OPPO understand that relative to Sharp’s TP, we would need to clarify whether the configuration is for a particular destination.

Huawei wonder if the gain is worth the spec complexity.  Apple think some simplification is possible.

Agreement:

Capture in the RRC spec that when a dedicated configuration is received for SL-RLC1, the intermediate relay UE releases the corresponding default configuration for SL-RLC1 on the same link.  Spec impact to be evaluated in CR implementation (refer to TP in R2-2507591, subject to simplification where possible).

Proposal 1.	(SRAP-6) Add procedural text in the RRC specification to describe the SRB1 behaviour for Intermediate U2N Relay UE, dedicated signalling can be used to configure RLC channel for SRB1 for both of a remote UE and an intermediate relay UE.

Other issues

R2-2506984	Discussion on SRAP layer issue	ZTE Corporation, Sanechips	discussion	Rel-19	NR_SL_relay_multihop

Noted

Discussion:

ZTE understand after offline discussion that the current spec captures this behaviour, but some clarification may be needed.

LG think the TP is not quite aligned with the definitions of the terminology.

OPPO understand ZTE’s concern is that the “connected child UE” terminology is not consistent, and this can be handled in CR implementation.

Proposal 1.	Only the first intermediate relay UE needs to remove the SRAP header. Adopt the TP in annex clause.

R2-2507633	SRAP error handling and related TP	Samsung	discussion

Noted

Discussion:

Apple think there is some irregular terminology in the TP.  Samsung agree that some of it is a placeholder for parameter names that would need to be defined.  OPPO have the same understanding as Apple, and they think the functionality is already provided by the current error handling.

Samsung agree that the error check for multihop may not be necessary since there is only a single path, but they can conceive of an error case where the relay UE receives a packet for a child UE that it does not serve.  OPPO think this case is already handled based on whether the child UE is configured to this relay UE by the network, and if not the packet will be dropped.  ZTE have the same understanding as OPPO.

Proposal. RAN2 to agree the TP in the Appendix of this document, capturing in the SRAP specification the case of unknown/incorrect next-hop ID on the downstream, and the case of the UE ID of the indirect parent being compared to the sl-LocalIdentity corresponding to L2 ID of the ingress link on the upstream.

### 8.13.4	Others

Impact to specs not listed above, including capability aspects of 38.331.

R2-2507354	Last relay UE capability	Qualcomm Incorporated, vivo, Samsung, Xiaomi, OPPO	discussion	NR_SL_relay_multihop-Core

Noted

Proposal 1	Remote operation is not a prerequisite for the last relay UE, and keep the current specification as it is.

## 8.14	Additional topological enhancements

(NR_WAB_5GFemto; leading WG: RAN3; REL-19; WID RP-243009)

Time budget: 0 TU

Tdoc Limitation: 0 tdocs

Work on this WI will only be triggered by LS from RAN3 so work on this WI is not expected to start RAN2#127bis or RAN2#128.

No contributions expected for this meeting

## 8.15	NavIC L1 SPS A-GNSS support

(LCS_NAVIC_L1_SPS_NR_LTE-Core; leading WG: RAN2; REL-19; WID RP-251552

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

## 8.16	BDS B2b in A-GNSS

LCS_BDS_B2b_LTE_NR; leading WG: RAN2; REL-19; WID RP-250767)

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

## 8.17	IoT-NTN TDD mode

(IoT_NTN_TDD; leading WG: RAN1; REL-19; WID RP-243293)

Time budget: 0TU

Tdoc Limitation: 1 tdoc

Corrections to all specs.

Including the lists of open issues, if any, raised in the following email discussions:

[Post131][310][IoT NTN TDD] Stage2 CR (Iridium)

[Post131][312][IoT NTN TDD] MAC CR (Toyota)

[Post131][313][IoT NTN TDD] 36.304 CR (Xiaomi)

[Post131][314][IoT NTN TDD] capability CR (Samsung)

- Incoming LSs

R2-2506714	LS on updated text proposal for 36.300 for IoT NTN TDD mode (R1-2506535; contact: Qualcomm)	RAN1	LS in	Rel-19	IoT_NTN_TDD	To:RAN2

Noted

RAN2 confirms that the new text in the RAN1 TP is already captured in 36.300

R2-2506741	LS Reply on precompensation for NB-IoT NTN TDD mode (R4-2512550; contact: Iridium)	RAN4	LS in	Rel-19	IoT_NTN_TDD-Core	To:RAN1	Cc:RAN2

Noted

R2-2507710 	Reply LS on precompensation for NB-IoT NTN TDD mode (R1-2508064; contact: Qualcomm)

Noted

- ASN.1 review

R2-2507063	RIL status on IoT NTN TDD	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_TDD

PropAgree: V220, V221, Z051, Z052, Z053, Z054, N021

V220, V221, Z051, Z052, Z053, Z054, N021 are Agreed

PropReject: N022

-	Nokia thinks this was rejected because the discussion in RAN1 was still ongoing. Now the discussion in RAN1 has concluded and we should capture something in RAN2. QC agrees

-	HW thinks RAN1 sent an LS to RAN4 for confirmation and we need to wait for RAN4 first

ToDo: X501

X501 and N022 are ToDo

R2-2507062	Rapporteur correction on IoT NTN TDD	Huawei, HiSilicon	CR	Rel-19	36.331	19.0.0	5161	-	F	IoT_NTN_TDD

Revised in R2-2507262

R2-2507262	Rapporteur correction on IoT NTN TDD	Huawei, HiSilicon	CR	Rel-19	36.331	19.0.0	5161	1	F	IoT_NTN_TDD	R2-2507062

-	ZTE is not sure about the change of pattern into frame in the field description on radioFrameOffsetForMeasurement. QC agrees something needs to be fixed

Endorsed (but can further check the use of ‘frame’ in field description for radioFrameOffsetForMeasurement and the typos)

Revised in R2-2507772

R2-2507772	Rapporteur correction on IoT NTN TDD	Huawei, HiSilicon	CR	Rel-19	36.331	19.0.0	5161	2	F	IoT_NTN_TDD

Revised in R2-2507788 to reflect the meeting agreements

R2-2507788	Rapporteur correction on IoT NTN TDD	Huawei, HiSilicon	CR	Rel-19	36.331	19.0.0	5161	3	F	IoT_NTN_TDD

Discussed in [Post131bis][306]

[Post131bis][306][IoT NTN TDD] RRC CR (Huawei)

Scope: Update the RRC CR with meeting agreements

Intended outcome: Endorsed CR in R2-2507788

Deadline:  short

=> Endorsed in R2-2507788

[X501]

R2-2507456	Discussion on RIL [X501] for IoT NTN TDD	Samsung	discussion	Rel-19

Proposal 1: Reject X501 and introduce correction to field description to indicate it is the IoT NTN TDD pattern of the neighbouring cell of the associated satellite.

Proposal 2: Agree text proposal in Annex A.

R2-2507674	Discussion on the RIL X501 (radioFrameOffset-r19) for IoT NTN TDD	Beijing Xiaomi Electronics	discussion	Rel-19	IoT_NTN_TDD-Core

Proposal 1: The radioFrameoffset-r19 should be configured per cell rather than per satellite.

The radioFrameoffset-r19 should be configured per cell rather than per satellite.

Continue in offline 309 to discuss how to implement this

Proposal 2: The radioFrameoffset should be included in SIB4-NB and SIB5-NB respectively.

The radioFrameoffset should be included in SIB4-NB and SIB5-NB respectively.

Proposal 3: The provided TP can be considered.

[AT131bis][309][IoT NTN TDD] radioFrameoffset per cell (Samsung)

Scope: Discuss how to implement radioFrameoffset per cell

Intended outcome: summary of the offline discussion (in R2-2507778)

Deadline for offline discussion summary: Friday 2025-10-17 09:00

R2-2507778 	Report of [AT131bis][309][IoT NTN TDD] radioFrameoffset per cell	Samsung	discussion	Rel-19	IoT_NTN_TDD

Proposal 1: Introduce satellite ID per cell if radioFrameOffset is configured, with a restriction that it can only be used for IoT NTN TDD and is configured together with radioFrameOffset. Use TP in Annex as a baseline.

-	ZTE wonders if this would be a new satellite ID

Introduce satellite ID per cell if radioFrameOffset is configured, with a restriction that it can only be used for IoT NTN TDD and is configured together with radioFrameOffset. Use TP in Annex as a baseline in the Post meeting email discussion

[Other]

Continue in offline 311 on all the RRC related proposals

R2-2507048	Discussion on IoT NTN TDD mode	OPPO	discussion	Rel-19	IoT_NTN_TDD

Proposal 1	In NTN TDD mode, UE is configured with a (starting) subframe id within the 8 UL NB-IoT subframes for all UL SPS transmissions.

Proposal 2	In NTN TDD mode, UE doesn’t start onDurationTimer within the UL segmented transmission gap.

R2-2507064	Neighbour cell measurement in IoT NTN TDD	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_TDD

Proposal 1: RAN2 clarifies that UE should perform neighbour cell measurement within the 8.28ms duration from the start of the valid downlink duration.

Proposal 2: RAN2 clarifies that inter-frequency neighbour cell measurement for RRC reestablishment is supported in invalid UL and DL periods in IoT NTN TDD.

R2-2507442	Remaining issues for NB-IoT NTN TDD mode	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_TDD

Proposal 1	For SPS interval, the values of semiPersistSchedIntervalUL-r15 are interpreted as new values that are multiple of 90ms.

Observation 1.	As NB-IoT does not support frequency priority, the UE may select TDD mode cell over FDD cell.

Proposal 2	Clarify that it is up to UE whether to perform TDD mode neighbor cell measurements when performing normal mode neighbor cell measurements.

R2-2507612	Remaining issues on support of TDD mode for NB-IoT-NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_TDD

Observation 1: From UE’s perspective, the time separation between an UL and a DL occasion in the NB-IoT TDD NTN frame structure is 24 subframes.

Proposal 1: RAN2 to capture UE AS operation during the gaps between UL and DL occasions in NB-IoT TDD NTN mode, by reusing the AS operation defined for GNSS measurement gap, with the exception of the neighbor cell measurements which can be performed outside the serving cell’s UL-DL pair.

-	QC is not sure this is necessary. Also Samsung wonders if this is correct.

-	Nordic does not support this

Proposal 2: If P1 can be agreed, adopt the TP in Annex 1 for TS 36.300.

Observation 2: The RAR window and CR timer may occur primarily during the UL-DL occasion gap if the start of the window is based on UE-eNB RTT.

Proposal 3: The start of the RAR window and Contention Resolution timer shall be aligned with the start of the DL occasions, when the UE-eNB RTT is shorter than the UL-DL gap.

Proposal 4: If Proposal 3 is agreeable, RAN2 to adopt the TP implementation to TS36.321 in Annex 2.

[AT131bis][311][IoT NTN TDD] RRC open issues (Huawei)

Scope: Discuss the remaining RRC related proposals

Intended outcome: summary of the offline discussion (in R2-2507773)

Deadline for offline discussion summary: Friday 2025-10-17 09:00

R2-2507773 	Report of [AT131bis][311][IoT NTN TDD] RRC open issues	Huawei	discussion	Rel-19	IoT_NTN_TDD

Proposal 1: Come back to the SPS collision issue (due to SPS postponement) in the next meeting.

Come back to the SPS collision issue (due to SPS postponement) in the next meeting.

Proposal 2: Check RAN4 progress and see if anything needs to be clarified for UE behaviour on neighbour measurement using TDD frame offset in the next meeting.

Check RAN4 progress and see if anything needs to be clarified for UE behaviour on neighbour measurement using TDD frame offset in the next meeting.

Proposal 3: RAN2 clarifies that inter-frequency neighbour cell measurement for RRC reestablishment is supported in invalid UL and DL periods in IoT NTN TDD in the Stage 2 spec as a new scenario.

RAN2 clarifies that inter-frequency neighbour cell measurement for RRC reestablishment is supported in invalid UL and DL periods in IoT NTN TDD in the Stage 2 spec as a new scenario (submit a Stage 2 CR for this at the next meeting)

Proposal 4: RAN2 understands if UE supports both IoT NTN band and IoT TDD band, it is up to UE implementation whether to select an IoT NTN cell or an IoT TDD cell. FFS on spec impact.

RAN2 understands if UE supports both IoT NTN band and IoT TDD band, it is up to UE implementation whether to select an IoT NTN cell or an IoT TDD cell. FFS on spec impact.

Agreements:

1.	The radioFrameoffset-r19 should be configured per cell rather than per satellite.

2.	The radioFrameoffset should be included in SIB4-NB and SIB5-NB respectively

3.	Introduce satellite ID per cell if radioFrameOffset is configured, with a restriction that it can only be used for IoT NTN TDD and is configured together with radioFrameOffset. Use TP in Annex as a baseline in the Post meeting email discussion

4.	RAN2 clarifies that inter-frequency neighbour cell measurement for RRC reestablishment is supported in invalid UL and DL periods in IoT NTN TDD in the Stage 2 spec as a new scenario (submit a Stage 2 CR for this at the next meeting)

5.	RAN2 understands if UE supports both IoT NTN band and IoT TDD band, it is up to UE implementation whether to select an IoT NTN cell or an IoT TDD cell. FFS on spec impact.

## 8.18	LTE-based 5G Broadcast

(LTE_terr_bcast_Ph2; leading WG: RAN1; REL-19; WID RP-250794)

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

### 8.18.1	Organizational

Incoming LS, rapporteur input etc.

R2-2507467	WI TerrBcast ASN.1 comments file	Qualcomm Incorporated	discussion	Rel-19	LTE_terr_bcast_Ph2-Core	Late

Noted

R2-2507468	WI TerrBcast ASN.1 review file	Qualcomm Incorporated	discussion	Rel-19	LTE_terr_bcast_Ph2-Core	Late

Noted

R2-2507469	Corrections to LTE-based 5G Broadcast Phase 2 after ASN.1 review	Qualcomm Incorporated	CR	Rel-19	36.331	19.0.0	5168	-	F	LTE_terr_bcast_Ph2-Core	Late

Revised by e-mail to include the agreements from this meeting

Revised in R2-2507926

R2-2507926	Corrections to LTE-based 5G Broadcast Phase 2 after ASN.1 review	Qualcomm Incorporated	CR	Rel-19	36.331	19.0.0	5168	1	F	LTE_terr_bcast_Ph2-Core

Revised in R2-2507927

R2-2507927	Corrections to LTE-based 5G Broadcast Phase 2 after ASN.1 review	Qualcomm Incorporated	CR	Rel-19	36.331	19.0.0	5168	2	F	LTE_terr_bcast_Ph2-Core

The CR is agreed in principle

### 8.18.2	RRC corrections

Corrections to TS 36.331 which require Tdoc submission as per RIL list.

R2-2507581	RRC corrections on LTE-based 5G Broadcast	Samsung	discussion	Rel-19	Late

RIL S903 is agreed

Already covered by the rapporteur CR

### 8.18.3	Other corrections

Corrections to other specifications including 36.321 and UE capabilities

R2-2507339	Consideration on cyclic shift for PMCH	Samsung	discussion	Rel-19

Proposal 1: Specify UE capability to support “alpha3” for cyclic shift for PMCH in TS 36.306 and TS36.331. Adopt TP1 and TP2 respectively.

- QCM think we need this capability

Specify UE capability to support “alpha3” for cyclic shift for PMCH in TS 36.306 and TS36.331. Adopt TP1 and TP2 respectively.

RRC TP will be included in rapporteur RRC CR

Prepare a CR for 36.306

R2-2507924	Correction on the cyclic shift capability for PMCH	Huawei, HiSilicon, Samsung	CR	Rel-19	36.306	19.0.0	1934	-	F	LTE_terr_bcast_Ph2-Core

The CR is agreed in principle

[AT131bis][503][LTE Broadcast] CRs for 36.331 and 36.306 (Qualcomm/Huawei)

Scope: Update/prepare the CRs for 36.331 and 36.306 according to the agreements

Intended outcome: CRs for approval

Deadline:  Friday 2025-10-17, 08:00

## 8.19	TEI19

Time budget: 1 TU

Tdoc Limitation: 1 tdoc for new proposals and 1 tdoc for old proposals for RAN2-led.

1 additional tdoc for primary co-sourcing company on top of the limit is allowed for co-sourced contribution with 4 or more companies.

Companies are encouraged to submit co-sourced contributions, which will have priority for discussion in RAN2#130

### 8.19.1	RAN2-led

SL Relay (SL breakout session)

R2-2506876	Extension of SFN-DFN mechanism for SL multi-hop relay	NEC, Ericsson	discussion	Rel-19	TEI19

Noted

Proposal 1	Intermediate relay UE forwards the SFN-DFN offset and timing offset(s) if present provided by its parent UE and an additional timing offset to a child UE.

Proposal 2	Intermediate relay UE sets the additional timing offset according to the relation between its DFN timeline and the DFN timeline of its parent.

Proposal 3	The remote UE determines the SFN time line of the serving network based on its DFN time line, the SFN-DFN offset and the additional timing offsets forwarded in the relay chain.

Proposal 4       Adopt the TPs captured in Clause 5

Discussion:

ZTE think we should justify the scenario first; they wonder if a multihop remote will actually be in PRS coverage.  Qualcomm have a similar view and think the SFN-DFN offset should not be treated in isolation; we have a remote UE indication in LPP that the LMF can take into account, and the LMF may assume it is a single-hop remote when it is actually much further away due to multihop, so they think a dedicated analysis is needed rather than just an RRC CR.

Qualcomm think the ability to adjust the offset for upstream timing is a new UE capability different from the ability to compute SFN-DFN offset; they are also not sure if we can compensate easily for degradation in the accuracy of the offset as it travels along a chain of relays, and it seems necessary to involve RAN4 to check this point.

Ericsson think if we need to support positioning for multihop, we have to make some sort of assumption, and the easiest thing would be to take E-CID-like measurements; they understand that if the UE can use the SFN-DFN offset and propagation delay compensation to determine where the PRS are located, it may be able to take measurements that support some kind of positioning.

Qualcomm think positioning for multihop is only for GNSS, because PRS is unlikely to be visible, and the offset would only be useful for GNSS timing.

Ericsson think one use case is that the remote UE will understand the SFN timing of the serving cell and be able to find the PRS.  Qualcomm think for single-hop, detectability of the PRS is plausible, but not for multihop.

Lenovo think from the relay point of view, the remote UE and intermediate relay UE actually can be in coverage; in Rel-18 for single hop, cell edge was a use case, and for multihop they think it is possible that the remote UE could be close in enough to receive PRS.

Ericsson agree with Lenovo’s comment and think one additional hop does not necessarily add a big distance since maximum sidelink range is only ~1 km.

Xiaomi wonder if we could restrict to the in-coverage case.  Ericsson think we could support the signalling and let the deployment decide when it is useful.

Apple have similar concerns to Qualcomm about the probability that the UE can see the PRS.

Ericsson wonder if it is intended to allow the UE to sync with GNSS; they understand that if this is the only possibility, the feature is probably not needed.

Xiaomi think there are different coverage cases as well as the GNSS case, and we may need to discuss which ones make sense.

Qualcomm wonder if the remote UE knows if it is in coverage or not; the LMF may assume the remote UE is in coverage of the serving cell.  They see LPP impact to inform the LMF that this is a multihop remote UE.

Ericsson would like to keep it simple and are thinking of an NRPPa+RRC-based solution for UE Rx-Tx measurements, which should allow the LMF to position the UE without the weight of LPP and without depending on PRS.  Qualcomm wonder what gNB would be the target of the SRS transmission for Rx-Tx.

NEC wonder what the alternative to an RRC CR solution is.  Qualcomm do not think there is an obvious alternative, but we need a discussion of whether/how to support positioning for multihop.

ZTE agree with Qualcomm that we took multiple meetings to discuss the single-hop case, and having just the offset is not a complete solution here; they are not convinced about the use case.

R2-2506948	Discussion on SFN-DFN offset in multi-hop scenario	Lenovo	discussion	Rel-19

Proposal 1: support forwarding the SFN-DFN offset at the L2 U2N Relay UE or at the L2 Last U2N Relay UE in a multi-hop scenario.

Proposal 2: the intermediate relay UE sets sfn-DFN-OffsetSupported to support once the intermediate relay UE obtains the SFN-DFN offset from the connected parent relay UE if the previous setting is absent.

R2-2506951	Introduction of SFN-DFN offset in Multi-hop scenario [PosMultiplehop]	Lenovo	CR	Rel-19	38.331	19.0.0	5493	-	B	TEI19

LTM_enh_SR

R2-2507406	Introducing SR resources in LTM cell switch MAC CE [LTM_enh_SR]	Ericsson, Continental Automotive, T-Mobile USA, BT Plc., Sharp, Charter Communications, Rakuten Mobile, Verizon, InterDigital, Qualcomm Incorporated, AT&T, Vodafone, MediaTek Inc., NTT Docomo, LG Electronics, Deutsche Telekom, Telia Company, Turkcell	CR	Rel-19	38.331	19.0.0	5530	-	B	TEI19

Check offline if there is a better way to capture the configuration IE while ensuring that the UE can easily release the configuration.

Revised in R2-2507916

R2-2507916	Introducing SR resources in LTM cell switch MAC CE [LTM_enh_SR]	Ericsson, Continental Automotive, T-Mobile USA, BT Plc., Sharp, Charter Communications, Rakuten Mobile, Verizon, InterDigital, Qualcomm Incorporated, AT&T, Vodafone, MediaTek Inc., NTT Docomo, LG Electronics, Nokia, Deutsche Telekom, Telia Company, Turkcell, ZTE Corporation, Samsung	CR	Rel-19	38.331	19.0.0	5530	1	B	TEI19

The CR is in principle agreed

R2-2507407	Introducing SR resources in LTM cell switch MAC CE [LTM_enh_SR]	Ericsson, Continental Automotive, T-Mobile USA, BT Plc., Sharp, Charter Communications, Rakuten Mobile, Verizon, InterDigital, Qualcomm Incorporated, AT&T, Vodafone, MediaTek Inc., NTT Docomo, LG Electronics, Deutsche Telekom, Telia Company, Turkcell	CR	Rel-19	38.321	19.0.0	2130	-	B	TEI19

Revised in R2-2507917

R2-2507917	Introducing SR resources in LTM cell switch MAC CE [LTM_enh_SR]	Ericsson, Continental Automotive, T-Mobile USA, BT Plc., Sharp, Charter Communications, Rakuten Mobile, Verizon, InterDigital, Qualcomm Incorporated, AT&T, Vodafone, MediaTek Inc., NTT Docomo, LG Electronics, Nokia, Deutsche Telekom, Telia Company, Turkcell, ZTE Corporation, Samsung	CR	Rel-19	38.321	19.0.0	2130	1	B	TEI19

The CR is in principle agreed

R2-2507408	Introducing SR resources in LTM cell switch MAC CE [LTM_enh_SR]	Ericsson, Continental Automotive, T-Mobile USA, BT Plc., Sharp, Charter Communications, Rakuten Mobile, Verizon, InterDigital, Qualcomm Incorporated, AT&T, Vodafone, MediaTek Inc., NTT Docomo, LG Electronics, Deutsche Telekom, Telia Company, Turkcell	CR	Rel-19	38.306	19.0.0	1367	-	B	TEI19

Revised in R2-2507918

R2-2507918	Introducing SR resources in LTM cell switch MAC CE [LTM_enh_SR]	Ericsson, Continental Automotive, T-Mobile USA, BT Plc., Sharp, Charter Communications, Rakuten Mobile, Verizon, InterDigital, Qualcomm Incorporated, AT&T, Vodafone, MediaTek Inc., NTT Docomo, LG Electronics, Nokia, Deutsche Telekom, Telia Company, Turkcell, ZTE Corporation, Samsung	CR	Rel-19	38.306	19.0.0	1367	1	B	TEI19

The CR is in principle agreed

R2-2507919	Introducing SR resources in LTM cell switch MAC CE [LTM_enh_SR]	Ericsson, Continental Automotive, T-Mobile USA, BT Plc., Sharp, Charter Communications, Rakuten Mobile, Verizon, InterDigital, Qualcomm Incorporated, AT&T, Vodafone, MediaTek Inc., NTT Docomo, LG Electronics, Nokia, Deutsche Telekom, Telia Company, Turkcell, ZTE Corporation, Samsung	CR	Rel-19	38.300	19.0.0	1054	-	B	TEI19

The CR is in principle agreed

[AT131bis][005][TEI19] LTM_enh_SR (Ericsson)

Intended outcome: Agree in principle to CRs and check with Samsung if there is an alternate way.

Deadline: Friday

R2-2507106	DRX adaptation for voice activity	Apple	discussion	Rel-19

Proposal 1: Introduce a new peer-silence-indication MAC CE that the UE can use to indicate the number of on-Durations for which the UE will be in a Reduced Monitoring mode (i.e., skip PDCCH monitoring or use an alternate SSSG pattern for PDCCH monitoring).

Proposal 2: UE enters the Reduced Monitoring mode upon transmitting the peer-silence-indication MAC CE.

Observation 4: Standard UL data arrival procedures enable the UE to exit the reduced monitoring mode and return to normal operation.

Proposal 3: If there is UL data in the buffer in the reduced monitoring mode, the UE exits the reduced monitoring mode.

-	Ericsson thinks that there are means in the network to detect. This will introduce delays from the network and if the UE is not listening (and network missed the indication) the network may think that there issues and adapt it’s scheduling.

-	ZTE also thinks that the network can detect silent periods and even if you recognize UL silence the UE doesn’t know the DL side.   ZTE Thinks that even if current packet is silent packet doesn’t mean the second one will also be silent packet.   Apple thinks that even if there is a delay by another 160ms it is ok from the UE perspective.

-	Xiaomi is supportive from UE side, but asks how the MAC CE is triggered given this is linked to application layer.  Will it be up to implementation.   Apple thinks that we can discuss solution and is worried about power.

-	CATT asks how the UE predicts and also thinks that DL services may be delayed, so prefers not to do skipping.

-	Vivo thinks that the silence period is dependent on UE and codec so this is related to other SA WGs.

-	Samsung also shares concerns that silent period cannot be predicted accurately.   Benefit is marginal as one UE can be in silent but not another one.

-	Oppo thinks that providing more information to the network is a good direction and we should consider the voice traffic performance.  We can think of this in 6G.

-	ZTE explains that this a tradeoff between performance and power saving.

-	CMCC has same concern with ZTE and Samsung as it impact voice performance.

-	Apple hopes that we keep this in mind and consider it for 6G.

=> Noted

R2-2507604	Adapting TTT based on mobility state in NR	Ericsson	discussion	Rel-19	TEI19

Proposal 1	TTT scaling can be implemented on the network side. The UE side TTT scaling mechanism from LTE should not be introduced in NR.

Proposal 2	For 6G more robust methods for determining mobility state (e.g. based on RSRP) should be explored. Also consider how other mobility parameters than TTT can be tuned.

Noted

R2-2507689	Discussion on speed dependant scaling of measurement related parameters	KDDI	discussion	Rel-19	TEI19

Proposal 1	Clarify the applicability and limitations of each candidate option, including identifying effective deployment scenarios such as highways, railways, and heterogeneous networks.

As a result, three candidate approaches were identified:

- Option 1: No standardization; rely on vendor-specific network implementation.

- Option 2: Reuse LTE-based mechanisms, such as handover count-based mobility state estimation.

- Option 3: Use RSRP variation-based estimation, such as relaxed measurement criteria.

Proposal 2	Initiate a survey among operators and vendors to gather feedback on the feasibility and preference for each option. This will ensure that the standardization process is informed by practical insights and deployment realities across different regions and technologies.

Proposal 3	Consider the inclusion of medium mobility state definitions in the standard, based on commercial network usage.

Proposal 4	Conduct a survey among operators to assess whether Option 1 is acceptable, specifically considering the risk that there is no guarantee network vendors will implement the required features. The survey should clarify if operators can tolerate the possibility that NR may not provide the functionalities they need due to vendor-specific implementations.

Noted

Discussion

-	Docomo prefers option 2 as it solves the high speed scenario.   Nokia thinks we should consider option 2 if the operators do really have a problematic scenario, otherwise we can do option 1 and let the networks handle it.

-	Samsung explains that there are some issues on their network side that option 1 doesn’t solve.   Further option 2 is implemented in LTE in the field and it is useful in certain scenarios.   So option 2 is a compromise solution.

-	Huawei also thinks it is beneficial to specify something and we should pursue option 2.

-	Qualcomm understands that in LTE it didn’t perform very well in the field.  KDDI and Samsung explain that they show in the paper with real trials the benefit.  ZTE confirms the benefits on this in the field.

-	Apple thinks that if operator has a problem in field they can ask their network vendors to count.   LG also thinks that the network can handle it.

-	Qualcomm thinks that for the existing UEs the network will have to do something anyways, and just doing it for Rel-19 won’t help much.    Samsung thinks that we can also use magic sentence.  Docomo thinks that even for Rel-19 it would be helpful as the system will keep on going for many years.

-	CATT also supports option 2 and we can add UE capability whether it is supported in UE side.

-	Qualcomm thinks that this only works for homogeneous deployments.

-	Ericsson thinks that the inter-vendor scenario doesn’t happen very often.   KDDI thinks that in Japan this happens in some high speed area.

-	Apple thinks that practically speaking the networks have to solve this anyways as there are legacy UEs.   Huawei thinks that it is complicated for the networks so if at least some UEs implemented they will benefit.

-	Mediatek thinks option 1 preferable.

Agreements

Reuse LTE-based mechanisms, such as handover count-based mobility state estimation.

Introduce optional UE capability

[POST131bis][006][TEI19] Speed dependent scaling (KDDI/Samsung)

Intended outcome: Agreeable CRs, 38.331, 38.306

Deadline:  Long

NTN  (NTN Breakout session)

(Why TEI19, all CRs have WI code?)

R2-2507243	Redirection from E-UTRAN TN to NB-IoT NTN [IoT-TN-NTN-redir]	Google	discussion	Rel-19	TEI19

- Redirection from E-UTRAN TN to NB-IoT NTN

Observation 1	UE is able to switch from an E-UTRAN TN cell to an NB-IoT NTN cell according to [EUTRAN-to-NBIoTNTN], but this only occurs when UE is in idle mode and completely out of E-UTRAN TN coverage.

Observation 2	Based on the existing TEI-19 enhancement, E-UTRAN TN cells are already capable of broadcasting NB-IoT satellite information via SIB33, making it possible for UEs in the connected state to utilize the information for potential transitions to NB-IoT NTN cells.

Proposal 1	Redirection from E-UTRAN TN to NB-IoT NTN is supported in TEI-19, as parts of the [IoT-TN-NTN-redir] or [EUTRAN-to-NBIoTNTN] enhancement.

-	Samsung would be fine with this

-	Ericsson is worried by the number of TEI19 for all the mobility cases but could accept to have this as part of the already agreed TEI

-	vivo thinks in case this would need a new identifier

-	ZTE wonders how to handle the UE capability in this case.

-	Google thinks we need a new capability

We introduce support for redirection from E-UTRAN TN to NB-IoT NTN in TEI-19, as parts of the [IoT-TN-NTN-redir]

Draft Stage 2, RRC and UE capability CR accordingly

Proposal 2	If P1 is agreed, RAN2 considers the TP in Annex as the starting point for drafting CRs.

- UE behaviour following unsuccessful redirection from TN to NTN

Observation 3	A UE being redirected from TN to NTN may not find a suitable NTN cell according to redirectedCarrierInfo. This may happen in all the TEI-19 redirection scenarios within the same RAT, including from NR TN to NR NTN, from E-UTRAN TN to eMTC NTN, or from NB-IoT TN to NB-IoT NTN.

Proposal 3	If a UE is redirected from TN to NTN within the same RAT and subsequently fails to find a suitable NTN cell based on the redirectedCarrierInfo, the UE implementation will determine whether to camp on any other suitable cell in either the TN or the NTN.

-	Xiaomi thinks the current spec already covers I case and we don’t need p3

-	Google is ok to leave this to UE implementation

Proposal 4	If P3 is agreed, RAN2 to capture a NOTE reflecting the agreement in both TS 36.306 (sub-clause 5.2.7) and TS 38.304 (sub-clause 5.2.6).

R2-2507910	Correction for the redirection from E-UTRAN TN to NB-IoT NTN [IoT-TN-NTN-redir]	Google, Samsung, Ericsson, vivo	CR	Rel-19	36.331	19.0.0	5173	-	F	TEI19

-	Ericsson notes that there are some typos in coversheet

Agreed in principle

R2-2507911	Correction for the redirection from E-UTRAN TN to NB-IoT NTN [IoT-TN-NTN-redir]	Google, Samsung, Ericsson, vivo	CR	Rel-19	36.306	19.0.0	1933	-	F	TEI19

-	Ericsson notes that there are some typos in coversheet

Agreed in principle

R2-2507289	RRC correction and stage 2 for TN to NTN redirection	Samsung, Xiaomi	discussion	Rel-19	TEI19

Proposal 1: Introduce Stage 2 description for NR TN to NR NTN.

Agreed

Proposal 2: Agree the description in Annex A.

Add: “The UE may be provided with NTN payload assistance to assist in searching for NTN cells for the purpose of redirection from TN to NTN.” to Stage2

Proposal 3: Introduce Stage 2 description for IoT TN to IoT NTN.

Agreed

Proposal 4: Agree the description in Annex A.

Add: “The UE may be provided with NTN payload assistance to assist in searching for NTN cells for the purpose of redirection from TN to NTN.” to Stage2

Proposal 5: Same solution for handling SMTC when redirecting from LTE TN to NR NTN: the SMTC is specified to be present for this release for NR TN to NR NTN redirection.

Same solution for handling SMTC when redirecting from LTE TN to NR NTN

Proposal 6: Same solution for handling SMTC when redirecting from LTE TN to NR NTN: the offset is adjusted according to the propagation delay.

We come back to this in the next meeting after the discussion for LTE TN to NR NTN

Proposal 7: Agree the text proposal in Annex C.

R2-2507921	Introduction of TN to NTN redirection for IoT [IoT-TN-NTN-redir]	Samsung, Google, Ericsson, vivo	CR	Rel-19	36.300	19.0.0	1438	-	B	TEI19

Agreed in principle

R2-2507923	Introduction of redirection from NR TN to NR NTN [NR_TN_NTN_redir]	Samsung, Xiaomi	CR	Rel-19	38.300	19.0.0	1055	-	B	TEI19

Agreed in principle

- inter-RAT cell-selection

R2-2507346	Asisstance for inter-RAT cell-selection from NB-IoT NTN to NR-NTN	EchoStar, Boost Mobile, Qualcomm, Aalyria, Terrestar, Skylo, Sateliot	CR	Rel-19	36.306	19.0.0	1929	-	B	IoT_NTN_enh-Core

-	Ericsson thinks if we do something it should cover eMTC as well. Samsung thinks that in case this could be easily extended to eMTC

-	vivo thinks it’s not essential for eMTC

-	Ericsson thinks this has impact on SA5 and they should be liased

-	Aalyria thinks that in case we can send an LS to SA5

-	Xiaomi supports this proposal for NB-IoT but don’t see the need to support eMTC

-	Ericsson don’t think this should be supported for the NR NTN to NB-IoT direction. Allyria thinks this is actually the most important case. QC also thinks this makes sense in the other direction

We introduce assistance for inter-RAT cell-selection from NB-IoT/eMTC NTN to NR-NTN and from NR NTN to NB-IoT/eMTC NTN

Come back in the next meeting with revised CRs

R2-2507356	NB-IoT NTN to NR NTN Cell Selection	EchoStar, Boost Mobile, Qualcomm, Aalyria, Terrestar, Skylo, Sateliot	CR	Rel-19	36.331	19.0.0	5163	-	B	IoT_NTN_enh-Core

R2-2507358	Asisstance for inter-RAT cell-selection from NR NTN to NB-IoT NTN	EchoStar, Boost Mobile, Qualcomm, Aalyria, Terrestar, Skylo, Sateliot	CR	Rel-19	38.306	19.0.0	1361	-	B	NR_NTN_enh-Core

R2-2507360	NR-NTN to NB-IoT NTN Cell Selection	EchoStar, Boost Mobile, Qualcomm, Aalyria, Terrestar, Skylo, Sateliot	CR	Rel-19	38.331	19.0.0	5526	-	B	NR_NTN_Ph3-Core

### 8.19.2	Other WG-led

R2-2506713	Reply LS on non-RedCap UE UL SRS frequency hopping for positioning (R1-2506531; contact: ZTE)	RAN1	LS in	Rel-19	TEI19	To:RAN3	Cc:RAN2

Noted

R2-2506715	Reply LS on UL Tx switching for TEI19 (R1-2506538; contact: MediaTek)	RAN1	LS in	Rel-19	TEI19	To:RAN4, RAN2

Noted

R2-2507022	Discussion on SA2 reply LS on paging capability loss issue	vivo	discussion	Rel-19	NR_LPWUS-Core, TEI19

Wait for RAN3 discussion

Should be moved to NR Others

R2-2507100	Restriction on RAT utilization	Apple, OPPO, InterDigital, Huawei, HiSilicon, Nokia, Samsung, Ericsson	draftCR	Rel-19	25.304	18.0.0	B	ECRATU

MBS (MBS breakout session)

R2-2507139	Rapporteur correction on CAS muting for LTE based 5G broadcast [5GB_CASMuting]	Huawei, HiSilicon, Samsung	CR	Rel-19	36.331	19.0.0	5162	-	F	TEI19	Revised

R2-2507237	5G Broadcast CAS Muting in stage 2 spec [5GB_CASMuting]	ZTE Corporation, Sanechips, Samsung, Huawei	CR	Rel-19	36.300	19.0.0	1436	-	F	TEI19

Cover page need to be revised, e.g. untick ME box, remove TEI ID from WI code, add other CRs, copy relevant RAN3 agreement, change to Cat. B

Approve by e-mail

Revised in R2-2507811

- ZTE received some comment to untick ME box

- Lenovo asks why?

- QCM thinks this does not impact UE, but wonders if we need this CR at all

- ZTE clarifies that there is an agreement in RAN3 but it has not been captured

R2-2507263	Rapporteur correction on CAS muting for LTE based 5G broadcast [5GB_CASMuting]	Huawei, HiSilicon, Samsung	CR	Rel-19	36.331	19.0.0	5162	1	F	TEI19	R2-2507139

The CR is agreed in principle

[AT131bis][504][TEI19] 5G CAS muting stage-2 CR (ZTE)

Scope: Revise R2-2507237 according to agreements

Intended outcome: CR for approval

Deadline:  Friday 2025-10-17, 08:00

R2-2507811	5G Broadcast CAS Muting in stage 2 spec [5GB_CASMuting]	ZTE Corporation, Sanechips, Samsung, Huawei	CR	Rel-19	36.300	19.0.0	1436	1	F	TEI19

The CR is agreed in principle

## 8.20	NR Others

Tdoc limit: 2

Specific items may be allocated to a breakout session for treatment.

Impacts from Other RAN WGs and TSGs that has no separate TU budget in RAN2. LS ins for Rel-19 specific WIs/SIs that has no RAN WI.

Additional tdocs on top of limit can be allowed for co-sourced contribution with 3 or more companies

### 8.20.1	RAN4

RRC Signaling for power domain enhancement

R2-2506730	LS on RRC signalling for power domain enhancement (R4-2511759; contact: Huawei)	RAN4	LS in	Rel-19	NR_ENDC_RF_Ph4-Core	To:RAN2

Noted

CSSF enhancement

R2-2506736	Reply LS on CSSF optimization for NR RRM Phase 5 (R4-2512161; contact: Apple)	RAN4	LS in	Rel-19	NR_RRM_Ph5-Core	To:RAN2

Noted

Rx BSF optimization

R2-2506739	LS on Rx BSF optimization for NR RRM Phase 5 (R4-2512333; contact: CICT	RAN4	LS in	Rel-19	NR_RRM_Ph5-Core	To:RAN2

Noted

R2-2506788	Report of [Post131][225][NR_Others] On Rx BSF optimization (CATT)	CATT	discussion	Rel-19	NR_RRM_Ph5-Core

Noted

According to RAN4 LS, a new item for indicating UE preference to quit FBS is added in UAI.

P2

- ?? Prohibit timer is not used for indicating UE preference to quit FBS in UAI.

Discussion

P2

-	Nokia think prohibit timer is useful. CATT explains that after UE report the UAI, NW can handle via proper configuration, so the prohibit timer is not so critical.

-	Ericsson wonders whether P2 implies some restriction to NW configuration.

-	Qualcomm think this does not mean UE needs to change its preference multiple times, i.e., it is like a one-time report.

-	Huawei think we should allow UE to report either way, i.e., keep it in the FBS state or quitting it, and think we need the prohibit timer. Qualcomm think we should have a prohibit timer if this is the case.

-	CATT indicate that in the R4 LS it is clear that they will not discuss the UE behaviour after reporting the UAI.

[AT131bis][202][NR_Others] On Rx BSF optimization (CATT)

Scope: Further discuss the UAI content, the need of the prohibit timer

Intended outcome: Summary with proposals in R2-2507732.

Deadline: before Friday CB

R2-2507732	Discussion on Rx BSF UAI	CATT	discussion 	Rel-19	NR_RRM_Ph5-Core

Noted

On UAI design:

P1: whether FBS operation is preferred or not is indicated in UAI.

P2: prohibit timer is used, and the candidate timer length can be {s0, s0dot5, s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s20, s30}.

P3: send a LS to RAN4 on RAN2 agreement (if P1 is agreed).

On scenario restriction:

P4: RAN2 to discuss the following understanding:

Understanding 1: In case the FBS triggering condition is configured, UE only applies it when the scenario related conditions of applicability of requirements defined in 38.133 are met.

Understanding 2: NW only configures the FBS triggering condition when the scenario related conditions of applicability of requirements defined in 38.133 are met.

On scenario restriction:

P4: RAN2 to discuss the following understanding:

Understanding 1: In case the FBS triggering condition is configured, UE only applies it when the scenario related conditions of applicability of requirements defined in 38.133 are met.

Understanding 2: NW only configures the FBS triggering condition when the scenario related conditions of applicability of requirements defined in 38.133 are met.

Discussions in CB

-	Apple think understanding #2 is the usual way, and understanding #1 is complex. Vivo share this view.

-	Qualcomm think the understanding #1 is ok, Qualcomm think this ‘consideration of applicability’ include RRC configured condition and the RSRP condition.

-	Nokia think with understanding #2 is very difficult for NW to configure. Ericsson think we can capture that ‘it is up to network when to configure’. ZTE share this view, and think there are many same cases.

Whether FBS operation is preferred or not is indicated in UAI.

Prohibit timer is introduced, and the candidate timer length can be {s0, s0dot5, s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s20, s30}.

In case the FBS triggering condition is configured, UE only applies it when the scenario related conditions of applicability of requirements defined in 38.133 are met. It is up to network when to configure the FBS triggering.

Send a LS to RAN4 to inform RAN2 agreement

[Post131bis][217][NR_Others] LS to RAN4 on Rx BSF optimization (CATT)

Intended outcome: Approve the LS as per the agreements

Deadline:  Short

=> Approved in R2-2507935

R2-2507935	Reply LS on Rx BSF optimization for NR RRM Phase 5	RAN2	LS out	Rel-19	NR_RRM_Ph5-Core	To:RAN4

=> Approved

[Post131bis][218][NR_Others] CR for TS 38.331 on Rx BSF optimization (CATT)

Intended outcome: Discuss and prepare the CR, for submission to the next meeting

Deadline:  Long

R2-2506789	Introduction of Rx BSF optimization for NR RRM Ph5	CATT	draftCR	Rel-19	38.331	18.6.0	B	NR_RRM_Ph5-Core

Will be updated in the post meeting email discussion

R2-2507613	Fast Beam Sweeping Factor	Nokia	discussion	Rel-19	NR_RRM_Ph5-Core	Late

UE capability for 6Rx UE

R2-2506735	LS on Release Independence of 6Rx (R4-2511898; contact: T-Mobile)	RAN4	LS in	Rel-19	NR_ENDC_RF_Ph4-Core	To:RAN2	Cc:RAN1

Noted

R2-2506987	[DRAFT] Reply LS on Release Independence of 6Rx	Qualcomm Incorporated, T-Mobile 	LS out	Rel-19	NR_ENDC_RF_Ph4-Core	To:RAN4	Cc:RAN1

Noted

R2-2507171	Discussion on release independent of 6Rx	vivo	discussion	Rel-19

Noted

Reply to RAN4, indicating that it is possible to specify the early implementation of the UE capability signalling on the support of maximum 6 DL MIMO layers for PDSCH. RAN2 assume that whether 6Rx supporting shall be early implemented should be decided by RAN4.  Update the draft LS in R2-2507733.

Discussion in CB

-	Qualcomm explain that the draft LS R2-2507733 was further updated in R2-2507742, which should be treated.

R2-2507742	[DRAFT] Reply LS on Release Independence of 6Rx	Qualcomm, T-Mobile	LS out	Rel-19	NR_ENDC_RF_Ph4-Core	To:RAN4	Cc:RAN1

Final LS is approved in R2-2507743.

UE capability for Low band CA

R2-2506733	LS on capability of NR_LBCA_Sw (R4-2511863; contact: Huawei)	RAN4	LS in	Rel-19	NR_LBCA_Sw	To:RAN2

Noted

R2-2506947	Discussion on UE capability of low band CA via switching	Huawei, HiSilicon	discussion	Rel-19	NR_LBCA_Sw

Noted

Proposal 1: RAN2 to discuss the following two alternatives for low band switching capability reporting:

alternative 1: reuse existing BC list for CA to indicate low band switching with a new FeatureSetCombination

alternative 2: introduce a new BC list to indicate low band switching.

Proposal 2: Do not consider higher-order band combinations on low band switching capability reporting in Rel-19.

Discussion

P2

-	Apple think this is about two things, the UE capability and the NW configuration. Qualcomm think this statement is too generic. MediaTek share the understanding as P2 and think nothing preclude UE from reporting so.

R2-2507124	UE capability for LBCA via switching	Apple	discussion	Rel-19	NR_LBCA_Sw

Noted

Proposal 1: RAN2 to select one from the following two alternatives to support LBCA_Sw UE capability signaling.

- Alt-1: Introducing a separate band combination list for LBCA_Sw (similar as UL Tx switching)

- Alt-2: Introducing a separate FeatureSetCombination for LBCA_Sw (similar as DAPS)

Proposal 2: RAN2 to discuss about the applicable deployments of LBCA_Sw and confirm if LBCA_Sw applies to both CA for NR standalone and NR-DC.

Proposal 3: RAN2 to discuss about whether to introduce explicit filtering bit into UECapabilityEnquiry for LBCA via switching if Alt-1 (separate band combination list) is agreed.

Discussion

P2

-	Ericsson think we focus on CA for now and we can extend later if needed. Huawei share this view and think there is no clear requirement from NR DC in R4.

R2-2507601	Consideration on the LBCA Capability Signaling	ZTE Corporation	discussion	Rel-19	NR_LBCA_Sw

Noted

Proposal 1: The LBCA capability can be signaled by introducing an LBCA-specific FeatureSetCombinationID.

Proposal 1a: The UE can indicate whether it supports the LBCA case by using an LBCA-specific FeatureSetCombinationID. There is no need to add other LBCA-specific parameters at this stage.

R2-2507603	Backwards compatibility for low NR band carrier aggregation switching	Ericsson	discussion

Noted

Proposal 1	Introduce an alternative FeatureSetCombinationId for a UE supporting low band carrier switching.

Introduce an alternative FeatureSetCombinationId for a UE supporting low band carrier switching.

We will focus on NR standalone case for the LBCA capability signaling.

On capability for intra-band non-collocated EN-DC/NR-CA deployment

R2-2507606	Consideration on the Type 2 and Type 4 UE Capability Reporting	ZTE Corporation	discussion	Rel-19	NonCol_intraB_ENDC_NR_CA_Ph2-Core

Noted

Proposal 1: Allow the UE to report the super BC with type 4 capability (i.e. interBandMRDC-WithOverlapDL-Bands-r19), with the restriction that this capability is applicable only to the fallback BC with single CC on the NR side.

Proposal 2: Allow the UE to report the super BC with type 2 capability (i.e. requirementTypeIndication-r18), with the restriction that this capability is applicable only to the fallback BC with at most 2 CCs on the NR side.

Proposal 3: For a given super BC, if the UE does not support the corresponding type 2 capability, it shall not report the type 4 capability for that super BC.

Proposal 4: If the above 3 proposals are agreed in RAN2, send an LS to RAN4 for the information.

Discussion

-	Apple think R4 already agreed that type 2 is restricted to single CC case.

-	Ericsson think if R4 already agree then maybe we do not need to do anything further.

RAN2 assume that UE is allowed to report the super BC with type 2 and type 4 capability, with the restriction that this capability is applicable only to the fallback BC with single CC on the NR side for the EN-DC case and with two CCs for the CA case (as defined in the RAN4 spec).

Chair: the other documents will be handled in other sessions.

R2-2506742	LS on UE Capability for Rel.19 Ku band VSAT (R4-2512658; contact: Chunghwa Telecom, Sharp)	RAN4	LS in	Rel-19	NR_NTN_Ku_bands	To:RAN2

Noted

R2-2507193	UE Capability for Rel-19 Ku Band VSAT (R4 60-1/60-2)	Sharp, CHTTL, SES	discussion	Rel-19	NR_NTN_Ku_bands

Observation 2	There can be two options to support Rel-19 VSAT UE capabilities:

- Option 1: Direct extension of Rel-18 Capability to be applicable for both FR1-NTN and FR2-NTN

- Option 2: Introduce new Rel-19 capability bits applicable for both FR1-NTN and FR2-NTN.

Proposal 1	Introduce new Rel-19 capability bits applicable for both FR1-NTN and FR2-NTN.

-	QC thinks we should keep the existing capability for FR2 and introduce a new one for FR1 only (option 3)

-	Sharp thinks that based on RAN4 agreement the capability should be the same for FR1 and FR2 and then thinks we should avoid the proposal suggested by QC

-	Xiaomi supports option 3 but is also fine with option 2

-	Samsung thinks option 2 is aligned to the RAN$ decision and supports that one.

-	ZTE thinks the capability should be per UE (no differentiation between FR1 and FR2) and then thinks we should go for option 2

-	vivo thinks option 2 is simpler (if we go for option 3 we have to say that the setting needs to be the same)

-	HW supports option 3 because they are not sure there is a RAN4 decision not to differentiate between FR1 and FR2. Could also accept to go for option 2

-	Thales thinks option 3 is not the common understanding in RAN4.

-	Ericsson and QC would at least like to check internally with RAN4 first

WA: Introduce new Rel-19 capability bits applicable for both FR1-NTN Ku and FR2-NTN Ku (FFS if we use exactly the FR1-NTN Ku and FR2-NTN Ku terminology or if we find another alternative to further align to RAN4 terminology)

-	Apple thinks we should have a CR to fix the description for legacy UEs, starting from Rel-18

Come back in the next meeting on the need of Rel-18 CRs to clarify the description for legacy UEs

R2-2507194	Draft 38.306 CR for Rel-19 NTN Ku Band	Sharp, CHTTL, SES	draftCR	Rel-19	38.306	19.0.0	NR_NTN_Ku_bands

Submit a revision to the next meeting based on meeting agreements

R2-2507195	Draft 38.331 UE capability CR for Rel-19 NTN Ku Band	Sharp, CHTTL, SES	draftCR	Rel-19	38.331	19.0.0	NR_NTN_Ku_bands

Submit a revision to the next meeting based on meeting agreements

R2-2506933	Introduction of Ku band	Huawei, HiSilicon	CR	Rel-19	38.331	19.0.0	5492	-	B	NR_NTN_Ku_bands

R2-2506934	Introduction of Ku band	Huawei, HiSilicon	CR	Rel-19	38.306	19.0.0	1356	-	B	NR_NTN_Ku_bands

R2-2507383	The introduction of NTN VSAT FR1 capabilities in 38.306	Nokia, Nokia Shanghai Bell	draftCR	Rel-19	38.306	19.0.0	NR_NTN_Ku_bands

R2-2507384	The introduction of NTN VSAT FR1 capabilities in 38.331	Nokia, Nokia Shanghai Bell	draftCR	Rel-19	38.331	19.0.0	NR_NTN_Ku_bands

Moved here from 8.8.4

R2-2507524	UE capability for R19 VSAT UEs	ZTE Corporation,  Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

### 8.20.2	Other WGs

MINT related

R2-2506707	LS on Broadcasting Information on Disaster Condition of a PLMN from E-UTRAN in Case of Disaster Condition (C1-255678; contact LGE)	CT1	LS in	Rel-19	MINT_Ph2	To:RAN2	Cc:SA2

Noted

R2-2507567	Introduction of MINT in EPS	LG Electronics Inc., Nokia	discussion	Rel-19	MINT_Ph2

Noted

Proposal1: Introduce disaster broadcast information fields for MINT in EPS in existing SIB30.

Proposal 2: Introduce a separate disaster roaming access barring for disaster roaming in EPS. Access attempt indicated as disaster roaming in EPS by upper layers only applies the disaster roaming access barring check but not others (other existing access barring control mechanisms)

Proposal 3: If other approaches than proposal2 is considered, send an LS to CT1 and SA2 to ask for detailed requirements for access control against disaster roaming in EPS.

Proposal 4: Update section 4.2 of 36.304 such that NAS maintains applicable disaster roaming information and CN type for available PLMNs including potential disaster PLMNs for available PLMNs, and AS reports applicable disaster roaming information for available PLMNs autonomously including potential disaster PLMNs and CN type.

Proposal 5: To discuss whether to introduce a disaster roaming capability as optional capability without signaling in 36.306.

R2-2506826	Support for MINT in EPS (MINT_Ph2)	Google	discussion	Rel-19	R2-2506190

Noted

Proposal 1: 	Discuss which of these options is the most suitable approach for providing configuration of disaster roaming information.

Option 1: 		Reuse SIB30 with existing configurations, i.e., the existing configurations are shared between E-UTRA/EPC and E-UTRA/5GC.

Option 2:		Reuse SIB30 with R19 configurations.

Option 3:		Define a new SIB.

Proposal 2: 	Support the ACB parameters (i.e., for access control for emergency call, mobile originated signalling, and mobile originated data) for disaster roaming UEs.

Proposal 3: 	Support the SSAC parameters (i.e., for MMTEL voice and MMTEL video) for disaster roaming UEs.

Proposal 4: 	Support barring skip parameters for MMTEL voice, MMTEL video, and SMS for disaster roaming UEs.

Proposal 5: 	ompone the following options:

Option 1: 		Don’t support the ACB parameter for CSFB for disaster roaming UEs.

Ootion 2: 		Support the ACB parameter for CSFB for disaster roaming UEs.

Option 3: 		Send an LS to CT1/SA1 to ask whether CSFB is applicable to a disaster roaming UE. RAN2 decides whether to support the ACB parameter for CSFB for disaster roaming UEs based on CT1/SA1 reply.

Proposal 6: 	ompone the following options:

Option 1: 		Don’t support the EAB parameters for disaster roaming UEs.

Option 2: 		Support the EAB parameters for disaster roaming UEs.

Option 3: 		Send an LS to CT1/SA1 to ask whether EAB is applicable to a disaster roaming UE. RAN2 decides whether to support the EAP parameters for disaster roaming UEs based on CT1/SA1 reply.

Proposal 7: 	ompone the following options:

Option 1: 		Don’t support the ACDC parameters for disaster roaming UEs.

Option 2: 		Support the ACDC parameters for disaster roaming UEs.

Option 3: 		Send an LS to CT1/SA1 to ask whether ACDC is applicable to a disaster roaming UE. RAN2 decides whether to support the EAP parameters for disaster roaming UEs based on CT1/SA1 reply.

Proposal 8: 	Access barring parameters for disaster roaming are included with the disaster roaming information in the same SIB.

Proposal 9: 	Support per-PLMN disaster roaming access barring parameters.

Proposal 10: 	Reuse the existing access barring check for disaster roaming specific access barring

Proposal 11: 	If the disaster roaming specific access barring parameters are not broadcast on the cell, disaster roaming UEs should use the legacy access barring parameters (if omponent) for access barring check.

R2-2507175	Impacts of MINT-EPS feature on RAN2 specifications	Lenovo	discussion	Rel-19	MINT_Ph2

Noted

Proposal 1: Introduce new SIBX for configuring disaster roaming information in MINT-EPS.

Proposal 2: Apply new access barring mechanism based on barring factor and barring time for disaster roaming UEs and broadcast the corresponding parameters via SIB2.

Proposal 3: Confirm that the MINT-EPS feature does not affect TS 36.304.

Proposal 4: Specify the MINT-EPS feature for the UE as an optional feature w/o capability signaling in TS 36.306.

Proposal 5: Introduce the stage 2 description of MINT-EPS feature in the existing description of MINT-5GS feature in TS 36.300, clause 24.10.

Discussion

-	Lenovo explain that they want new SIB due to some concern on signalling overhead, but fine to go with majority.

Issue 1, new sib or sib 30

-	Qualcomm think using existing SIB is very simple.

-	Samsung wonder whether we do need to introduce new field like commonPLMNsWithDisasterConditionEPS or can we reuse. Qualcomm also think we do not need this new filed. Lenovo think technically this might be useful so perhaps we can have this new filed.

Introduce disaster broadcast information fields for MINT in EPS in existing SIB30.

Issue 2, whether to introduce a separate disaster roaming access barring

-	Goolge think reuse LTE is simpler from UE implementation point of view.

-	Lenovo, LG E, Nokia, Samsung think we should introduce.

-	Google think CT1 never discuss this. Lenovo think we can decide and inform them. LG E fine to do so.

Introduce a separate disaster roaming access barring for disaster roaming in EPS. Access attempt indicated as disaster roaming in EPS by upper layers only applies the disaster roaming access barring check but not others

Ask CT1 about the handling of the emergency call for the disaster roaming case

[AT131bis][204][NR_Others] On support for MINT in EPS (LG E)

Intended outcome: Draft LS to CT1, CC SA2, SA1 to inform our conclusion (draft LS in R2-2507735), review the CRs (R2-2507568/70/71/72) and update if needed

Deadline: before Friday CB

R2-2507737	Summary of [AT131bis][204] ][NR_Others] On support for MINT in EPS	LGE	discussion	Rel-19	MINT_Ph2

Noted

LS

R2-2507735	[DRAFT] Reply LS on Broadcasting Information on Disaster Condition of a PLMN from E-UTRAN in Case of Disaster Condition	LG Electronics	RAN2	Rel-19	MINT_Ph2	To:CT1	Cc:SA1, SA2

LS is updated by replacing the last paragraph in section 1 with ‘RAN2 understands that during access control for disaster roaming access in 5GS, Access Identity 3 related access control only applies and access category related access control is bypassed. Therefore RAN2 made similar changes to the EPS case. As a consequence, RAN2 understands that disaster roaming access for emergency call is not treated differently from other disaster roaming access for non-emergency call in terms of access control. RAN2 would like to ask if this is in line with the intention of access control toward disaster roaming access.’

Updated LS is approved in R2-2507744

CRs

R2-2507738	Introduction of MINT in EPS	LG Electronics Inc., Nokia	CR	Rel-19	36.300	19.0.0	1437	1	B	MINT_Ph2

RAN2 to in-principle agree the CR to 36.300 based on the content in R2-2507738 (replacing the term ‘EPC’ to ‘EPS’ and ‘5GC’ to ‘5GS’)

R2-2507739	Introduction of MINT in EPS	LG Electronics Inc., Nokia	CR	Rel-19	36.306	19.0.0	1932	1	B	MINT_Ph2

The CR to 36.306 in R2-2507739 is in-principle agreed.

R2-2507568	Introduction of MINT in EPS [MINT_Ph2_EPS]	LG Electronics Inc., Nokia	CR	Rel-19	36.331	19.0.0	5171	-	B	MINT_Ph2

Revised in R2-2507740

R2-2507740	Introduction of MINT in EP S[MINT_Ph2_EPS]	LG Electronics Inc., Nokia	CR	Rel-19	36.331	19.0.0	5171	1	B	MINT_Ph2

Revised in R2-2507741

R2-2507741	Introduction of MINT in EPS	LG Electronics Inc., Nokia	CR	Rel-19	36.331	19.0.0	5171	2	B	MINT_Ph2

Will be further discussed in the post meeting email discussion

Discussions on CR to 36.331

whether barring parameters for disaster roaming are included in SIB2 (Option 1) or SIB30 (Option 2)

-	Samsung think both options are fine, but think it is slightly better to put it in SIB2. Ericsson, Lenovo share this view.

-	Qualcomm think Option 2 is better, and would like to avoid impact to all other UEs that are receiving SIB2. Qualcomm has strong concern in requiring changes to SIB2. Google share this view and think if we put all the related information for disaster roaming in a single SIB then it does not impact other UEs who do not care these info.

Barring parameters for disaster roaming are included in SIB30

On need for CR to 36.304

CR to 36.304 is not needed

[Post131bis][219][NR_Others] CR for support for MINT in EPS (LG E)

Intended outcome: Update the CRs to 36.331 and 36.300 for agreement in-principle

Deadline:  Short

=> Agreed in principle in:

R2-2507936 (36.331)

R2-2507937 (36.300)

R2-2507936	Introduction of MINT in EPS	LG Electronics Inc., Nokia	CR	Rel-19	36.331	19.0.0	5171	3	B	MINT_Ph2

=> Agreed in principle

R2-2507937	Introduction of MINT in EPS	LG Electronics Inc., Nokia	CR	Rel-19	36.300	19.0.0	1437	2	B	MINT_Ph2

=> Agreed in principle

R2-2506827	Introduction of MINT in EPS	Google	CR	Rel-19	36.331	18.6.0	5155	1	B	TEI19	R2-2506192

R2-2506829	Introduction of MINT in EPS	Google	CR	Rel-19	36.306	18.5.0	1924	1	B	TEI19	R2-2506193

R2-2506830	Introduction of MINT in EPS	Google	CR	Rel-19	36.300	18.5.0	1431	1	B	TEI19	R2-2506194

The above three CRs are not pursued.

R2-2507569	CR to 36.304 Introduction of MINT in EPS	LG Electronics Inc.	CR	Rel-19	36.331	19.0.0	5172	-	B	MINT_Ph2	Withdrawn

R2-2507570	CR to 36.304 Introduction of MINT in EPS	LG Electronics Inc., Nokia	CR	Rel-19	36.304	19.0.0	0886	-	B	MINT_Ph2

R2-2507571	CR to 36.306 Introduction of MINT in EPS	LG Electronics Inc., Nokia	CR	Rel-19	36.306	19.0.0	1932	-	B	MINT_Ph2

R2-2507572	CR to 36.300 Introduction of MINT in EPS	LG Electronics Inc., Nokia	CR	Rel-19	36.300	19.0.0	1437	-	B	MINT_Ph2

R2-2506827	Introduction of MINT in EPS	Google	CR	Rel-19	36.331	18.6.0	5155	1	B	TEI19	R2-2506192

R2-2506829	Introduction of MINT in EPS	Google	CR	Rel-19	36.306	18.5.0	1924	1	B	TEI19	R2-2506193

R2-2506830	Introduction of MINT in EPS	Google	CR	Rel-19	36.300	18.5.0	1431	1	B	TEI19	R2-2506194

On RAT restriction

Moved from 8.19.2

R2-2506705	Reply LS on UE usage of the RAT restrictions (C1-255319; contact: Apple)	CT1	LS in	Rel-19	ECRATU	To:RAN2	Cc:CT4, RAN

Noted

R2-2507100	Restriction on RAT utilization	Apple, OPPO, InterDigital, Huawei, HiSilicon, Nokia, Samsung, Ericsson	draftCR	Rel-19	25.304	18.0.0	B	ECRATU

Discussion

-	Lenovo think the behaviour in 5.2.6.1.3 is ‘shall not’, instead of should. Apple think this is optional for UE.

-	Samsung think ‘Maintain a list of equivalent PLMN identities and provide the list to AS.’ is not need for the cell selection part.

Content of the draft CR is agreeable, will have a real CR submitted to the next R2 meeting for technical endorsement.

On suspension of trace production

R2-2506756	LS on temporary suspension of trace production (S5-253909; contact: Ericsson)	SA5	LS in	Rel-19	TraceQoE_OAM	To:RAN3, RAN2

Noted

R2-2507394	Discussion on LS temporary suspension of trace production	L.M. Ericsson Limited	LS out	Rel-19	To:SA5	Cc:RAN3

Noted

Proposal 1	Send a reply LS to SA5, stating that the addition of the mentioned attributes has no impact on RAN2 and that RAN2 consents to the attributes being added (see draft LS in the Annex).

[AT131bis][205][NR_Others] Draft LS on temporary suspension of trace production  (Ericsson)

Intended outcome: Draft LS in R2-2507736

Deadline: before Friday CB

R2-2507736	Reply LS on temporary suspension of trace production	RAN2	LS out	Rel-19	TraceQoE_OAM	To:SA5	Cc:RAN3

Revised the LS by deleting ‘as a UE in RRC IDLE/INACTIVE cannot be informed about the suspension’.

The final LS is approved in R2-2507745

Other topics with LSin

R2-2506749	Reply LS on energy saving indication from CN to RAN (S2-2507784; contact: LGE)	SA2	LS in	Rel-19	EnergySys	To:RAN3	Cc:RAN2

Noted

R2-2506758	Reply to RAN2 LS on Number of UEs in RRC_INACTIVE state with data transmission (S5-254084; contact: China Telecom)	SA5	LS in	Rel-19	PM_KPI_5G_Ph4	To:RAN2	Cc:RAN3

Noted

Chair: the other documents will be handled in other sessions.

R2-2506706	Reply LS to SA4 on the RAN simulation assumptions for ULBC (C1-255650; contact: Qualcomm)	CT1	LS in	Rel-19	FS_ULBC	To:SA4	Cc:SA2, SA1, RAN1, RAN2, RAN4

R2-2506716	Reply LS on the RAN simulation assumptions for ULBC (R1-2506541; contact: Qualcomm)	RAN1	LS in	Rel-19	FS_ULBC	To:SA4	Cc:RAN4, RAN2, SA2, CT1

R2-2506732	Response LS on the RAN simulation assumptions for ULBC (R4-2511782; contact: Xiaomi)	RAN4	LS in	Rel-20	FS_ULBC	To:SA4	Cc:RAN2, RAN1, SA2

R2-2506746	Reply LS on the RAN simulation assumptions for ULBC (S2-2507578; contact: Qualcomm)	SA2	LS in	Rel-20	FS_ULBC	To:SA4	Cc:RAN1, RAN2, RAN4, SA1, CT1

R2-2506754	LS on bundling period and SPS for ULBC (S4aA250258; contact: Qualcomm)	SA4	LS in	Rel-20	FS_ULBC	To:RAN2	Cc:RAN1

# 9	NR Rel-20

## 9.1	AI/ML for PHY Ph2

(NR_AIML_air_Ph2, leading WG: RAN1; REL-20; WID: RP-252445)

Time budget: 0 TU

Tdoc Limitation: 0 tdoc

## 9.2	Ambient IoT Ph2

(Ambient_IoT_Solutions_Ph2, leading WG: RAN1; REL-20; WID: RP-252894)

Time budget: 0.5 TU

Tdoc Limitation: 1 tdoc

### 9.2.1	Organizational

R2-2507032	Work Plan for Solutions for Ambient IoT (Internet of Things) in NR Phase 2	Huawei, T-Mobile USA	Work Plan	Rel-20

Noted

### 9.2.2	Topology 2

Architecture Assumption

R2-2506930	Discussion on topology 2 for A-IoT	OPPO	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

Proposal 1: RAN2 to agree to adopt RRC based solution for implementing Topology 2.

-	Xiaomi indicates that this is reflected in WID.

Noted

Reader-gNB Information Exchange: Contents of the AIOT procedure messages over Uu

R2-2506985	Support of Topology 2 for A-IoT	Xiaomi	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

Proposal 11: Upon reception of A-IoT service request, the gNB signals the A-IoT paging related information (e.g. paging ID, etc.) to the UE reader via RRC dedicated signaling. FFS what specific information related to A-IoT paging needs to be signaled in Uu.

Proposal 12: In Topology 2, the R2D and D2R upper layer data is embedded in the DL and UL RRC messages as containers respectively, and transferred over Uu between the gNB and UE reader.

-	Qualcomm indicates that there are other proposals to introduce new messages so we should wait.

Noted

R2-2506963	Discussion on Topology 2	vivo	discussion	Ambient_IoT_Solutions_Ph2

Proposal 8: The RRC Inventory Request may at least include the following information:

A-IoT Device Identification Requested;

Inventory Assistance Information: Expected D2R Message Size, Approximate Number of Target A-IoT Devices and Time Interval for report;

Follow-on Command Indication;

Optional A-IoT Resource allocation info.

-	Qualcomm asks what time interval is.  Vivo explains that these are the information provided by CN to gNB.  Qualcomm then would agree that we should provide this to reader.

Noted

Discussion

Expected D2R Message Size, Approximate Number of Target A-IoT Devices and Time Interval for report;

-	Xiaomi indicates that we have to be careful as there some information that can be transparently forwarded by the gNB and some that need to be transformed.

-	Ericsson thinks that not all information needs to be sent.  Xiaomi, Interdigital agrees with Ericsson.  Interdigital also thinks we should revisit the information one by one.

-	Nokia thinks that SA2 is discussing whether the information can be delivered to the reader so we can’t assume that the gNB is aware of this information.   Ofinno explains that there are a few options including partial information, so we can only agree that some information will be passed from gNB to reader.

-	Qualcomm thinks that the gNB has to understand the information and encodes the RRC messages.

Resource Allocation: Trigger for Resource Allocation (CN only and/or UE request)

R2-2506943	Discussion on Topology-2 for Ambient IoT	CATT	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

Proposal 2b: RAN2 agrees to adopt Alternative 2 for triggering the gNB to allocate A-IoT resources to UE readers, i.e., BS configures/allocates the A-IoT radio resource to the UE reader, based on the service request from CN.

Noted

R2-2507454	Initial Considerations for Topology 2	InterDigital France R&D, SAS	discussion	Rel-20

Proposal 5:	Dynamic signalling from the intermediate UE to the network is supported to adapt the resource allocation/usage at the intermediate UE during a procedure.  FFS what adaptation mechanisms to support, e.g., modify the timing of the resources, requesting more resources, extending the time usage of the resources, or indicating the non-usage of resources.

Noted

Discussion on gNB allocating resources

-	Xiaomi thinks CN trigger should be baseline.

-	Transsion asks if the allocation is done after selection.  Xiaomi explains that the selection can be happen before and resources allocate is related to request from CN.

-	ZTE Thinks that both CN and Reader requested should be supported.  WE may have collisions so reader may need to request.   Qualcomm also supports InterDigital proposal.  Mediatek, Kyocera, Apple, Nokia and Vivo agrees and the reader has a better understanding of the radio situation with other devices, but there are cases at beginning where the basestation has a good idea.  Kyocera indicates that the reader is responsible for scheduling.

-	Samsung things it is too early to agree and gNB can handle it as the reader would report something to the gBN after the procedure is completed.

-	Ericsson thinks that the baseline is the gNB but the reader can request.  Interdigital thinks that outdoor UEs that are mobile it would be important to allower reader to request.   LG also thinks that the gNB is not aware of the collision.

-	CMCC thinks that if there are sub-sequent paging from reader then the UE reader can ask for more resources.

-	Mediatek ask if the assumption is that there is a very close dynamic understanding from UE reader.    Qualcomm had same question and if the UE reader is expected to provide reporting that is the same as UE provided assistance.

-	Huawei thinks that this is linked to for how long the resources is available.   We should discuss what kind of information from the UE reader is needed.

-	Nokia thinks we can provide simple request from UE reader to gNB.

Agreements

- 1.  	gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN.  FFS what information

2.	gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling.   The resource allocation can be triggered by CN request.   UE reader may provide assistance information related to AIoT transmissions.  FFS what the information may be useful to be provided to gNB.  It is up to gNB how the information is used.  It is understood that assistance information is not mandated for the gNB to determine resource allocation.

3. 	the reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

Resource Validity

R2-2507033	Support for Device 1 operation in Deployment Scenario 2 with Topology 2	Huawei, HiSilicon	discussion	Rel-20

Proposal 7:	Adopt option 1, i.e., “The resources remain valid until the network releases them explicitly” (FFS signaling details, e.g., by RRC reconfiguration or by message containing session complete/release-like info).

Noted

R2-2507212	Discussion on Topology 2 for AIoT 	LG Electronics Inc.	discussion	Ambient_IoT_Solutions_Ph2

Proposal 2. The following AIoT resource allocation methods should be supported.

The UE reader receives the AIoT radio resources configuration in RRC signalling. The AIoT radio resources remain valid until the network releases them explicitly.

The UE reader receives the AIoT radio resources configuration in RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the AIoT radio resources remain valid for that time period, unless the resource configuration is explicitly released by the network.

Noted

Discussion

-	Ericsson, Apple, Nokia, CATT and ompon thinks that a timer is not needed, the gNB is always in control.   Lenovo and CATT are not sure the gNB can set the time.

-	Nokia asks if the reader can tell the reader that it has completed.

-	Qualcomm thinks that we are going too much into detail, we need to decided if it is periodic resources or continuous or one shot.  Mediatek thinks that we could have a one shot resource allocation, or we can have cases where readers can communicate with multiple devices.   Huawei thinks that periodic is not likely and one shot doesn’t make sense.   The assumption is that the network gives you recourses and the procedure should be completed.

-	Vivo agrees that we can have one shot and periodic.

-	Xioami thinks that anyways the gNB can release the connection whenever it wants.   One shot is not really validity but rather transmission opportunity

-	Interdigital explains that allocating resources continuously doesn’t make sense as there may be period of no transmission so the gNB can provide non-contigous finite resources.

Continue discussion next meeting on how resource allocation looks like.

Resource validity in Temporary Out of Connection

R2-2507429	Aspects for Ambient IoT Topology 2	Ericsson	discussion	Rel-20

Proposal 6	Adopt Option 1 in TR 38.169 for handing out of connection, i.e., the UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition .

Noted

R2-2507173	Rel-20 A-IoT: Topology 2 aspects	Qualcomm Incorporated	discussion	Rel-20	Ambient_IoT_Solutions_Ph2-Core

Proposal 10:	The A-IoT resources configured by the serving cell are considered as being valid while the UE is ongoing HO or RLF event (i.e. until HO complete or end of RLF recovery procedure or until a validity timer expires).

Noted

Discussion

-	Qualcomm thinks that the RLF recovery is very short and the UE may recover in the same cell so the inference sholdn’t be a problem.   Ericsson explains that the interference is among the UE readers.

-	Huawei thinks that when we get HO command you stop the transmission.   Qualcomm thinks that it should be after HO complete.

Not Treated:

Resource Granularity

R2-2506901	Discussion on Topology 2 for A-IoT	CMCC	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

Proposal 11: A-IoT radio resources for UE-readers should be allocated in an orthogonal manner, such that the resources are not shared with or interfered by other UEs.

R2-2506963	Discussion on Topology 2	vivo	discussion	Ambient_IoT_Solutions_Ph2

RAN2 confirms that the A-IoT radio resource configuration is allocated per UE reader via dedicated RRC signalling. Finer configuration granularities, such as per A-IoT message or per A-IoT procedure, are excluded.

R2-2506922	Discussion for Topology 2 for Rel-20 Ambient IoT	Lenovo	discussion	Rel-19

Proposal 4: RAN2 is suggested to further consider other resource granularity options (e.g., per transmission/pair) to support specific procedures (e.g., command procedure).

Simultaneous Uu/AIOT

R2-2507348	Initial consideration on Ambient-IoT topology 2	ZTE Corporation, Sanechips	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

Proposal 1: RAN2 to discuss whether to support both in-band and out-of band deployment for AIoT topology 2.

Contributions on support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions.  Only for traffic types DO-DTT and DT.

R2-2506766	Discussion on TP2 in A-IOT	Transsion Holdings	discussion	Rel-19

R2-2506875	Discussion on Topology 2 with intermediate UE as Reader	NEC	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

R2-2506886	Discussion on Topology 2 for Ambient IoT	China Telecom	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

R2-2506914	Discussion on A-IoT topology 2	Spreadtrum, UNISOC	discussion	Rel-20

R2-2506943	Discussion on Topology-2 for Ambient IoT	CATT	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

R2-2506956	Discussion on A-IoT phase 2 deployment scenario	Tejas Network Limited	discussion	Rel-20

R2-2506968	Discussion on Deployment Scenario 2 with Topology 2 in A-IoT	SHARP Corporation	discussion

R2-2507038	Discussion on A-IoT resource allocation for T2	HONOR	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

R2-2507102	Discussion on Topology 2	Apple	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

R2-2507198	RAN2 impacts to support D2T2 for DT and DO-DTT traffic	Ofinno	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

R2-2507211	Discussion on Topology 2 for A-IoT	KT Corp.	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

R2-2507240	Discussion on Ambient IoT Topology 2	ETRI	discussion	Rel-20

R2-2507269	Considerations for Deployment Scenario 2 with Topology 2	Panasonic	discussion	Rel-20

R2-2507297	Discussion on Topology 2 for AIoT	Continental Automotive	discussion	Rel-20

R2-2507318	Discussion on Topology 2 for Ambient IoT	Sony	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

R2-2507458	Ambient IoT aspects in Topology 2	Nokia	discussion	Rel-20

R2-2507501	Initial consideration of A-IoT radio resource management for Topology 2 	Kyocera 	discussion	Rel-20

R2-2507513	Discussion on Topology 2 for Ambient IoT	TCL	discussion

R2-2507560	Discussion on A-IoT resources for UE reader	ASUSTeK	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

R2-2507585	Discussion on Topology 2 for Ambient IoT	CEWiT	discussion

R2-2507619	Initial discussion on introduction of Topology 2	NTT DOCOMO, INC.	discussion	Rel-20

R2-2507651	Discussion on requirements for UE as reader	Rakuten Mobile, Inc	discussion	Late

## 9.3	AI/ML for mobility

(NR_AIML_Mob, leading WG: RAN2; REL-20; WID: RP-252899)

Time budget: 0 TU

Tdoc Limitation: 0 tdoc

## 9.4	Mobility Enh Ph5

(NR_Mob_Ph5; leading WG: RAN2; REL-20; WID: RP-252113)

time budget: 0 TU

Tdoc Limitation: 0 tdoc

## 9.5	XR Enhancements Ph4

(NR_XR_Ph4; leading WG: RAN2; REL-20; WID: RP-252755)

Time budget: 0 TU

Tdoc Limitation:0  tdocs

## 9.6	SON/MDT Ph5

(NR_SON_MDT_Ph5-Core; leading WG: RAN3; REL-19; WID: RP-251869)

Time budget: 0 TU

Tdoc Limitation: 0 tdocs

## 9.7	IoT NTN Ph4

(IoT_NTN_Ph4; leading WG: RAN2; REL-20; WID: RP-252473

Time budget: 0.5 TU

Tdoc Limitation: 1 tdocs

### 9.7.1	Organizational

- Workplan

R2-2506831	Work Plan for IoT NTN Ph4	vivo	Work Plan	Rel-20	IoT_NTN_Ph4-Core

Noted

- Incoming LS from SA2:

Moved here from 9

R2-2506747	LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC (S2-2507636; contact: Qualcomm)	SA2	LS in	Rel-20	FS_5GSAT_Ph4_ARC	To:RAN2, SA4, CT1, SA3, SA1, RAN1

Questions to RAN2:

Question 1 (To RAN2): Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC?

Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically?

Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20?

Question 9 (To RAN2): Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media?

Question 10 (To RAN2): If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

Noted

Draft a Reply LS in R2-2507765

- Incoming LS from SA4 and related replies from other groups:

Questions from the original RAN4 Ls in R2-2505068/ S4-251584:

SA4 kindly asks SA2 and RAN2 to comment on

- the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and

- specifically, whether a packet overhead of 1 byte of MAC header is realistic

Draft a reply LS to R2-2505068/ S4-251584 in R2-2507766

Moved here from 8.20.2

R2-2506706	Reply LS to SA4 on the RAN simulation assumptions for ULBC (C1-255650; contact: Qualcomm)	CT1	LS in	Rel-19	FS_ULBC	To:SA4	Cc:SA2, SA1, RAN1, RAN2, RAN4

Noted

R2-2506716	Reply LS on the RAN simulation assumptions for ULBC (R1-2506541; contact: Qualcomm)	RAN1	LS in	Rel-19	FS_ULBC	To:SA4	Cc:RAN4, RAN2, SA2, CT1

Noted

R2-2506732	Response LS on the RAN simulation assumptions for ULBC (R4-2511782; contact: Xiaomi)	RAN4	LS in	Rel-20	FS_ULBC	To:SA4	Cc:RAN2, RAN1, SA2

Noted

R2-2506746	Reply LS on the RAN simulation assumptions for ULBC (S2-2507578; contact: Qualcomm)	SA2	LS in	Rel-20	FS_ULBC	To:SA4	Cc:RAN1, RAN2, RAN4, SA1, CT1

Noted

R2-2506754	LS on bundling period and SPS for ULBC (S4aA250258; contact: Qualcomm)	SA4	LS in	Rel-20	FS_ULBC	To:RAN2	Cc:RAN1

SA4 kindly asks RAN2 to confirm whether the 120ms and/or 240ms bundling periods would be valid for the SPS operation in NB-IoT NTN, and provide feedback, if any.

Noted

Reply also to this LS in R2-2507766

R2-2507445	[Draft] Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC	Qualcomm Incorporated	LS out	Rel-20	IoT_NTN_Ph4	To:SA2	Cc:RAN1, SA4, SA1, CT1

Revised in R2-2507765 based on the outcome of offline 304

R2-2507765	[Draft] Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC	Qualcomm Incorporated, vivo	LS out	Rel-20	IoT_NTN_Ph4	To:SA2	Cc:RAN1, SA4, SA1, CT1

Remove Draft, put RAN2 as source, add one extra line before the last sentence

Revised in R2-2507784

R2-2507784	Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC	 vivo	LS out	Rel-20	IoT_NTN_Ph4	To:SA2	Cc:RAN1, SA4, SA1, CT1

Approved

R2-2507444	[Draft] Reply LS on the RAN simulation assumptions for ULBC	Qualcomm Incorporated	LS out	Rel-20	FS_ULBC	To:SA4	Cc:RAN1, SA2, CT1

Revised in R2-2507766 based on the outcome of offline 305

R2-2507446	[Draft] Reply LS on bundling period and SPS for ULBC	Qualcomm Incorporated	LS out	Rel-20	FS_ULBC	To:SA4	Cc:RAN1

Revised in R2-2507766 based on the outcome of offline 305

R2-2507766	[Draft] Reply LS on the RAN simulation assumptions, bundling period and SPS for ULBC	Qualcomm Incorporated	LS out	Rel-20	FS_ULBC	To:SA4	Cc:RAN1, SA2, CT1

-	vivo thinks we can say RAN2 “may” provide

Remove Draft, for answer to Q2 change to RAN2 “may” provide

Revised in R2-2507790

R2-2507790	Reply LS on the RAN simulation assumptions, bundling period and SPS for ULBC	Qualcomm Incorporated	LS out	Rel-20	FS_ULBC	To:SA4	Cc:RAN1, SA2, CT1

Approved

### 9.7.2	Other

Contributions should focus on down-selecting between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 and any responses to other WG LSs

R2-2506832	Discussion on Support of IMS Voice over NB-IoT NTN	vivo	discussion	Rel-20	IoT_NTN_Ph4-Core

- Part 1: SA4 LS on RAN assumption

Proposal 1: RAN2 responds to SA4 that a packet overhead of 1 byte of MAC header for voice packets is not realistic based on the current MAC spec.

-	QC thinks also 2 bytes is not realistic.

-	Ericsson thinks this depends on the TBS

Continue in offline 305

Observation 1: In Legacy NB-IoT, the L2 protocol header size for service data transmission is specified as follows,

•	For CP architecture, the RRC overhead is 1 byte for UL and 2 bytes for DL, whereas for UP architecture, the RRC overhead is 0 bytes;

•	For UP architecture, the PDCP protocol header has a size of 1 byte (i.e., PDCP SN=7bits), whereas for CP architecture, the PDCP protocol overhead is 0 bytes;

•	With no RLC concatenation and segmentation, the AM RLC protocol header has a fixed size of 2 bytes (AM RLC is the only applicable RLC mode for the CP architecture), whereas the UM RLC protocol header has a fixed size of 1 byte;

•	Except for certain special cases (e.g., the last sub-header in the MAC PDU), the MAC protocol header has a fixed size of 2 bytes.

Proposal 2: RAN2 responds to SA4 with the L2 protocol header size for service data based on the current specifications for NB-IoT.

Continue in offline 305

Proposal 3: RAN2 responds to SA4 that RAN2 is working on the down-selection of solutions and has not yet concluded on the option.

Continue in offline 305

- Part 2: SA2 LS on support of IMS voice

Continue in offline 304

Proposal 4: For SA2 Q1, RAN2 responds to SA2 that 3GPP RAN only takes responsibility for the configuration and application of the ROHC function, while the implementation of the functionality of the ROHC framework falls outside of RAN2's scope.

Observation 3: To handle the voice packet with different sizes, e.g., due to talk/silence transition, two SPS configuration mechanisms with dynamic scheduling used as a supplementary means can be one of the candidate solutions.

Proposal 5: For SA2 Q3, RAN2 responds to SA2 with the confirmation that RAN scheduling can handle the voice packets of different sizes.

Proposal 6: For SA2 Q6, RAN2 responds to SA2 that it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20.

Proposal 7: For SA2 Q9 and 10, RAN2 responds to SA2 that, as per legacy RAN design, the voice packet is mapped on DRB with UM RLC considering real-time characteristic of voice service, while SRB with AM RLC is used to transmit packets with relaxed latency requirements. SRB is not preferred for voice packets from the RAN2 point of view.

- Part 3: SA4 LS on SPS

Proposal 8: RAN2 postpones the SA4 LS on SPS until it initiates the standardization work on SPS objective.

Continue in offline 305

- Down-select between CP and UP solutions

Observation 4: Due to the low data rate 1~3kbps for the GEO voice service, from RAN’s perspective, protocol overhead should be taken as one of the criteria for down-selecting solutions.

Observation 5: As specified in SA2 TR 23.700-19, there are 11 candidate solutions in total for supporting the GEO voice service. From the perspective of protocol overhead for voice packets, these solutions are categorized into four types: UP IP, CP non-IP, UP non-IP, and CP IP based solutions.

Observation 6: For UP IP based solution, the existing NB-IoT ROHC mechanism between UE and eNB should be enhanced to compress the protocol overhead (i.e., RTP/UDP/IP) of the GEO voice packet, and RAN scheduling should be able to efficiently handle the packet with variable packet size generated in different ROHC compression states.

Observation 7: For UP non-IP based solution (without IP/UDP header), how to perform RTP protocol overhead reduction is up to SA2 work. However, RAN should be indicated by CN that ROHC function is not applied in case UP non-IP based solution is used.

Observation 8: For CP IP based solution, RAN2 may need to further AS overhead reduction (e.g., RRC protocol overhead reduction), to satisfy the requirement of low data rate, and RAN scheduling should be able to efficiently handle the packet with variable packet size generated in different ROHC compression state between UE and MME.

Observation 9: For CP non-IP based solution (without IP/UDP header), how to perform RTP protocol overhead reduction is up to SA2 work, and RAN2 may need to further AS overhead reduction (e.g., RRC protocol overhead reduction), to satisfy the requirement of low data rate.

Observation 10: The overall protocol overhead of UP based solution is lower than that of CP based solution which has NAS protocol overhead with a minimum size of 7 bytes.

Observation 11: For UP based solutions, the existing RAN Function framework can be reused, while new RAN functions (e.g., RRC reconfiguration, SRB management for IMS voice service) should be introduced in NB-IoT CP architecture to support CP based solutions.

Proposal 9: From the RAN2 perspective, RAN2 selects the UP solution with a further down-selection between UP IP and UP non-IP.

Moved here from 9.7.1

R2-2507447	Discussion on SA4 and SA2 LS replies on voice over NB-IoT	Qualcomm Incorporated	discussion	Rel-20	IoT_NTN_Ph4

Proposal 1	Clarify to SA4 that total MAC header of 1 byte is feasible in case UE has only one PDU that fits in the available TBS.

Proposal 2	For UP solution, clarify to SA4 that 4 bytes of RoHC header (not 3 bytes) is more reasonable to consider.

Proposal 3	For CP solution, indicate that RRC layer adds 2 bytes of overhead. If the size of NAS PDU carrying voice packet is fixed based on codec rate, the RRC layer adds just 1 byte of overhead.

Proposal 5	Send LS reply to SA4 that RAN2 recommends avoiding periodicities such as 120ms and 240ms and agree draft LS in [5].

Proposal 6	For Q1, respond to the Question 1 that RoHC implementation is out of 3GPP scope and RAN2 observation is that the decompression failure of one or more packets can result in ROHC state fall back at compressor.

Proposal 7	For Q3, respond in the LS that NB-IoT over GEO channel cannot handle the voice packets of different sizes changing dynamically without significant performance impact on voice service.

Proposal 8	For Q6 and Q7, respond in the LS that it is feasible to support more than 2 DRBs or define a new SRB.

Proposal 9	For Q10, respond in the LS that it is possible for a new SRB, um-Uni-Directional-UL-r15 i.e., in RLC UM mode is configured explicitly or RLC UM is specified as a default configuration implicitly.

Proposal 10	To SA2, RAN2 agree the draft LS reply provided in [6].

R2-2507641	NB-IoT NTN voice over GSO	Ericsson	discussion	Rel-20	IoT_NTN_Ph4-Core

Proposal 1	RLC UM for voice over GSO is supported regardless of whether UP or CP solution is selected.

Proposal 2	The NB-IoT NTN voice over GSO solution shall not break the QoS framework regardless of whether UP or CP solution is selected. That is, all data of one bearer gets the same packet treatment.

Proposal 3	RAN2 supports the UP based solution for NB-IoT NTN voice over GSO.

Proposal 4	Send LS to inform SA2 that RAN2 has selected the UP solution and ask whether there are any major issues with that, see draft in Appendix B.

Proposal 5	NB-IoT UEs without TA reporting capability are supported in voice over GSO.

Proposal 6	Reply to SA4 that a packet overhead of 1 byte of MAC header is not realistic.

Proposal 7	Reply to SA4 that RTP, UDP, IP and Non-IP headers are outside of RAN2 scope.

Proposal 8	Reply to SA4 that RAN2 supports signalling of RoHC profiles, as specified in 36.323 clause 5.5 and 36.331 PDCP-Config-NB IE, but the functions and compression is outside of RAN2 scope. Note that the compression profiles for RTP are not supported but can be added for Rel-20.

Proposal 9	Reply to SA4 that packet overhead for UP solution in PDCP, RLC, and MAC is 4 bytes and packet overhead for CP solution in RRC, RLC, and MAC is 6 bytes.

Proposal 10	Reply to SA2 that RAN2 supports signalling of RoHC profiles, as specified in 36.323 clause 5.5 and 36.331 PDCP-Config-NB IE, but the functions and compression is outside of RAN2 scope. Note that the compression profiles for RTP are not supported but can be added for Rel-20.

Proposal 11	Reply to SA2 that dynamic scheduling can handle dynamic varying sizes, but will add at least one RTT of 542 ms for UL traffic.

Proposal 12	Reply to SA2 that if the size change between two sizes and two periodicities that evenly divides each other, then likely a specially tuned semi-persistent scheduling can support IMS voice without an extra RTT of delay for UL.

Proposal 13	Reply to SA2 it is feasible from a specifications point of view to add more than 2 DRBs per UE in Rel-20.

Proposal 14	Reply to SA2 that it is possible to define a new SRB for voice media but it has the drawback of needing RRC reconfiguration for CP solution which is not supported today.

Proposal 15	Reply to SA2 that it is possible to define a new SRB for voice media with RLC-UM, but it has the drawback of needing RRC reconfiguration for CP solution, which is not supported today, and the CP solution changes are larger compared to the UP solution.

R2-2507137	Discussion on the Support of IMS Voice over NB-IoT NTN Connected to EPC	MediaTek Inc.	discussion

Proposal 1: RAN2 provides to following answers to SA2’s questions to RAN2 in the LS:

-	Q1: The number of consecutive packets lost needed to trigger RoHC state fallback at the compressor depends on the header type used. Up to 62 packets loss may be needed to trigger the fallback.

-	Q3: Semi-persistent scheduling (SPS) can overcome the signalling delay due to long RTT in NTN. The voice packet size variation may cause over-scheduling when SPS is applied, but this should be tolerable to some extent.

-	Q6: It is feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20.

-	Q9: While it is technically possible, we do not prefer designing a new SRB for voice. The main concern is the standardization efforts required to significantly reduce CP header size.

-	Q10: We prefer UP over CP solution.

Proposal 2:	RAN2 specifies that we prefer UP over CP solution and leave the decision between IP and non-IP solution to SA2.

[AT131bis][304][IoT NTN Ph4] Reply LS to SA2 (vivo)

Scope: Discuss the content of a reply LS to SA2

Intended outcome: Draft LS (in R2-2507765) and report of the discussion in R2-2507904

F2F time: FFS

New deadline for Draft LS: Thursday 2025-10-16 11:00

[AT131bis][305][IoT NTN Ph4] Reply LS to SA4 (QC)

Scope: Discuss the content of a reply LS to SA4

Intended outcome: Draft LS (in R2-2507766) and report of the discussion in R2-2507770

F2F time: FFS

New deadline for Draft LS: Friday 2025-10-17 11:00

R2-2507904	Report of [AT131bis][304][IoT NTN Ph4] Reply LS to SA2	vivo	discussion	Rel-20	IoT_NTN_Ph4

Proposal 1: Regarding SA2 Q1, RAN2 does not have the observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC, and thinks the implementation of ROHC functions (e.g., compression and decompression) falls outside of RAN2 scope.

-	QC would prefer to say that RAN2 did not have sufficient time to investigate this for now

-	Xiaomi thinks it’s not about time but competence. Ericsson agrees

-	Aalyria thinks that RAN2 cannot suggest changes to ROHC but thinks we could still have a view on the impact to ROHC performance in this case

Regarding SA2 Q1, RAN2 is not able to make any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC, and thinks the implementation of ROHC functions (e.g., compression and decompression) falls outside of RAN2 scope

Proposal 2: Regarding SA2 Q3, RAN2 responds to SA2 with scheduling methods can handle the voice packets of different sizes (e.g., via semi-persistent scheduling and dynamic scheduling), and RAN2 will further study and discuss the details of the scheduling mechanism to avoid the potential issues.

-	Ericsson thinks this correct only of DL, not for UL. For UL we can either solve this via overprovisioning of resource or accept delays

-	Oppo thinks we should also involve SA4 to know which delays could be accepted

-	Session chair thinks that if we are not sure about the QoS requirements for this case we could ask SA2 if the QoS requirements for Voice over IoT-NTN would be the same as for voice over TN.

Regarding SA2 Q3, RAN2 responds to SA2 that scheduling methods may handle the voice packets of different sizes (e.g., via semi-persistent scheduling and dynamic scheduling, including over-provisioning of resources in UL), but RAN2 will further study and discuss the details of the scheduling mechanism to avoid the potential issues.

Continue if offline 304 to check if we can add a question on QoS requirements for Voice over IoT NTN

Proposal 3: Regarding SA2 Q6, RAN2 responds to SA2 that it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20.

Regarding SA2 Q6, RAN2 responds to SA2 that it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20.

Proposal 4: Regarding SA2 Q9 and Q10, RAN2 responds to SA2 that it is technically feasible to introduce SRB with RLC UM for voice packet, and further discuss whether additional RAN2 impacts should be provided in the reply LS.

-	vivo thinks we should add more about RAN2 impacts in doing so.

Regarding SA2 Q9 and Q10, RAN2 responds to SA2 that it is technically feasible to introduce SRB with RLC UM for voice packet

Session chair wonders if we can reply to SA2 that:

- RAN2 indicates that we prefer UP over CP solution and leaves the decision between IP and non-IP solution to SA2.

# of companies in favour this:

13

# of companies not in favour this:

3

HW comments that the down-selection is expected to happen in December

We add a sentence in the same reply LS saying that majority of companies in RAN2 prefer the UP over CP solution (because of the higher impacts on RAN2 specs of the CP solution). RAN2 also understands that the final decision will be taken at the checkpoint in December plenary.

Agreements regarding the reply LS to SA2:

1.	Regarding SA2 Q1, RAN2 is not able to make any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC, and thinks the implementation of ROHC functions (e.g., compression and decompression) falls outside of RAN2 scope

2.	Regarding SA2 Q3, RAN2 responds to SA2 that scheduling methods may handle the voice packets of different sizes (e.g., via semi-persistent scheduling and dynamic scheduling, including over-provisioning of resources in UL), but RAN2 will further study and discuss the details of the scheduling mechanism to avoid the potential issues.

3.	Regarding SA2 Q6, RAN2 responds to SA2 that it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20.

4.	Regarding SA2 Q9 and Q10, RAN2 responds to SA2 that it is technically feasible to introduce SRB with RLC UM for voice packet

5.	We add a sentence in the same reply LS saying that majority of companies in RAN2 prefer the UP over CP solution (because of the higher impacts on RAN2 specs of the CP solution). RAN2 also understands that the final decision will be taken at the checkpoint in December plenary.

R2-2507770	Report of [AT131bis][305][IoT NTN Ph4] Reply LS to SA4	Qualcomm	discussion	Rel-20	IoT_NTN_Ph4

Proposal 1	Add in SA4 response that no conclusion has been reached yet for down selection of CP/UP and any AS layer optimization (may be updated based on online discussion).

Proposal 2	RAN2 assumes either 1 byte or 2 byte or 3 byte MAC header is possible depending on whether the TB size matches with the grant. However, in most common scenario, total MAC header size is likely to be 3 bytes.

RAN2 assumes the MAC header size may be 1 to 3 bytes. However, in most common scenario, the total MAC header size is likely to be 3 bytes.

Proposal 3	For PDCP and RLC in UP solution and for RRC and RLC in CP solution, indicate header size according to the current specification for NB-IoT (i.e., 1 byte for PDCP, 1 byte for RLC UM, 2 byte for RRC, and 2 byte for RLC AM).

For transmitting voice packets via the UP solution RAN2 expects 1 byte for PDCP + 1 byte for RLC UM + the MAC header size

For transmitting voice packets via the CP solution RAN2 expects 2 bytes for RRC + 2 bytes for RLC AM (the only mode currently specified for CP) or 1 byte for RLC UM (if RLC UM will be introduced for SRB in case a CP based solution will be selected) + the MAC header size

Proposal 4	Discuss whether to indicate to SA4 that minimum RoHC header size is 3 bytes but due to frequent talk spurts, RoHC header size of 4 byte will be used frequently.

RAN2 will provide further feedback on the expected average RoHC header size

Proposal 5	Indicate to SA4 that RAN2 has not started work on SPS for voice but SPS periodicities like 120ms and 240ms periodicities that do not divide 10240 may need to be avoided for NB-IoT.

We indicate to SA4 that RAN2 has not started work on SPS for voice but thinks that SPS periodicities like 120ms and 240ms periodicities that do not divide 10240 would require additional specification work in RAN2 to resolve the issues

Agreements regarding the reply LS to SA4:

1.	RAN2 assumes the MAC header size may be 1 to 3 bytes. However, in most common scenario, the total MAC header size is likely to be 3 bytes.

2.	For transmitting voice packets via the UP solution RAN2 expects 1 byte for PDCP + 1 byte for RLC UM + the MAC header size

3.	For transmitting voice packets via the CP solution RAN2 expects 2 bytes for RRC + 2 bytes for RLC AM (the only mode currently specified for CP) or 1 byte for RLC UM (if RLC UM will be introduced for SRB in case a CP based solution will be selected) + the MAC header size

4.	RAN2 will provide further feedback on the expected average RoHC header size

5.	We indicate to SA4 that RAN2 has not started work on SPS for voice but thinks that SPS periodicities like 120ms and 240ms periodicities that do not divide 10240 would require additional specification work in RAN2 to resolve the issues

R2-2506874	Discussion on supporting IMS voice call over GSO for NB-IoT	CATT	discussion	Rel-20	IoT_NTN_Ph4

R2-2506882	Discussion of NB-IoT voice over GEO	China Telecom	discussion	Rel-20	IoT_NTN_Ph4

R2-2506908	Discussion on UP solution vs. CP solution for voice support over NB-IoT-NTN	CMCC	discussion	Rel-20	IoT_NTN_Ph4

R2-2506912	Discussion on CP and UP solutions for GEO voice	Spreadtrum, UNISOC	discussion	Rel-20

R2-2506919	Considerations on voice support over IoT-NTN	Lenovo	discussion	Rel-19

R2-2506945	Discussion on support of IMS voice call over GSO	Transsion Holdings	discussion	Rel-20

R2-2506982	Discussion on IMS voice over GSO	Xiaomi	discussion	Rel-20	IoT_NTN_Ph4

R2-2506991	Discussion on IMS voice over GEO	CSCN	discussion	Rel-20	IoT_NTN_Ph4

R2-2507039	Discussion on IoT-NTN to support IMS voice call	HONOR	discussion	Rel-20	IoT_NTN_Ph4

R2-2507049	Discussion on voice support over NB-IoT NTN	OPPO	discussion	Rel-20	IoT_NTN_Ph4

R2-2507065	General consideration on voice over NB-IoT NTN via GSO	Huawei, HiSilicon	discussion	Rel-20	IoT_NTN_Ph4-Core

R2-2507085	Comparison of solutions for voice call support over NB-IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-20	IoT_NTN_Ph4-Core

R2-2507125	Discussion on voice over GEO	Apple	discussion	Rel-20	IoT_NTN_Ph4

R2-2507136	Discussion on support of voice over NB-IoT-NTN via GEO	Nokia, Nokia Shanghai Bell	discussion	Rel-20	IoT_NTN_Ph4

R2-2507196	UP Solution vs CP Solution for Voice Support over NB-IoT-NTN	Sharp	discussion	Rel-20	IoT_NTN_Ph4

R2-2507208	Discussion on voice support over NT-IoT-NTN	ETRI	discussion	Rel-20	IoT_NTN_Ph4

R2-2507260	Discussion on how to cupport voice call via IoT-NTN	LG Electronics Inc.	discussion	Rel-20	IoT_NTN_Ph4

R2-2507290	Initial discussions on voice over NB-IoT NTN	Samsung	discussion	Rel-20

R2-2507324	Consideration on IMT voice over IoT NTN	InterDigital Communications	discussion	Rel-20

R2-2507362	Voice over GSO based on NB-IOT NTN	NEC	discussion	Rel-20	IoT_NTN_Ph4-Core

R2-2507448	Discussion on CP and UP solutions	Qualcomm Incorporated	discussion	Rel-20	IoT_NTN_Ph4

## 9.8	E-UTRA TN to NR NTN HO

(LTE_TN_NR_NTN_HO; leading WG: RAN2, Rel-20; WID  RP-252890)

Time budget: 0 TU

Tdoc Limitation: 0 tdocs

# 10	6GR Rel-20 - Study on 6G Radio Access Technology

New SID: Study on 6G Radio; leading WG: RAN1; REL-20; started: Aug. 25; target: May. 27; SID: RP-251881

Time budget: 4 TUs

Tdoc limit:6.   Co-sourced contributions will count towards tdoc limit.

Guidelines:

Proposal limit: 7 proposals per contribution.  Proposals should focus on addressing the issues that should be discussed, prioritized and addressed at this stage of the work (i.e. proposals on how to advance the work and technical areas to address).   Observations to justify proposals, which are copied in conclusion section are recommended.  Contributions should address lessons learned from 5G and justify the need/gains.   Observations and Proposals should fit in one page in conclusion section at the end of contribution (i.e. reasonable length proposals and font size).

Inter-WG and Inter-TSGs issues: Companies are encouraged to identify inter-WG and/or inter-TSG dependencies/decisions that impact RAN2 design.  Intention is to coordinate closely with other WGs and prioritize accordingly.

NOTE: AIs will be further refined after RAN1#131bis

NOTE: As endorsed in RP-252909, section 1, RAN2 will wait for RAN Plenary study on migration options to start after March 206 and wait for further guidance on whether to consider 6G-6G DC and/or NR-6GR DC.  For now the 6G study in RAN2 should focus on standalone deployment and enhancements.

## 10.1	Organizational

Reserved for rapporteur inputs, including work plan, skeleton TR and LSs

R2-2506760	LS on Study on Modernization of Specification Format and Procedures for 6G (SP-251228; contact: Nokia, Samsung, CMCC, ETSI)	SA	LS in	Rel-20	FS_6GSpecs	To:RAN1, RAN2, RAN3, RAN4, RAN5, SA1, SA2, SA3, SA4, SA5, SA6, CT1, CT3, CT4, CT6	Cc:RAN, CT

Delegates are reminded about the ongoing Study on Modernization of Specification Format and Procedures for 6G and encouraged to participate to reflect the needs and ways of working of RAN2.

Noted

R2-2506761	LS on Guidance on 6G data related work tasks (SP-251261; contact: CMCC	SA	LS in	Rel-20	To:SA2, SA5	Cc:SA3, SA6, RAN, RAN2, RAN3

Noted

R2-2506904	Work Plan for 6G SI RAN2	CMCC, NTT DOCOMO, AT&T, Vodafone	Work Plan	Rel-20	FS_6G_Radio

AI/ML use cases are not starting in November

-	Xiaomi asks about NTN as they don’t see it explicitly mentioned.   CMCC explains that NTN Will be discussed as part of the different system functions and not separated.   Interdigital thinks that we need to also agree on how much NTN specific things we need to do and maybe we will know what ambition we have in a few meetings.

-	Oppo thinks that we have a lot of AI for 5G so we should start it a bit later than November.

-	Tmobile thinks that use and deployment scenarios should be discussed in plenary.

-	Nokia agrees that we should keep things in RAN and even new things should go to plenary first.

We should not duplicate discussions from RAN

-	Ericsson asks why there is a gap in security for two meetings as we might need to communicate with other WGs.   CMCC explains is that there may be a point where we have to wait for their inputs after some discussion.

RAN2 will ensure that discussions on security take place when LSs are sent regardless of Work plan.

-	Qualcomm asks about data collection can be postponed at least until February and give SA2 some time to discuss.  CMCC thinks that we can identify a few things in RAN and requirements.

-	ZTE thinks we need to be flexible and not be too static as per workplan, so we should just note.  Qualcomm agrees.

Chair indicates that more sub-agenda items will be created in the future and confirms there is no official parallel sessions for 6G next year.

-	Samsung thinks that for DC migration and DC the RAN can start earlier.

Confirm understanding that for 5G NR to 6GR migrations even if work plan it is starting from August, we may start earlier depending on RAN guidance/assigned tasks.

-	Oppo understands that these features are not exhaustive and more can come up.

Confirm that this is not an exhaustive list of topics/features and the plan will be flexible and adjusted by chair when creating the agenda.

Noted

R2-2506903	Draft skeleton of the TR 38.760-2 Study on 6G Radio RAN2 aspects	CMCC	discussion	Rel-20	FS_6G_Radio

-	Nokia thinks this is aligned with 4G and 5G so it is quite good, but we are missing security and we didn’t agree we have a data framework yet, section 9, so we should remove it.   CMCC explains that it is in the study so this is to just capture the study part of data framework.

-	Xiaomi asks why we need section 8 on identity and also deployment scenarios.   CMCC explains that we had a separate section on 5G for identity.   For deployment scenarios it is just to refer to what we have in the plenary to simplify the discussion in RAN2.

-	Oppo asks why we have both section 9 on data framework and one on section 10.3 on AI data collection.    CMCC thinks that section 9 is for multiple use cases data transfer framework and 10.3 is specific to AI.

-	Oppo thinks that UP sections assumes that we have all the UP stack as it is.  CMCC thinks that this can be updated.

-	LG asks why ARQ and HARQ is a separate section and also multi-carrier framework .  Also mobility can be under RRC.  Also data framework and AI data management shouldn’t be duplicated.

-	Ericsson thinks that this is a bit too early to accept a structure given that we have quite a bit of discussions.   ZTE thinks that we should only discuss skeleton when we are ready with capturing TPs after a whle.

-	Tmobile thinks that service aware RAN will deserve it’s own section and under 6G identities we should add an explanation. Also new section for FWA.   CMCC thinks that service aware will impact specific functionalities so need for special section.

-	Docomo thinks that multicarrier is not needed.

-	Lenovo asks how we plan to capture things.  Mediatek that perhaps the rapporteur can think of initially just capturing agreements in the annex.   ZTE and CMCC thinks that we should follow our RAN2 way of working where not everything is captured, just things that we have consensus and agree to capture.    Nokia and ZTE think that the TR is a stepping stone to stage 2 and we can capture why we did something.    Qualcomm and Samsung agrees we shouldn’t capture everything but there will be some consensus solutions that we can capture, as we need to do some evaluation.  Samsung things that we should be very careful on what we capture in the TR.

-	Qualcomm thinks that it should be AI/ML for radio interface.   Xiaomi agrees and we should capture at least the things that will be useful for normative phase.

The TR skeleton will be revisited and revised after some pre-liminary discussions take place in RAN2

RAN2 will not capture all the proposed solutions in the TR.

Noted

## 10.2	General aspects

Including contributions on principles, guidelines, new services, deployment scenarios and architectures, and other general aspects including UE capability framework, etc.

More detailed aspects related to energy efficiency and power savings can be discussed as part of UP/CP/Common design.

Design principles, services, architecture and requirements for 6GR (1hr)

R2-2506910	General considerations on 6GR	CMCC	discussion	Rel-20	FS_6G_Radio

Critical Issues from 5G

Proposal 3: The following key issues from 5G commercialization must to be addressed:

- Protocol Complexity and Innovation Bottleneck

- The NR RRC configuration and UE capability framework has been criticized as overly complex, difficult to process and maintain on the gNB-UE both side, consuming excessive signaling overhead;

- Fixed and coupled protocol stack causes high complexity, high cost and performance limitation;

-	Xiaomi asks for what protocol stack we are referring to.  CMCC explains it is UP but also CP for modular design.

- Limitation on AI/sensing data collection and model transfer

- Fragmented access control mechanism, including cell barring, UAC and RRC connection rejection;

- Dispersed signaling configurations for mobility/measurement;

- Fragmented design of multi-carrier operation results in inefficient utilization of sparse spectrum, inflexible UL and DL carrier association.

-	Qualcomm thinks it is fragmented spectrum.

Noted

[3mins]

R2-2506949	General aspects for 6G Radio protocol	Samsung, Verizon	discussion	Rel-20	FS_6G_Radio

Observation 7: 5G NR user plane protocol stack can be leveraged as a baseline while exploring enhancements for 6G, focusing on simplified designs, cross-layer optimization, and mechanisms to support immersive services.

Observation 10: RAN2 can begin studying 5G NR-6GR MRSS only after RAN1 makes progress on this aspect.

-	Nokia asks why we should wait for RAN1, we are confident that RAN1 will have it.   Samsung things that RAN1 may need to make some progress for further details.  Nokia thinks we can assume we have it for study item purposes.  Qualcomm and ZTE also thinks that we should wait for RAN1 so we understand what MRSS mean.

Observation 11: Inter-RAT handover with 5G should be handled through the core network. No immediate study by RAN2 is necessary.

Proposal 1: The design principles for 6G Radio protocol should encompass the following:

- Optimized mobility framework to achieve the stringent latency and reliability requirements for emerging interactive and immersive applications;

-	Xiaomi asks if we would like better performance than NR.  Samsung things that we can define the requirements from RAN study but we can design signaling to achieve this.   Apple asks if immersive is referring to XR.   Samsung things yes for now but it can evolve.   Nokia clarifies that we shouldn’t discuss schemes that assume MRSS is not supported.

(2) Optimized power and energy saving technologies to maximize energy efficiency for both UE and NW;

-	Lenovo asks what we mean by optimized, does it mean that 5G is the baseline.  Samsung things that we can reduce the options and how to streamline the protocol.

(3) Implementation-friendly and simplified UP Protocol to support high-speed and immersive traffic efficiently;

-	LG asks what is implementation friendly.  Samsung things we need to keep in mind hardware processing and complexity.

(4) Enhanced AS security to improve system stability by mitigating potential security threats;

(5) Scalable and forward-compatible protocols to accommodate diverse device types

-	Oppo thinks that we should wait for RAN plenary.  Samsung things that the purpose of the proposal is a principle that when we design we should consider there various types.

Proposal 2: RAN2 to await progress on 6G deployment scenarios and 6G RAN architecture from TSG RAN and RAN3 WG to identify associated 6G Radio protocol issues.

-	Vivo asks what deployment scenarios.  Samsung explains that we have to make sure that RAN discussed deployment scenarios.

Noted

[3mins]

---

R2-2506855	General considerations for 6G in RAN2	Huawei, HiSilicon	discussion	Rel-20	FS_6G_Radio

Observation 4-1: Immersive communication services have a composite requirement, which means the high data rate, low latency and high reliability need to be satisfied simultaneously for a minimum number of users per TRP.

Observation 4-2：For the new AI services (e.g., AI agent), transforming data into tokens is becoming a popular approach for efficient transmission and processing compared to transmitting raw input data. Token based communication introduces numerous new requirements and unique characteristics.

Observation 6-1: Device types are being discussed in TSG RAN and RAN1 currently. RAN2 should await and align with the conclusions from TSG RAN and RAN1, and further identify the potential impacts to RAN2.

Proposal 1: RAN2 should maintain an open approach when discussing candidate technologies, supported by thorough evaluation and analysis.

Proposal 2: RAN2 should prioritize discussions on requirements and solutions for the 6G standalone scenario, including enhanced spectrum utilization and aggregation. Discussions regarding requirements and solutions specific to additional migration mechanisms should await decisions from the RAN plenary.

Proposal 4: RAN2's studies should consider the requirements of new application services emerging in the 6G era, such as immersive communication services and new AI services, by considering their characteristics.

Proposal 5: RAN2's studies should consider the requirements of new AI services to be served by networks/operators (e.g. digital twin).

-	Mediatek asks to what extent RAN2 needs to discuss requirements and how much RAN plenary.   Huawei thinks that RAN plenary has some numbers but RAN2 needs to discuss some characteristics of mobile AI traffic.   For AI we can discuss data collection part requirements.

-	Interdigital ask what new requirements for AI services in CN for digital twin, QoS differention, data collection and termination, and sensing.  Huawei confirms.

-	Huawei clarifies that digital twin is a different services.

-	Samsung thinks that RAN2 is not an expert group for AI mobile and data collection, so we need some inputs.   Huawei thinks that RAN2 can discuss the traffic and SA2 is not an expert either.

-	Ericsson thinks that we have 5G A mobile AI traffic characteristics study so how do we handle this.   Huawei thinks that for 6G we also have tokenized traffic to consider.

Noted

[3mins]

R2-2506860	Overview on 6G Radio Access Technology	CATT	discussion	Rel-20	FS_6G_Radio

Observation 1: The 6G network is designed as user-centric network to optimize the user experience, which involves, e.g., UE-specific service subscription, optimal mobility, smart link selection, customized energy saving, efficient control and scheduling.

Observation 2: 6GR design should consider some important KPIs, e.g., energy efficiency, spectrum efficiency, service continuity, latency and security.

Observation 5: 6G shall be able to support most of existing 5G services, including positioning services.

Observation 7: The RAN is responsible for determining the UE positioning methods to be supported within its functional scope.

Observation 13: Service Awareness is beneficial for addressing the data characteristics and service requirements of immersive communication, making it suitable as the information foundation for service transmission and scheduling.

Observation 14: RAN2 should consider the impacts of adaptive QoS introduced by immersive communication.

Observation 15: Multi-modality and other potential issues (e.g., FEC mechanisms) should be discussed in SA2 and RAN from the scratch.

Proposal 1: From RAN2 point of view, the design target of 6GR should take into account:

- User-centric network design for optimized user experience;

- Substantial gains in term of KPIs, e.g., energy efficiency, spectrum efficiency, service continuity, latency and security;

- Simplicity and efficiency design;

- Harmonized and integration of TN and NTN.

Proposal 3: In order to support location service in 6G, RAN2 further study positioning-related functions, architecture, and procedures, with collaboration from SA2 and RAN3 as needed.

-	Xiaomi thinks that RAN is discussing positioning so should we wait.   Apple also thinks that it is not clear what we would discuss.  CATT thinks that we can discuss the 6G architecture.   Nokia thinks that we need to address the emergency call requirements and asks if RAN2 should lead the architecture, but we anyways need to confirm with SA2.  CATT thinks that we can work closely with SA2 and RAN3 how to support positioning.

Proposal 5: For immersive communication, the following aspects can be considered: service awareness, adaptive QoS, multi-modality, etc.

Noted

[3mins]

---

R2-2507070	How to make the best possible 6G	Ericsson	discussion	Rel-20

Proposal 1	For 6G, RAN2 should discuss and define the problem to understand if a solution is needed. If it is, only the best solution based on performance evaluations should be introduced.

-	Apple asks what evaluations we are referring to.   Ericsson is not referring to simulations but more a pen and paper analysis.   We should understand the real problem we are addressing before we start discussing the solution.  Nokia thinks this is a good principle and keep it in mind.   ZTE thinks that we also need to consider implementation complexity.   Ericsson agrees.

Proposal 2	3GPP should:

- avoid specifying several solutions/options for the same problem

- avoid overly complex solution, for example if 80 % of the performance comes from 20 % of the complexity, 3GPP should not chase the last 20% of the gain by adding the additional 80 % of the complexity.

- avoid specifying solutions which do not have a clear market demand, for example RAN2 should not do endless enhancements on top of a functionality that has not been implemented.

Proposal 3	Specify a single unified protocol stack for 6G addressing the targeted 6G use cases sufficiently well, where different use cases are supported by different configurations of the same unified protocol stack. Aim to limit use case-specific functionality. Do not specify any unjustified optimizations.

-	Qualcomm thinks that unified and modularization don’t work together.  Ericsson thinks that we shouldn’t over optimize for things that will not be implemented.

Proposal 4	RAN2 should develop a protocol stack to maximize performance over the Uu interface. RAN2 should not make an inferior design to accommodate for not-yet-agreed deployment scenarios, architectures or RAN internal interfaces.

Noted

[3mins]

R2-2507079	6GR Design	Nokia	discussion	Rel-20	FS_6G_Radio

Proposal 1: 6GR shall be able to fulfil all regulatory requirements on emergency voice calls, positioning and PWS.

Proposal 2: 6GR shall support FWA in an optimised manner.

-	Vivo asks if this a RAN2 domain and what we can discuss.   Nokia thinks that we want to be able to identify these types of devices from the RA procedure.

Proposal 3: 6GR shall follow a bottom-up approach where a baseline addressing the minimum shared requirements across all services is first agreed, and only then additional functions can be added on top to address more advanced use cases and services.

-	Oppo thinks that for 5G we designed the services for eMBB.  Is the proposal proposing that we identify all services and then design.   Nokia explains that the bottom capability is IoT devices and not eMBB, so we shouldn’t have a design that would require removal of features to support a new feature like IoT.

-	Mediatek thinks that these proposals are philosophical ideas but not sure what would be agreable for them.

-	Lenovo thinks that this interesting approach as it is more like a modular approach.

-	Ericsson thinks that this is from the plenary that we will have the minimum capabilities.   For unified approach we just meant to not specify features for niche use cases and focus on features that apply to many use case.

-	Fainity explains that RAN1 is also discussing the baseline and wonders if we should wait for RAN1 progress.   Nokia thinks that we can work in parallel and we have an understanding on what minimum capability would mean.   We should discuss in RAN2 on what is the minimum set and then feed it into RAN plenary.   For example CA is not part of minimum capability.   RAN4 is also discussing this.

-	Huawei thinks that we should analyze the real requirements and then discuss functionality needed rather than taking 5G as a baseline.

-	ZTE thinks that we are confusing minimum capability and system configuration, we need to separate these two.  RAN2 design should be modular and configurable from system configuration perspective.   Nokia agrees in theory it should work like this but in practice it didn’t happen like this.  The configuration should assume from the beginning.  ZTE thinks that we need RAN1 help to design the minimum initial access requirement.

-	Xiaomi thinks that we should have a discussion on minimal capability from RAN2 perspective, e.g. what is a basic feature.

-	CMCC thinks that the minimum capability set would also help us see if we can simplify the signaling structure.

-	Vivo thinks that such design should not give a negative impact to eMBB, this is an important requirement.

Proposal 4: 6GR radio protocols shall make it possible to implement parallel pipelines, burst-efficient data transfers, and zero-copy handling, all while keeping real-time workloads minimal.

-	LG thinks that even with current specifications we can have zero-copy handling via implementation.  Nokia thinks that 5G already allows that but the worry is that we end up with a design that no longer achieves this.

Proposal 5: 6GR uses the sub-layers of Layer 2 of 4G/5G as model baseline. Any consideration of merging sub-layers should stem from a thorough functional analysis, rather than being pursued as an objective in its own right.

Noted

[3mins]

Not treated:

R2-2507205	Views on 6G general aspects	NTT DOCOMO INC..	discussion

Observation1: RAN will start the study in March 2026 and will make a decision by September 2026 whether expand WG SI scope to cover additional migration option(s). RAN will also task relevant RAN WGs for any specific technical analysis.

Proposal1: RAN2 study should be based on the above 6G RAN architecture and migration requirements captured in TR 38.914.

- The 6G RAN architecture shall support standalone RAN architecture.

- The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

- The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

- The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

- The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

- The 6G RAN architecture shall allow for control plane and user plane separation.

- The 6G RAN architecture shall support sharing of the RAN between multiple operators.

- The 6G RAN architecture shall allow for the operation of network slicing.

- The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

- The 6G RAN architecture shall support enhanced service awareness in RAN.

- The 6G RAN architecture shall allow non-public networks.

Proposal2: RAN2 should wait for liaison from RAN regarding additional migration options study.

[3mins]

R2-2507371	Overall framework for 6G from higher layer perspective	NEC	discussion	Rel-20	FS_6G_Radio

Proposal 1: RAN2 aim to support the multiple services by the single technology framework based on the SA architecture, with some exceptions which require distinctive design, e.g. MBS, sildelink.

Proposal 2: RAN2 aim to support the different types of deployment scenarios including coverage node/cell, capacity node/cell, and wider coverage node/cell (NTN) from Day1.

Proposal 3: RAN2 study a feasibility of a new node type with zero always-on signal but uplink Rx is on for being adaptively waken-up

Proposal 4: RAN2 basically waits for RAN3 progress on RAN internal functional split. If the existence of RAN internal functional split would impact on radio protocol, RAN2 should be able to initiate the corresponding study.

[2mins]

R2-2507303	Design principles for 6G	ZTE Corporation, Sanechips	discussion

Proposal 1:	RAN2 study should first focus on the required functionality in 6GR, and only once the functionality is stable, the overall protocol layer architecture and the distribution of functionality across the different protocol layers should be studied with an aim to avoid inter-layer redundancy/overhead for similar functionality

Proposal 2:	Unique functionality should be developed for satisfying a given set of similar 6GR requirements by avoiding defining multiple solutions to satisfy requirements which are similar (e.g., mobility and multi-carrier operation).

Observation 1: Optimal support of CU/DU split and DC (including inter-RAT DC for migration) will have substantial impact on RAN2 protocol design

Proposal 3:	Interim RAN2 decisions on overall protocol design (before the check points for DC and CU/DU split) should ensure an optimized operation for non-DC and non-CU/DU split architecture whilst ensuring forward compatibility with optimal support for both DC (MR-DC / 6G-DC) and CU/DU split.

Proposal 4:	For 6GR, RAN2 study shall cover the use cases including NTN, HRLLC, SON/MDT, Slicing, Relays (including repeaters and RIS), UAV, HST, AI/ML and Sensing scenarios

Proposal 5: 	For the existing use cases in 5G, RAN2 study should identify at an early stage, functionality that can be reused from 5G as a baseline to minimize the additional work in these areas (e.g. for SON/MDT, TSN, Slicing – depending on SA2 output etc) and focus on functionality that needs to be studied from scratch for 6G (e.g. Sensing)

Proposal 6:	The RAN2 protocol architecture for 6GR should support separation of CP/UP data transfer and functionality to transport signalling (Radio signalling and NAS signalling) and data (UP data, data for sensing/AI). The design should allow termination points for both data and signalling to be either in RAN or in CN.

Proposal 7: 	The support of shared SSB in MRSS will impact the design in RAN2 (e.g. for system information and initial access). RAN2 should postpone the discussion on MRSS-specific enhancements (e.g. shared reference signal or common signaling) pending input from RAN1.

[5mins]

R2-2506950	General aspects on RAN2 6G	OPPO	discussion	Late

Proposal 1	For principles of 6GR RAN2 designs, consider new aspects, e.g., minimalist and essential design, eliminating redundant functions, enhanced security, native support for new emerging service, energy efficiency, on top of those which can be re-used from 5G NR principles. Endorse the following TP, as the principles/guidelines:

Proposal 2	R2 study 6G RAN architecture that is able to aggregate spectrum of 1) lower frequency which is to offer robust connection for coverage, together with 2) higher frequency which is to offer on-demand connection for capacity, covering both collocated and non-collocated scenarios.

Proposal 3	R2 study 6G spectrum aggregation architecture improvement on top of 4/5G design, e.g., in terms of 1) improved spectrum efficiency, 2) unified design of various aggregation solution(s), 3) dynamic load balance, and 4) improved link robustness. FFS on the applicable scenario (intra/inter-band, (non)collocated, RRC states and etc.)

[3mins]

R2-2507387	General considerations on 6GR	China Unicom	discussion	Late

Proposal 3: During RAN2 study on support AI and sensing services, it ompon be considered that reusing the current computing infrastructure and stuty solutions for different distributed options of computing resources.

Proposal 4: As a comprehensive use case of AI/ML and sensing, digital twin should be further studied by RAN2.

[2min]

Non-Terrestrial Networks (NTN) and 6GR (1hr)

TN/NTN harmonization and NTN capabilities

R2-2507644	6G Radio Access Technology general aspects for NTN	THALES, Airbus, Echostar, Novamint, Fraunhofer IIS	discussion	Rel-20	FS_6G_Radio

Proposal 1	Consider the support of a harmonized 6G radio interface design that supports further integration of terrestrial and non-terrestrial networks

-	Qualcomm ask what do you mean by further integration.   Thales thinks it is more integrated and for example design procedures that are applicable to both TN and NTN to allow smooth mobility.

Proposal 2	Study the 6G radio interface/access to support multi-orbit architecture and all the practical NTN deployment scenarios, orbits, and related service link characteristics in the table above.

Proposal 3	The 6G radio interface/access shall be defined to support the different NTN deployment architectures, e.g. transparent and multiple regenerative payloads with different 6G system functional splits between on board/on ground NTN nodes

Proposal 4	The 6G Radio design shall be able to support NTN with all the duplex modes, i.e. FDD, TDD and HD-FDD at the UE and the network level

Proposal 5	Study the support of the following services for NTN with 6G Radio

- Positioning, Navigation and Timing (PNT) services.

- Broadcast services over an intended area.

- Multicast services to a group of user equipment distributed over an intended area.

- PWS (Public Warning System) services over an intended area.

Proposal 6	RAN2 to study in priority the following NTN capabilities and services for the 6GR as part of the Rel-20 :

- GNSS independent NTN operation

- Extended coverage

- Flexible duplex mode support at UE level

- Support of HD-FDD at Network side

- Massive messaging capability

- Positioning, Navigation and Timing

- Enhanced network verified UE location service

- Seamless TN/NTN mobility in connected mode

- 6G NTN coexistence with IoT-NTN and NR-NTN

- ICAS (Integrated Communication And Sensing)

- Broadcast , multicast services

-	CMCC thinks that these are RAN level requirements.   Thales thinks that this are aspects that impact RAN2.

Noted

[5mins]

R2-2506992	Discussion on the general aspects of 6G NTN	CSCN	discussion	Rel-20	FS_6G_Radio

Observation 1: 	In the scenario of multi-beam cell, the cell may have irregular shape (i.e. non-circular) due to the inclusion of multiple circular beams within it.

Proposal 1:	RAN2 to study the impacts on the irregular shape of NTN cells, for example, mobility.

Proposal 2:	RAN2 to study a new case 2 with cell coverage in two consecutive periods partly overlap in multi-beam deployment.

Proposal 3:	RAN2 to study a fine-grained signaling design for 6G NTN.

Proposal 4:	RAN2 to study the impact of in-line interference between GEO and NGSO, for example, the blind zones.

Proposal 5:	RAN2 to study energy-efficient design for 6G NTN (e.g., SSB periodicity extension, on-demand SSB).

Noted

[2mins]

R2-2507138	Consideration of 6G NTN	MediaTek Inc.	discussion

Observation 1:  HARQ feedback disabling is useful for 6G-NTN.

Observation 2:  The SMTC timing adjustment could be further studied after measurement gap discussion.

Observation 3:  Low overhead access methods, e.g., RACH-less HO/CHO, should be studied for 6G-NTN.

Observation 4:  Time-based/Location-based mobility mechanisms are useful for 6G-NTN.

Observation 5: 	Cell-reselection or re-direction assistant information for TN-NTN mobility is useful for 6G-NTN, e.g., TN coverage information in an NTN cell and NTN-related information in a TN cell.

Observation 6:  Satellite switch with re-sync (including hard switch and soft switch) is useful for 6G-NTN.

Observation 7:  For initial satellite deployment and provision of IoT service, discontinuous coverage should be considered for 6G-NTN.

Observation 8:  Store-and-forward should be considered for 6G-NTN.

Proposal 1:	RAN2 should study the 5G-NTN specific features and evaluate which features should be included for 6G-NTN.

Proposal 2:	Among 5G-NTN features, at least the following shall be considered for 6G-NTN:

-	HARQ feedback disabling for DL/UL

-	Low overhead access (e.g., RACH-less like), in particular for mobility

-	Time-/Location-based mobility mechanism

-	Assistant information for TN-NTN mobility

-	Satellite switch with resync

-	Discontinuous coverage

-	Store and forward

Proposal 3:  RAN2 should start to study the new 6G NTN deployment scenarios/requirement after approved by RAN plenary.

Proposal 4:  Any 6G feature specific to NTN should be motivated with analysis.

Noted

- [4mins]

Discussion

-	Vodafone asks that we should discuss the TN design and then understand what would need to be done for that specific functionality to support NTN while we are discussing TN in the same release.    What is specific NTN related energy saving that are not going to be designed for TN, so if we design a TN specific energy efficient we should just take it.

-	Qualcomm agrees with Vodafone, also we don’t even know what the baseline TN looks like yet.

-	CATT thinks that we need to harmonize the design.   Oppo thinks that we should have common design and for NTN specific features we should do it later.   Nokia agrees with Oppo.

-	CMCC thinks that we can discuss and consider TN and NTN within the specific function to make sure it meets the scenarios.   Xiaomi agrees and we should learn from 5G.

-	Ericsson thinks that we can consider bringing forward the problems we want solve and then discuss the solutions.

-	Samsung also agrees with Qualcomm and even in RAN1 they don’t start discussing until February.  Thales’ paper has a lot of additional NTN specific enhancements but perhaps we can take baby steps on the essential features like in Mediatek’s papers.

-	LG agrees with CMCC.

-	Huawei agrees we should have a harmonized design but the question is how we approach it.   We should understand the essential feaure/requirements.   ZTE agrees and there are common requirements and we can discuss the TN design and understand whether something different needs to be done for NTN.   Vivo agrees with ZTE and Huawei, and we need to figure out what requirements we need to consider in harmonized design, for example for large RTT we can have larger timers.   For NTN specific enhancement are needed.

-	Dish agrees with CMCC and we should make sure that the configurations are considered for NTN from the beginning.

-	Sharp also thinks a common design is important and TN should be the baseline.

-	Interdigital also thinks common design is important, we should discuss what NTN specific things we would consider with TN, like what Mediatek has proposed.  Vodafone also thinks we need to identify what are essential requirements.

-	Apple, Honor,  also thinks that for next meeting we can let people identify what is common with TN and what is NTN specific and whether it is an essential requirement.

=>	For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the  common design with TN and what are NTN specific and whether they are essential.

UE capabilities for 6GR (1hr)

Lessons learned and requirements

R2-2506773	General Consideration on 6GR UE Capability	Xiaomi	discussion	Rel-20	FS_6G_Radio

Lesson learnt from 5G (1): Due to the late start of UE capability discussion, there’s limited time for cross-WG coordination for an efficient and proper UE capability design (e.g., postpone of supporting a useful feature, unclear definition of granularity, etc).

Proposal 1: Study UE capability management, framework, signalling framework design as early as possible with tight and efficient coordination cross WGs (RAN1/4, RAN3/SA2).

Lesson learnt from 5G (2): Incomplete UE capability management solution in Day 1 may bring drawbacks to later releases, e.g., duplicate reporting over air interface, forward-compatibility issue, etc.

Proposal 2: Study UE capability management (e.g., entities storing UE radio capability, UE capability exchange between gNB and CN, UE capability exchange between gNBs, etc) based on the following principles:

-	Support forward-compatibility for UE capability exchange between nodes;

-	Minimize the frequency of UE reports its full capability during UE’s mobility;

-	Minimize duplicate reporting of the same UE capability over the air interface.

Consider 5GNR UE capability management and retrieval framework as starting point for 6GR, including:

-	Storing UE static capability at CN;

-	RAN obtains UE capability from CN when available;

-	RAN obtains UE capability from UE only when CN stored UE capability is not available or not complete.

-	Lenovo ask how the RAN knows that CN capabilities are not complete.   Xiaomi thinks that we can solve it during study phase.

-	Samsung things that we can discuss this in later phase.  Xiaomi explains that we should learn from 5G and start early discussions so other WGs can trigger the work.

-	Mediatek thinks that this has SA2 dependencies.  Xiaomi thinks that RAN2 has the lead on capability so we have understand dependencies with other WGs and then trigger.

Lessons learnt from 5G (3): During 5GNR practice, there are some cases that network configurations are mismatch with UE capability (e.g., some capabilities don’t have any configuration defined, some capabilities are reported unnecessarily).

Lesson learnt from 5G (4): Temporary restriction on UE capability reporting in 5GNR misused UAI without clear network behaviour specified, which raises a risk for those capabilities being ignored by the network.

Proposal 3: UE capability framework design should follow the below principles:

-	UE capabilities with signalling should be defined only if there’s a corresponding configuration provided by the network;

-	UE should be aware of network supported features before UE capability reporting;

-	UE should be allowed to perform AS-triggered UE capability update without network request, instead of using UAI;

-	6GNR UE capability granularity level should support sufficient UE flexibility based on the real deployment needs. 5GNR UE capability granularity levels are considered as starting point for further analysis;

Lesson learnt from 5G (5):

-	Unnecessary political discussion on mandatory with capability signalling and optional with capability signalling.

-	Complex approaches of defining reduced capabilities requires readers to have the basic understanding of the capability framework, in order to understand what capabilities are supported/not supported for different features/device types (if any).

Lesson learnt from 5G (6): 5GNR introduced different mechanisms to reduce capability message size, but didn’t efficiently and fundamentally solve the issue.

Proposal 4: Study 6GR UE capability signalling framework in the following aspects:

-	Signalling categories (i.e., mandatory/optional with/without capability signalling) for 6GR, aiming for 1) a harmonized design for minimum mandatory capabilities of different device types (if any), 2) easy readability to understand what capabilities should be supported in order to support a device type (if any)/feature;

-	Simplified method to reduce UE capability message size (e.g., signalling simplification, overhead reduction, avoid unnecessary capability reporting, etc);

Lesson learnt from 5G (7): Different components of UE capability are spread across different specifications, making it difficult to connect between signalling structure and field description and difficult to maintain for cross-spec alignment.

Proposal 5: Study the specification structure between 331/306/822 based on the following principle:

-	Avoid split between ASN.1 and field description;

-	Easy approach to identify capabilities (mandatory/optional) introduced for different features/device types (if any) in each release.

Lesson learnt from 5G (8): 6GR should avoid introducing multiple solutions for feature/device type (if any) early identification.

Proposal 6: Based on 6GR initial access procedure, study 1) how and from which step to differentiate device types (if any) within the harmonized system framework; 2) a unified and scalable approach for early feature identification with different purposes (e.g., early service identification, early capability acquisition, early information acquisition during state transition).

Lesson Learnt from 4G (9): Defining UE categories only by achievable throughput is not only challenging to introducing new UE categories in later releases due to marketing competition and singalling overhead, but likely to leave market fragmentation as well.

Lesson learnt from 5G (10): Introducing the lower end device type later than commercial devices limits the support of low-end device type.

Proposal 7: RAN2 to study higher layer parameters/factors of minimum mandatory capabilities.

Noted

Discussion

-	Apple asks how we will solve these problems with the other WGs so we should have the study early on so we can inform other working groups.

[5mins]

R2-2506988	Considerations on UE capability signalling in 6G	Qualcomm Incorporated	discussion	Rel-20	FS_6G_Radio

Observation 1:	Band combination related UE capability parameters are the main contributor to UE capability size increase.

Observation 2:	The size of featureSetCombinations (merely containing pointers to feature sets) becomes prominent factor when the number of supported band combination increases.

Observation 3:	The Feature Set scheme serves the intended purpose to reuse the same set of UE capabilities across different band combinations.

-

Observation 4:	Roughly the half of RF-parameters is baseband related UE capability parameters, which would be reuseable across band combinations.

Observation 5:	“Multi-dimensional” aspect of UE capabilities, such as CC BW and MIMO-layers, leads to the advertisement of a large number of feature sets within each featureSetCombination (corresponding to one featureSetCombinationId).

-	 Qualcomm explains that this increases the number of features set combinations it supports.  Ericsson hasn’t observed that many.

Proposal 1:	RAN2 to study solutions to reduce UE capability signalling overhead, considering the limitations of air interface, network interfaces and network storage.

-	Xiaomi asks what is meant by network storage as network stores the UE capabilities.  Qualcomm explains that we introduced this segmentation but network cannot support all segments.

Proposal 2:	RAN2 to study more generic solution allowing the UE to update its radio capabilities within connected mode.

-	Nokia asks if this is for any UE capability or just within the UAI scope.   Qualcomm would like to apply it to all.   CATT asks if this would introduce requirements on the network side.  Qualcomm’s preference is to have some network behaviour to honor UE capabilities.

-	BT asks what is different with UAI.  QC wants to avoid UAI and have a solution that can support any type of use cases.   The goal is to have something that when the UE changes capability the network needs to adapt.

-	Vodafone asks what happens when the connection changes, do these capabilities disappear.  Qualcomm thinks that the UE capability stored in the network shouldn’t change.

-	Nokia asks if the change is only applicable to RAN or also to the CN.   QC explains it is only RAN.

-	Oppo asks if the UE has already changed the capability or it plans to change it.   If we change it already then current configuration will not work.  QC agrees but we can follow the MUSIM solution.

-	Ofinno thinks whats important is a clear network behaviour when capability changes.

-	Transsion wants to clarify for what type of UEs.   QC wants to make it generic so we don’t need to discuss every use case.    ZTE doesn’t agree that this should apply to anything as this will cause a lot of complexity for the network.

-	Samsung would like to solve the issue.

-	Huawei thinks that if it is too dynamic it will cause network issues

Proposal 3:	RAN2 to study solutions allowing the UE to activate a feature capability after IODT with a given set of network vendor, while avoiding inter-operability problems due to the lack of IODT with other network vendors.

-	CMCC how RAN2 would do this as this is RAN5.   Qualcomm thinks we can discuss solutions later.

-	Oppo thinks this proposal is in a good direction.

-	Lenovo asks how this is supposed to work.   QC explains that the solutions are not all clear but we can ause something like RACS.

-	Huawei thinks that this should be discussed in the plenary.

-	Ericsson also thinks that we should look at some IODT problems and then if we can’t agree we can bring it to the plenary.  CMCC thinks this is also related to plenary.  Samsung thinks we can discuss in RAN2.

Proposal 4:	RAN2 to study RACS and its enhancements in coordination with SA2/CT1.

-	Fraunhofer asks why RACS wasn’t widespread as a solution.   Qualcomm explains that it is because it was added in Rel-16, so if we do it from day one we get a chance to get deploy.

-	Docomo is highly interested in RACS.

-	Ericsson thinks it reduces the amount of time it has to update the capabilities, but this is how it should work with a properly configured CN.  Ericsson thinks the problem is signaling the same things all the time and the unnecessary repetition.   UEs don’t make use of the flexibility that we have designed with the signaling.   They usually support same set of features in a band for example, and we don’t need to again and again repeat MIMO layers etc on those bands.

-	Xiaomi thinks that we have to solve the issue of per band combination and work with RAN1/RAN4.  WE need to understand what is the critical problem.

-	Huawei thinks that RAN2 should study these different aspects of signaling.

-	Interdigital also thinks that we should prioritise what we study first, as some aspects need progress right away if dependencies with other groups.

Noted

[3mins]

R2-2507126	Initial considerations on UE capability framework in 6G	Apple	discussion	Rel-20	FS_6G_Radio

Observation 1: UE capability size and handling have become very inefficient due to the philosophy of pre-transferring of all the UE capabilities even when the NW might not use many of them.

Observation 2: NR has supported equivalent dynamic UE capability update in multiple topics via UAI/RRCReconfigurationComplete procedure.

Observation 2: NR has supported dynamic UE capability update in multiple topics via UAI/RRCReconfigurationComplete procedure.

Proposal 1: RAN2 should study how to reduce the signaling overhead by avoiding pre-provision of static configuration, and study efficient transfer of context aware capability transfer.

Proposal 2: RAN2 should study dynamic change of capability to make UE capability size manageable and effective.

-	Apple proposes that the starting point for use cases are the UAI cases and need for gap.   Xiaomi thinks that the framework should be applicable to all, but of course we can discuss use cases like the current one and future ones.   For example, we can consider the temporary capability change as a starting point of use cases, e.g. MIMO layer, BW, etc.

Noted

[2mins]

R2-2507607	Consideration on 6G UE Capability	ZTE Corporation	discussion	Rel-20	FS_6G_Radio

UE Capability Reporting Mechanism

Observation 1: Multiple UAI-based functionalities for UE radio capability update have been commercialized in 5G networks.

Proposal 1: The basic UE capability reporting procedure can be retained from 4G/5G, where the UE reports capabilities based on a network request. RAN2 to study how to optimize the filtering mechanism to enable additional granularity for the filtering of UE capabilities.

Proposal 2: Regarding dynamic capability updates, RAN2 should first discuss whether there are new scenarios in 6G that require enhancements to the current 5G UE capability update mechanism.

Proposal 3: For the 6G UE capability reporting, the UE should be able to report capabilities using both explicit and implicit methods. RAN2 to study how to optimize the RACS mechanism in 6G to improve its feasibility.

Proposal 4: Define different device types, with mandatory features specified for each device type per release version.

Proposal 5:  RAN2 to study more flexible structure for RF/baseband decoupling and DL/UL decoupling.

Proposal 6: For the 6G UE Capability structure, RAN2 to study how to avoid repeated reporting problem in the 5G.

Observation 7: Because of some problematic UEs, the network has to adopt conservative settings or disable the new features entirely for all of the UEs to prevent widespread issues, which can significantly hinder new feature commercialization.

Proposal 7: RAN2 should discuss mechanisms to enable network identification of UE/chipset information in 6G.

-	Xiaomi thinks that the information is already in the CN so it can be shared with RAN. ZTE explains that it is difficult to achieve. Tmobile explains that this is an important problem in 5G and we should enable it in 6G. Qualcomm thinks that this is going into the solution domain. Apple thinks that we have similar issues on the device side so we should be open to provide information on both sides.

-	Samsung thinks that the solution is clear but not really the problem.

Noted

Agreements on UE capabilities for next meeting

For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities. Understand if there is impact to other WGs.

Companies can focus on the practical IODT problems in the field. Possible ways to address them.

[3mins]

ISAC discussion

R2-2506772	General considerations on RAN2 6G study	Xiaomi	discussion	Rel-20	FS_6G_Radio

Observation 5: In Rel-20 5GA ISAC SI, RAN3 starts the study on architecture, procedure in parallel with RAN1 without waiting for RAN1 input.

Observation 6: In Rel-16 NR positioning SI, RAN2 is the primary WG to lead the study on architecture, procedure in parallel with RAN1 without waiting for RAN1 input.

Lesson learnt from 5G (6): RAN2 is the primary working group (WG) responsible for studying the architecture, procedures, and protocols of NR positioning, working in parallel with RAN1.

Proposal 5: In 6G RAN2 study on ISAC, the role split should be:

-	RAN2 leads the study on overall architecture, procedure for Sensing, in coordination with SA2, RAN3

-	RAN2 is responsible for the protocol between UE and SF

-	RAN3 is responsible for the study on interface/protocol between RAN and CN and internal RAN architecture, interface and protocol

-	RAN2 can start the study on sensing architecture and procedures in Feb meeting, including: sensing function location; sensing node selection/management and procedure, etc., without waiting for RAN1 progress.

Noted

-	Lenovo asks about what sensing mode we support and when the sensing function is located and depending on where it is located then it would change whether RAN2 should lead the discussion.   Xiaomi thinks that SA2 is discussing the architecture.

-	Mediatek thinks that in RAN2 we are going to make some assumptions given how SA2 works and the time it may take to make a decision.   We assume the SF is in the CN somewhere.  Xiaomi doesn’t agree with the assumption as there are two possible assumption.  Interdigital thinks that we can progress some part of the work without knowing where it located.   We can assume flexibility of where the termination point may be and this is also related to the data transfer discussions.

-	Mediatek is concerned that RAN3 doesn’t have a responsibility in study for now. RAN2 should work on sensing function and sensing entity and start in a harmonized way.

-	Apple agrees that RAN2 leads the study on overall architecture, procedure for Sensing, in coordination with SA2, RAN3, similar to positioning.

-	Ericsson thinks that leading word is a bit misleading, as if it is gNB based it is RAN3, but if it is with UE involvement then it has to be RAN2. The main question is when do we start.  Starting in April is good enough. Ericsson thinks we should first discuss whether there is a need to have a protocol between the UE and sensing function in network. MEdiatek thinks that this is an agnostic question without having any dependencies on other groups.

-	Docomo would like to understand what is the separation between the 10.3.3 and this agenda.

-	Oppo would like to see some discussion in SA2 and RAN1.  Mediatek is ok to start in April but we shouldn’t assume that we will decisions from SA2.

RAN2 will start sensing study in April 2026

Not treated

R2-2506767	General discussion in 6G	Transsion Holdings	discussion

R2-2506773	General Consideration on 6GR UE Capability	Xiaomi	discussion	Rel-20	FS_6G_Radio

R2-2506797	Considerations on 6GR general aspects	vivo	discussion	Rel-20

R2-2506806	Views on 6GR	Fainity Innovation	discussion

R2-2506893	6G General Aspects	Sharp	discussion	Rel-20	FS_6G_Radio

R2-2506917	General considerations on 6GR	Spreadtrum, UNISOC	discussion	Rel-20

R2-2506975	Enhancement of Public Safety Support	Fraunhofer IIS, Fraunhofer HHI	discussion

R2-2506988	Considerations on UE capability signalling in 6G	Qualcomm Incorporated	discussion	Rel-20	FS_6G_Radio

R2-2507132	General considerations on 6GR	Fujitsu	discussion	Rel-20	FS_6G_Radio

R2-2507141	Consideration on general aspects for 6G	LG Electronics Inc.	discussion	Rel-20	FS_6G_Radio	Withdrawn

R2-2507147	On 6GR UE capability	MediaTek Inc.	discussion	Rel-20	FS_6G_Radio

R2-2507176	Discussion on general aspects on RAN2 study for 6GR	Lenovo	discussion	Rel-20	FS_6G_Radio

R2-2507184	Scenarios and architectural principles for 6G RAN2 design	Ofinno	discussion	Rel-20	FS_6G_Radio

R2-2507185	UE capability framework and key features for 6G	Ofinno	discussion	Rel-20	FS_6G_Radio

R2-2507201	Discussions on General Aspects of 6GR Layer 2	Futurewei	discussion	Rel-20

R2-2507307	General consideration on 6GR	ITL	discussion	Rel-20

R2-2507312	Design of 6GR Radio Protocols	InterDigital	discussion	Rel-20	FS_6G_Radio

R2-2507319	Discussion on 6GR Rel-20 general aspects	Sony	discussion	Rel-20	FS_6G_Radio

R2-2507340	Consideration on general aspects for 6G	LG Electronics Inc.	discussion	Rel-20	FS_6G_Radio

R2-2507361	Single Frequency HetNet Deployment Scenario for 6GR	Jio Platforms	discussion	Rel-20

R2-2507393	Discussion on general aspects for 6GR	TCL	discussion	Rel-20

R2-2507450	6G AI/ML Data Collection Requirements 	T-Mobile USA Inc.	discussion	Rel-20	FS_6G_Radio

R2-2507502	Consideration of general aspects and principles for 6G study 	Kyocera 	discussion	Rel-20

R2-2507506	Views on 6GR design principles and strategies	ETRI	discussion

R2-2507511	Initial Considerations for 6GR Access Technology	Hanbat National University	discussion	Rel-20

## 10.3	Radio protocol architecture

#### 10.3.1	User plane

Identification of essential user plane functions and considerations for user plane architecture.

UP architecture and functions

R2-2507250	6G Radio protocol architecture - User Plane Aspects	Lenovo	discussion	Rel-20	FS_6G_Radio

Observation 1: The 5G protocol stack has evolved with new optimizations and features added across various releases resulting in the duplication of functionalities across different protocol layers, leading to an overall complex protocol behaviour and inherent inefficiencies. For 6G, the aim should be therefore to streamline the protocol stack compared to 5G, by removing functionalities which are found to be inefficient/unnecessary or redundant.

-	Docomo asks what functions are duplicated.  Lenovo explains, SN, windows, etc. that why the proposal is to first understand the functionality we support.

Proposal 1: It is proposed that RAN2 first agree on the complete set of required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.

Observation 3: 5G protocol stack was generally designed in an information agnostic way, e.g. neither content of incoming data packet nor the contribution to application layer performance are considered during radio resource allocation. To some very limited extend application-layer awareness in AS was introduced for XR services.

Proposal 3: RAN2 should study support of native application-awareness at the Access Stratum.

-	Nokia thinks that we have QFI so what do you have in mind.   Lenovo explains that all packets within a DRB and treated the same and there is no differentiation between the packets and dependencies of packets.  For 6G we should be more flexible and maybe provide more meta data to enable this.  Nokia so we are extending a bit what we did for XR.

-	CMCC asks if this for UL or DL.   Lenovo explains for both and of course we have some dependencies on SA.

-	docomo likes this proposal

Noted

[2 min]

R2-2507200	On 6G User Plane	NTT DOCOMO, INC.	discussion	Rel-20

Observation 1.	As a first step for discussing the 6GR User Plane architecture from scratch, it is appropriate to evaluate the necessity of the Layer 2 functions defined in the NR User Plane.

Proposal 1.	6G Layer 2 supports the following functions, using NR as a baseline and applying modifications or optimizations as needed: header compression (ROHC and UDC), security protection for UP/CP data, in-sequence delivery, discarding of outdated data, ARQ, (re)segmentation, multiplexing, HARQ and MAC CE.

Proposal 2.	RAN2 studies which sublayer supports each legacy function and order of the functions to be processed in Layer 2.

Proposal 3.	RAN2 studies whether SDAP sublayer can be merged to PDCP sublayer, i.e., PDCP can support mapping between QoS Flow and DRB, coordinating with other WGs.

-	Mediatek thinks that only the required functions are needed to be considered for SDAP and Reflective QoS is never deployed but it is still using up a header.

-	Samsung asks if the itnentino is to remove the SDAP functions or just move them to PDCP.  Docomo explains it is to move.

-	Ofinno asks how we can merge since there is not a one to one mapping for SDAP and PDCP.

-	Nokia clarifies that this not only for reflective QoS but also for QoS flow mapping.

Noted

[2 min]

R2-2506809	Discussion on 6G user plane enhancements	Qualcomm France	discussion	Rel-20

Observation 1.	6G MAC should address the shortcomings in NR’s RACH design (e.g. RACH partitioning) and aim for a more resource efficient and adaptive design.

Observation 2. 	6G can benefit from lower uplink latency and better support for increasingly more diverse traffic. These objectives can be achieved by enhancements that are more QoS-aware, more adaptive and better coordinated with upper layers such as RLC.

Proposal 1. 	The following MAC functions are supported in 6G:

-	Functions related to random access, including CBRA and CFRA;

-	Functions related to UL scheduling, including SR, BSR, DSR, LCP, HARQ, CG, PHR;

-	Functions related to bandwidth management, including BWP, CA;

-	Functions related to link management, including BFD/R, UL timing;

-	Functions related to energy savings, including UE DRX and cell DTX/DRX.

-	Huawei asks how we can progress discussion on some of these procedures as we depend on RAN1.  Qualcomm explains that this just refer to functions and we can still discuss these procedures.

-	Vivo asks what is the intention with the RA partitioning.   For power savings we need to discuss further as this is very specific.   Qualcomm explains that we at least know about these two schemes and when we have more we can consider.

-	Oppo asks what about LP-WUS.  What about BW managements.  Qualcomm thinks that LP-WUS is RAN1 led topic so it will depend on eventual what RAN1 will depend on.

-	Ericsson thinks that this is surprising that we have to wait for RAN1 for RA and scheduling.  RAN2 should discuss overall system and inform RAN1 on what matters.  And we should coordinate earlier on the power saving mechanism.

-	Apple ask if these functions are mandatory or optional.   Qualcomm thinks that these functionalities should be mandatory if RAN2 decides to include them.

Noted

Discussions

-	LG indicates that architecture and protocol decisions depend on DC, so we should focus on standalone.

-	Samsung asks based on what can we discuss the functionalities and understand the necessity of the functionality.

-	Huawei thinks that this is align with the bottom up approach.

-	CMCC would prefer not to go one by one, but rather find out what we require for new requirements.

-	ZTE thinks that we should identify what functionality is needed for new requirements and which ones we can use from 5G.  We should prioritize the functionalities that address new use cases/services.  We should avoid spending time on minor enhancements to existing functionalities.   Vivo agrees with ZTE and we should discuss requirements for the new services.

-	Mediatek reminds everyone of the 80-20 rule from Ericsson.

-	Transsion thinks that the QoS model is not clear in SA2.

-	Ericsson is a bit concerned the problems that we had.   We should understand what is not working and what we should change.  It is good to have simulation results to provide the benefit.

-	LG wants to clarify that this should be for data and not for AI/ML.   ZTE thinks that protocol layer should enable data transfer and any data is data.

[2 mins]

UP processing

R2-2506854	On 6G user plane architecture considerations and user plane functions	MediaTek UK	discussion	Rel-20

Proposal 2: 6G user plane is designed to be hardware-processing friendly while keeping memory requirements low.

-	example, fixed headers

-	Interdigital asks if there is any recommendations.   Mediatek thinks that anything related to data transfer it should be a simple as possible and deterministic.   The piping of data should be very simple and efficient.

-	Ericsson thinks another consideration is moving the data the least amount of time across the layers.   Mediatek points out it is important to minimize data movement.

-	Mediatek would like to have similar design so we can benefit from economies of scale.

-	Apple agrees and encourages companies to identify what functions are processing heavy and complex and how we can simplify.

-	Vivo asks what are the principles to decide whether it is friendly or not.  Mediatek thinks that one principle is that we have good justification for it.

-	ZTE asks whether we have any examples of things that weren’t processing friendly.  MEdiatek explains that we have done a very good job in 5G and we should keep those principles in mind.  But there may be some things related to memory and timers.

-	Huawei thinks we shouldn’t sacrifice other KPIs for this.

-	Sharp thinks that we already made headers fixed and that’s why we had removed concatenation in RLC.  The only flexible headers remaining are in MAC.     MEdiatek thinks that 5G was very good and 6G will be better.

Noted

[2 mins]

Not treated:

R2-2507216	User plane functions for 6G	Samsung	discussion	FS_6G_Radio

Observation 1. User plane protocol has been tested and proven to be stable over decades.

Observation 2. UP protocol was already optimized by removing redundant functions.

Observation 3. Data pre-processing was allowed to support high data rates in NR.

Proposal 1. Data pre-processing is allowed in 6G user plane protocol the same as in NR.

Proposal 2. To study how to further utilize data pre-processing in 6G user plane protocol.

Proposal 3. At least one post-concatenation function is mandatory in 6G user plane protocol.

Proposal 4. To study pre-concatenation function in 6G user plane protocol.

[3 mins]

Not treated

R2-2506940	Discussion on 6GR User Plane functions and architecture	Huawei, HiSilicon	discussion	FS_6G_Radio

Observation1-1:	From the processing complexity point of view, the design target is to ensure that the Layer2 does not become a bottleneck between the user experienced data rate it provides to Layer 3 and the uplink data rate that physical layer provides.

Proposal5:	RAN2 to study the potential directions for increased spectral efficiency from the perspective of, e.g., L2 header size reduction, PDCP concatenation.

Proposal6:	RAN2 to study the potential directions of processing complexity reduction from the perspective of e.g., PDCP concatenation, PDCP/RLC Tx/Rx operation, and MAC PDU formats.

[2mins]

QoS and framework

R2-2506891	Service Aware RAN RAN2 consideration	T-Mobile USA Inc.	discussion

Observation 1: Lack of application-level performance visibility in RAN.

Observation 2: Coarse granularity of the 5G QoS framework.

Observation 3: Limited ability of RAN to optimize resources for new service types.

-	Samsung asks what is the exact limitation from resource perspective.   Tmobile explains hat from application point of view RAN has no idea and it cannot adjust.

Observation 4: Necessity of treating XR, AI, and other emerging services as primary in 6G.

Accordingly:

Proposal 1: RAN2 should study enhancements to the Radio Interface architecture and protocols with service awareness as a design principle, including novel scheduling methodologies.

-	Docomo asks what are novel scheduling methodologies you have in mind.   Tmobile explains that it is mainly referring to scheduling to be more efficient.

-	Huawei asks what other services other than XR do you have in mind and what are the traffic characteristics.   Tmobile explain XR is one, AI applications, and we can discuss further.

-	Vivo asks if this means that we need application visibility in RAN.  From 5G we learned that we needed some parameters and which layer.   Tmobile this is intending to open the discussion and we can continue looking at the solutions.

-	MEdiatek agrees and asks if they consider only operator managed services or in general.  Tmobile explains that it is more general.

-	ZTE thinks that we should identify dependencies with other groups and we should separate UL and DL in the discussions.   For DL do we assume we have something and work in RAN2, and for UL we should work with CT1.   Tmobile thinks we can work closely with SA2.

Noted

R2-2507313	Requirements for L2 protocols	InterDigital	discussion	Rel-20	FS_6G_Radio

Observation 2.1-1: Interactive AI-based services are uplink heavy, latency sensitive and data sessions are characterized by periodic and/or irregular data bursts with varying volumes and inter-burst interval.

Proposal 1: 	The 6GR QoS framework shall support uplink transfer of delay critical bursts of varying volumes and inter-burst interval in a resource efficient manner (e.g., without overprovisioning of radio resources).

-	Xiaomi asks what is the delay requirement for these types of services.    Interdigital thinks that some of this examples have provided targets of 200ms, but this depends on use cases.

-	Apple wonders whether we are going to introduce a new scheduling mechanism or if we can use some of the existing solutions can handle these requirements.  Interdigital explains the intention is to first understand the requirements and then we can look at solutions on how to achieve this at the end.

-	Mediatek thinks that this is more related to scheduling and wonders how QoS can help.  Interdigital explains that we use QoS similar to XR to understand the priority of the packets and achieve resource efficiency.

Observation 2.4.4-1: Advanced immersive applications and/or codecs can adapt in rate and/or modality to implement graceful QoE degradation.

Proposal 6:	6GR QoS framework supports QoS ranging for QoE and/or rate-based adaptations e.g., the UE autonomously selects parameters within the NW-configured range of values.

-	Lenovo asks what type of parameters you have in mind and the motivation.  Interidigtla explains it is general but one example can be PDCP discard and the UE can apply a timer within a configure value range based on traffic.

-	Qualcomm asks what is QoE, what metric is it, will we specify something or is it a subjective metric.  Interdigital doesn’t think that we will specify spefic metrics.

Proposal 7:	6GR QoS framework support service-based differentiation and sub-flow granularity e.g., based on L2 application-level awareness.

-	Oppo asks if the AI based service whether we consider all types including tokenized AI.   Interdigital explains that we only had limited examples to show that AI application generate a lot more UL traffic but we can study other services.

-	Fujitsu asks what sub-flow refers to here as it has SA2 relationship.    Interdigital thinks that RAN2 should first discuss and define what RAN needs for RAN awareness and then discuss with other groups.

Noted

Discussion

-	Nokia thinks it is fine to study these but we shouldn’t do this in isolation of SA2 and SA4.  And for tokenized traffic we need to wait for SA4 to tell us that this is something out there.    Interdigital agrees that we can get traffic characterization from SA4 and the main point was that for some initial discussions in RAN we don’t need to involve SA2 immediately.

-	Ericsson thinks that the framework is not bad but rather that it is not fully used.  So we should look into the problems before going into the sub-flows.

-	Qualcomm understand that service-awareness is important but has some concerns on UE complexity perspective.   We shouldn’t introduce different solutions for every application as this creates fragmentation.  We should strive to have unified solutions.  Also RAN alone doesn’t have much idea on the application level.   We should be careful and aim to design something that is applicable to all applications.

-	ZTE thinks that we should do this in conjunction with SA2 and we should rely on a standardized mechanism for both UL and DL and learn from problems we have had in XR.   ZTE points out that we have more dynamic QoS and every QoS changes require NAS re-negotiation.  So we should try to handle it in RAN with some information from SA.

-	Huawei thinks that we need to understand what are the services, AI services and immersive so far.  We should have a standardized way to handle these services.

-	CMCC thinks we should look into tokenized AI.

-	Vivo thinks we should discuss this from the beginning and we shouldn’t just enable the awareness, and study mechanism to support this.   We should at least identify the requirements and what RAN needs.

-	Apple thinks that we need to know how to take advantage of this awareness.

-	Samsung would like to avoid the problem with coordination in RAN and SA.

-	Lenovo thinks that one lesson learned that that in some cases we thought it was beneficial but SA4 told us otherwise, so we should be careful.

-	ZTE as part of study RAN with cooperation (when needed) with SA4 should identify what is needed and indicate to SA2 what is required from RAN perspective.

-	Xioami asks how to avoid overlap with XR 5GA and this topic.  Huawei explains that the 5GA is limited to UL traffic.

-	Ericsson suggest that we can move the study of mobile AI traffic characteristics into 6G.

[3 mins]

Not treated

R2-2506808	General considerations on RAN2 6G UP design	Beijing Xiaomi Mobile Software	discussion	Rel-20

[Lesson from 5G-10] 5G doesn't allow differentiated QoS fulfilment for distinct PDU set types within a QoS flow.

[Lesson from 5G-11] 5G cannot well support the synchronization dependency among QoS flows.

Proposal 5: 6G should consider the service awareness techniques introduced in 5G for XR as a start point, and strive to further improve the QoS experience with the following consideration:

- Support Service Aware QoS handling and finer QoS granularity;

- Support the correlation requirement (e.g. sync requirement) among different QoS flows;

[3mins]

R2-2507113	Views on Directions of 6G User Plane Enhancements	Apple	discussion	Rel-20	FS_6G_Radio

Proposal 5: RAN2 should study (based on coordination with SA) how differentiated radio treatment can be provided to packets within the same QoS flow/DRB, to accommodate the characteristics of some emerging use cases in 6G.

Proposal 6: RAN2 should study how 6G user plane protocols can be enhanced to prevent or reduce the delay of more critical QoS flows(s) caused by head of line blocking issues within a DRB.

[2mins]

R2-2507615	6GR Common Aspects	Nokia, Nokia Shanghai Bell	discussion	Rel-20	FS_6G_Radio

From 10.3.3

Proposal 1: the QoS framework of 6G is based on that of 5G, with a control of uplink QoS flows that shall allow absolute prioritisation, guarantee a bit rate, avoid starvation, adjust the priority based on the looming possibility of discard, and enforce bit rate limits, while being observable.

[2 mins]

UP latency

R2-2506854	On 6G user plane architecture considerations and user plane functions	MediaTek UK	discussion	Rel-20

Observation 1: Latency associated with eMBB data transfer will be an increasingly important metric to serve emerging use-cases during 6G’s lifetime.

Proposal 1: 6G user plane is designed to reduce the latency associated with eMBB data transfer compared to NR.

-	Nokia asks what is the root cause.  Mediatek explains that our data rates don’t matter anymore and our pipeline is good enough.  What the differentiator with 6G is latency and every user can get the service right away.   We should streamline our protocol to achieve this.

-	Apple indicates that IMT requirements will not change for latency and what really matters at the end is the end to end latency that matters, but the air interface is a small part.   Mediatek explains that UL initiation of traffic is subject to delays from configuration to transmission etc.   Ericsson, Docomo, agrees with Mediatek and has same observation and it is many times we have 20-40 ms delay.

-	Docomo explains that latency would give operators a motivation to deploy and monetize 6G.

-	LG thinks that both radio interface and end-to-end delay should be considered.

-	Interdigital agrees and what we have learned from 5G is also the time it takes to get a grant from SR/BR and transmit etc.   ZTE agrees and we should find a way to reduce the latency.

Noted

[2 mins]

R2-2506940	Discussion on 6GR User Plane functions and architecture	Huawei, HiSilicon	discussion	FS_6G_Radio

Proposal1:	RAN2 user plane design shall support the targeted 6G KPI requirements (e.g., user experienced data rate, latency, reliability, composite requirement) and new service requirements (e.g., AI traffic and immersive communications).

Noted

[2 mins]

Scheduling

R2-2507071	User plane: Let's keep it simple!	Ericsson	discussion	Rel-20

Observation 5	Indications to manage queueing latency in RAN to transport layer are subject to delays considering 5G’s need for pre-processing.

Observation 6	The latency introduced by the SR/BSR procedure to obtain the initial grant in NR is the dominant contributor to delay and hence limits the end-to-end performance of UL- and DL-heavy applications.

Proposal 2	Study support for faster queue management as an integral component of 6G RAN (e.g. based on queue indications).

Proposal 3	Study means to reduce latency and improve accuracy for uplink buffer information reporting and thereby decrease latency and enhance e2e performance.

Proposal 4	Study uplink scheduling framework to ensure QoE fairness with fine-grained control over QoS characteristics among LCH(s).

Noted

[3mins]

R2-2506850	Discussion on the 6G user plane features	OPPO	discussion	Rel-20	FS_6G_Radio

Observation 5: In 5G, UL transmission via DG provides flexible scheduling without the waste of reserved uplink resources of PUSCH, but with more UL scheduling latency due to the SR/BSR/DCI transmission/reception. The UL transmission via CG is of low latency but may cause resource waste due to pre-configuration. The UL transmission via RACH MsgA/Msg3 supports diverse scenarios and RRC state but has unnecessary overhead (e.g., preamble transmission) and latency for initial access and UL scheduling.

Proposal 5: For UL scheduling schemes in 6G, RAN2 studies to: 1) support 5G UL scheduling schemes as the baseline, i.e., DG and type-1/type-2 CG. 2) further investigate UL scheduling schemes for contention-based CG resources for fast access.

-	Docomo thinks 1)  is very important.   Nokia asks if we need both type-1 and type2 or can we consider a more unified scheme.  Oppo thinks that these are for different pruposes and type 1 is more deterministic.   Qualcomm thinks the second one should be studied further.

-	Qualcomm wonders if we should continue studying SPS since there was no deployment.

Noted

[2 mins]

Discussion on contention based UL resource

-	ZTE, Mediatek agrees as it is beneficial to address the latency related to SR/BSR delays.   Huawei thinks that RAN2 cannot agree to this alone as there needs to be simulations from RAN1.

-	Apple thinks that we discussed SR/BSR latency in 5G and this is why we introduced CG, so doesn’t see the motivation to have yet another scheme.

-	Interdigital sees the benefit of contention based UL resource.  This can be beneficial for  SR/BSR, small data and fast access and wonders if this is for connected mode or can it be for other modes, like inactive.   ZTE Thinks that we should first design the channel and then discuss where it can be used.   Ericsson explains that according to simulation this contention based channel is best for SR/BSR.

-	Ericsson doesn’t think RAN1 can simulate this and we would have to do it in RAN2 to understand how useful it is and system performance.

-	LG is also supportive of this contention based UL resource, as network doesn’t have to provide dedicated grant to all UEs.

-	Sony thinks that this I related to resource efficiency.

R2-2506894	Initial Considerations for 6GR User Plane	Sharp	discussion	Rel-20	FS_6G_Radio

Observation 5: BSR and DSR have duplication which results in signalling overhead or even inefficiency.

Proposal 5: RAN2 to study a unified buffer status reporting including total buffered data volume and delay status of the buffered data.

[2 mins]

L2 re-transmission

R2-2506798	Considerations on 6GR user plane	vivo	discussion	Rel-20

Proposal 5:	Study solutions to better support new emerging services with low latency and high reliability requirements than NR. Effective feedback to enable fast ARQ-like L2 retransmission should be considered.

Noted

[2 mins]

R2-2507113	Views on Directions of 6G User Plane Enhancements	Apple	discussion	Rel-20	FS_6G_Radio

Proposal 3: RAN2 should study to investigate if L2 ARQ efficiency can be enhanced in 6GR based on tight coordination with HARQ.

Noted

[2 mins]

R2-2507157	6G UP design	Nokia, Nokia Shanghai Bell	discussion

Proposal 4: The RLC in 6G should be built on Rel-19 RLC, incorporating Rel-19 XR mechanisms. RAN2 further study if 6G RLC supports avoidance of redundant retransmission under reliable HARQ or faster retransmission using HARQ feedback, where such considerations require coordination with RAN1.

Noted

[2 mins]

R2-2507127	Considerations on User plane for 6G	LG Electronics Inc.	discussion	Rel-20	FS_6G_Radio

Proposal 3	Introduce per-packet transmission mode suitable for mixed type packet flow

a. Whether to perform retransmission is determined per packet basis

b. Single DRB supports both lossless and lossy transmission

Noted

[2 mins]

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB).

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

Not treated:

R2-2506768	Discussion on user plan in 6G	Transsion Holdings	discussion

R2-2506798	Considerations on 6GR user plane	vivo	discussion	Rel-20

R2-2506808	General considerations on RAN2 6G UP design	Beijing Xiaomi Mobile Software	discussion	Rel-20

R2-2506809	Discussion on 6G user plane enhancements	Qualcomm France	discussion	Rel-20

R2-2506828	Discussion on User Plane for 6G RAN	TCL	discussion	Rel-20

R2-2506845	6GR UP Architecture and Functions	CATT	discussion	Rel-20	FS_6G_Radio

R2-2506850	Discussion on the 6G user plane features	OPPO	discussion	Rel-20	FS_6G_Radio

R2-2506854	On 6G user plane architecture considerations and user plane functions	MediaTek UK	discussion	Rel-20

R2-2506883	Consideration of User Plane Functions for 6GR	China Telecom	discussion	Rel-20	FS_6G_Radio

R2-2506894	Initial Considerations for 6GR User Plane	Sharp	discussion	Rel-20	FS_6G_Radio

R2-2506905	Consideration on 6GR User Plane	CMCC	discussion	Rel-20	FS_6G_Radio

R2-2506913	Discussion on user plane protocol of 6GR	Spreadtrum, UNISOC	discussion	Rel-20

R2-2506938	6GR User plane aspects	Fujitsu	discussion	Rel-20	FS_6G_Radio

R2-2506940	Discussion on 6GR User Plane functions and architecture	Huawei, HiSilicon	discussion	FS_6G_Radio

R2-2506952	Considerations for ARQ-less 6G user plane	KT Corp.	discussion	Rel-20	FS_6G_Radio

R2-2507034	Discussion on User plane for 6GR	HONOR	discussion	Rel-20	FS_6G_Radio

R2-2507071	User plane: Let's keep it simple!	Ericsson	discussion	Rel-20

R2-2507113	Views on Directions of 6G User Plane Enhancements	Apple	discussion	Rel-20	FS_6G_Radio

R2-2507127	Considerations on User plane for 6G	LG Electronics Inc.	discussion	Rel-20	FS_6G_Radio

R2-2507157	6G UP design	Nokia, Nokia Shanghai Bell	discussion

R2-2507186	Overview of User Plane	Ofinno	discussion	Rel-20	FS_6G_Radio

R2-2507200	On 6G User Plane	NTT DOCOMO, INC.	discussion	Rel-20

R2-2507202	Discussions on 6G User Plane	Futurewei	discussion	Rel-20

R2-2507216	User plane functions for 6G	Samsung	discussion	FS_6G_Radio

R2-2507241	Discussion on User Plane Design for 6GR	ETRI	discussion	Rel-20

R2-2507250	6G Radio protocol architecture - User Plane Aspects	Lenovo	discussion	Rel-20	FS_6G_Radio

R2-2507302	6G User plane functionality and dependencies	ZTE Corporation, Sanechips	discussion

R2-2507313	Requirements for L2 protocols	InterDigital	discussion	Rel-20	FS_6G_Radio

R2-2507320	Discussion on 6GR Rel-20 User plane aspects	Sony	discussion	Rel-20	FS_6G_Radio

R2-2507333	Discussion on Radio Protocol Architecture – User Plane	Rakuten Mobile, Inc	discussion	Rel-20

R2-2507372	Overview of 6G User Plane protocol architecture	NEC	discussion	Rel-20	FS_6G_Radio

R2-2507389	Discussion on user plane functions	Tejas Network Limited	discussion	Rel-20

R2-2507461	Considerations for 6G User Plane Functions and Protocols	CEWiT	discussion

R2-2507512	Initial Considerations to Support User Plane Function in the AI-native RAN Architecture	Hanbat National University	discussion	Rel-20

R2-2507542	Discussion on 6G User Plane design	ASUSTeK	discussion	Rel-20

R2-2507575	Discussion on 6G AS User plane design	Google Korea LLC	discussion	FS_6G_Radio

R2-2507579	Considerations on User Plane for 6GR	KDDI Corporation	discussion

R2-2507645	6G User Plane design aspects for NTN	THALES, Airbus, Echostar, Novamint, Fraunhofer IIS	discussion	Rel-20	FS_6G_Radio

#### 10.3.2	Control plane

RRC modelling, connection management, initial and system access, including  system information, paging etc..

RRC State Model

R2-2506900	Discussion on 6G control plane	CMCC	discussion	Rel-20	FS_6G_Radio

Observation 3: CONNECTED and IDLE states are well commercialized in 4G and 5G.

Observation 4: RRC_INACTIVE has not been widely enabled in 5G networks due to the following reasons: 1) Network planning complexity for RAN Notification Area (RNA); 2) Limited power saving gain compared to RRC_IDLE; 3) Lack of use cases requiring low state transition delay.

Proposal 6: CONNECTED and IDLE mode are supported for 6G. RAN2 should further study two candidate RRC state architecture evolution paths:

Option 1 – Evolve and Simplify the RRC_INACTIVE State;

Option 2 – Introducing a new power-saving state paradigm including a lightweight RRC_CONNECTED sub-state.

-	Interdigital asks if there is a difference from a functional point of view between these two options.  CMCC clarifies that for option 2 the UE doesn’t release the connection.

-	Huawei asks if there is paging complexity with RRC INACTIVE.  CMCC explains that we should avoid having a complicated paging procedure to guarantee commercialization.

-	Kyocera asks if we have option 1 we don’t need option 2.   CMCC hopes that we should only have one at the end, but we can study both.

Noted

[3min]

R2-2507146	On 6G RRC design	Nokia	discussion	Rel-20	FS_6G_Radio

Proposal 3: Study how to enable energy-efficient RRC states and low-latency state transitions in 6G. Baseline 6G RRC states should be based on 5G IDLE, INACTIVE and CONNECTED functionalities.

Proposal 4: Study how to enable energy-efficient user plane transmissions of small packets from INACTIVE state in 6G, using NR SDT design as baseline.

-	Xiaomi thinks that the functionalities are the most important part of the discussion and asks what enhancements are needed on top for those states.   Nokia explains that we want to discuss what we want to achieve, low latency, energy efficiency, and accessing the network.

-	Vivo asks if we want to consider SDT from beginning or wait until RRC states are stable.  Nokia thinks that we need to solve the problem of energy efficient small data transmission.

-	CATT asks that low latency is not just for transition.  Nokia explains that we need to fullfill IMT requirements for latency and see if we can do real things.

-	Tmobile agrees with Nokia’s view

Noted

[2min]

R2-2507072	Controlling the 6G access stratum	Ericsson	discussion	Rel-20

Proposal 3: 6GR can use the same RRC states as 5G (RRC_IDLE, RRC_INACTIVE, RRC_CONNECTED).

Proposal 4: Study methods to address shortcomings and improve RRC_INACTIVE, such as simplified RNA configuration, relaxed requirement on Xn connectivity, and improved I-RNTI format.

-	Oppo asks how we can achieve the inactive fast transitions without the Xn connectivity.  Ericsson explains that we can do paging in different ways, either CN or RAN relayed via CN.

-	Huawei asks if the Xn connectivity requires involvement from SA or RAN3.  Ericsson confirms it requires RAN3.

-	Samsung asks what is the multiple types of paging.  Ericsson clarifies that we can keep RAN paging but we should simplify it.

Noted

[2min]

R2-2507433	Control Plane for 6GR	InterDigital, Inc.	discussion	Rel-20	FS_6G_Radio

Proposal 3: Define the 6G state model with:

1.	Idle: Initial attach/recovery only.

2.	Semi-connected/inactive: Default low-power state with stored UE context, RAN-based paging, UE controlled mobility.

3.	Connected: High-throughput state with full network-controlled mobility and bearer setup.

Proposal 4: Introduce fast transition between Semi-connected/Inactive and Connected states targeting sub-10 ms resume latency, using a stored UE context in the semi-connected state.

-	Apple asks if recovery should also be in inactive as it happens quite a bit.   Interdigital agrees and the point is that there will be cases in idle where recovery is needed, and there will be recovery in other states.

-	Lenovo asks if paging and tracking area will be in inactive.  Interdigital explains that initial attach is for idle and then the UE can go to inactive for further procedures.  Transion asks if idle is only for attached.

-	Xiaomi asks how we can guarantee that the network doesn’t send to idle.

Noted

[2min]

R2-2506857	Energy efficient and unified RRC state modelling	Huawei, HiSilicon	discussion	Rel-20	FS_6G_Radio

Observation 4:	6G IDLE state is still needed for the UE’s initial power-on phase.

Proposal 6:	6GR design should maximize UEs in 6G CONNECTED state, rather than releasing them to 6G IDLE state.

Observation 5:	Simple RRC state modelling simplifies the network operation for UE management. 6GR should avoid introducing unnecessary RRC state, i.e., each additional RRC state should be first well justified.

Proposal 7:	6GR design should simplify RRC state modelling: starting from or focusing on essential RRC states, i.e., 6G CONNECTED state and 6G IDLE state.

-	ZTE asks what is the motivation and how we can achieve the fast transitions.  Huawei thinks that if the UE can do both stay in connected and enjoy power saving, the UE can stay in CONNECTED.   ZTE asks what about mobility.

-	Ericsson asks if we are in connected is it still NW controlled mobility.  Huawei thinks that we can consider to support both NW based and UE based mobility.

-	Nokia asks what happens to UE when in connected mode and it is doing UE based mobility.  How do we manage the context.  Huawei thinks that these are all questions we need to consider.

Noted

[2min]

R2-2507270	RRC states and connection management for 6GR	Panasonic	discussion	Rel-20

Proposal 1: For 6GR RRC state design, consider 2-state RRC modeling by unifying RRC_IDLE and RRC_INACTIVE into a single RRC non-connected state.

-	LG asks if we can transmit data in non-connected state.   Panasonic thinks that we should consider data transmission in idle, keep context.

-	Vivo asks how we can unify these in two states given that we have cases where we have context and we don’t.  Panasonic thinks we can do both.

-	Fraunhofer asks how we can avoid signaling overhead and transitions.

Noted

Discussions

-	Idle mode

-	Connected mode

-	Inactive state:  low latency transitions, energy efficient, UE based mobility, UE context, small data transfer

-	Qualcomm explains that we introduced inactive for fast transition so we still need and for that we need UE context.   Paging and RNA are some problematic.   Don’t like starting with Connected and going to connected.

-	Apple thinks this other state should have UE based mobility and focus assuming that 6G UEs will stay more inactive state than in idle mode.

-	ZTE thinks that for MT traffic we need paging.

-	Huawei thinks from UE behavior inactive is no UE data kind of state.

-	Fainity asks if this will apply to all device types.

-	Jio thinks that we shouldn’t sacrifice the flexibility of inactive.

-	Mediatek wonders if there is a reduced idle mode functionality if that would imply less mobility management involvement from CN.  InterDigital confirms.

-	Sharp thinks that inactive is an important state and we can merge with connected.

-	LG describes history, we went from 4states in HSPA to 2 states in LTE because transitions were too complicated.  In NR we realized that there were shortcoming in LTE with state transitions so we introduced inactive.   It is not a good idea to introduce a Connected mode sub-states as it introduces complexity.

-	Oppo explains that standalone was introduced in china but yet inactive wasn’t deployed.

-	CMCC thinks companies need to think what is missing in idle mode and connected mode.   For example, in connected mode we don’t have power saving mode, so we should think how to support this.

-	Samsung thinks that main complication in NR is that inactive is optional so we should consider making it mandatory.

-	Docomo thinks we should study inactive enhancements.  ZTE explains that it is a mandatory feature with IoT bit.  The reason why they were not using is the lesson learned from 5G, we made it mandatory for the UE, optional for network, and we made mistake to not enable small data so it wasn’t use.

-	Xiaomi explains that we used inactive for the 20ms requirement and we need it again.

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

RRC Signalling Design and Reconfigurations

R2-2506957	Control plane architecture and 6G RRC protocol design	MediaTek Inc.	discussion	Rel-20

Observation 4	The UE configuration structure in 5G RRC signalling has become complex due to incremental addition of new features.

Observation 5	UE configuration structure has an inter-WG dependency to RAN1/RAN4, since significant part of the UE configuration parameters are defined by RAN1/RAN4.

Observation 6	Delta signalling introduces ambiguities in RRC signalling, which are sources of security vulnerabilities. The required effort by the UE vendors to build devices which are robust against exploitation of the ambiguities is very high.

Proposal 6	RAN2 to study from the beginning improved UE configuration structure for 6G RRC as compared to 5G RRC, with a focus on simplicity and maintainability upon future extensions (such as new feature additions).

Proposal 7	RAN2 to study from the beginning more usable mechanisms for delta signalling for 6G RRC as compared to previous generations, with a focus on implementation robustness and simplicity rather than specification convenience.

-	Qualcomm wonders if the delta signaling are not useful or if there is something we can do to make it better.   Mediatek explains that it is used and there are things we can do to make it less ambiguous.

-	Fujistu asks how there is security vulnerabilities if RRC message is protected.

-	Xiaomi thinks that the principal should be applicable in general and not just in delta signaling.

-	Vivo asks how we solve the problem of inter-WG dependencies.  Mediatek thinks we should design the format in such a way that we don’t need to change when RAN1/4 adds something new or when we add new features.

-	Nokia thinks we have to study how to improve the RRC structure.  Delta signaling makes things complicated but it is helpful since it minimizes.   So study how to give configuration to UEs efficiently while keeping the size small.

-	ZTE asks what is robustness mean.  Mediatek explains is to avoid the vulnerabilities introduced by NR delta signaling.

Noted

[3min]

R2-2507111	Views on Directions of 6G Control Plane Enhancements	Apple	discussion	Rel-20	FS_6G_Radio

Observation 2: Most of UE dedicated configurations are same for UEs with the same capabilities in the same deployment.

Observation 3: By providing partial/delta configuration based on the reference configuration, the RRC message size can be reduced, thereby saving the system resources and reducing UE processing time.

Proposal 3: RAN2 study on 6G RRC configuration model focuses on the following aspects:

	1) Reducing configuration latency (e.g. RAN node that decides the config can directly send it.)

	2) Reducing RRC configuration size (e.g. based on reference configuration)

	3) Ensuring configuration validity (e.g. Do not provide configurations that may change in advance)

Noted

[3min]

R2-2507172	Views on Control Plane for 6G	Qualcomm Incorporated	discussion	Rel-20	FS_6G_Radio

Proposal 1:  	ASN.1 is used for encoding of RRC signaling for 6G air interface.

Proposal 2:  	RAN2 will study possible enhancements aimed at reducing duplication, increasing efficiency and improving readability of ASN.1 for RRC signaling.

Proposal 3:  	As a design goal, RAN2 will aim for modular design of RRC for 6G, e.g. consisting of baseline module and additional vertical-specific/use-case-specific modules.

-	Ericsson supports modular design but doesn’t understand how we can do it for use cases as we specify functions.  We should consider modularizing based on features, functions, etc.  Qualcomm was thinking that eMBB and IoT had different ASN.1 signaling, but the goal is that not one size fits all.   Memory footprint for ASN.1 should be smaller for IoT devices for example.

-	Huawei is good with modular design, but given modular design we may end up doing duplication and that introduces problems.

-	CMCC also thinks this is important.

-	Samsung is ok but is not sure what how it looks like.   Xiaomi explains that for SLPP we did use this modular design.

-	Vivo would like to ensure that eMBB is not impacted.

-	ZTE Thinks that when we design this modular we need to consult RAN1/RAN4 on how to define.

Proposal 4:  	6G design will allow the UE to keep/apply the good (part of) configuration and avoid re-establishment procedure.

-	LG asks what is the intention, shouldn’t the network provide a good configuration.   Qualcomm thinks thinks that the networks always have good intention but it’s not always possible, as the network is not always aware of the capability and temporary we could not be operating at 100% capability.

-	ZTE thinks the intention is ok but at the end of the day we need to have an awareness on the network side what configuration the UE has applied.  Qualcomm confirms and this is just a starting principle  but we would need to discuss different solutions.  The intention is that the UE shouldn’t be forced to go to RLF.

-	Ericsson thinks this would be a risky direction as we so far have relied on the network configuring the UE properly.  If there is something that causes bad network configuration we should address the root cause of that issue.

-	Nokia thinks that we should study how to do an efficient RRC re-establishment failure, but before we do this we should understand the issue and according to some explanations it seems to be more linked to dynamic capabilities.    Qualcomm thinks in addition to more efficient, the aim should also be to reduce the number of re-establishments.

-	Xiaomi ask if this is related to modularization. Qualcomm thinks that indeed we can make a connection there as if we have good modularization we could use it, for example indicate which model can be applied.

-	Interdigital thinks that we can minimize the re-establishment failures and modularization can help.   Apple also supports this direction and if we have a good structure it helps.

-	Mediatek has some sympathy for the proposal, but points out that we have spoken this since UMTS, with partial failure, and some of the issues were related to the network knowing.

-	Jio thinks this is a good proposal and the network should know so it is avoided in future.

-	Transsion thinks that this can be linked to the dynamic capability change.

-	CMCC thinks that this is a rare case in 5G and this adds new complexities.  Huawei thinks that we need to study use cases, for example target network may not understand the configuration of the source configuration and we may be able to address the issue with modular.

-	Samsung would like to first understand when this situation happens and then we can discuss the solution.

-	Qualcomm encourages companies to speak to their IODT teams and when it comes to UEs we get unexpected configurations.

For next meeting, can study the reasons why these failures happens and understand the root cause of the problem.

Noted

Agreements

  - ASN.1 is used for encoding of RRC signaling for 6G air interface (same as NR).

  - Study overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signalling

  - Study how to efficiently, reliably and unambiguously configure UEs while keeping signalling size small (e.g. improvements to delta signaling or no delta signaling).

  - RAN2 will study modular design of RRC for 6G.  Further study how to modularize RRC e.g. based on features, functions, verticals, etc.

[3min]

Spectrum Aggregation

R2-2507072	Controlling the 6G access stratum	Ericsson	discussion	Rel-20

Proposal 6 Study a single framework for spectrum aggregation based on carrier aggregation, enhanced with fast setup of additional carriers, flexible combining of UL/DL carriers and improved resiliency.

-	Vivo asks if this is for all states or only for connected state.

-	ZTE asks which WG would be the responsible for the downselection of framework.  Ericsson thinks that we should focus on the realistic deployments where we can get the most gains.  For WG we should have our own analysis of the whole framework and RAN2 should be responsible for the signaling.

-	Apple thinks that from R2 point of view we already support flexibility combining of UL/DL and also what is further enhancmeents on fast setup.   Ericsson thinks that we should design something that is feasible from other WG perspective and if what we have for NR works then we can use it.

-	Xiaomi asks if this includes UL only carrier.   Ericsson thinks that it is similar to SUL which wasn’t too successful as it didn’t have a DL for synchronization etc.   Ericsson thinks that the idle mode UE would have to know that there is another carrier so it would impact SI, so we can at least discuss those aspects together with RAN4.   Qualcomm thinks that we should not couple idle and connected mode.

-	Ericsson explains that we look mainly at collocated deployments as they are cheaper but it isn’t always possible to have co-llocated deployements.  For collocated we should focus on doing it in CA based solution with single MAC control.

Noted

[2min]

R2-2506799	Considerations on 6GR control plane	vivo	discussion	Rel-20

Observation 1: The cell model of single cell with multi-carriers (SCMC) is not only enabling the operators to utilize fragmented spectrums with potential NES gain (e.g., SIB is transmitted only on anchor carrier), but also beneficial for the UE’s power saving to camp on SCMC when the anchor carrier is deployed in low bands.

Proposal 3: 6GR shall study single cell with multi-carriers (SCMC) to aggregate multiple carriers in the same or different bands as a single cell, with the assumption of same/diverse coverage and co-located/non-co-located deployment among the carriers.

-	Ericsson explains that RAN4 thinks that this related to measurement gap, for RAN2 related to system information, etc.  We need to understand the problem.  Vivo explains that because bw is limited some SI on some carriers won’t be transmitted.

-	Qulcomm asks if we are aggregating across FRs.   Vivo From RAN2 perspective we should do everything.   Qualcomm is concerned that we have a single cell across FRs, not like normal aggregation.

-	LG asks if we have multiple carriers why don’t we map the carriers to a cell, otherwise it is complicated so what is the real problem.

-	Xiaomi thinks that question is whether all the carriers can transmit a SSB.

-	CMCC thinks that we have deployed CA and it works very well, but for idle mode we should optimize the idle mode and limit the signalling overhead.

-	Mediatek thinks the problem is in connected mode and it is connected to the configuration required for every cell.  So if you aggregate multiple spectrum we have significant overhead from configuration point of view.   We should optimize more for aggregating multiple pieces of spectrum.

-	Interdigital points out that we have done this in NB-IoT where we have introduced anchor carrier and the main reason was the limited BW.  Whether we have the same problem in 6G is not clear.

-	Ericsson and Apple thinks that those are the problem we should address but not jump into conclusion on how to address them.

-	LG thinks that single cell is a modelling issue anyways, we should avoid the overhead channels for multicarrier.

Noted

[2min]

R2-2507232	Considerations for 6G Control Plane	Samsung, Verizon	discussion	FS_6G_Radio

Observation 4: Conventional Carrier Aggregation with many carriers is less efficient in signalling and resource management aspects.

Proposal 4: For efficient usage of fragmented frequency spectrum, the single cell concept with fragmented carriers is supported. The N-carrier Single Cell (NSC) consists of DL/UL anchor carriers and one or more DL/UL non-anchor carrier(s). The initial access is performed on the anchor carrier of the NSC.

Noted

[2 min]

Agreements on Spectrum aggregation

- 1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

- 2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

System Information

R2-2507146	On 6G RRC design	Nokia	discussion	Rel-20	FS_6G_Radio

Proposal 5: Study how to enable energy-efficient system information transmission and paging operation to allow good power saving possibilities for both networks and UE. Baseline design should be based on 5G-A network and UE energy saving operation.

Noted

[2min]

R2-2507232	Considerations for 6G Control Plane	Samsung, Verizon	discussion	FS_6G_Radio

Observation 5: On-demand SI and area-specific SIB introduced in NR are beneficial for sustainability.

Proposal 5: SI broadcast framework is inherited from NR, and further improvements of the SI broadcast can be studied for sustainability, e.g., Spatial SI broadcast, On-demand MIB/SIB1/area-specific SIB1, finer granularity of SI change notification, transmit only the information relevant to schedule the changed SIBs instead of transmitting the entire SIB, reference configuration for SIB and so on.

-	Mediatek would like to understand if we don’t send the information in the SIBs what would happen if a new UE arrives in a cell that the SIBis not broadcasted.   Samsung thinks that we can use ondemand SI and there are benefits in certain rural area.

-	Qualcomm asks if spatial SI is used only for ondemand SI.   Qualcomm asks if the intention is to put some information area specific and some cell specific.   Samsung confirms that there are both types.

-	ZTE asks about on-demand MIB whether they need to be discussed together with ondemand SSB.   Samsung is open to all scenarios.

Noted

R2-2506856	Discussion on 6G control plane	Huawei, HiSilicon	discussion	Rel-20	FS_6G_Radio

Observation 1-1:	 The current SI scheduling mechanism has become overly complex due to multiple enhancements aimed at better utilizing time-domain opportunities however at late stage.

Observation 1-2:	 The current SI scheduling mechanism restricts the network's ability to enter sleep mode for energy saving.

Observation 1-3:	As the typical implementation for SI acquisition in 5G, UE needs to re-acquire SIBs upon cell change even if the SIBs of the new cell are the same with the stored versions. It causes unnecessary SI broadcast signaling and energy consumption for both UE and network.

-	Fujitsu asks if this is only for SIB1 or other SIBS as well.   Huawei indicates that this is for other SIBs and in the real world it is difficult coordinate the area specific SIBs.

Proposal 1: 6G System information design should consider energy saving friendly (for both network and UE) SI scheduling/acquisition mechanism.

Observation 2-1:	 In 5G, the SIB1 size limitation may prevent the network from enabling certain features in some scenarios.

Proposal 2: RAN2 should study the issues related to SIB size restriction (especially for SIB1).

-	Xiaomi thinks that the size itself should first be decided by RAN1.  Is there something specific RAN2 should discuss in parallel.    Huawei thinks that we can wait or we can study if some information can be split and moved out.

-	CATT asks if the intention of splitting SIB1 would also help for energy saving.

-	Vivo asks what criteria can we use to determine the SIB1 split.  Huawei explains that we can keep the mandatory information in SIB1.

-	Nokia thinks that we can study in RAN2 and give RAN1 recommendation on available payload.   How can we split and how does it help.    Huawei explains that there are somethings that can be studied in RAN1 and splitting would imply that different information can be scheduled in different time slots.

-	Apple thinks that we can study how to minimize the SIB1 design or avoid the transmission.    LG we can think of an approach to split between common channels similar to the split with SIB1 and SIB2.   We need to understand what is the expected size for SIB1.

-	Interdigital thinks that there are two issues in NR, always on SIB1 and size of SIB1 due to all the optionality of features.   If we simplify the system the size should also decrease.   If we have ondemand SIB1 then do we need to reduce the size.

Noted

[2min]

R2-2507069	Consideration on 6G control plane	ZTE Corporation, Sanechips	discussion	Rel-20	FS_6G_Radio

Proposal 3: The concept of RMSI, cell and area specific system information, SI modification indication via paging and on demand system information should be inherited from 5G while the following enhancements should be studied for 6G system information design:

-	Network energy saving for system information (e.g., on-demand SI, cell DTX/DRX)

-	Security protection for system information (e.g., integrity protection; pending input from SA3)

-	Support for various device types (e.g., whether separate SSB/MIB/SIB are required for different device types; pending input from RAN1)

-	Ericsson asks if the intention is to have a separate design.  ZTE explain that it is not the target to have different designs but we should keep an eye.

-	Support for 5G–6G MRSS (e.g., whether joint SSB/MIB/SIB can be considered; pending input from RAN1)

-	Apple asks why do we need to care.   ZTE indicates that if we have separate SIBs then we don’t need to care but if it is the same SIB that would have an impact on the SIB design.   Apple thinks we can wait for RAN1.

-	Nokia thinks that RMSI was a term that RAN1 came up with because they didn’t know about SIB1.

Noted

[2min]

R2-2506887	SSB Transmission Consideration in 6GR	T-Mobile USA; Ericsson	discussion (moved from 10.2)

Observation 1	The 20ms SSB periodicity in NR illustrates how lean carrier design enables substantial network energy savings relative to LTE, primarily by reducing always-on transmissions.

Observation 2	For broadcast transmissions, full benefits of lean design can be achieved if sparsity can be maintained across all transmissions and receptions on the same time scale for any carrier.

Proposal 1	Study mechanisms to extend the default SSB periodicity and to introduce on-demand SSB transmissions with the goal to establish sparsity across all transmissions targeting UEs in both connected and non-connected states in the first 6GR release.

Observation 3	The 20ms default SSB periodicity in NR in cells supporting initial access, remains a key limiting factor that restricts network’s ability to transition into deep sleep states.

Observation 4	Blind initial cell searches are rare in practice, as most UEs leverage prior knowledge, such as PLMN scanning logic, during cell search.

Observation 5	Constraining adjustments to the default SSB periodicity based on the assumption that UEs frequently perform blind searches is not justified.

Proposal 2	Extended values for default SSB periodicity should be considered for UEs in idle/inactive modes when studying 6GR features.

Proposal 3	Study on-demand SSB transmission for both connected and non-connected UEs.

-	CATT asks why do you need to send on-demand SSB in connected.   Ericsson clarifies that on-demand doesn’t necessarily mean that the UE requests anything.  The network would know when the transmission is needed.

-	Ericsson thinks for mobility the network is aware and can provide SSB transmissions.

Noted

[2mins]

R2-2507433	Control Plane for 6GR	InterDigital, Inc.	discussion	Rel-20	FS_6G_Radio

Proposal 1: Support on-demand System Information, including SSB transmission without SIB1 (SIB1-less), multi-carrier aware access, and cases where SSB may not always be present.

-	Nokia asks if the UE would measure multiple carriers.  Interdigital explains that it is something similar to NBIoT where only one carrier is broadcasting information and the device can access other carriers.

-	Qualcomm asks how can the UE get scheduling information without SIB1.   Interidigital explains that we consider some UL signalling to request.

-	Docomo asks if the UE can receive paging in carriers where SIBs are not transmitted.  Interdigital explains that it can be possible similar to NB-IoT.

-	Transsion ask how the UE discover the cell if the SSB is not presented.   Interdigital thinks that the assumption that the cell is completely off but rather that the periodicity may be long or on demand.

Noted

[1min]

Discussion

enable energy-efficient system information transmission and paging operation to allow good power saving possibilities for both networks and UE

6G System information design should consider energy saving friendly (for both network and UE) SI scheduling/acquisition mechanism.

SSB periodicity

SIB1 size

-	Lenovo has done some test to understand how much paging is done and found out that it is over designed.  System information is very stable and doesn’t change very much and we should design something that is useful in the field.    Xiaomi agrees with Lenovo and we should not do a very complex design.

-	CATT thinks that from network perspective the most important part is the energy efficiency, on demand SIBx.   CMCC thinks we should include SSBs as the savings would be limited with only ondemand SIB.  Ericsson indicates that the periodicity of SSB is also very important.

-	Mediatek points out the flexibility and extensibility of scheduling design and we should study.

-	Nokia thinks that we should ensure that we communicate with RAN1 to tell them what we need.

-	Samsung thinks that SI update is important.

-	BT would like to understand the impact of this schemes on the UE.

-	Ericsson thinks we should ensure public warning system can be delivered.   LG thinks that we haven’t yet agreed that it would be transmitted by SIBs.  Ericsson clarifies that it doesn’t mean that, we just have to make sure that requirements are met.

-	Thales thinks that we should consider NTN

Area SIBs

Agreements on System Information

- -	System information design should consider energy efficiency and low complexity for both network and UE.

-	System information design should ensure that the public warning requirements are met.

-	Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

-	Study how to improve flexibility and extensibility of scheduling SI design

-	Study mechanisms to improve SI update mechanism

-	Study aspects related to SIB1 size.  Understand the issues and desired content/size from RAN2 point of view.

-	Study areas specific SIB design.  Understand 5G pain points and what improvements can be considered.

NOTE: this is a starting list

Paging

R2-2507111	Views on Directions of 6G Control Plane Enhancements	Apple	discussion	Rel-20	FS_6G_Radio

Proposal 6: RAN2 study on 6G paging focuses on network energy saving (e.g. non-uniformed PO distribution), UE power saving (e.g. subgroup and WUS based paging), UE implementation simplification (e.g. single PO set to monitor paging), and user experience improvement (e.g. carrying richer MT paging info).

-	Xiaomi asks if this WUS is LPWUS or a general.  Apple explains it is general.

-	Vivo asks if we add more info in paging do we need to consider security aspects.   Apple is open to look at the security for paging.

Noted

[2min]

R2-2507172	Views on Control Plane for 6G	Qualcomm Incorporated	discussion	Rel-20	FS_6G_Radio

Proposal 5:  	RAN2 studies the feasibility of unifying IDLE and Inactive state procedures from the following aspects:

-	Unified location area management

-	Unified paging procedure

-	UE context storage in RAN and/or CN

-	CATT doesn’t think that context storage is RAN2 scope and the context storage depends on the RRC state.

-	Honor asks which node triggers the paging.  Qualcomm thinks that is up to discussion.

Noted

[2min]

R2-2506900	Discussion on 6G control plane	CMCC	discussion	Rel-20	FS_6G_Radio

Observation 2: In 5G, the enhancements of paging mechanism are essential in following scenarios:

In scenarios involving high frequency bands and massive device connections, paging capacity constraints emerge, resulting in increased paging delay.

In multi-carrier scenario, sending the same paging message across all carriers within a Tracking Area (TA) incurs excessive overhead.

In NTN scenario with multiple SSB periodicity, the paging mechanism fails to adapt to multiple SSB periodicity, potentially leading to paging failures or unnecessary paging monitoring.

Proposal 5: 6G paging should consider the following enhancements:

In multi-carrier scenario, paging message can be transmitted on either one or more carriers respectively to alleviate paging overhead or increase paging capacity for different purposes.

-	Apple asks how it works and whether paging can be done in multiple carriers.  Interdigital explains that in NBIoT the UE can be paged in other carriers and it is simple.

-	Qualcomm would like to avoid cases where the UE has to measure multiple carriers.  Also never heard that paging capacity is the problem.  We should first know what is the problem and UE complexity is more important.   CMCC agrees that complexity is important to avoid and we can rely on UEs to perform measurements in anchor carrier.

-	Lenovo thinks that we should avoid having to announce whether UE is anchor or other carrier.

-	LG sees the benefit from the energy saving perspective.

-	Nokia agrees capacity is not the main issue, unless we are dealing with some other different devices and we should consider random access capacity.

-	Ericsson asks if this is for same band or different band.  CMCC thinks that both cases should be studied.

Companies can continue thinking for next meeting what is the actual problem and how to solve it in a simple way.

In NTN scenario, the paging mechanism can adapt to the SSB periodicity by configuring multiple sets of paging parameters corresponding to different SSB periodicity.

Noted

[2min]

Agreements

Paging design should consider energy efficiency and simplicity for both UE and Network.

Initial Access and random access

Not treated:

R2-2507072	Controlling the 6G access stratum	Ericsson	discussion	Rel-20

Proposal 5	Study a common random-access procedure for all UEs (including lowest-complexity UEs), with a baseline assumption of 4-step RA, CBRA/CFRA, a simplified and generic Msg1 indication framework, and a larger MSG3 size in the cell edge.

[2min]

R2-2507574	Discussion on 6G AS control plane design	Google Korea LLC	discussion	FS_6G_Radio

Proposal 6: The 2-step RACH procedure should be considered the baseline for initial access in addition to the 4-step RACH procedure.

[2min]

R2-2506846	Discussion on 6GR control plane	OPPO	discussion	Rel-20	FS_6G_Radio

Proposal 3	For 6G initial access, RAN2 study schemes to reduce always-on signal transmission/reception by enabling adaptive/on-demand triggering of related signal (e.g., SSB, SIB, RACH, Paging).

Proposal 5	For 6G initial access, RAN2 study the scheme(s) for flexible offloading/re-distribution (e.g., for RACH, Paging) across frequency-domain resources.

[2min]

R2-2507069	Consideration on 6G control plane	ZTE Corporation, Sanechips	discussion	Rel-20	FS_6G_Radio

Proposal 1: A unified multi-cell/carrier coordinated initial access procedure should be supported for efficient utilization of network resources, e.g. spectrum, time and spatial domain resources, satellite orbits, to provide overall coverage, high throughput with reduced energy consumption at both network and UE side.

[2min]

Not treated

R2-2506769	Discussion on control plan in 6G	Transsion Holdings	discussion

R2-2506774	Discussion on 6GR control plane protocol design	Xiaomi	discussion	Rel-20	FS_6G_Radio

R2-2506819	Overview of 6GR Control Plane	CATT	discussion	Rel-20	FS_6G_Radio

R2-2506859	6G Radio protocol architecture	Lenovo	discussion	FS_6G_Radio

R2-2506884	Discussion on control plane aspects in 6GR	China Telecom	discussion	Rel-20	FS_6G_Radio

R2-2506890	RAN2 Enhancements Considerations for Fixed Wireless Access	T-Mobile USA Inc.	discussion

R2-2506891	Service Aware RAN RAN2 consideration	T-Mobile USA Inc.	discussion

R2-2506895	Initial Considerations for 6GR Control Plane	Sharp	discussion	Rel-20	FS_6G_Radio

R2-2506932	Initial Considerations on RRC Protocol Architecture for 6GR	TCL	discussion

R2-2507035	Considerations of Control plane for 6G Radio	HONOR	discussion	Rel-20	FS_6G_Radio

R2-2507073	RRC, ASN.1 and other signalling aspects for 6G	Ericsson	discussion	Rel-20

R2-2507096	RRC Signaling Framework with more close integration with the slices	Panasonic	discussion	Rel-20

R2-2507131	Initial consideration on Control plane aspects in 6G	Fujitsu	discussion	Rel-20	FS_6G_Radio

R2-2507142	Consideration on control plane aspects for 6G	LG Electronics Inc.	discussion	Rel-20	FS_6G_Radio	Withdrawn

R2-2507182	Discussion on RRC state for 6G	ETRI	discussion

R2-2507187	Control Plane aspects for 6G	Ofinno	discussion	Rel-20	FS_6G_Radio

R2-2507203	Discussions on 6G Control Plane	Futurewei	discussion	Rel-20

R2-2507321	Discussion on 6GR Rel-20 Control plane aspects	Sony	discussion	Rel-20	FS_6G_Radio

R2-2507332	Discussion on Radio Protocol Architecture – Control Plane	Rakuten Mobile, Inc	discussion	Rel-20

R2-2507341	Consideration on control plane aspects for 6G	LG Electronics Inc.	discussion	Rel-20	FS_6G_Radio

R2-2507373	Overview of 6G Control Plane aspects	NEC	discussion	Rel-20	FS_6G_Radio

R2-2507392	Discussion on 6GR Control Plane	Fraunhofer IIS, Fraunhofer HHI	discussion

R2-2507466	Initial consideration for RRC modeling in 6GR 	Kyocera	discussion

R2-2507503	Initial consideration for 6G initial access aspects 	Kyocera 	discussion	Rel-20

R2-2507556	Discussion on basic procedures of control plane for 6GR	TCL	discussion

R2-2507578	Considerations on Control Plane for 6GR	KDDI Corporation	discussion

R2-2507646	6G Control Plane design aspects for NTN	THALES, Airbus, Echostar, Novamint, Fraunhofer IIS	discussion	Rel-20	FS_6G_Radio

#### 10.3.3	Common User plane and Control plane

Access stratum security aspects, in alignment with requirements from SA3.

Transfer of various type of data (including AI/ML data, sensing, etc. ) and general AI/ML framework considerations.

NOTEs: Detailed AI/ML use case specific proposals are not expected in this meeting.   Specific technical details/procedures related to sensing are not expected until RAN1 starts 6G sensing work.

Data Collection/Management/Transfer

Principles and requirements

R2-2507514	Considerations on 6G AI/ML Data Collection and Management	AT&T Labs, Inc	discussion

Proposal 1: The following core principles are followed to design an AI/ML framework for 6GR air interface:

- A unified flexible LCM framework for model management, model transfer, model training, and model testing

- A unified data collection framework to enhance management efficiency

- Network visibility to drive innovation while proactively addressing security and privacy concerns

- Network control over data collection to ensure network performance is not impacted while providing potential new value opportunities via hosting/routing/augmenting the data

- Scalability to accommodate various emerging and future use cases.

Proposal 2: AI/ML framework in 6GR should support multiple termination points for AI/ML data within the network with MNO visibility

Proposal 3: 6GR is designed to differentiate AI/ML data management traffic from user plane traffic and control plane traffic

Proposal 5: For 6GR, study the feasibility of a unified, service-agnostic data collection framework. All requirements proposed for AI/ML specific data collection framework are applicable for the unified framework.

-	Xiaomi asks what use case and data types they have in mind.   AT&T indicates AI/ML data

-	Oppo asks if we need to consider more than functionality based LCM and model based.  ATT thinks we can fold both in the same framework.

-	ZTE asks what unified is, is it different services, use cases, working group.  We should first clarify the requirement and then see how we can build a unified framework.  ATT thinks that the work has to be done with other groups but we don’t want to consider on use case bases like in 5G.

Noted

[3 mins]

R2-2506909	Consideration on general AI/ML framework and data collection	CMCC	discussion	Rel-20	FS_6G_Radio

Proposal 2: 6G RAN should support large volume of data transmission (e.g. for model training) and model transfer/delivery, as well as avoid duplicated data collection and reporting.

-	Apple asks where the requirement comes from.   CMCC explains that we studied both CP and UP solution and is concerned that the RRC is not sufficient to carry large data.  And some data can be kept in the RAN.

-	CATT thinks that avoiding duplicate data collection is related to how many UEs the network selects for data collection.   CMCC thinks that we can’t always avoid but we can target to minimize it.

Proposal 4: A unified RAN data collection framework should be supported for diversified data collected from 6G new services, e.g. AI and sensing.

-	Qualcomm asks how we can do all this unification.    CMCC would like to reuse the framework as much as possible, and of course if at the end of the study we can’t unify everything we can adjust.   There are some data that doesn’t need to go to the CN.    Qualcomm thinks that we should first understand the requirement then framwork.

Proposal 5: The following requirements in 5G-A can be taken as starting point for AI data collection and transfer in 6G:

- The data collected is secured and data integrity and confidentiality for that data is ensured.

- User data privacy, anonymity and user consent is respected.

- The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose. This includes initiating, terminating, and fully managing data transfer.

- MNO has full visibility for standardized data.

- The design is future-proof and extendable.

-	Apple asks if this is mainly applicable for AI/ML or also for sensing.  CMCC thinks we can start with AI/ML and we can wait  for sensing as we don’t know all the requirements/details

Proposal 6: It is proposed to follow 5G-A mechanism that the user consent can be configured by OAM.

-	LG asks if user consent is dependent on use cases.   CMCC confirms that it depends on the type of information and sensitive information from the UE.    In R18 SA3 introduced user consent based on OAM.

Noted

[3 mins]

Discussion on requirements

- The data collected is secured and data integrity and confidentiality for that data is ensured.

- User data privacy, anonymity and user consent is respected.

- The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose. This includes initiating, terminating, and fully managing data transfer.

- MNO has full visibility for standardized data.

- The design is future-proof and extendable.

-	Xiaomi thinks that these requirements are only for AI/ML data types.   Apple thinks that this is only for standardized data.

-	Huawei thinks that it natural that we continue with the AI/ML but we should also consider the requirements for all other use cases like sensing.

-	Fraunhaufer wonders if we should also talk about UE power consumption.

-	Nokia thinks that we should exclude the transparent (ex. 1a) and we shouldn’t spend any time in 3GPP discussion.    Samsung doesn’t think the intention is to spend time on discussing thinks that are not 3GPP specific but we can’t capture that we rule out transparent tranfer.

-	Qualcomm would like to separate the use cases for UE sided or network sided data collection.

-	Vivo thinks that we can take these as baseline.

-	Interdigital think that when we think about requirements we should not  focus on a simple use cases but we should think of having generic requirements that can apply to multiple use case.

-	Verizon thinks that these requirements apply to both network and UE side data collection.

-	Dish thinks that this requirement is important and it should be applicable to all data in the network.   BT agrees that data is data and we should apply it to both UE side and NW side.

-	Qualcomm thinks that we should study non-standarized data collection.

-	Oppo wonders which part is not applicable to network sided.  Apple thinks that there may be some UE privacy difference.   We should capture that the transparent solution is not precluded.   Xiaomi and Samsung think that we can capture transparent non-3GPP solution are supported like for 5GA.

-	Ericsson thinks that we should study the termination points and it is important that we unify the solutions across multiple working groups.   Mediatek thinks that termination points are related to data consumers and also depend on other working groups.

-	Nokia thinks that for unification we need to discuss which particular aspects we are unifying.

-	ZTE would like to understand if this IP based or non-IP based and who will decide.

-	Apple is concerned that the third sub-bullet is outside of our term of references.

R2-2507153	Qualcomm's Views on 6G AI/ML Framework for RAN2	QUALCOMM Europe Inc. - Italy	discussion	Rel-20	FS_6G_Radio

Proposal 3: 6G Requirements for UE data collection for network use-cases: [Qualcomm 7153]

- The UE data collection for network use-cases should have minimal impact to the UE battery, CPU and APU Processing and  memory utilization.

- UE data collection for network use-cases should have minimal impact to UE power saving features such as DRX, inactive state, and other NES features.

-	Lenovo ask what this means.   Qualcomm wants to avoid request for data collection that has impact on the features.

- There should be minimal interruptions and minimum retransmissions of data, whether due to mobility or other reasons.

- Lenovo asks if we would have a new requirement as this is up to the network scheduling.  Qualcomm thinks the point is to not create retransmission just because of design that require moving of points.

- The UE should be able to postpone requests for UE data collection for network use-cases based on UE internal considerations.

- Ericsson ask what they had in mind with this.   CMCC thinks that the network would never want to impact UE performance so the UE should follow network.   Qualcomm thinks that if we can put a requirement on the network that it shall release configuration when the user has low battery for example.   Docomo thinks that we are open to thinking further but this may have impacts.

- There should be a way to differentiate any data collection traffic to avoid charging the user.

- User data privacy, anonymity and user consent is ensured (not a RAN2 requirement).

Noted

[2 mins]

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions

For next meeting companies can consider at least the following aspects:

- Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc);

  - For each type of data, study:

  - Applicable use case(s)

  - End point pairs (i.e., producer and consumer), including UE and RAN, UE and CN, RAN and CN, RAN and OAM;

  - Data size in a single reporting and total data size

  - Frequency of data reporting;

  - QoS requirements (e.g., latency, priority, GBR, packet error rate, etc);

R2-2506775	Consideration on 6GR data transfer, AI/ML framework and security	Xiaomi	discussion	Rel-20	FS_6G_Radio

Proposal 2: Study data collection and transfer in the following aspects:

- Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc);

  - For each type of data, study:

  - Applicable use case(s)

  - End point pairs (i.e., producer and consumer), including UE and RAN, UE and CN, RAN and CN, RAN and OAM;

  - Data size in a single reporting

  - Frequency of data reporting;

  - QoS requirements (e.g., latency, priority, GBR, packet error rate, etc);

- For SON/MDT, QoE related data, assumption on data size/latency/frequency used in 5GNR can be considered as baseline for 6GR SON/MDT, QoE related data;

- For AI/ML related data, assumption on data size/latency/frequency used in 5GNR can be considered as baseline for 6GR AI/ML related data;

- In terms of data transfer framework for sensing related data, RAN2 needs to wait for further input from RAN1 evaluation on measurement metrics (e.g., data size, reporting frequency, QoS, etc) and RAN2 study on sensing architecture/protocol (e.g., producer, consumer);

- NOTE: Coordination with RAN1, RAN3 and SA is expected.

-  Apple thinks that some data are actual measurements and not data.  Xiaomi thinks that we would need to just understand the different use cases and we can conclude which one is a measurement and which one is not.

Noted

[2 mins]

Not treated:

R2-2507602	AIML Framework and Data Transfer Design	MediaTek USA	discussion	Rel-20

Proposal 1 (objective of data transfer design): It shall be possible to avoid high Uu load due to non-RT data transfer, e.g. by specific network control. L2 enhancements for Uu efficiency of non-RT file transfer can be considered. Once TSG RAN has settled the scope for 6G UE sensing, sensing cases might need to be analyzed for further impact to Data transfer objective (FFS).

Proposal 5: For data collection from the UE for UE-side purpose including model training and vendor performance monitoring, it shall be possible to collect vendor-specific data.

[2 mins]

R2-2507114	Views on Directions of 6G AI/ML general framework and data transfer	Apple	discussion	Rel-20	FS_6G_Radio

Proposal 3: For 6G NW-side data collection:

- Data transfer is terminated in RAN as 5G, and no need to be unified with UE-side data collection.

- Take 5G logging measurement framework as baseline and study its enhancement (e.g. support data collection across all RRC states).

Proposal 4: Due to potential overlapping with 6G “data transfer” and 5G UE-side data collection, postpone the RAN2 discussion on 6G UE-side data collection after their conclusion is clear.

Proposal 5: As one lesson learnt in 5G, RAN2 leaves the discussion on visibility and controllability requirements on 6G UE-side data collection and model transfer to RAN plenary and SA1/SA3.

Proposal 6: RAN2 discuss the detailed list of all data transport tasks for services belonging to “data transfer” in 6G RAN study before deciding whether a unified “data transfer” solution in RAN2 for different 6G services is feasible.

[2 mins]

R2-2506763	Initial consideration for 6GR AI	OPPO	discussion	Rel-20

Proposal 5: 6G data framework should study whether/how to unify the data transfer design between UE-side data collection and network-side data collection. The additional common parts can be studied case by case when identified.

[1 min]

Model transfer requirements

R2-2507583	Guidelines for 6G AI_ML for the air interface model delivery options	BT Plc, T-Mobile USA, Orange, Deutsche Telekom, Turkcell, Verizon, Vodafone, KDDI	discussion (Moved from 10.2)

Proposal 1: RAN2 should focus the AI/ML for the air interface study on 3GPP solutions only.

Proposal 2: 3GPP AI model transfer solutions for the air interface should be deeply studied by 3GPP as part of 6G Study Item.

Proposal 3: The AI model transfer solution shall consider at least the below requirements:

- Size: from RAN2 point of view, aim to support various sizes of the model parameter transfer (FFS on model size);

- Continuity: service continuity of model transfer during UE mobility needs to be supported;

- Controllability: NW decides on if and when to transfer over the air interface;

- Latency: relaxed latency requirement and infrequent update;

Proposal 4: RAN2 needs to perform the study based on at least the following principles for the AI model transfer:

- Model transfer/delivery: traffic should be transferred at a different priority, e.g., lower than user traffic.

- Differentiability: model transfer/delivery traffic should be differentiated from other user traffic.

- Security: there should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models cannot be downloaded.

- Addressability: Models need to be addressable such that the UE can request the transfer/delivery of a specific one

[3 mins]

Architecture (if time allows)

R2-2506786	Considerations on 6G data transfer and AI framework	CATT, CBN	discussion	Rel-20	FS_6G_Radio

Observation 1: There are some limitations based on legacy CP/UP solution for large volumes of data in both UL and DL of 5G AI/ML, to study new mechanism to avoid the issues that arise in 5G is necessary.

Observation 3: From QoS requirement aspect, introduction of a new plane can more flexibly address diverse QoS requirements for large amount data transfer and avoid the repetitive discussion process caused by defining e.g. new SRBs.

Observation 4: From data generation and routing aspect, compared with legacy SRB/DRB, introduction of a new plane can more flexibly configure different start and terminate points of data transmission on the network side, and the delay due to data forwarding via intermediate node can be avoided.

Observation 5: From use case scalability aspect, introduction of a new plane can unify design for large size data transmission of multiple use cases, and future-proof for use cases added in later releases.

Proposal 2: RAN2 to support the data transfer via a RAN data plane for 6G large amount of new service data.

[2 mins]

R2-2507092	Considerations for AI/ML and sensing in 6G	Samsung	discussion	Rel-20	FS_6G_Radio

Proposal 3. Study the following RAN2 impact from the introduction of service plane, in close and timely coordination with SA2 and RAN3.

- Whether/how to introduce new RB(s), minimizing impact to traffic on existing RB(s)

- How to support NW-side visibility/controllability for data transfer

- How to support UE-gNB and/or UE-CN interaction

- Whether/how to support security for 6G-supported data

- Whether/what other use-cases (e.g., SON, MDT, QoE, Positioning, or new 6G services) can be supported over service plane

Proposal 4. Data transfer over service plane supports both NW-side data collection and standardized UE-side data collection for AI/ML model training.

[2 mins]

R2-2506763	Initial consideration for 6GR AI	OPPO	discussion	Rel-20

Proposal 4: For 6G data transfer, the outcome from B5G study can be considered as the baseline and RAN2 study should focus on the following scenarios:

- Scenario 1: UE transfers collected data to OAM, i.e. collected data is terminated at OAM, via RAN using CP/UP.

- Scenario 2: UE transfers collected data to RAN, i.e. collected data is terminated at RAN, via CP/UP.

[2 mins]

Inter-WG group coordination

R2-2507081	Discussions on AIML framework and data transfer	NTT DOCOMO, INC.	discussion

Proposal 6: RAN2 shall coordinate with RAN3 and SA WGs on following issues:

- SA1: The data transfer requirements of emerging use cases like sensing.

- RAN3, SA2, SA5: The QoS requirements for new services and E2E solution for data transfer and/or data collection, new plane and/or RAN-CN interface.

- RAN3, SA3, SA5: The security and UE consent of model and data transfer and/or data collection solutions, e.g. MAC CE encryption and integrity protection.

[2 mins]

AIML

LCM framework and requirements [30 mins]

R2-2506800	Considerations on 6GR AI framework	vivo	discussion	Rel-20

Observation 1: While 5G introduced AI/ML functions through a use-case-specific approach, its fragmentation and lack of coordination make it unsuitable for the AI-native vision of 6G. highlighting the urgent need for a generic, extensible, and unified framework.

Observation 2: UE capability transfer and applicability determination are primary procedures to determine whether an AI/ML model/functionality can be viewed as practical from the deployment entity’s perspective, especially for the case of UE-sided model.

Proposal 1: RAN2 aims to design a generic and extensible 6G AI/ML framework, enabling new AI/ML use cases to be easily introduced and integrated, and supporting both one-sided models (including UE-sided and NW-sided models) and two-sided models.

-	Apple thinks it is too early to think of a unified framework.   ZTE doesn’t think it should consider the network sided model.   Nokia thinks that we develop principles that allow us to apply functions to multiple use cases.

Proposal 2: 6G AI/ML signaling framework should support at least the following functions/procedures:

- UE AI/ML Capabilities Exchange;

- Applicable Functionality Reporting;

- Inference Configuration and Reporting;

- Performance Monitoring Configuration and Reporting;

- Functionality (De-)Activation and Fallback/Switching to the non-AI/ML Functionality;

- Data collection, both UE-side Data Collection and NW-side Data Collection;

- Model Delivery/Transfer.

-	Apple thinks that we didn’t study this in 5G so we should remove it.

-	LG indicates that we are considering dynamic capability reporting so we can use the same framework.  Vivo thinks that we can share it amongst multiple use cases.   Nokia thinks that we should discuss this as the network should be able to reject a model transfer.   Interdigital thinks that the last two bullets can be removed.

-	NEC asks what is the relationship of this LCM with the unified SA framework.  Vivo explains that we are trying to unify the radio related LCM aspects.

-	MEdiatek would like to reduce the interactions between the network and UE for UE sided and have some autonomy.

-	Qualcomm thinks that every feature we add in 6G should be future compatible with AI/ML and we assume that every feature may have AI/ML.

-	BT thinks that we should keep model transfer in the list.

Noted

Agreements on LCM Study 6G AI/ML LCM framework that supports at least the following functions/procedures: - UE AI/ML Capabilities Exchange; - Applicable Functionality Reporting; - Inference Configuration and Reporting; - Performance Monitoring Configuration and Reporting; - Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

[3 mins]

Not treated

R2-2506786	Considerations on 6G data transfer and AI framework	CATT, CBN	discussion	Rel-20	FS_6G_Radio

Proposal 4: For 6G AI/ML framework, develop an enhanced LCM framework to enable future-proof framework applicable to diverse emerging use cases, incorporating:

- Advanced training techniques, e.g. online training

- Continuity of AI/ML features

[1 min]

R2-2506775	Consideration on 6GR data transfer, AI/ML framework and security	Xiaomi	discussion	Rel-20	FS_6G_Radio

Proposal 4: 6GR AI/ML framework should follow the below principles:

- Mitigate the requirement for UEs to maintain excessive models or parameters;

- Avoid on-device training.

- Ensure robust user privacy protection.

[1 min]

L2/L3 AIML Use Cases (If time allows)

R2-2507314	Framework for AI/ML and Transfer of Various Data Types	InterDigital	discussion	Rel-20	FS_6G_Radio

Proposal 5: Prioritize evaluation of use cases where no conventional alternative exists or AIML offers significant improvements to system performance.

Proposal 6: Establish a set of complexity metrics for AI/ML evaluation, including normalized FLOPs, space complexity, and derived energy consumption profiles.

[2 mins]

R2-2507229	Views on common user plane and control plane	ZTE  Corporation, Sanechips	discussion	FS_6G_Radio

Proposal 7: RAN2 to conduct a case by case study on 6G AI/ML use cases and the use case proponent shall provide the following aspects for the use case discussion: 1) Use case description, including the motivation, justification; 2) Performance gain, including the methodology for the gain evaluation; 3) Specification impact, including the evaluations based on each compnent of the LCM.

[1 min]

AS security

LSs

R2-2506743	LS on Early Alignment on Access Stratum security aspects (RP-252891; contact: Vodafone)	RAN	LS in	Rel-20	FS_6G_Radio	To:SA	Cc:RAN2, RAN3, SA3, SA2

Noted

R2-2506762	Reply to LS on Early Alignment on Access Stratum security aspects (SP-251268; contact: Vodafone)	SA	LS in	Rel-20	To:RAN, SA3, RAN2, RAN3	Cc:SA2

Noted

Security for MAC CEs & Other L2 Control

R2-2506896	Initial Considerations for 6GR UP/CP Common Aspects	Sharp	discussion	Rel-20	FS_6G_Radio

Proposal 1: RAN2 to study security protection (i.e. ciphering and integrity protection) of control messages (e.g. MAC CE, PDCP/RLC control PDUs).

Proposal 2: RAN2 to study a unified security framework for data and control messages.

-	Lenovo asks what kind of risks there is with MAC CE, is it only DOS.  Sharp explains that we may have to consider replay attacks but SA3 is the main expert.   Lenovo asks if there has been an analysis on the severity of the problem.  Vodafone thinks that this also depends on the type of MAC CEs and that’s the discussion that SA3 should take.

-	CMCC asks for all data and control need ciphering and integrity.   Sharp thinks that at least for control integrity protection should be necessary.

-	Vivo reminds that we already identified issues in 5G and SA3 acknowledged it but they didn’t do anything in 5G.   Should we first wait for SA3 to acknowledge that MAC control has an issue.

Noted

[2 mins]

R2-2506937	Discussion on Security Requirements in 6GR	CMCC	discussion	Rel-20	FS_6G_Radio

Observation 1: There’s no security protection mechanism on 5G MAC CE, including LTM Cell Switch Command.

Observation 2: Supporting security protection on MAC CE from 6G day1 could facilitate to the smoother discussion across various 6G topics.

Observation 3: With security protection function centralized in one layer like PDCP, it may introduce extra inter-layer interactions  and extra time delay, and it may results in confusion at the UE between MAC CE and other data.

Proposal 2: The MAC layer should be responsible for security protection operation on MAC CE:

- Not all types of MAC CEs will require security protection;

- Detailed security functions to be supported at the MAC layer, such as ciphering, integrity protection and other potential functions should be further discussed in RAN2 and SA3.

Noted

[2 min]

R2-2507074	Common user and control plane aspects for 6G	Ericsson	discussion	Rel-20

Observation 1: Working groups tend to map control elements to their channels rather than choosing the channel offering the required properties in terms of robustness, message size and security.

Proposal 2: Study placement of access stratum security for data and signalling while avoiding having security in multiple layers.

-	Xioami asks where the multiple layer problem comes from.  Ericsson explains that would be a problem if we MAC layer security so we should first analyse whether we need to use MAC for the signals that need security.  Xiaomi indicates that if we have CU/DU split then we’d have to move PDCP to DU to do security.   Ericsson assumes no CU/DU split but we don’t know how the split will look.  RAN3 should consider the impact to our protocols and RAN2 should be involved.

-	LG asks how can we do SI security if it doesn’t pass PDCP.

Noted

[2 min]

Discussion

-	Qualcomm thinks that first we need to understand the RAN2 involvement in the security.  The real question is do we have to do security for L2 control and how can we decide if we don’t know about MAC CEs yet.  Also security should be in the node where the data originates.  Fujitsu agrees that security should be in the same node where data is originated.

-	Vodafone points out that we know how generally MAC CEs look like so that can be a baseline for understanding how it may look like in 6G.    Is it a big complexity if we have security in MAC for control and PDCP.   Nokia agrees with Vodafone that we do have an idea of what MAC control looks like.

-	Verizon thinks that we have to assume the CU/DU split as we have deployments that we want to use for 6G.   Docomo agrees.

-	ZTE thinks that we have good reasons why we put some signaling in lower layers for some radio level requirements.   Now we have security requirements and we shouldn’t change our radio requirements because of security.   We need to understand which of the L2 message need to meet these requirements.  We shouldn’t move anything to upper layers.

-	Mediatek thinks that we should first look at the functionalities and we should avoid duplication, so we should try to have a goal for a unified solution.

-	Sharp thinks from UE perspective we should avoid having too much duplication.

-	Huawei also thinks it is not urgent to have these discussion and we should wait for DU/CU split to have these discussions.   Also even for now it is not clear which MAC control would have these problem and it may not be worth to have security on MAC layer only for LTM for example.   Apple doesn’t thinks we should wait as we have some proposals that may require security and we should ask SA3.

-	Lenovo thinks DU/CU is deployed.  We should avoid having duplicated solution, but even if we have security in two layers it is not duplication as we can use same type.

-	Interdigital thinks that we already determined that we had a problem in 5G for LTM so we should ensure that this is fixed in 6G from the beginning and even if we don’t have DU/CU split it doesn’t matter as higher layer processing is still high so we will still need MAC control for fast changes.   Xiaomi agrees, but we are also discussing unifying mobility so we may need to wait.   Oppo agrees and even in LTE we don’t have CU/DU split.

-	Vivo thinks that this would introduce cross layer signaling if we want to do things in the same layer.

-	LG doesn’t think we should change security we have today and the question is really whether we need security for some of MAC control signaling.    Samsung thinks that some information is moved to lower layers and that’s a motivation.

-	Docomo thinks that this discussion is needed to have here in RAN2 and we shouldn’t just wait for SA3.  Mediatek agrees and because this has hardware implication it should be resolved as soon as possible.

-	Transsion thinks we can wait.

-	ZTE thinks that we can categorize broad categories of signalling and we indicate to SA3 that these are the types of signaling that we are studying to put in lower layers.   Vodafone is concerned that if we try to categorize all MAC CEs it may take a long time.   The question is if we come to the conclusion that we may need L2 control what are the security implication.   Qualcomm thinks that if we want to be proactive, if such control information needs to be protected it should not be done as data.  Oppo thinks that it would be good to inform SA3 that we we are studying L2 and L3 signaling.

-	Xiaomi thinks that the question is whether certain information we send is security sensitive.

-	ZTE thinks that we don’t know what type of security SA3 will design for us, but they have to first discuss and then we can discuss here.

-	Mediatek thinks that SA3 needs to know what information is carried so we can do an analysis on the current MAC CEs for now.    If we go in the direction of having a full stack on the DU that would bring it’s own problems.

-	Ericsson is afraid that SA3 might say that everything should be secure and that increased delays.  ZTE indicates that we have to indicate that from radio perspective overhead and processing is important.

-	Ericsson asks if some information in L1 like CSI, etc may need security and if it does we shouldn’t have secuiryt in multiple layers.  Those type of sensitive information should be sent by L2.

Send LS to SA3 to indicate the existing 5G MAC CE information and that some of these control information may be carried over in 6G L2.  Ask them what information would require security.  Explain RAN2 concerns of overhead (size and mobility security context exchange) and processing.  Please identify only critical information that needs to be secure and what type of security (i.e. integrity, ciphering).

Indicate that if there are information that critical to be protected RAN2 and SA3 should work jointly to develop a solution.

Nice to get a response as soon as possible.

[POST131bis][018][6G] LS to SA3 on security (Vodafone)

Intended outcome:  Approve LS to SA3

Deadline:  1 week

=> Approved in R2-2507945

R2-2507933	LS on Early Alignment on Access Stratum security aspects	RAN2	LS out	Rel-19	FS_6G_Radio	To:SA3	Cc:RAN1, RAN3, SA2

R2-2507945	LS on Early Alignment on Access Stratum security aspects	RAN2	LS out	Rel-19	FS_6G_Radio	To:SA3	Cc:RAN1, RAN3, SA2

=> Approved

Not treated:

False base station and Integrity for Broadcast/System Information (SIB/SI)

R2-2507226	Discussion on access stratum security aspects	KT Corp.	discussion

Proposal 2. Security protection function to avoid cyberattacks should be considered in 6G SI.

R2-2507218	Considerations for Energy Saving and AS Security in 6GR 	Samsung, Verizon	discussion	Rel-20	FS_6G_Radio

Observation 7: In 5G NR, system information broadcast is prone to potential threats from False Base Stations.

Proposal 7: RAN2 to study enhanced AS layer security to protect broadcast system information message and mitigate threat from false base stations.

[2 min]

R2-2506937	Discussion on Security Requirements in 6GR	CMCC	discussion	Rel-20	FS_6G_Radio

Observation 4: Although SA3 studied the security design of system information during the SI stage, the final NR specification does not capture the security protection of system information.

Observation 5: A key question to SA3 regarding system information security is whether the system information in 6G network is genuinely faces security threats or what are new potential risks.

Proposal 1: RAN2 is proposed to incorporate the security protection for MAC CEs and System Information as a requirement for subsequent alignment between SA3 and RAN2, and send LS to SA3 to clarify RAN2’s requirements. The detail solution should be determined by SA3.

[2 mins]

R2-2506787	Considerations on 6G AS security	CATT	discussion	Rel-20	FS_6G_Radio

Proposal 5: Enhance the integrity protection of broadcasting system information in 6G Day 1.

Proposal 6: Apply integrity protection only for essential system Information.

[2 mins]

Inter-WG Coordination

R2-2506775	Consideration on 6GR data transfer, AI/ML framework and security	Xiaomi	discussion	Rel-20	FS_6G_Radio

Proposal 7: RAN2 waits for SA3’s input on requirements for access stratum security before starting any related study.

[2 mins]

R2-2507229	Views on common user plane and control plane	ZTE  Corporation, Sanechips	discussion	FS_6G_Radio

Observation 2 : The solution for the transmission of low layer command (e.g. legacy MAC CE based and/or DU terminated local RRC based) will impact the discussion in SA3 for the security protection of low layer command.

Proposal 1: RAN2 should identify the type of lower layer signalling that is likely to be specified for 6G and convey this information to SA3 and RAN2 should study the format for the transmission of lower layer signalling (e.g. MAC CE based or Local RRC based), and inform SA3 the corresponding conclusion.

[2 mins]

R2-2507397	Early alignment on the air interface security	Vodafone GmbH	discussion	Rel-20

Proposal: It is proposed that this RAN 2 meeting sends an LS to SA3 with the following action:

RAN2 kindly ask SA3 to provide security requirements for RRC Layer and below (e.g. MAC) by June 2026, and RAN2 invites SA3 to engage in early dialogue to facilitate this.

R2-2507398	Draft LS on Early Alignment on Access Stratum security aspects 	Vodafone GmbH	LS out	Rel-20	To:SA3	Cc:RAN 1,RAN 3, SA 2

[2 min]

Network and UE Energy Efficiency

Design targets

R2-2506855	General considerations for 6G in RAN2	Huawei, HiSilicon	discussion	Rel-20	FS_6G_Radio (moved from 10.2)

Observation 3-1: Energy-saving features typically require support from both UEs and networks. The earlier these energy-saving features are supported, the more UEs and networks will be able to adopt them—thus allowing the intended energy-saving gains to be achieved and maximized.

Proposal 3: 6GR should support Energy Saving features from Day-1. RAN2 should strive to develop solutions that address both network energy efficiency and UE power saving, to enhance their chance of being commercialized.

Noted

[1 min]

R2-2507074	Common user and control plane aspects for 6G	Ericsson	discussion	Rel-20

Observation 8: Network energy saving techniques introduced in NR can provide limited benefits due to backwards compatibility constraint and limited UE support. 6G’s NES functionality must hence be specified and implemented by UEs and networks from day one.

Proposal 5: Network energy efficiency should be considered as a key requirement for all features developed for 6G. Configuration options (e.g. placement of reference signals and paging occasions) that optimize energy efficiency should be the primary/mandatory choice

Noted

[1 min]

R2-2507133	Initial consideration on 6GR CP/UP common aspects	Fujitsu	discussion	Rel-20	FS_6G_Radio

Proposal 3: RAN2 to study and decide fundamental and essential power saving functions in Day1 (both NW and UE side).

Proposal 4: RAN2 to study efficient mechanism to enable independent or joint power saving between UE and NW.

Noted

[1 min]

Discussion

RAN2 should strive to develop solutions that address both network energy efficiency and UE power saving

-	Lenovo asks if we have solutions in mind that can do both.   Apple explains that there were things like DTX/DRX.

-	Interdigital thinks that we need to strike a balance between UE and network energy saving.  We can look at network energy saving first and then see how it impacts the UE.

-	Xiaomi thinks that we can do some analysis in RAN2 on things that don’t require RAN1.

-	Mediatek thinks that we need to consider both sides.  Qualcomm would like to ensure that it doesn’t impact performance.

Design goal

1. 	Strive to develop solutions that consider network energy efficiency and UE power saving

Alignment of power saving features

R2-2507615	6GR Common Aspects	Nokia, Nokia Shanghai Bell	discussion	Rel-20	FS_6G_Radio

Proposal 4: RAN2 to discuss and decide which UE power saving features are inherited from 5G to 6G while avoiding specifying multiple features for the same purpose.

-	Lenovo supports this but wonders how we approach this and what would be the criteria.

Noted

[2 mins]

R2-2507074	Common user and control plane aspects for 6G	Ericsson	discussion	Rel-20

Observation 9: Studies have shown significant reduction in the network energy consumption if default SSB periodicity is extended to at least 160 ms.

Observation 11: To extend the deep sleep opportunities for improving network energy savings, adaptations of common channel and signalling transmissions/receptions, such as paging and random-access occasions, should be coordinated accordingly.

Proposal 4: Study LP-WUS and RRM neighbour cell measurement relaxation for IDLE/INACTIVE and review and align the toolbox of LP-WUS, C-DRX, SCell (de-)activation and PDCCH-switching features for CONNECTED mode.

-	Vivo asks what is the intention of studying in RAN2.   Ericsson thinks that we can study them all and then we can downselect the features we already have.

-	ZTE asks how we can align these features.   Ericsson thinks that should be part of the study and see which functions we want to align.

-	Nokia asks what is the basic assumption on the SSB periodicity

Noted

[2 mins]

R2-2507180	Energy efficiency and AS security for 6GR	InterDigital	discussion	Rel-20	FS_6G_Radio

Proposal 2:  For energy efficiency in control channel monitoring, the following features are assumed from day-1 for all UEs: LP-WUS/WUR, C-DRX, PDCCH skipping, and cell DTX/DRX.

Noted

[1 mins]

R2-2507113	Views on Directions of 6G User Plane Enhancements	Apple	discussion	Rel-20	FS_6G_Radio (moved from 10.3.1)

Proposal 7: RAN2 should study how C-DRX flexibility can be enhanced to improve UE power efficiency.

Noted

For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose.   Focus on solutions that have RAN2 impacts.

[1 mins]

Time domain

R2-2507218	Considerations for Energy Saving and AS Security in 6GR 	Samsung, Verizon	discussion	Rel-20	FS_6G_Radio

Observation 1: 5G networks face inherent limitations in energy efficiency due to fundamental trade-offs between power savings and QoS requirements, always-on signaling, and lack of harmonization between UE and network power-saving technologies.

Observation 3: In 5G NR, Cell DTX/DRX were introduced to enhance network energy efficiency in the RRC Connected state but were not extended to Idle and Inactive Modes, leaving untapped potential for energy savings.

Proposal 1: RAN2 should study a unified Cell DTX/DRX mechanism that works in RRC Connected, Idle and Inactive Modes.

[2 mins]

R2-2507340	Consideration on general aspects for 6G	LG Electronics Inc.	discussion	Rel-20	FS_6G_Radio (moved from 10.2)

Proposal 5: Study Cell DTX/DRX for 6G, focusing on significantly reducing downlink and uplink activity during OFF periods (and potentially achieving complete suspension when feasible), in order to maximize network energy saving while minimizing UE experience degradation.

[2 mins]

Frequency domain

R2-2507615	6GR Common Aspects	Nokia, Nokia Shanghai Bell	discussion	Rel-20	FS_6G_Radio

Proposal 3: From day one 3GPP should design 6G energy saving mechanisms taking into account joint network and UE optimizations for both coverage and capacity layers, single and multiple carrier deployments and allowing reduced transmissions of always-on signals.

[2 mins]

R2-2507218	Considerations for Energy Saving and AS Security in 6GR 	Samsung, Verizon	discussion	Rel-20	FS_6G_Radio

Observation 2:5G networks also encountered challenges from increasing network complexity due to mixed deployments and heterogeneous device integration, as well as compatibility gaps arising from the gradual development of new features.

Proposal 2: RAN2 should study carrier separation architecture from 6GR's initial phase, focusing on network deployment strategies and UE initial access procedures to optimize both network energy efficiency and UE power consumption.

[2 mins]

Not treated

R2-2506770	Discussion on energy efficient in 6G	Transsion Holdings	discussion

R2-2506801	Considerations on 6G data collection and data transfer	vivo, NTT DOCOMO, INC.	discussion	Rel-20

R2-2506851	Discussion on the RAN2-related 6G security aspects	OPPO	discussion	Rel-20	FS_6G_Radio

R2-2506892	AI RAN RAN2 consideration	T-Mobile USA Inc.	discussion	Withdrawn

R2-2506897	6GR AI/ML Framework	Sharp	discussion	Rel-20	FS_6G_Radio

R2-2506911	Discussion on data transfer and general AIML framework for 6G	Spreadtrum, UNISOC	discussion	Rel-20

R2-2506918	Discussion on transfer of various type of data and general AI/ML framework	Transsion Holdings	discussion	Rel-20

R2-2506955	Discussion on data transfer and AI/ML framework in 6G	Huawei, HiSilicon	discussion	Rel-20	FS_6G_Radio

R2-2506974	Support for Location Dependent Data Collection 	Fraunhofer IIS, Fraunhofer HHI	discussion

R2-2507036	Discussion on Common User plane and Control plane for 6GR	HONOR	discussion	Rel-20	FS_6G_Radio

R2-2507128	Considerations on Common User plane and Control plane for 6G	LG Electronics Inc.	discussion	Rel-20	FS_6G_Radio

R2-2507188	Overview of Common User Plane and Control Plane	Ofinno	discussion	Rel-20	FS_6G_Radio

R2-2507204	Discussions on 6G Common Aspects of UP and CP	Futurewei	discussion	Rel-20

R2-2507225	Discussion on data transfer design to support various type of data	KT Corp.	discussion

R2-2507239	Considerations on Common User plane and Control plane for 6G AI/ML and Sensing	LG Electronics Inc.	discussion	Rel-20

R2-2507268	Discussion on the radio protocols for transfer of various type of data	ITRI	discussion	NR_newRAT-Core

R2-2507291	Considerations for 6G unified CP and UP designs for TN and NTN	Lenovo	discussion	Rel-20	FS_6G_Radio

R2-2507293	6GR Common User Plane and Control Plane aspects 	Lenovo	discussion

R2-2507317	Discussion on common user plane and control plane for 6GR	ITL	discussion	Rel-20

R2-2507322	Discussion on 6GR Rel-20 Common UP & CP aspects	Sony	discussion	Rel-20	FS_6G_Radio

R2-2507335	Data Plane for AI-ML data collection	Rakuten Mobile, Inc	discussion	Rel-20

R2-2507357	Enhancements on New Type of Data in 6G	Jio Platforms	discussion	Rel-20

R2-2507374	Transfer of various data and AIML framework	NEC	discussion	Rel-20	FS_6G_Radio

R2-2507388	Consideration on general AI/ML framework and data transfer	China Unicom	discussion	Late

R2-2507425	On AIML data transfer, interface protocols, framework	Nokia	discussion	Rel-20	FS_6G_Radio

R2-2507449	On 6G AS security	NTT DOCOMO, INC.	discussion	Rel-20	Withdrawn

R2-2507486	AI-Native Radio Protocols for 6G	Qualcomm Incorporated	discussion

=> Revised in R2-2507655

R2-2507655	AI-Native Radio Protocols for 6G	Qualcomm Incorporated, MediaRek Inc.	discussion

R2-2507515	Considerations on 6GR general AI/ML framework	TCL	discussion

R2-2507543	Discussion on 6G AIML framework	ASUSTeK	discussion	Rel-20

R2-2507545	Discussion on 6G Data Collection and Management	Futurewei Technologies	discussion	Rel-20

R2-2507564	Discussion on Common User plane and Control plane	ETRI	discussion	Rel-20

R2-2507580	Initial Considerations on the Impact of AS Security and New Services on 6G Common CP and UP	China Telecom	discussion	Rel-20	FS_6G_Radio

## 10.4	Mobility

General mobility principles, types (e.g. L3, CHO, LTM, RLM/RLF, cell reselection), and measurements. Including Inter-RAT and intra-RAT mobility.

Mobility framework and targets

R2-2507075	6G Mobility	Ericsson	discussion	Rel-20

Observation 5	The mobility procedures in NR serve different purposes, but in the end multiple options have been specified addressing similar requirements.

Proposal 3	The mobility framework in 6G supports both short interruption time and high robustness in the first release.

-	Lenovo as what short is, is it the 0ms interruption and if so do you think this is really needed.  Ericsson thinks that we can aim as close to 0 but we don’t want to have multiple solutions.    Xiaomi thinks that the important requirement is to ensure seamless HO for application, but we should have target on robustness.   Ericsson thinks that conditional framework provides high robustness.

Proposal 4	6G mobility has a single framework that supports:  - both conditional and immediate execution - both UL and DL pre-synchronization - operation with and without candidate pre-configurations - short and flexible execution command

-	Samsung asks what is this short and flexible execution.  Ericsson thinks that RRC can handle the short execution without MAC CE.

Noted

[3 min]

R2-2507385	Initial Thoughts on 6G MMM (Mobility, Measurements and Migration)	Nokia	discussion	Rel-20	FS_6G_Radio

Proposal 1: Study a 6GR unified handover (UHO) procedure unifying selected mobility procedures defined in 5G and 5G-Advanced. The study shall consider the following aspects:

•	both UE and NW implementation aspects of the handover procedure.

•	eMBB use-case shall be well addressed.

•	the value added by each procedure to baseline UHO procedure.

-	ZTE asks eMBB is the most important.  Nokia confirms it is the baseline.

-	Vivo asks if configuration should also be unified.  Nokia explains that the intention to avoid what happened in 5G and we had a lot of mobility features added throughout the releases.

Proposal 2: Study procedures to enable interruption time reduction during mobility, ensure robustness and high cell edge throughput. In particular RAN2 considers the following:

•	For interruption time reduction consider early DL synchronization, early UL synchronization and early decoding.

-	Mediatek asks this is the considering LTM like procedures.   Nokia doesn’t want to limit the discussion yet to LTM even though we like the LTM, but we have RACH-less so we should consider all of them.

•	For ensuring mobility robustness study the procedures that show clear gains versus complexity.

•	For ensuring high cell edge throughput prioritize procedures that offers the highest gains.

-	ZTE points out that for 0ms interruption can be achieved by beam level so when we talk about cell level mobility we can focus on nearly 0ms.

-	Mediatek asks what is prioritized procedures.   Nokia explains we should not consider marginal gains and high complexity, so we should focus on solutions with high gains.

Noted

[3min]

R2-2506858	Discussion on 6GR mobility designs	Huawei, HiSilicon	other	Rel-20	FS_6G_Radio

Observation 1:	6GR mobility and beam management framework should jointly consider practical deployment scenarios, handover performance and UE/NW implementation complexity from day 1.

Observation 2: 	UE and network implementation complexity is a critical factor in commercial adoption of mobility solutions.

Proposal 2:	The 6GR handover procedure should be designed based on the following targets:

	Simplification and unification;

	Minimization latency/interruption time/data loss;

-	CATT asks about the data loss.  Huawei explains the intentions is to minimize but no data loss may not always be necessary.

	Enhancement of robustness.

Noted

[2min]

R2-2507169	Consideration on 6G Mobility	ZTE Corporation	discussion	Rel-20	FS_6G_Radio

Observation 3	Seamless handover can be achieved by a combination of mTRP and L3 HO/LTM in 6G, e.g. beam-level mobility based on mTRP within “super” cell and LTM across “super” cells.

-	Sony asks what is super cell.  ZTE explains that in the real field we group TRPs in super cells to minimize amount of handovers and we should consider some form of inter-TRP mobility.

-	Xiaomi indicates that we don’t need paging so why.  ZTE explains that the common channels need to change and the current framework requires a full configuration, but we only to change a sub-set of configuration.

-	Oppo thinks that beam level mobility is RAN1.  ZTE thinks that cell is not yet defined as we beam level is also mobility.   Oppo thinks that RAN1 needs to be discussed.   ZTE thinks that RAN2 should also study and understand what are the scenarios and then understand the RAN1 centric ones and coordinate.

-	Samsung agrees with ZTE and RAN1 and RAN2 have same target and come up with different solutions so we should consider this together in 6G.

-	Lenovo is concerned that we are doing things without understand the problems.

-	Huawei thinks that we can start discussions on overall mobility and worry about the split later.

-	CMCC understands that within hyper cell the UE will perform beam level and only perform cell level across hyper cell.

Proposal 2	For mobility in RRC_CONNECTED, RAN2 to consider the following requirements in 6G mobility design:

•	Low interruption time, e.g. nearly 0ms interruption time;

•	Robustness improvement;

•	Throughput improvement, e.g. avoid throughput degradation during mobility;

•	Unified and simplified signalling design.

Noted

[2min]

Discussion

Interruption time target, robustness, unified.

-	Huawei thinks that the problem was not that they weren’t added from day on but rather that they weren’t integrated.   Also measurement reporting is complicate and RAN1 and RAN2 were working on similar things.   Maybe faster RACH procedure can help and we may not need RACHless.

-	Apple asks about this single framework, what is unified, layer, trigger?.  Ideally we shouldn’t have subsequent enhancements after day1.  ZTE thinks that the first one is configuration and procedure and the other part is measurements.  Mediatek agrees with ZTEs explanation, but we need to take a step back and understand what is the problem.  In 5G it was difficult to enhance as there were a lot of dependencies and separate pieces of functionality.   We can maybe have a system where we can introduce enables a bit more easily.   Future extensibility is important so we should consider what we may introduce in the future.

-	Qualcomm thinks that we should aim to support RACH-less for UL synch.

-	Jio doesn’t want to give up on DAPS.

-	LG asks whether this

-	Interdigital is aligned with a single framework and we should move away from harmony as harmony in music is multiple voices at once.

-	Docomo and CMCC think interruption time is not the most important but rather service continuity and ensure the throughput doesn’t go.   ZTE agrees and one solution is early CSI acquisition.

-	ZTE thinks that robustness is very important.

-	Mediatek thinks that we need to simplify parametrization.   Apple thinks that architecture will impact the mobility procedure, for example do we need a key change for every mobility.   Ericsson agrees with Apple and we need to first understand what is mobility and all the different aspects.

-	Nokia thinks that we can wait until next meeting to understand what is unified.

-	Vivo thinks that when we discuss unified we should provide some requirements to RAN1.

-	LG Thinks that unified that from a configuration perspective is one step.

-	Huawei is concerned that early DL and UL

For interruption time reduction consider early DL synchronization, early UL synchronization

6G mobility has a single framework that supports:  - both conditional and immediate execution - both UL and DL pre-synchronization - operation with and without candidate pre-configurations - short and flexible execution command

Agreements

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

Not treated

R2-2506899	Consideration on the mobility in 6GR	CMCC	discussion	Rel-20	FS_6G_Radio

Observation 1: An interim milestone is set as TSG#113 (September/2026) for RAN plenary to  make a decision on additional migration option(s), including 6G-6G DC, therefore, RAN2 can focus on mobility solutions for standalone architecture before that time.

Observation 2: L3 measurement based immediate HO can achieve almost 99% HO success probability on FR1, and it makes sense to take it as baseline of 6G mobility.

Observation 3: CHO can avoid the deterioration of UE experience and handover successful probability, even in high speed scenario.

Proposal 1: For 6G mobility, it is proposed to take L3 measurement based immediate HO as baseline, besides, RACH-less HO,CHO and (C)LTM are supported from 6G day 1.

Observation 4: The minimum interruption time of LTM can almost be 13ms, while L3 measurement based immediately HO latency is also tens of milliseconds e.g., 20ms, the benefits of LTM seems not so attractive.

Proposal 2: The following aspects should be considered in 6G mobility design:

1.	Support both beam level and cell level measurement results based mobility, and study a unified 6G measurement framework;

2.	Pursue a unified mobility configuration and procedure to avoid redundancy in RRC configuration, and LTM can be taken as baseline;

3.	Further study to reduce mobility interruption time, e.g., to achieve ms-level mobility interruption time.

[2min]

R2-2507135	Views on Mobility and RRM for 6G	Qualcomm Incorporated	discussion	Rel-20	FS_6G_Radio

Proposal 2: 6G should support both intra-6G mobility to a prepared cell (i.e., via pre-configuring UE with one or more candidate cell configurations before the cell switch) and intra-6G mobility to a non-prepared cell (i.e., cell switch to a target cell whose configuration is not provided prior to the cell switch) from day-1.

Observation 3: RACH-less handover is supported in NR for intra-gNB handovers (for both TN and NTN), but was specified for inter-gNB handovers only in case of NTN and mobile IAB-DU migration procedure. Also, unlike RACH-less LTM, RACH-less handover doesn’t support early UL/DL sync procedures.

Proposal 3: 6G should support both “RACH-less” intra-6G mobility (e.g., via early DL/UL sync towards target/candidate cells) and RACH-based intra-6G mobility.

[2min]

Measurement Framework

R2-2507135	Views on Mobility and RRM for 6G	Qualcomm Incorporated	discussion	Rel-20	FS_6G_Radio

Observation 4: 5G measurement framework is quite fragmented and supports different measurement resource and reporting framework for L1/L3 measurements, serving/neighboring cells, SSB vs. CSI-RS.

Proposal 4: 6G should strive to support a unified measurement configuration framework e.g., via unification of measurement resource configuration structure (for L1/L3 measurements, for xserving/neighbor cells/beams, for different types of RS) and unification of measurement report configuration structure.

-	Xiaomi asks what about the reporting.   Qualcomm explains that we can discuss reporting later.

-	CATT asks how can we unify given that CSI measurements are in RAN1 scope.

-	Apple thinks that this a good intention but this should come much later as we don’t yet know the L1 reference signals and what L2/L3 reference signals should be.   Qualcomm thinks that this is a design goal.  Apple thinks that we should go one step further and consider future design and then design the other WGs.

-	ZTE thinks that we can unify the measurement configuration but the reporting configuration will depend on the CU/DU split.    Qualcomm thinks we can unify by sending the report to CU.  ZTE won’t to avoid increasing network implementation complexity.

Noted

[2min]

R2-2507562	Mobility for 6GR	MediaTek Inc.	discussion

Proposal 3: RAN2 assumes that separate type of mobility RSs are used for IDLE and CONNECTED mode mobility procedure. RAN2 should discuss the impact to mobility procedure based on this assumption.

-	Interdigital thinks that this may be premature and what do you think the impacts are.   Mediatek thinks that just the configuration part.  Xiaomi thinks that this is RAN1 business.  Ericsson explains that this is more about the modelling from RAN2 point of view.   The reference signal locations can be different in the two states, so we need to account for this in RAN2.

Proposal 4: RAN2 should aim to have a harmonized framework/procedure for different use cases. Considering the following direction:

•	Measurement of Beam Management (BM) and CONNECTED mode mobility should be based on the same RS.

•	Unified measurement report (e.g. in MAC-CE). No separate reporting in L1 and L3.

•	Report per RS – FFS on beam consolidation

Noted

[2min]

R2-2507385	Initial Thoughts on 6G MMM (Mobility, Measurements and Migration)	Nokia	discussion

Proposal 3: RAN2 to study how to reduce measurement overhead and scheduling limitations when compared to 5G. Gap-assisted measurements should consider the following aspects:

•	non-uniform gap pattern to support efficient gap-based measurements of deactivated serving cells or for load balancing

•	dynamic network-controlled gap use

•	gap distribution in time and resource domain across UEs to enable efficient scheduling at the network.

Proposal 4: Study RRM measurements framework for different RRC states under single objective to simplify and unify measurement configurations. Consider reusing measurement results across RRC states and idle mode measurements for fast CA setup in 6G Day One.

Noted

[2min]

Discussion

-	Ericsson agrees that we should look at all the RSs and aim to create a single framework.

-	Xiaomi thinks that we should identify the lesson learned and pain point from 5G design.    Apple and Nokia indicate that there was RAN4 papers on measurement gap.  Apple thinks that measurement gap was a big mess and we have to acknowledge that this may happen again in 6G so we should address a future proof way of handling the measurement gap.   ZTE thinks that RAN4 has this topic as highest priority.

-	Qualcomm thinks that we should think of a signaling framework that accounts for different services, e.g. measurement, positioning, MUSIM gap.   Mediateks agrees that there were problems and even if RAN4 is discussing we can also look and highlight the problem.

Not treated:

Cell Reselection

R2-2507217	Study on 6G Mobility Framework	Samsung, Verizon	discussion	Rel-20	FS_6G_Radio

Proposal 6: NR cell reselection framework is the baseline of 6G cell reselection framework.

[2min]

R2-2506899    Consideration on the mobility in 6GR         CMCC   discussion        Rel-20   FS_6G_Radio

Proposal 3: For mobility in RRC_IDLE/INACTIVE, i.e., cell reselection, both single cell associate with one carrier and single cell with multiple carriers (hyper cell) are considered.

[1min]

R2-2507143    Consideration on mobility aspects for 6G   LG Electronics Inc.        discussion        Rel-20   FS_6G_RadioProposal 5 Study enhanced cell reselection for network to enable camping cell distribution efficiently and to support network slicing in a simpler but fully functional manner.

[1min]

R2-2506885	Discussion on mobility aspects in 6GR	China Telecom	discussion	Rel-20	FS_6G_Radio

Proposal 2: Slice aware cell reselection for 6GR shall be considered and the design in NR could be taken as a starting point for further study.

[1min]

Inter-RAT

R2-2507294	Discussion on 6G mobility	NTT DOCOMO, INC.	discussion	Rel-20

Proposal 7: For interworking between 6GR and NR, cell reselection, handover, and redirection can be the solution as baseline.  The points RAN2 should discuss are following.

- SIB for cell reselection. (e.g., whether extending SIB5 or introducing new SIB in NR spec.)

- Inter-RAT measurement configuration.

[2mins]

NTN (will only treat NTN related papers in General section this meeting)

R2-2507647	6G Mobility aspects for NTN	THALES, Airbus, Echostar, Novamint, Fraunhofer IIS	discussion	Rel-20	FS_6G_Radio

Proposal 2	Study a design for 6G procedures supporting TN to NTN and NTN to TN mobility for UEs in Idle and Connected mode.

Proposal 3	Study mobility mechanisms for 6G satellite networks (NTN to NTN), at both idle and connected modes. The study should address:

•	Inter-beam mobility

•	Inter-cell mobility

•	Inter-satellite mobility

Proposal 5	Consider the support of idle and connected mode mobility between

•	6G NTN and NR NTN

•	6G NTN and LTE TN

•	6G TN and 5G NR-NTN

Proposal 6	The 6G radio interface/access shall be defined to support multi-connectivity or Dual Connectivity between two NTN accesses (e.g. GSO and NGSO based) as well as between 6G NTN and 6G TN with simultaneous traffic flow in both/different access links.

R2-2506776	Discussion on 6G mobility	Xiaomi	discussion	Rel-20	FS_6G_Radio

Lesson learnt from 5G (6): 5G NTN-NTN lacks the support of the multi-orbit scenarios, and mobility between NTN and TN (including cell reselection procedure) has not been sufficiently addressed in 5G.

Proposal 7: In 6G RAN2 study on mobility for NTN scenario, 5G NTN mobility solution can be used as starting point. RAN2 further study, including:

-	Incorporation of orbital information of the target cell in HO and cell reselection procedures for multi-orbit scenarios.

-	Development of optimized NTN-TN mobility solutions to support seamless service continuity and efficient mobility in integrated NTN-TN deployments.

R2-2506771	Discussion on mobility in 6G	Transsion Holdings	discussion

R2-2506802	Considerations on 6GR mobility	vivo	discussion	Rel-20

R2-2506811	Discussion on 6GR Mobility	CATT	discussion	Rel-20	FS_6G_Radio

R2-2506853	Discussion on measurement and mobility framework for 6GR	TCL	discussion

R2-2506889	Views on 6G Mobility	Fainity Innovation	discussion

R2-2506898	Initial Consideratioins for 6GR Mobility	Sharp	discussion	Rel-20	FS_6G_Radio

R2-2506916	General considerations on mobility for 6GR	Spreadtrum, UNISOC	discussion	Rel-20

R2-2506939	Initial consideration on 6GR Mobility	Fujitsu	discussion	Rel-20	FS_6G_Radio

R2-2506973	On 6G-Mobility	Fraunhofer HHI, Fraunhofer IIS	discussion

R2-2507037	Discussion on Mobility management for 6GR	HONOR	discussion	Rel-20	FS_6G_Radio

R2-2507095	Initial considerations on 6G Mobility	OPPO	discussion	Rel-20	FS_6G_Radio

R2-2507120	Planning for 6G Mobility Study	Apple	discussion	Rel-20	FS_6G_Radio

R2-2507143	Consideration on mobility aspects for 6G	LG Electronics Inc.	discussion	Rel-20	FS_6G_Radio

R2-2507189	Key considerations for mobility in 6G	Ofinno	discussion	Rel-20	FS_6G_Radio

R2-2507206	Discussions on 6G Mobility	Futurewei	discussion	Rel-20

R2-2507221	Discussion on 6G Mobility and measurement	Lenovo	discussion	Rel-20	FS_6G_Radio

R2-2507247	Discussion on 6G mobility	ETRI	discussion	Rel-20	FS_6G_Radio

R2-2507278	Considerations for 6G mobility design	Panasonic	discussion	Rel-20

R2-2507292	Views on Mobility for 6GR	KDDI Corporation	discussion	Rel-20

R2-2507323	Discussion on 6GR Rel-20 mobility aspects	Sony	discussion	Rel-20	FS_6G_Radio

R2-2507336	Discussion on mobility aspects of 6G Radio Access Technology	Rakuten Mobile, Inc	discussion	Rel-20

R2-2507365	Cell-Pair Specific Inter-RAT Mobility Configuration	Jio Platforms	discussion	Rel-20

R2-2507366	Fundamentals of 6G Mobility	Jio Platforms	discussion	Rel-20

R2-2507375	Overview of mobility procedures in 6G	NEC	discussion	Rel-20	FS_6G_Radio

R2-2507391	Discussion on 6G Mobility framework	Tejas Network Limited	discussion	Rel-20

R2-2507432	Connected Mobility for 6GR	InterDigital, Inc.	discussion	Rel-20	FS_6G_Radio

R2-2507463	6G Mobility Framework	CEWiT	discussion

R2-2507487	Discussion on 6G Mobility	Google	discussion	Rel-20

R2-2507500	Consideration of mobility for 6G study 	Kyocera 	discussion	Rel-20

R2-2507544	Discussion on 6G Mobility design	ASUSTeK	discussion	Rel-20

R2-2507584	High level requirements for 6GR mobility	BT plc	discussion

# 11	Breakout session reports

No documents shall be submitted to this AI or its sub-AIs. It is only for at-meeting-generated contents.

## 11.1	Session on R18 and R19 Mobility

R2-2507701	Report from session on R18 SL, R18/19 MOB, and R19 NES	Session chair (Ericsson)	Report

Approved

## 11.2	Session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others

R2-2507702	Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others	Vice Chairman (CATT)	Report

RO type indication in SBFD CR should be removed from SBFD and treated in separate CR that also includes mobility WI.

Approved

## 11.3	Session on NES, NR NTN and IoT NTN

R2-2507703	Report from session on NES, NR NTN and IoT NTN	Session chair (ZTE)	Report

Approved

## 11.4	Session on positioning and sidelink relay

R2-2507704	Report from session on positioning and sidelink relay	Session chair (MediaTek)	Report

Approved

## 11.5	Session on XR and LTE-based 5G Broadcast

R2-2507705	Report from session on XR and LTE-based 5G Broadcast	Session chair (Huawei)	Report

Approved

## 11.6	Session on maintenance and SON/MDT

R2-2507706	Report from session on maintenance and SON/MDT	Session chair (Ericsson)	Report

Approved

# Closing of the meeting

The meeting was closed by the chair at 12:47 CEST on Friday, 17th of October.

# Annex A:	List of participants

RAN2#131bis participants list is attached to this report.

Total number of participants: 460

# Annex B:	List of Tdocs

The list of tdocs from RAN2#131bis is attached to this report.

Total of 1142 tdoc numbers were allocated out of which 1311 tdocs were made available.

# Annex C:	Incoming liaison statements

| TDoc | Title | Source | Status | Rel | Related WIs | To | Cc | Original LS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R2-2506704 | LS on delayed A-IoT D2R NAS messages (C1-255165; contact: Huawei) | CT1 | noted | Rel-19 | Ambient_IoT_Solutions, AmbientIoT-CT | RAN2 | SA2 | C1-255165 |
| R2-2506705 | Reply LS on UE usage of the RAT restrictions (C1-255319; contact: Apple) | CT1 | noted | Rel-19 | ECRATU | RAN2 | CT4, RAN | C1-255319 |
| R2-2506706 | Reply LS to SA4 on the RAN simulation assumptions for ULBC (C1-255650; contact: Qualcomm) | CT1 | noted | Rel-19 | FS_ULBC | SA4 | SA2, SA1, RAN1, RAN2, RAN4 | C1-255650 |
| R2-2506707 | LS on Broadcasting Information on Disaster Condition of a PLMN from E-UTRAN in Case of Disaster Condition (C1-255678; contact LGE) | CT1 | noted | Rel-19 | MINT_Ph2 | RAN2 | SA2 | C1-255678 |
| R2-2506708 | LS on the maximum supported AIoT NAS container length (C1-255679; contact: Lenovo) | CT1 | noted | Rel-19 | AmbientIoT-CT | RAN2 | RAN1, RAN3 | C1-255679 |
| R2-2506709 | LS on Structure updates of AIoT Identifiers (C4-253575; contact: CICT) | CT4 | noted | Rel-19 | AmbientIoT-CT | SA2, RAN3 | SA3, RAN2 | C4-253575 |
| R2-2506710 | LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#122 (R1-2506426; contact: NTT DOCOMO, AT&T) | RAN1 | noted | Rel-19 | NR_AIML_air, NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_Mob_Ph4, NR_NTN_Ph3, NR_MC_enh2, TEI19, NR_LBCA_Sw | RAN2 | RAN4 | R1-2506426 |
| R2-2506711 | LS on updated Rel-19 RAN1 UE features lists for LTE after RAN1#122 (R1-2506429; contact: NTT DOCOMO, AT&T) | RAN1 | noted | Rel-19 | IoT_NTN_Ph3, IoT_NTN_TDD, LTE_terr_bcast_Ph2 | RAN2 | RAN4 | R1-2506429 |
| R2-2506712 | LS on Ambient IoT Stage-2 TP (R1-2506523; contact: CMCC) | RAN1 | noted | Rel-19 | Ambient_IoT_Solutions | RAN2 |  | R1-2506523 |
| R2-2506713 | Reply LS on non-RedCap UE UL SRS frequency hopping for positioning (R1-2506531; contact: ZTE) | RAN1 | noted | Rel-19 | TEI19 | RAN3 | RAN2 | R1-2506531 |
| R2-2506714 | LS on updated text proposal for 36.300 for IoT NTN TDD mode (R1-2506535; contact: Qualcomm) | RAN1 | noted | Rel-19 | IoT_NTN_TDD | RAN2 |  | R1-2506535 |
| R2-2506715 | Reply LS on UL Tx switching for TEI19 (R1-2506538; contact: MediaTek) | RAN1 | noted | Rel-19 | TEI19 | RAN4, RAN2 |  | R1-2506538 |
| R2-2506716 | Reply LS on the RAN simulation assumptions for ULBC (R1-2506541; contact: Qualcomm) | RAN1 | noted | Rel-19 | FS_ULBC | SA4 | RAN4, RAN2, SA2, CT1 | R1-2506541 |
| R2-2506717 | Reply LS on on CB Msg3 EDT for IoT NTN Ph3 (R1-2506553; contact: MediaTek) | RAN1 | noted | Rel-19 | IoT_NTN_Ph3 | RAN2 |  | R1-2506553 |
| R2-2506718 | Reply LS on CSI-RS based CFRA using SBFD RO (R1-2506556; contact: ZTE) | RAN1 | noted | Rel-19 | NR_duplex_evo-Core | RAN2 | RAN4 | R1-2506556 |
| R2-2506719 | Reply LS on RA-RNTI for PRACH adaptation (R1-2506587; contact: Ericsson) | RAN1 | noted | Rel-19 | Netw_Energy_NR_enh | RAN2 |  | R1-2506587 |
| R2-2506720 | LS on Rel-19 higher layers parameters list Post RAN1#122 (R1-2506626; contact: Ericsson) | RAN1 | noted | Rel-19 | NR_AIML_air, NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_Mob_Ph4, NR_XR_Ph3, NR_NTN_Ph3, IoT_NTN_Ph3, IoT_NTN_TDD, NR_MC_enh2, NR_LBCA_Sw, LTE_terr_bcast_Ph2, TEI19 | RAN2, RAN3 | RAN4 | R1-2506626 |
| R2-2506721 | Reply LS on NR Femto node shared by PLMN and PNI-NPN (R3-252337; contact: LGE) | RAN3 | noted | Rel-19 | 5G_Femto, NR_WAB_5GFemto-Core, eNPN | SA2 | RAN2 | R3-252337 |
| R2-2506722 | Reply LS on Logged Data Handling During Handover (R3-255824; contact: Nokia) | RAN3 | noted | Rel-19 | NR_AIML_air-Core | RAN2 | SA5 | R3-255824 |
| R2-2506723 | Reply LS on emergency call back and paging (R3-255883; contact: Qualcomm) | RAN3 | noted | Rel-17 | NR_newRAT-Core, NR_redcap-Core | SA2 | RAN2, CT1, RAN | R3-255883 |
| R2-2506724 | Reply LS on MBS Communication Service Type (R3-255896; contact: Nokia) | RAN3 | noted | Rel-18 | NR_QoE_enh-Core | SA4 | RAN2, SA5 | R3-255896 |
| R2-2506725 | LS on compatibility issue for PEI and emergency PDU session (R3-255906; contact: ZTE) | RAN3 | noted | Rel-17 | NR_UE_pow_sav_enh-Core | SA2 | CT1, RAN2 | R3-255906 |
| R2-2506726 | LS on the handling of inter-DU L2 reset for LTM (R3-255907; contact: Ericsson) | RAN3 | noted | Rel-18 | NR_Mob_enh2-Core | RAN2 |  | R4-2511781 |
| R2-2506727 | LS on allocation of CN assigned subgroup ID for LP-WUS (R3-255941; contact: NTT DOCOMO) | RAN3 | noted | Rel-19 | NR_LPWUS-Core | SA2 | RAN2, RAN1 | R3-255941 |
| R2-2506728 | LS on geographical area scope MDT (R3-255960; contact: CATT) | RAN3 | noted | Rel-19 | NR_ENDC_SON_MDT_Ph4-Core | RAN2, SA5 |  | R3-255960 |
| R2-2506729 | LS on 8Rx UE receiver capability definition update request (R4-2509151; contact: China Telecom, Ericsson) | RAN4 | noted | Rel-18 | NR_ENDC_RF_FR1_enh2-Perf | RAN2 |  | R4-2509151 |
| R2-2506730 | LS on RRC signalling for power domain enhancement (R4-2511759; contact: Huawei) | RAN4 | noted | Rel-19 | NR_ENDC_RF_Ph4-Core | RAN2 |  | R4-2511759 |
| R2-2506731 | Reply LS on maximum transmission power for STxMP (R4-2511781; contact: vivo) | RAN4 | noted | Rel-20 | NR_MIMO_evo_DL_UL-Core | RAN1 | RAN2 | R4-2511781 |
| R2-2506732 | Response LS on the RAN simulation assumptions for ULBC (R4-2511782; contact: Xiaomi) | RAN4 | noted | Rel-20 | FS_ULBC | SA4 | RAN2, RAN1, SA2 | R4-2511782 |
| R2-2506733 | LS on capability of NR_LBCA_Sw (R4-2511863; contact: Huawei) | RAN4 | noted | Rel-19 | NR_LBCA_Sw | RAN2 |  | R4-2511863 |
| R2-2506734 | LS on Rel-19 RAN4 UE feature list for NR (version 2) (R4-2511884; contact: CMCC) | RAN4 | noted | Rel-19 | NR_ENDC_RF_Ph4, NonCol_intraB_ENDC_NR_CA_Ph2, NR_ATG_enh, NR_RRM_Ph5, Netw_Energy_NR_enh, NR_LPWUS, NR_Mob_Ph4, NR_XR_Ph3, NR_FR1_lessthan_5MHz_BW_Ph2, NR_LBCA_Sw, NR_FR1_7MHz_BW, NR_IoT_NTN_req_test_enh, NR_AIML_air, NR_NTN_Ku_bands, NR_NTN_Ph3 | RAN2 | RAN1 | R4-2511884 |
| R2-2506735 | LS on Release Independence of 6Rx (R4-2511898; contact: T-Mobile) | RAN4 | noted | Rel-19 | NR_ENDC_RF_Ph4-Core | RAN2 | RAN1 | R4-2511898 |
| R2-2506736 | Reply LS on CSSF optimization for NR RRM Phase 5 (R4-2512161; contact: Apple) | RAN4 | noted | Rel-19 | NR_RRM_Ph5-Core | RAN2 |  | R4-2512161 |
| R2-2506737 | LS on CB-msg3-EDT (R4-2512173; contact: MediaTek) | RAN4 | noted | Rel-19 | IoT_NTN_Ph3-Core | RAN2 |  | R4-2512173 |
| R2-2506738 | LS on event triggered L1-RSRP reporting if eventDetectionTimeWindowLength-r19 is configured (R4-2512232; contact: Qualcomm) | RAN4 | noted | Rel-19 | NR_MIMO_Ph5-Core | RAN1 | RAN2 | R4-2512232 |
| R2-2506739 | LS on Rx BSF optimization for NR RRM Phase 5 (R4-2512333; contact: CICT | RAN4 | noted | Rel-19 | NR_RRM_Ph5-Core | RAN2 |  | R4-2512333 |
| R2-2506740 | LS on definition of CSI-RS based L1 intra/inter-frequency measurement (R4-2512334; contact: Apple) | RAN4 | noted | Rel-19 | NR_Mob_Ph4-Core | RAN2 | RAN1 | R4-2512334 |
| R2-2506741 | LS Reply on precompensation for NB-IoT NTN TDD mode (R4-2512550; contact: Iridium) | RAN4 | noted | Rel-19 | IoT_NTN_TDD-Core | RAN1 | RAN2 | R4-2512550 |
| R2-2506742 | LS on UE Capability for Rel.19 Ku band VSAT (R4-2512658; contact: Chunghwa Telecom, Sharp) | RAN4 | noted | Rel-19 | NR_NTN_Ku_bands | RAN2 |  | R4-2512658 |
| R2-2506743 | LS on Early Alignment on Access Stratum security aspects (RP-252891; contact: Vodafone) | RAN | noted | Rel-20 | FS_6G_Radio | SA | RAN2, RAN3, SA3, SA2 | RP-252891 |
| R2-2506744 | LS on specification of dataset and model parameters exchange (RP-252966; contact: Qualcomm, InterDIgital) | RAN | noted | Rel-20 | NR_AIML_air_Ph2 | SA | SA2, SA5, RAN2 | RP-252966 |
| R2-2506745 | Reply LS on FS_VMR_Ph2 solution impacts to RAN (Additional ULI) (S2-2504110; contact: Qualcomm) | SA2 | noted | Rel-19 | VMR_Ph2 | RAN3 | RAN2 | S2-2504110 |
| R2-2506746 | Reply LS on the RAN simulation assumptions for ULBC (S2-2507578; contact: Qualcomm) | SA2 | noted | Rel-20 | FS_ULBC | SA4 | RAN1, RAN2, RAN4, SA1, CT1 | S2-2507578 |
| R2-2506747 | LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC (S2-2507636; contact: Qualcomm) | SA2 | noted | Rel-20 | FS_5GSAT_Ph4_ARC | RAN2, SA4, CT1, SA3, SA1, RAN1 |  | S2-2507636 |
| R2-2506748 | Reply LS to Reply LS on the removal of service type information (S2-2507689; contact: LGE) | SA2 | noted | Rel-19 | AmbientIoT-ARC, Ambient_IoT_Solutions | RAN3, RAN2 | RAN1 | S2-2507689 |
| R2-2506749 | Reply LS on energy saving indication from CN to RAN (S2-2507784; contact: LGE) | SA2 | noted | Rel-19 | EnergySys | RAN3 | RAN2 | S2-2507784 |
| R2-2506750 | LS on AIoT Device Permanent ID Length (S2-2507793; contact: Huawei) | SA2 | noted | Rel-19 | AmbientIoT-ARC | RAN2 | RAN1, CT4, SA3 | S2-2507793 |
| R2-2506751 | Reply LS on signalling feasibility of dataset and parameter sharing (S2-2508104; contact: Samsung) | SA2 | noted | Rel-19 | NR_AIML_air-Core | RAN2 | RAN, RAN1, RAN3, SA, SA3, SA5 | S2-2508104 |
| R2-2506752 | LS on UE data collection and data transfer (S2-2508119; contact: Nokia) | SA2 | noted | Rel-20 | FS_AIML_CN_Ph2 | RAN2 | RAN1, RAN3 | S2-2508119 |
| R2-2506753 | Reply LS on paging ID length (S3-252933; contact: CATT) | SA3 | noted | Rel-19 | AmbientIoT-SEC | SA2, RAN2, CT4 | RAN3 | S3-252933 |
| R2-2506754 | LS on bundling period and SPS for ULBC (S4aA250258; contact: Qualcomm) | SA4 | noted | Rel-20 | FS_ULBC | RAN2 | RAN1 | S4aA250258 |
| R2-2506755 | Reply LS on per-UE UE performance metrics (S5-253854; contact: Huawei) | SA5 | noted | Rel-18 | NR_AIML_NGRAN-Core | RAN3 | RAN2 | S5-253854 |
| R2-2506756 | LS on temporary suspension of trace production (S5-253909; contact: Ericsson) | SA5 | noted | Rel-19 | TraceQoE_OAM | RAN3, RAN2 |  | S5-253909 |
| R2-2506757 | Reply LS on signalling feasibility of dataset and parameter sharing (S5-254083; contact: Huawei) | SA5 | noted | Rel-19 | NR_AIML_air-Core | RAN2 | RAN, SA, RAN1, RAN3, SA2, SA3 | S5-254083 |
| R2-2506758 | Reply to RAN2 LS on Number of UEs in RRC_INACTIVE state with data transmission (S5-254084; contact: China Telecom) | SA5 | noted | Rel-19 | PM_KPI_5G_Ph4 | RAN2 | RAN3 | S5-254084 |
| R2-2506759 | Reply to LS on Continuous MDT (S5-254110; contact: Ericsson, Magenta, CATT, ZTE, Huawei) | SA5 | noted | Rel-19 | NR_AIML_NGRAN_enh-Core | RAN3 | RAN2 | S5-254110 |
| R2-2506760 | LS on Study on Modernization of Specification Format and Procedures for 6G (SP-251228; contact: Nokia, Samsung, CMCC, ETSI) | SA | noted | Rel-20 | FS_6GSpecs | RAN1, RAN2, RAN3, RAN4, RAN5, SA1, SA2, SA3, SA4, SA5, SA6, CT1, CT3, CT4, CT6 | RAN, CT | SP-251228 |
| R2-2506761 | LS on Guidance on 6G data related work tasks (SP-251261; contact: CMCC | SA | noted | Rel-20 |  | SA2, SA5 | SA3, SA6, RAN, RAN2, RAN3 | SP-251261 |
| R2-2506762 | Reply to LS on Early Alignment on Access Stratum security aspects (SP-251268; contact: Vodafone) | SA | noted | Rel-20 |  | RAN, SA3, RAN2, RAN3 | SA2 | SP-251268 |
| R2-2507134 | LS on input data from UE for case 3b (S2-2507583; contact: Samsung) | SA2 | noted | Rel-19 | NR_AIML_air, NR_AIML_air-Core, AIML_CN | RAN2 | CT1, CT4 | S2-2507583 |
| R2-2507710 | Reply LS on precompensation for NB-IoT NTN TDD mode (R1-2508064; contact: Qualcomm) | RAN1 | noted | Rel-19 | IoT_NTN_TDD-Core | RAN4 | RAN2 |  |

61 incoming LS which were all noted.

# Annex D:	Outgoing liaison statements

| TDoc | Title | Rel | Related WIs | To | Cc |
| --- | --- | --- | --- | --- | --- |
| R2-2507727 | Reply LS on the handling of inter-DU L2 reset for LTM | Rel-18 | NR_Mob_enh2-Core | RAN3 |  |
| R2-2507743 | Reply LS on Release Independence of 6Rx | Rel-19 | NR_ENDC_RF_Ph4-Core | RAN4 | RAN1 |
| R2-2507744 | Reply LS on Broadcasting Information on Disaster Condition of a PLMN from E-UTRAN in Case of Disaster Condition | Rel-19 | MINT_Ph2 | CT1 | SA1, SA2 |
| R2-2507745 | Reply LS on temporary suspension of trace production | Rel-19 | TraceQoE_OAM | SA5 | RAN3 |
| R2-2507746 | LS on SBFD and CA | Rel-19 | NR_duplex_evo-Core | RAN1 | RAN4 |
| R2-2507768 | LS on OD-SSB | Rel-19 | Netw_Energy_NR_enh-Core | RAN1 | RAN4 |
| R2-2507784 | Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC | Rel-20 | IoT_NTN_Ph4-Core | SA2 | RAN1, SA4, SA1, SA3, CT1 |
| R2-2507789 | LS on Location-based SMTC enhancement | Rel-19 | NR_NTN_Ph3-Core | RAN4 |  |
| R2-2507790 | Reply LS on the RAN simulation assumptions, bundling period and SPS for ULBC | Rel-20 | FS_ULBC | SA4 | RAN1, SA2, CT1 |
| R2-2507909 | LS on OCC for RACH-less HO | Rel-19 | NR_NTN_Ph3-Core | RAN1 | RAN4 |
| R2-2507915 | LS on integrity failure | Rel-19 | Ambient_IoT_solutions | SA3 | SA2, RAN3, CT1 |
| R2-2507920 | LS on Security parameter in A-IoT paging | Rel-19 | Ambient_IoT_solutions | SA3 | CT1, RAN3 |
| R2-2507922 | Reply LS on input data from UE for Case 3b | Rel-19 | AIML_CN, NR_AIML_Air, NR_AIML_air-Core | SA2 | CT1, CT4 |
| R2-2507928 | Reply LS on UE data collection and data transfer | Rel-20 | FS_AIML_CN_Ph2 | SA2 | RAN1, RAN3 |
| R2-2507929 | Reply LS on signalling feasibility of dataset and parameter sharing | Rel-20 | NR_AIML_air_Ph2 | SA2 | RAN1, RAN3, SA3, SA5 |
| R2-2507930 | LS to SA2 on the paging in Store and Forward | Rel-19 | IoT_NTN_Ph3-Core | SA2 | RAN3 |
| R2-2507931 | Reply LS on AIoT Device Permanent ID Length | Rel-19 | Ambient_IoT_Solutions | SA2 | RAN1, CT4, SA3 |
| R2-2507932 | Reply LS on the maximum supported AIoT NAS container length | Rel-19 | Ambient_IoT_Solutions | CT1 | RAN3 |
| R2-2507935 | Reply LS on Rx BSF optimization for NR RRM Phase 5 | Rel-19 | NR_RRM_Ph5-Core | RAN4 |  |
| R2-2507945 | LS on Early Alignment on Access Stratum security aspects | Rel-20 | FS_6G_Radio | SA3 | RAN1, RAN3, SA2 |

20 outgoing LS.

# Annex E:	List of agreed in principle CRs

| TDoc | Title | Source | Rel | Spec | Related WIs | CR | Rev | Cat |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R2-2506790 | Correction on nrPreviousCell logging in RLF report | CATT | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh2-Core | 5483 |  | F |
| R2-2506791 | Correction on nrPreviousCell logging in RLF report | CATT | Rel-19 | 38.331 | NR_ENDC_SON_MDT_enh2-Core | 5484 |  | A |
| R2-2506824 | Corrections on LPHAP, carrier phase, bandwidth aggregation and frequency hopping for positioning | CATT, Ericsson, Nokia, ZTE Corporation | Rel-18 | 38.305 | NR_pos_enh2-Core | 0187 | 3 | F |
| R2-2506825 | Corrections on LPHAP, carrier phase, bandwidth aggregation and frequency hopping for positioning(R19 Cat. A CR) | CATT, Ericsson, Nokia, ZTE Corporation | Rel-19 | 38.305 | NR_pos_enh2-Core | 0198 |  | A |
| R2-2506969 | Correction on AdditionalSpectrumEmission in SL positioning | ZTE Corporation | Rel-18 | 38.331 | NR_pos_enh2-Core | 5494 |  | F |
| R2-2506970 | Correction on AdditionalSpectrumEmission in SL positioning | ZTE Corporation | Rel-19 | 38.331 | NR_pos_enh2-Core | 5495 |  | A |
| R2-2507004 | Correction on subcarrierSpacing values in IE SCS-SpecificCarrier | Huawei, HiSilicon | Rel-17 | 38.331 | NR_ext_to_71GHz-Core | 5500 |  | F |
| R2-2507005 | Correction on subcarrierSpacing values in IE SCS-SpecificCarrier | Huawei, HiSilicon | Rel-18 | 38.331 | NR_ext_to_71GHz-Core | 5501 |  | A |
| R2-2507006 | Correction on subcarrierSpacing values in IE SCS-SpecificCarrier | Huawei, HiSilicon | Rel-19 | 38.331 | NR_ext_to_71GHz-Core | 5502 |  | A |
| R2-2507066 | Correction on uac-BarringFactorForAI3 absence case [MINT] | Huawei, HiSilicon | Rel-17 | 38.331 | TEI17 | 5505 |  | F |
| R2-2507067 | Correction on uac-BarringFactorForAI3 absence case [MINT] | Huawei, HiSilicon | Rel-18 | 38.331 | TEI17 | 5506 |  | A |
| R2-2507068 | Correction on uac-BarringFactorForAI3 absence case [MINT] | Huawei, HiSilicon | Rel-19 | 38.331 | TEI17 | 5507 |  | A |
| R2-2507076 | Correction on UE capability for MP split | ZTE Corporation, Sanechips, CATT | Rel-18 | 38.306 | NR_SL_relay_enh-Core | 1357 |  | F |
| R2-2507077 | Correction on UE capability for MP split | ZTE Corporation, Sanechips, CATT | Rel-19 | 38.306 | NR_SL_relay_enh-Core | 1358 |  | A |
| R2-2507164 | Correction on UL Tx switching MIMO coherence capabilities | ZTE Corporation | Rel-17 | 38.331 | NR_RF_FR1_enh-Core | 5515 |  | F |
| R2-2507165 | Correction on UL Tx switching MIMO coherence capabilities | ZTE Corporation | Rel-18 | 38.331 | NR_RF_FR1_enh-Core, NR_MC_enh-Core | 5516 |  | A |
| R2-2507166 | Correction on UL Tx switching MIMO coherence capabilities | ZTE Corporation | Rel-19 | 38.331 | NR_RF_FR1_enh-Core, NR_MC_enh-Core | 5517 |  | A |
| R2-2507167 | Correction on UL Tx switching MIMO coherence capabilities | ZTE Corporation | Rel-18 | 38.306 | NR_MC_enh-Core | 1359 |  | F |
| R2-2507168 | Correction on UL Tx switching MIMO coherence capabilities | ZTE Corporation | Rel-19 | 38.306 | NR_MC_enh-Core | 1360 |  | A |
| R2-2507191 | Correction on uplink power control for Type-1 CG-PUSCH [PL RS Type 1 CG] | Ofinno | Rel-18 | 38.331 | TEI18 | 5518 |  | F |
| R2-2507263 | Rapporteur correction on CAS muting for LTE based 5G broadcast [5GB_CASMuting] | Huawei, HiSilicon, Samsung | Rel-19 | 36.331 | TEI19 | 5162 | 1 | F |
| R2-2507274 | MAC correction on UE transmissions during Cell DRX | Huawei, HiSilicon | Rel-18 | 38.321 | Netw_Energy_NR-Core | 2128 |  | F |
| R2-2507275 | Stage-2 correction on UE transmissions during Cell DRX | Huawei, HiSilicon | Rel-18 | 38.300 | Netw_Energy_NR-Core | 1043 |  | F |
| R2-2507276 | MAC correction on UE transmissions during Cell DRX | Huawei, HiSilicon | Rel-19 | 38.321 | Netw_Energy_NR-Core | 2129 |  | A |
| R2-2507277 | Stage-2 correction on UE transmissions during Cell DRX | Huawei, HiSilicon | Rel-19 | 38.300 | Netw_Energy_NR-Core | 1044 |  | A |
| R2-2507368 | Correction on Rel-19 NES UE capability | ZTE Corporation, Sanechips, Ericsson | Rel-19 | 38.306 | Netw_Energy_NR_enh-Core | 1362 |  | F |
| R2-2507390 | Correction on pdcp-Config for SRB4 | Samsung, Ericsson | Rel-17 | 38.331 | NR_QoE-Core | 5527 |  | F |
| R2-2507395 | Correction on R18 8Rx UE receiver capability definition | China Telecom, Ericsson, Huawei, HiSilicon | Rel-18 | 38.306 | NR_ENDC_RF_FR1_enh2-Perf | 1365 |  | F |
| R2-2507396 | Correction on R18 8Rx UE receiver capability definition | China Telecom, Ericsson, Huawei, HiSilicon | Rel-19 | 38.306 | NR_ENDC_RF_FR1_enh2-Perf | 1366 |  | A |
| R2-2507519 | Correction to CPAC MRO | ZTE Corporation, CMCC, Huawei, Ericsson, Sanechips | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh2-Core | 5538 |  | F |
| R2-2507520 | Correction to CPAC MRO | ZTE Corporation, CMCC, Huawei, Ericsson, Sanechips | Rel-19 | 38.331 | NR_ENDC_SON_MDT_enh2-Core | 5539 |  | A |
| R2-2507522 | Correction on broadcast service in NTN | ZTE Corporation, Sanechips | Rel-19 | 38.304 | NR_NTN_Ph3-Core | 0448 |  | F |
| R2-2507546 | Correction on pdcp-Config for SRB4 and SRB5 | Samsung, Ericsson | Rel-18 | 38.331 | NR_QoE_enh-Core | 5542 |  | F |
| R2-2507547 | Correction on dependency of group-based beam reporting | Nokia | Rel-18 | 38.331 | NR_MIMO_evo_DL_UL-Core | 5543 |  | F |
| R2-2507548 | Correction on dependency of group-based beam reporting | Nokia | Rel-19 | 38.331 | NR_MIMO_evo_DL_UL-Core | 5544 |  | A |
| R2-2507620 | Correction on setting timeSinceCHO-Reconfig when the failure is due to RLF | Ericsson | Rel-17 | 38.331 | NR_ENDC_SON_MDT_enh-Core | 5555 |  | F |
| R2-2507621 | Correction on setting timeSinceCHO-Reconfig when the failure is due to RLF | Ericsson | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh-Core | 5556 |  | A |
| R2-2507622 | Correction on setting timeSinceCHO-Reconfig when the failure is due to RLF | Ericsson | Rel-19 | 38.331 | NR_ENDC_SON_MDT_enh-Core | 5557 |  | A |
| R2-2507631 | Correction on application of restrictions to mIAB-MT | Samsung | Rel-18 | 38.304 | NR_mobile_IAB-Core | 0449 |  | F |
| R2-2507636 | Correction to F field in MAC subheader for SL-SCH | NTT DOCOMO, INC., Qualcomm Incorporated, Samsung | Rel-16 | 38.321 | 5G_V2X_NRSL-Core | 2132 |  | F |
| R2-2507637 | Correction to F field in MAC subheader for SL-SCH | NTT DOCOMO, INC., Qualcomm Incorporated, Samsung | Rel-17 | 38.321 | 5G_V2X_NRSL-Core | 2133 |  | A |
| R2-2507638 | Correction to F field in MAC subheader for SL-SCH | NTT DOCOMO, INC., Qualcomm Incorporated, Samsung | Rel-18 | 38.321 | 5G_V2X_NRSL-Core | 2134 |  | A |
| R2-2507712 | Correction on the execution of SCG LTM | CATT | Rel-18 | 38.331 | NR_Mob_enh2-Core | 5554 | 1 | F |
| R2-2507713 | Correction on the execution of SCG LTM | CATT | Rel-19 | 38.331 | NR_Mob_enh2-Core | 5558 | 1 | A |
| R2-2507714 | Corrections on validation of reported idle/inactive and reselection measurements | Samsung | Rel-18 | 38.331 | NR_Mob_enh2-Core | 5519 | 1 | F |
| R2-2507715 | Corrections on validation of reported idle/inactive and reselection measurements | Samsung | Rel-19 | 38.331 | NR_Mob_enh2-Core | 5564 |  | A |
| R2-2507716 | Corrections on Rel-18 UE capabilities for LTM | Huawei, HiSilicon | Rel-18 | 38.306 | NR_Mob_enh2 | 1363 | 1 | F |
| R2-2507717 | Corrections on Rel-18 UE capabilities for LTM | Huawei, HiSilicon | Rel-19 | 38.306 | NR_Mob_enh2 | 1364 | 1 | A |
| R2-2507718 | Miscellaneous corrections on mobility enhancements | ZTE Corporation, Sanechips | Rel-18 | 38.331 | NR_Mob_enh2-Core | 5540 | 1 | F |
| R2-2507719 | Miscellaneous corrections on mobility enhancements | ZTE Corporation, Sanechips | Rel-19 | 38.331 | NR_Mob_enh2-Core | 5541 | 1 | A |
| R2-2507723 | Miscellaneous corrections for stage-2 in Rel-19 Mobility Enhancements | Apple Inc | Rel-19 | 38.300 | NR_Mob_Ph4-Core | 1040 | 1 | F |
| R2-2507739 | Introduction of MINT in EPS | LG Electronics Inc., Nokia | Rel-19 | 36.306 | MINT_Ph2 | 1932 | 1 | B |
| R2-2507779 | Miscellaneous Stage 2 corrections for NR NTN phase 3 | THALES (Rapporteur) | Rel-19 | 38.300 | NR_NTN_Ph3-Core | 1047 | 1 | F |
| R2-2507792 | Correction on PC5 Relay RLC channel configuration | Apple, CATT | Rel-19 | 38.331 | NR_SL_relay-Core | 5510 | 1 | A |
| R2-2507793 | U2U Relays, Peer Remote UE Control Plane Procedures | Ericsson, Nokia | Rel-18 | 38.300 | NR_SL_relay_enh-Core | 1045 | 1 | F |
| R2-2507794 | U2U Relays, Peer Remote UE Control Plane Procedures | Ericsson, Nokia | Rel-19 | 38.300 | NR_SL_relay_enh-Core | 1048 |  | A |
| R2-2507795 | Correction to SI reception by remote UE for multi path | LG Electronics | Rel-18 | 38.331 | NR_SL_relay_enh-Core | 5422 | 4 | F |
| R2-2507796 | Correction to SI reception by remote UE for multi path | LG Electronics Inc. | Rel-19 | 38.331 | NR_SL_relay_enh-Core | 5563 | 1 | A |
| R2-2507797 | Correction on processing of sidelink grant on Dedicated SL-PRS resource pool | ASUSTeK | Rel-18 | 38.321 | NR_pos_enh2-Core | 2131 | 1 | F |
| R2-2507798 | Correction on processing of sidelink grant on Dedicated SL-PRS resource pool | ASUSTeK | Rel-19 | 38.321 | NR_pos_enh2-Core | 2135 |  | A |
| R2-2507811 | Introducing CAS Muting [5GB_CASMuting] | ZTE Corporation, Sanechips, Samsung, Huawei | Rel-19 | 36.300 | TEI19 | 1436 | 1 | F |
| R2-2507833 | Clarification of supported band pairs for UL TX switching | Ericsson | Rel-18 | 38.331 | NR_MC_enh-Core | 5553 | 1 | F |
| R2-2507834 | Clarification of the single SCS per frequency restriction | Apple, Nokia, Samsung Inc. | Rel-15 | 38.300 | NR_newRAT-Core | 1049 |  | F |
| R2-2507835 | Clarification of the single SCS per frequency restriction | Apple, Nokia, Samsung Inc. | Rel-16 | 38.300 | NR_newRAT-Core | 1050 |  | A |
| R2-2507836 | Clarification of the single SCS per frequency restriction | Apple, Nokia, Samsung Inc. | Rel-17 | 38.300 | NR_newRAT-Core | 1051 |  | A |
| R2-2507837 | Clarification of the single SCS per frequency restriction | Apple, Nokia, Samsung Inc. | Rel-18 | 38.300 | NR_newRAT-Core | 1052 |  | A |
| R2-2507838 | Stage 2 additions on MDT for slicing | Ericsson | Rel-19 | 37.320 | NR_ENDC_SON_MDT_Ph4-Core | 0147 |  | F |
| R2-2507908 | Clarification of the single SCS per frequency restriction | Apple, Nokia, Samsung Inc. | Rel-19 | 38.300 | NR_newRAT-Core | 1053 |  | A |
| R2-2507910 | Correction for the redirection from E-UTRAN TN to NB-IoT NTN [IoT-TN-NTN-redir] | Google, Samsung, Ericsson, vivo | Rel-19 | 36.331 | TEI19 | 5173 |  | F |
| R2-2507911 | Correction for the redirection from E-UTRAN TN to NB-IoT NTN [IoT-TN-NTN-redir] | Google, Samsung, Ericsson, vivo | Rel-19 | 36.306 | TEI19 | 1933 |  | F |
| R2-2507916 | Introducing SR resources in LTM cell switch MAC CE [LTM_enh_SR] | Ericsson, Continental Automotive, T-Mobile USA, BT Plc., Sharp, Charter Communications, Rakuten Mobile, Verizon, InterDigital, Qualcomm Incorporated, AT&T, Vodafone, MediaTek Inc., NTT Docomo, LG Electronics, Nokia, Deutsche Telekom, Telia Company, Turkcel | Rel-19 | 38.331 | TEI19 | 5530 | 1 | B |
| R2-2507917 | Introducing SR resources in LTM cell switch MAC CE [LTM_enh_SR] | Ericsson, Continental Automotive, T-Mobile USA, BT Plc., Sharp, Charter Communications, Rakuten Mobile, Verizon, InterDigital, Qualcomm Incorporated, AT&T, Vodafone, MediaTek Inc., NTT Docomo, LG Electronics, Nokia, Deutsche Telekom, Telia Company, Turkcell, ZTE Corporation, Samsung | Rel-19 | 38.321 | TEI19 | 2130 | 1 | B |
| R2-2507918 | Introducing SR resources in LTM cell switch MAC CE [LTM_enh_SR] | Ericsson, Continental Automotive, T-Mobile USA, BT Plc., Sharp, Charter Communications, Rakuten Mobile, Verizon, InterDigital, Qualcomm Incorporated, AT&T, Vodafone, MediaTek Inc., NTT Docomo, LG Electronics, Nokia, Deutsche Telekom, Telia Company, Turkcell, ZTE Corporation, Samsung | Rel-19 | 38.306 | TEI19 | 1367 | 1 | B |
| R2-2507919 | Introducing SR resources in LTM cell switch MAC CE [LTM_enh_SR] | Ericsson, Continental Automotive, T-Mobile USA, BT Plc., Sharp, Charter Communications, Rakuten Mobile, Verizon, InterDigital, Qualcomm Incorporated, AT&T, Vodafone, MediaTek Inc., NTT Docomo, LG Electronics, Nokia, Deutsche Telekom, Telia Company, Turkcell, ZTE Corporation, Samsung | Rel-19 | 38.300 | TEI19 | 1054 |  | B |
| R2-2507921 | Introduction of TN to IoT NTN redirection [IoT-TN-NTN-redir] | Samsung, Google, Ericsson, vivo | Rel-19 | 36.300 | TEI19 | 1438 |  | B |
| R2-2507923 | Introduction of Redirection from NR TN to NR NTN [NR_TN_NTN_redir] | Samsung | Rel-19 | 38.300 | TEI19 | 1055 |  | B |
| R2-2507924 | Correction on the cyclic shift capability for PMCH | Huawei, HiSilicon, Samsung | Rel-19 | 36.306 | LTE_terr_bcast_Ph2-Core | 1934 |  | F |
| R2-2507927 | Corrections to LTE-based 5G Broadcast Phase 2 after ASN.1 review | Qualcomm Incorporated, Samsung | Rel-19 | 36.331 | LTE_terr_bcast_Ph2-Core | 5168 | 2 | F |
| R2-2507936 | Introduction of MINT in EPS | LG Electronics Inc., Nokia, Ericsson, Google, Qualcomm Inc | Rel-19 | 36.331 | MINT_Ph2 | 5171 | 3 | B |
| R2-2507937 | Introduction of MINT in EPS | LG Electronics Inc., Nokia, Ericsson, Lenovo, Google, Qualcomm Inc. | Rel-19 | 36.300 | MINT_Ph2 | 1437 | 2 | B |

80 Agreed in principle CRs.

# Annex F:	List of email discussions during the meeting

Discussions during the RAN2#131bis meeting:

List of AT meeting email discussions

[AT131bis][005][TEI19] LTM_enh_SR (Ericsson)

[AT131bis][007][AI PHY] LS to SA2 on dataset parameter options (Samsung)

[AT131bis][008][AI PHY] Reply LS on positioning (Samsung)

[AT131bis][009][AI PHY] Reply LS to SA2 on data collection (Nokia)

[AT131bis][012][AI PHY] Offline on RILs (Ericsson)

[AT131bis][013][AIoT] LS to CT1 (Lenovo)

[AT131bis][014][AIoT] LS to SA2 (Huawei)

[AT131bis][015][AIoT] LS to SA3 (CMCC)

[AT131bis][016][AIoT] LS to SA3 on integrity failure (Xiaomi)

[AT131bis][101][MOB] LS to RAN3 of L2 reset (Ericsson)

[AT131bis][104][MOB] Corrections on Rel-18 UE capabilities for LTM (Huawei)

[AT131bis][105][MOB] Capabilities (CATT)

[AT131bis][106][MOB] TP for CLTM execution condition configuration (Huawei)

[AT131bis][107] TP for [Z157] Option 2 (ZTE)

[AT131bis][108][MOB] MAC corrections (vivo)

[AT131bis][201][LPWUS] Impact with co-existence with LPWUS and paging adaptation (CATT)

[AT131bis][202][NR_Others] On Rx BSF optimization (CATT)

[AT131bis][203][MIMOevo] Remaining MAC issues (Samsung)

[AT131bis][204][NR_Others] On support for MINT in EPS (LGE)

[AT131bis][205][NR_Others] Draft LS on temporary suspension of trace production (Ericsson)

[AT131bis][206][SBFD] LS to R1 and R4 on SBFD and CA (InterDigital)

[AT131bis][301][NES] RRC open issues (Ericsson)

[AT131bis][302][NES] SSB-less vs OD-SSB case 1 (Apple)

[AT131bis][303][NR NTN] RRC open issues (Ericsson)

[AT131bis][304][IoT NTN Ph4] Reply LS to SA2 (vivo)

[AT131bis][305][IoT NTN Ph4] Reply LS to SA4 (QC)

[AT131bis][306][NR NTN] LS on OCC for RACH-less HO (Thales)

[AT131bis][307][NES] MAC open issues (Interdigital)

[AT131bis][309][IoT NTN TDD] radioFrameoffset per cell (Samsung)

[AT131bis][310][R18 NR NTN] Event D2 (CSCN)

[AT131bis][311][IoT NTN TDD] RRC open issues (Huawei)

[AT131bis][401][Relay] Rel-19 relay RRC non-RIL issues (Huawei)

[AT131bis][500][XR] Organizational – Session on XR and LTE-based 5G Broadcast (Session chair)

[AT131bis][501][XR] Discuss remaining RLC issues (vivo)

[AT131bis][502][XR] Discuss remaining MAC issues (Qualcomm)

[AT131bis][503][LTE Broadcast] CRs for 36.331 and 36.306 (Qualcomm, Huawei)

[AT131bis][504][TEI19] 5G CAS muting stage-2 CR (ZTE)

[AT131bis][601][Maint] Correction on RACH-less (Ericsson)

[AT131bis][602][Maint] Correction SRB4 and SRB5 (Samsung)

[AT131bis][603][Maint] UL TX switching (Ericsson)

[AT131bis][604][Maint] SCS per frequency restriction (Apple)

[AT131bis][605][SONMDT] Corrections on SONMDT UE Capabilities (CATT)

[AT131bis][606][SONMDT] Stage-2 corrections for SONMDT (Ericsson)

[AT131bis][xxx][A-IoT] A-IoT MAC issues (Huawei)

44 at-meeting offline discussions.

# Annex G:	Post-meeting email discussions after RAN2#131bis

# Guidelines for email discussions:

General guidelines for SHORT email discussions, to be concluded approved endorsed at current meeting.

- Aim to have the final version of the agreed documents provided by the rapporteur at or shortly after the deadline.

- Please provide comments on the first version of the document in good time before the deadline. This allows the rapporteur to make an update addressing all companies' comments and there still be time for a quick round of comments on the update.

- If you have provided comments in the discussion then please indicate to the rapporteur if you are ok with the update provided (preferably via reflector). This avoids the rapporteur having to wait before they can conclude that their update is acceptable to you.

- Rapporteurs, if not already available, please request your tdoc number from the Secretary when you initiate your email discussion and then provide the final version as soon as you are confident that it is agreeable. You do not need to wait for a reminder from chair, session chair or Secretary before sending the final version.

- To avoid any confusion, Secretary, chair, or session chair will send an email to confirm the final status of the document.

For emails discussion to the next meeting (long):

- Rapporteurs, feel free to set an intermediate deadline for companies to provide initial comments, so that the conclusions and proposals can be prepared and distributed before the final deadline.

- Participants, please respect any intermediate deadline indicated by the rapporteur, and preferably provide your feedback as soon as possible.

# Deadlines

Oct. 24th 	1000 UTC	Deadline short email discussions

Oct. 31st 	1000 UTC	Deadline long email discussions

Nov. 7th	1000 UTC	Submission Deadline RAN2#132

Weekends are inactive periods.

It is recommended to not send emails or update files on the server during inactive periods while It is not strictly prohibited. Rapporteurs may kick-off discussions during inactive period. However, no intermediate deadlines and no interactive discussion, no decision making may occur during the inactive period.   It shall be possible for a delegate to stay away from reflector and 3GPP server during the inactive period, and still fully participate. Rapporteur announcements during the inactive period, if any, or other updates, can be taken into account after the inactive period.

# Short email discussions, Deadline Oct. 24th, 10:00 UTC

Please request R2-131bis TDoc numbers for the following email discussions from MCC if not already allocated. Approval / endorsement will be declared at or shortly after the deadline.

[POST131bis][002][Organizational] Post email discussions

Intended outcome: share email discussion list

[POST131bis][011][AI PHY] RRC CR (Ericsson)

Intended outcome:  Continue reviewing and updating CRs and get input on remaining ASN.1 open issues using the general ASN.1 process.

Deadline:  short

=> Endorsed in R2-2507934 (38.331)

[POST131bis][018][6G] LS to SA3 on security (Vodafone)

Intended outcome:  Approve LS to SA3

Deadline:  1 week

=> Approved in R2-2507945

[Post131bis][109][MOB] MAC corrections (vivo)

Scope: Capture agreements from the meeting and update issues list.

Intended outcome: Agreeable in principle CR in R2-2507728.

Deadline: Short.

=> Endorsed in R2-2507728 (38.321)

[Post131bis][110][MOB] RRC corrections (Ericsson)

Scope: Capture agreements from the meeting and update RILs list.

Intended outcome: Agreeable in principle CR in R2-2507729 and update of RILs list in R2-2507730 (RIL Comments file) and R2-2507881 (Excel sheet).

Deadline: Short.

=> Endorsed in:

R2-2507729 (38.331)

R2-2507730

=> Noted in R2-2507881

[Post131bis][208][LPWUS] CR for TS 38.331 (vivo)

Intended outcome: Update the CR for endorsement

Deadline:  Short

=> Endorsed in R2-2507939 (38.331)

[Post131bis][213][SBFD] CR for TS 38.331 (Huawei)

Intended outcome: Update the CR for endorsement

Deadline:  Short

=> Endorsed in R2-2507944 (38.331)

[Post131bis][216][MIMOevo] CR for TS 38.331 (Ericsson)

Intended outcome: Update the CR for endorsement

Deadline:  Short

=> Endorsed in R2-2507938 (38.331)

[Post131bis][217][NR_Others] LS to RAN4 on Rx BSF optimization (CATT)

Intended outcome: Approve the LS as per the agreements

Deadline:  Short

=> Approved in R2-2507935

[Post131bis][219][NR_Others] CR for support for MINT in EPS (LGE)

Intended outcome: Update the CRs to 36.331 and 36.300 for agreement in-principle

Deadline:  Short

=> Agreed in principle in:

R2-2507936 (36.331)

R2-2507937 (36.300)

[Post131bis][302][NES] RRC CR (Ericsson)

Scope: Update the RRC CR with meeting agreements

Intended outcome: endorsed CR in R2-2507783

Deadline:  short

=> Endorsed in R2-2507783 (38.331)

[Post131bis][303][NR NTN] RRC CR (Ericsson)

Scope: Update the RRC CR with meeting agreements

Intended outcome: endorsed CR in R2-2507785

Deadline:  short

=> Endorsed in R2-2507946 (38.331)

[Post131bis][304][LTE TN NR NTN] RRC CR (CATT)

Scope: Update the RRC CR with meeting agreements

Intended outcome: endorsed CR in R2-2507786

Deadline:  short

=> Endorsed in R2-2507786 (36.331)

[Post131bis][305][IoT NTN] RRC CR (Huawei)

Scope: Update the RRC CR with meeting agreements

Intended outcome: endorsed CR in R2-2507787

Deadline:  short

=> Endorsed in R2-2507787 (36.331)

[Post131bis][306][IoT NTN TDD] RRC CR (Huawei)

Scope: Update the RRC CR with meeting agreements

Intended outcome: endorsed CR in R2-2507788

Deadline:  short

=> Endorsed in R2-2507788 (36.331)

[Post131bis][307][IoT NTN] LS to SA2 (Huawei)

Scope: discuss the LS to SA2 on paging enhancements

Intended outcome: Approved LS in R2-2507930

Deadline:  short

=> Approved in R2-2507930

[Post131bis][403][Relay] Rel-19 relay RRC CR (Huawei)

Scope: Update the CR in R2-2507488 in line with decisions of RAN2#131bis.

Intended outcome: Endorsed CR (to be taken into ASN.1 review)

Deadline: Short

=> Endorsed in R2-2507943 (38.331)

[POST131bis][508][XR] Update 38.331 CR (Huawei)

Scope: Update and review 38.331 CR

Intended outcome: Agreeable CR

Deadline:  Short

=> Endorsed in R2-2507813 (38.331)

18 email discussions.

# Long email discussions, for R2-132, Oct. 31st 10:00 UTC (unless otherwise stated)]

Please request R2-132 TDoc numbers for the following email discussions by 3GU according to normal tdoc submission procedure.

[POST131bis][003][RRC] RAN1 parameters (Ericsson)

Intended outcome: Report to RAN1 the final parameter names

Deadline:  Long

[POST131bis][004][UE caps] UE capability CR (Xiaomi)

Intended outcome: Update UE capability CRs post RAN1/RAN4 LS and combine with RAN2 CRs.

Deadline:  long

[POST131bis][006][TEI19] Speed dependent scaling (KDDI/Samsung)

Intended outcome: Agreeable CRs, 38.331, 38.306

Deadline:  Long

[POST131bis][010][AI PHY] stage 2 CR (Vivo)

Intended outcome:  CR to be submitted for approval next meeting

Deadline:  Long

[POST131bis][017][AIoT] MAC CR  (Huawei)

Intended outcome: update and review revised MAC CR with agreements from this meeting and highlight any new open issues.

Deadline:  Long

- [POST131bis][020][RRC] Readability of RRC procedural text (Huawei)

Intended outcome: Identify sections that can benefit from update procedure and doesn’t introduce inconsistencies in previous releases. No draft CR or TPs for RAN2#132

Deadline: long

[Post131bis][207][LPWUS] CR for TS 38.304 (CATT)

Intended outcome: Update and review the CR and identify any additional other open issues.

Deadline:  Long

[Post131bis][209][LPWUS] CR for TS 38.321 (Apple)

Intended outcome: Update the CR and identify any additional other open issues.

Deadline:  Long

[Post131bis][210][LPWUS] CR for TS 38.300 (Ericsson)

Intended outcome: Update the CR and identify any additional other open issues.

Deadline:  Long

[Post131bis][211][LPWUS] CR for TS 38.306 (Huawei)

Intended outcome: Update the CR and identify any additional other open issues.

Deadline:  Long

[Post131bis][212][SBFD] CR for TS 38.321 (Samsung)

Intended outcome: Update the CR and identify any additional other open issues

Deadline:  Long

[Post131bis][214][SBFD] CR for TS 38.300 (CATT)

Intended outcome: Update the CR and identify any additional other open issues

Deadline:  Long

[Post131bis][215][MIMOevo] CR for TS 38.321 (Samsung)

Intended outcome: Update the CR and identify any additional other open issues

Deadline:  Long

[Post131bis][218][NR_Others] CR for TS 38.331 on Rx BSF optimization (CATT)

Intended outcome: Discuss and prepare the CR, for submission to the next meeting

Deadline:  Long

[Post131bis][301][NES] OD-SSB  CSI-RS measurements (Xiaomi)

Scope: Discuss procedural text for OD-SSB and CSI-RS measurements and for SMTC setup for SSB adaptation/OD-SSB

Intended outcome: email discussion summary

Deadline:  long

[Post131bis][402][Relay] Rel-19 relay SRAP CR (OPPO)

Scope: Update the CR in R2-2507150 in line with decisions of RAN2#131bis and identification of any other open issues

Intended outcome: Endorsable CR to RAN2#132

Deadline: Long

[POST131bis][505][XR] Update 38.321 CR (Qualcomm)

Scope: Update and review 38.321 CR and identification of any other open issues

Intended outcome: Agreeable CR

Deadline:  Long

[POST131bis][506][XR] Update 38.322 CR (vivo)

Scope: Update and review 38.322 CR and identification of any other open issues

Intended outcome: Agreeable CR

Deadline:  Long

[POST131bis][507][XR] Update 38.323 CR (LGE)

Scope: Update and review 38.323 CR and identification of any other open issues

Intended outcome: Agreeable CR

Deadline:  Long

19 long email discussions.

# Annex H:	History

| Document history |  |  |
| --- | --- | --- |
| Date | TSG RAN WG2 Tdoc | Subject |
| 04.11.2025 |  | First draft version |
| 07.11.2025 |  | Second draft version |
| 14.11.2025 | R2-2508002 | Version submitted for approval |
| Author: Dr. Juha Korhonen ETSI Mobile Competence Centre (MCC) Tel. +33 (0)4 92 94 42 00 email: Juha.Korhonen@etsi.org |  |  |