3GPP TSG-RAN WG2 Meeting #132	R2-2508096

Dallas, USA,  Nov. 17th - 21st, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussions on RRM measurement event prediction

Agenda Item:	9.3.3

Document for:	Discussion and Decisions

- Introduction

During the RAN#109 meeting, the WID on “Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR” was revised[1] and the objective of the work on RRM measurement event prediction is as below,

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]:

- Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported.

- L3 Cell level predication is supported

Note 1: For UE sided model, spatial domain prediction is not supported.

Note 2: For UE sided model, L3 beam level prediction is not supported.

- For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]:

- UE capability request and response procedure

- Applicability report for both full and partial configuration approach

- Inference configuration and inference report

- Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality

- UE sided data collection

- For measurement event prediction:

  - Measurement event A1~A6 are in the scope in this release

Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation.

Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible.

This paper discusses LCM functionality management for RRM measurement event prediction, including UE capability, applicability, inference configuration and reporting.

- UE capability

According to the conclusion in RANP #109 meeting[2], for direct event prediction, RAN2 will discuss KPI for performance monitoring until Q1 2026 and RAN4 will discuss requirements until Q2 2026. The check point in RAN#112 for finally decide on whether direct event prediction will be supported depending on RAN2 and RAN4 conclusions. From a commercial deployment perspective, it is therefore preferable that the Rel-20 work on RRM measurement event prediction focuses on the indirect case, since any functionality achievable by direct prediction can also be supported through indirect prediction, while indirect prediction provides a clearer and more practical basis for performance monitoring. Unlike direct prediction, the indirect case can reuse existing measurement reporting and evaluation mechanisms, making its feasibility and verification path more straightforward. As a result, the introduction of indirect prediction is less dependent on cross-WG alignment.

Proposal 1: The work on RRM measurement event prediction should focus on the indirect case in Rel-20 for specification and commercial simplicity.

In the companion contribution[2], the UE capability procedure for RRM measurement prediction was discussed, which can be mostly reused for RRM measurement event prediction. Moreover, the information on choice of indirect or direction prediction, and the corresponding measurement event A1~A6 should be supported as well to facilitate the NW decision on inference configuration or inference related parameters.

Proposal 2: RAN2 to support:

  - The functionality granularity to be at the sub-use cases level, i.e. each sub-use case in each scenario.

  - NW indicates whether specific information(e.g. Accuracy, Preferred measurement gap, Frequency range, supported associated ID) could be indicated in UECapabilityEnquiry message per RAT/frequency/cell(group).

  - UE may report the supported associated IDs together with corresponding supported functionalities in the UECapabilityInformation according to NW indication.

  - UE may report the type of event prediction, such as indirect/direct, A1~A6, together with corresponding supported functionalities in the UECapabilityInformation according to NW indication.

- Applicability

In the companion contribution [2], the applicability procedure for RRM measurement prediction was discussed, which can also be mostly reused at least for indirect RRM measurement event prediction.

Proposal 3: RAN2 to support Option A and Option B to be configured in the same RRCReconfiguration message with following information:

  - Option A: One of multiple sets of full inference configuration, which is for actual measurement and prediction in the same or independent measurement objects

- Temporal domain case A: The prediction window

- Temporal domain case B: The skipping pattern or up-sampling rate or virtual RS configuration

- Frequency domain:

      - Co-located information

      - Gap-less configuration for prediction

  - Option B: One of multiple sets of inference related parameters

- Temporal case A: PW length

- Temporal case B: Skipping pattern

- Frequency domain: Measured and predicted frequency, co-locate information

Proposal 4: RAN2 to support both single-cell specific associated ID and multi-cell specific associated ID with 24bits, which is unique within a PLMN, and can be used for both Option A and Option B.

Proposal 5: RAN2 to support to report following information in the initial or updated applicability reporting for both Option A and Option B:

  - Inapplicability cause

  - Candidate associated IDs

Proposal 6: RAN2 to support the applicability reporting during handover procedure and RRC reestablishment procedure.

- UE needs to know whether the related inference configurations or inference related parameters are still valid upon initiation of RRC Reestablishment.

- Inference configuration

At the RAN2 #131 meeting, the following were agreed for inference configuration[3].

Agreements at #131 meeting

- For RRM measurement event prediction, the following are included in the inference configuration:

  - The length of PW for the associated AIML model

  - Event-related information (event type and event-related parameters)

In the companion contribution[2], the inference configuration for RRM measurement prediction was discussed, which can also be mostly reused at least for indirect RRM measurement event prediction.

Inference activation/deactivation:

Proposal 7: RAN2 to support:

  - The event-triggered activation/deactivation, i.e. UE activates/de-activates the applicable functionalities when certain event condition is satisfied/unsatisfied.

  - The on-demand activation/deactivation by NW through MAC CE or DCI.

  - The autonomous deactivation of the configured measurement gap for a particular frequency band for which the infer-frequency prediction is activated.

How to configure:

Proposal 8: RAN2 to support:

  - Temporal domain: The inference configuration for actual measurement and prediction in the same or independent measurement objects.

  - Frequency domain: The inference configuration for prediction without gap configuration; otherwise UE may ignore the gap configuration.

- Inference reporting

At the RAN2 #129bis and #131 meeting, the following were agreed for inference reporting [3][4],

Agreements at #129bis meeting

- For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

- Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP

Agreements at #131 meeting

- For RRM measurement event prediction, the event prediction report can include:

  - RRM measurements (For normative phase to define what types of measurements)

  - Time-related information about the predicted event (e.g., the time/window of occurrence)

- For measurement event determined, when the UE is configured with temporal domain case B, and frequency domain prediction, UE reports measurement event by following an approach similar legacy procedure.   This can be achieved without spec impact for measurement reporting.  FFS for normative phase if additional information needs to be added.

When to report:

The activation or deactivation of the report configuration together with the bounded inference configuration is straightforward. For event prediction, the inference configuration and the reporting configuration could also be independently configured. In this case, as long as the linkage these two configurations are also configured, activating one can automatically activate the other, which avoids the need for separate signaling steps and reduces overhead compared with configuring or enabling them independently.

Event triggering based on a predicted measurement value in the future may be achieved either by enhancing the existing event configuration or by defining a new event. Both approaches are technically feasible, and the difference lies only in the eventual specification design rather than in the functional behaviour. The underlying requirement is simply that the triggering condition may reference a future predicted value rather than an immediately measured one.

Different from legacy handover, NW may rely on the predicted event to start handover preparation but still executes the handover command only when the actual measurement value confirms the predicted measurement results. Supporting separate triggering conditions for the predicted value and the actual value allows the NW to make use of the predictive information for early handover preparation, while still relying on the actual measurement result to ensure reliability before executing the final handover command. This enables the prediction to provide its timing advantage without forcing the NW to fully trust the predicted value, thereby combining the benefit of early reporting with the assurance of real measurement confirmation.

By binding the predicted and actual measurement reports to the same time instance, the NW can rely on the predicted value for early processing and later overwrite it with the actual measurement result. At the same time, the NW can compare the two results to evaluate prediction accuracy, without introducing extra signalling or additional report types.

Proposal 9: RAN2 to support:

  - The activation/deactivation of report configuration together with bounded inference configuration.

  - The event triggering based on the predicted measurement value in the future based on enhancement on current event configuration or on new event.

  - The event triggering independently based on leaving or entering condition of actual predicted measurement value or predicted measurement value.

  - The bounding between predicted measurement reporting and the actual measurement reporting for the same time instances

What to report:

In prediction measurement reporting, the NW may need not only the measurement value itself but also the contextual information that allows it to interpret, validate, or replace the reported result. Depending on deployment needs, the NW may require identification of the reporting object, access to both actual and predicted values, visibility of when the predicted event is expected to occur, an indication of its confidence level, or the ability to update or cancel a previously reported prediction. Therefore, it is necessary that the reporting framework can carry such information when configured, without enforcing any fixed combination of fields.

Proposal 10: RAN2 to support to report at least one of the following types of information in one measurement reporting:

  - MeasID

  - Cell ID(s)

  - Most recent multiple actual measurement results

  - Predicted measurement results

  - Expected occurrence time of the predicted event

  - Confidence of the predicted event(e.g. accuracy, probability)

  - Indication of updating and/or canceling of previous predicted measurement reporting.

After reporting:

Using the predicted measurement result as the basis for conditional handover allows the NW to start the handover preparation with visibility of the future measurement condition, rather than relying solely on the most recent actual measurement. Compared with conditional handover based on the latest actual measurement, using the predicted result allows the NW to prepare the handover closer to the time when the condition is expected to be satisfied, reducing premature or delayed preparation and improving the accuracy of handover timing, while the final execution could still be safeguarded by the UE’s confirmation with the actual measurement condition.

Proposal 11: RAN2 to support the conditional handover if the condition provided to the UE after predicted measurement reporting is verified.

- Conclusion

In this contribution, we have the following proposals on functionality management of UE-sided model,

- UE capability

Proposal 1: The work on RRM measurement event prediction should focus on the indirect case in Rel-20 for specification and commercial simplicity.

Proposal 2: RAN2 to support:

  - The functionality granularity to be at the sub-use cases level, i.e. each sub-use case in each scenario.

  - NW indicates whether specific information(e.g. Accuracy, Preferred measurement gap, Frequency range, supported associated ID) could be indicated in UECapabilityEnquiry message per RAT/frequency/cell(group).

  - UE may report the supported associated IDs together with corresponding supported functionalities in the UECapabilityInformation according to NW indication.

  - UE may report the type of event prediction, such as indirect/direct, A1~A6, together with corresponding supported functionalities in the UECapabilityInformation according to NW indication.

- Applicability

Proposal 3: RAN2 to support Option A and Option B to be configured in the same RRCReconfiguration message with following information:

  - Option A: One of multiple sets of full inference configuration, which is for actual measurement and prediction in the same or independent measurement objects

- Temporal domain case A: The prediction window

- Temporal domain case B: The skipping pattern or up-sampling rate or virtual RS configuration

- Frequency domain:

      - Co-located information

      - Gap-less configuration for prediction

  - Option B: One of multiple sets of inference related parameters

- Temporal case A: PW length

- Temporal case B: Skipping pattern

- Frequency domain: Measured and predicted frequency, co-locate information

Proposal 4: RAN2 to support both single-cell specific associated ID and multi-cell specific associated ID with 24bits, which is unique within a PLMN, and can be used for both Option A and Option B.

Proposal 5: RAN2 to support to report following information in the initial or updated applicability reporting for both Option A and Option B:

  - Inapplicability cause

  - Candidate associated IDs

Proposal 6: RAN2 to support the applicability reporting during handover procedure and RRC reestablishment procedure.

- UE needs to know whether the related inference configurations or inference related parameters are still valid upon initiation of RRC Reestablishment.

- Inference configuration

Proposal 7: RAN2 to support:

  - The event-triggered activation/deactivation, i.e. UE activates/de-activates the applicable functionalities when certain event condition is satisfied/unsatisfied.

  - The on-demand activation/deactivation by NW through MAC CE or DCI.

  - The autonomous deactivation of the configured measurement gap for a particular frequency band for which the infer-frequency prediction is activated.

Proposal 8: RAN2 to support:

  - Temporal domain: The inference configuration for actual measurement and prediction in the same or independent measurement objects.

  - Frequency domain: The inference configuration for prediction without gap configuration; otherwise UE may ignore the gap configuration.

- Inference reporting

Proposal 9: RAN2 to support:

  - The activation/deactivation of report configuration together with bounded inference configuration.

  - The event triggering based on the predicted measurement value in the future based on enhancement on current event configuration or on new event.

  - The event triggering independently based on leaving or entering condition of actual predicted measurement value or predicted measurement value.

  - The bounding between predicted measurement reporting and the actual measurement reporting for the same time instances

Proposal 10: RAN2 to support to report at least one of the following types of information in one measurement reporting:

  - MeasID

  - Cell ID(s)

  - Most recent multiple actual measurement results

  - Predicted measurement results

  - Expected occurrence time of the predicted event

  - Confidence of the predicted event(e.g. accuracy, probability)

  - Indication of updating and/or canceling of previous predicted measurement reporting.

Proposal 11: RAN2 to support the conditional handover if the condition provided to the UE after predicted measurement reporting is verified.

References

- OPPO, Interdigital, RP-252889, “Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR,” Sep. 2025.

- RAN2 Chair (InterDigital), RP-252896, “Way forward on AI/ML Mobility Scope”, Sep., 2025.

- RAN2 Chair (InterDigital), “RAN2 #131 Chair Notes”, Aug., 2025.

- RAN2 Chair (InterDigital), “RAN2 #129b Chair Notes”, Apr., 2025.