3GPP TSG RAN WG2 Meeting #132	 R2-2508192

Dallas, USA,  Nov. 17th - 21st

Agenda Item:			10.4

Source:			Fraunhofer HHI, Fraunhofer IIS

Title:			On 6G Mobility

Document for:		Discussion and Decision

# Introduction

At the RAN2#131-bis meeting [1], the following agreements were reached:

1 Study mobility with the following requirements in 6G mobility design: - Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions. - Robustness of mobility procedures - Energy efficiency for both UE and NW 2 Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

In addition, a unified framework for mobility was discussed at the last meeting, though no agreement was reached. This contribution considers potential enhancements for 6G mobility, focusing on the design of a unified signalling framework for mobility procedures, interruption time minimization, the maintenance of HARQ processes during handover, and the use of uplink reference signal measurements (e.g., SRS) as a basis for mobility decisions.

# Enhancement for 6G-Mobility

## Unified Mobility Signalling

In 5G, several mobility enhancement have been introduced e.g., CHO, DAPS, LTM, CLTM etc. Their main objectives include reducing service interruption time, improving robustness, and enabling faster cell switching.

Although these features target similar goals, they are implemented as independent mechanisms, with distinct structures, signaling flows, and procedures. This fragmentation leads to increased overhead and power consumption for both the UE and the network. As sustainability and energy efficiency become key design principles for 6G systems on both the UE and gNB sides a unified handover (UHO) framework is necessary.

Proposal 1: RAN2 is invited to first discuss the meaning and scope of the unified handover.

The handover process consists of several phases, including configuration, measurement, reporting, preparation, command, and execution. RAN2 should identify which of these elements can be harmonized under a unified structure.

For instance, independent frameworks currently exist for L3 and L1 measurements. They differ in RRC signaling structures, UE behaviors, and reporting procedures. For example:

- Reference signals are provided through different RRC configurations (e.g., MeasObject, CSI-MeasConfig).

- Measurement evaluation and triggering occur at different protocol layers.

- L3 RRM reports are transmitted via RRC messages, whereas L1 reports use MAC CEs or UCI.

- Reporting configurations are generated by different network entities.

RAN2 should therefore study and determine where and how to initiate work toward a single, unified framework for mobility signaling.In later stages, the study should may include aspects such as early TCI activation, early TA acquisition, fast recovery etc.

Proposal 2: RAN2 to study a unified handover framework covering:

- Configuration alignment (e.g., RRC and reference signal definitions)

- Measurement harmonization (e.g., L1 and L3 metrics, thresholds, and events)

- Reporting unification (e.g., RRC, MAC CE, UCI procedures)

- Layer coordination (e.g., evaluation and triggering alignment)

- Coordination between network entities

In our view, UHO studies should begin with configuration aspects in the mobility procedures, as these rely heavily on RRC signalling. In 5G, the UE maintains a full configuration, and gNBs send RRCReconfiguration messages to indicate delta changes. However, in practice, large sets of configuration IEs are often retransmitted even if unchanged. Furthermore, the fullConfig IE can force the UE to discard its configuration entirely, introducing unnecessary latency, signalling overhead, and power consumption.

These inefficiencies are especially visible in frequent or diverse mobility scenarios. For instance, in L3 handover, long configuration lists are often repeated unnecessarily. In LTM, forcing a full reconfiguration slows down recovery from radio link failure. In conditional mobility e.g., conditional L3 handover or conditional LTM, unchanged IEs are duplicated across multiple target configurations, further increasing signalling burden.

Observation 1: Current RRC signalling repeats unchanged configuration elements and may trigger full reconfigurations, increasing latency, signalling load, and power usage.

To address this, a unified signalling design for target cell preparation and configuration is needed. The benefits are clear: smaller RRC message sizes, faster mobility execution, and reduced UE processing and power consumption.

Proposal 3: RAN2 is to ensure signalling efficiency for target cell preparation and configuration in different handover scenarios.

Proposal 4: RAN2 is to design a unified signalling framework that supports flexible and incremental configuration updates across mobility events.

## Interruption Time Minimization

In the RAN2#131-bis meeting, it was agreed that one of the key objectives for 6G mobility design is to minimize interruption time, and companies were encouraged to compare the expected gains against implementation complexity when evaluating solutions.

Across 5G releases, several mechanisms were introduced to reduce interruption time: DAPS and CHO in Release 16, LTM in Release 18, and CLTM and RACH-less HO/CHO in Release 19. However, in practice, not all of these mechanisms have been deployed. In many cases, implementation complexity and practical deployment challenges have prevented their commercial adoption. Each mobility enhancement has advantages and disadvantages in terms of performance, complexity, and applicability.

For example, although DAPS can enable zero-millisecond interruption for downlink transmission, the dual-stack architecture introduces significant complexity on the UE side, making large-scale commercial deployment difficult in 5G networks. In addition, DAPS handover has limited compatibility with other key features such as carrier aggregation, dual connectivity, and multi-TRP operation. As a result, DAPS may have low priority in future 6G mobility studies.

Observation 2: Due to high implementation complexity, DAPS did not become commercially beneficial in practice.

CHO and CLTM naturally provide low interruption time and high robustness. However, they require in advance configuration of multiple target candidates, which increases signaling overhead and creates management burden for the network. Furthermore, legacy handover is simple and lightweight from a signaling perspective, but it does not provide features that guarantee low interruption.

LTM provides low interruption time and high throughput. However, in network-controlled LTM, carrying the handover execution command via MAC CE makes LTM vulnerable from a security perspective. To fully benefit from LTM in 6G, this security weakness should be addressed.

Proposal 5: RAN2 to study enhancements to address security vulnerabilities associated with LTM, so that low-interruption mobility can be achieved securely in 6G.

## HARQ Maintenance

Hybrid Automatic Repeat Request (HARQ) has been a key mechanism in 5G to ensure reliable data transmission. By running multiple processes in parallel, each with a dedicated process ID, the system can transmit new packets while waiting for ACK/NACK feedback. This parallelism reduces delay and improves robustness, especially in high throughput scenarios.

In the current design, HARQ entities are reset at handover. The UE flushes all HARQ buffers, ongoing processes are terminated, and the target cell starts new HARQ entities. Packets awaiting retransmission or acknowledgement are lost, resulting in higher latency and lower spectral efficiency. The impact is even more severe at higher frequencies (e.g., mmWave, sub-THz) where link quality fluctuates quickly and handovers occur frequently. In LTM, where latency degradation is the primary concern, restarting HARQ retransmissions from scratch at every event is inefficient and not suitable for 6G.

For example, in a dense urban scenario with mmWave coverage, a UE may perform multiple mobility events within seconds. Resetting HARQ at each handover forces retransmission of partially delivered packets, increasing both latency and signalling overhead. Similarly, in high-speed vehicular mobility, frequent handovers would repeatedly discard useful HARQ states, degrading efficiency.

Observation 3: Current HARQ design forces the UE to start new processes at each handover, causing redundant retransmissions and higher latency.

We propose that RAN2 study HARQ maintenance in 6G mobility procedures. In this manner, the target gNB would be able to resume ongoing retransmissions seamlessly, without forcing the UE to restart from the first transmission attempt. The potential benefits are substantial. Latency can be reduced by avoiding redundant retransmissions, radio resources can be saved, and user experience can be improved in scenarios with frequent handovers.

Proposal 6: RAN2 study HARQ maintenance across handovers to allow seamless continuation of retransmissions and acknowledgements. This would reduce latency, save radio resources, and improve user experience in scenarios with frequent mobility.

## Use of SRS-Measurements for UE Mobility:

UE Mobility in 3GPP is primarily based on utilizing measurements performed on downlink reference signals. Use of uplink reference signals, such as SRS, where configured, could be used to determine uplink measurements and would benefit from reduced need to perform measurement and reporting.

For UE positioning, the neighbouring cells can be configured to perform measurement on sounding reference signals that are configured by the serving cell for a given UE. This is already done for uplink positioning (UL-TDOA). For mobility, the neighbouring cells can perform measurement on SRS transmitted by the UE and report the measurement to the serving cell, so that the serving cell can initiate handover or send handover configuration to the UE when needed.

Observation 4:  In addition to UE reported measurements and events, the network shall also utilise existing reference signals like the SRS to perform measurement at different RAN nodes, to determine measurements and events.

It is observed that for co-located case, AI/ML models could be utilised to perform inter-frequency domain prediction. By analogy, if the SRS from UE is received at a neighbouring cell, and the neighbouring cell performs measurement, then it could also apply analogously an AI/ML model to predict measurement in an inter-frequency cell, where the SRS is received. Hence, utilising SRS measurement at a neighbouring cell could enable an inter-frequency prediction at a target cell that is not collocated with the special cell.

Observation 5: Inter-frequency measurement for co-located cells may be predicted using measurements of uplink reference signals like SRS.

Training such AI model on the network side needs training data and validation. To enable the network to train and perform monitoring, the UE shall indicate which of the measurements are conducted measurement and which of the measurement are predicted measurement.

Hence, we propose that RAN2 studies the use of uplink reference signal to determine the measurement and events.

Proposal 7: RAN2 studies the use of uplink reference signal (like SRS) to determine or predict uplink measurement for UE mobility in addition to current measurement and reporting framework based on downlink reference signals.

# Conclusions

In this contribution, we discussed various aspects related to enhancements for 6G mobility. Based on the discussions, we have made the following observations and proposals:

Observation 1: Current RRC signalling repeats unchanged configuration elements and may trigger full reconfigurations, increasing latency, signalling load, and power usage.

Observation 2: Due to high implementation complexity, DAPS did not become commercially beneficial in practice.

Observation 3: Current HARQ design forces the UE to start new processes at each handover, causing redundant retransmissions and higher latency.

Observation 4:  In addition to UE reported measurements and events, the network shall also utilise existing reference signals like the SRS to perform measurement at different RAN nodes, to determine measurements and events.

Observation 5: Inter-frequency measurement for co-located cells may be predicted using measurements of uplink reference signals like SRS.

Proposal 1: RAN2 is invited to first discuss the meaning and scope of the unified handover.

Proposal 2: RAN2 to study a unified handover framework covering:

- Configuration alignment (e.g., RRC and reference signal definitions)

- Measurement harmonization (e.g., L1 and L3 metrics, thresholds, and events)

- Reporting unification (e.g., RRC, MAC CE, UCI procedures)

- Layer coordination (e.g., evaluation and triggering alignment)

- Coordination between network entities

Proposal 3: RAN2 is to ensure signalling efficiency for target cell preparation and configuration in different handover scenarios.

Proposal 4: RAN2 is to design a unified signalling framework that supports flexible and incremental configuration updates across mobility events.

Proposal 5: RAN2 to study enhancements to address security vulnerabilities associated with LTM, so that low-interruption mobility can be achieved securely in 6G.

Proposal 6: RAN2 study HARQ maintenance across handovers to allow seamless continuation of retransmissions and acknowledgements. This would reduce latency, save radio resources, and improve user experience in scenarios with frequent mobility.

Proposal 7: RAN2 studies the use of uplink reference signal (like SRS) to determine or predict uplink measurement for UE mobility in addition to current measurement and reporting framework based on downlink reference signals.

# References

[1] Chair Notes RAN2#131-bis, Prague, Czech Republic, Oct. 13th-17th

[2]  TS 22.870, "Study on 6G Use Cases and Service Requirements," 3GPP, 2025-06.