3GPP TSG-RAN WG2 Meeting #132										R2-2508209

Dallas, USA, November 17th-21st, 2025

Agenda Item:	10.2.1

Source: 	Sharp

Title:  	6GR UE Capability Framework

Document for: 	Discussion

# Introduction

In RAN2_131bis meeting, RAN2 had following agreements on UE capabilities [1]:

Agreements on UE capabilities for next meeting

For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities.   Understand if there is impact to other WGs.

Companies can focus on the practical IODT problems in the field.   Possible ways to address them.

This document discusses 6GR UE capability framework.

# Discussion

## Capability signaling size reduction

In NR, the size of UE capability signaling has become excessive due to the high degree of design flexibility. NR assumed to support various spectrum combinations, aggregation schemes (e.g., CA, DC), and deployment scenarios. This complexity forces UEs to report a massive amount of information to the network. In Rel-15, there are 179 optional feature groups (not counting mandatory and RF/RRM-related features), while Rel-16 already defines 422 optional feature groups (excluding RF/RRM-related features). In addition, each feature group may contain multiple sub-components, further increasing message size and processing complexity. The current framework requires transmission of the full capability set whenever the network needs to retrieve capability information. There is no standardized mechanism for partial or differential reporting, even when only minor parts of the capability set have changed. This results in substantial signaling overhead during connection establishment, re-establishment, or mobility procedures, leading to longer setup time and higher processing burden for both UE and network.

To cope with the large size of signaling, RRC segmentation was introduced. However, segmentation adds latency and consumes valuable radio resources. Critically, this wide flexibility rarely translates into real-world use. Most UEs deploy only a few specific sets of features. The current mechanism may be considered inefficient as it forces the transfer of irrelevant capabilities for the common case.

The shift to the new generation (i.e., 6G) is a crucial opportunity to eliminate this inefficiency. To achieve the goals of simplification, efficiency, and reduced network overhead, the RAN2 6G study should aim to reduce the size of UE capability signaling.

Observation 1: NR capability signaling has become excessive due to the high degree of design flexibility.

Proposal 1: RAN2 study aims to reduce the size of UE capability signaling in 6GR.

To achieve this goal of signaling reduction, the 6GR study should investigate specific optimization mechanisms. One approach is reintroducing a streamlined UE Category system, similar to that used in LTE. This easily allows the network to quickly infer a standard set of capabilities with a minimal signaling footprint. Additionally, the study can explore introducing a capability group concept that avoids the repetitive transfer of individual feature information.

Proposal 2: RAN2 study to consider for 6GR:

- 1) introducing UE category as in LTE

- 2) introducing capability group to cover multiple features.

## Dynamic capability change

In RAN2#131bis meeting, RAN2 had some initial discussions on dynamic capability change in connected mode. The main issues were whether the use cases of dynamic capability change exist or not.

In NR, the dynamic change of UE capability in connected mode is not allowed. However, UAI supports UE’s preference reporting on reduced capability. NR UAI is not a capability report, so it does not mandate any network behaviours.

We may consider cases of sudden internal UE problems such as UE processing power change, UE battery status change, and overheating. If a UE processing power is shared with other user-specific functions, e.g. on-device AI, UE’s AS/NAS processing may be impacted. UE’s low battery and overheating are well-known scenarios as well. In these cases, it would be better to allow the temporary degradation of UE capability for flexible UE implementation at the early stage of 6G.

Observation 2: In NR, it is not possible for the UE to degrade the UE capability due to sudden internal UE problems, e.g. UE processing power change, UE battery status change, and overheating.

Proposal 3: RAN2 to study how to support dynamic capability change in connected mode at least in case of UE processing power change, UE battery status change, and overheating.

However, the dynamic capability update should not require huge standardization efforts or high signaling exchanges. Hence, RAN2 study should consider the efficiency of effectiveness of the dynamic capability update.

Proposal 4: RAN2 to study mechanisms for dynamic capability update in connected mode, focusing on low signaling overhead and minimal impact.

The current UE capability framework already provides mechanisms such as applicabilityConfigList, featureSetCombination, and UE capability ID referencing, which can be leveraged to manage variability without introducing entirely new signaling.

Before introducing new procedures, it is essential to evaluate whether dynamic capability behavior can be supported by extending or re-using existing RRC signaling framework.

Proposal 5: RAN2 to use the existing UE capability exchange framework as a baseline to discuss connected mode dynamic capability change.

Furthermore, also based on the temporary capability restriction via UAI, a UE could implement partial configuration failure handing while the UE is unable to apply (part of) the configuration resulting from RRCReconfiguration message. However, in 5GNR, ﻿it is up to UE implementation how to apply RRCReconfiguration message for partial configuration failure handling. To the NW, it may may cause unpredictable UE behaviour.

Therefore, by using the existing UE capability exchange framework as a baseline for dynamic capability change, it would be doubtful whether 6GR should support temporary capability restriction and partial configuration failure handling approach via UAI, which may fragile the 6G capability framework. RAN2 is suggested to study how to replace the NR temporary capability restriction approach via UAI transmission and partial configuration failure handing by the 6G capability framework.

Observation 3 In NR, temporary capability restriction (e.g., MUSIM case) does not mandate any network behaviours.

Observation 4 The partial configuration failure handling after temporary capability restriction may cause unpredictable UE behaviour to RAN.

Proposal 6: RAN2 to study how to replace the NR temporary capability restriction transmission and partial configuration failure handing by the 6G capability framework.

## Cross-WG Coordination

The lack of a general version-control or capability mechanism complicates long-term maintenance and interoperability between NR and future 6GR systems.

In addition, we think that the 6G capability framework study may be coordinated with RAN1, RAN3, RAN4, SA2, and SA5 respectively to maintain consistency in feature activation behavior, capability storage and retrieval, and management interfaces.

Proposal 7: RAN2 to initiate early coordination with RAN1, RAN3, RAN4, SA2, and SA5 to exchange views on Dynamic and Forward Compatible UE Capability Management.

# Conclusion

Based on the discussion given above, RAN2 is requested to discuss the observations and agree the following proposals:

Observation 1: NR capability signaling has become excessive due to the high degree of design flexibility.

Proposal 1: RAN2 study aims to reduce the size of UE capability signaling in 6GR.

Proposal 2: RAN2 study to consider for 6GR:

- 1) introducing UE category as in LTE

- 2) introducing capability group to cover multiple features.

Observation 2: In NR, it is not possible for the UE to degrade the UE capability due to sudden internal UE problems, e.g. UE processing power change, UE battery status change, and overheating.

Proposal 3: RAN2 to study how to support dynamic capability change in connected mode at least in case of UE processing power change, UE battery status change, and overheating.

Proposal 4: RAN2 to study mechanisms for dynamic capability update in connected mode with minimal impact or focus on low signaling overhead.

Proposal 5: RAN2 to use the existing UE capability exchange framework as a baseline to discuss connected mode dynamic capability change.

Observation 3 In NR, temporary capability restriction (e.g., MUSIM case) does not mandate any network behaviours.

Observation 4 The partial configuration failure handling after temporary capability restriction may cause unpredictable UE behaviour to RAN.

Proposal 6: RAN2 to study how to replace the NR temporary capability restriction transmission and partial configuration failure handing by the 6G capability framework.

Proposal 7: RAN2 to initiate early coordination with RAN1, RAN3, RAN4, SA2, and SA5 to exchange views on Dynamic and Forward Compatible UE Capability Management.

# References

[1] RAN2_131bis meeting notes.