3GPP TSG-RAN WG2 Meeting #132											R2-2508238

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	10.3.2.1

Source: 	Sharp

Title:  	RRC modelling and connection control in 6GR

Document for: 	Discussion

Introduction

In RAN2#131bis meeting, RAN2 has initially discussed RRC aspects on RRC state modelling for 6GR system, and the conclusion includes [1] :

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

In this contribution, we provide views for RRC state modelling and connection management for 6GR.

Discussion

RRC state

Figure 1. UE state machine and state transitions in NR[2]

In NR, three RRC states are supported, i.e., RRC_CONNECTED, RRC_IDLE and RRC_INACTIVE as depicted in Figure 1. From the UE perspective, there are not many differences between RRC_IDLE and RRC_INACTIVE. The main difference is the network behaviour that the core network maintains the UE contexts for fast reconnection. From RAN perspective, RRC Establishment, RRC Resume procedure and corresponding UE behaviours were specified in RAN specifications.

Instead, RRC_INACTIVE has some benefits, i.e., fast transition to the connected, and small data transmission can be supported. Thus, it would be nice to merge some characteristics of RRC_INACTIVE into either mode (RRC idle or RRC connected) to reduce the complexity while the benefits are kept.

Proposal 1: Characteristics of inactive mode/sub-state can be merged into either connected mode or idle mode.

RRC functionality for 6G

In 5G, small data transmission (SDT) was introduced to support fast data transmission without state transition. Considering that many Internet applications generate small-size sparse traffic in which frequent state transition may be required without SDT procedure, SDT is still useful or more useful in 6G.

Proposal 2: Small data transmission without mode transition should be supported in 6G.

A major complexity of RRC_INACTIVE was paging complexity. More specifically, RNA is considered as an additional paging mechanism different from traditional idle mode paging. Thus, separate location update mechanisms were supported. We see that these duplicate paging functionalities should be avoided in 6G. It can be merged into idle mode paging or simplified to a single-cell operation.

Proposal 3: RNA should be same as either TA or a single-cell coverage.

RRC connection management

For connection management, to implement RRC state transition between RRC_CONNECTED and RRC_IDLE (or RRC_INACTIVE), three-step RRC connection establishment procedure and one-step RRC connection release procedure controlled by the network have been used in LTE and NR. In our view, those have been working well, and we consider them as a baseline.

Furthermore, the concept of RLM/RLF is being used for radio link recovery. In 6G, bad coverage problem will still exist. The radio link recovery is vital. Therefore, we see that existing concept and procedure should also be kept in 6GR. With this in mind, RRC connection re-establishment procedure framework in NR can also be a reference.

Proposal 4: NR connection management framework of RRC connection establishment procedure/RRC release procedure/RRC re-establishment procedure is considered as baseline of 6GR.

Conclusions

Based on the discussion above, the following proposals are requested to be discussed:

Proposal 1: Characteristics of inactive mode/sub-state can be merged into either connected mode or idle mode.

Proposal 2: Small data transmission without mode transition should be supported in 6G.

Proposal 3: RNA should be same as either TA or a single-cell coverage.

Proposal 4: NR connection management framework of RRC connection establishment procedure/RRC release procedure/RRC re-establishment procedure is considered as baseline of 6GR.

Reference

- Draft chairman notes on RAN2#131bis meeting

- TS 38.300	NR and NG-RAN Overall Description, Stage-2