3GPP TSG-RAN WG2 Meeting #132		R2-2508255

Dallas, USA, 17th – 21st Nov., 2025

Source:	vivo

Title:	Discussion on MAC open issues for UEI BMR

Agenda Item:	8.12.2

Document for:	Discussion and Decision

- Introduction

In RAN2#131bis meeting, the remaining MAC issues related to UEI BMR have been listed and discussed in [1], while two remaining issues have no conclusion.

In this contribution, we will discuss these two open issues.

- Discussion

  - MAC open issues

Issue#1: For case a) the TAT for mode-B PUSCH is expired while the TAT for PUCCH is running, release PUCCH:

RAN1 has agreed some details about first PUCCH indication and mode-B PUSCH as below:

Agreement from RAN1#116bis On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all modes For above procedures, cross-CC beam reporting is supported for both modes. - FFS: Details. Agreement from RAN1#118 On beam report transmission procedure for UE-initiated/event-driven beam reporting, for regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 is at least type 1 CG-PUSCH. - FFS: PUCCH as the second channel - FFS: Whether the PUSCH can be with UL data Agreement from RAN1#118 On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured. - Above applies to both cross-CC and same-CC beam report. - Note: Above is applied to the case that second UL channel is PUSCH. FFS: Whether the first and second channels can be from the same/different CC. Agreement from RAN1#118bis On beam report transmission procedure for UE-initiated/event-driven beam reporting, resource mapping/configuration between first and second UL channel in Mode-B, at least Option-1 is supported - Option-1 (one-to-one): Only one periodic PUCCH resource for the first channel and only one pre-configured resource for second UL channel can be associated with the CSI report configuration for UE-initiated/event-driven beam reporting. - Down-select one of the following in RAN1#118bis - Option-1A: Same periodicity between first PUCCH resource and pre-configured resource for second UL channel. - Option-1B: No restriction in terms of periodicity. Agreement from RAN1#119 Confirm the following working assumption in RAN1#117: On beam report transmission procedure for UE-initiated/event-driven beam reporting - For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved. - FFS: details on the dedicated RRC signaling - Above applies at least for the single CC case. Agreement from RAN1#120 On beam report transmission procedure for UE-initiated/event-driven beam reporting, support the following option of dropping rule for the Case-1: the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR - Option-1: LRR > first PUCCH > normal SR Note: When the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR, only one of them is transmitted based on the above priority rule Agreement from RAN1#121 On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, introduce an RRC parameter to indicate one from {SpCell, PUCCH-Scell} as the cell of the configured PUCCH. - Note: It is up to NW implementation to configure same or different PUCCH resource IDs in different serving cells

According to the above agreement, for mode-B UEI reporting, the network configures a 1-to-1 association between PUCCH and Type 1 CG PUSCH. And PUCCH and associated Type 1 CG PUSCH can be from different CCs within the same CG. Thus, they can be associated with different TAGs. However, TAT expiration behaviors of the two TAGs corresponding to PUCCH and Type 1 CG PUSCH may be different and how to resolve this issue needs further discussion.

If the TAT for PUCCH has expired while the TAT for Type 1 CG PUSCH is running, the PUCCH should be released as legacy. As the PUCCH is used to notify the transmission of Type 1 CG PUSCH carrying the beam report, to avoid NW blind detection, Type 1 CG PUSCH should not be transmitted if PUCCH is released. Furthermore, as Type 1 CG PUSCH for mode-B UEI reporting is only used to carry the beam report, it should also be released to improve resource utilization efficiency.

If the TAT for Type 1 CG PUSCH has expired while the TAT for PUCCH is running, the Type 1 CG PUSCH should be released as legacy. As Type 1 CG PUSCH is released, the PUCCH transmission is meaningless. However, a concern for the release of PUCCH is that the PUCCH can be used to carry HARQ-ACK. If the PUCCH is released, HARQ-ACK cannot be transmitted.  In our view, this concern is not critical as it can be avoided by network scheduling. For example, the network indicates a PUCCH that has a different PUCCH resource id from the PUCCH configured for mode-B UEI reporting in the PDCCH to carry HARQ-ACK.  Besides, the PUCCH configured for mode-B UEI reporting may also be configured to carry SR/LLR. For this case, if TAT for Type 1 CG PUSCH has expired, the PUCCH cannot be released because of the transmission of positive SR/LLR. Therefore, UE releases PUCCH only when the corresponding PUCCH resource id is associated only with Type 1 CG PUSCH for mode-B UEI reporting.

Proposal 1: For mode-B UEI reporting, PUCCH and Type 1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for Type1 CG PUSCH has expired while the TAT for PUCCH is running, and if the configured PUCCH resource id is only associated with this type-1 CG PUSCH, UE releases the PUCCH.

Issue#2: DRX active time for mode-A UEI reporting

According to the discussion in RAN2#131bis meeting, two options have been identified in [1] as below:

- Option 1: UE stays in active time until receiving PDCCH scheduling. No MAC impact.

- Option 2: UE stays in active time until the next PUCCH resource occasion. This leads to MAC impact.

For mode-A UEI reporting, UEIRI is used to request PUSCH to carry beam report, which is similar to SR. Therefore, DRX active time for SR should be reused for mode-A UEI reporting, i.e., Option 1. The concern for Option 1 is that DRX active time will be indefinite if the NW misses the PUCCH or the UE misses the PDCCH. However, long-term PUCCH missing is very rare, as the UE will keep the PUCCH transmission when the serving beam is bad, until the network updates the serving beam based on the beam report. Furthermore, if the serving beam has poor quality for a long time, BFR/RLF procedure will be triggered as a fallback mechanism. As for the case that the serving beam recovers soon after transmitting the UEIRI on PUCCH, we think it is a corner case and happens rarely. For the normal case that NW detects the PUCCH and UE receives the scheduling PDCCH successfully, as the scheduling PDCCH is received between two PUCCH occasions, the duration of DRX active time for Option 1 is shorter than Option 2. Therefore, it is unnecessary to optimize it for some corner cases.

Proposal 2: For DRX, after transmitting UEIRI on PUCCH, UE stays in active time until receiving PDCCH scheduling or receiving PDCCH/PDSCH updating the current beam RS.

- Conclusion

In this contribution, we discuss the remaining MAC open issues on UE-initiated/event-driven beam management. Based on the above discussion, we have the following proposals.

Proposal 1: For mode-B UEI reporting, PUCCH and Type 1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for Type1 CG PUSCH has expired while the TAT for PUCCH is running, and if the configured PUCCH resource id is only associated with this type-1 CG PUSCH, UE releases the PUCCH.

Proposal 2: For DRX, after transmitting UEIRI on PUCCH, UE stays in active time until receiving PDCCH scheduling or receiving PDCCH/PDSCH updating the current beam RS.

- References

- R2-2507734, [AT131bis][203][MIMOevo] Remaining MAC issues, Samsung