3GPP TSG-RAN WG2 Meeting #132	R2-2508358

# Dallas, USA, 17-21 November 2025

Agenda item:		10.3.2

Source:	T-Mobile USA

Title:	6G Service Aware RAN – Operator View

Document for:		Discussion and Decision

# Introduction

Modern mobile networks must efficiently manage diverse application traffic with varying service requirements (e.g., latency, reliability, bandwidth). However, networks lack real-time insight into the specific service characteristics and quality expectations of individual application traffic sub-flows. This limitation prevents the network from optimizing resource allocation and traffic differentiation in a manner aligned with end-user experience. The absence of dynamic, application-originated feedback on traffic requirements and quality of service degrades the network's ability to make informed decisions, especially during congestion scenarios.

The way to address this problem is by enabling applications, whether running on user equipment or application servers—to continuously communicate detailed, qualitative service characteristics for different traffic sub-flows to the network. This mechanism allows the network to adapt its behavior to better meet application-level performance needs, improving overall service quality and user satisfaction.

# Discussion

## Background and Assumptions

The proposed Service Aware RAN concept applies universally to all applications, including over-the-top (OTT) third-party applications that operate without any T-Mobile-specific configuration. It is agnostic to the mechanisms used for applying QoS to packets, making it suitable for all traffic types—ranging from best-effort flows to those utilizing standardized 5QIs or network slicing.

In this context, the term network refers primarily to the mobile access network, with a focus on the RAN. However, it may also encompass relevant components of the core network and the application hosting environment, where applicable.

## Conceptual Overview

Applications—whether running on the user equipment (UE) or hosted on an external application server—communicate their service characteristics to the network. These characteristics describe the traffic handling requirements from the application's perspective and are used to inform the network about how different portions of traffic should be treated.

Each application may generate multiple sets of service characteristics, corresponding to distinct traffic sub streams that require differentiated handling. For example:

- A control message may require low latency, minimal data volume, and extremely high reliability.

- An image transfer may demand high throughput, moderate latency, and tolerance for some packet loss.

These service characteristics are scoped to a single application or service instance and are not intended to compare or prioritize traffic across different applications. That is, they do not serve as a mechanism to distinguish between, for example, a high-priority industrial control application and a low-priority consumer app.

Applications periodically send updated service characteristics to the network while active. The exact update interval could be per packet or over a longer period of time.

Service characteristics include quantitative parameters such as:

- Latency

- Reliability

- Throughput

- Priority

## Network Utilization of Service Characteristics

The Service Characteristics provided by applications serve as dynamic, application-level cues that guide the network in making intelligent decisions about resource allocation, traffic handling, and service assurance. These cues are used in several key ways:

#### Traffic Differentiation and Prioritization

The network uses Service Characteristics to distinguish between different traffic segments within the same application. For example:

- A control message requiring ultra-low latency and high reliability is prioritized over a concurrent image upload from the same app.

- This intra-application differentiation allows the network to apply fine-grained scheduling and buffering strategies.

#### Congestion Management

During periods of congestion, the network can use Service Characteristics to selectively discard or delay traffic that has lower impact on user experience. For instance:

- If an application indicates that image uploads are tolerant to delay or loss, the network may deprioritize that traffic to preserve resources for latency-sensitive control messages.

- This minimizes perceptible degradation and maintains service continuity.

#### Dynamic Resource Allocation

The network dynamically adjusts radio and transport resources based on real-time updates from applications:

- If an AR app signals increased throughput needs due to a new scene being rendered, the RAN can allocate additional bandwidth.

- Conversely, if an app reports that current delivery exceeds its needs, the network may reallocate resources to other users or services.

##### Closed-Loop Optimization

Bidirectional feedback enables closed-loop control:

- From Application to Network: The app reports whether current delivery meets its needs (e.g., “latency too high”).

- From Network to Application: The network informs the app of current capabilities (e.g., “high bandwidth not available”), prompting the app to adjust its behavior (e.g., reduce video resolution or frame rate).

#### Policy Enforcement and Slice Adaptation

In networks supporting slicing or policy-based traffic management:

- Service Characteristics can be mapped to slice attributes or QoS policies.

- The network can steer traffic to appropriate slices or adjust slice parameters to meet evolving service demands.

#### AI/ML-Driven Optimization

Service Characteristics serve as valuable input for AI/ML models deployed in the RAN:

- Models can predict future service needs based on historical patterns.

- Anomalies (e.g., sudden drop in reliability) can trigger automated mitigation actions.

- Anomaly prevision (e.g., prediction of anomalies to preserve service experience).

- Dynamic allocation and adaptation of network slices to serve the real-time demands of diverse services.

- Data collection, model training, and inferencing to optimize performance based on service needs (e.g., channel state information feedback, mobility, and energy savings).

### Proposal

Proposal 1: RAN2 should study enhancements to the Radio Interface architecture and protocols with service awareness as a design principle, including novel scheduling methodologies.

# Conclusion

This contribution emphasizes the need for Service Aware RAN in 6G.

Proposal 1: RAN2 should study enhancements to the Radio Interface architecture and protocols with service awareness as a design principle, including novel scheduling methodologies.

# References

[1] RP-251881, NTT Docomo (Moderator), "Study on 6G Radio"

[2] RP-251986, T-Mobile USA, AT&T, Nokia, Ericsson, Telstra, Verizon, “Requirements for Service Aware 6G RAN”

[3] R3-256716, T-Mobile USA, “6G Service Aware RAN Architecture Option”