# 3GPP TSG RAN WG2 Meeting #132      	                        							R2-2508384
Dallas, US, Nov. 17 – 21, 2025

Agenda item:	10.4

Source:	Apple

Title:	6G Mobility Discussion

WID/SID:	FS_6G_Radio – Release 20

Document for:	Discussion and Decision

# Introduction

RAN2 had a preliminary discussion in RAN2-131bis on Mobility and the below points were agreed to be studied as part of this.

Agreements

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

In this paper, we focus on the above aspects with some proposals to move this study forward keeping in mind the principles from RAN SID (RP-252912) that the design of 6GR should be simple, efficient, with optimal user experience.

# Mobility Topics

## Typical Anatomy

The discussion in RAN2-131bis on interruption was a fruitful one where there was consensus among companies to minimize interruption and throughput degradation. To us, the interruption can be viewed as the time where the UE cannot send or receive (essentially resume) user packets, and this needs to be reduced for 6G.

Observation 1: Interruption time for practical purposes can be considered as the time where the UE cannot transfer and receive user packets/service. Ensuring better service continuity despite interruption is possible by avoiding “reconfiguring or re-structuring” the data-path as much as possible.

In the interest of allowing flexibility for the gNB to control the complete mobility configuration, 5G NR has started off with the mobility framework where a lot of layers participate and get impacted by a typical L3 NR handover. Figure 1 provide the typical impacted components.

Re-established

Intra-DU LTM

Reset

Reset/Resync

L3 Handover

Figure 1: Mobility impacts to RAN architecture

Towards the later releases of NR, RAN2 tried to focus on the actual moving parts of mobility to reduce the operational overhead and arrived at focusing on PHY/MAC configuration handling as this is the main component that changes with mobility.

Observation 2: In later releases of NR, LTM brought in the aspect where the data bearers (and security on these) are not changed ensuring that the data-path is not re-established at handover. LTM also showed that for a typical handover, only the physical layer and MAC layer configuration changes, but everything else (RB config for eg) usually remains the same. In other words, only NR equivalent CellGroupConfig changes typically in handover.

Observation 3: It is unavoidable in 6G to have only LTM-like handovers where the CU is not changed. But mobility can be more efficient if RAN2 can optimize for the frequent cases (like 5G equivalent intra-CU) at the cost of increasing the latency for infrequent (inter-CU) cases.

For 6G it would be a good direction to explore the case where part of the RAN is maintained in the cloud – so the configuration to the UE is from a “centralized” server (from UE perspective) and only the PHY/MAC configuration is TRP specific. The intent here is that the configuration for the UE does not need to change whenever there is a handover, except for the critical PHY/MAC config which is associated with the TRPs. Our companion paper [2] discusses the RAN cloudification in RAN3 and it would be good if RAN2 takes advantage of this for Mobility.

Proposal 1: RAN2 to study as primary mobility use-case where only physical and MAC configuration is changed as part of mobility and optimize for this use-case. Other mobility use-cases are to be considered as secondary.

## Dissecting Layer 2 for Mobility

With the introduction of LTM, RAN2 has started using L2 (specifically MAC layer with MAC CEs) to deliver “critical” mobility control messages to facilitate mobility in a low latency method. It is important to note that these L2 MAC messages do NOT have any L2 ARQ protection. In Rel-19 and in Rel-20, the MAC CE are further used for transfer of measurement events and reports, and also for changing of measurement configuration.

gNB PHY MAC RLC PDCP RRC UE RRC Mobility messages RRC PDCP MAC CEs: (NO L2 ARQ PROTECTION!) LTM trigger Measurement Report Measurement Configuration RLC MAC PHY

Figure 2: Mobility impacts to RAN architecture

Observation 4: The measurement handling for mobility involves UE dealing with PHY reference signals and reporting measurements on these. For mobility to be efficient and robust, the configuration of the measurement to the UE and UE reporting of measurements should be fast and reliable.

Observation 5: Faster control plane configuration and UE feedback is needed also for reducing interruption and increasing mobility robustness. In order for the UE to avoid link failures, it is important that the PHY/MAC configuration is provided in a timely manner AND applied at the UE faster. Faster UE reporting and application of such configuration is also needed.

Observation 6: With Rel-18, critical mobility control message – “handover trigger” was introduced with MAC CE. In Rel-19 and Rel-20, RAN2 proceeded further with adding more critical mobility control messages – measurement event reporting, measurement reporting, and even measurement configuration change, all with MAC CE. And none of these MAC CEs have L2 ARQ protection.

If we study methods of enhancing L2 to provide low-latency and low overhead method of delivering critical mobility control messages, while still being “reliable”, we see no reason to have additional L3 based mobility control message delivery for mobility other than maybe providing periodical semi-static configuration.

Proposal 2: For fast, efficient yet robust mobility, RAN2 to evaluate on using low overhead (MAC like)  Layer-2 for all mobility related “critical and short” control configuration exchange and for measurement reporting. To study the feasibility of L2 where the requirements for this Layer-2 are to operate with lower latency with security while still being able to carry the control information in a reliable manner.

## Unification of sorts

RAN2 had a discussion in RAN2-131bis on unification of mobility and companies were not sure what it would mean and where the unification would apply. To us, riding on the direction of designing a “better” Layer-2 that has low-latency operation with low overhead while still being reliable, the unification would then be using this layer for all mobility related main operations.

In other words, other than semi static configuration from the RRC layer, all other important mobility functions: measurement configuration, measurement reporting, triggering of mobility etc, should be done at this layer, and where possible with a single UE procedure.

Observation 7:  A “better” Layer-2 that has low-latency operation with low overhead while still being reliable provides the platform for carrying all important mobility functions: measurement configuration, measurement reporting, triggering of mobility. RRC layer can then be used just for semi-static periodic configuration where needed. Unification would then be using a single layer for mobility and using a single trigger for handover.

Proposal 3: Unification of mobility procedures would mean RAN2 evaluation of using the same “single” Layer-2 for both “critical” and compact configuration (including measurement configuration and reporting), as well as triggering handover. RRC layer is to be used for semi-static non-frequent configuration where needed.

## Mobility components for enhancements

Over the course of several NR releases, RAN2 has introduced many mobility features incrementally (for CHO, SN mobility etc). Many of the 5G NR mobility enhancements were not adopted due to the complexity and/or practical deployment issues. It would be prudent that RAN2 picks the topics for 6G DAY 1, that are commercially useful.

While we acknowledge that first release in any technology cannot have all the bells and whistles, RAN2 should use the study-item phase to evaluate the options that were in 5G with the aim of improving it for 6G as well as attempting to make these simpler and practical.

Observation 8:  Further minimizing interruption during handover is possible with the late 5G NR enhancements like RACH-less handover, early DL/UL sync, early CSI acquisition). However, 6G should support diverse device types with varying levels of PHY/RF/baseband capabilities. So not all 6G UEs are expected to support these.

Proposal 4: RACH-less handover, early DL/UL synchronization, early CSI acquisition are supported in 6G from Day 1, and these features are optional at the UE.

## IRAT Mobility

MRSS is considered as the main tool for migration from 5G, but RAN has already agreed to design IRAT mobility which would be needed independent of migration directions. Also MRSS requires some sort of harmonization between 5G and 6G at physical layer for smooth co-existence.

Observation 9: Irrespective of whether we support DC or other migration options with 5G, inter-RAT handover is critical and basic must have for 6G Day 1 operation. It is critical to optimize this compared to 4G-5G handover. Since MRSS is likely a Day 1 feature, the 6G physical layer is expected to operate in a cohesive manner with 5G physical layer. We should take advantage of this for more efficient IRAT handover between 5G and 6G and 6G CP and 6G UP optimizations need to be considered for IRAT mobility.

Observation 10: For UP, architecture evaluations should be done in UP agenda on how to align the 6G user plane protocol convergence with 5G UP to ensure that data-path interruption can be minimal at IRAT handover

Proposal 5: For efficient IRAT mobility, RAN2 to study 6G user-plane architecture to have a common convergence layer with 5G UP so that data-path interruption is minimal.

Proposal 6: For efficient IRAT mobility, RAN2 should evaluate on 6G UE performing IRAT RACH-less handover, IRAT early DL/UL synchronization and IRAT early CSI acquisition as optional features at the UE. Consult RAN1 and RAN4 for feasibility of this.

# Conclusions

Observation 1: Interruption time for practical purposes can be considered as the time where the UE cannot transfer and receive user packets/service. Ensuring better service continuity despite interruption is possible by avoiding “reconfiguring or re-structuring” the data-path as much as possible.

Observation 2: In later releases of NR, LTM brought in the aspect where the data bearers (and security on these) are not changed ensuring that the data-path is not re-established at handover. LTM also showed that for a typical handover, only the physical layer and MAC layer configuration changes, but everything else (RB config for eg) usually remains the same. In other words, only NR equivalent CellGroupConfig changes typically in handover.

Observation 3: It is unavoidable in 6G to have only LTM-like handovers without inter-CU, but with cloudification of RAN [3] and SW defined RAN (and potentially centralized) RAN, mobility can be more efficient if RAN2 can optimize for the frequent cases (like 5G equivalent intra-CU) even if the inter-CU style of handover increases the latency considering they would be infrequent.

Proposal 1: RAN2 to study as primary mobility use-case where only physical and MAC configuration is changed as part of mobility and optimize for this use-case. Other mobility use-cases are to be considered as secondary.

Observation 4: The measurement handling for mobility involves UE dealing with PHY reference signals and reporting measurements on these. For mobility to be efficient and robust, the configuration of the measurement to the UE and UE reporting of measurements should be fast and reliable.

Observation 5: Faster control plane configuration and UE feedback is needed also for reducing interruption and increasing mobility robustness. In order for the UE to avoid link failures, it is important that the PHY/MAC configuration is provided in a timely manner AND applied at the UE faster. Faster UE reporting and application of such configuration is also needed.

Observation 6: With Rel-18, critical mobility control message – “handover trigger” was introduced with MAC CE. In Rel-19 and Rel-20, RAN2 proceeded further with adding more critical mobility control messages – measurement event reporting, measurement reporting, and even measurement configuration change, all with MAC CE. And none of these MAC CEs have L2 ARQ protection.

Proposal 2: For fast, efficient yet robust mobility, RAN2 to evaluate on using low overhead (MAC like)  Layer-2 for all mobility related “critical and short” control configuration exchange and for measurement reporting. To study the feasibility of L2 where the requirements for this Layer-2 are to operate with lower latency with security while still being able to carry the control information in a reliable manner.

Observation 7:  A “better” Layer-2 that has low-latency operation with low overhead while still being reliable provides the platform for carrying all important mobility functions: measurement configuration, measurement reporting, triggering of mobility. RRC layer can then be used just for semi-static periodic configuration where needed. Unification would then be using a single layer for mobility and using a single trigger for handover.

Proposal 3: Unification of mobility procedures would mean RAN2 evaluation of using the same “single” Layer-2 for both “critical” and compact configuration (including measurement configuration and reporting), as well as triggering handover. RRC layer is to be used for semi-static non-frequent configuration where needed.

Observation 8:  Further minimizing interruption during handover is possible with the late 5G NR enhancements like RACH-less handover, early DL/UL sync, early CSI acquisition). However, 6G should support diverse device types with varying levels of PHY/RF/baseband capabilities. So not all 6G UEs are expected to support these.

Proposal 4: RACH-less handover, early DL/UL synchronization, early CSI acquisition are supported in 6G from Day 1, and these features are optional at the UE.

Observation 9: Irrespective of whether we support DC or other migration options with 5G, inter-RAT handover is critical and basic must have for 6G Day 1 operation. It is critical to optimize this compared to 4G-5G handover. Since MRSS is likely a Day 1 feature, the 6G physical layer is expected to operate in a cohesive manner with 5G physical layer. We should take advantage of this for more efficient IRAT handover between 5G and 6G and 6G CP and 6G UP optimizations need to be considered for IRAT mobility.

Observation 10: For UP, architecture evaluations should be done in UP agenda on how to align the 6G user plane protocol convergence with 5G UP to ensure that data-path interruption can be minimal at IRAT handover

Proposal 5: For efficient IRAT mobility, RAN2 to study 6G user-plane architecture to have a common convergence layer with 5G UP so that data-path interruption is minimal.

Proposal 6: For efficient IRAT mobility, RAN2 should evaluate on 6G UE performing IRAT RACH-less handover, IRAT early DL/UL synchronization and IRAT early CSI acquisition as optional features at the UE. Consult RAN1 and RAN4 for feasibility of this.

# References

- RP-252912	Study on 6G Radio

- R3-258274     6G RAN Requirements for cloud native deployments