# 3GPP TSG RAN WG2 Meeting #132      	                        R2-2508455
Dallas, US, Nov. 17 – 21, 2025

Agenda item:	10.3.3.1

Source:	Apple

Title:	Discussion on 6G AI/ML general framework

WID/SID:	FS_6G_Radio – Release 20

Document for:	Discussion and Decision

# 1 Introduction

The SID of 6G Radio (SID RP-251881) was agreed in RAN#108 [1]. The SID objective on 6G AI/ML framework can be found as follows:

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In RAN2#131b [2], 6G AI/ML framework were discussed, and the following initial agreements were made:

Agreements on LCM Study 6G AI/ML LCM framework that supports at least the following functions/procedures: - UE AI/ML Capabilities Exchange; - Applicable Functionality Reporting; - Inference Configuration and Reporting; - Performance Monitoring Configuration and Reporting; - Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

In this contribution, we share our view on 6G AI/ML general framework, including:

- Starting point of 6G AI/ML framework study

- Study directions of 6G AI/ML framework

# 2 Discussion

## 2.1 Starting point of 6G AI/ML framework study

For general consideration of 6G AI/ML LCM framework, the first question is how to understand “general” and what can be inherited from 5G AI/ML LCM framework.

In Table 1, we provide a summary on general aspects of 5G LCM common to all 5 use cases studied in 5G (i.e. AI/ML based beam management, CSI prediction, AI/ML based positioning, AI mobility and AI/ML based CSI compression), which can be served as the starting point of 6G AI/ML framework study. Meanwhile, we also provide our view on general 6G study directions on top of the starting point.

|  | 5G LCM aspects common to all 5 studied use cases as starting point of 6G LCM | View on 6G study directions on top of the starting point |
| --- | --- | --- |
| UE capability | Legacy static FG/FG group capability framework. | - Whether dynamic UE capability depends on general 6G capability discussion. - APU framework was introduced on top of CPU for CSI related use cases. Study whether APU framework can be extended to general 6G AI/ML capability. |
| Data collection configuration (not data transfer) | UE can request to start/stop data collection and preferred configuration(s) from NW provided candidate configuration list. | Study whether UE can actively suggest configuration(s) when NW doesn’t provide any candidate configuration. |
| Model training | Offline training, whose model development details are transparent to 3GPP. | Study whether consider online model training / fine-tuning, under specific AI/ML use case and practical UE processing capability. |
| Applicability reporting | - Support initial and updated applicability reporting for inference configuration, determined by NW-side additional condition, UE-side additional condition and model availability. - Support associated ID as NW-side additional condition. | Study enhancements with more UE autonomy (e.g. the UE may select parameter value from NW configured value range to make AI/ML configuration applicable). |
| Inference Configuration and Reporting | N/A (it highly depends on AI/ML use case) | Depend on AI/ML use case |
| Functionality (De-)Activation and fallback | Follow NW indication (RRC or L1/L2 signalling depends on use case) for applicable inference configuration(s) | Consider enhancement with more UE autonomy on activation / deactivation of AI/ML inference configuration. |
| Performance monitoring | Support NW makes decision based on UE reported measurement / specified metric. | Study solution of UE making decision (e.g. UE autonomous fallback to non-AI). |

Table 1 5G AI/ML LCM aspects common to all 5 use cases (as starting point of 6G) and 6G study directions

From Table.1, we can observe that 5G AI/ML LCM framework has already specified quite a lot of general aspects common to all the 5 studied use cases. Thus, we think these common aspects are sufficient to serve as the starting point of 6G AI/ML LCM framework.

Observation 1: 5G LCM framework has specified quite a lot of general aspects common to all use cases (Table 1).

Proposal 1: 6G AI/ML LCM framework takes the following general aspects of 5G LCM common to all the use cases studied in 5G cases as the starting point:

- UE capability: Legacy static FG/FG group capability framework.

- Data collection configuration: UE can request to start/stop data collection and preferred configuration(s) from NW provided candidate configuration list.

- Model training: Offline training, whose model development details are transparent to 3GPP.

- Applicability reporting: Support initial and updated applicability reporting for inference configuration, which is determined by NW-side additional condition, UE-side additional condition and model availability. Support associated ID as NW-side additional condition.

- AI/ML (De-)Activation and fallback: Follow NW indication for applicable inference configuration(s).

- Performance monitoring: NW makes decision based on UE reported measurement / specified metric.

## 2.2 Study directions of 6G AI/ML framework

### 2.2.1: 6G direction 1: UE-initiated AI/ML management framework

From Table.1, we can observe that 5G AI/ML LCM framework belongs to “Network decision, network-initiated AI/ML management”, which is only one of 4 identified frameworks captured in Section 7.2.1.1 of TR 38.843 [3]. As the UE has better local observation and device internal information which can’t be shared with NW due to UE privacy, the other 3 UE-initiated management solutions (i.e. “Network decision, UE-initiated AI/ML management”, “UE decision, event-triggered as configured by NW” and “UE-autonomous, UE’s decision is reported to NW”) were also captured in TR 38.843 [3] (in Appendix for reference). The direction of UE initiated AI/ML management was also discussed in 5G but not concluded due to limited TU in 5G. For example, UE autonomous fallback to non-AI operation was discussed and is still on the table of Rel-20 AI mobility. Thus, we think that this direction was already justified by existing 5G AI/ML use case and 6G AI/ML should further study it as a general direction.

Observation 2: 5G LCM framework belongs to “Network decision, network-initiated AI/ML management”. 5G also studied other 3 UE-initiated management solutions captured in TR 38.843 with justification that UE has better local observation and device internal info which can’t be shared with NW. But no conclusion was made due to limited TU.

Thus, we propose:

Proposal 2: Study UE-initiated AI/ML management for at least data collection, functionality applicability and (de)activation, performance monitoring and fallback. Consider the frameworks captured in TR 38.843:

- “Network decision, UE-initiated AI/ML management”

- “UE decision, event-triggered as configured by NW”

- “UE-autonomous and UE’s decision is reported to NW”.

### 2.2.2: 6G direction 2: traffic prediction for UP and power enhancement

In 5G AI/ML study, all studied use cases focus on various measurement prediction:

- L1 measurement prediction: AI/ML based beam management (e.g. predicted top-K L1 RSRP and beam index), CSI prediction (e.g. predicted PMI in future), CSI compression (e.g. compressed channel information) and AI/ML based positioning (e.g. new PRS measurements)

- L3 measurement prediction: AI mobility (e.g. predicted L3 RSRP).

We think it is RAN2 common understanding that new L1/L3 measurement prediction will be further studied in 6G AI/ML new use cases (e.g. AI/ML based RS reduction and AI based LTM/CHO). The details can be discussed later.

However, besides measurement prediction, we think that 6G can also study traffic prediction for UP performance enhancement and UE/NW energy saving enhancement. First, we think that academic study on traffic prediction is already mature. Table 2 from [4] provides a good summary of various existing approaches of traffic prediction.

Table 2 Summary of traffic prediction approaches in academia (from [3])

Then, we have done a simulation based on 5G air interface to evaluate the accuracy of traffic prediction with LSTM as an example approach. The details of simulation assumption and LSTM details are summarized as follows:

- Simulation assumption:

  - Traffic model: actual Google Stadia gaming dataset

    - It includes application specific data set which includes combination of many traffic models including human interaction (https://github.com/wn-upf/Stadia_cloud_gaming_dataset_2020).

    - An example trace of data used in simulation is provided in Figure.1.

  - OTA transmission simulation model:

    - 5G air interface with 10 UEs in one cell.

    - BSR reporting with periodicity of 10 TTI.

- LSTM implementation details:

  - 40 hidden nodes are used to predict size of next packet at the UE.

  - 60 hidden nodes are used to predict arrival time of next packet at the UE.

  - Inference for prediction runs when a new packet arrives at the buffer.

  - Complexity: 21K FLOP

Figure 1 An example trace of traffic model in simulation (RTCP: real time control protocol, STUN Session Traversal Utilities for NAT, DTLS: Datagram Transport Layer Security)

Figure 2 CDF curves of prediction errors for both packet size and arrival time of next packet at the UE

Figure 3 Scheduling latency gain of BSR with predicted traffic info over legacy BSR

We provide simulation results in Figure 2 and Figure 3:

- Figure 2 shows the CDF curves of prediction errors for both packet size and arrival time of the next packet at the UE. It can be observed that the prediction error is quite small: normalized MSE of packet size prediction@90% <0.01 and normalized MSE of packet arrival time prediction@90% <0.2.

- Figure 3 shows the system performance gain in term of scheduling latency calculated as the time difference between the packet arrival time in UE and the packet reception time in gNB. It can be observed that up to 21% latency gain can be achieved compared with legacy BSR without traffic prediction.

Observation 3: Academic study on traffic prediction is mature. Simulation results show that AI/ML can predict future traffic with high accuracy in the studied cases (normalized MSE of packet size prediction@90% <0.01 and MSE of packet arrival time prediction@90% <0.2). And up to 21% performance gain over 5G can be achieved.

Please note that BSR prediction is just an example to show benefit of traffic prediction. We think that traffic prediction can be used as a general tool to enhance various User Plane performances. For example:

- Throughput enhancement: traffic prediction utilized for BSR and LCP.

- Robust enhancement: traffic prediction utilized for PDCP duplication.

- UE/NW power saving enhancement: traffic prediction utilized for UE CDRX and Cell DTX/DRX.

Thus, we propose:

Proposal 3: Study traffic prediction as a general tool to enhance User Plane performance and UE/NW energy saving.

### 2.2.3: 6G direction: others

According to table 1, we also identify the following other 6G directions:

- Dynamic capability reporting for 6G AI/ML

- General AI/ML capability (e.g. APU)

- Online training / model fine-tuning

However, we propose to postpone the discussion of these directions in RAN2 due to different reasons.

First, for dynamic capability reporting for 6G AI/ML, we think it technically makes sense to support dynamic capability reporting in 6G AI/ML. For example, 5G applicability reporting is essentially a kind of dynamic UE capability reporting. However, please note that 6G general UE capability framework is discussing whether to support dynamic capability reporting. To avoid duplicated discussion, we think it can be discussed after conclusion of 6G general UE capability is clear.

Proposal 4: RAN2 postpone the discussion on dynamic capability reporting for 6G AI/ML after the conclusion of whether 6G general capability supports dynamic reporting is clear.

Then, for general AI/ML capability (e.g. APU), we are open for discussion. However, we tend to think that it should be the outcome of 6G AI/ML use case study. Taking example of APU, its main study aspect is whether APU can be extended to non-CSI AI/ML use case. Thus, we prefer to postpone the discussion after the scope of 6G AI/ML use case is clear.

Observation 4: On general AI/ML capability (e.g. APU), its main study aspect is whether APU can be extended to non-CSI AI/ML use case. Thus, RAN2 needs to first wait for the scope of 6G AI/ML use case.

Proposal 5: RAN2 postpone the discussion on general AI/ML capability (e.g. APU) after the scope of 6G AI/ML use case is clear.

Finally, for online training / model fine-tuning, we are open for discussion. However, we can foresee at least the following differences from offline training:

- Its specification impacts may depend on specific AI/ML use case

In 5G offline training, the design of data collection can be AI/ML use case agnostic due to no latency requirement (i.e. the time of data collection and the time of model training are decoupled). However, online training / model fine-tuning will put a new latency requirement on data collection, which is expected to be different across different 6G AI/ML use cases.

- Its feasibility needs to be evaluated subject to valid / practical UE device processing capability

5G offline training doesn’t require extra UE device processing capability because the time of data collection and the time of model training are decoupled. However, online training / model fine-tuning will obviously set a new requirement on UE device processing, which is also expected to be different across different 6G AI/ML use cases. Thus, its feasibility needs to be evaluated subject to valid / practical UE device processing capability.

Observation 5: For online training / model fine-tuning, the following differences from offline training are foreseen:

- Its specification impacts may depend on specific AI/ML use case

- Its feasibility needs to be evaluated subject to valid / practical UE device processing capability

Since it is not clear what is the 3GPP specification impact due to online training, it is necessary to first evaluate its necessity under specific new 6G AI/ML use case and specific spec changes, subject to valid / practical UE device processing capability. Thus, we propose:

Proposal 6: RAN2 postpone the discussion on online training / model fine-tuning after the 6G AI/ML use case discussion, and its feasibility needs to be evaluated under specific new 6G AI/ML use case and spec changes, subject to valid / practical UE device processing capability.

# 3 Conclusion

In this contribution, we share our general consideration on 6G AI/ML framework. Our observation and proposals are summarized in one page as follows:

Starting point of 6G AI/ML framework study

Observation 1: 5G LCM framework has specified quite a lot of general aspects common to all use cases (Table 1).

Proposal 1: 6G AI/ML LCM framework takes the following general aspects of 5G LCM common to all the use cases studied in 5G cases as the starting point:

- UE capability: Legacy static FG/FG group capability framework.

- Data collection configuration: UE can request to start/stop data collection and preferred configuration(s) from NW provided candidate configuration list.

- Model training: Offline training, whose model development details are transparent to 3GPP.

- Applicability reporting: Support initial and updated applicability reporting for inference configuration, which is determined by NW-side additional condition, UE-side additional condition and model availability. Support associated ID as NW-side additional condition.

- AI/ML (De-)Activation and fallback: Follow NW indication for applicable inference configuration(s).

- Performance monitoring: NW makes decision based on UE reported measurement / specified metric.

Study directions of 6G AI/ML framework

Observation 2: 5G LCM framework belongs to “Network decision, network-initiated AI/ML management”. 5G also studied other 3 UE-initiated management solutions captured in TR 38.843 with justification that UE has better local observation and device internal info which can’t be shared with NW. But no conclusion was made due to limited TU.

Proposal 2: Study UE-initiated AI/ML management for at least data collection, functionality applicability and (de)activation, performance monitoring and fallback. Consider the frameworks captured in TR 38.843:

- “Network decision, UE-initiated AI/ML management”

- “UE decision, event-triggered as configured by NW”

- “UE-autonomous and UE’s decision is reported to NW”.

Observation 3: Academic study on traffic prediction is mature. Simulation results show that AI/ML can predict future traffic with high accuracy in the studied cases (normalized MSE of packet size prediction@90% <0.01 and MSE of packet arrival time prediction@90% <0.2). And up to 21% performance gain over 5G can be achieved.

Proposal 3: Study traffic prediction as a general tool to enhance User Plane performance and UE/NW energy saving.

Directions need to be postponed in RAN2

Proposal 4: RAN2 postpone the discussion on dynamic capability reporting for 6G AI/ML after the conclusion of whether 6G general capability supports dynamic reporting is clear.

Observation 4: On general AI/ML capability (e.g. APU), its main study aspect is whether APU can be extended to non-CSI AI/ML use case. Thus, RAN2 needs to first wait for the scope of 6G AI/ML use case.

Proposal 5: RAN2 postpone the discussion on general AI/ML capability (e.g. APU) after the scope of 6G AI/ML use case is clear.

Observation 5: For online training / model fine-tuning, the following differences from offline training are foreseen:

- Its specification impacts may depend on specific AI/ML use case

- Its feasibility needs to be evaluated subject to valid / practical UE device processing capability

Proposal 6: RAN2 postpone the discussion on online training / model fine-tuning after the 6G AI/ML use case discussion, and its feasibility needs to be evaluated under specific new 6G AI/ML use case and spec changes, subject to valid / practical UE device processing capability.

# 4 References

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO.

[2] RAN2#131b, Chair Notes.

[3] TR 38.843-v2.0.1, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), 2023-12.

[4] Aaron Chen, Jeffrey Law and Michal Aibin (2021). A Survey on Traffic Prediction Techniques Using Artificial Intelligence for Communication Networks. Telecom. 2. 518-535. 10.3390/telecom2040029.

# Appendix: UE-initiated solutions captured in TR 38.843

#### 7.2.1.1	Signalling procedures for model and functionality life cycle management

As per the functional framework in Figure 4.4-1, in this clause the signalling procedures for different scenarios for model-ID-based management and/or functionality-based management are exemplified. The procedures can at least be considered for UE-side models. From clause 4.2, these can include scenarios for which the management decision is taken by the network or by the UE. For network-side decision, this can be either network-initiated, or UE-initiated and requested to the network. While for UE-side decision, this can be either event-triggered as configured by the network and where the UE’s decision is reported to the network, or UE-autonomous, with or without UE’s decision being reported to the network.

Note:	The mapping of these scenarios to specific use cases can be left to RAN1.

Note:	The scenarios discussed below shall not imply support for all potential functionality and/or model Management Instructions (e.g., (de)activation, selection, switching, fallback, etc.) for every use case.

Note:	In the figures below, Management Request/Management Instruction/Management Decision Report may include details about the model/functionality selection, (de)activation, switching or fallback.

-	Decision by the network

o	Network-initiated

Figure 7.2.1.1-1: Network decision, network-initiated AI/ML management

The case where the LCM decision is taken and initiated by the network is depicted in Figure 7.2.1.1-1.

Note:	The Management Instruction may be a result of model/functionality performance monitoring at the network.

Note:	The Management Instruction may include information about the model or functionality.

o	UE-initiated and requested to the network

Figure 7.2.1.1-2: Network decision, UE-initiated AI/ML management

The case where the LCM decision is taken by the network but where the request is initiated by the UE is depicted in Figure 7.2.1.1-2.

Note:	The Management Request may be a result of model/functionality monitoring at the UE.

Note:	In response to the Management Request, the network may send a Management Instruction to the UE.

Note:	The Management Request may include information about the model or functionality.

Note:	The network may accept or reject the Management Request from the UE.

Note:	The Management Request may include information related to model/functionality performance metrics.

Note:	The Management Instruction may include information about the model or functionality.

-	Decision by the UE

o	Event-triggered as configured by the network, UE’s decision is reported to the network

Figure 7.2.1.1-3: UE decision, event-triggered as configured by the network

The case where the LCM decision is taken by the UE according to prior network configuration is depicted in Figure 7.2.1.1-3.

Note:	Use case-specific events/conditions may be configured by the network for event-triggered AI/ML management at the UE.

Note:	UE may send a Management Decision Report to the network following event-triggered AI/ML management at the UE.

Note: The Management Decision Report may include information about the model or functionality.

o	UE-autonomous, UE’s decision is reported to the network

Figure 7.2.1.1-4: UE autonomous, decision reported to the network

The case where the LCM decision can autonomously be taken by the UE is depicted in Figure 7.2.1.1-4.

Note:	The UE may be configured to send a Management Decision Report to the network upon performing a model/functionality Management Decision.

o	UE-autonomous, UE’s decision is not reported to the network

For the case where the LCM decision can autonomously be taken by the UE and where the decision is not reported to the network, the AI/ML management is transparent from a network perspective.