3GPP TSG-RAN WG2 Meeting #132	R2-2508469

Dallas, USA, Nov. 17th - 21st, 2025

Source:	vivo

Title:	Discussion on Topology 2

Agenda Item:	9.2.2

Document for:	Discussion and Decision

- Introduction

R2#131bis has reached below agreements [1] regarding Topology 2:

Agreements

- 1.  	gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN.  FFS what information

2.	gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling.   The resource allocation can be triggered by CN request.   UE reader may provide assistance information related to AIoT transmissions.  FFS what the information may be useful to be provided to gNB.  It is up to gNB how the information is used.  It is understood that assistance information is not mandated for the gNB to determine resource allocation.

3. 	the reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

In this contribution, we will discuss the support for Topology 2 as follows:

- UE Reader selection & Uu Behavior

- Inventory and command procedure

- IoT Radio Resource Allocation and Validity

- Coordination between Uu operation and A-IoT operation

- Discussion

For A-IoT topology 2, RRC-based solution in Uu interface is the only solution since UP-based solution and NAS-based solution have been ruled out by SA2. One example of the signaling procedure is specified in [2] as:

Figure 1: Message flow for A-IoT Inventory in Topology 2 - RRC-based solution

## UE Reader selection & Uu Behavior

### UE Reader selection

During study phase for Topology2 aspect, RAN2 has agreeed that[2]:

From TR 38769-j00 For the UE reader selection, it is up to RAN3 and SA2 on whether BS or CN selects the UE reader(s) and what information is shared between BS and CN. From RAN2 perspective, it is assumed, at least for RRC based solution, there may be radio related reasons for the BS to be involved in the UE reader selection.

In RAN architecture aspects, it is specified that “In RRC based solution, Reader selection may need coordination between A-IoT-enabled gNB and A-IoT CN”. As RRC based solution has been selected, further discussion is needed on how to perform reader selection. In Rel-19, the A-IoT CN node initiates the inventory procedure by sending the Inventory Request message to the gNB, the gNB may select readers based to the Requested Service Area Information (list of A-IoT Area IDs and reader list) included in the the Inventory Request message. For Topology2, the UE reader may be selected in the same manner, i.e., based on A-IoT Area IDs and reader list.

Although UE Reader can perform AIoT operations only in RRC_CONNECTED state, the UE may be in a non-Connected state (e.g., RRC_IDLE or RRC_INACTIVE) when the network selects it as a UE reader. In case that the UE is in non-Connected state, the NW shall page the UE to enter Connected state firstly before performing A-IoT Inventory procedure. Therefore, we propose the following:

- UE Reader in all RRC states may be selected.

In addition to the A-IoT Area and/or reader list information, the NW may select the UE reader considering based on further criteria, such as:

- The UE is an AIoT-enabled UE, which relates to its capability.

- The receiving signal strength is sufficient, e.g., the UE is allowed to act as a UE reader only if the serving cell RSRP is above a certain RSRP threshold . The RSRP threshold is used to control interference from the AIoT interface to the Uu interface.

In RRC_CONNECTED state, the network can easily apply the above criteria through implementation. When the UE is in non-Connected state, the NW shall page the UE to perform the inventory procedure. If the Uu RSRP is too low, potentially causing the UE to fail to complete the A-IoT procedure properly, the UE may not accept the paging to become a UE reader. In other word, whether the UE to respond to paging for A-IoT purpose depends on whether the Uu RSRP threshold is met. Therefore, we propose the following:

- The A-IoT-enabled UE responds to a Uu Paging message for A-IoT purpose only if selection criteria are met, FFS selection criteria.

### UE Reader Camping and Access

Before providing AIoT services, a UE reader must find a cell that supports UE reader access and establish a radio connection in that cell. This cell is expected to support the following:

- RRC based A-IoT message transmission on the Uu interface.

- AIoT radio resource allocation for the UE reader.

Before accessing the cell, the UE reader relies on the system message to determine whether a cell supports UE reader access. To facilitate this, a UE-reader-support indicator can be provided in SIB1, enabling the UE reader to identify whether a cell supports UE reader access during the cell searching procedure. For a cell shared by multiple PLMNs, a separate UE-reader-support indicator can be provided for each PLMN.

- Introduce an indicator in SIB1 to indicate a cell supports the UE reader.

A UE reader operates as an access node for AIoT devices and it must be served by an AMF that supports AIoT services. After establishing a radio connection, the serving gNB should initiate the UE context setup procedure by sending an INITIAL UE MESSAGE to a selected AMF capable of providing AIoT service. To enable this, the UE reader must indicate to its serving gNB that it is accessing as a UE reader before the serving gNB selects AMF and sends the INITIAL UE MESSAGE. To minimize the delay for UE context setup on the NG interface, it is preferable for the UE reader to send a UE reader indication to the serving gNB during random access procedure. An example flowchart is provided below.

UE reader

gNB

AMF

Random Access (+ reader indication)

INITIAL UE MESSAGE

INITIAL UE CONTEXT SETUP REQUEST

SecurityModeProcedure

UECapbilityProcedure

INITIAL UE CONTEXT SETUP RESPONSE

Figure 2: Flow chart example of UE reader indication in access procedure

- The UE acting as an A-IoT reader should indicate to gNB that it is accessing as a UE reader during the Uu access procedure.

### UE Reader capability

RAN2 received SA2 LS regarding UE reader capability.

S2-2509657: SA2 is starting to conclude R20 Ambient IoT study for key issue #1: support AIoT services under the RRC-based option for UE Reader connectivity and will document the agreements in TR 23.700-30. One aspect of the overall conclusion is on UE reader authorization and SA2 has agreed the following principles: - UE Reader subscription information is available to AMF - If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, AMF informs NG-RAN that the UE is authorized to operate as a UE Reader. SA2 is discussing whether the core network needs to be aware whether a particular UE can operate as a UE reader based on the UE capability, including (but not limited to) the frequency bands a UE is operated in. SA2 would like to ask the following question to RAN2: Q1: whether UE can operate as a reader also depends on the available frequency bands that can be assigned to a UE reader for Ambient IoT operation?. SA2 would like to ask the following question to RAN3: Q1: if the answer to Q1 is yes, whether NG-RAN can inform and update the AMF whether UE is able to operate as a reader based on the available frequency bands that can be assigned to a UE reader for Ambient IoT operation?. SA2 will finalize the R20 study in Feb meeting 2026, therefore SA2 hopes RAN2 can provide feedback in an early time.

In our view, the selection and operation of a UE as a UE reader depends on the following factors:

- AIoT service requirement, such as target AIoT devices, target AIoT area. These requirements constrain the candidate pool of potential UE readers and can be ascertained by the AIoT CN node or the gNB.

- UE reader capability. As noted by SA2, UE reader capability at a minimum includes supported frequency bands for AIoT operation. This capability is essential for reader selection. To improve selection accuracy and AIoT procedure performance, more granular capability details may be considered. The UE should report its UE reader capability to the gNB. If the AIoT CN node is responsible for reader selection, it should also be made aware of the UE reader capability.

- Available gNB resource for AIoT radio. Clearly, the gNB must have radio resources available for AIoT that are compatible with the UE reader’s capabilities in order for the UE to operate as a reader.

Based on the above, whether a UE can act as a reader depends on both its UE reader capability and the availability of suitable frequency bands that the gNB can assign for Ambient IoT operation. Therefore, the answer to SA2’s question to RAN2 should be Yes.

- RAN2 replies to SA2 that UE can operate as a reader depending on both the UE Reader capability and the available frequency bands that can be assigned to the UE reader for A-IoT operation

Regardless of how UE reader authorization and selection is performed, UE reader capability reporting remains essential. The specific details of the capability can be further studied. Thus, we propose:

- UE reports its UE Reader capability for A-IoT operation. The exact set of UE Reader capability is FFS.

## Inventory and command procedure

### New SRB

For A-IoT Topology 2, as RRC-based solution is the only option, RRC message should be used to transmit A-IoT Inventory and Command PDUs, e.g. in the form of containers, along with related control information such as A-IoT session assistance information and resource allocation for the A-IoT interface. Legacy RRC messages are transmitted via SRB1, which has the highest priority, infinite Prioritized Bit Rate (PBR), higher reliability requirement, no service continuity requirement, etc. These transmission attributes of SRB1 seems unsuitable for A-IoT data, as A-IoT data may have lower priority, different PBR, and different reliability requirements compared to SRB1. Furthermore, A-IoT data is transmitted between device and CN, which may require service continuity during handover scenarios.

- RRC messages containing A-IoT data may have different transmission requirements from those of SRB1, e.g., lower priority, distinct PBR and reliability requirements.

Introducing a new SRB, e.g. SRBx, may be a feasible and reasonable solution for more efficient forwarding A-IoT data over the Uu interface. The new SRBx would be used to carry RRC messages containing A-IoT data or related information and could be configured on-demand with appropriate priority, PBR, reliability parameters, etc.

- Introduce a new SRB, e.g. SRBx, to transfer RRC messages containing A-IoT data or related information between an A-IoT UE reader and its serving gNB.

The new SRBx should be managed (i.e., addition/modification/release) by the serving gNB of the A-IoT UE reader, e.g. via RRC Reconfiguration procedure. When the serving gNB determines that a UE will act as a A-IoT UE reader to transmit A-IoT data or information over the Uu interface, it configures SRBx for that UE. Consistent with legacy rules, SRBx must be configured after AS security activation.

- The New SRBx can be configured to an A-IoT UE reader only after AS security activation.

Furthermore, normal UE without any DRB/MRB is not permitted to stay in RRC_CONNECTED mode for an extended period, which is unsuitable for an A-IoT UE reader. When an A-IoT UE reader has no user services of its own but only A-IoT transmission requirements, it should be allowed to stay in RRC_CONNECTED state until A-IoT transmission is complete. In other words, a configuration with the new SRBx for A-IoT data without any DRB/MRB is supported for a A-IoT UE reader.

- For an A-IoT UE reader, a configuration with new SRBx for A-IoT data, without any DRB/MRB, is supported.

### Inventory procedure

In the Uu interface, the RRC inventory procedure is triggered by Inventory Request of NG interface. According to recent RAN3 progress, Inventory Request from the NG interface may include the following fields:

- Message type,

- AIOTF Identifier,

- IoT Correlation Identifier,

- IoT Device Identification Requested,

- Inventory Assistance Information: Expected D2R Message Size, Approximate Number of Target A-IoT Devices and Time Interval for report,

- Follow-on Command Indication.

The first three fields indicate the type and identification of A-IoT service, which are also useful for the Uu interface. However, RRC message can use their own encoding method, e.g. unified ASN.1 method for Message type, node ID, and session ID which can be encoded or included directly as is. Details can be left to Stage 3.

The field of A-IoT Device Identification Requested carries necessary paging ID information in the device’s NAS PDU format, which should be encapsulated in an RRC container.

In our understanding, there are two options for decision of detailed AIoT paging parameters: UE reader or its serving gNB. If the independent control of the AIoT interface is granted to the UE reader, UE reader can conveniently adjust the paging parameter based on access and collision situations (e.g. when collisions occur frequently, increase RO resources and conversely, timely recycle RO resources to prevent waste and unnecessary delay) and the latency and system efficiency are relatively better. On the contrary, if the serving gNB is fully responsible for the control of the AIoT interface, the only benefit may be to reduce the complexity of the UE reader and the corresponding drawback is that the Uu interaction and overhead will be increased, while the latency performance and system efficiency are decreased rapidly.

- If the serving gNB is fully responsible for deciding AIoT paging parameters, the Uu interaction and signaling overhead will increase, while the latency performance and system efficiency will degrade significantly.

The current UE performance is extremely powerful. System performance and efficiency are more worthy goals to pursue.

- The UE reader is responsible for the control of AIoT interface, e.g., decision of AIoT paging parameters and subsequent scheduling.

Hence, the Inventory Assistance Information and Follow-on Command Indication fields provide critical information for the inventory procedure and are useful for UE reader to allocate the paging parameters and schedule subsequent operations on the A-IoT interface. Therefore, this information should be included in the RRC message.

Furthermore, RRC Inventory Request is typically the first A-IoT related message on the Uu interface. It can also carry resource allocation information for the A-IoT interface, as the serving gNB can provide appropriate resources based on the approximate scale of the inventory and follow-on command indication. This allocation method reduces delay and enables A-IoT interface to start operating as soon as possible. Alternatively, resource allocation can also be transmitted via separate procedure, e.g. reusing RRC Reconfiguration procedure, similar to the legacy resource allocation method.

- The RRC Inventory Request may at least include the following information:

- A-IoT Device Identification Requested (terminated in AIoT device, i.e. in the form of RRC container);

- Inventory Assistance Information: Expected D2R Message Size, Approximate Number of Target A-IoT Devices and Time Interval for report (terminated in UE reader and included in the fields of RRC message);

- Follow-on Command Indication (terminated in UE reader and included in the fields of RRC message);

- Optional A-IoT Resource allocation info.

In the second procedure, the RRC Inventory Report is sent from UE reader to its serving gNB. Consistent with the same rule, Inventory Report on the NG interface should include the following:

- Message type;

- AIOTF Identifier;

- A-IoT Correlation Identifier;

- Global gNB ID;

- Reader Report List: Each item includes Reader Index and Device Report List;

- Device Report List: Each item includes A-IoT NAS PDU and optional RAN A-IoT Device NGAP ID (e.g. NGAP ID is present in the case of subsequent Command).

The first three fields can be handled in the same manner as in the RRC Inventory Request and left to stage3. On the NG interface, the A-IoT message operates per-gNB granularity, and there may be multiple cell readers under the same gNB, e.g. Global gNB ID and Reader Index differentiate device report list from each cell reader under a specific gNB. For a UE reader, only one device report list is needed as there is only one A-IoT reader.

The key issue is how to differentiate devices within a UE reader and establish one-to-one device ID mapping between the Uu interface and NG interface for accurate UL and DL addressing and routing. There are 3 options for device IDentification between a UE reader and its gNB:

- Option 1: Device local ID allocated by gNB;

- Option 2: Device local ID allocated by UE reader;

- Option 3: Reusing AS ID in R19 A-IoT interface.

Option 1 aligns most closely with the principle of centralized control by the gNB. However, Option 1 requires the UE reader to first report device information to the gNB and wait for device local ID allocation, which introduces signaling overhead and additional latency. Both option 2 & 3 involves allocating the device ID by the UE reader, which is closer to the source of device ID requirement, reducing signaling overhead and latency. Compared to Option 2, Option 3 is simpler and more reasonable, as the AS ID in Rel-19 A-IoT interface is a unique identifier within the scope of a UE reader. A newly or separately allocated ID, as in Option 2, offers no advantages and only add complexity.

- The AS ID in Rel-19 A-IoT interface is the simplest method to uniquely identify a device within the scope of a UE reader.

From the gNB’s perspective, there may be multiple UE readers and even coexistence with gNB reader(s) under the same gNB. gNB should establish and maintain a one-to-one mapping between the AS ID of a specific UE reader and the RAN A-IoT Device NGAP ID, as shown in the following mapping table (Table 1). When the RRC Inventory Report message first includes a new AS ID from A-IoT interface, the gNB allocates a new RAN A-IoT Device NGAP ID to this device and associates it with the pair of UE reader Index and AS ID, completing the establishment of the ID mapping relationship. Subsequently, A-IoT Command messages from CN are addressed using this RAN A-IoT Device NGAP ID. The gNB receives CN message, identifies the corresponding UE reader Index and device AS ID from the RAN A-IoT Device NGAP ID, and delivers the A-IoT Command message to the appropriate UE reader and, subsequently, to the correct device.

Table 1 Example of Device ID mapping

|  | AS ID | RAN A-IoT Device NGAP ID |
| --- | --- | --- |
| UE reader 1 | AS ID 1 | NGAP ID 1 |
|  | AS ID 2 | NGAP ID 2 |
|  | … | … |
|  | AS ID m | NGAP ID m |
| UE reader 2 | AS ID 1 | NGAP ID m+1 |
|  | … | … |
|  | AS ID n | NGAP ID m+n |
| UE reader 3 | … | … |

- The gNB can generate a device ID for use on the NG interface (i.e. RAN A-IoT Device NGAP ID) based on the pair of UE reader Index and device AS ID of Rel-19 A-IoT interface. the gNB can then use the mapping relationship for subsequent A-IoT unicast command addressing and routing.

Hence, we propose:

- The RRC Inventory Report should include at least a Device Report List: each item includes A-IoT NAS PDU and an optional Device ID within the scope of this UE reader.

- The Device AS ID in the A-IoT interface (i.e., allocated by UE reader, as in Rel-19) can be reused as Device ID in A-IoT RRC messages between the UE reader and its serving gNB.

### Command procedure

After the Inventory procedure, the RRC Command procedure is triggered by a Command Request from the NG interface. According to the newest TS38.413 CR for A-IoT, the Command Request from the NG interface may include the following fields:

- Message type;

- AIOTF Identifier;

- A-IoT Correlation Identifier;

- RAN A-IoT Device NGAP ID;

- A-IoT NAS PDU;

- Command Assistance Information: Estimate of Expected D2R Message Size;

The first three fields are also left to stage 3. The RAN A-IoT Device NGAP ID should be translated into the appropriate pair of UE reader Index and Device AS ID using the Device ID mapping (e.g., Table 1). Thus, the gNB can include the Device AS ID in the RRC Command Request sent to the appropriate UE reader, which then delivers it to the correct device. This ensures proper DL routing of A-IoT data.

- For routing and transmitting an A-IoT Command Request from CN, the gNB can translate the RAN A-IoT Device NGAP ID in the NG message into the appropriate pair of UE reader Index and Device AS ID.

Hence, we propose:

- RRC Command Request should include at least the following information:

- Device AS ID;

- A-IoT NAS PDU;

- Command Assistance Information: Estimate of Expected D2R Message Size.

### Failure case

On the NG interface, nearly all of DL A-IoT Request messages will have a corresponding separate Failure message to report the reason for NG interface failure. Similarly, RAN2 needs to discuss whether failure may also occur on the Uu interface and the reasons for failures should be reported from the UE reader to the gNB. If Failure reporting is deemed necessary, the failure causes for the Uu interface should be designed separately based on specific Uu interface requirements and should not reuse those defined for the NG interface. Furthermore, it is up to gNB implementation on how to convert a failure report on the Uu interface into NG failure message. If NG failure cause needs to be extended, it is responsible to RAN3.

- The RRC Inventory/Command Failure case, if necessary, may report AIoT related failure cause(s) on the Uu interface to the serving gNB.

## A-IoT Radio Resource Allocation and Validity

### A-IoT resource allocation granularity

During study phase, RAN2 agreed that the A-IoT radio resources are not shared among UE readers, and are only configured to the UE readers via dedicated signaling by the network, as highlighted as following:

From TR 38769-j00 The radio resources used by A-IoT radio interface between the A-IoT device(s) and UE reader are controlled by the network. The radio resources, which are dedicated for a UE reader, are only configured to the UE reader via dedicated signalling. The mechanisms for shared resource pool amongst UE readers are not considered in this release. The UE reader in coverage of BS scenario is supported.

During the study phase, configuration granularities beyond per UE reader were proposed. Examples are as below:

- Per A-IoT message (e.g., paging, MSG1, etc.)

- Per D2R and R2D transmission pair

- Per A-IoT procedure

In our view, these finer configuration granularities represent over-optimization, causing significant Uu signaling overhead between the UE reader and BS without clear benefits. Therefore, we think all of the above configuration granularities should not be considered. Therefore, it is proposed to confirm that the agreement made during SI phase is directly used for further standardization work.

- RAN2 confirms that the A-IoT radio resource configuration is allocated per UE reader via dedicated RRC signalling. Finer configuration granularities, such as per A-IoT message or per A-IoT procedure, are excluded.

### FFS how resource allocation looks like

At the last RAN2#131bis meeting, chair left a guideline to encourage company contributions to provide more details on how resource allocation looks like as following.

Continue discussion next meeting on how resource allocation looks like.

Generally, the A-IoT resource allocation mechanism should be flexible to support different uses case i.e., Inventory request to trigger one, multiple or all A-IoT device(s) to response. In case of triggering one specific device to response, allocating one set of T&F resources to the UE reader is the simplest and can be baseline for initial resource allocation (see left picture in below Figure 3). If the UE reader determines that the initial resource allocation may not be enough due to serious collisions or transmission failures, the UE reader can request the gNB to allocate more A-IoT resources afterwards. However, in case of triggering multiple or all A-IoT device(s) to respond, allocating multiple sets of T&F resources is more reasonable given that the number of A-IoT device(s) may be large in the service area. As illustrated in right picture in below Figure 3, the multiple sets of T&F resources occur periodically within a limited time window (shown as valid time period). The validity time period concept is beneficial for the following use cases:

- Preventing an RRC CONNECTED UE reader from permanently occupying periodic resources, thereby improving TDM resource utilization efficiency.

- Addressing the loss of explicit release signaling for the resource configuration by the network during temporary out-of-connection scenarios (e.g., RLF).

- Reducing potential A-IoT service interruption and ensuring A-IoT service continuity for at least a specific period.

Based on above analysis, with a periodic resource pattern, the gNB only needs to configure once to indicate the multiple T&F resource sets to the UE reader, instead of multiple times. Therefore, RRC singnaling efficiency can be achieved.

Figure 3: Illustration of how resource allocation looks like

- For the A-IoT radio resources allocation to the UE reader, both of the following alternatives will be supported for different inventory use cases to trigger one, multiple or all A-IoT device(s) to response.

- Alt-1: One Set of T&F resources;

- Alt-2: Multiple Sets of T&F resources periodically within Valid time period.

Regarding the candidate signaling options for defining the Multiple Sets of A-IoT resources to a UE reader, three options under consideration:

- Option 1: RRC-based Configured Grant resource (e.g., like SL CG type-1 configuration IE SL-ConfiguredGrantConfig)

- Option 2: A-IoT resource pools (e.g., similar to the SL resource pool configuration IE SL-ResourcePool), with each pool configured exclusively to a UE reader

- Option 3: A-IoT specific Gap configuration (e.g., similar to the Uu Gap configuration needed for certain feature)

For Option 1, the RRC parameters are similar to the SL CG type-1 configuration IE SL-ConfiguredGrantConfig. However, this option introduces specification complexity. For example, RRC must provide a set of parameters for the configured grant (e.g., the periodicity, the time and the frequency resource location etc), and a new MAC CE for Configured Grant Confirmation is required. Additionally, existing RRC parameters, such as HARQ process ID and the PUCCH resource for HARQ feedback, are not applicable to A-IoT radio interface. Consequently, adopting Option 1 would require further work to decouple the HARQ concept from the existing configured grant mechanism.

For Option 2, the signaling is similar to the SL resource pool configuration IE SL-ResourcePool, However, since the A-IoT radio interface supports only two physical channels types, the parameters for SL resource pool can be significantly simplified. For instance, a single resource pool for PRDCH and PDRCH, defined by a repeating bitmap indicating available slots with periodicity within a SFN cycle, may suffice. However, the stringent timeline between A-IoT R2D transmission and corresponding D2R transmission, as defined in RAN1 specification, may lead to resource fragmentation within the A-IoT resource pool defined by this bitmap approach. As a result, Option 2 is less favorable due to potential low resource utilization.

For Option 3, the Uu Gap configuration signaling, such as the IE MUSIM-GapConfig can be reused as baseline. By introducing a Gap configuration that provides periodic time window, a balance between BS control flexibility and resource utilization can be achieved to effectively operate on A-IoT radio interface. For instance, within a given resource set, time-domain resources are available to A-IoT by default (no bitmap required). Besides, in case that some potential conflict between Uu and A-IoT operation may occur and is detected by the UE reader, it can be addressed according to the collision handling between Uu and A-IoT, per the discussion in Subclause 2.4. Based on this analysis, Option 3 is slightly preferred.

- RAN2 to discuss and decide which of the following signalling options to adopt for defining Multiple Sets of A-IoT resource to a UE reader:

        - Option 1: RRC-based Configured Grant resource (e.g., similar to SL CG type-1 resource)

        - Option 2: A-IoT resource pools, with each Pool configured exclusively for a UE reader (e.g., similar to SL resource pool)

        - Option 3: A-IoT specific Gap configuration (e.g., similar to Uu gap configuration needed for certain feature)

### A-IoT resource allocation validity

At the RAN2#131bis meeting, RAN2 agreed that “the reader schedules the AIoT interface transmissions within the resources allocated by the gNB”. However, it hasn’t been discussed yet whether the reader is permitted to continoue scheduling the AIoT interface transmissions using the gNB-allocated T&F resources when temporary out-of-connection scenarios (RLF, handover) occur. In our understanding, the assumption is that the resources have been reserved exclusively for the UE reader once configured. Therefore, as long as the A-IoT resources is still within the configured valid time period and not explicitly released by the gNB, the radio interference is under control and avoided by the gNB. There is no problem for the UE reader to continue scheduling A-IoT transmissions if it has on-going A-IoT service. Therefore, it is proposed to further clarifying the UE reader scheduling behavior as follows.

- The UE reader can continue scheduling A-IoT transmissions using the gNB-allocated T&F resources in temporary out-of-connection scenarios (RLF, handover), as long as that the A-IoT transmissions occur within the valid time period and the resources have not been explicitly released by the gNB.

### FFS what the information may be useful to be provided to gNB

At the last RAN2#131bis meeting, companies reached consensus that UE reader may provide information related to AIoT transmissions to assist gNB’s resource allocation. Meanwhile, it is still FFS what information may be useful to be provided. In our view, the assistance information from the UE reader should be able to address the following purposes as below:

- Motivation #1: Improve resource allocation efficiency. For instance, when the UE reader has completed the A-IoT service procedure, the gNB should be able to know and timely recycle the unused A-IoT resources if any, which can be re-allocated to other UE readers if needed.

- Motivation #2: Guarantee A-IoT service performance. For instance, when the UE reader encounters serious access collisions or A-IoT transmission failures during the A-IoT service procedure, the gNB should be able to add more A-IoT resources to the UE reader, such that the A-IoT service procedure can be completed.

Based on the above considerations, we believe that the following A-IoT info is useful to be provided to gNB. The reasons are analyzed as below:

- The Inventory Complete Indication: In Rel-19 A-IoT Topology 1, Inventory Complete Indication IE is included in the INVENTORY REPORT message during NGAP procedure, so that the A-IoT CN node can be aware that the inventory operation is completed in the RAN reader. Similarly, for Topology 2, the Inventory Complete Indication can be provided from the UE reader to its serving gNB. The serving gNB has two actions correspondingly, one is that the Inventory Complete Indication will be further forwarded to the A-IoT CN node similar as in Rel-19, and the other is that the Inventory Complete Indication will be used atthe serving gNB by itself to determine whether to trigger explicit release of the A-IoT resources to the UE reader.

- The Estimated Number of A-IoT devices to be inventoried: In Rel-19 A-IoT Topology 1, Approximate Number of Target A-IoT Devices is provided as the assistance information from the A-IoT CN node to RAN reader. Moreover, Approximate Number of Target A-IoT Devices IE is optional in the INVENTORY REQUEST message, which means that such assistance information in Topology 2 may also not always be available. On one hand, in case that such assistance information from CN is not available, the gNB may choose to blindly reserve some A-IoT resources upon RRC Inventory Request and allocate them to the UE reader, which may probably be either insufficient or overbooked. On the other hand, UE reader is response for schedule the A-IoT transmissions within the allocated A-IoT resources and thus UE reader is at the best position to know the Estimated Number of A-IoT devices to be inventoried. Therefore, providing such information to the gNB from the UE reader can be beneficial to overcome the disadvantages of blind resource allocation. Moreover, the estimation methods can be totally left to the UE reader implementation e.g., similar to the probability estimation methods (such as Maximum Likelihood Estimation, MLE) to estimate the unread tags in RFID EPC Gen2 protocol. Consequently, we don’t over specify UE behavior to estimate the A-IoT device number, and simply support providing the outcome over the air, i.e., the Estimated Number of A-IoT devices to be inventoried to the gNB, which should be enough for further adjustment of the allocated resources.

- The Average Time during which the UE reader has successfully inventoried a single A-IoT device: This information is a statistical average in the history and can be combined with the Estimated Number of A-IoT devices to be inventoried to determine the required time-domain resources in the future. Based on this information, the gNB can determine whether and how to adjust the allocated resources e.g., modify periodicity between two adjacent T&F resource sets, extend valid time period, or add more sets of T&F resources etc.

After thorough discussion and decision on the useful A-IoT info to be provided to gNB, a relevant issue left is that whether introduce new or enhance existing dedicated RRC message (e.g., UAI) to carry that info, which is suggested to be finalized during Stage-3 phase.

- Support the following A-IoT info to be provided to gNB for adjustment (i.e., release/modification/add) of the configured A-IoT resources. FFS whether introduce new or enhance existing dedicated RRC message (e.g., UAI) to carry that info.

- The Inventory Complete Indication;

- The Estimated Number of A-IoT devices to be inventoried;

- The Average Time during which the UE reader has successfully inventoried a single A-IoT device.

## Coordination between Uu operation and A-IoT operation

As an intermediate node between gNB and AIoT device, a UE reader has the following restriction as described in the WID[3]:

- The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band

- The UE reader operates as a UE on Uu interface and as a reader in A-IoT interface, but simultaneous Uu and A-IoT transmission/reception in the same band is not supported.

From the UE reader’s perspective, Uu traffic may arrive while A-IoT traffic is ongoing on its A-IoT interface, or A-IoT traffic may arrive while Uu traffic is ongoing on its Uu interface. The Uu traffic includes DL/UL data traffic transception and signaling/functional traffic (e.g. beam failure recovery, measurements, RLF etc). The AIoT traffic may include inventory, and command procedure. As simultaneous operation of Uu and A-IoT in the same band is not supported, a mechanism is needed for the UE reader to switch between Uu operation and the A-IoT operations. For example, if a UE reader is engaged in an ongoing A-IoT service, a voice call may arrive, a beam failure may occur, or a measurement report may be triggered. In such cases, the UE reader must determine whether to prioritize Uu traffic handling or continue A-IoT traffic until completion. Similarly, if the UE reader is engaged in ongoing Uu traffic, A-IoT traffic may arrive. It is unclear whether the gNB implementation should delay A-IoT traffic until Uu traffic is completed or allow the UE to switch between Uu and A-IoT traffic based on specific rules.

- RAN2 to consider the mechanism for the UE reader to coordinate between Uu and AIoT operations, for instance:

        - Mechanism to handle the Uu traffic occurance (e.g., incoming call, measurement report and BFR etc) for UE reader with ongoing AIoT service.

        - Mechanism to handle the incoming A-IoT traffic for a UE reader with ongoing Uu traffic.

- Conclusion

On a basis of above analysis, we make the following observations and proposals:

UE Reader selection & Uu Behavior

- UE Reader in all RRC states may be selected.

- The A-IoT-enabled UE responds to a Uu Paging message for A-IoT purpose only if selection criteria are met, FFS selection criteria.

- Introduce an indicator in SIB1 to indicate a cell supports the UE reader.

- The UE acting as an A-IoT reader should indicate to gNB that it is accessing as a UE reader during the Uu access procedure.

- RAN2 replies to SA2 that UE can operate as a reader depending on both the UE Reader capability and the available frequency bands that can be assigned to the UE reader for A-IoT operation

- UE reports its UE Reader capability for A-IoT operation. The exact set of UE Reader capability is FFS.

Inventory and command procedure

- RRC messages containing A-IoT data may have different transmission requirements from those of SRB1, e.g., lower priority, distinct PBR and reliability requirements.

- If the serving gNB is fully responsible for deciding AIoT paging parameters, the Uu interaction and signaling overhead will increase, while the latency performance and system efficiency will degrade significantly.

- The AS ID in Rel-19 A-IoT interface is the simplest method to uniquely identify a device within the scope of a UE reader.

- The gNB can generate a device ID for use on the NG interface (i.e., RAN A-IoT Device NGAP ID) based on the pair of UE reader Index and device AS ID of Rel-19 A-IoT interface. the gNB can then use the mapping relationship for subsequent A-IoT unicast command addressing and routing.

- For routing and transmitting an A-IoT Command Request from CN, the gNB can translate the RAN A-IoT Device NGAP ID in the NG message into the appropriate pair of UE reader Index and Device AS ID.

- Introduce a new SRB, e.g. SRBx, to transfer RRC messages containing A-IoT data or related information between an A-IoT UE reader and its serving gNB.

- The New SRBx can be configured to an A-IoT UE reader only after AS security activation.

- For an A-IoT UE reader, a configuration with new SRBx for A-IoT data, without any DRB/MRB, is supported.

- The UE reader is responsible for the control of AIoT interface, e.g., decision of AIoT paging parameters and subsequent scheduling.

- The RRC Inventory Request may at least include the following information:

- A-IoT Device Identification Requested (terminated in AIoT device, i.e. in the form of RRC container);

- Inventory Assistance Information: Expected D2R Message Size, Approximate Number of Target A-IoT Devices and Time Interval for report (terminated in UE reader and included in the fields of RRC message);

- Follow-on Command Indication (terminated in UE reader and included in the fields of RRC message);

- Optional A-IoT Resource allocation info.

- The RRC Inventory Report should include at least a Device Report List: each item includes A-IoT NAS PDU and an optional Device ID within the scope of this UE reader.

- The Device AS ID in the A-IoT interface (i.e., allocated by UE reader, as in Rel-19) can be reused as Device ID in A-IoT RRC messages between the UE reader and its serving gNB.

- RRC Command Request should include at least the following information:

- Device AS ID;

- A-IoT NAS PDU;

- Command Assistance Information: Estimate of Expected D2R Message Size.

- The RRC Inventory/Command Failure case, if necessary, may report AIoT related failure cause(s) on the Uu interface to the serving gNB.

AIoT Radio Resource Allocation and Validity

- RAN2 confirms that the A-IoT radio resource configuration is allocated per UE reader via dedicated RRC signalling. Finer configuration granularities, such as per A-IoT message or per A-IoT procedure, are excluded.

- For the A-IoT radio resources allocation to the UE reader, both of the following alternatives will be supported for different inventory use cases to trigger one, multiple or all A-IoT device(s) to response.

- Alt-1: One Set of T&F resources;

- Alt-2: Multiple Sets of T&F resources periodically within Valid time period.

- RAN2 to discuss and decide which of the following signalling options to adopt for defining Multiple Sets of A-IoT resource to a UE reader:

        - Option 1: RRC-based Configured Grant resource (e.g., similar to SL CG type-1 resource)

        - Option 2: A-IoT resource pools, with each Pool configured exclusively for a UE reader (e.g., similar to SL resource pool)

        - Option 3: A-IoT specific Gap configuration (e.g., similar to Uu gap configuration needed for certain feature)

- The UE reader can continue scheduling A-IoT transmissions using the gNB-allocated T&F resources in temporary out-of-connection scenarios (RLF, handover), as long as that the A-IoT transmissions occur within the valid time period and the resources have not been explicitly released by the gNB.

- Support the following A-IoT info to be provided to gNB for adjustment (i.e., release/modification/add) of the configured A-IoT resources. FFS whether introduce new or enhance existing dedicated RRC message (e.g., UAI) to carry that info.

- The Inventory Complete Indication;

- The Estimated Number of A-IoT devices to be inventoried;

- The Average Time during which the UE reader has successfully inventoried a single A-IoT device.

Coordination between Uu operation and A-IoT operation

- The UE reader operates as a UE on Uu interface and as a reader in A-IoT interface, but simultaneous Uu and A-IoT transmission/reception in the same band is not supported.

- RAN2 to consider the mechanism for the UE reader to coordinate between Uu and AIoT operations, for instance:

        - Mechanism to handle the Uu traffic occurance (e.g., incoming call, measurement report and BFR etc) for UE reader with ongoing AIoT service.

        - Mechanism to handle the incoming A-IoT traffic for a UE reader with ongoing Uu traffic.

- Reference

- R2#131bis Chair Note

- 3GPP TR 38769-j00 Study on solutions for Ambient IoT (Internet of Things) in NR.

- RP-252894 New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2.