3GPP TSG-RAN WG2 Meeting #132	           R2-2508496

Dallas, USA, November 17th – 21st, 2025

Agenda item:	9.3.2

Source:	China Telecom

Title:	LCM for RRM measurement prediction

Document for:	Discussion

- Introduction

In this paper, we would like to discuss LCM for UE-sided model for RRM measurement prediction based on AI/ML, especially inference configuration and reporting.

- Discussion

During study phase, RAN2 had the following inference configuration and reporting related agreements for UE-sided model for RRM measurement prediction:

RAN2#129bis - For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements. FFS details - The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework RAN2#130 - Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology). The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction) Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them. RAN2#131 For the interpretation of “skipping pattern” in temporal domain Case B, RAN2 confirm that it refers to SSB configuration to indicate the timing of NW's SSB transmission—not timing of UE's SSB measurement/skipping. For temporal domain Case A, NW can indicate the list of target cells for which it expects results (if available) as part of inference configuration. This list is optional. For temporal domain case A, one or more instances of predicted measurement results in PW per one cell are reported in one measurementReport message For temporal domain Case B, the inference report can include the latest measurement results (regardless of actual results or predicted results). For frequency domain prediction, the inference report can include the latest measurement results of the predicted cell. (No change to the existing measurement report)

Regarding Rel-19 AI/ML based beam management functionality, the prediction configuration for beam management, the RS resource configuration for actual measurement, and the report quantity configuration for inference result reporting are put in one CSI-ReportConfig and the three configuration are linked in this way. Furthermore, the RS resources to be predicted is indicated by resourcesForChannelPrediction contained in the prediction configuration, the RS resource configuration for actual measurement is indicated by legacy parameter resourcesForChannelMeasurement in CSI-ReportConfig. The UE predicts the resource indicated by resourcesForChannelPrediction based on measurements performed on resourcesForChannelMeasurement.

Similar to AI/ML based beam management functionality, the UE also need to know the resource for RRM measurement prediction and the resource for actual measurement based on which the RRM measurement prediction is performed. In other words, the linkage between the RRM measurement prediction configuration and the resource configuration for actual measurement need to be considered in the RRC signalling design.

On the other hand, according to RAN2#129bis agreements, the inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework. In CSI framework, the CSI resource ID are configured in CSI reporting configuration, and in this way the CSI resource and the reporting configuration are linked. However, unlike the CSI framework, one measurement ID is used to associate one measurement object with one measurement reporting configuration in the legacy measurement configuration framework. Measurement object indicates the resource on which the UE shall perform the measurements. Reporting configuration indicates the criterion that triggers the UE to send a measurement report, RS type and reporting format. One measurement object can’t be associated with another measurement object. Although one reporting configuration can be associated with multiple measurement objects, that means multiple measurement IDs will be used to identify each pair of reporting configuration and measurement object respectively. Thus the legacy measurement configuration need to be enhanced to link the RRM measurement prediction configuration and the resource configuration for actual measurement.

Proposal 1: Enhance the legacy measurement configuration signaling to link the RRM measurement prediction configuration and the resource configuration for actual measurement.

One straight way is introducing a new ID to associate multiple measurement objects (including predicted measurement object and actual measurement object) and an inference reporting configuration. Another way is including measurement object identity in the measurement reporting configuration to indicate resource the UE need to perform RRM measurement prediction (i.e. predicted measurement object).

Proposal 2: Consider the following two potential signaling options to enable inference resource configuration and inference result reporting configuration:

- Option 1: introduce a new ID to associate multiple measurement objects (including predicted measurement object and actual measurement object) and an inference reporting configuration.

- Option 2: include measurement object identity in the measurement reporting configuration to indicate the predicted measurement object.

According to RAN2#130 and RAN2#131bis agreements, the inference configuration for RRM measurement prediction may optionally include several parameters which depend on the cases. How to put the inference configuration in the measurement configuration framework also need to be considered. For Rel-19 AI/ML based beam management feature, the inference configuration is put in CSI-ReportConfig. Considering the legacy measurement configuration framework, the inference configuration can be carried in measurement object and reporting configuration, i.e. the resource configuration for RRM measurement prediction can be indicated by measurement object, and the other inference configuration can be carried in measurement reporting configuration.

Proposal 3: The inference configuration can be carried in measurement object and reporting configuration.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: Enhance the legacy measurement configuration signaling to link the RRM measurement prediction configuration and the resource configuration for actual measurement.

Proposal 2: Consider the following two potential signaling options to enable inference resource configuration and inference result reporting configuration:

- Option 1: introduce a new ID to associate multiple measurement objects (including predicted measurement object and actual measurement object) and an inference reporting configuration.

- Option 2: include measurement object identity in the measurement reporting configuration to indicate the predicted measurement object.

Proposal 3: The inference configuration can be carried in measurement object and reporting configuration.