3GPP TSG-RAN WG2 #132	R2-2508555

Dallas, USA, 17 – 21 November 2025

Agenda item:		10.2.2 – TN/NTN integration

Title:		TN/NTN Integration

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

According to the last RAN2 meeting’s agreement, companies are invited to identify essential requirements and functions for Non-Terrestrial Networks (NTN)[1]. Additionally, functions that can be incorporated into a common design with Terrestrial Networks (TN), as well as those that are unique to NTN, should be identified.

This document first lists specified NTN functions from Release 17 to Release 20. Using this list, the minimal essential set of NTN functions for initial deployment are identified. It also presents an initial analysis of which functionalities may be suitable for common design with TN, which may not be applicable but specific to NTN.

# 2. Discussion

2.1	Essential functions for NTN in 6G Day1

To address challenges present in satellite networks, such as long propagation delays, big cell coverage areas, moving cell coverage, and similar radio signal strengths at both the centre and the edge of cells, many functions for NTN have been introduced from Rel-17 to Rel-20. we group these functions into three categories for 6G NTN:

- Minimum essential functions, which are necessary for the 6G system to operate for NTN.

- Vital functions, the absence of which means the system may function but not efficiently.

- Optimization functions, which offer additional performance benefits.

Table 1. Essentiality of NTN functions

| Release | Enhancements to NR/IOT radio interface | Essentiality for 6G NTN (essential, vital, optimization) |
| --- | --- | --- |
| Rel17 (RP-211557; RP-211601) | - UE pre-compensation for uplink synchronization from initial access to all subsequent uplink transmission, as well as UE TA report | Essential: at least pre-compensation for initial access |
|  | - value extension on protocol parameters/timers/variables from top to bottom | Essential: to cope with the long RTT. |
|  | - HARQ feedback disabling and blind retransmission scheduling | Vital: big number of HARQ processes would also avoid HARQ process stalling, and without blind retransmission scheduling, long RTT will result in slower HARQ retransmission, and long packet delivery delay |
|  | - SMTC and measurement Gap adaptation for NTN neighbour cells measurement | Essential: without adaptation to propagation delay, the gap/SMTC timing may misalign at UE and network, which will result into failed communication |
|  | - Multiple TAC(s) broadcast and update | Essential: one TAC per satellite cell especially for moving cell, will lead to frequent TAU even for stationary UEs |
|  | - Time-based and Location-based triggered connected mode mobility and cell reselection initiation | Essential: due to the lack of signal strength difference from cell centra to cell edge, other types of events do not work |
|  | - Support discontinuous coverage scenario | Optimization: for more UE power saving |
| Rel18 (RP-232669; RP-223519) | - Satellite switch with re-sync | Vital: without this, UE may suffer RLF/RRC reestablishment, and lead to handover signalling storming |
|  | - Uplink coverage enhancements including PUCCH repetition for msg4 HARQ-ACK and PUSCH DMRS bundling | Vital: for normal phone workable in satellite network |
|  | - TN coverage area assistance information for reduced measurement and power consumption | Optimization: UE may rely on UE intelligence to know TN coverage; it is for more UE power saving |
|  | - Extended the time-based and location-based mobility enhancement to earth moving cell scenario | Skip |
|  | - UE location verification/positioning based on multi-RTT measurement with single satellites | Vital: |
| Rel19 (RP-251954; RP-252504) | - Downlink Coverage enhancements, including repetition of PDCCH in common search space, repetitions of PDSCH carrying Msg4, repetition of PDSCH carrying SIB within 20ms, and extended SSB periodicity of 160ms. | Vital: for normal device workable |
|  | - Uplink Capacity/Throughput enhancement via supporting of OCC | TBC: depending on performance target, can merge into other system performance enhancement features |
|  | - Support of MBS broadcast service and its intended service areas (target the issue the intended MBS area is smaller than a satellite cell) | Optimization: only the MBS intended service area is NTN specific function for large cell scenario. without this, nothing is broken but the UE out of intended service area will also receive MBS |
|  | - Support RedCap and eRedcap (target the issues in half-duplex collision cases at the UE side caused by Timing Advance (TA) mismatch for Half-Duplex UE in paired spectrum) | TBC: Only essential for half duplex low-end UE |
|  | - Support of regenerative payload and Ng interface management (dynamic Ng setup and removal) | Skip, it ends up not much difference from transparent mode. |
|  | - Contention based Msg3 EDT | Optimization: for achieve shorter latency |
|  | - Store and forward operation | Optimization: for a specific use case |
|  | - Support of public warning systems in IOT NTN as well (especially ETWS geofencing) | Optimization: a unified Geo fencing could be considered in higher layer |
| Rel-20 (RP-252473) | - Voice over GSO | optimization: feature for specific use case. this requires UE to support very low data rate voice encoding scheme |
| Through all releases | - To support different inter-RAT, inter NTN-TN and NTN-NTN idle and connected mode mobility | Essential: for ubiquitous coverage |

Based on the above analysis:

Proposal 1: RAN2 support the following minimum essential functions for NTN in 6G Day 1

- UE pre-compensation for uplink synchronization as well as TA report

- Adaptation of protocol parameters/timers/variables/SMTC and gap configuration to long RTT

- Multiple TAC(s) broadcast and update

- Time-based and Location-based (idle and connected mode) mobility

- support inter-RAT, inter TN-NTN, NTN-NTN idle and connected mode mobility

Proposal 2: RAN2 consider following functions are vital and make best effort to support for NTN in 6G Day 1

- HARQ feedback disabling and blind retransmission scheduling

- Uplink and downlink coverage enhancements for all channels including common PDCCH/PDCH, DMRS and PUCCH

- Satellite switch with re-sync

- positioning based on multi-RTT measurement with single RTP

2.2	Functionality in common design with TN

Among all NTN functions we have, some NTN functions can be commonly designed with TN easily, such as optional HARQ feedback, some NTN function looks very special to NTN, but a common design might be still possible.

UE TA and Frequency Pre-compensation

UE TA and frequency pre-compensation are similar, and they are the key differences between NTN and TN operation. Because frequency pre-compensation is fully in the RAN1 scope, here we focus on TA pre-compensation.

In TN, the UE measures downlink arrival timing on the serving cell and uses it to send the preamble with an initial TA of 0 (no pre-compensation). Different preamble format lengths are designed and used for different cell sizes and maximum RTTs. After the first uplink transmission, closed-loop synchronization continues as the network measures the difference between the UE's actual and expected uplink arrival times and sends TAC to correct any TA error.

In NTN, due to significant RTT delays, the UE must pre-compensate for RTT in all uplink transmissions, including the initial preamble. The network broadcasts valid ephemeris and Common TA parameters. For synchronization, before connecting to an NTN cell, the UE uses its GNSS position, ephemeris, and Common TA parameters to calculate the RTT to the RP (satellite) and autonomously pre-compensates the TA accordingly.

Pre-compensation continues after connecting to an NTN cell for uplink transmissions, operating alongside closed-loop TA management. The UE must combine pre-compensation and closed-loop TAC carefully.

For initial uplink transmission, maintaining a common design with TN means either creating a PRACH channel with an extremely long preamble—allowing RA procedures without pre-compensation in large cells, which seems impractical and requires RAN1 input—or extending pre-compensation to TN. As in NTN, with L1/L2-triggered mobility, a UE can use TA measurement to synchronize uplink with LTM candidate cells before receiving a cell switch command.

For subsequent uplink transmission, it has been discussed in the past that we may solely rely on closed-loop synchronization in NTN (e.g., in case that IOT device cannot support data transmission and GNSS reception at the same time). The framework would be same as the close loop today but with a wider TA error correction range. This could be adopted from 6G day1 as a unified design for NTN and TN.

Another indirect specification change is the UE TA report during Random Access procedures and afterwards. This is the only way for the network to know the total TA used by the UE if the UE pre-compensation is still in use.

Furthermore, the Rel-20 SID Study on GNSS-resilient NR-NTN operation is being studied in RAN1 [RP-251863]. Proposed solutions for the UE’s temporary operation without GNSS information are provided by contributing companies. RAN1 categorizes these solutions into two sets [Chairman Note RAN1#122bis]:

A. Enhancing PRACH tolerance during initial access: This allows reuse of existing PRACH channels/signals to accommodate larger timing and frequency errors when GNSS information is unavailable.

B. Reducing time/frequency uncertainty: This enables the UE to either directly estimate the TA and Doppler offset relative to the reference point based on DL signals or estimate its own position for TA and Doppler pre-compensation.

The study results are expected to pave the way for a unified design for UE TA and frequency pre-compensation applicable to both NTN and TN, including high-speed TN scenarios (e.g., HST, ATG).

Proposal 3: RAN2 assume a unified design on TA management for TN/NTN could include the following aspects, which should be confirmed by coordination with RAN1

- Enhanced PRACH formats and/or preamble sequences which could be used for different cell sizes, including the NTN cell

- An enhanced closed-loop synchronization with a wider error correction range

- A pre-compensation framework applicable to both TN/NTN

Satellite switch with resync

Satellite switch with resync is a satellite-specific function used for satellite switchover. As shown in figure below, all UEs in the coverage will reconnect to the same gNB via a different satellite after switching. PCI of the cell is intentionally kept. This is to avoid handover or RRC Re-establishment. Because relay satellite changes and RTT changes, UE must re-synchronize.

The basic idea is to indicate the switchover timing. At the indicated time/time window, all UEs firstly suspend the connection, them starts to re-sync to “the same cell with same PCI” but through “a different satellite”, finally UE will resume the transmission afterward. One similar case is NES triggered group handover from capacity cell to coverage cell, where all UE shall switch over to another cell at the indicated time/time window. In case the capability and coverage cell are intra-gNB, the procedure could be very similar.

Proposal 4: RAN2 study a common design of a group UE switch over between intra-6G NB cells for satellite switch over, NES cell switching off etc.

Coverage enhancements in uplink and downlink

When RAN2 was working on coverage enhancements for NTN scenario, overlapping with the general coverage enhancements relevant WID has been observed.

General, coverage enhancement requires repetitions for different channels including common channels, initial access channels, reference signalling channels.

Based on SID of 6G. we believe that coverage enhancement would be common design targeting diverse device types and deployment scenarios. Although the coverage enhancement is led by RAN1, RAN2 should assume a common design for TN and NTN.

Enhanced overall coverage, focus on cell-edge performance and UL coverage. Target scalable and forward compatible design for diverse device types.

Proposal 5: RAN2 assume that coverage enhancement should be considered in 6G day1 to cover different use cases including NTN and wait for RAN1 progress at first.

Other functions can be easily commonly designed for TN and NTN:

- HARQ feedback disabling: To support a range of devices in 6G from low-end to high-end, HARQ feedback-based retransmission could be considered as an optional feature from day one. One use case other than NTN is to support URLLC service, where HARQ feedback-based retransmission can be disabled, instead transmission repetition/duplication could be utilized to achieve reliability and short latency.

- RACH-less handover: RACH-less handover was agreed for NTN then generalized to TN, IAB etc. already

- Time-/Location-based idle and connected mode mobility mechanism: In connected mode, events for handover condition and measurement report condition should be unified designed to support signal strength-based event, time-based event, location-based event and hight-based event as for UAV.  Ultimately, it is up to network to configure which one UE should use. We shall not limit any event specifically to a scenario like NTN. For example, apart from NTN, time-based event could be used for cell switching off case. Location-based event can be used for load control in certain geo-area. Same for idle mode mobility, time and location-based measurement initiation could be applicable to TN as well.

- Adaptation of protocol parameters/timers/variables/SMTC and gap configuration for long RTT: a common design only needs configurable parameters—such as ra-ResponseWindow, t-Rassembly, or K value—to support both short and very long RTT scenarios.

- Multiple TAC(s) broadcast and update: The system design should allow for both single and multiple TAC broadcasts and updates, depending on the network's requirements.

Functions is NTN special:

- Inter-RAT, inter TN-NTN, NTN-NTN idle and connected mode mobility: as long satellite info play an important role in NTN relevant mobility, satellite relevant assistance information cannot be skipped and this is different from intra-TN mobility

- positioning based on multi-RTT measurement with single RTP: positioning based multi-RTT from signal RTP works and only necessary for satellites scenario. For TN network, legacy multi-RTT positioning shall work better

Proposal 6: following functions could be considered with common design for TN and NTN:

- HARQ feedback disabling

- RACH-less handover

- Time-/Location-based idle and connected mode mobility mechanism

- Adaptation of protocol parameters/timers/variables/SMTC and gap configuration to long RTT

- One or multiple TAC(s) broadcast and update

# 3. Conclusion

In this contribution we discussed the essential/vital functions required for 6G NTN day1 using IOT NTN and NR NTN functions specified from Rel17 to Rel19 as baseline, other functions could be delayed to later releases. Moreover, based on our initial analysis, most of the functions can be commonly designed for TN and NTN. The most controversial aspects may be 1) TA and frequency self-compensation/self-evaluation for uplink transmission and 2) the dependence on GNSS capability. These will define how much TN/NTN are "harmonized" or "integrated". In details we have following proposals:

Proposal 1: RAN2 support the following minimum essential functions for NTN in 6G Day 1

- UE pre-compensation for uplink synchronization as well as TA report

- Adaptation of protocol parameters/timers/variables/SMTC and gap configuration to long RTT

- Multiple TAC(s) broadcast and update

- Time-based and Location-based (idle and connected mode) mobility

- support inter-RAT, inter TN-NTN, NTN-NTN idle and connected mode mobility

Proposal 2: RAN2 consider following functions are vital and make best effort to support for NTN in 6G day 1

- HARQ feedback disabling and blind retransmission scheduling

- Uplink and downlink coverage enhancements for all channels including common PDCCH/PDCH, DMRS and PUCCH

- Satellite switch with re-sync

- positioning based on multi-RTT measurement with single RTP

Proposal 3: RAN2 assume a unified design on TA management for TN/NTN could include the following aspects, which should be confirmed by coordination with RAN1

- Enhanced PRACH formats and/or preamble sequences which could be used for different cell sizes, including the NTN cell

- An enhanced closed-loop synchronization with a wider error correction range

- A pre-compensation framework applicable to both TN/NTN

Proposal 4: RAN2 study a common design of a group UE switch over between intra-6G NB cells for satellite switch over, NES cell switching off etc.

Proposal 5: RAN2 assume that coverage enhancement should be considered in 6G day1 to cover different use cases including NTN and wait for RAN1 progress at first.

Proposal 6: following functions could be considered with common design for TN and NTN:

- HARQ feedback disabling

- RACH-less handover

- Time-/Location-based idle and connected mode mobility mechanism

- Adaptation of protocol parameters/timers/variables/SMTC and gap configuration to long RTT

- One or multiple TAC(s) broadcast and update

# 4. References

[1] RAN2#131bis Chair notes