3GPP TSG-RAN-WG2 Meeting #132	R2-2508607

# Dallas, USA, Nov 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2147 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on stored RSRP update for positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18, the UE autonomous TA adjustment for the positioning SRS in the RRC inactive state was introduced. In TS 38.331, the RRC layer indicates the lower layer to update the TA and stored RSRP when the TA needs to be updated. However, according to the TS 38.213, the UE physical layer autonomously updates NTA at cell reselection according to the RAN4 requirements, not based on RRC layer indication. There is a misalignment between the higher layer and the physical layer regarding UE autonomous TA adjustment for positioning SRS. Considering that the physical layer measurements/parameters are used to define the RAN4 requirements, the physical layer is more feasible to determine when to update the timing advance. Therefore, the RRC layer is not feasible to indicate the MAC layer to update the stored RSRP since the RRC layer doesn’t know when the TA is updated and the MAC layer should update the stored RSRP according to the physical layer indicatiion. Impact analysis Impacted 5G architecture options: NR SA Impact analysis Impacted functionality: RRC inactive UE positioning Inter-operability: - No inter-operability issue is foreseen. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The MAC layer update the sotred RSRP for the positioning according to the physical layer indicaiton. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TA update and the stored RSRP update may occur at different occasion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.26.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.331 CR 5589 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

5	MAC procedures

5.26	Positioning SRS transmission in RRC_INACTIVE

5.26.1	General

Periodic and semi-persistent Positioning SRS with or without positioning SRS bandwidth aggregation can be configured for Positioning SRS transmission in RRC_INACTIVE.

SRS for positioning Tx frequency hopping can also be configured for Positioning SRS transmission in RRC_INACTIVE.

The MAC entity shall,

1>	if the TA of the configured Positioning SRS is valid according to clause 5.26.2, and the conditions for positioning SRS transmission in clause 7.3.1 of TS 38.213 [6] and clause 6.2.1.4 of TS 38.214 [7] are satisfied:

2>	instruct to the lower layer according to TS 38.214 [7] to transmit Periodic or Semi-Persistent Positioning SRS that is activated according to clause 5.18.17 or clause 5.18.37.

5.26.2	TA validation for SRS transmission in RRC_INACTIVE

RRC configures the following parameters for validation for SRS transmission in RRC_INACTIVE:

-	inactivePosSRS-RSRP-ChangeThreshold: RSRP threshold for the increase/decrease of RSRP for time alignment validation;

-	inactivePosSRS-ValidityAreaRSRP: RSRP threshold for the increase/decrease of RSRP for time alignment validation when SRS positioning validity area is configured.

The MAC entity shall:

1>	if the UE receives configuration for SRS transmission in RRC_INACTIVE:

2>	store the RSRP of the downlink pathloss reference with the current RSRP value of the downlink pathloss reference as in TS 38.331 [5].

1>	else if the UE is configured with SRS transmission in RRC_INACTIVE:

2>	if Timing Advance Command MAC CE is received as in clause 5.2, or;

2>	if Timing Advance Command or Absolute Timing Advance Command is received for Random Access procedure that is successfully completed:

3>	update the stored the RSRP of the downlink pathloss reference with the current RSRP value of the downlink pathloss reference.

2>	if the UE is configured Positioning with SRS with validity area and the Physical layer indicates the MAC to update the stored RSRP:

3>	update the RSRP of the downlink pathloss reference with the current RSRP value of the downlink pathloss reference of the camped cell as specified in TS 38.331 [5].

The MAC entity shall consider the TA to be valid when the following conditions are fulfilled:

1>	compared to the stored downlink pathloss reference RSRP value, the current RSRP value of the downlink pathloss reference of the camped cell as specified in TS 38.331 [5] has not increased/decreased by more than inactivePosSRS-RSRP-ChangeThreshold, if configured, or inactivePosSRS-ValidityAreaRSRP, if configured, when SRS positioning validity area is configured; and

- inactivePosSRS-TimeAlignmentTimer is running or inactivePosSRS-ValidityAreaTAT is running when SRS positioning validity area is configured.

End of change