3GPP TSG RAN WG1 #132	R2-2508675

Dallas, USA,  Nov. 17th - 21st 2025

Agenda Item:	8.5.2

Source:	Ericsson

Title:	Maintenance for R19 NES

Document for:	Discussion

# 1	Introduction

This contribution presents some maintenance issues related to Rel-19 NES functionality.

# 2 	On-demand SSB SCell operation

## 2.1 Impact of od-ssb-PositionsInBurst on ssb-ToMeasure

In RAN2#131bis meeting it was discussed whether UE needs exact value on the measurement configuration for the ssb-ToMeasure or not. Currently MO configures one value ssb-ToMeasure but the OD-SSB that is activated has a parameter od-ssb-PositionsInBurst which may change as per the MAC CE. The topic was postponed:

R2-2507761	Report of [AT131bis][301][NES] RRC open issues	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1	Discussion on the impact of od-ssb-PositionsInBurst on ssb-ToMeasure is postponed to next meeting.

Discussion on the impact of od-ssb-PositionsInBurst on ssb-ToMeasure is postponed to next meeting

The parameter ssb-ToMeasure is used in TS 38.133 in multiple places and we give two examples here:

8.1.2	Requirements for SSB based radio link monitoring

...

-	Psharing factor = 1, if the RLM-RS resource outside GAP is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

9.2.5.3 Scheduling availability of UE during intra-frequency measurements UE shall be capable of measuring without measurement gaps when the SSB is completely contained in the active bandwidth part of the UE, or the UE indicates no-gap-no-interruption for intra-frequency measurement, or the UE indicates no-gap-with-interruption for intra-frequency measurement. When any of the conditions in the following clauses is met, there are restrictions on the scheduling availability; otherwise, there is no scheduling restriction. Note that the SSB symbols indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged [2], if it is configured; otherwise, all L SSB symbols within the SMTC window duration defined in clause 4.1 of TS 38.213 [3] are included.

Since RAN4 is adopting a union of ssb-toMeasure per carrier, it seems ok to configure a union of od-ssb-PositionsInBurst as ssb-ToMeasure in the MO that is considered for OD-SSB. Thsi needs to be captured in the field description of the SSB-ToMeasure field in the MO.

RAN2 to agree that union of od-ssb-PositionsInBurst is configured as ssb-ToMeasure in the MO that is considered for OD-SSB and captured this in the field description of the SSB-ToMeasure field in the MO.

## 2.2 Remaining field description issues and RIL O009

It can be seen that there are some controversy in the chair notes related to assuming servingcellMO for case1:

RAN2 assumes the following table describes how the CSI-RS based serving cell measurements would work when OD-SSB is configured

| Scenario | servingCellMO contains | servingCellMO-OD contains | What is measured (OD-SSB not activated) for serving cell | What is measured (OD-SSB activated) for serving cell |
| --- | --- | --- | --- | --- |
| AO-SSB and OD-SSB both configured and on same frequency | AO-SSB CSI-RS OD-SSB on same freq |  | AO-SSB, CSI-RS | OD-SSB, CSI-RS |
| AO-SSB and OD-SSB both configured and on different frequency | AO-SSB CSI-RS | OD-SSB CSI-RS2 | AO-SSB, CSI-RS | OD-SSB, CSI-RS2 |
| Only OD-SSB configured | CSI-RS | OD-SSB CSI-RS2 | CSI-RS | OD-SSB, CSI-RS2 |
| Only OD-SSB configured |  | OD-SSB CSI-RS2 |  | OD-SSB, CSI-RS2 |

The above table that was concluded first has CSI-RS measurements when OD-SSB is not activated for Case1 while the below agreements that were done later do not state servingcellMO for case1:

RAN2 confirms that legacy servingCellMO is used for deriving serving cell measurement results in the following cases:

- - if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is not configured.

- - if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is not configured with od-ssb-absoluteFrequency

- - if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is configured with od-ssb-absoluteFrequency and od-ssb transmission is not activated

RAN2 confirms that servingCellMO-OD is used for deriving serving cell measurement results in the following cases:

- - if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is configured with od-ssb-absoluteFrequency and OD-SSB transmission is activated.

- - if absoluteFrequencySSB is not configured in ServingCellConfigCommon and od-ssb is configured and od-ssb is activated.

Since the servingCellMO seems to be optionally configured for SCells in most cases, it may be ok to assume it can be optionally configured also for Case1. The intention of the addition “or if OD-SSB I configured” under the case “For SSB-less SCells,..” is to deviate from the requirement of the specific capability. Since it is under the case “For SSB-less SCells,..” it should not impact case2a/b. However, for clarity reasons, it may be good to list OD-SSB as a separate case.

RAN2 to Confirm the assumption of servingcellMO for Case 1, choose between

  - servingcellMO is not configured for case1

  - servingcellMO is optionally configured for case1

The conditional presence of servingCellMO-OD needs to be updated to the latest agreement to be able to use it for case1. Also the name of the cond could be updated to consider also case1.

servingCellMO-OD-r19                MeasObjectId                                                         OPTIONAL, -- Cond Diff_Freq

servingCellMO-OD

measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell instead of servingCellMO in IE ServingCellConfig.

| Conditional Presence | Explanation |
| --- | --- |
| 2TA-TDD-Only | The field is optionally present, Need N, for a TDD cell, in the mimoParam-v1850 if additionalPCI-ToAddModList is present in ServingCellConfig and if tag2 is present in ServingCellConfig. It is absent otherwise. |
| AsyncCA | This field is mandatory present for SCells whose slot offset between the SpCell is not 0. Otherwise it is absent, Need S. |
| MeasObject | This field is mandatory present for the SpCell if the UE has a measConfig, and it is optionally present, Need M, for SCells. For (e)RedCap UEs, this field is optionally present, Need M. For SSB-less SCell(s), this field is not present if intraF-NeighMeasForSCellWithoutSSB is not supported by the UE, otherwise this field is optionally present, Need M. |
| SCellOnly | This field is optionally present, Need R, for SCells. It is absent otherwise. |
| ServingCellWithoutPUCCH | This field is optionally present, Need S, for SCells except PUCCH SCells. It is absent otherwise. |
| SyncAndCellAdd | This field is mandatory present for a SpCell upon reconfiguration with reconfigurationWithSync and upon RRCSetup/RRCResume. The field is optionally present for an SpCell, Need N, upon reconfiguration without reconfigurationWithSync. The field is mandatory present for an SCell upon addition, and absent for SCell in other cases, Need M. |
| TCI_ActivatedConfig | This field is optional Need N for SCells if sCellState is configured, otherwise it is absent. This field is optional Need S for the PSCell when the SCG is indicated as deactivated or is being activated, otherwise it is absent. This field is absent for the PCell. |
| TDD | This field is optionally present, Need R, for TDD cells. It is absent otherwise. |
| TDD_IAB | For IAB-MT, this field is optionally present, Need R, for TDD cells. It is absent otherwise. |
| TypeDCI0-3 | This field is mandatory present if ScheduledCellListDCI-0-3 is configured, otherwise it is absent, Need R. |
| TypeDCI1-3 | This field is mandatory present if ScheduledCellListDCI-1-3 is configured, otherwise it is absent, Need R. |
| Diff_Freq | This field is mandatory present if od-ssb-absoluteFrequency indicates different frequency than absoluteFrequencySSB of the serving cell. It is absent otherwise. |

RAN2 to rename cond Diff_Freq as MeasObjectOD and adopt the below TP

This field is mandatory present if od-ssb-absoluteFrequency indicates different frequency than absoluteFrequencySSB of the serving cell or if absoluteFrequencySSB is absent and OD-SSB is configured in IE CellGroupConfig. It is absent otherwise.

The below RIL is concerning the same issue in the chairnotes:

## O009

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O009 | NES | 1 | Condition for the presence of servingcellMO | R2-25xxxx | Qianxi Lu |  | V025 | ToDo |

[Description]: The following R2 conclusion

servingcellMO is not configured for OD-SSB measurement in Case1 SCell and OD-SSB specific MO is used for both CSI-RS measurements and OD-SSB measurements after OD-SSB is activated.

However, due to the following R2 agreement, apparently the first part above is wrong

RAN2 assumes the following table describes how the CSI-RS based serving cell measurements would work when OD-SSB is configured

I.e., R2#131bis made two agreements which are colliding with each other, and cause problem to CR work in the post R2 meeting discussion.

So R2 should revise the agreement to make it more accurate.

[Proposed Change]: E.g., if the table considering CSI-RS measurement is to the one to follow

| Conditional Presence | Explanation |
| --- | --- |
| MeasObject | This field is mandatory present for the SpCell if the UE has a measConfig, and it is optionally present, Need M, for SCells. For (e)RedCap UEs, this field is optionally present, Need M. For SSB-less SCell(s), this field is not present if intraF-NeighMeasForSCellWithoutSSB is not supported by the UE and if OD-SSB is not configured in IE SCellConfig, otherwise this field is optionally present, Need M. |

[Comments]:

RAN2 to first agree if measObject is configured for case1 and then address RIL O009

3	SSB adaptation activation and RIL [H130]

In RAN2#131bis meeting, there was a discussion on RIL [H130]:

# H130

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H130 | NES | 1 | SSB adaptation for deactivated SCell | R2-25xxxxx | Huawei (Lili) |  | V034 |  |

[Description]:

According to 38.321, when an SCell is deactivated, the MAC entity shall not monitor PDCCH on the SCell or for the SCell:

1> if the SCell is deactivated: 2> not transmit SRS on the SCell; 2> not report CSI for the SCell; 2> not transmit on UL-SCH on the SCell; 2> not transmit on RACH on the SCell; 2> not monitor the PDCCH on the SCell; 2> not monitor the PDCCH for the SCell; 2> not transmit PUCCH on the SCell;

The above PDCCH monitoring behaviour inherits from LTE R10. With this restriction, the SSB adaptation indicated by DCI cannot apply to deactivated SCell. Even though we think dynamically changing the SSB periodicity to a smaller value for fast SCell activation is a valid use case, we prefer not to change the PDCCH monitoring behaviour or to revert RAN2 previous agreement of not introducing new MAC CE. This fast SCell activation can be enabled by OD-SSB operation. Under OD-SSB discussion, it is allowed to use the OD-SSB MAC CE to dynamically change the OD-SSB configuration including SSB periodicity. Alternatively, the network can use RRC reconfiguration to update the SSB periodicity of a deactivated SCell.

[Proposed Change]:

[[

od-ssb-r19                      SetupRelease {OD-SSB-r19}                                       OPTIONAL,   -- Need M

adap-SSB-Config-r19             SetupRelease {Adap-SSB-Config-r19}                              OPTIONAL    -- Cond SSB-AdaptM

]]

| Conditional Presence | Explanation |
| --- | --- |
| SCellAdd | The field is mandatory present upon SCell addition; otherwise it is absent, Need M. |
| SCellAddMod | The field is mandatory present upon SCell addition; otherwise it is optionally present, need M. |
| SCellAddSync | The field is optionally present, Need N: - in the masterCellGroup at - SCell addition, - reconfiguration with sync, - resume of an RRC connection. - in the secondaryCellGroup, when the SCG is not indicated as deactivated at: - SCG activation while the SCG was previously deactivated, - SCell addition, - reconfiguration with sync. It is absent otherwise. |
| SCG | The field is mandatory present in an SpCellConfig for the PSCell. It is absent otherwise. |
| SCellSIB20-Opt | This field is optionally present, Need M, if the field sCellSIB20 is configured. It is absent otherwise. |
| SCG-Opt | The field is optionally present, Need M, in an SpCellConfig for the PSCell. It is absent otherwise. |
| SSB-Adapt | The field is optionally present, Need M, for an activated SCell. It is absent otherwise. |

In the RIL above, the related text in 38.321 is interpreted as when an SCell is deactivated, the MAC entity shall not monitor PDCCH for the SCell. Therefore, it was assumed that SSB adaptation indicated by DCI format 2_9 cannot apply to deactivated SCell. However, this DCI is for group common signaling, so it should be possible for a UE to receive DCI 2_9 from the Pcell for SSB adaptation for deactivated Scell(s). This is also the intention for this Rel-19 feature.

RAN2 to confirm that it is possible for a UE to receive DCI format 2_9 from the Pcell for SSB adaptation for deactivated Scell(s).

RAN2 to clarify the confirmation above in the specifications.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to agree that union of od-ssb-PositionsInBurst is configured as ssb-ToMeasure in the MO that is considered for OD-SSB and captured this in the field description of the SSB-ToMeasure field in the MO.

Proposal 2	RAN2 to Confirm the assumption of servingcellMO for Case 1, choose between

a.	servingcellMO is not configured for case1

b.	servingcellMO is optionally configured for case1

Proposal 3	RAN2 to rename cond Diff_Freq as MeasObjectOD and adopt the below TP

Proposal 4	RAN2 to first agree if measObject is configured for case1 and then address RIL O009

Proposal 5	RAN2 to confirm that it is possible for a UE to receive DCI format 2_9 from the Pcell for SSB adaptation for deactivated Scell(s).

Proposal 6	RAN2 to clarify the confirmation above in the specifications.

# References

RP-252332, “Revised WID on enhancements of network energy savings for NR”, 3GPP TSG RAN Meeting #109, Beijing, PR China, Sep 2025

TS 38.213-j10