3GPP TSG-RAN WG2 Meeting #132	R2-2508692

# Dallas, USA, 17th – 21st Nov, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1385 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Per band and Per band combination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO, NR_FeMIMO, NR_MIMO_evo_DL_UL, NR_AIML_air, NR_MIMO_Ph5, |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Regarding the interpretation on ‘per band and per band combination’, RAN1 provided below feedback in R1-2507981: - When UE indicates both per band and per BC capability, if the capability/component is not counted across CCs, then the minimum capability between per BC capability and per band capability should be applied for a band in case of band combination (CA) - RAN1 will continue to discuss the cases - When the capability/component is counted across CCs regardless of whether CA is configured or not - When the capability/component is not counted across CCs and CA is not configured - RAN1 will review by RAN1 #123 for which FGs special rules/handling may need to be defined in Rel. 19, e.g., in some cases a clear “ranking” between capabilities may need to be clarified in the Notes - For Case 2 and Case 3, RAN2’s understanding is generally correct - For per band and per BC capabilities, it is expected that UE indicates the same granularity as the pre-requisite. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce the definition of ‘per band and per band combination’ granularity. - Introduce the general principle for final capability and prerequisite for capability with granularity of ‘per band and per band combination’. Impact Analysis Impacted 5G architecture options: NR SA, NR-DC Impacted functionality: MIMO, AI/ML air Inter-operability: If the network is implemented according to this CR and UE is not, UE will not be able to signal all the capability correctly and network may assume that UE does not support them and may not configure them. If UE implementation is not aligned with the additional clarifications, it can cause wrong intrepration of the capability signalling resulting in configuration failure of the features. If the UE is implemented according to the CR and network is not, network will not be aware of the UE capability and may not configure the UE for these supported features. If network implementation is not aligned with the additional clarifications, it can cause wrong intrepration of the capability signalling resulting in configuration failure of the features. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Final capability and prerequisite of features with ‘per band and per band combination’ is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

eRedCap UE: a UE with enhanced reduced capabilities as specified in clause 4.2.22.1.

Fallback band combination: A Uu band combination that would result from another Uu band combination (parent band combination) by releasing at least one SCell or uplink configuration of SCell, or SCG, or SUL. A PC5 band combination that would result from another PC5 band combination (parent band combination) by releasing at least one sidelink carrier. An intra-band non-contiguous band combination is not considered to be a fallback band combination of an intra-band contiguous band combination. A fallback band combination supports the same channel bandwidth(s) for each carrier as its parent band combination(s).

Fallback per band feature set: A feature set per band that has same or lower capabilities than the reported capabilities from the reported feature set per band for a given band.

Fallback per CC feature set: A feature set per CC that has same or lower capabilities than the capabilities of UE (e.g. supported MIMO layers, BW, modulation order) while keeping the numerology the same from the reported feature set per CC for a given carrier per band. The supportedMinBandwidthDL/supportedMinBandwidthUL defines the lower bound of the bandwidth supported by the UE.

Per band and per band combination: A UE capability parameter defined in both per band and per BC with same feature components.

RedCap UE: The UE with reduced capabilities as specified in clause 4.2.21.1.

SON report(s): A SON report corresponds to one report from UE such as Random Access report, Radio Link Failure report, Connection Establishment Failure report, Mobility History Information report, Successful Handover report, and Successful PSCell change report.

Switching SCell (sSCell): The SCell configured with cross-carrier scheduling to PCell/PSCell.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

MaxDLDataRate:	Maximum DL data rate

MaxDLDataRate_MN:	Maximum DL data rate in the MN

MaxDLDataRate_SN:	Maximum DL data rate in the SN

MaxULDataRate:	Maximum UL data rate

MaxSLtxDataRate:	Maximum SL data rate in transmission

MaxSLrxDataRate:	Maximum SL data rate in reception

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A-CSI	Aperiodic-CSI

AI/ML	Artificial Intelligence/Machine Learning

ATG	Air To Ground

BAP	Backhaul Adaptation Protocol

BC	Band Combination

BPS	Body Proximity Sensing

BT	Bluetooth

CCS	Cross Carrier Scheduling

CLTM	Conditional L1/L2 Triggered Mobility

CMR	Channel Measurement Resource

CPAC	Conditional PSCell Addition/Change

DAPS	Dual Active Protocol Stack

DL	Downlink

DSR	Delay Status Report

EHC	Ethernet Header Compression

FS	Feature Set

FSPC	Feature Set Per Component-carrier

FWA	Fixed Wireless Access

GSO	Geosynchronous Orbit

HSDN	High Speed Dedicated Network

IAB-MT	Integrated Access Backhaul Mobile Termination

IDC	In-Device Coexistence

MAC	Medium Access Control

MHI	Mobility History Information

MBS	Multicast/Broadcast Service

MCG	Master Cell Group

MN	Master Node

MO-SDT	Mobile Originated Small Data Transmission

MRB	MBS Radio Bearer

MR-DC	Multi-Radio Dual Connectivity

MSD	Maximum Sensitivity Degradation

MT-SDT	Mobile Terminated Small Data Transmission

mTRP	Multiple TRP

MUSIM	Multi-Universal Subscriber Identity Module

NCJT	Non-Coherent Joint Transmission

NCR	Network Controlled Repeater

NCR-MT	NCR Mobile Termination

NCSG	Network Controlled Small Gap

NES	Network Energy Savings

NGSO	Non-Geosynchronous Orbit

NTN	Non-Terrestrial Network

P-CSI	Periodic CSI

PDCP	Packet Data Convergence Protocol

PSI	PDU Set Importance

QoE	Quality of Experience

RLC	Radio Link Control

RTT	Round Trip Time

SCG	Secondary Cell Group

SDAP	Service Data Adaptation Protocol

SDL	Supplementary Downlink

SN	Secondary Node

sTRP	Serving TRP

SUL	Supplementary Uplink

TN	Terrestrial Network

TRP	Transmit/Receive Point

UDC	Uplink Data Compression

UL	Uplink

VSAT	Very Small Aperture Terminal

WLAN	Wireless Local Area Network

XR	eXtended Reality

2nd CHANGES

# 4	UE radio access capability parameters

## 4.2	UE Capability Parameters

### 4.2.1	Introduction

The following clauses define the UE radio access capability parameters. Only parameters for which there is the possibility for UEs to signal different values are considered as UE radio access capability parameters. Therefore, mandatory features without capability parameters that are the same for all UEs are not listed here.

The network needs to respect the signalled UE radio access capability parameters when configuring the UE and when scheduling the UE.

For capabilities that required to be set consistently for all FDD-FR1 bands (i.e. capabilities that are supposed to be per UE), the UE shall also set capability values for all SUL bands with same values for FDD-FR1 bands if SUL band is supported by the UE.

The UE may support different functionalities between FDD and TDD, and/or between FR1 and FR2. The UE shall indicate the UE capabilities as follows. In the table of UE capability parameter in subsequent clauses, "Yes" in the column by "FDD-TDD DIFF" and "FR1-FR2 DIFF" indicates the UE capability field can have a different value for between FDD and TDD or between FR1 and FR2 and "No" indicates if it cannot. "(Incl FR2-2 DIFF)" in the column by "FR1-FR2 DIFF" indicates the UE capability field can have a different value for between FR2-1 and FR2-2. Regarding to the per UE capabilities that are FDD/TDD differentiated(i.e. capabilities indicated as "Yes" in the column by "FDD-TDD DIFF"), the corresponding capabilities indicated by the FDD capability is applied to SUL/SDL if SUL/SDL band is supported by the UE. "FD" in the column indicates to refer the associated field description. "FR1 only" or "FR2 only" in the column indicates the associated feature is only supported in FR1 or FR2 and "TDD only" indicates the associated feature is only supported in TDD and not applicable to SUL/SDL carriers. "N/A" in the column indicates it is not applicable to the feature (e,g. the signalling supports the UE to have different values between FDD and TDD or between FR1 and FR2).

1>	set all fields of UE-NR/MRDC-Capability except fdd-Add-UE-NR/MRDC/Sidelink-Capabilities, tdd-Add-UE-NR/MRDC/Sidelink-Capabilities, fr1-Add-UE-NR/MRDC-Capabilities and fr2-Add-UE-NR/MRDC-Capabilities, to include the values applicable for all duplex mode(s) and frequency range(s) that the UE supports;

1>	if UE supports both FDD (or SUL/SDL) and TDD and if (some of) the UE capability fields have a different value for FDD (or SUL/SDL) and TDD:

2>	if for FDD (and, if the UE supports SUL/SDL, for SUL/SDL), the UE supports additional functionality compared to what is indicated by the previous fields of UE-NR/MRDC-Capability/SidelinkParameters:

3>	include field fdd-Add-UE-NR/MRDC/Sidelink-Capabilities and set it to include fields reflecting the additional functionality applicable for FDD;

2>	if for TDD, the UE supports additional functionality compared to what is indicated by the previous fields of UE-NR/MRDC-Capability/SidelinkParameters:

3>	include field tdd-Add-UE-NR/MRDC/Sidelink-Capabilities and set it to include fields reflecting the additional functionality applicable for TDD;

1>	if UE supports both FR1 and FR2 and if (some of) the UE capability fields have a different value for FR1 and FR2:

2>	if for FR1, the UE supports additional functionality compared to what is indicated by the previous fields of UE-NR/MRDC-Capability:

3>	include field fr1-Add-UE-NR/MRDC-Capabilities and set it to include fields reflecting the additional functionality applicable for FR1;

2>	if for FR2, the UE supports additional functionality compared to what is indicated by the previous fields of UE-NR/MRDC-Capability:

3>	include field fr2-Add-UE-NR/MRDC-Capabilities and set it to include fields reflecting the additional functionality applicable for FR2;

NOTE 1:	The fields which indicate "shall be set to 1" or "shall be set to supported" in the following tables means these features are purely mandatory and are assumed they are the same as mandatory without capability signalling.

NOTE 2:	For the case where the UE is allowed to support different functionality between FDD and TDD and between FR1 and FR2 according to the specification, the UE capability indication is clarified in Annex B.

NOTE 2a:	In this release of the specification, if the UE is allowed to support different functionalities between FDD and TDD, and/or between FR1 and FR2, these functionalities are signalled per band with the text "UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively".

For optional features, the UE radio access capability parameter indicates whether the feature has been implemented and successfully tested. For mandatory features with the UE radio access capability parameter, the parameter indicates whether the feature has been successfully tested. In the table of UE capability parameter in subsequent clauses, "Yes" in the column by "M" indicates the associated feature is mandatory and "No" indicates the associated feature is optional. "CY" in the column indicates the associated feature is conditional mandatory and the condition is described in the field description and the associated feature is considered mandatory with capability parameter, when the described condition is satisfied. "FD" in the column indicates to refer the associated field description. Some parameters in subsequent clauses are not related to UE features and in the case, "N/A" is indicated in the column.

UE capability parameters have hierarchical structure. In the table of UE capability parameter in subsequent clauses, "Per" indicates the level the associated parameter is included. "UE" in the column indicates the associated parameter is signalled per UE, "Band" indicates it is signalled per band, "BC" indicates it is signalled per band combination, "FS" indicates it is signalled per feature set (per band per band combination), "FSPC" indicates it is signalled per feature set per component carrier (per CC per band per band combination), and "FD" in the column indicates to refer the associated field description.

NOTE 3:	Unless otherwise specified, for dependent capabilities with prerequisite capability in a finer granularity, the UE should indicate support of the prerequisite capability in at least one finer granularity. And the dependent capability is supported only in the finer granularity where the prerequisite capability is supported, e.g. a UE indicating support of supportNewDMRS-Port-r16 (dependent capability which is defined per band) should indicate at least one band combination where singleDCI-SDM-scheme-r16 (prerequisite capability which is defined per feature set) is supported in the corresponding band. In this case, supportNewDMRS-Port-r16 is considered supported only in the corresponding band of the band combination where singleDCI-SDM-scheme-r16 is supported.

NOTE X: For a capability with same component signalled "per band" and "per BC":

If the capability/comprised parameter(s) is not counted across CCs, the minimum capability between "per BC" and "per band" should be applied for a band in case of band combination (CA).

When "per BC" is signalled but some subset of the bands in the BC doesn’t signal "per band" capability, the capability/comprised parameter(s) is not supported in the band without "per band" capability, the capability/comprised parameter(s) is supported only in the band with "per band" capability signalled.

When the UE signals "per band" but does not include "per BC" for a certain BC, the UE supports the capability/comprised parameter(s) as indicated in the "per band" without further per BC limitations.

For "per band and per band combination" capabilities with prerequisite capability in "per band and per band combination", the UE should indicate support of the prerequisite capability in the corresponding band/BC, respectively.

2nd CHANGES