3GPP TSG-RAN WG2#132	R2-2508947

Dallas, USA, Nov 17th – 21st 2025

Source: 	Fujitsu

Agenda item:	10.3.2.3

Title:	Discussion on Initial and System Access in 6G

Document for:	Discussion and decision

# Introduction

The SID of Rel-20 Study on 6G Radio was approved in RAN#108 and updated in RAN#109 [1].

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

At the RAN2#131bis meeting in Prague following agreements related to system information were achieved [2]:

Agreements on System Information

-	System information design should consider energy efficiency and low complexity for both network and UE.

-	System information design should ensure that the public warning requirements are met.

-	Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

-	Study how to improve flexibility and extensibility of scheduling SI design

-	Study mechanisms to improve SI update mechanism

-	Study aspects related to SIB1 size.  Understand the issues and desired content/size from RAN2 point of view.

-	Study areas specific SIB design.  Understand 5G pain points and what improvements can be considered.

NOTE: this is a starting list

In this contribution, we would like to provide our views on initial and system access aspects more particular about system information provisioning and single cell multiple carrier concept.

# Discussion

## Initial and System Access

In LTE/5G when the UE power on, then the first step is cell search which involves detecting the cell based on synchronization signal decoding, acquisition of the essential system information like MIB and SIB1, camping on the cell based on the cell selection criteria using the cell access information in SIB1 and then transitioning to the connected state using the random access procedure applying the RACH and PRACH information included in the SIB1. We assume the initial access by a UE in a 6G system would be like that in the 4G and 5G system as shown in Figure 1.

Figure 1 Initial Access

The cell search procedure involves detecting the synchronization signal which is RAN1 scope. After detecting the cell acquiring the essential system information is RAN2 scope. In the next section we summarize the system information provisioning in 4G LTE and 5G NR.

### System information

LTE System Information (SI) provisioning is based on periodic broadcast of all SI. 5G NR from Day 1 introduced on-demand system information provisioning for Other System Information (i.e. SIB2, SIB3 etc) catering to the energy efficiency and signalling overhead requirement. However, the minimum system information (i.e. MIB and SIB1) were always periodically broadcasted up to Rel-18 as shown in Figure 2. In Rel-19, the mechanism to provide SIB1 on-demand in a NES cell was introduced based on SIB1 Request Config information received by the UE from an anchor cell. 6GR system information provisioning design shall cater to the network energy saving requirement such that system information can be provisioned in on-demand manner based on UE request in the standalone scenario with and without support of the anchor cell as proposed in several company contributions submitted to RAN2#131bis meeting [3-6].

Figure 2 Evolution of SI provisioning in 5G NR

In summary we propose the 6GR system information provisioning needs to consider the following:

Proposal#1

- 6GR system information can be provisioned in on-demand manner based on the UE request in the standalone scenario with and without support of the anchor cell.

- The concept of periodic non-overlapping staggered SI windows and the concept of BCCH modification period shall be revisited in study item. 6GR system information broadcast shall explore alternate concepts to the existing ones to be used for SI broadcasting and SI change during the study phase.

- 6GR study shall strive to simplify the on-demand system information request procedure to save the UE power, the network energy and the radio resources consumption for provisioning SI in on-demand manner.

### MIB

We expect there will be MIB in 6G system. We further assume that the MIB will be periodically broadcast on the PBCH. Based on the RAN1 study related to synchronization signals (i.e. SSB), if RAN1 decides that SSB periodic transmission in time domain is adapted in the standalone single cell system under certain conditions then the MIB broadcast can also be adapted along with SSB. The contents of the MIB will be decided by RAN2 in consultation with RAN1 as the MIB payload is expected to be limited, similar to LTE and NR. Therefore, the contents of the MIB should be judiciously decided considering the most essential information. It is a bit premature to discuss the contents of the MIB without having agreements on the supported scenarios like standalone cell meeting the energy saving requirement.

Observation#1: RAN2 assume MIB will be periodically broadcast on PBCH. If SSB periodicity is adapted, then MIB periodicity is also adapted.

Observation#2: Contents of MIB partly depends on the scenarios to be supported for NES cell.

### SIB1

On-demand SIB1 as shown in Figure 2 is introduced in Rel-19 for 5G NR. Further, UE acquires SIB1 request configuration for NES cell from another cell. The on-demand SIB1 mechanism of 5G NR Rel-19 may not get deployed as the scope is limited and hence may not be considered as baseline for 6GR. The SIB1 is defined as cell-specific in 5G NR. However, in real practical deployments majority of SIB1 contents like cell access information, RACH/PRACH config etc is same for cells operating on the same frequency. Random access procedure optimization in real deployments is achieved by configuring preamble root sequence index and PRACH config for load balancing such that intra-frequency cells that satisfy the MOD(PCI, N) = N-1 have the same parameters. One way to keep the SIB1 contents same for RACH config and PRACH config would be to include the N-1 configurations as a list in SIB1. This may increase the size of SIB1 marginally and can be further studied during Stage-3. The UEs in the respective intra-frequency cells choose the N-1th index to apply the right set of parameters. We therefore think there is no reason to keep the scope of SIB1 solely cell specific. Depending on the deployment scenario the SIB1 can be either cell-specific or can be area-specific. Alternatively, as proposed by some company contributions submitted to RAN2#131bis meeting the area-specific SIB1 contents can be grouped together, the cell-specific contents can be grouped separately and contents which is not required to be included in SIB1 can be moved to OSI [7-8].

If SIB1 becomes area-specific, then once the UE has acquired SIB1 it remains valid across several cells during UE cell re-selection and there is no need for the UE re-acquire SIB1 if the area validity check satisfies. This helps the UE to save battery and hence meet the design requirement on energy saving for UEs. From the network perspective regardless of the SIB1 is cell-specific or area-specific, if SIB1 is provided in on-demand manner whether the deployment is standalone or energy saving cell is supported by anchor cell, it helps to meet the network energy saving requirement.

Observation#3: In real deployment majority of SIB1 contents for at least intra-frequency cells are same and hence no need to restrict SIB1 scope as solely cell-specific.

Obervation#4: If SIB1 is area-specific then UE need not re-acquire SIB1 upon cell reselection where area scope of SIB1 is valid thus saving UE battery.

We therefore propose

Proposal#2: SIB1 in 6G can be specified as either cell-specific or area-specific depending on operator deployment scenario. RAN2 to further study what contents of SIB1 can be area-specific.

### Other SIBs broadcast in SI window

In LTE and 5G NR the periodic broadcast of other system information (OSI) is based on the concept of non-overlapping staggered SI windows which are of same size and the concept of BCCH modification period for the change in system information. The broadcast of OSI in the SI windows either periodic or on-demand in 5G NR is based on the scheduling information of the SIB/SI messages in SIB1. Several company contributions submitted to RAN2#131bis meeting argued that scheduling information is SIB1 increases the SIB1 size. Further when the contents of some SIB are changed, the short message SI change notification does not indicate which SIB has changed and UE needs to re-acquire the SIB1 even if contents of SIB1 are not changed. This is not desirable from UE battery consumption.

Obervation#5: SI change notification in 5G does not indicate which SIB has changed.

From network perspective the periodic OSI broadcast sometimes create scheduling restriction for user data scheduling especially during moderate to high cell traffic conditions. In the CU-DU distributed split RAN architecture deployment the minimum system information (i.e. MIB and SIB1) broadcasting is handled by the DU and the OSI broadcasting is handled by the CU. The concept of SI windows for OSI broadcasting requires coordination between the CU and DU on the F1 interface. Similar coordination on the F1 interface is required when SI change notification through paging/short message is sent to the UEs in the cell considering the BCCH modification period. Given that 6GR is expected to provision system information in on-demand manner, the concept of SI window shall be revisited considering the constraints it introduces for scheduling the user data when SI is broadcasted and unnecessary UE battery wastage for re-acquiring SIB1 when SI change notified. Alternate concepts to existing ones used for SI broadcasting and SI change shall be explored during the study item considering the energy saving requirement and alleviating the scheduling constraints. One approach to avoid scheduling info in SIB1 and hence the staggered SI window broadcast is to provision the OSI after the UE has transitioned to connected mode through on-demand unicast manner. Only essential SI like MIB and SIB1 is either periodically broadcast or SIB1 provided on-demand. Essential information acquired by UE is sufficient for Idle mode operation and transitioning to connected mode. OSI is provided on-demand in connected mode upon UE request as shown in Figure 3. OSI once acquired and if made area-specific is stored by UE and remains valid within the area scope during Idle mode mobility.

Figure 3 System Information Provisioning in 6G

One can argue that OSI also comprises cell re-selection information (i.e. SIB2, SIB3 etc) which is required during Idle mode mobility and for acquiring such information the UE is mandated to transition to connected mode is not desirable. On the other hand, providing all OSI through broadcast in SI windows is also not desirable. Therefore, it can be further studied what OSI is always provided in unicast and what OSI can be provided in broadcast in on-demand manner.

Obervation#6: Provisioning all OSI in connected mode through on-demand unicast avoids the need of periodic broadcast window.

Observation#7: If periodic broadcast SI windows are avoided then scheduling info for OSI is not needed to be included in SIB1, thus reducing SIB1 size and SIB1 re-acquisition during SI change.

We therefore propose

Proposal#3: RAN2 to consider more granular SI change indication indicating which SIB has changed, to avoid re-acquiring SIB1

Proposal#4: RAN2 to further study the feasibility of OSI provisioning in connected mode through on-demand unicast.

### Area scope of Other SIBs

In 5G NR the OSI can be either cell-specific or area-specific. This is indicated through the inclusion of the area scope identifier in SIB1. If the area scope identifier is not included, then the particular SIB/SI message is cell specific. In real 5G deployments the area-specific OSI is not implemented and hence by default the OSI is cell-specific. As such there is no technical constraint to implement the area-specific OSI (at least for some SIB/SI message) because the area scope identifier provides that flexibility. Based on floor discussions at RAN2#131bis on the area-specific SIBs, there seems some misunderstanding with some companies which think area-specific SIBs needs to co-ordinated in adjacent neighbour cells. This understanding is based on the logic that the area scope identifier gets mapped to a particular group of adjacent cells in a geographic area. This understanding in our view is not correct because the area scope identifier is agnostic to geographic area. Any groups cells within the operator network for which the OSI contents is same regardless of the geography of the cell can be configured with the same area identifier. We believe the scope of area identifier scope for OSI was discussed at depth during the 5G study item phase and there seem no shortcoming from a technical point of view. It is more to do with network vendor and operator willingness to plan the area-scope for the OSI in commercial deployment. Apart from the area scope identifier the SIB also has a valuetag referring to version of the SIB. For area validity, the UE needs to check the area scope identifier and valuetag of the stored SIB with the area scope identifier and valuetag included in SIB1 of the reselected cell. If there is match UE need not acquire the corresponding SIB else UE needs to re-acquire that SIB.

Observation#8: Area scope identifier is agnostic to geographic area and any groups of cells within the operator network regardless of geography can be configured for area-specific OSI.

Observation#9: Area validity for OSI is based on checking both the area scope identifier and valuetag of the stored SIB with that included in the SIB1 of the reselected cell.

Proposal#5: RAN2 to agree the area-specific OSI based on area scope identifier for 6GR.

## Spectrum aggregation

One of the key issues in wireless communication is how to improve spectrum efficiency. In 5G NR, two types of multi-carrier operations are specified: Carrier Aggregation (CA) and Supplementary UL (SUL), where CA is primarily used to increase UE data rate and SUL is targeted for better UL coverage.

In 6GR, the operators would have more discrete spectrums across different bands. In TR 38.914, 6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR, and 6GR aims to support flexible utilization of spectrum resources for DL and UL over different carriers/bands.

In RAN2#131bis meeting [2], there are following agreements:

Agreements on Spectrum aggregation

1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

From our perspective, one of the technologies for spectrum aggregation can be a single cell with multiple carriers (SCMC). In a single cell with multiple carriers, more than one uplink carriers and/or downlink carriers (CCs) can be included in one cell as shown in Figure 4. The CCs in the cell can be in the same band or in different bands, but it is preferred that they are in the same frequency range (e.g., either FR1 or FR2).

Figure 4 - A single cell with multiple CCs

In the single cell with multiple carriers, the PCI of the cell can be derived based on the synchronization signals in the DL carrier where SSB is transmitted. This technology has some benefits compared with legacy CA in 5G NR:

- Less common signal or channel which is beneficial for network energy saving and less radio resource overhead. For example, synchronization signals like SSBs, system information like SIB1/SIs and paging can be transmitted in one or part of the DL carriers. The other carriers can be used to e.g. provide capacity for eMBB traffic or higher reliability for URLLC traffic for UEs in RRC_Connected, and these carriers can be SSB-less carriers or can have on-demand SSB transmissions.

- Faster carrier switching or aggregation compared with legacy CA. In 5G NR carrier aggregation, the UE should be configured with one or more SCells first and then activate or deactivate an SCell based on the activation status in RRC reconfiguration message or the received SCell activation/deactivation MAC CE, which is slow and inefficient. In a single cell with multiple carriers, the switching or aggregation of the carriers in the cell can be performed by L1 signalling, like BWP switching in 5G NR, which is faster and can also save some signalling overhead.

- Random access distributed to carriers to select a better carrier based on UE traffic/type or reduce random access resource congestion. In a single cell with multiple carriers, the resource for random access can be configured in more than one carrier, so that the UE can select a carrier with low load or matching the UE’s traffic or type for random access.

- Flexible uplink and downlink carrier linking. In 5G NR carrier aggregation, the linkage between an uplink carrier and a downlink carrier is based on the cell configuration which is fixed. In a single cell with multiple carriers, the uplink carriers and downlink carriers can be linked flexibly based on the traffic load or coverage enhancements.

Based on the above mentioned benefits, we propose to study the technology of a single cell with multiple carriers in 6GR. To be specific, the following aspects can be further studied:

  - The carrier(s) on which common signals/channels are transmitted by NW. The common signals/channels may include SS, MIB, SIB, Paging, RACH, RAR and so on.

  - How UE determines/selects a carrier for SIB reception, paging reception and initial access.

  - Signalling for the configuration of common signals/channels on each carrier.

  - Consideration on RRC_Idle/Inactive UEs and RRC_Connected UEs.

  - RLM/RLF determination

  - Relationship/co-existence with legacy CA or cell aggregation.

Proposal#6: RAN2 to study a single cell with multiple carriers in 6GR.

# Conclusion

In this paper we presented some views on initial and system access aspects particularly system information provisioning and have come up with the following observations and proposals to be discussed and agreed:

Proposal#1: Requirements for SI provisioning in 6G

- 6GR system information can be provisioned in on-demand manner based on the UE request in the standalone scenario with and without support of the anchor cell.

- The concept of periodic non-overlapping staggered SI windows and the concept of BCCH modification period shall be revisited in study item. 6GR system information broadcast shall explore alternate concepts to the existing ones to be used for SI broadcasting and SI change during the study phase.

- 6GR study shall strive to simplify the on-demand system information request procedure to save the UE power, the network energy and the radio resources consumption for provisioning SI in on-demand manner.

Observation#1: RAN2 assume MIB will be periodically broadcast on PBCH. If SSB periodicity is adapted, then MIB periodicity can also be adaptive.

Observation#2: Contents of MIB partly depends on the scenarios to be supported for NES cell.

Observation#3: In real deployment majority of SIB1 contents for at least intra-frequency cells are same and hence no need to restrict SIB1 scope as solely cell-specific.

Obervation#4: If SIB1 is area-specific then UE need not re-acquire SIB1 upon cell reselection where area scope of SIB1 is valid thus saving UE battery.

Proposal#2: SIB1 in 6G can be specified as either cell-specific or area-specific depending on operator deployment scenario. RAN2 to further study what contents of SIB1 can be area-specific.

Obervation#5: SI change notification in 5G does not indicate which SIB has changed.

Obervation#6: Provisioning all OSI in connected mode through on-demand unicast avoids the need of periodic broadcast window.

Observation#7: If periodic broadcast SI windows are avoided then scheduling info for OSI is not needed to be included in SIB1, thus reducing SIB1 size and SIB1 re-acquisition during SI change.

Proposal#3: RAN2 to consider more granular SI change indication indicating which SIB has changed, to avoid re-acquiring SIB1.

Proposal#4: RAN2 to further study the feasibility of OSI provisioning in connected mode through on-demand unicast.

Observation#8: Area scope identifier is agnostic to geographic area and any groups of cells within the operator network regardless of geography can be configured for area-specific OSI.

Observation#9: Area validity for OSI is based on checking both the area scope identifier and valuetag of the stored SIB with that included in the SIB1 of the reselected cell.

Proposal#5: RAN2 to agree the area-specific OSI based on area scope identifier for 6GR.

Proposal#6: RAN2 to study a single cell with multiple carriers in 6GR.

# References

- RP-252912, Revised SID: Study on 6G Radio.

- R2-25xxxx, Chairman Notes, RAN2#131Bis meeting, 13-17 Oct, 2025, Prague.

- R2-2507131,	Initial consideration on Control plane aspects in 6G, Fujitsu.

- R2-2507327, Views on 6G control plane, NTT DOCOMO, INC.

- R2-2507373,	Overview of 6G Control Plane aspects, NEC.

- R2-2506799,	Considerations on 6GR control plane, vivo.

- R2-2507232,	Considerations for 6G Control Plane, Samsung, Verizon.

- R2-2507111,	Views on Directions of 6G Control Plane Enhancements, Apple.