3GPP TSG-RAN2 #132	R2-2509064

# Dallas, TX, USA, November 17 – November 21, 2025

Agenda item:	10.2.3 (FS_6G_Radio)

Source:	LG Electronics

Title: 	General considerations on 6G

Document for:	Discussion and decision

1.	Introduction

In RAN2#131bis meeting, it was discussed on general aspects for 6G, including the discussion on which service type should be considered for 6G Day-1 and whether/how to support the multiple device types in 6G.

This document further discusses general aspects of 6G, including the support of XR services, unified access checking procedure, and early indication to support device type.

2.	Discussion

2.1	XR features

2.1.1 XR functions standardized in NR

In 5G, the traffic characteristics of XR services have been studied, and the normative work has been completed in Rel-18 and Rel-19. The XR services require high data rate and low latency communications, and following functions were introduced in 5G to support XR traffics.

- Awareness

  - PDU Set based handling

- Power saving

  - DRX with non-integer periodicities

  - Retransmission-less CG

- Capacity

  - Multiple CG PUSCH transmission

  - BSR with low quantization error

  - Delay Status Report

  - PDU Set based discard

  - PSI based discard

  - PDCP SN gap report

  - Logical channel priority adjustment

  - Avoidance of unnecessary RLC retransmission

  - Remaining time based RLC retransmission

  - Remaining time based RLC polling

  - Uplink rate control

The above functions are minimum set to support XR services, and we believe they should be supported from 6G Day-1. Some modifications may be needed according to protocol changes in 6G, but the basic principle, e.g. PDU Set based handling, DRX with non-integer periodicities, Delay status report, Fine-granularity BSR, Retransmission enhancement, should be kept in 6G.

Proposal 1. The functions standardized in NR Rel-18 and Rel-19 should be standardized from 6G Day-1.

2.1.2	XR functions studied but not standardized in NR

We think the functions standardized in NR Rel-18 and Rel-19 are not enough to support more divergent traffics expected in 6G. Given the 5G functions as baseline, it is required to study more advanced functions to support XR type traffics in 6G.

In should be noted that RAN2 already had deep study in Rel-17 for XR, and the outcome of the study was captured in TR 38.835. During the study, many functions were discussed, but not all functions were standardized. The list of functions studied in TR 38.835 but not standardized are as shown below:

- DRB mapping to different L2 entities

- Multiple active DRX patterns

- Jitter consideration on CG

We think those functions are essential to support divergent XR traffics in 6G, and thus we propose to standardize them from 6G Day-1.

Proposal 2. The functions studied but not standardized in NR should be standardized from 6G Day-1.

2.1.3	XR functions not studied in NR

In addition to already studied functions, we think it is necessary to introduce additional functions to support the advancement of the XR related technologies. Following aspects need to be considered:

- Data burst handling

  - The burstiness of XR traffic characteristics becomes remarkable when operating with AI systems. The generative AI application produces data bursts with irregular periodicities and irregular data volume. Thus, it should be studied how to efficiently transmit the data burst within a stringent delay bound. For the network to efficiently schedule UL resource for data bursts, UEs may need to provide more detailed information for data bursts, e.g. data burst size, data burst delay bound, end of data burst, etc. These aspects should be considered when designing BSR/DSR in 6G.

- Per-packet handling

  - In 5G, it is assumed that packets transmitted through a single DRB have similar QoS characteristics, and thus there is no need to apply different QoS handling for those packets. However, new emerging services like AI and XR make such assumption obsolete. It is identified that AI and XR-like traffic flow has a mixed characteristic within the same traffic flow, and it’s time to consider per-packet based QoS handling within a single DRB. Mixture of lossless and lossy, and delay-critical and delay-tolerant characteristics in a single DRB should be considered from 6G Day-1.

- L4S support

  - TCP congestion control mechanism is now changing from loss-based mechanism (e.g. Tahoe, Reno) to delay-based mechanism (e.g. BBR, L4S). The L4S (Low Latency, Low Loss, Scalable throughput) is rapidly growing delay-based congestion control mechanism, and it is expected that the L4S becomes dominant in 6G. However, in 5G, the L4S is considered only in network interface (e.g. ECN marking in UPF and gNB) and not considered in radio protocols, which makes the L4S performance gain not fully achieved. The 6G radio protocols should be designed considering L4S from Day-1.

Proposal 3: Standardize more enhanced XR functions from 6G Day-1, e.g. Data burst handling, Per-packet handling, L4S support.

2.2	Unified access checking procedure

In NR, there are two access control mechanisms:

- 1) Cell status and special reservations for control of cell selection and reselection procedures

- 2) Unified Access Control, which allows preventing selected access categories or access identities from sending initial access messages for load control reasons.

It is well understood that Unified Access Control introduced in NR successfully unified the fragmented access control features defined in LTE, based on a common access control procedure with access categories and access identifies. In contrast, access control related to cell status and cell reservations in NR remains fragmented like LTE, which unnecessarily complicates access barring checking, resulting in the specifications hard to understand and error prone.

In NR, there are several cell status indications introduced in different releases, as follows:

- cellBarred (IE type: "barred" or "not barred") Indicated in MIB message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is ignored by UEs supporting NTN for NTN access, or by UEs supporting ATG for ATG access. - cellBarredATG (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to ATG UEs. - cellBarred2RxXR (IE type: "barred") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to 2Rx XR UEs. - cellBarred-eRedCap1Rx (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to eRedCap UEs. - cellBarred-eRedCap2Rx (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to eRedCap UEs. - cellBarredFixedVSAT (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. This field is only applicable to VSAT UEs using NTN access. - cellBarredMobileVSAT (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. This field is only applicable to VSAT UEs using NTN access. - cellBarredNES (IE type: "not barred") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to UEs indicating any of the values in nes-CellDTX-DRX as specified in TS 38.306 [24]. - cellBarredNTN (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. This field is ignored if the UE does not support NTN access. - cellBarredRedCap1Rx (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to RedCap UEs. - cellBarredRedCap2Rx (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to RedCap UEs. - halfDuplexRedCapAllowed (IE type: "true") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to (e)RedCap UEs. - iab-Support (IE type: "true") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is specified per PLMN or per SNPN. This field is only applicable to IAB-MT. - ncr-Support (IE type: "true") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to NCR-MT. - mobileIAB-Support (IE type: "true") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is specified per PLMN or per SNPN. This field is only applicable to mobile IAB-MT.

In 5G, access barring checking procedure related to cell staus is specified in fragmentend manner across 38.304 and 38.331 without a clear splitting rule. Behaviours are distributedly specified across procedure text in 38.304 and 38.331 procedure text (5.2.2.4.1, 5.2.2.4.2) and 38.331 ANS.1 field description (for those in MIB and SIB1) , and some behaviours are duplicated across the specifications. This made the access checking procedure hard to understand and quite error prone. Note that there have been quite many CRs approved in 5G to clarify and/or correct access control related cell status fields or just to resolve misalignment between 38.304 and 38.331. RAN2 acknolwedged this problem and tried some clean-up efforts, but limited clean-up was done in the end because there was a serious worry of introducing unintended big errors if big changes were made to the existing procedure text (note again that the existing text is already error prone). 6G specifications should not suffer from the same problem.

Observation 1. In NR, access barring checking procedures related to cell status fields are distributed across TS 38.304 and TS 38.331 with duplications, leading to hard-to-follow and error-prone procedures (as evidenced by many clarification/correction CRs).

In 5G, some cell barring status fiels use the code point {“barred”, “not barred”} and others use the code point {“not barred”},{“barred”} or {true}. Therefore, description of behaviours for different field types are slightly different, and the meaning of absence of those fields are alo different across different types. This made the acces checking procedure unnecessarily complicated, hard to understand for readers and error prone. It would be vauable to study if multiple types are cell status field are indeed necessary in 6G and whether we could minimize the type of cell status fields, aiming for a common handling of the cell status fields.

Observation 2. Multiple code-point conventions and absence semantics across cell status fields force per-field handling and increase implementation/readability errors.

In 5G, the handling of cellBarred in MIB is not consistent. cellBarred in MIB governs access barring for UEs, but this field may be ignored or overridden in some cases. For example, cellBarred in MIB is ignored by UE for NTN access or ATG access and separate cell barring fields in SIB1 dedicated for NTN/ATG replace cellBarred in MIB.  For NES capable UE, cellBarred is ignored if cellBarredNES is present inSIB1. RAN2 can evaluate wether a consistent handling of cellBarred is possible for all cases (both known and unknown).

Observation 3. Handling of cellBarred in MIB is inconsistent (ignored or overridden in case NTN/ATG/NES-specific cell status field is present in SIB1), creating ambiguity about precedence and inconsistent UE behavior.

In 5G, some cell status fields are common for all PLMNs and NPNs while others support PLMN/NPN-specific cell status fields. Whenever new cell status field is introduced, it was discussed on the fly whether it is per PLMN/NPN or common. In 6G, more flexible approach can be considered. For example, other than cellBarred in MIB, any cell status field can be configured as either of PLMN/NPN-common or PLMN/NPN-specific by default. This allows operators to get higher degree of freedom in controlling access as per their needs.  RAN2 may study whether cellBarred in MIB can be also configured per PLMN/NPN-specific (considering the size restriction of MIB).

For some cell barring status fields, intra frequency reselection infication (IFRI) were introduced but not for others. In 5G, there were no clear criteria on whether to introduce a new IFRI when a new cell barring statu is introduced.  In 6G, it is better to establish a guding rule on whether a separate IFRI is needed or not, or whether it is better for network to be able to configure a separate IFRI, if needed, for each of any cell barring status fields via a configuration option.

Observation 4. Whether a cell-status field is common vs PLMN/NPN-specific, and whether an IFRI is introduced, have been decided case-by-case without guiding criteria, yielding inconsistent operator control.

Based on these observations, we propose that RAN2 group study how to simplify/unify access control related to cell status fields in 6G

- For cell barring status fields, a common type of cell status IE is used for (possibly) all different cell status fields. For example, a single type of IE with code point {“barred”, “not barred”} is used for all different cell status fields in 6G, instead of defining multiple types of cell status IEs (like 5G has defined {“barred”, “not barred”},{“barred”}{“not barred”} and{“true”}).

- A clear rule is defined to make ensure a clear split of specifications across xx.304 and xx.331, aiming for maximal consistency and readability as well as minimal duplications.

- The simplified/unified access checking procedure related to	cell status and cell reservations is forward compatible such that the common procedure is applicable to new cell status/reservation fields introduced later than 6G Day-1, with no/minimal changes. One potential approach could be that a fixed size array of cell status fields is pre-defined in 6G Day-1, where first N fields of the arrays are mapped to cell status fields to be introduced in 6G Day-1 and the rest are reserved for new cell status fields to be introduced in 6G Day-2 or beyond. This approach may be similar to UAC, where an array of Access Categories has been predefined in initial 5G release (standalone drop) and a common access control procedure was defined to be applied to any AC (ACs defined in the initial 5G release as well as new ACs to be introduced later).

- Any cell status field is configured as either of PLMN/NPN-common or PLMN/NPN-specific, FFS for cellBarred in MIB.

- Establishing a guiding principle to determine whether IFRI that is specific to each cell status field is needed or not. Enabling a configuration option for any of cell status fields can be also considered.

Proposal 4. Study a simplified/unified access checking procedure related to cell status for 6G.

2.3	Early indication for device type

In 5G NR, various early indications for device types or functional capabilities are supported. Here, the early indication for device type (via Msg1 or Msg3) can be performed as follows:

- via Msg1 (by RA resource partitioning) or

- via Msg3 (by separated LCID values for (e)RedCap UE, or

In addition, there are several early indications for feature/functionalities as follows:

- Msg1/MsgA PRACH: applicability of slice group, Msg1/Msg3 repetition, SDT, SBFD-aware UE (by using SBFD RO)

- Msg3/MsgA PUSCH (by separated LCID): Msg4 PDSCH repetition, PUCCH repetition for Msg4 HARQ-ACK

On the other hand, since it is expected to support the diverse device types in 6G, the separate handling for each data type at early stage in initial access would be beneficial, considering the different capabilities of each device type. For example, if the XR-like devices support up to 100 MHz bandwidth while RedCap-like devices support only up to 20 MHz bandwidth, indicating the device type before the UE capability transfer would help the network to allocate resource for message exchanges efficiently from the early stage. Therefore, as early indication for (e)RedCap UE is introduced in order to avoid the restriction of resource allocation for normal UE, it would be beneficial to indicate the UE type as soon as possible to allow suitable resource allocation for each UE type while keeping the resource efficiency.

Further, the indication of UE type should be scalable with forward compatibility. In NR, various early indications are introduced in later releases, causing additional issues. In detail, Msg3-based early indication, (e)RedCap UE is introduced in later releases, causing additional complexity on defining separated LCID values for combination of device type and/or features (e.g., additional LX field). For Msg1-based early indication, resource fragmentation issue has occurred in NR, since the number of partition increases exponentially due to feature combination.

Therefore, in order to support the early indication without complexity issues identified in NR, the early indication framework should be considered in 6G from Day-1 with scalability.

Proposal 5. Study early indication of device type from 6G Day-1 with scalability for diverse device types.

3. Conclusion

Based on the above discussions, we present the following proposals:

Support of XR services

Proposal 1. The functions standardized in NR Rel-18 and Rel-19 should be standardized from 6G Day-1.

Proposal 2. The functions studied but not standardized in NR should be standardized from 6G Day-1.

Proposal 3: Standardize more enhanced XR functions from 6G Day-1, e.g. Data burst handling, Per-packet handling, L4S support.

Unified access checking framework

Observation 1. In NR, access barring checking procedures related to cell status fields are distributed across TS 38.304 and TS 38.331 with duplications, leading to hard-to-follow and error-prone procedures (as evidenced by many clarification/correction CRs).

Observation 2. Multiple code-point conventions and absence semantics across cell status fields force per-field handling and increase implementation/readability errors.

Observation 3. Handling of cellBarred in MIB is inconsistent (ignored or overridden in case NTN/ATG/NES-specific cell status field is present in SIB1), creating ambiguity about precedence and inconsistent UE behavior.

Observation 4. Whether a cell-status field is common vs PLMN/NPN-specific, and whether an IFRI is introduced, have been decided case-by-case without guiding criteria, yielding inconsistent operator control.

Proposal 4. Study a simplified/unified access checking procedure related to cell status for 6G.

Early indication for device type

Proposal 5. Study early indication of device type from 6G Day-1 with scalability for diverse device types.